Bathymetry Compilation for Proposed Marine Protected Areas in East Antarctica

Technical Report

Geoscience Australia
Record 2013/24

O. A. Wilson

Department of Resources, Energy and Tourism

Minister for Resources and Energy: The Hon Gary Gray AO MP
Secretary: Mr Blair Comley, PSM

Geoscience Australia

Chief Executive Officer: Dr Chris Pigram
This paper is published with the permission of the CEO, Geoscience Australia

© Commonwealth of Australia (Geoscience Australia) 2013

With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence. (http://www.creativecommons.org/licenses/by/3.0/au/deed.en)

Geoscience Australia has tried to make the information in this product as accurate as possible. However, it does not guarantee that the information is totally accurate or complete. Therefore, you should not solely rely on this information when making a commercial decision.

Geoscience Australia is committed to providing web accessible content wherever possible. If you are having difficulties with accessing this document please contact clientservices@ga.gov.au.

ISSN 1448-2177 (Print)
ISSN 2201-702X (PDF)

ISBN 978-1-922201-55-3 (Print)
ISBN 978-1-922201-56-0 (PDF)

GeoCat 74551

Bibliographic reference: Wilson, O. A. 2013. Bathymetry Compilation for Proposed Marine Protected Area in East Antarctica: Technical Report. Record 2013/24. Geoscience Australia: Canberra.

Version: 1301

Contents

List of Abbreviations	iv

Executive Summary	1

1. Introduction	2

2. Data Sources	4

3. Data Processing	12

3.1. Singlebeam Data	12

3.1.1. Adjustment of Sound Velocity	12

3.1.2. Loading data into CARISTM HIPS (CARISTM, 2011a)	13

3.1.3. Processing of Navigation Data	14

3.1.4. Processing of Depth Data	14

3.2. Multibeam Data	14

3.2.1. Processing of Navigation Data	14

3.2.2. Adjustment of Sound Velocity	15

3.2.3. Processing of Depth Data	15

4. Data Presentation	16

4.1. Projection	16

4.2. Surface Creation	17

4.3. Export of Soundings	18

5. Data Deliverables	19

5.1. Filetypes	19

5.1.1. Interpolated Gridded Surface	19

5.1.2. ArcMapTM GIS Files	19

5.1.3. GeoTIFs	19

5.1.4. Soundings	19

5.1.5. CARISTM Easyview	19

5.2. Individual MPAs	21

5.2.1. Overall Marine Protected Area (MPA) Locations	21

5.2.2. Mertz proposed Marine Protected Area	22

5.2.3. Wilkes proposed Marine Protected Area	23

5.2.4. Drygalski proposed Marine Protected Area	24

5.2.5. Prydz proposed Marine Protected Area	25

5.2.6. Mac.Robertson proposed Marine Protected Area	26

5.2.7. Enderby proposed Marine Protected Area	27

5.2.8. Gunnerus proposed Marine Protected Area	28

Acknowledgements	29

References	30

Appendix A. Determination of Approximate Mean Sound Velocity.	31

List of Abbreviations

AAD		Australian Antarctic Division

ANU		Australian National University

ANUDEM	Australian National University Digital Elevation Modelling

CCAMLR	Commission for the Conservation of Antarctic Marine Living Resources

CUBE		Combined Uncertainty and Bathymetry Estimation

GA		Geoscience Australia

JCU		James Cook University

MPA		Marine Protected Area

MSL		Mean Sea Level

RSMPA	Representative System of Marine Protected Areas

TPU		Total Propagated Uncertainty

UTM		Universal Transverse Mercator

USGS		United States Geological Survey

GIS		Geographic Information System

Executive Summary

The Australian Antarctic Division (AAD) has developed a proposal for the establishment of seven Marine Protected Areas (MPAs) located around east Antarctica for the purposes of marine ecosystem conservation. As seafloor morphology is a key component of marine ecosystems, this bathymetry compilation for the proposed MPAs was produced to support the AAD proposal. All bathymetry data available to Geoscience Australia at the time of compilation were used. This included multibeam and singlebeam acoustic data which were verified and processed to ensure the data were as accurate as possible. Processing included sound velocity corrections, navigation verification and the rejection of erroneous data points. Once processed, the data were gridded to 100m resolution and projected into suitable WGS84 UTM zones. The gridded data was exported into several formats to facilitate ease of use. The formats include xyz files, ESRI rasters, geoTIFs, CARISTM image files and soundings.

# Introduction

This bathymetry compilation was produced by Geoscience Australia to help inform the development of a Representative System of Marine Protected Areas (RSMPA) in east Antarctica. The results were presented at the Commission for the Conservation of Antarctic Marine Living Resources (CCAMLR) Workshop on Marine Protected Areas held in Brest, France in August 2011.

There are seven proposed MPAs (listed as follows from east to west); Mertz, Wilkes, Drygalski, Prydz, Mac.Robertson, Enderby and Gunnerus. These areas were selected based on previous bioregionalisation studies conducted by the AAD and are shown in Figure 1. It should be noted that the MPA boundaries shown in this report are as specified in 2011 and that revised boundaries were released in December 2012.

This technical report details the sourcing, processing and presentation of bathymetric data for the seven proposed MPAs.

Figure 1: Location of the seven proposed MPAs on the east Antarctic coast (South Pole Stereographic Projection). The three Australian Antarctic Bases (Mawson, Davis and Casey) are shown.

# Data Sources

The tables below detail the sources of bathymetric data that were used to create the bathymetric compilations for each proposed MPA. The source institutions listed hold Intellectual Property ownership of the raw data. The data presented in this report may differ from the raw data which has been processed at Geoscience Australia to assure its quality and to enable data integration for each prepared MPA.

Table 1a: Mertz MPA Multibeam Data Sources.

| Source Institution | Survey Name/ Code | GA Survey Ref No. | Vessel | Voyage Start | Voyage End | System |
| --- | --- | --- | --- | --- | --- | --- |
| University of California, San Diego | Dunedin-Antarctica-Fremantle CLIVAR/18S | GA-2493 | RV Roger Revelle | 6/02/2007 | 2/03/2007 | Simrad EM-120 12kHz |
| Lamont Doherty Earth Observatory | NBP0008 | GA-2327 | RV Nathaniel B Palmer | 20/12/2000 | 25/01/2001 | Seabeam 2112 12kHz |
| Laboratoire des Sciences du Climat et de L’Environment | CADO | GA-2345 | RV Marion Dufresne | 23/01/2003 | 17/02/2003 | Thompson Marconi TSM 5625 |

Table 1b: Mertz MPA Singlebeam Data Sources.

| Source Institution | Survey Name/ Code | GA Survey Ref No. | Vessel | Voyage Start | Voyage End | System |
| --- | --- | --- | --- | --- | --- | --- |
| AAD | 199293090 | GA-2167 | RV Aurora Australis | 11/03/1993 | 03/04/1993 | EK38 |
| AAD | 199394070 | GA-2171 | RV Aurora Australis | 01/01/1994 | 01/3/1994 | EK38 |
| AAD | 199495010 | GA-2172 | RV Aurora Australis | 31/08/1994 | 19/10/1994 | EK38 |
| AAD | 199495040 | GA-2175 | RV Aurora Australis | 13/12/1994 | 02/02/1995 | EK38 |
| AAD | 199495070 | GA-2177 | RV Aurora Australis | 11/04/1995 | 08/05/1995 | EK38 |
| AAD | 199596010 | GA-2178 | RV Aurora Australis | 17/07/1995 | 02/09/1995 | EK38 |
| AAD | 199798070 | GA-2187 | RV Aurora Australis | 03/04/1998 | 22/05/1998 | EK38 |
| AAD | 199900010 | GA-2191 | RV Aurora Australis | 13/07/1999 | 07/09/1999 | EK38 |
| AAD | 199900040 | GA-2193 | RV Aurora Australis | 22/11/1999 | 21/01/2000 | EK38 |
| AAD | 200102030 | GA-2200 | RV Aurora Australis | 28/10/2001 | 12/12/2001 | EK38 |
| AAD | 199697010 | GA-2513 | RV Aurora Australis | 22/08/1996 | 25/09/1996 | EK38 |
| AAD | 200809030 | GA-3662 | RV Aurora Australis | 30/12/2008 | 20/02/2009 | EK38 |
| AAD | 200708030 | GA-4330 | RV Aurora Australis | 13/12/2009 | 28/10/2009 | EK38 |

Table 2a: Wilkes MPA Multibeam Data Sources

| Source Institution | Survey Name/ Code | GA Survey Ref No. | Vessel | Voyage Start | Voyage End | System |
| --- | --- | --- | --- | --- | --- | --- |
| University of California, San Diego | Dunedin-Antarctica-Fremantle CLIVAR/18S | GA-2493 | RV Roger Revelle | 6/02/2007 | 2/03/2007 | Simrad EM-120 12kHz |

Table 2b: Wilkes MPA Singlebeam Data Sources

| Source Institution | Survey Name/ Code | GA Survey Ref No. | Vessel | Voyage Start | Voyage End | System |
| --- | --- | --- | --- | --- | --- | --- |
| AAD | 199394070 | GA-2171 | RV Aurora Australis | 01/01/1994 | 01/3/1994 | EK38 |
| AAD | 199798070 | GA-2187 | RV Aurora Australis | 03/04/1998 | 22/05/1998 | EK38 |
| AAD | 199900040 | GA-2193 | RV Aurora Australis | 22/11/1999 | 21/01/2000 | EK38 |
| AAD | 199900050 | GA-2194 | RV Aurora Australis | 24/01/2000 | 16/02/2000 | EK38 |
| AAD | 200708010 | GA-3651 | RV Aurora Australis | 29/08/2007 | 16/10/2007 | EK38 |
| AAD | 200809030 | GA-3662 | RV Aurora Australis | 30/12/2008 | 20/02/2009 | EK38 |

Table 3a: Drygalski MPA Multibeam Data Sources

| Source Institution | Survey Name/ Code | GA Survey Ref No. | Vessel | Voyage Start | Voyage End | System |
| --- | --- | --- | --- | --- | --- | --- |
| University of California, San Diego | Dunedin-Antarctica-Fremantle CLIVAR/18S | GA-2493 | RV Roger Revelle | 6/02/2007 | 2/03/2007 | Simrad EM-120 12kHz |
| Alfred Wegener Institute | ANT-XXIII/9 | GA-2432 | RV Polar Stern | 2/02/2007 | 11/04/2007 | Atlas Hydrosweep DS2 |
| National Science Foundation | NBP0101 | GA-2365 | RV Nathaniel B Palmer | 5/02/2001 | 19/03/2001 | Seabeam 2112 12kHz |

Table 3b: Drygalski MPA Singlebeam Data Sources

| Source Institution | Survey Name/ Code | GA Survey Ref No. | Vessel | Voyage Start | Voyage End | System |
| --- | --- | --- | --- | --- | --- | --- |
| AAD | 288 | GA-0031 | RV Nella Dan | 19/10/1979 | 19/03/1980 | Raytheon, 3.5kHz |
| AAD | 295 | GA-0032 | RV Nella Dan | 18/10/1980 | 19/03/1981 | Raytheon, 3.5kHz |
| AAD | 110 | GA-0033 | RV Nella Dan | 17/11/1981 | 24/03/1982 | Raytheon, 3.5kHz |
| AAD | 199697050 | GA-0149 | RV Aurora Australis | 06/02/1995 | 12/04/1995 | EK38 |
| AAD | 199697050 | GA-0186 | RV Aurora Australis | 28/01/1997 | 29/03/1997 | EK38 |
| AAD | 200102070 | GA-1765 | RV Aurora Australis | 24/01/2002 | 07/03/2002 | EK38 |
| AAD | 199394020 | GA-1821 | RV Aurora Australis | 12/10/1993 | 17/11/1993 | EK38 |
| AAD | 198586050 | GA-2160 | RV Nella Dan | 04/01/1986 | 22/02/1986 | EK38 |
| AAD | 198687070 | GA-2161 | RV Nella Dan | 06/02/1987 | 22/02/1987 | EK38 |
| AAD | 199091060 | GA-2164 | RV Aurora Australis | 03/01/1991 | 19/03/1991 | EK38 |
| AAD | 199293010 | GA-2165 | RV Aurora Australis | 17/10/1992 | 23/11/1992 | EK38 |
| AAD | 199293070 | GA-2166 | RV Aurora Australis | 05/01/1993 | 09/03/1993 | EK38 |
| AAD | 199394040 | GA-2170 | RV Aurora Australis | 19/11/1993 | 28/12/1993 | EK38 |
| AAD | 199394070 | GA-2171 | RV Aurora Australis | 01/01/1994 | 01/03/1994 | EK38 |
| AAD | 199495010 | GA-2172 | RV Aurora Australis | 31/08/1994 | 19/10/1994 | EK38 |
| AAD | 199697020 | GA-2180 | RV Aurora Australis | 26/09/1996 | 24/11/1996 | EK38 |
| AAD | 199798020 | GA-2183 | RV Aurora Australis | 23/09/1997 | 11/11/1997 | EK38 |
| AAD | 199798040 | GA-2185 | RV Aurora Australis | 29/11/1997 | 22/01/1998 | EK38 |
| AAD | 199798050 | GA-2186 | RV Aurora Australis | 26/01/1998 | 23/02/1998 | EK38 |
| AAD | 199798070 | GA-2187 | RV Aurora Australis | 03/04/1998 | 22/05/1998 | EK38 |
| AAD | 199899010 | GA-2188 | RV Aurora Australis | 15/07/1998 | 31/07/1998 | EK38 |
| AAD | 199899060 | GA-2190 | RV Aurora Australis | 06/03/1999 | 07/09/1999 | EK38 |
| AAD | 199900020 | GA-2192 | RV Aurora Australis | 01/10/1999 | 19/11/1999 | EK38 |
| AAD | 199900040 | GA-2193 | RV Aurora Australis | 22/11/1999 | 21/01/2000 | EK38 |
| AAD | 199900060 | GA-2198 | RV Aurora Australis | 18/02/2000 | 30/03/2000 | EK38 |
| AAD | 200102050 | GA-2201 | RV Aurora Australis | 16/12/2001 | 23/01/2002 | EK38 |
| AAD | 200203040 | GA-3649 | RV Aurora Australis | 03/01/2003 | 19/03/2003 | EK38 |
| AAD | 200809020 | GA-3661 | RV Aurora Australis | 23/11/2008 | 20/02/2009 | EK38 |
| AAD | 200809030 | GA-3662 | RV Aurora Australis | 30/12/2008 | 20/02/2009 | EK38 |
| AAD | 200809050 | GA-3667 | RV Aurora Australis | 24/02/2009 | 26/03/2009 | EK38 |
| AAD | 200910020 | GA-3668 | RV Aurora Australis | 07/12/2009 | 22/01/2010 | EK38 |
| AAD | 200910030 | GA-3671 | RV Aurora Australis | 25/01/2010 | 27/02/2010 | EK38 |
| AAD | 200910040 | GA-3672 | RV Aurora Australis | 08/03/2010 | 28/03/2010 | EK38 |
| AAD | 201011010 | GA-3674 | RV Aurora Australis | 21/10/2010 | 02/12/2010 | EK38 |
| AAD | 200910010 | GA-4311 | RV Aurora Australis | 30/10/2009 | 04/12/2009 | EK38 |

Table 4a: Prydz MPA Multibeam Data Sources

| Source Institution | Survey Name/ Code | GA Survey Ref No. | Vessel | Voyage Start | Voyage End | System |
| --- | --- | --- | --- | --- | --- | --- |
| National Science Foundation | NBP0101 | GA-2365 | RV Nathaniel B Palmer | 5/02/2001 | 19/03/2001 | Seabeam 2112 12kHz |
| Alfred Wegener Institute | ANT-XXIII/9 | GA-2432 | RV Polar Stern | 2/02/2007 | 11/04/2007 | Atlas Hydrosweep DS2 |
| Geoscience Australia | Davis Base Harbour Survey | GA-4301 | RV Howard Burton | 15/02/2010 | 12/03/2010 | Kongsberg EM3002 300kHz |

Table 4b: Prydz MPA Singlebeam Data Sources

| Source Institution | Survey Name/ Code | GA Survey Ref No. | Vessel | Voyage Start | Voyage End | System |
| --- | --- | --- | --- | --- | --- | --- |
| AAD | 288 | GA-0031 | RV Nella Dan | 19/10/1979 | 19/03/1980 | Raytheon, 3.5kHz |
| AAD | 295 | GA-0032 | RV Nella Dan | 18/10/1980 | 19/03/1981 | Raytheon, 3.5kHz |
| AAD | 110 | GA-0033 | RV Nella Dan | 17/11/1981 | 24/03/1982 | Raytheon, 3.5kHz |
| AAD | 199697050 | GA-0149 | RV Aurora Australis | 06/02/1995 | 12/04/1995 | EK38 |
| AAD | 199697050 | GA-0186 | RV Aurora Australis | 28/01/1997 | 29/03/1997 | EK38 |
| AAD | 200102070 | GA-1765 | RV Aurora Australis | 24/01/2002 | 07/03/2002 | EK38 |
| AAD | 199394020 | GA-1821 | RV Aurora Australis | 12/10/1993 | 17/11/1993 | EK38 |
| AAD | 198586050 | GA-2160 | RV Nella Dan | 04/01/1986 | 22/02/1986 | EK38 |
| AAD | 198687070 | GA-2161 | RV Nella Dan | 06/02/1987 | 22/02/1987 | EK38 |
| AAD | 199091060 | GA-2164 | RV Aurora Australis | 03/01/1991 | 19/03/1991 | EK38 |
| AAD | 199293010 | GA-2165 | RV Aurora Australis | 17/10/1992 | 23/11/1992 | EK38 |
| AAD | 199293070 | GA-2166 | RV Aurora Australis | 05/01/1993 | 09/03/1993 | EK38 |
| AAD | 199394040 | GA-2170 | RV Aurora Australis | 19/11/1993 | 28/12/1993 | EK38 |
| AAD | 199394070 | GA-2171 | RV Aurora Australis | 01/01/1994 | 01/03/1994 | EK38 |
| AAD | 199495010 | GA-2172 | RV Aurora Australis | 31/08/1994 | 19/10/1994 | EK38 |
| AAD | 199495020 | GA-2173 | RV Aurora Australis | 22/10/1994 | 29/11/1994 | EK38 |
| AAD | 199596040 | GA-2179 | RV Aurora Australis | 19/01/1996 | 31/03/1996 | EK38 |
| AAD | 199798020 | GA-2183 | RV Aurora Australis | 23/09/1997 | 11/11/1997 | EK38 |
| AAD | 199798040 | GA-2185 | RV Aurora Australis | 29/11/1997 | 22/01/1998 | EK38 |
| AAD | 199798050 | GA-2186 | RV Aurora Australis | 26/01/1998 | 23/02/1998 | EK38 |
| AAD | 199899040 | GA-2189 | RV Aurora Australis | 28/10/1998 | 27/12/1998 | EK38 |
| AAD | 199899060 | GA-2190 | RV Aurora Australis | 06/03/1999 | 07/09/1999 | EK38 |
| AAD | 199900020 | GA-2192 | RV Aurora Australis | 01/10/1999 | 19/11/1999 | EK38 |
| AAD | 199900040 | GA-2193 | RV Aurora Australis | 22/11/1999 | 21/01/2000 | EK38 |
| AAD | 200001060 | GA-2195 | RV Aurora Australis | 31/12/2000 | 10/03/2001 | EK38 |
| AAD | 199192060 | GA-2197 | RV Aurora Australis | 09/01/1992 | 27/03/1992 | EK38 |
| AAD | 199900060 | GA-2198 | RV Aurora Australis | 18/02/2000 | 30/03/2000 | EK38 |
| AAD | 200102050 | GA-2201 | RV Aurora Australis | 16/12/2001 | 23/01/2002 | EK38 |
| AAD | 200203040 | GA-3649 | RV Aurora Australis | 03/01/2003 | 19/03/2003 | EK38 |
| AAD | 200506030 | GA-3650 | RV Aurora Australis | 02/01/2006 | 14/03/2006 | EK38 |
| AAD | 200809010 | GA-3659 | RV Aurora Australis | 12/10/2008 | 21/11/2008 | EK38 |
| AAD | 200809020 | GA-3661 | RV Aurora Australis | 23/11/2008 | 26/12/2008 | EK38 |
| AAD | 200809050 | GA-3667 | RV Aurora Australis | 24/02/2009 | 26/03/2009 | EK38 |
| AAD | 200910020 | GA-3668 | RV Aurora Australis | 07/12/2009 | 22/01/2010 | EK38 |
| AAD | 200910030 | GA-3671 | RV Aurora Australis | 25/01/2010 | 27/02/2010 | EK38 |
| AAD | 200910040 | GA-3672 | RV Aurora Australis | 08/03/2010 | 28/03/2010 | EK38 |
| AAD | 201011010 | GA-3674 | RV Aurora Australis | 21/10/2010 | 02/12/2010 | EK38 |
| AAD | 200910010 | GA-4311 | RV Aurora Australis | 30/10/2009 | 04/12/2009 | EK38 |

Table 5a: Mac.Robertson MPA Multibeam Data Sources

| Source Institution | Survey Name/ Code | GA Survey Ref No. | Vessel | Voyage Start | Voyage End | System |
| --- | --- | --- | --- | --- | --- | --- |
| National Science Foundation | NBP0101 | GA-2365 | RV Nathaniel B Palmer | 5/02/2001 | 19/03/2001 | Seabeam 2112 12kHz |
| Alfred Wegener Institute | ANT-XXIII/9 | GA-2432 | RV Polar Stern | 2/02/2007 | 11/04/2007 | Atlas Hydrosweep DS2 |

Table 5b: Mac.Robertson MPA Singlebeam Data Sources

| Source Institution | Survey Name/ Code | GA Survey Ref No. | Vessel | Voyage Start | Voyage End | System |
| --- | --- | --- | --- | --- | --- | --- |
| AAD | 288 | GA-0031 | RV Nella Dan | 19/10/1979 | 19/03/1980 | Raytheon, 3.5kHz |
| AAD | 295 | GA-0032 | RV Nella Dan | 18/10/1980 | 19/03/1981 | Raytheon, 3.5kHz |
| AAD | 110 | GA-0033 | RV Nella Dan | 17/11/1981 | 24/03/1982 | Raytheon, 3.5kHz |
| AAD | 199697050 | GA-0149 | RV Aurora Australis | 06/02/1995 | 12/04/1995 | EK38 |
| AAD | 199697050 | GA-0186 | RV Aurora Australis | 28/01/1997 | 29/03/1997 | EK38 |
| AAD | 200102070 | GA-1765 | RV Aurora Australis | 24/01/2002 | 07/03/2002 | EK38 |
| AAD | 199394020 | GA-1821 | RV Aurora Australis | 12/10/1993 | 17/11/1993 | EK38 |
| AAD | 198586050 | GA-2160 | RV Nella Dan | 04/01/1986 | 22/02/1986 | EK38 |
| AAD | 198687070 | GA-2161 | RV Nella Dan | 06/02/1987 | 22/02/1987 | EK38 |
| AAD | 199091060 | GA-2164 | RV Aurora Australis | 03/01/1991 | 19/03/1991 | EK38 |
| AAD | 199293070 | GA-2166 | RV Aurora Australis | 05/01/1993 | 09/03/1993 | EK38 |
| AAD | 199394040 | GA-2170 | RV Aurora Australis | 19/11/1993 | 28/12/1993 | EK38 |
| AAD | 199394070 | GA-2171 | RV Aurora Australis | 01/01/1994 | 01/03/1994 | EK38 |
| AAD | 199495010 | GA-2172 | RV Aurora Australis | 31/08/1994 | 19/10/1994 | EK38 |
| AAD | 199798020 | GA-2183 | RV Aurora Australis | 23/09/1997 | 11/11/1997 | EK38 |
| AAD | 199798040 | GA-2185 | RV Aurora Australis | 29/11/1997 | 22/01/1998 | EK38 |
| AAD | 199899060 | GA-2190 | RV Aurora Australis | 06/03/1999 | 07/09/1999 | EK38 |
| AAD | 199900020 | GA-2192 | RV Aurora Australis | 01/10/1999 | 19/11/1999 | EK38 |
| AAD | 199900040 | GA-2193 | RV Aurora Australis | 22/11/1999 | 21/01/2000 | EK38 |
| AAD | 200001060 | GA-2195 | RV Aurora Australis | 31/12/2000 | 10/03/2001 | EK38 |
| AAD | 199192060 | GA-2197 | RV Aurora Australis | 09/01/1992 | 27/03/1992 | EK38 |
| AAD | 199900060 | GA-2198 | RV Aurora Australis | 18/02/2000 | 30/03/2000 | EK38 |
| AAD | 200102050 | GA-2201 | RV Aurora Australis | 16/12/2001 | 23/01/2002 | EK38 |
| AAD | 200203040 | GA-3649 | RV Aurora Australis | 03/01/2003 | 19/03/2003 | EK38 |
| AAD | 200506030 | GA-3650 | RV Aurora Australis | 02/01/2006 | 14/03/2006 | EK38 |
| AAD | 200809030 | GA-3662 | RV Aurora Australis | 30/12/2008 | 20/02/2009 | EK38 |
| AAD | 200910030 | GA-3671 | RV Aurora Australis | 25/01/2010 | 27/02/2010 | EK38 |

Table 6a: Enderby MPA Multibeam Data Sources

| Source Institution | Survey Name/ Code | GA Survey Ref No. | Vessel | Voyage Start | Voyage End | System |
| --- | --- | --- | --- | --- | --- | --- |
| Alfred Wegener Institute | ANT-XXIII/9 | GA-2432 | RV Polar Stern | 2/02/2007 | 11/04/2007 | Atlas Hydrosweep DS2 |

Table 6b: Enderby MPA Singlebeam Data Sources

| Source Institution | Survey Name/ Code | GA Survey Ref No. | Vessel | Voyage Start | Voyage End | System |
| --- | --- | --- | --- | --- | --- | --- |
| AAD | 200506030 | GA-3650 | RV Aurora Australis | 02/011/2006 | 14/03/2006 | EK38 |
| AAD | 200809030 | GA-3662 | RV Aurora Australis | 30/12/2008 | 20/02/2009 | EK38 |

Table 7: Gunnerus MPA Singlebeam Data Sources (no multibeam data available)

| Source Institution | Survey Name/ Code | GA Survey Ref No. | Vessel | Voyage Start | Voyage End | System |
| --- | --- | --- | --- | --- | --- | --- |
| AAD | 200506030 | GA-3650 | RV Aurora Australis | 02/011/2006 | 14/03/2006 | EK38 |

# Data Processing

## Singlebeam Data

### Adjustment of Sound Velocity

The majority of AAD singlebeam data was provided in MGD77 format. MGD77 Format was developed by the National Geosphysical Data Center under the US Department of Commerce. It is designed as an exchange format for marine geophysical data (MGD) (bathymetry, magnetics, gravity and seismic navigation) and was first conceived of in 1977 (NGDC, 2010). In this format, the header specifies the speed of sound value used to calculate depth from the two-way travel time measured by the singlebeam instrument (Figure 2).

Figure 2: Excerpt from MGD77 formatted data file. The speed of sound field is highlighted in blue.

All AAD singlebeam data provided in MGD77 format (as shown in Figure 2) specified that a speed of sound value of 1500 m/s was used to calculate depth. The two-way travel time field was not populated. It is assumed that a value of 1500 m/s was actually used in calculations and not added into the header at some point during a file conversion process.

A value of 1500 m/s is a generalised figure for the speed of sound in water . It is shown by the collected sound velocity profiles embedded in the multibeam data to be an unrealistic value for Antarctic waters and if used would lead to significant depth errors. Therefore, the AAD singlebeam datasets required some adjustment. Using the sound velocity profiles stored with the Antarctic multibeam data, a mean sound velocity value was calculated (for sound velocity profiles and mean calculations, see Appendix A). The mean sound velocity calculated from the profiles used with the multibeam data was found to be:m/s, with a standard deviation of ~16 m/s and m/s.

The following formula is used to calculate singlebeam depth values from two way travel time. Time in this case refers to half the two way travel time :

The conversion factor to adjust the depth values of the MGD77 Singlebeam data for a more realistic speed of sound (1460m/s) was determined as follows:

All the depth values in the MGD77 formatted singlebeam files were then multiplied by this conversion factor. The header was adjusted accordingly.

### Loading data into CARISTM HIPS (CARISTM, 2011a)

CARISTM HIPS is a comprehensive hydrographic processing software system. It is comprised of numerous tools to facilitate the accurate import, processing and export of bathymetry data.

All MGD77 files were opened in CARISTM HIPS software using the Generic Data Parser import tool. This tool allows the user to specify the details of an import format. The details of that format can then be saved as *.par file and used again for the same filetypes in the future. The customised *.par file used for the import of the MGD77 data is stored in the following directory on the data drive: AAD_MPA_Deliverables\01_RAW_DATA\08_ALL_SINGLBEAM_MGD77.

The data files were taken from long voyages with continuous data collection. In order to import them into CARISTM HIPS, they first had to be split up into separate files that covered time periods of less than 24 days. This is a restriction of the software, which is explained in the following paragraph. A script was written for this purpose: ‘Split_MGD77’ stored on the data drive in: AAD_MPA_Deliverables\01_RAW_DATA\08_ALL_SINGLBEAM_MGD77\MGD77_COMPLETE_1460ms.

This was necessary because CARISTM HIPS 7.0 organises the viewing of data by line and stores timestamps internally as scaled 32-bit integers as an offset in milliseconds from the reference time (which is taken as the first timestamp in the data set). This means a single HIPS line can only store data that spans 2 ^ 31 / 1000.0 seconds (= 2147483.648, approximately 24 days).

### Processing of Navigation Data

Once the MGD77 files had been imported into CARISTM HIPS, the navigation data could be viewed on the display window and edited using the tool ‘Navigation Editor’. All anomalous data was then ‘rejected with interpolation’. That is, if a position point was significantly different from the ship's course, the position could be rejected and the course interpolated between the preceding and subsequent position point. Full lines were processed for navigation error, not just within the MPA boundaries.

It should be noted that despite all positioning data for the singlebeam data being provided to a precision of 5 decimal places of a degree, this does not necessarily reflect the accuracy of the data. Data collected from the vessel RSV Aurora Australis is listed as having been positioned by DGPS. However, data collected from the MV Nella Dan was positioned by the Transit Satellite Navigation System Tracor MK (GA, 1980).

The navigation tracks of the vessel using Transit Satellite Navigation occasionally showed a distinct zigzag pattern. It can be assumed that this occurred when the satellite positioning was intermittently available and the vessel alternated between satellite positioning and dead reckoning.

These patterns were not edited out of the datasets unless the bathymetry appeared anomalous. However, the poor accuracy of the positioning should be considered when depth is required at an exact position.

### Processing of Depth Data

Once the data files were corrected for navigation, a ‘zero tide’ file was applied to allow for data editing in CARISTM HIPS. A zero tide file is a file formatted as a tide file, but with zero values for all tide readings. This file can be applied to satisfy the software’s requirement for the application of tide when no tide application is necessary.

Since the majority of the data was in water deeper than 500m, it is assumed that all depth data is referenced to the vertical datum MSL.

The data was then area-based edited, using the 3D Subset Editor facility in CARISTM HIPS, meaning that the data was assessed visually, addressing small areas at a time. This meant that depth outliers could be visually identified. Lines that were obviously erroneous, that showed consistent significantly different depth values to surrounding data from other sources, were completely removed. Lines with some noise were 'cleaned' by rejecting the anomalous depth values.

## Multibeam Data

### Processing of Navigation Data

The multibeam data was loaded into CARISTM HIPS and the navigation data within the MPA regions was edited for ‘spikes’ and navigation ‘jumps’ using the ‘Navigation Editor’ tool. This tool allows the navigation data to be isolated from other data and viewed and edited easily. As with the singlebeam data, the majority of positioning anomalies were rejected with interpolation.

### Adjustment of Sound Velocity

The multibeam swath was viewed by line (where a line is a fragment of the dataset determined at collection) using the ‘Swath Editor’ tool in CARISTM HIPS. The swath editor allows the line to be broken down and viewed from numerous angles in order to more easily identify erroneous data. In areas where an obvious sound velocity error appeared, the ‘Refraction Editor’ was used to adjust the swath. Typically, sound velocity error can be identified by a concave or convex bend across the swath. The Refraction Editor allows the applied sound velocity to be adjusted, thereby adjusting the depth calculations across the beams.

### Processing of Depth Data

All data is assumed to be referenced to the vertical datum MSL, so again a zero tide file was applied.

Since no Total Propagated Uncertainty TPU values were provided with the survey data, the automated processing algorithm CUBE could not be used as a processing tool. TPU is a prerequisite for CUBE, as CUBE hypothesises a surface based on the depth points and their uncertainty. The multibeam swath tracks were therefore edited manually using the Subset Editor tool in CARISTM HIPS. The Subset Editor allowed the data to be selected and viewed in both 2D and 3D viewers. Anomalous data points could then be selected and rejected.

# Data Presentation

## Projection

The most appropriate projection for the presentation of the bathymetric datasets was WGS84 UTM zones. This is the most effective way of minimising distortion and maintaining positional accuracy for this region of the world. This decision was strongly influenced by how the CARISTM HIPS software deals with projections.

When data is imported into CARISTM HIPS, it is converted into HDCS Data, which stores position data as geographic coordinates (i.e. Latitude and Longitude). Users are then prompted to select a ‘map definition’, that is a projection for displaying the data on the screen. This does not affect how the data is stored.

In order to process the data a ‘Fieldsheet’ must be created. Fieldsheets also require a map definition, which describes the reference system the grid is stored in and exported as. This map definition cannot be changed once it is defined and so this is the point at which the most appropriate projection should be selected. Choosing an inappropriate projection at this stage would distort the positions and lead to the export of erroneous positioning data.

Although a regional projection such as the southern polar stereographic projection makes apparent sense visually, the UTM zones have the least distortion in the Antarctic coastal areas (i.e. the sum of squares of the residuals with respect to the actual global shape is minimised). A stereographic South Pole projection is only appropriate for latitudes south of -80° (SCAR, 2011a). Figure 4 shows where the UTM zone projections and stereographic projections (UPS South) are most appropriate for the Antarctic region. For this reason, the bathymetric data provided is grouped into UTM zones from the ‘fieldsheet’ stage through to the creation of images and soundings etc. All data is displayed in the Master GIS *.mxd file for ease of viewing.

Figure 3: Projections: UTM zones and UPS South zone in the Antarctic Region (SCAR, 2011b).

## Surface Creation

Singlebeam and multibeam data were gridded separately in UTM zone specific fieldsheets in CARISTM HIPS. They were then opened and combined using CARISTM BASE Editor (CARIS, 2011b), prioritising the multibeam data where data overlapped. The gridded surfaces were then cropped to fit within the proposed MPA boundaries.

The combined xyz data was then exported as a single ASCII file to be opened in the GIS software ESRI® ArcMapTM 9.3 (ESRI®, 2008) and converted to a Raster using the 'Point to Raster' tool in the Conversion toolbox.

All data was gridded to 100m resolution in both applications.

In areas of high sounding density, interpolated surfaces could also be made, for example in the Prydz, Drygalski and Mac.Robertson MPAs (Fig. 5). Interpolated surfaces are surfaces that are created by taking the gridded data and interpolating across areas where there is no data. The interpolation depths are calculated based on modelled existing data.

To interpolate a surface, the bathymetric point data was exported to a shapefile. Regions were then selected and interpolated using the ‘Topo to Raster’ tool in the Spatial Analyst toolbox. This tool uses a process developed by Michael Hutchinson at ANU termed ANUDEM (Hutchinson, 1989). The ArcMapTM tool only interpolates rectagonal regions and so where this was unsuitable, the ArcMapTM mosaic tool was used to patch rectagonal regions together. All interpolated gridding was to a resolution of 100m.

One pre-made interpolated grid was also included. The interpolated Mertz grid was provided by Rob Beaman of JCU (Beaman et al., 2011). (Note: this interpolated grid does not cover the full extent of the proposed Mertz MPA).

## Export of Soundings

Soundings layers were created in CARISTM Bathy Database at 10 km intervals. This spacing allowed for ease of viewing at a reasonable zoom level in GIS software applications. The sounding files created in CARISTM are a HOB file type. HOB files are CARISTM format that is used for soundings. The HOB file was then exported from CARISTM BASE Editor to an S57 file type (*.000). The S57 filetype is a vector interchange format used for maritime charts. It was developed by the International Hydrographic Organisation (IHO) and is an international standard.

# Data Deliverables

## Filetypes

This dataset was produced for use by the AAD and has now been publicly released by Geoscience Australia. The bathymetric data can be downloaded from the following webpage:

https://www.ga.gov.au/products/servlet/controller?event=FILE_SELECTION&catno=75593

The following filetypes are available:

### Interpolated Gridded Surface

In areas of high data density (Prydz, Drygalski and Mac.Robertson MPAs), interpolated surfaces are available in Raster formats. These grids can be visualised using the CARISTM EasyView software or in ESRI ArcMapTM.

### ArcMapTM GIS Files

ESRI ArcMapTM shapefiles and raster files have been provided as they are helpful in the visualisation of bathymetry data. The shapefiles include all survey tracks, the proposed MPA boundaries, and the Antarctic coastline. Satellite bathymetry (from ETOPO2) is provided as a background raster file.

### GeoTIFs

GeoTIFs of the bathymetric data have been provided for each proposed MPA. These files are geographically referenced images and can be displayed in GIS software or can be viewed simply as image files.

### Soundings

The soundings can be viewed in ArcMapTM by loading the shapefiles and labelling with the attribute, DEPTH.

### CARISTM Easyview

This is free software provided by CARISTM that allows viewing of several spatial data formats including those listed below. CARISTM Easyview also has a 3D viewer, which is very useful for generating images. Supported data formats:

AutoCAD files (DWG and DXF)

BASE Surface files (CSAR, HNS, BMS, HCS, DEF, and BAG)

Sounding Set files (CSAR, BPS)

BSB images (KAP, CAP)

CARISTM files (DES)

CARISTM Spatial Agent File files (SAF)

CRL images (CRL)

DGN files (DGN)

ECW images (ECW)

GeoTIFF images (TIF)

HCRF images (CHR)

Hydrographic Object Binary files (HOB)

IGA images (IGA)

JPEG 2000 images (JP2)

S-57 files (000)

Shape files (SHP)

TFW images(TFW)

VPF library (LHT)

## Individual MPAs

### Overall Marine Protected Area (MPA) Locations

Figure 4, an adapted version of Figure 1 is shown here as a reference for the locations of the MPAs.

Figure 4: Location of the seven proposed Antarctica MPAs (boundaries shown by thick black outlines). The blue lines represent singlebeam data ship tracks. The pale green lines represent multibeam coverage and ship tracks. The coloured regions represent the interpolated bathymetric grids derived from the bathymetry data. The Australian Antarctic bases are shown in bold text.

### Mertz proposed Marine Protected Area

There is a significant amount of bathymetry data available within this MPA. An interpolated surface has previously been published for much of this region (Beaman, 2011). These data were submitted to the AAD and included in the datasets presented here.

Figure 5: Mertz MPA Bathymetry. The outline of the MPA is represented by the thick black lines. The coloured areas detail the bathymetry data, with the coloured block showing the interpolated dataset published by Beaman et al., 2011. The blue lines represent singlebeam ship tracks.

### Wilkes proposed Marine Protected Area

There is very little data within this MPA, with only several singlebeam surveys and one multibeam survey crossing the MPA boundary.

Figure 6: Wilkes MPA Bathymetry. The MPA boundaries are shown by the thick black lines. The coloured areas represent bathymetry data and the blue lines singlebeam data ship tracks.

### Drygalski proposed Marine Protected Area

The Drygalski MPA contains a significant amount of bathymetry data and an interpolated surface was calculated to cover much of the MPA area.

Figure 7: Drygalski MPA Bathymetry.The outline of the MPA boundary is shown in thick black lines. The coloured areas represent the bathymetry data both as data and interpolated datasets.

### Prydz proposed Marine Protected Area

As a result of this MPA including the Australian Base Davis, this area was data rich with both numerous singlebeam and multibeam surveys. An interpolated surface was created.

Figure 8: Prydz MPA Bathymetry. Davis Antarctic base is shown in bold text. The coloured areas represent interpolated bathymetry grids. The blue lines show singlebeam ship tracks.

### Mac.Robertson proposed Marine Protected Area

Mac.Robertson MPA includes the Australian Base, Mawson and so also has numerous shiptracks, with singlebeam and multibeam data. An interpolated surface was created for most of the MPA, where the majority of data lies.

Figure 9: Mac.Robertson MPA Bathymetry. Mawson Antarctic base is shown in bold text. The coloured areas represent interpolated and gridded bathymetry. The blue lines represent singlebeam ship tracks.

### Enderby proposed Marine Protected Area

The data from only one multibeam survey is available for this remote and infrequently visited MPA. The lack of data did not warrant the creation of an interpolated surface.

Figure 10: Enderby MPA Bathymetry. The black lines represent the boundary of the MPA. The blue lines represent singlebeam ship tracks and the coloured areas represent bathymetry data.

### Gunnerus proposed Marine Protected Area

One singlebeam line passes in the bounds of the Gunnerus MPA (Fig.11).

Figure 11: Gunnerus MPA Bathymetry. The thick black lines mark out the MPA boundaries. The blue lines represent singlebeam ship tracks.

Acknowledgements

I would like to thank the following for all of their help and advice in producing this bathymetry compilation. Mike Sexton of Geoscience Australia for sourcing data and offering knowledge from his extensive experience in bathymetry, Henk Brolsma for data provision from the AAD, Michaela Guest for her correspondence and support from the AAD and Chris Carson for facilitating the relationship between Geoscience Australia and the AAD and his extensive reviewing of this record. Furthermore, Scott Nichol and Justy Siwabessy for their input as peer reviewers.

References

Beaman, R.J., O'Brien, P.E., Post, A.L., and De Santis, L., 2011, A new high-resolution bathymetry model for the Terre Adélie and George V continental margin, East Antarctica: Antarctic Science 23(1), 95-103. Available at: http://www.deepreef.org/publications/peer-review/82-gvdem.html

CARISTM HIPS. Sofware version 7.1. Release Date: May, 2011.

CARISTM Bathy DataBASE. Software Version 3.1. Release Date: April, 2011.

ESRI®. ArcMapTM Software Version 9.3. Release Date: 2008.

Geoscience Australia. Bureau of Mineral Resources Geology and Geophysics Record 1980/68. 1980. Marine Magnetic Surveys Aboard the M.V Nella Dan 1979/1980- Operational Report.

National Geophysical Data Center (NGDC). 2010. National Oceanic and Atmospheric Administration, United States Department of Commerce. The Marine Geophysical Data Exchange Format- MGD77/MGD77T. http://ngdc.noaa.gov/mgg/dat/geodas/docs/mgd77.htm. rev.

Scientific Committee on Antarctic Research_a (SCAR). 2011. The Antarctic Spatial Data Infrastructure. Projection Parameters UPS/UTM WGS84. http://portal.uni-freiburg.de/AntSDI/standardsspecifications/refsystemandprojections/projections/paramsupsutm#utm. Accessed: June 2011.

Scientific Committee on Antarctic Research_b (SCAR). 2011. The Antarctic Spatial Data Infrastructure. UTM/UPS Zones Antarctica. http://portal.uni-freiburg.de/AntSDI/standardsspecifications/refsystemandprojections/projections/utm.gif/view. Accessed: June 2011.

Determination of Approximate Mean Sound Velocity.

The sound velocity profiles (SVPs) used in the collection of the multibeam data are tabulated below (Not all voyages had SVP records). The mean sound velocity value was calculated from each discreet SVP and then a grand mean was calculated from all mean values. This was deemed the most efficient way to derive a grand mean with the data available:

Table 8: Mean Sound Velocity Values by Survey

| GA Voyage Reference Number | Mean Sound Velocity | Standard Deviation |
| --- | --- | --- |
| GA-2493 | 1471 | 12.3 |
| GA-2327 | 1487 | 2.2 |
| GA-2493 | 1458 | 7.6 |
| GA-2432 | 1452 | 7.2 |
| GA-2365 | 1451 | 11.8 |
| GA-4301 | 1441 | 0.2 |
| Grand Mean | 1460 | 16.6 |

Table 9: Sound Velocity Profiles from survey GA-2493 (first profile).

| Depth (m) | Velocity (m/s) |
| --- | --- |
| 0 | 1460 |
| 19.2 | 1460 |
| 56.7 | 1461 |
| 69.4 | 1461 |
| 79.59 | 1458 |
| 89.1 | 1454 |
| 98.6 | 1453 |
| 102.4 | 1452 |
| 126.4 | 1453 |
| 130.9 | 1453 |
| 138.4 | 1454 |
| 147.9 | 1454 |
| 158.6 | 1455 |
| 169.9 | 1456 |
| 193.8 | 1459 |
| 200.1 | 1459 |
| 216.4 | 1460 |
| 227.7 | 1461 |
| 237.7 | 1461 |
| 245.8 | 1462 |
| 295.7 | 1463 |
| 323 | 1463 |
| 339.1 | 1464 |
| 488 | 1466 |
| 593 | 1468 |
| 617.8 | 1468 |
| 626.2 | 1473 |
| 635.9 | 1471 |
| 651.5 | 1471 |
| 702.4 | 1471 |
| 732.3 | 1471 |
| 745.9 | 1478 |
| 749.5 | 1479 |
| 759.6 | 1494 |
| 770.9 | 1484 |
| 778.6 | 1482 |
| 792.8 | 1479 |
| 804.7 | 1478 |
| 827.7 | 1477 |
| 846.6 | 1476 |
| 866 | 1476 |
| 873.6 | 1476 |
| 887.7 | 1476 |
| 889.5 | 1476 |
| 897.1 | 1488 |
| 902.3 | 1488 |
| 908.8 | 1485 |
| 910.5 | 1485 |
| 924 | 1482 |
| 936.2 | 1481 |
| 943.2 | 1487 |
| 950.8 | 1485 |
| 960.1 | 1484 |
| 967.7 | 1490 |
| 976.4 | 1486 |
| 986.8 | 1484 |
| 993.8 | 1484 |
| 997.9 | 1490 |

Table 10: Sound Velocity Profiles from survey GA-2327

| Depth (m) | Velocity (m/s) |
| --- | --- |
| 6.7 | 1488 |
| 36.09 | 1488.9 |
| 61.5 | 1489.7 |
| 81.19 | 1488.6 |
| 102.5 | 1484.3 |
| 147.1 | 1485.2 |
| 192.3 | 1487.5 |
| 248.9 | 1488 |
| 299.79 | 1488.3 |
| 364.9 | 1488.6 |
| 424.3 | 1490.6 |
| 475.2 | 1490.6 |
| 595.4 | 1488.5 |
| 701.2 | 1487.6 |
| 807 | 1486.6 |
| 919.5 | 1485.1 |
| 998.9 | 1482.7 |

Table 11: Sound Velocity Profiles from survey GA-2493 (second profile)

| Depth (m) | Velocity (m/s) |
| --- | --- |
| 0 | 1456.5 |
| 8.9 | 1456.6 |
| 43.4 | 1458.1 |
| 47.2 | 1457.8 |
| 58.6 | 1453.2 |
| 69.4 | 1451.4 |
| 82.1 | 1448.8 |
| 92.3 | 1448.4 |
| 100.5 | 1448.4 |
| 105.6 | 1447.9 |
| 111.9 | 1446.6 |
| 115.7 | 1446.7 |
| 122 | 1448.3 |
| 127.1 | 1447.7 |
| 145.4 | 1450.8 |
| 154.2 | 1452 |
| 157.4 | 1452.3 |
| 162.4 | 1452.6 |
| 175 | 1456.3 |
| 193.8 | 1459.2 |
| 204.5 | 1460 |
| 222 | 1460.6 |
| 237.7 | 1461.1 |
| 258.9 | 1461.2 |
| 269.5 | 1461.6 |
| 301.3 | 1462 |
| 320.5 | 1462.7 |
| 328 | 1462.7 |
| 358.9 | 1463.1 |
| 408.3 | 1463.8 |
| 468.4 | 1465 |
| 530 | 1466 |
| 578.5 | 1466.8 |
| 720.9 | 1469 |
| 823.6 | 1470.2 |
| 974.6 | 1472.2 |
| 997.9 | 1472.5 |

Table 12: Sound Velocity Profiles from survey GA-2432

| Depth (m) | Velocity (m/s) |
| --- | --- |
| 0 | 1450 |
| 32 | 1449 |
| 36 | 1451 |
| 41 | 1447 |
| 45 | 1443 |
| 54 | 1443 |
| 54 | 1444 |
| 63 | 1446 |
| 77 | 1449 |
| 90 | 1452 |
| 99 | 1455 |
| 122 | 1458 |
| 144 | 1458 |
| 162 | 1458 |
| 162 | 1460 |
| 221 | 1461 |
| 662 | 1469 |

Table 13: Sound Velocity Profiles from survey GA-2365

| Depth (m) | Velocity (m/s) |
| --- | --- |
| 6.7 | 1460 |
| 19.1 | 1439 |
| 35.3 | 1440.5 |
| 48.5 | 1439.6 |
| 73 | 1439.9 |
| 99.9 | 1439.9 |
| 123.9 | 1440.8 |
| 151.8 | 1440.8 |
| 201.5 | 1441.8 |
| 248.5 | 1442.4 |
| 294.1 | 1442.1 |
| 404.2 | 1443.5 |
| 496.2 | 1445.3 |
| 609.2 | 1447.6 |
| 705.1 | 1449 |
| 790.6 | 1450.8 |
| 916.2 | 1453.3 |
| 1011.4 | 1455.8 |
| 1150.8 | 1458.6 |
| 1244.4 | 1460.7 |
| 1344.8 | 1463.1 |
| 1453.6 | 1465.6 |
| 1590.8 | 1468.9 |
| 1733.9 | 1472.6 |
| 2001.4 | 1478.4 |

Table 14: Sound Velocity Profiles from survey GA-4301

| Depth (m) | Velocity (m/s) |
| --- | --- |
| 0.06 | 1440.9 |
| 0.08 | 1441 |
| 4.57 | 1441.2 |
| 19.5 | 1441.3 |
| 50 | 1441.3 |
| 350 | 1441.3 |