Australian Seismological Report 2012

Geoscience Australia
RECORD 2013/32

Compiled by: Glanville, D.H.

Authored by: Balfour, N., Barton, T., Borleis, E., Burbidge, D.R., Cuthbertson, R., Dent, V., Glanville, D.H., Hardy, D., Love, D., Rynn, J.M.W., Wallace, A., Weatherley, D.

Department of Resources, Energy and Tourism

Minister for Resources and Energy: The Hon Gary Gray AO MP
Secretary: Mr Blair Comley, PSM

Geoscience Australia

Chief Executive Officer: Dr Chris Pigram
This paper is published with the permission of the CEO, Geoscience Australia

© Commonwealth of Australia (Geoscience Australia) 2013

With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence. (http://www.creativecommons.org/licenses/by/3.0/au/deed.en)

Geoscience Australia has tried to make the information in this product as accurate as possible. However, it does not guarantee that the information is totally accurate or complete. Therefore, you should not solely rely on this information when making a commercial decision.

Geoscience Australia is committed to providing web accessible content wherever possible. If you are having difficulties with accessing this document please contact clientservices@ga.gov.au.

ISSN 2201-702X (PDF)

ISBN 978-1-922201-65-2 (PDF)

GeoCat 75912

Bibliographic reference: Glanville, D.H. (comp), Balfour, N., Barton, T., Borleis, E., Burbidge, D.R., Cuthbertson, R., Dent, V., Glanville, D.H., Hardy, D., Love, D., Rynn, J.M.W., Wallace, A., Weatherley, D., 2013. Australian Seismological Report 2012. Record 2013/32. Geoscience Australia: Canberra.

Version: 1308

Contents

1 Introduction	1

2 Australia	4

3 Geoscience Australia’s Geophysical Network Operations 2012	7

3.1 Introduction	7

3.2 Network Operations 2012	8

3.2.1 Network Changes in 2012	9

3.2.2 Proposed Network Changes in 2013-2014	10

3.2.3 GA waveform data availability	11

3.2.4 Seismographic Paper Records Scanning Project	12

3.2.5 GA rapid earthquake response deployment activity 2012	18

4 The 2012 Earthquake Hazard Map	20

5 Australian Centre for Geomechanics/ Public Seismic Network	23

6 Australian Seismometers in Schools Network (AuSIS)	26

7 Australian Capital Territory	28

8 New South Wales	30

8.1 New South Wales Seismicity Report	34

8.1.1 Seismicity	34

8.1.2 Network	35

8.1.3 Procedures	35

9 Northern Territory	40

10 Queensland	42

10.1 Queensland Seismicity Report	43

10.1.1 Introduction	43

10.1.2 Seismicity	44

10.1.3 Network	44

10.1.4 ES&S Procedures	44

10.1.5 Queensland Earthquake Database (QEDB)	49

11 South Australia	50

11.1 South Australia Seismicity Report	51

11.1.1 Seismicity	51

11.1.2 Network	52

11.1.3 Procedures	52

11.1.4 Presentations	53

12 Tasmania	57

12.1 Tasmania Seismicity Report	58

12.1.1 Seismicity	58

12.1.2 Network	58

12.1.3 Procedures	59

13 Victoria	62

13.1 Victoria Seismicity Report	65

13.2 Seismicity	65

13.2.1 Network	66

13.2.2 Procedures	66

14 Western Australia	70

15 Australian Region	72

16 World	74

References	76

Appendices	77

A.1 Australian Earthquakes	77

A.2 New South Wales Earthquakes	94

A.3 South Australian Earthquakes	97

A.4 Western Austral ian Earthquakes	101

A.5 Australian Region Earthquakes	110

A.6 World Earthquakes	134

A.7 Geoscience Australia Seismic Data Downloading Procedure using SEEDLINK	139

Purpose	139

Definitions	139

Procedure	139

A.8 IRIS Seismic Data Downloading Procedure using SEEDLINK	142

Purpose	142

Definitions	142

Procedure	142

# Introduction

This report details earthquakes detected and located by Geoscience Australia (GA), The Department of Manufacturing, Innovation, Trade, Resources and Energy (DMITRE) and Environmental Systems and Services (ES&S) in 2012.Detailed summaries of earthquakes that occurred or were felt in Australia are provided, along with earthquakes detected within the Australian region and the worldwide. The latter are recorded as part of earthquake monitoring conducted in the operation of the Joint Australian Tsunami Warning Centre (JATWC).Individual State and Territory earthquake summaries are included, providing more detailed information regarding earthquakes in that State/Territory.Tables are also provided detailing the largest earthquakes ever recorded in that State/Territory and largest earthquakes (e.g. Top 10) for that State/Territory in 2012, where appropriate.The date and time of the earthquake are provided in Universal Time (UTC).Seismic networks operating in Australia are also detailed by their various operators in this report.

Earthquakes are located by Geoscience Australia using the following procedures:

An automated system using Antelope® software generates seismic event solutions. These are evaluated by the seismologist on duty to determine validity, size and location.

Blasts are differentiated when possible and are usually not located, and (generally) not included in the earthquake database.

Earthquakes worldwide, determined to have a magnitude (Mb or MWp) of 6.0 or higher by the duty seismologist, are entered into the Geoscience Australia earthquakes database as a preliminary solution.

Earthquakes located within the Australian Region, encompassing Latitude 10oN to -70oS and Longitude 85oE to -150oW, determined to have a magnitude of 5.0 or higher by the duty seismologist, are entered into the Geoscience Australia earthquakes database as a preliminary solution.

Earthquakes within Australia determined to have a magnitude of 4.5 or higher by the duty seismologist, are entered into the Geoscience Australia earthquakes database as a preliminary solution.

Earthquakes within Australia, determined to have a magnitude of 3.5 or higher by the duty seismologist and located near a populated area, are entered into the Geoscience Australia earthquakes database as a preliminary solution.

Earthquakes felt within Australia are located by the duty seismologist and entered into the Geoscience Australia earthquakes database as a preliminary solution.

All preliminary solutions are reviewed by Seismic Analysts using the Antelope® software tool DBloc2. For each solution the arrivals and stations are reviewed and the location and magnitude of the earthquake are updated and the solution finalised.

Seismic Analysts then analyse Australian seismic station records for smaller local events that are then located, a magnitude is determined and the event is entered into the Geoscience Australia earthquakes database as a final solution. This requires a clear signal on 3 or more seismic stations.

For events outside Australia, locations are determined using the IASPEI-91 velocity model and Mb, Msx and MWp magnitudes are calculated whenever suitable.

For events inside Australia, both IASPEI-91, and local Australian velocity models are used to determine location, with the model yeilding the best fit to arrivals being selected. Local magnitudes (Ml) are calculated using the amplitude of maximum displacement on the Vertical component of a seismogram that is converted to a Wood-Anderson seismometer response, using the following 3 formulae outlined in papers (Figure 1.1):

For Western side of Tasman Line: Gaull and Gregson (1991)

For South Australian Seismic Zone: Greenhalgh and Singh (1986)

For Eastern side of Tasman Line: Michael-Leiba and Malafant (1992)

Figure 1.1 Magniatude Formulae Zones used for calculating Ml by GA.

More information can be accessed at the Geoscience Australia Recent Earthquakes Website: http://www.ga.gov.au/earthquakes/

More information about the Joint Australian Tsunami Warning System can be accessed at:

http://www.bom.gov.au/tsunami/

If you have felt an earthquake you can lodge a felt report on our website. We use this information to construct isoseismal maps detailing intensity and felt radius for Australian earthquakes near population centres, and to gain a better understanding of the ground shaking characteristics for Australian earthquakes.

http://www.ga.gov.au/earthquakes/staticPageController.do?page=felt-earthquake

Or follow the link at:

http://www.ga.gov.au/earthquakes/

More earthquake and seismic information for South Australia can be found at:

http://www.pir.sa.gov.au/minerals/earthquakes

More earthquake and seismic information for New South Wales, Queensland, Tasmania and Victoria is available on the Environmental Systems and Services website at:

http://www.esands.com/

Other organisations and private seismic stations include:

Australian seismometers in schools network.

Facebook: www.facebook.com/ausisnetwork, Website: www.ausis.edu.au or email ausis@anu.edu.au

Australian Earthquake Engineering Society

http://www.aees.org.au/

Edward Pigot Seismic Observatory (EPSO), Coonabarabran, NSW, Australia

http://www.map.id.au/seismic/

Regional seismic users group website

http://www.rsuw.daleh.id.au/

# Australia

Figure 2.1 Map of 2012 Australian Earthquakes recorded by GA (circle size scales with magnitude).

Table 2.1 Australian Top 11 Earthquakes 2012 (Recorded by Geoscience Australia)

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (km) | Approximate Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5.7 | ML | 23/03/2012 | 09:25:19 | -26.124 | 132.124 | 0 | Near Ernabella, SA (MW 5.4). |
| 5.4 | ML | 19/06/2012 | 10:53:29 | -38.304 | 146.200 | 10 | SW of Moe, Vic. |
| 4.5 | MB | 05/11/2012 | 19:31:49 | -11.546 | 129.045 | 39 | Offshore Bathurst Island, NT. |
| 4.4 | ML | 20/07/2012 | 09:11:30 | -38.307 | 146.230 | 0 | Near Moe, Vic. |
| 4.3 | ML | 16/03/2012 | 11:24:36 | -26.174 | 131.824 | 15 | Musgrave Ranges, SA. |
| 4.2 | ML | 08/06/2012 | 11:31:00 | -30.759 | 150.413 | 0 | NW of Tamworth, NSW. |
| 4.1 | ML | 08/06/2012 | 11:32:39 | -30.748 | 150.400 | 0 | NW of Tamworth, NSW. |
| 4.0 | ML | 28/11/2012 | 22:35:22 | -27.757 | 125.943 | 0 | SW of Warburton, WA. |
| 3.9 | ML | 29/11/2012 | 19:21:18 | -27.756 | 140.659 | 5 | Near Innamincka, SA. |
| 3.9 | ML | 25/03/2012 | 20:53:20 | -30.324 | 136.821 | 0 | Near Andamooka, SA. |
| 3.9 | ML | 21/03/2012 | 17:07:09 | -28.231 | 136.001 | 0 | SE of Oodnadatta, SA. |

Figure 2.2 Number of Australian Earthquakes by Magnitude recorded by GA during 2012. (Note that not all small earthquakes [less than magnitude 3] can be located)

Table 2.2 Australian Top 10 Earthquakes

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Approximate Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7.3 | ML | 29/04/1941 | 01:35:39 | -26.920 | 115.800 | 0 | Meeberrie, WA. |
| 6.9 | ML | 26/01/1892 | 16:48:00 | -40.400 | 149.500 | 0 | Tasman Sea, NE TAS. |
| 6.9 | ML | 14/10/1968 | 02:58:50 | -31.620 | 116.980 | 10 | Meckering, WA. |
| 6.8 | ML | 12/05/1885 | 23:37:00 | -39.900 | 148.900 | 0 | Tasman Sea, NE TAS. |
| 6.7 | ML | 24/03/1970 | 10:35:17 | -22.050 | 126.610 | 8 | L McKay, WA. |
| 6.7 | ML | 22/01/1988 | 12:05:00 | -19.838 | 133.994 | 5 | Tennant Creek, NT. |
| 6.6 | ML | 16/08/1929 | 21:28:23 | -16.990 | 120.660 | 33 | Broome, WA. |
| 6.6 | ML | 23/04/1979 | 05:45:10 | -16.660 | 120.270 | 26 | Broome, WA. |
| 6.5 | ML | 10/05/1897 | 05:26:00 | -37.333 | 139.750 | 0 | Beachport, SA. |
| 6.5 | ML | 27/06/1941 | 07:55:49 | -25.950 | 137.340 | 0 | Simpson Desert, NT. |

Table 2.3 Time since last Magnitude 5, 6, and 7 in Australia as of end 2012.

| Magnitude | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Approximate Location | Time since |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5.4 | 19/06/2012 | 10:53:29 | -38.304 | 146.200 | 10 | SW of Moe, Vic. | 195 Days |
| 6.3 | 10/08/1997 | 09:20:31 | -16.050 | 124.423 | 10 | Collier Bay WA. | 15 yrs |
| 7.3 | 29/04/1941 | 01:35:39 | -26.920 | 115.800 | 0 | Meeberrie WA. | 71 yrs |

Appendix A.1 includes a table detailing all 2012 Australian Earthquakes detected by Geoscience Australia.

# Geoscience Australia’s Geophysical Network Operations 2012

Tim Barton

GA Observatories and Engineering Services Group Section Leader

## Introduction

The Geophysical Network (GN) Section in the Minerals & Natural Hazards Division of GA is responsible for the operation of the Australian National Seismograph Network (ANSN), affiliated international seismic stations and support to International Monitoring stations under Australia's obligations for the Comprehensive Nuclear Test Ban Treaty which includes seismic, infrasound and hydroacoustic facilities. The section also provides technical support for other observatory activities undertaken by GA in geomagnetism and geodesy. This support extends across continental Australia, throughout the Pacific, Indian and Southern Oceans, PNG and the AAT.

In addition, the section is responsible for the ingestion of real time data streams for over 200 seismic stations worldwide to support the operations of the Australian Tsunami Warning System (ATWS) at GA.

For all data streams GN is required to ensure that data is fit for purpose to meet client needs. To do this the section invests significant effort into State of Health (SoH) monitoring, Quality Assurance (QA) and Quality Control (QC) of seismic and ancillary data using commercial and in-house developed tools as well as ongoing review and checking of metadata.

The GN section is also responsible for the delivery of data to stakeholders both internal and external to GA. The focus for external stakeholders is on international warning centres and as such the bulk of the available resources are devoted to the serving of real time data streams. As GA does not currently operate a Data Management Centre (DMC) the delivery of legacy data continues to be a problematical issue in terms of meeting client expectations. In 2012 we continued to serve all of the ANSN real time data to the IRIS DMC who provide a suite of tools designed to meet most client demands.

The network design and operation is based around a series of defined requirements. These have evolved over the last 30 or so years and were first articulated in a review of BMR activities in the fields of earthquake seismology and geomagnetism undertaken during 1981 (BMR Record 1981/15). The main recommendations of this report were:

'That a national seismograph network should be developed to provide the capability for

Locating all Australian earthquakes with magnitudes greater than 4.0 on the Richter Scale;

Locating all earthquakes with magnitude greater than 3.0 on the Richter Scale in the continent's most seismically active and populous areas.

Detecting all Australian earthquakes with magnitudes greater than 3.0 on the Richter Scale'

The current operational metrics for the ANSN are:

Allow the detection, location (position, depth) and magnitude of all earthquakes of magnitude 3.5 or greater anywhere in Australia within 24 hours of an earthquake event;

Allow the detection of 99.9% of earthquakes of magnitude 6.0 or greater anywhere in the region of the Indian Ocean, Southwest Pacific, and the Indonesian archipelago within 10 minutes of the earthquake having occured.

## Network Operations 2012

Figure 3.1 Australian National Seismic Network (ANSN) Stations

The current network stations are listed in Table 1 as at 31 December 2012. All stations are part of the AU network except where noted. Those marked * are stations which were upgraded under the JUMP re-equipment program commenced in 2008-09. Station equipment and sensors are not presented here. However, these may be obtained using IRIS DMC BUD monitor tools, seehttp://www.iris.edu/data/bud/ In general most ANSN stations have broadband sensors except for those that are a part of the JUMP program as these are short period. Many stations also have a strong motion sensor.

### Network Changes in 2012

New stations

BN1H – Brisbane Hard Rock (Qld) site commissioned.

BW1H - Bowen Hard Rock (Qld) site commissioned.

BW2S - Bowen Soft Rock (Qld) site commissioned.

CN1H – Cairns Hard Rock (Qld) site commissioned.

CN2S – Cairns Soft Rock (Qld) site commissioned.

GD1S – Gladstone Soft Rock (Qld) site commissioned.

NTLH – Newcastle Hard Rock (NSW) site commissioned (December 2012).

PSAR - Pilbara Seismic Array (WA) commissioned (April 2012).

RK1H - Rockhampton Hard Rock site commissioned.

TV1H – Townsville Hard Rock site commissioned.

TV2S - Townsville Soft Rock site commissioned.

Station Equipment upgrades and significant changes

ARPS - Mt Arapiles (Vic). Equipment infrastructure housing upgrade completed to include solar panels installation and radio replacement

COEN – Coen (Qld). Completed earthworks to re-direct water flow, sealed vault floor and replaced pump to rectify leaky vault

DLN - Dalton (NSW). Taurus data logger and Next-G modem installed for post event data recovery.

FORT (Forrest, WA). – New solar power system, Q330/Marmot, STS-2 installed and VSAT relocated from airport to new site.

FITZ - Fitzroy Crossing (WA). Housing rebuilt and 1.2m VSAT dish replaced with 1.8m dish to improve communication reliability.

GIRL - Giralia (WA). BB STS-2 seismometer installed.

HTT - Hallett (SA). Temperature sensor installed.

INKA – Innaminka (SA). Civil works were completed for the new station.

MANU - Manus Island (PNG). Faulty STS-2 seismometer replaced, installed new weather station and GPS.

MEEK - Meekathara (WA). Civil works completed to rectify vault water issues with STS-2 and PA23 re-installed back into vault. Installed 3G modem as a VSAT backup. Relocated vault water pump and remounted dual and single float switch arrangements.

MILA - Mila (NSW). Q330HR/Marmot and BB sensor installed to replace SP sensor.

MULG - Mulgathing (SA). – Civil works were completed for the new station.

MGCD - Mangrove Creek Dam (NSW). Q330HR/Marmot installed to existing SP sensor.

MTSU – Mt Surprise (Qld). Equipment housing replaced, power system upgrades and VSAT relocation.

PTPS - Pt Pirie (SA). Faulty R130 digitiser replaced.

RKGY - Rocky Gully (WA). Mains power documented and site block diagrammed for future relocation of seismometer to address winter solar power issues.

RIV - Riverview (NSW). Power system upgrade, Q330HR/Marmot and BB sensor installed to replace SP sensor.

RMQ - Roma (Qld). Solar power system upgrade, VSAT relocated from airport to new site, Q330HR/Marmot installed to existing borehole SP sensor.

XMIS – Christmas Island (Indian Ocean) Damaged VSAT BUC and LNB replaced.

YNG - Young (NSW). Q330HR/Marmot installed to existing posthole SP sensor.

QFS - Quorrobolong Fire Station (NSW). Kelunji Classic dial-up site decommissioned.

CHI – Chichester Dam (NSW). Kelunji Classic dial-up site decommissioned.

Other station visits and minor retrofits

ARMA – Armidale (NSW).

BBOO – Buckleboo (SA).

BLDU – Ballidu (WA). VSAT modem upgrade

CARL – Carlisle (WA).JUMP

CN1H - Cairns (Qld). JUMP

CN2S – Cairns(Qld). JUMP

EIDS – Eidsvold (Qld).

GHSS - Government House, (SA). JUMP

MORW – Morawa (WA). VSAT modem upgrade

NAPP - Napperby (SA). JUMP

QLP – Quilpie (Qld).

RABL – Rabaul (PNG).

STKA – Stephens Creek (NSW). Radios and surge arrestors replaced and Next-G modem installed

TWOA - Flinders University (SA). JUMP

WRKA - Warakurna (WA). GPS weather station replaced

XMI – Christmas Island (Indian Ocean)

### Proposed Network Changes in 2013-2014

In 2013, subject to available resources and necessary approvals, the following network changes may occur. Should resources be available some of these activities may be rescheduled for 2013-2014 financial year.

Victoria JUMP station upgrades - Reoccupy the old ES&S Greenvale site, new site at Scienceworks, new site at Deakin University, new JUMP equipment at Ballarat (BRAT). Site approvals have been granted and these are now subject to scheduling by GA.

NSW JUMP station upgrade - Site selection for WOLS in Wollongong.

ACT JUMP station upgrade - Parliament House, PHB, new equipment.

NSW miscellaneous sites - Decommission KIM as equipment is beyond end of life.

SA regional - Installation of systems at OOD, LCRK, MULG & INKA sites where civil works have been completed.

Relocation of RKGY to address winter solar panel shading issues from mature trees. Bid for funding for a BB borehole sensor and construction of borehole.

Other station upgrades to be determined where SP sensors require upgrades to BB sensors.

### GA waveform data availability

All of GA's seismic data may be freely used under Creative Commons Attribution 3.0 Australia Licence, please refer to http://www.ga.gov.au/copyright.html for further information.

#### AutoDRM

AutoDRM is available for limited periods of time from the GA archive. Currently this is around 30 days from present time. AutoDRM has limited support at GA and is available through its inclusion in the Antelope Software Package. Its continued availability is contingent upon continued support from the vendor.

AutoDRM is also available from the IRIS DMC. GA has been providing a large number of stations to the IRIS DMC for a number of years and since late October 2011 all of the operational upgraded JUMP stations have also been supplied to the IRIS DMC.

#### SEEDLINK

Real time data is available from the GA Seedlink or IRIS DMC Seedlink server.

#### Notes on data sample rates and storage time limits

In general, all of ANSN sites stream either 20sps or 40sps data streams for seismic and strong motion channels. Details of individual stations can be found in the dataless SEED volumes or viewed in the IRIS DMC MetaData Aggregator.

GA undertakes considerable efforts to maintain up to date information to the IRIS DMC as part of GA's QA processes.

The digitiser technologies employed on all of GA's real-time stations includes locally stored high sample rate data (HSD), either at 100sps or 200sps, for seismic and strong motion channels. Depending on the digitiser, HSD is stored locally from between 7 days (2Gb GDAS) to ~30 days for Q330HR/Marmot and ~30 days for Reftek R130 (4Gb) units. These data are not available through any of our automated data delivery systems at this time. These data are downloaded using a semi-automated process either on request with justification or under internal GA guidelines for >M3.5 events.

The HSD downloaded data is stored on internal servers at GA and currently is not made available due to resourcing restrictions. However, on occasion requests from external users for copies of these data may be made available if we are able to assist.

### Seismographic Paper Records Scanning Project

#### Background

The project commenced in August 2009 in response to a storage issue identified at GA which provided the driver for a business case to preserve the records digitally and allow for the sentencing for disposal of completed scanned records under the Australia Archives Act.

Extensive testing and stakeholder engagement was undertaken to deliver a scanned product that captured all the information from the original record. In fact in some cases, particularly for annotated smoked paper records the scans were an improvement over naked eye viewing.

The collection is derived from a number of agencies, some of which are now defunct, and consists of records from over 200 seismic stations in Australia, AAT and PNG. Records date from the 1890's until the present. There are 51 pallets containing 3060 boxes with additional boxes being discovered from other areas in GA. It is currently estimated there are 750,000 records to be scanned.

Prior to disposal, the provenance of the records must be established. For most stations this is relatively straightforward and merely requires a demonstration that GA or its predecessors established, owned or were custodians of the station and the data it produced.

#### Current status

As of late November 2012 a total of 431,343 paper records have been scanned and associated metadata captured. This work, currently funded until June 30, 2013, is now being undertaken in a separate Division at GA with reduced staffing following the organisation’s recent restructure.

We have now completed scanning about half of our holdings.

The project continues to receive requests from international researchers for access to these records.

Two side projects are currently being undertaken on an opportunity basis in the Geophysical Network Section. The first is a compilation of individual station histories based upon BMR/AGSO/GA publications from the 1940's to present. This project has been made feasible using searchable pdf's generated from a digitisation project undertaken by the GA Library of Reports, Records and BMR/AGSO Journals over the past 2 years. The second project is to define a series of criteria to preserve a subset of the original records. Preliminary work indicates that a selection of type examples ought to be preserved such as smoked paper, photographic and ink record examples. In addition significant earthquake events (e.g. Australian events of M>5.5 and world events of M>7.5) may warrant preservation of original records for cultural heritage purposes. This is currently a work in progress. Several original records of significant events appear to be missing from the record boxes. It is assumed these have been removed by researchers and not returned. Several cases of misfiled records have also been noted.

#### Data availability

At this time there are no plans to make these scanned records available for external use due to resource limitations. The focus has been to ensure that resources continue to be made available to complete the scanning project. This is partly due to the order in which records are stored precluding scanning strategies based, for example, on "scan all station XXX since 19xx" or "scan all records from 19xx to 19xx".

The database being compiled during the project has enabled international researchers to visit GA and access some of the scanned records on an ad-hoc basis. At this time we would not like to encourage this to be too frequent an occurrence as it impacts on the scanning productivity.

Table 3.1 Geoscience Australia operated real time telemetry seismic stations

| Network | Station Code | Station Name |
| --- | --- | --- |
| AU | ARMA | Armidale, NSW |
| AU | ARPS | Mount Arapiles, Vic |
| AU | ASAR | AS01 to AS19 Alice Springs Array Elements |
| AU | BBOO | Buckleboo Station, SA |
| AU | BLDU | Ballidu, WA |
| AU | BN1H | Brisbane Hard Rock, Qld |
| AU | BN2S* | Brisbane Soft Rock, Qld |
| AU | BW1H* | Bowen Hard Rock, Qld |
| AU | BW2S* | Bowen Soft Rock, Qld |
| G | CAN | Mt Stromlo, Australian Capital Territory (Geoscope) |
| AU | CARL* | Carlisle, Perth, WA |
| IU | CASY | Casey, AAT (IRIS IU) |
| AU | CMSA | Cobar, NSW |
| AU | CN1H* | Cairns Hard Rock, Qld |
| AU | CN2S* | Cairns Soft Rock, Qld |
| AU | CNB | Kowen Forest, Australian Capital Territory |
| II | COCO | Cocos Island, Indian Ocean (IRIS/IDA II) |
| AU | COEN | Coen, Qld |
| AU | CORO* | Coronation Park, Launceston, Tasmania |
| AU | CTA | Charters Towers, Qld (CTBTO IM) |
| IU | CTAO | Charters Towers, Qld (IRIS IU) |
| AU | DPH* | Darwin Parliament House, NT |
| AU | DRS* | Darwin Rock Store, NT |
| AU | EIDS | Eidsvold Station, Qld |
| AU | FITZ | Fitzroy Crossing, WA (CTBTO IM) |
| AU | FORT | Forrest, WA |
| AU | GC1S* | Gold Coast Soft Rock, Qld |
| AU | GC2F* | Gold Coast Fill, Qld |
| AU | GEES* | Gees Lookout, Launceston, Tasmania |
| AU | GHSS* | Government House, Adelaide, South Aust. |
| AU | GIRL | Giralia, WA |
| AU | GLAD* | Gladstone, Tasmania |
| AU | GD1S* | Gladstone Soft Rock site, Qld |
| AU | HTT | Hallett, SA |
| AU | KDU | Kakadu, NT |
| AU | KLBR | Kellerberrin, WA |
| AU | KMBL | Kambalda, WA |
| AU | KNRA | Kununurra, WA |
| AU | LHI | Lord Howe Island |
| AU | MANU | Manus Island, Papua New Guinea |
| AU | MAW | Mawson, Antarctica (CTBTO IM) |
| IU | MBWA | Marble Bar, WA (IRIS IU) |
| AU | MCQ | Macquarie Island |
| AU | MEEK | Meekatharra, WA |
| AU | MGCD | Mangrove Creek Dam, NSW |
| AU | MILA | Mila, NSW |
| AU | MOO | Moorlands, Tasmania |
| AU | MORW | Morawa, WA |
| AU | MTKN* | Mount Kenneth, Perth, WA |
| AU | MTN | Manton Dan, NT |
| AU | MTSU | Mount Surprise, Qld |
| AU | MUN | Mundaring Weir, WA |
| AU | NAPP* | Napperby, SA |
| AU | NFK | Norfolk Island |
| AU | NIUE | Niue, Pacific Ocean |
| AU | NTLS* | Newcastle Soft Rock, NSW |
| AU | NTLH* | Newcastle Hard Rock, NSW |
| AU | PTPS* | Port Pirie, SA |
| AU | QIS | Mt Isa, Qld |
| AU | QLP | Quilpie, Qld |
| AU | RABL | Rabaul, Papua New Guinea |
| AU | RIV | Riverview, NSW |
| AU | RK1H * | Rockhampton Hard Rock, Qld |
| AU | RK2S* | Rockhampton Soft Rock, Qld |
| AU | RKGY | Rocky Gully, WA |
| AU | RMQ | Roma, Qld |
| AU | SYM | GA Symonston, ACT (Test site) |
| AU | STKA | Stephens Creek Reservoir, NSW (CTBTO IM) |
| AU | SYDH* | Kingswood College, Sydney, NSW |
| AU | SYDS* | David Phillips Sportsfield, NSW |
| AU | TOO | Toolangi, Vic |
| AU | TW1H* | Toowoomba Hard Rock, Qld |
| AU | TWOA* | Flinders University, Adelaide, South Aust. |
| AU | WHYH* | Whyalla, SA |
| AU | WOLH* | Wollongong Hard Rock, NSW |
| AU | WRKA | Warakurna, WA |
| AU | XMI | Christmas Island Airport |
| AU | XMIS | Christmas Island Grants Well |
| AU | YNG | Young, NSW |

* indicates a former JUMP/Triggered station

#### Geoscience Australia seismic stations under development

Table 3.2 Stations under development

| Station Code | Station Name |
| --- | --- |
| BN1H*(i) | Brisbane Hard Rock, Qld |
| CN1H*(i) | Cairns Hard Rock, Qld |
| CN2S*(i) | Cairns Soft Rock, Qld |
| RK1H*(i) | Rockhampton Hard Rock, Qld |
| TV1H*(i) | Townsville Hard Rock, Qld |
| TV2S*(i) | Townsville Soft Rock, Qld |
| INKA(ii) | Innamincka, SA |
| LCRK(ii) | Leigh Creek, SA |
| MULG(ii) | Mulgathing, SA |
| OOD(ii) | Oodnadatta, SA |
| PSAR(iii) | Pilbara Seismic Array, WA |
| PHB*(iv) | Parliament House Canberra, ACT |
| Greenvale*(v) | Greenvale Reservoir, Vic |
| BRAT*(vi) | Ballarat Reservoir, Vic |
| Melbourne*(vii) | Scienceworks, Melbourne, Vic |
| Deakin Uni*(viii) | Deakin University, Geelong, Vic |
| NTLH*(ix) | Newcastle Hard Rock, Newcastle, NSW |
| WOLS*(x) | Wollongong Soft Rock, Wollongong, NSW |

Notes:

(i) JUMP replacement stations were commissioned during the year by the Queensland Government and GA. Data is currently only available for these stations from GA. Inclusion of these data streaming to the IRIS DMC is yet to be completed.

(ii) Civil works completed. Equipment install in 2013 subject to operational funds.

(iii) Station in PRODUCTION, data streamed to IRIS DMC. Dual comms to BoM Melbourne still to be commissioned in early 2013.

(iv) Equipment procured, awaiting staff resources to determine a timing solution with no GPS access before install can proceed. Station currently has a dial up Classic installed.

(v) Hard Rock Site selected, Deed of licence being negotiated with Melbourne Water, equipment and civil components in storage. Install planned for early 2013.

(vi) Existing site operated by ES&S which is still technically a Victorian JUMP station, equipment in stock. Install planned for early 013.

(vii) Negotiations in progress with Scienceworks for install in a basement in the facility, equipment in stock. Install planned for early 2013.

(viii) Site located on campus, Deed of licence being negotiated with Deakin University, equipment and civil components in storage. Install planned for early 2013. This site will replace BANM.

(ix) Hard Rock Site to replace NLD, install completed, station in PRODUCTION December 2012.

(x) New site yet to be located, equipment and civil components in storage.

#### Geoscience Australia operated non-real time seismic stations

Table 3.3 Non-real time seismic stations

| Station Code | Station Name |
| --- | --- |
| DLN(xi) | Dalton, NSW |
| SY01 to 06(xii) | Various sites in Western Sydney |
| QFS(xiii) | Quorrobolong Fire Station, NSW |
| KIM(xiv) | Newcastle, NSW |
| CHI(xv) | Chichester Dam, NSW |

Notes:

(xi) Station previously referred to as DAL was renamed to an ISC code of DLN which had been used for many years by ANU in the vicinity. Station consists of L4C single component seismometer in a borehole with a CMG5T accelerometer linked to a 3 channel Nanometrics Taurus digitiser at 200 sps and a Next-G modem. For technical reasons this is not currently streamed real time and data is downloaded manually when required. The system will be upgraded to a CMG-3TB with R130 digitiser in early 2013.

(xii) These are temporary stations installed in 2006. They currently consist of L4C3D three component seismometer in pits linked to a 3 channel Nanometrics Taurus digitiser at 200 sps and a Next-G modem. For technical reasons these are not currently streamed real time and data is downloaded manually when required.

(xiii) QFS was a Kelunji Classic on dial-up. Decommissioned following the installation of NTLH.

(xiv) KIM is a Kelunji Classic on dial-up. Its current status is unknown. It will be decommissioned following the installation of NTLH. The equipment is not owned by GA who has been covering the operational costs for some years. Some investigation will be required prior to its removal.

(xv) CHI is a Kelunji Classic on dial-up. Decommissioned in 2012.

#### High Sample Data for 2012

##### Operating principles

GA has developed as set of guidelines for the semi-automatic download of high sample data from seismic stations within Australia and its Territories. Depending on the digitiser, the majority of ANSN stations with seismometer and accelerometer data are sampled at either 100 sps or 200 sps.

For segmented standard sampled data streams at 20 sps or 40 sps availability is best accessed through the IRIS DMC. Real time data may be streamed by SEEDLINK from GA or IRIS.

##### Earthquake Magnitude and Distance

In the event of an Earthquake within Australia and Territories

Magnitude 3.0 - 4.0: collect data from stations within 2000km

Magnitude 4.0 - 4.5: collect data from stations within 3000km

Magnitude > 4.5:collect data from stations within 4000km

Consideration is also given to requests by Earthquake Hazard Section staff at GA for events which are evaluated as significant for a particular reason. In such cases the stations recovered will be decided upon depending on the nature of the event.

##### Download parameters

Start-time: 30 second before the P arrival time

End-time: 20 minutes after P arrival time

Format: MiniSEED

GA currently operates a number of different data digitiser systems with different storage capacities.

Q330HR/Marmot – 200 sps data for 6 channels up to 30 days

GDAS – 100 sps or 200 sps data for 6 channels up 7 to 14 days subject to size of onboard memory

Reftek R130-1- 200 sps data for 6 channels up to 30 days subject to size of onboard memory

Taurus- 200 sps data for 3 channels up to 30 days subject to size of onboard memory

### GA rapid earthquake response deployment activity 2012

#### Guiding Principles

In the event of a large (>ML.4.5) or an otherwise significant earthquake occurring it may be desirable to deploy a temporary array of recording instruments in the epicentral region to determine accurate hypocentres of aftershocks. These can be used to more precisely locate the main shock, and to delineate the faults associated with the earthquake. It also presents an opportunity to acquire strong motion data close to epicentres. This data provides important constraints on determining the local seismic hazard. A similar deployment may be undertaken in areas of earthquake swarm activity. The deployment of an array will accurately locate the hypocentres of the swarm events and help to identify the association with any tectonic features and the spatial and temporal behaviour of the swarm.

#### Rapid Earthquake Response Deployments

##### Triggers for Rapid Deployments

A rapid aftershock deployment should be considered if an earthquake occurs that is:

Larger than magnitude (>ML 4.5) and:

Occurs within or close to a population centre or

Occurs close to major infrastructure (e.g.: large dam; energy facility; etc).

##### Field Deployment

At least 4 seismic recorders should be deployed within 24 to 36 hours around the earthquake epicentre.

One should be placed as close to the known epicentre as possible and allow both seismometer and accelerometer recording.

The other three should be approximately 5 km, 10 km and 15 km radial distance from the epicentre unit at equal 120 degree azimuths.

One of these recorders should be at a location where data can be downloaded at short notice to enable active monitoring of aftershocks.

If aftershocks are occurring, when possible the data should be downloaded and analysed frequently. If the data cannot be accessed remotely, the recorders will have to be visited regularly to download the data for analysis. The preferred option is to have an active communication link with the station.

Maintenance of the recorders should be undertaken at intervals no greater than the onboard storage capacity of memory. This is generally 30 days for 2 Gb memory for 6 ch @ 100sps on a Kelunji Echo recorder.

##### Duration of Deployments.

A review of deployments should be undertaken after 21 days to determine whether the activity falls to near-background levels and recovery made at 30 days. If instruments have additional storage capacity then a second review should be done to determine whether a service or recovery is appropriate. For prolonged deployments then the activity changes to a long term temporary deployment which will require planned resourcing and additional or different equipment. An example of such an activity is the current GA Sydney deployment which commenced in 2006 and is still active.

##### 2012 GA capacity

In 2012 GA had a total of 12 rapid deployment kits consisting of an EchoPro, a Lennartz LE-3D seismometer each in a Pelican case (total weight 20 kg for airline baggage limits) and solar panel.

These kits were upgraded substantially in 2012 and now include Next-G comms.

As at November 2012 there are 8 kits stored in GA in Canberra, 0 at the GSWA Core store at Carlisle in Perth and 4 at PIRSA Glenside in Adelaide. All deployments undertaken in 2012 had instruments installed within 36 hours of events by GA staff.

#### Rapid Earthquake Response deployments 2012

Event: 		Moe, Victoria. M5.4 on 19 June 2012

Deployment:		20-21 June 2012

Service visit:		22 June – 1 July 2012

Table 3.4 Moe rapid deployment sites

| Station | Latitude | Longitude |
| --- | --- | --- |
| MOE8 | ‘Morwell Motel’ |  |
| MOE3 | -38.181811 | 146.31003 |
| MOE4 | -38.239601 | 146.102646 |
| MOE1 | -38.285933 | 146.23434 |
| MOE2 | -38.37475 | 146.262855 |
| The replacement or back-up sites are: |  |  |
| MOE5 | -38.266832 | 146.508993 |
| MOE6 | -38.493009 | 146.144483 |
| MOE7 | -38.319577 | 145.965831 |

High-sample rate data for this event from surrounding permanent stations was recovered by GA.

# The 2012 Earthquake Hazard Map

Ed. D.R. Burbidge

Extract from Executive Summary

In July 2009 Geoscience Australia (GA) initiated a project to update the Australian earthquake hazard map using the latest available methods and data. A report summarising the work done since that time is now available online. Among other applications, the maps presented in the report have been designed so that they can be used to inform the hazard component of the next revision to Australia’s earthquake loading code AS1170.4 “Structural design actions: Part 4 Earthquake actions in Australia”. The key improvements in the 2012 Australian Earthquake Hazard Map are summarised below.

The earthquake catalogues used to generate the existing earthquake hazard map found in AS1170.4-2007 were based only on events up until the late 1980s, although a later revision in the 1990s also considered earthquakes that had occurred up until then. The earthquake catalogue used here includes events up until 2011; a total of 8,000 earthquakes above magnitude 3. The catalogue is a combined version of several other regional catalogues provided by external agencies and seismologists. This combined catalogue represents the most complete known catalogue of earthquakes ever compiled for Australia.

Historically, a variety of local magnitude ML formulae have been used to measure the size of earthquakes across Australia. We developed a more consistent catalogue through conversion of the various magnitude measurements of the earthquakes into a more consistent ‘pseudo ML’ scale. Furthermore, we introduced a systematic logic to select preferred magnitude types (e.g. moment magnitude MW, surface-wave magnitude MS, etc.) when the use of ML is not appropriate.

Probable aftershocks, foreshocks and mine blasts have been identified in the catalogue using a combination of methods based on the latest thinking of how earthquakes cluster in intraplate regions. The method uses both manual event classification by an expert and an automatic classification system. While the method has probably not removed every aftershock, mine blast or earthquake swarm event, to the best of our knowledge, the declustered catalogue used here is cleaner than any previous Australian catalogue.

Earthquake source zones applied in the hazard map are developed using a unique combination of three different layers, which capture seismic characteristics at sub-national, regional and high-activity point (hotspot) scales. The zones defined balance the stability and conservatism requirements of a hazard map for the building code while still allowing it to be flexible enough to accommodate a wide range of uses. In each zone we estimate earthquake probability by fitting the catalogue data to truncated Gutenberg-Richter (Gutenberg and Richter, 1944) relations using a new automatic algorithm. The method provides robust values for the Gutenberg-Richter values, a and b, and requires little to no manual adjustment.

The maximum magnitude of earthquakes within the majority of the zones is based on the neotectonic domains work of Clark et al. (2011) and the work of Leonard and Clark (2011). This makes the map one of the first in the world to apply a semi-quantitative measure of Mmax for majority of the source zones in the map rather than simply adding a constant to the observed maximum magnitude from a historic catalogue.

Ground-motion prediction equations for previous maps were based on limited intensity data and often involved applying relations developed for other countries. Is this report we applied recently developed equations (two of them Australia-specific), based on modern methods and data. Weightings applied to the various ground-motion equations are determined by comparing their predictions to observed Australian ground-motion data. These equations were used to calculate the ground motion at a range of response spectral accelerations, rather than just calculating the hazard for peak ground acceleration (PGA), as has been done in many previous assessments.

A suite of maps is calculated using GA’s Earthquake Risk Model (EQRM). The EQRM is open-source, allowing the results to be tested or modified independently. In addition to determining earthquake hazard (i.e. likelihood of ground-shaking), the use of EQRM also provides the capability to calculate earthquake risk (e.g. likelihood of damage) in the future, provided the necessary site classification, exposure and vulnerability models are available.

The final 2012 Australian earthquake hazard maps for a range of return periods and response spectral periods are presented in the report. Figure 4.1 shows the 0.0 s response spectral period (or PGA) earthquake hazard in 3D for a return period of 2500 years. The hazard is represented as the geometric mean of horizontal component ground-motion. This particular combination of response spectral period and return period is most comparable to those for the map used in the current building code. However, Figure 4.1 represents just one of many different combinations of return and response spectral period hazard calculated in this assessment.

As a result of the updates and improvements outlined above, the 2012 maps represent a significant improvement over previous national earthquake hazard maps. Accordingly, GA has recommended that the 2012 maps, such as the one shown in Figure 4.1, be considered by Standards Australia as a basis for developing hazard factors for the next revision of the AS1170.4 earthquake loading code.

For more information regarding the 2012 Earthquake Hazard Map see:

The interactive Earthquake Hazard Map:

http://www.ga.gov.au/darwin-view/hazards.xhtml

Or the full report on the 2012 Earthquake Hazard Map:

http://www.ga.gov.au/products/servlet/controller?event=GEOCAT_DETAILS&catno=74811

Figure 4.1 Earthquake hazard on rock for the 2500 year return period at a response spectral period of 0.0 s (PGA) shown in 3D evaluated for the geometric mean of the horizontal components.

# Australian Centre for Geomechanics/ Public Seismic Network

Vic Dent & Dale Hardy

A brief history of this network, which began in 2006, is given in the 2011 Australian Seismological Report. In its current form, the network is divided into three sub-networks – Eastern Australia, Central Australia and Western Australia. The stations in the networks are owned or maintained by volunteers, and assisted by the Australian Centre for Geomechanics (ACG), based in Perth, WA, which provides web-based analytical software, as well as networking and communications software, and data storage.

In Eastern Australia, the network as it existed in 2011 (9 stations) continued in 2012 with little change. The only significant change was the installation of a new station at Lithgow High School to replace that at Katoomba, which had significant noise problems arising from the school air-conditioning system.

The stations in the Central Australian network, at Peterborough, Jamestown and Modbury Heights continued operation in 2012, and two new stations, in the Adelaide Hills region, were added at Lobethal and Mylor. Data from the Central Australian network are regularly used by personnel in DMITRE, Adelaide, in conjunction with data from their own network, to assist in locating South Australian earthquakes.

In Western Australia the network contracted, basically due to difficulties in maintaining internet connections through school internet security systems. Stations at schools in Broome, York, Lake Grace, Beacon and Pinjarra all closed during 2012. The network is now looking towards taking advantage of local Community Resource Centres and the station at the District High School in York, closed in November 2011, was re-opened at the York CRC in October 2012.

Several temporary sites were occupied for short periods in late 2012 in the Beacon/Koorda region, about 200 km north-east of Perth, to monitor swarm activity occurring there. Some useful recordings were made and a report is being prepared for presentation to the AEES.

More information on the network, and current seismograms from some stations, can be found at www.rsuw.daleh.id.au.

Table 5.1 Stations operating in 2012.

| Location | State | Latitude | Longitude | Opened | Sensor | Code! | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Gin Gin | QLD | -25.1045 | 151.867 | 06/08/2011* | geophone | FS03 |  |
| Dungog | NSW | -32.3971 | 151.7495 | 7/10/2011 | geophone |  |  |
| Redhead | NSW | -33.0197 | 151.697 | 1/12/2008 | Willmore | REDH | replaced MKPT |
| Orange | NSW | -33.3019 | 149.0937 | 2/05/2011 | geophone |  |  |
| Katoomba | NSW | -33.7284 | 150.314 | 6/05/2010 | geophone |  | Closed Aug 2011 |
| Lithgow | NSW | -33.4857 | 150.166 | 23/07/2012 | geophone |  | new in July 2012 |
| Tallong | NSW | -34.42 | 150.05 | 17/02/2011 | Willmore |  |  |
| Crookwell | NSW | -34.4523 | 149.4658 | 21/02/2011 | geophone |  | to willmore in 2012 |
| Gundaroo | NSW | -35.032 | 149.268 | 26/01/2008 | Willmore | GUND |  |
| Kambah | ACT | -35.3892 | 149.0768 | 01/02/2010* | geophone | KBH |  |
| Sandon | Vic | -37.1674 | 144.0516 | 27/09/2011* | Willmore | S88B |  |
| Peterborough | SA | -32.9787 | 138.8357 | 29/06/2011 | Willmore | PBR1 |  |
| Jamestown | SA | -33.2045 | 138.6092 | 29/06/2011 | geophone | JMS1 |  |
| Modbury | SA | -34.7417 | 138.7733 | 24/06/2011* | Willmore | THS |  |
| Morphett Vale | SA | -35.1303 | 138.8599 | 27/05/2010 | Willmore | MPTV |  |
| Lobethal | SA | -34.9052 | 138.8599 | 09/01/2012 | Willmore |  | new in Jan 2012 |
| Mylor | SA | -35.0488 | 138.7537 | 15/04/2012 | Willmore |  | new in Apr 2012 |
| Broome | WA | -17.9559 | 122.2325 | 27/06/2006 | Willmore |  | closed in April 2012 |
| Northampton | WA | -28.3538 | 114.647 | 25/07/2011 | Willmore |  |  |
| Beacon | WA | -30.4511 | 117.8687 | 19/03/2009 | Willmore | BEAC | closed 23 Dec 2012 |
| Beacon West | WA | -30.418 | 117.7463 | 08/09/2012 | geophone |  | temp-closed 24 Sep |
| Koorda KOO5 | WA | -30.6637 | 117.5411 | 20/09/2012 | geophone |  | temp-closed 12 Oct |
| Koorda KOO4 | WA | -30.6307 | 1176041 | 11/11/2011 | geophone |  | closed 18 Jan 2012 |
| York | WA | -31.8933 | 116.7603 | 15/05/2007 | geophone | lat was wrong | closed 28 Nov 2011 |
| York CRC | WA | -31.8917 | 116.7674 | 23/10/2012 | geophone |  | re-opened 23 Oct 12 |
| UWAmuseum | WA | -31.985 | 115.82 | 27/04/2006 | geophone |  |  |
| Pinjarra | WA | -32.6331 | 115.8723 | 18/02/2010 | geophone |  | closed in 2012 |
| LakeGrace | WA | -33.1036 | 118.459 | 28/01/2011 | geophone |  | closed 29 Oct 2012 |
| Gnowangerup | WA | -33.8728 | 117.8999 | 27/01/2008 | geophone | GNOW |  |

* denotes a prior-operating seismograph, added to PSN network on this data

!ISC registered codes

Figure 5.1 Map of Public Seismic Network Stations.

# Australian Seismometers in Schools Network (AuSIS)

Michelle Salmon and Natalie Balfour

Over the next year AuSIS will install 40 broadband seismometers in schools around Australia to promote careers in geoscience and to give students the opportunity to be involved in a national science experiment in conjunction with collecting valuable data for the seismological community. Over 125 schools around Australia applied to participate in the program making deciding between schools a difficult task. The map below shows the locations of schools that have been selected to host a broadband instrument. Five instruments have already been installed, 2 in schools in Canberra, one in the Mt Stromlo seismic vault, one at the Research School of Earth Sciences at ANU and one in Jindabyne. The data from these stations is now publically available from IRIS in near real-time. The New South Wales Department of Trade and Investment have provided the funding for two additional broadband instruments, 50 quake-catcher sensors (http://qcn.stanford.edu/), and 10 slinky seismometers (http://cgiss.boisestate.edu/~kasper/geoph297wiki/index.php/BSU_Network) to go into schools in NSW.

Table 6.1 Station Information

| Network Code | Station Code | Latitude | Longitude | Elevation (m) | Install date | Instrument | Sample rates |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S | AUMTS | -35.3188 | 148.9964 | 667 | 09/11 | CMG-6TD | 10, 100 |
| S | AUMHS | -35.3635 | 149.0888 | 615 | 12/11 | CMG-6TD | 10, 100 |
| S | AUDAR | -35.2530 | 149.1377 | 575 | 04/12 | CMG-6TD | 100 |
| S | AUSIS | -35.2839 | 149.1147 | 585 | 05/11 | CMG-6TD | 100 |
| S | AUSMG | -36.4161 | 148.6084 | 951 | 03/13 | CMG-6TD | 10, 100 |

AuSIS is a four-year project (2011-2014) funded by the Geophysical Education Observatory component of AuScope Australian Geophysical Observing System (AGOS). Stations will run for the duration of the project and we hope well beyond as they have been set up to require minimal maintenance. For recent updates on the project see our Facebook page: www.facebook.com/ausisnetwork, our website: www.ausis.edu.au or email: ausis@anu.edu.au

Figure 6.1 Map of selected schools to have installations in 2013.

# Australian Capital Territory

Figure 7.1 Map of 2012 Australian Capital Territory Earthquakes recorded by GA (circle size scales with magnitude).

Table 7.1 Australian Capital Territory Top 9 Earthquakes

| Magnitude | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- |
| 3.0 | 25/04/1970 | 10:54:50 | -35.430 | 149.100 | 18 | ACT |
| 3.0 | 19/08/1998 | 11:12:44 | -35.654 | 148.951 | 0 | ACT |
| 2.9 | 23/10/2000 | 01:20:25 | -35.472 | 148.837 | 1 | ACT |
| 2.8 | 10/01/1976 | 01:27:46 | -35.610 | 149.030 | 13 | ACT |
| 2.5 | 21/06/1999 | 13:59:35 | -35.193 | 149.164 | 7 | ACT |
| 2.5 | 19/02/1972 | 05:18:49 | -35.360 | 149.010 | 4 | ACT |
| 2.5 | 07/12/1975 | 10:31:26 | -35.340 | 149.090 | 0 | ACT |
| 2.5 | 12/02/1977 | 11:56:46 | -35.540 | 148.850 | 13 | ACT |
| 2.5 | 19/09/1976 | 02:04:52 | -35.900 | 149.000 | 20 | ACT |

# New South Wales

Figure 8.1 Map of 2012 New South Wales Earthquakes recorded by GA (circle size scales with magnitude).

Appendix A.2 includes a table detailing all 2012 New South Wales Earthquakes detected by Geoscience Australia.

Table 8.1 New South Wales Top 7 Earthquakes

| Magnitude | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- |
| 5.8 | 21/05/1961 | 21:40:03 | -34.547 | 150.503 | 19 | Bowral, NSW. |
| 5.7 | 27/12/1989 | 23:26:57 | -32.946 | 151.607 | 11 | Newcastle, NSW. |
| 5.6 | 18/11/1934 | 21:58:41 | -34.800 | 149.200 | 10 | Gunning, NSW. |
| 5.5 | 29/11/1886 | 17:00:00 | -34.750 | 148.800 | 0 | Bowning, NSW. |
| 5.5 | 30/05/1921 | 14:51:59 | -35.000 | 145.000 | 0 | Hay, NSW. |
| 5.5 | 10/03/1949 | 22:30:33 | -34.740 | 149.200 | 0 | Dalton, NSW. |
| 5.5 | 9/03/1973 | 19:09:14 | -34.170 | 150.320 | 21 | Picton, NSW. |

Figure 8.2 Isoseismal map of the June 8 Ml 4.4 Tamworth earthquake (GA)

Figure 8.3 Isoseismal map of the June 22 Ml 3.7 Boorowa earthquake (GA)

Figure 8.4 Isoseismal map of the April 19 Ml 3.7 Clive earthquake (GA)

Figure 8.5 Isoseismal map of the June 16 Ml 2.7 earthquake north of Canberra (GA)

## New South Wales Seismicity Report

Environmental Systems and Services

Compiled by: Elodie Borleis and Russell Cuthbertson

### Seismicity

This report deals primarily with the area covered by the ES&S New South Wales seismic network.Twenty-four events with magnitude greater or equal to ML 2.5 were located in New South Wales by ES&S in 2012, eight greater than or equal to magnitude ML 3.0 and two greater or equal to magnitude 4.0, excluding all aftershocks.

On the evening of 8 June, 2012 a magnitude 4.4 earthquake occurred near Lake Keepit. This event and a magnitude 4.3 earthquake that occurred approximately 100 seconds later were felt widely – up to 100km away.Data from the intensity reports sent to ES&S for the earthquake was used to create an isoseismal map.

Initial estimates of the location of these shocks were to the north of the dam. Two temporary stations (KP1T & KP2T) were installed to the northeast of the dam on 19th June. When combined with the permanent stations at Keepit Dam (2 sites) and Split Rock Dam, these two sites produced a network capable of obtaining more accurate locations of any aftershocks.

The temporary sites operated until early November but did suffer from loss of power during periods of extreme cold because of the lack of permanent instrument housing.

While the two temporary sites recorded a large number of blasts from several mines near Boggabri, there was only one aftershock recorded (22 June).

The possibility that the series of earthquakes was induced by the reservoir seems unlikely. Reservoir induced seismicity is a possibility for reservoirs exceeding 1,000,000 ML in volume or 100m in depth. Lake Keepit, with a FSL volume of 425,510 ML and a depth of 50m would be unlikely to be big enough to induce activity.

The mainshock and well located aftershock are almost 10km from the deeper parts of the reservoir.

Most reservoir induced activity is followed by a significant aftershock sequence. The lack of such in this instance provides further evidence for a lack of correlation between the reservoir and the recorded activity.

Induced seismicity often occurs immediately upon filling of a reservoir due simply to the loading effect. However significant delays (many years) between filling and induction occur in instances where pore pressures effects are considerable. In certain crustal stress regimes a rapid reduction in storage volume can trigger activity.

Storage level data obtained for Lake Keepit showed that there was a major increase in storage volume in late 2010 and a rapid drop in late 2012 - but this was after the earthquake sequence occurred.

The 20 month delay between the increase in water level and the earthquake sequence does not preclude an induction mechanism.

### Network

The New South Wales seismic monitoring network operated by the ES&S Seismology Research Centre now comprises 37 seismographs and accelerographs throughout New South Wales.

### Procedures

Standard procedures for earthquake location are:

Unusual, abnormal or unknown blasts are sometimes located and included in the catalogue (with a code to indicate the source type)

Overseas events are not located

Events outside the network operated by ES&S (in eastern Australia) are occasionally located.

Events that trigger the ES&S alarm service (operating in eastern Australia) are typically located within 30 minutes.

More detailed analyses of these events, as well as smaller events that did not trigger the alarm, are generally conducted within a week.

Data from GA sites and other independent stations are obtained whenever possible.

Locations are calculated using eqFocus software.

Figure 8.6 ES&S New South Wales Seismic Network

Table 8.2 New South Wales magnitude ML 2.5+ earthquakes recorded by Environmental Systems & Services

| Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Mag | Mmi | Acc | Location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2012-03-11 | 1821 | -36.302 | 148.688 | 5 | 2.9 | 0 | C | Berridale |
| 2012-03-20 | 1528 | -33.797 | 151.727 | 10 | 2.9 | 0 | D | Off Sydney |
| 2012-04-19 | 1909 | -35.328 | 148.642 | 8 | 3.9 | 3 | C | E of Tumut |
| 2012-04-29 | 0657 | -32.681 | 151.020 | 0 | 2.8 | 3 | C | Broke |
| 2012-06-05 | 0912 | -34.304 | 148.757 | 10 | 3.1 | 0 | B | Boorowa |
| 2012-06-08 | 1131 | -30.780 | 150.460 | 15 | 4.4 | 5 | B | Tamworth |
| 2012-06-08 | 1132 | -30.774 | 150.504 | 4 | 4.3 | 3 | C | Tamworth |
| 2012-06-14 | 0700 | -30.770 | 150.497 | 5 | 2.6 | 0 | C | Tamworth |
| 2012-06-22 | 0916 | -34.063 | 148.610 | 18 | 4.3 | 4 | D | N of Boorowa |
| 2012-07-20 | 1008 | -32.068 | 149.699 | 10 | 2.7 | 0 | E | Dunedoo |
| 2012-07-30 | 1725 | -37.666 | 150.663 | 6 | 3.9 | 0 | C | Off Gabo Is |
| 2012-07-30 | 1728 | -37.876 | 150.366 | 5 | 3.8 | 0 | C | Off Gabo Is |
| 2012-07-30 | 1956 | -37.664 | 150.544 | 5 | 2.8 | 0 | C | Off Gabo Is |
| 2012-07-31 | 0106 | -37.742 | 150.789 | 15 | 3.4 | 0 | D | Off Gabo Is |
| 2012-08-12 | 1401 | -32.958 | 148.773 | 2 | 2.9 | 0 | C | Molong |
| 2012-08-21 | 0625 | -34.312 | 149.228 | 1 | 2.6 | 0 | C | Crookwell |
| 2012-09-03 | 1913 | -37.538 | 151.362 | 10 | 2.5 | 0 | E | Tasman Sea |
| 2012-09-07 | 0700 | -28.754 | 149.431 | 10 | 2.8 | 0 | D | Moree |
| 2012-09-10 | 0623 | -33.539 | 151.787 | 11 | 3.4 | 0 | C | Off Woy Woy |
| 2012-09-17 | 1306 | -34.164 | 150.782 | 2 | 2.7 | 0 | C | Appin |
| 2012-10-17 | 1817 | -30.930 | 142.191 | 10 | 3.7 | 0 | D | NE of Broken Hill |
| 2012-11-24 | 0657 | -33.456 | 149.010 | 0 | 2.5 | 0 | D | Cadia |
| 2012-12-16 | 1609 | -37.497 | 151.438 | 10 | 2.7 | 0 | D | E of Mallacoota |
| 2012-12-21 | 1526 | -32.394 | 152.226 | 4 | 3.4 | 0 | C | Bulahdelah |

Figure 8.7 Map of 2012 New South Wales Earthquakes recorded by ES&S (circle size scales with magnitude).

Figure 8.8 ES&S Isoseismal map of the June 8 Ml 4.4 Tamworth earthquake

# Northern Territory

Figure 9.1 Map of 2012 Northern Territory Earthquakes recorded by GA (circle size scales with magnitude).

Table 9.1 Northern Territory Top 9 Earthquakes

| Magnitude | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- |
| 6.7 | 22/01/1988 | 12:05:00 | -19.838 | 133.994 | 5 | Tennant Creek, NT. |
| 6.5 | 27/06/1941 | 7:55:49 | -25.950 | 137.340 | 0 | Simpson Desert, NT. |
| 6.4 | 22/01/1988 | 3:57:28 | -19.826 | 133.984 | 4 | Tennant Creek, NT. |
| 6.3 | 22/01/1988 | 0:36:01 | -19.812 | 133.975 | 6 | Tennant Creek, NT. |
| 6.0 | 15/12/1873 | 4:00:00 | -26.250 | 127.500 | 0 | Gibson Desert, NT. |
| 6.0 | 20/12/1937 | 22:35:02 | -25.400 | 136.500 | 33 | Simpson Desert, NT. |
| 5.9 | 17/04/1938 | 8:56:22 | -25.500 | 137.200 | 0 | Simpson Desert, NT. |
| 5.8 | 22/01/1988 | 20:54:05 | -19.887 | 134.084 | 3 | Tennant Creek, NT. |
| 5.8 | 12/03/1999 | 0:29:14 | -11.119 | 130.666 | 100 | Arafura Sea. |

Table 9.2 Northern Territory Earthquakes 2012

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.9 | ML | 28/01/2012 | 19:52:56 | -19.784 | 133.926 | 16 | SW of Tennant Creek, NT. |
| 2.4 | ML | 08/04/2012 | 06:19:10 | -19.841 | 134.042 | 0 | Tennant Creek, NT. |
| 3.6 | ML | 12/04/2012 | 06:50:58 | -25.937 | 132.917 | 10 | W of Kulgera, NT. |
| 3.6 | ML | 19/04/2012 | 21:03:46 | -25.917 | 132.892 | 0 | Near Kulgera, NT. |
| 2.8 | ML | 03/06/2012 | 09:20:02 | -17.083 | 129.173 | 15 | SE of Kununurra, NT. |
| 3.2 | ML | 29/06/2012 | 12:28:56 | -23.931 | 136.882 | 10 | Simpson Desert, NT. |
| 3.5 | ML | 14/09/2012 | 17:10:04 | -20.951 | 129.475 | 5 | NE of Lake Mackay, NT. |
| 2.9 | ML | 27/09/2012 | 15:56:53 | -19.7 | 133.988 | 0 | Tennant Creek, NT. |
| 2.4 | ML | 13/10/2012 | 13:57:41 | -19.769 | 133.897 | 0 | Tennant Creek, NT. |
| 4.5 | MB | 05/11/2012 | 19:31:49 | -11.546 | 129.045 | 39 | Offshore Bathurst Island, NT. |
| 3.6 | ML | 14/12/2012 | 04:14:55 | -19.826 | 133.818 | 11 | SW of Tennant Creek, NT. |
| 1.9 | ML | 28/12/2012 | 17:52:44 | -19.842 | 133.982 | 0 | Tennant Creek, NT. |
| 2.0 | ML | 29/12/2012 | 08:26:46 | -19.846 | 134.014 | 0 | SW of Tennant Creek, NT. |

# Queensland

Figure 10.1 Map of 2012 Queensland Earthquakes recorded by GA (circle size scales with magnitude).

Table 10.1 Queensland Top 8 Earthquakes

| Magnitude | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- |
| 6.0 | 06/06/1918 | 18:14:24 | -23.500 | 152.500 | 15 | East of Rockhampton, QLD. |
| 5.7 | 18/12/1913 | 13:54:00 | -20.000 | 147.000 | 33 | Charters Towers, QLD. |
| 5.7 | 06/06/1918 | 19:20:00 | -23.500 | 152.500 | 0 | East of Rockhampton, QLD. |
| 5.7 | 14/05/1928 | 02:46:55 | -15.000 | 144.000 | 10 | Lakefield National Park, QLD. |
| 5.6 | 28/08/1883 | 16:55:00 | -25.500 | 151.670 | 20 | Gayndah, QLD. |
| 5.5 | 06/06/1918 | 18:23:00 | -23.500 | 152.500 | 0 | East of Rockhampton, QLD. |
| 5.5 | 04/05/1925 | 11:28:48 | -16.000 | 140.000 | 10 | NE of Mornington Island, QLD |
| 5.5 | 12/04/1935 | 01:32:24 | -25.500 | 151.670 | 10 | Gayndah, QLD. |

Table 10.2 Queensland Earthquakes 2012

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.3 | ML | 16/02/2012 | 10:48:30 | -27.054 | 147.980 | 0 | SW of Roma, QLD. |
| 2.9 | ML | 23/09/2012 | 16:29:13 | -26.33 | 151.561 | 0 | Ballogie, QLD. |
| 3.5 | ML | 04/12/2012 | 20:17:43 | -25.785 | 151.473 | 10 | Near Gayndah, QLD. |

## Queensland Seismicity Report

Environmental Systems and Services (ES&S), Centre for Earthquake Research in Australia (CERA) and the University of Queensland (UQ)

Russell Cuthbertson (ES&S), Jack Rynn (CERA) and Dion Weatherley (UQ)

### Introduction

This is a combined 2012 Report by ES&S, CERA and UQ. During 2012, earthquake monitoring and data analysis procedures changed in Queensland. The State Government of Queensland contract with ES&S was terminated on 30 August 2012. From 01 September 2012, the State Government began co-ordinating all seismometry activities (including earthquake monitoring, data analyses and maintenance) in cooperation with State Development, Sunwater and SEQWater. The Queensland JUMP network remained a cooperative program between Geoscience Australia (GA) and the State Government. CERA and UQ have independently compiled the Queensland Earthquake Database (QEDB 1865 – 2012).

### Seismicity

This report deals primarily with the area that was covered by the ES&S Queensland network: i.e. eastern Queensland, from Cairns in the north to just into NSW in the south.

There were three earthquakes of magnitude 4.0 or greater during the year: off Yeppoon (2012-01-05 ML 4.1) and at Mundubbera (2012-12-04 ML 4.0) – both in areas that have consistently had earthquake activity; and at Mitchell (2012-02-16 ML 4.2) in an area that has not had a history of large earthquakes.

### Network

As at 30 August 2012, ES&S were no longer responsible for the operation of the 11 State and Sunwater stations (noting that two of these were decommissioned). Subsequent removal of ES&S equipment from these sites meant that a majority of these ceased operation. Sunwater and the State began upgrading several of these stations, with the installation of ECHO recorders on loan from GA and UQ, to continue monitoring. Sunwater purchased an ES&S Eqserver system to handle data from those sites still in operation.

However, the State then made the decision to cease operations of all its stations, until 2013 because of fiscal constraints. No seismic data has been available from these stations from about 01 September through 31 December 2012.

SEQWater continue to operate their 6 seismograph stations (Nanometrics equipment, telemetered to their Karalee office) and 6 accelerograph stations (Kinemetrics, manually downloaded) in Southeast Queensland.

The Gladstone Area Water Board (GAWB) continues to operate 3 seismograph stations in the Gladstone area with ES&S.

The University of Queensland (UQ) continues to operate the Mt. Nebo (BRSA) and Monto (MNT) seismograph stations.

A private seismograph station near Gin Gin in Central Queensland (code FS03) also continues to operate.

### ES&S Procedures

Standard procedures for earthquake location are:

Unusual, abnormal or unknown blasts are sometimes located and included in the catalogue 
(with a code to indicate the source type)

Overseas events are not located

Events outside the network operated by ES&S (in eastern Australia) are occasionally located.

Events that trigger the ES&S alarm service (operating in eastern Australia) are typically located within 30 minutes.

More detailed analyses of these events, as well as smaller events that did not trigger the alarm, are generally conducted within a week.

Data from GA sites and other independent stations are obtained whenever possible.

Locations are calculated using eqFocus software.

Table 10.3 Queensland Network status 2012

| Station | Owner | Status at Dec 2012 | Comment |
| --- | --- | --- | --- |
| SEISMOGRAPH STATIONS |  |  | X – to be modernised in 2013 In station code : H = hard rock site S = soft rock site F = reclaimed land (fill) site |
| CHIL- Chillagoe | State | not operational | last data : 2012 – 08 – 10 X |
| TINA- Tinaroo Falls Dam | State/Sunwater | not operational | last data : 2012 – 09 – 03 X |
| BURD- Burdekin Falls Dam | State/Sunwater | not operational | last data : 2012 – 09 – 03 X |
| MCPQ- Mt. Cooper | (State/Sunwater) |  | decommissioned |
| UKAQ-Ukalunda | State/Sunwater | intermittent | last data : 2012 – 12 – 31X |
| DLBQ- Dalbeg | State/Sunwater | intermittent | last data : 2012 – 12 – 31X |
| PETE - Peter Faust Dam | State/Sunwater | not operational | last data : 2012 – 08 – 20X |
| FAIR- Fairbairn Dam | State/ Sunwater | not operational | last data : 2012 – 09 – 03X |
| MORG- Mt. Morgan | (State) |  | decommissioned |
| FRED - Fred Haigh Dam | State/Sunwater | not operational | last data : 2012 – 08 – 31X |
| BOON- Boondooma Dam | State/Sunwater | not operational | last data : 2012 – 09 – 03X |
| WWD - Wivenhoe Dam | SEQWater | operational |  |
| WSD- Somerset Dam | SEQwater | operational |  |
| WNP- North Pine Dam | SEQWater | operational |  |
| WRP- Regents Park | SEQWater | operational |  |
| WPC- Paradise Creek | SEQWater | operational |  |
| WOB- O’Briens Hill | SEQWater | operational |  |
| BRSA- Mt. Nebo | UQ | operational | X |
| MNT -Monto | UQ | operational |  |
| BEEN - Beenleigh | UQ |  | decommissioned |
| HNZ2 - Hinze Dam | UQ |  | decommissioned |
| FS03- Four Seasons | Private | operational | X |
| GLADS- Gladstone | GAWB | operational |  |
| EASTN - East End | GAWB | operational |  |
| AWOON - Awoonga Dam | GAWB | operational |  |
| COEN - Coen | GA ( ANSN ) | operational |  |
| MTSU - Mt. Surprise | GA ( ANSN ) | operational |  |
| CTAO - Charters Towers | GA ( ANSN ) | operational |  |
| ISQ - Mt. Isa | GA ( ANSN ) | operational |  |
| EIDS - Eidsvold | GA ( ANSN ) | operational |  |
| RMQ - Roma | GA ( ANSN ) | operational |  |
| QLP- Quilpie | GA ( ANSN ) | operational |  |
| ACCELEROGRAPH STATIONS |  |  |  |
| SW01- Wivenhoe Dam (upper) | SEQWater | operational |  |
| SW02- Wivenhoe Dam (lower) | SEQWater | operational |  |
| SS01 - Somerset Dam(upper) | SEQWater | operational |  |
| SS02 - Somerset Dam(lower) | SEQWater | operational |  |
| SN01 - North Pine Dam(upper) | SEQWater | operational |  |
| SN02 - North Pine Dam(lower) | SEQWater | operational |  |
| 7 ANSN stations | GA | operational |  |
| Qld. State stations |  | Not operational | Planned for several stations but not installed ; no data available |
| JUMP/Triggered STATIONS |  |  | Some now 24-7 live feed |
| CN1H – Cairns - Tunnel Hill | GA / State | operational | commissioned2012 – 02 – 13 |
| CN2S – Cairns - Edmonton | GA / State | operational | commissioned2012 – 02 – 13 |
| TV1H – Townsville - Castle Hill | GA / State | operational | commissioned2011 – 12 – 01 |
| TV2S – Townsville - Anderson Park | GA / State | operational | commissioned2012 – 06 – 25 |
| BW1H – Bowen - Bowen Reservoir | State | operational | commissioned2012 – 06 – 22 |
| BW2S – Bowen - Richmond Road | State | operational | commissioned2012 – 06 – 20 |
| RK1H – Rockhampton - The Range | GA / State | operational | commissioned2009 – 03 – 26 |
| RK2S – Rockhampton - Parkhurst | GA / State | operational | commissioned2009 – 03 – 24 |
| GD1S – Gladstone - Callemondah | State | operational | commissioned2011 – 10 – 05 |
| BN1H – Brisbane - Windsor | GA / State | operational | commissioned2012 – 02 – 28 |
| BN2S – Brisbane - Deagon | GA / State | operational | commissioned2008 – 10 – 29 |
| GC1S – Gold Coast - Coolangatta | GA / State | operational | commissioned2009 – 02 – 25 |
| GC2F – Gold Coast - Runaway Bay | GA / State | operational | commissioned2009 – 02 – 26 |
| TW1H – Toowoomba - Mt. Lofty | GA / State | operational | commissioned2009 – 02 – 12 |
| COEN - Coen | GA ( ANSN ) | operational |  |

Table 10.4 Queensland Magnitude 2.0+ earthquakes recorded by ES&S

| Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Mag | Mmi | Acc | Location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2012-01-05 | 1405 | -21.973 | 152.510 | 10 | 4.1 | 0 | C | Off Yeppoon |
| 2012-02-16 | 1048 | -27.051 | 147.963 | 10 | 4.2 | 0 | C | Mitchell |
| 2012-04-06 | 0643 | -16.996 | 146.117 | 10 | 3.0 | 0 | E | Off Cape Grafton |
| 2012-05-31 | 0807 | -16.605 | 146.144 | 5 | 2.7 | 0 | E | Off Cairns |
| 2012-07-30 | 0334 | -17.025 | 146.159 | 10 | 3.5 | 0 | D | Off Gordonvale |
| 2012-07-31 | 0508 | -17.051 | 146.246 | 7 | 2.8 | 0 | D | Off Gordonvale |
| 2012-09-07 | 0700 | -28.754 | 149.431 | 10 | 2.8 | 0 | D | Moree |
| 2012-09-23 | 1629 | -26.333 | 151.465 | 7 | 3.1 | 0 | C | Boondooma |
| 2012-12-04 | 2017 | -25.872 | 151.182 | 5 | 4.0 | 0 | C | Mundubbera |

Figure 10.2 Map of 2012 Queensland Earthquakes detected by ES&S

### Queensland Earthquake Database (QEDB)

CERA and UQ independently compiled the QEDB Edition 1 (1866 – 2009), which incorporated all available earthquake data files (including CERA, UQ, GA EARTHMON, GA Reports, ES&S, SEQWater, University of Central Queensland, PIRSA, AEES, AAAS, ISS, ISC and other published sources). Details are given in the publication for the 2012 International Geological Congress, Brisbane, by Rynn and Weatherley, 2012.

The QEDB Edition 2 (1865 – 2012) was commenced, which will include all available earthquake data files for 2010 through 2012, newspaper information (per TROVE) pre – 1937 and corrections to QEDB Edition 1.

# South Australia

Figure 11.1 Map of 2012 South Australian Earthquakes recorded by GA (circle size scales with magnitude).

Appendix A.3 includes a table detailing all 2012 South Australian Earthquakes detected by Geoscience Australia.

Table 11.1 South Australian Top 11 Earthquakes

| Magnitude | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- |
| 6.5 | 10/05/1897 | 05:26:00 | -37.333 | 139.750 | 0 | Beachport, SA. |
| 6.0 | 30/03/1986 | 08:53:48 | -26.333 | 132.517 | 5 | Marryat Creek, SA. |
| 6.0 | 19/09/1902 | 10:35:00 | -35.000 | 138.000 | 14 | Warooka, SA. |
| 5.8 | 26/03/1939 | 03:56:00 | -32.000 | 138.000 | 0 | Motpena, SA. |
| 5.8 | 04/05/1941 | 22:07:30 | -26.300 | 136.900 | 33 | Simpson Desert, SA. |
| 5.7 | 16/04/1887 | 22:10:00 | -34.300 | 135.800 | 10 | Eyre Peninsula, SA. |
| 5.7 | 23/03/2012 | 09:25:19 | -26.124 | 132.124 | 0 | Near Ernabella, SA (MW 5.4). |
| 5.6 | 04/05/1941 | 22:23:57 | -26.300 | 136.900 | 33 | Simpson Desert, SA. |
| 5.6 | 06/08/1948 | 03:29:23 | -37.360 | 139.680 | 18 | Robe, SA. |
| 5.5 | 26/10/1911 | 10:00:00 | -33.667 | 136.417 | 0 | Cleve, SA. |
| 5.5 | 18/04/1972 | 22:20:39 | -31.580 | 138.620 | 12 | Wilpena, SA. |

## South Australia Seismicity Report

Department for manufacturing, Innovation, Trade, Resources and Energy

David Love & Alison Wallace

### Seismicity

The largest earthquake of the year was magnitude 5.7 near Ernabella in the state’s far north.This caused a small surface rupture that was investigated by Geoscience Australia. This is the second surface rupture in the region, the previous one being in 1986.

Overall, earthquake activity was slightly lower than 2011 (see Australian Seismological Report 2011). 331 events were located by DMITRE, with 22 being above magnitude 3.

Apart from Ernabella, which had a number of aftershocks, there were no notable earthquake sequences in the year and no portable deployments were undertaken.

A geothermal project (Habanero, by Geodynamics) near Innamincka carried out a fracture stimulation test from 14th November to 2nd December. Many events were recorded up to approximately magnitude 3.9.Most stations of the SA network are more than 600km away, hence technically cannot be used for magnitude calculation.The largest of these was on the 29th November, and has been included in the database.The detailed data from the project will remain confidential for 2 years.

A mining induced event on 25th March at Olympic Dam was well recorded by the SA network, with a magnitude of 3.7.This has been included in the database.

No focal mechanisms or isoseismal maps have been produced for 2012 events.

### Network

The Mount Gambier (MGBR) analogue recorder was converted from analogue to digital on 23rd January.It is a vertical 1Hz seismometer in a 20m borehole in basalt.A new digital station was installed near Robe (ROBE), and commissioned on 12th December. It has a 3D 1Hz seismometer and accelerometer at surface.

Two new private stations were installed at Lobethal and Mylor in the hills east of Adelaide (see ACG-PSN report).

The SA Government network now consists of 5 analogue stations, 4 dial-up stations, 14 continuous, real-time stations and 1 visit and download station (suburban).GA has 7 stations within the state and there are 7 private stations.

### Procedures

There was a slight change to procedures in 2012. Geothermal and mining induced events have been included in the catalogue.

Standard procedure for earthquake location is:

Blasts are usually not located and are not included in the catalogue.

Overseas events are not located.

Interstate events are occasionally located.

Local events triggering the central computer are reviewed as soon as possible using eqFocus.

This preliminary review may contain data from non-DMITRE sites.

Reviewed events are put on the DMITRE website as soon as possible.

Final locations are calculated using the old software Eqlocl, one to two months after actual date, using all available data, including data from Geoscience Australia, Environmental Systems and Services, and ACG-PSN data.

Velocity model being used is called SA1A, with a single layer 38km deep.

Final locations are uploaded overnight to the DMITRE spatial library and are publicly available on the SARIG website.

GA data is requested via AutoDRM, and ACG-PSN phases are read from a web waveform display.

Initial magnitudes are calculated using the Richter formula. 8 stations are calibrated for this method. Final magnitudes are calculated using the Greenhalgh formula. 4 stations are calibrated for this method.

If the GA magnitude is significantly different, the preferred magnitude is listed between the two values.

### Presentations

A simple poster summarizing accurately known earthquakes depths in Australia was produced for IGC2012, and a summary paper on the 2011 Yongala sequence for AEES2012. This latter paper included a number of new focal mechanisms.

Figure 11.2 South Australian seismographs in 2012.Circles showing where changes have occurred or new stations have been installed.

Figure 11.3 Seismicity for 2012 detected and located by DMITRE.2012 seismicity shown in colour, and the complete catalogue of earthquakes (from 1840) shown in grey.

Table 11.2 Magnitude 2.7+ Seismicity located by DMITRE.

| Date | Time UTC | Sec. | Long | Lat | Depth (km) | Place | Mag | MMI | Acc (A-F) | Res | Stns |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2011-01-04 | 1955 | 25.8 | 138.765 | -32.178 | 13.0 | Hawker 45KM SE | 2.8 |  | D | 0.70 | 17 |
| 2011-01-09 | 1853 | 4.5 | 139.267 | -30.402 | 20.8 | Arkaroola 13KM SW | 2.7 | 3 | D | 0.56 | 14 |
| 2011-01-12 | 0122 | 13.2 | 140.759 | -38.629 | 27.1 | MT Gambier 89KM S | 3.3 |  | D | 0.60 | 21 |
| 2011-02-01 | 1946 | 19.7 | 138.693 | -33.086 | 4.0 | Yongala 7KM SW | 2.9 | 3 | C | 0.54 | 22 |
| 2011-02-01 | 1957 | 23.1 | 138.693 | -33.086 | 4.0 | Yongala 7KM SW | 3.0 | 3 | C | 0.56 | 25 |
| 2011-02-04 | 0841 | 7.0 | 133.119 | -34.889 | 15.2 | PT Lincoln 252KM W | 3.8 |  | D | 0.65 | 23 |
| 2011-02-05 | 0152 | 43.5 | 139.574 | -33.409 | 13.5 | Peterborough 84KM SE | 2.7 |  | C | 0.53 | 19 |
| 2011-02-22 | 1411 | 13.2 | 137.819 | -26.956 | 37.0 | Poeppel Corner 108KM S | 3.3 |  | E | 1.18 | 10 |
| 2011-02-26 | 1425 | 3.4 | 136.011 | -34.045 | 5.2 | Cummins 36KM NE | 2.8 |  | C | 0.52 | 20 |
| 2011-03-06 | 2234 | 22.4 | 139.663 | -37.728 | 32.1 | Beachport 41KM SW | 2.8 |  | E | 0.67 | 16 |
| 2011-03-12 | 1206 | 51.5 | 138.278 | -33.218 | 12.6 | Port Pirie 26KM E | 3.0 | 3 | C | 0.56 | 23 |
| 2011-03-16 | 1124 | 35.8 | 132.330 | -26.189 | 2.0 | Ernabella 20KM NE | 4.3 | 4 | E | 2.77 | 18 |
| 2011-03-20 | 0325 | 21.4 | 132.330 | -26.189 | 2.0 | Ernabella 20KM NE | 4.2 | 4 | E | 3.07 | 12 |
| 2011-03-21 | 1707 | 11.8 | 136.098 | -28.376 | 4.9 | William Creek 62KM N | 4.1 |  | D | 1.10 | 25 |
| 2011-03-23 | 0925 | 14.8 | 132.330 | -26.189 | 1.4 | Ernabella 20KM NE | 5.7 | 4 | E | 1.54 | 27 |
| 2011-02-23 | 1624 | 42.2 | 138.722 | -33.389 | 9.4 | Peterborough 47KM S | 2.7 | 3 | C | 0.55 | 22 |
| 2011-03-25 | 2053 | 22.2 | 136.850 | -30.440 | 0.0 | Olympic Dam Mine Induced | 3.7 | 4 | D | 0.71 | 19 |
| 2011-04-08 | 1156 | 9.6 | 132.153 | -26.315 | 28.7 | Ernabella | 3.7 | 4 | E | 1.82 | 21 |
| 2011-04-08 | 1845 | 50.6 | 139.939 | -28.822 | 22.1 | Cameron Corner 106KM W | 3.2 |  | E | 1.46 | 8 |
| 2011-04-12 | 0651 | 0.2 | 132.968 | -26.052 | 29.1 | Kulgera 40KM SW | 3.6 |  | E | 1.85 | 18 |
| 2011-04-19 | 2103 | 48.1 | 132.976 | -26.050 | 20.3 | Kulgera 39KM SW | 3.8 |  | E | 1.49 | 16 |
| 2011-04-26 | 1457 | 28.7 | 139.008 | -31.552 | 16.5 | Hawker 66KM NE | 2.8 |  | C | 0.65 | 20 |
| 2011-05-24 | 1906 | 33.0 | 141.738 | -31.318 | 20.9 | Broken Hill 77KM N | 3.6 |  | D | 0.92 | 27 |
| 2011-06-04 | 0409 | 23.6 | 138.436 | -33.334 | 13.4 | Port Pirie 43KM E | 3.0 |  | C | 0.56 | 26 |
| 2011-06-11 | 2107 | 26.3 | 138.558 | -32.332 | 18.1 | Hawker 51KM E | 2.9 |  | C | 0.48 | 23 |
| 2011-06-17 | 0339 | 0.6 | 135.512 | -27.697 | 10.0 | Oodnadatta 19KM SE | 3.2 |  | E | 1.47 | 12 |
| 2011-06-22 | 1421 | 15.7 | 138.710 | -33.070 | 2.7 | Yongala 6KM SW | 2.7 |  | C | 0.56 | 22 |
| 2011-06-26 | 1340 | 37.6 | 138.564 | -31.693 | 20.7 | Hawker 25KM NE | 2.8 |  | C | 0.54 | 21 |
| 2011-06-26 | 2007 | 1.2 | 136.980 | -33.296 | 14.8 | Cowell 43KM N | 2.8 |  | C | 0.61 | 22 |
| 2011-07-01 | 1617 | 54.9 | 138.768 | -31.700 | 13.6 | Hawker 38KM NE | 2.7 |  | C | 0.56 | 16 |
| 2011-07-09 | 0655 | 34.1 | 133.780 | -33.562 | 18.3 | Elliston 99KM W | 2.9 |  | C | 0.53 | 21 |
| 2011-07-10 | 0539 | 2.5 | 137.734 | -31.774 | 30.1 | Hawker 67KM W | 2.7 |  | C | 0.78 | 21 |
| 2011-08-19 | 1627 | 18.5 | 135.472 | -27.528 | 7.7 | Oodnadatta 4KM NE | 3.6 |  | E | 1.68 | 24 |
| 2011-09-09 | 0841 | 59.0 | 140.791 | -38.552 | 30.1 | PT Macdonnell 56KM S | 3.0 |  | D | 0.59 | 21 |
| 2011-09-09 | 0923 | 55.1 | 140.794 | -38.562 | 31.3 | PT Macdonnell 56KM S | 3.1 |  | D | 0.67 | 22 |
| 2011-09-11 | 0713 | 30.4 | 132.291 | -26.308 | 10.2 | Ernabella | 3.0 |  | E | 1.19 | 15 |
| 2011-09-20 | 1241 | 53.8 | 136.372 | -33.606 | 7.7 | Cleve 14KM NW | 2.8 |  | C | 0.62 | 20 |
| 2011-09-26 | 0522 | 36.5 | 132.275 | -26.292 | 32.2 | Ernabella | 3.5 |  | E | 1.75 | 16 |
| 2011-10-08 | 1028 | 34.8 | 137.065 | -25.474 | 33.1 | Simpson Desert | 3.0 |  | E | 2.26 | 9 |
| 2011-10-17 | 1816 | 57.7 | 141.900 | -30.661 | 17.9 | Broken Hill 152KM N | 3.9 |  | C | 0.58 | 28 |
| 2011-11-08 | 0356 | 57.1 | 127.209 | -29.907 | 32.5 | Eucla 260KM NW | 3.6 |  | E | 2.97 | 22 |
| 2011-11-13 | 0747 | 41.3 | 139.784 | -28.308 | 10.0 | Arkaroola 222KM N | 3.0 |  | E | 2.17 | 9 |
| 2011-11-23 | 1704 | 0.0 | 137.813 | -28.543 | 33.2 | Marree 127KM N | 2.7 |  | E | 1.86 | 18 |
| 2011-11-28 | 2235 | 15.4 | 125.975 | -28.017 | 10.0 | Warburton WA 212KM S | 4.0 |  | E |  | 24 |
| 2011-11-29 | 1921 | 0.0 | 140.760 | -27.811 | 4.0 | Habanero Induced | 3.9 |  | D |  | 0 |
| 2011-12-04 | 1751 | 6.4 | 139.233 | -30.026 | 17.3 | Arkaroola 36KM N | 3.5 |  | D |  | 22 |

# Tasmania

Figure 12.1 Map of 2012 Tasmanian Earthquakes recorded by GA (circle size scales with magnitude).

Table 12.1 Tasmanian Top 10 Earthquakes

| Magnitude | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- |
| 6.9 | 26/01/1892 | 16:48:00 | -40.400 | 149.500 | 0 | Tasman Sea NE TAS. |
| 6.4 | 13/07/1884 | 3:55:00 | -40.500 | 148.500 | 0 | Cape Barren Island NE TAS. |
| 6.4 | 19/08/1884 | 10:27:00 | -40.800 | 149.500 | 0 | NE of Gladstone, TAS. |
| 5.9 | 28/03/1884 | 13:12:00 | -41.200 | 148.900 | 0 | E of Bay of Fires, TAS. |
| 5.9 | 10/05/1884 | 21:30:00 | -41.200 | 148.900 | 0 | E of Bay of Fires, TAS. |
| 5.9 | 09/06/1884 | 9:20:00 | -40.800 | 149.500 | 0 | NE of Gladstone, TAS. |
| 5.9 | 20/03/1885 | 23:03:00 | -40.500 | 149.500 | 0 | E of Cape Barren Isl, TAS. |
| 5.4 | 28/12/1929 | 01:22:00 | -39.690 | 149.450 | 10 | Flinders Island, TAS. |
| 5.3 | 10/04/1922 | 20:47:00 | -40.000 | 147.500 | 33 | Cape Barren Island, TAS. |
| 5.3 | 01/01/1958 | 10:07:22 | -42.200 | 146.100 | 10 | Near Lake St. Clair, TAS. |

Table 12.2 Tasmanian Earthquakes 2012

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.5 | ML | 13/01/2012 | 20:46:28 | -42.034 | 145.674 | 5 | Near Queenstown, TAS. |
| 3.4 | ML | 22/01/2012 | 20:18:53 | -42.042 | 145.544 | 11 | Near Queenstown, TAS. |
| 2.8 | ML | 24/01/2012 | 18:34:17 | -42.035 | 145.53 | 10 | Near Queenstown, TAS. |
| 2.8 | ML | 25/02/2012 | 01:16:46 | -40.408 | 149.228 | 18 | E of Flinders Island, TAS. |
| 2.5 | ML | 03/09/2012 | 19:13:46 | -37.499 | 151.24 | 0 | Tasman sea. |
| 3.0 | ML | 16/09/2012 | 15:57:17 | -39.566 | 150.002 | 4 | E of Flinders Island, Australia. |
| 2.1 | ML | 21/09/2012 | 19:27:20 | -42.035 | 145.896 | 10 | NE of Queenstown, Tas. |
| 2.5 | ML | 01/10/2012 | 13:05:20 | -43.429 | 147.544 | 16 | Off Bruny Island, Tas. |
| 2.3 | ML | 24/11/2012 | 22:54:40 | -40.458 | 145.197 | 13 | N of Smithton, Tas. |

## Tasmania Seismicity Report

Environmental Systems and Services

Compiled by Elodie Borleis

### Seismicity

This report deals primarily with the area covered by the ES&S Tasmanian seismic network.The largest event for Tasmania was a magnitude ML 3.5 earthquake on January 22nd northeast of Queenstown. This was followed by a series of aftershocks, the largest of which was a magnitude ML 3.0 recorded on January 24th.

Nine events with magnitude greater or equal to ML 2.5 were located in Tasmania by ES&S in 2012.Excluding the aftershocks from the Queenstown earthquake, there were five of magnitude ML 3.0+.

### Network

The Tasmanian seismic monitoring network operated by the ES&S Seismology Research Centre now comprises 8 seismographs and accelerographs throughout Tasmania.All sites except SCOT are continuously telemetered.

### Procedures

Standard procedures for earthquake location are:

Unusual, abnormal or unknown blasts are sometimes located and included in the catalogue (with a code to indicate the source type)

Overseas events are not located

Events outside the network operated by ES&S (in eastern Australia) are occasionally located.

Events that trigger the ES&S alarm service (operating in eastern Australia) are typically located within 30 minutes.

More detailed analyses of these events, as well as smaller events that did not trigger the alarm, are generally conducted within a week.

Data from GA sites and other independent stations are obtained whenever possible.

Locations are calculated using eqFocus software.

Figure 12.2 ES&S Tasmanian Seismic Network

Table 12.3 Tasmanian magnitude ML 2.5+ earthquakes recorded by Environmental Systems & Services

| Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Mag | Mmi | Acc | Location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2012-01-22 | 2018 | -42.083 | 145.550 | 1 | 3.5 | 3 | C | Queenstown |
| 2012-01-24 | 1834 | -42.110 | 145.532 | 2 | 3.0 | 3 | D | Queenstown |
| 2012-02-25 | 0116 | -40.347 | 149.139 | 20 | 3.2 | 0 | F | E of Flinders Is |
| 2012-06-03 | 1054 | -41.255 | 144.665 | 2 | 2.8 | 0 | D | Off Tarkine Coast |
| 2012-08-19 | 1039 | -40.322 | 143.939 | 10 | 3.0 | 0 | E | S of King Island |
| 2012-08-19 | 1209 | -40.398 | 144.038 | 10 | 2.5 | 0 | E | S of King Island |
| 2012-09-16 | 1557 | -39.492 | 149.788 | 10 | 3.4 | 0 | E | S of Mallacoota |
| 2012-10-01 | 1305 | -43.648 | 147.171 | 11 | 3.1 | 0 | D | S of Bruny Island |
| 2012-12-31 | 0814 | -43.807 | 146.186 | 10 | 2.8 | 0 | E | Off Southern Tasmania |

Figure 12.3 Map of 2012 Tasmanian Earthquakes detected by ES&S (circle size scales with magnitude).

# Victoria

Figure 13.1 Map of 2012 Victorian Earthquakes recorded by GA (circle size scales with magnitude).

Table 13.1 Victorian Top 7 Earthquakes

| Magnitude | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- |
| 5.7 | 02/07/1885 | 16:03:00 | -39.000 | 146.000 | 0 | Cape Liptrap, VIC. |
| 5.7 | 14/08/65 | 12:53:13 | -38.700 | 144.300 | 10 | Bass Strait, VIC. |
| 5.7 | 03/05/66 | 19:07:54 | -37.100 | 147.170 | 24 | Mt Hotham, VIC. |
| 5.4 | 21/11/82 | 11:34:18 | -37.205 | 146.956 | 17 | NW of Dargo, VIC. |
| 5.3 | 14/07/03 | 10:29:00 | -38.430 | 142.530 | 0 | Warrnambool, VIC. |
| 5.3 | 24/12/60 | 16:42:10 | -38.880 | 143.590 | 0 | Cape Otway, VIC. |
| 5.3 | 20/06/69 | 11:15:28 | -38.470 | 146.300 | 19 | Boolarra, VIC. |

Table 13.2 Victorian Earthquakes 2012

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.6 | ML | 02/01/2012 | 12:46:51 | -38.631 | 146.349 | 62 | N of Toora, VIC. |
| 2.0 | ML | 16/02/2012 | 12:04:28 | -36.488 | 147.635 | 10 | Near Lake Dartmouth, VIC. |
| 2.3 | ML | 05/03/2012 | 15:19:07 | -36.622 | 147.898 | 10 | NE of Benambra, VIC. |
| 3.1 | ML | 24/03/2012 | 05:06:02 | -38.443 | 145.849 | 8 | Korumburra, VIC. |
| 2.6 | ML | 23/04/2012 | 00:29:05 | -38.59 | 143.051 | 10 | Port Campbell, VIC. |
| 2.6 | ML | 02/05/2012 | 21:55:02 | -38.431 | 145.85 | 0 | Korumburra, Vic. |
| 2.4 | ML | 06/05/2012 | 03:37:34 | -38.647 | 147.073 | 0 | Off Woodside Beach, VIC. |
| 2.2 | ML | 06/05/2012 | 20:01:08 | -38.472 | 147.061 | 0 | Near Darriman, VIC. |
| 2.5 | ML | 25/05/2012 | 03:45:38 | -38.214 | 145.846 | 10 | Near Korumburra, VIC. |
| 2.6 | ML | 15/06/2012 | 17:37:01 | -38.037 | 144.323 | 0 | NW of Geelong, Vic. |
| 5.4 | ML | 19/06/2012 | 10:53:29 | -38.304 | 146.2 | 10 | SW of Moe, Vic. |
| 3.1 | ML | 19/06/2012 | 11:14:47 | -38.346 | 146.198 | 4 | SW of Moe, Vic. |
| 3.1 | ML | 19/06/2012 | 14:37:34 | -38.293 | 146.18 | 10 | SW of Moe, Vic. |
| 2.4 | ML | 19/06/2012 | 15:18:23 | -38.299 | 146.196 | 6 | SW of Moe, Vic. |
| 2.7 | ML | 19/06/2012 | 16:55:44 | -38.312 | 146.237 | 14 | SW of Moe, Vic. |
| 2.4 | ML | 19/06/2012 | 18:00:41 | -38.212 | 146.324 | 10 | SE of Moe, Vic. |
| 2.7 | ML | 19/06/2012 | 19:17:17 | -38.282 | 146.223 | 10 | SW of Moe, Vic. |
| 2.4 | ML | 22/06/2012 | 17:56:34 | -38.474 | 145.849 | 0 | Korumburra, VIC. |
| 2.7 | ML | 22/06/2012 | 19:38:36 | -38.463 | 145.758 | 0 | Korumburra, VIC. |
| 3.1 | ML | 30/06/2012 | 03:02:35 | -38.32 | 146.269 | 12 | S of Moe, Vic. |
| 2.9 | ML | 08/07/2012 | 10:27:33 | -38.339 | 146.176 | 14 | Near Moe, Vic. |
| 2.4 | ML | 16/07/2012 | 22:04:43 | -38.291 | 146.192 | 0 | SW of Moe, Vic. |
| 2.4 | ML | 17/07/2012 | 01:56:00 | -38.159 | 146.256 | 0 | Moe, Vic. |
| 2.7 | ML | 18/07/2012 | 08:09:04 | -38.088 | 146.271 | 0 | N of Moe, Vic. |
| 2.9 | ML | 20/07/2012 | 04:42:01 | -38.335 | 146.191 | 13 | Near Moe, Vic. |
| 4.4 | ML | 20/07/2012 | 09:11:30 | -38.307 | 146.23 | 0 | Near Moe, Vic. |
| 2.2 | ML | 20/07/2012 | 09:25:08 | -38.23 | 146.239 | 1 | Near Moe, Vic. |
| 2.1 | ML | 20/07/2012 | 09:30:42 | -38.165 | 146.282 | 4 | Near Moe, Vic. |
| 2.8 | ML | 20/07/2012 | 10:00:16 | -38.302 | 146.205 | 0 | Near Moe, Vic. |
| 2.2 | ML | 20/07/2012 | 11:38:42 | -38.296 | 146.21 | 0 | Near Moe, Vic. |
| 2.4 | ML | 22/07/2012 | 01:53:09 | -37.082 | 147.141 | 10 | Dinner Plain, Vic. |
| 2.0 | ML | 24/07/2012 | 17:33:24 | -38.268 | 146.292 | 10 | S of Moe, Vic. |
| 2.4 | ML | 25/07/2012 | 18:31:03 | -38.592 | 147.324 | 10 | SE of Seaspray, Vic. |
| 3.5 | ML | 30/07/2012 | 17:25:19 | -37.74 | 150.505 | 2 | SE of Mallacoota, Vic. |
| 3.2 | ML | 30/07/2012 | 17:28:26 | -37.811 | 150.583 | 17 | SE of Mallacoota, Vic. |
| 2.9 | ML | 30/07/2012 | 19:56:05 | -37.827 | 150.576 | 15 | SE of Mallacoota, Vic. |
| 2.9 | ML | 31/07/2012 | 00:08:04 | -38.627 | 146.636 | 12 | Near Yarram, Vic. |
| 3.1 | ML | 31/07/2012 | 01:06:58 | -37.843 | 150.428 | 0 | SE of Mallacoota, Vic. |
| 2.3 | ML | 12/09/2012 | 20:34:13 | -38.509 | 145.77 | 0 | Korumburra, Vic. |
| 2.4 | ML | 01/10/2012 | 17:08:56 | -38.115 | 149.384 | 17 | Off Point Hicks, Vic. |
| 2.7 | ML | 02/10/2012 | 11:38:31 | -36.855 | 144.111 | 0 | SW of Bendigo, Vic. |
| 2.2 | ML | 05/10/2012 | 05:19:28 | -36.594 | 147.975 | 10 | Near Mt Murphy, Vic. |
| 3.1 | ML | 11/10/2012 | 22:47:09 | -37.893 | 143.591 | 10 | Rokewood, Vic. |
| 3.0 | ML | 27/10/2012 | 04:43:14 | -38.31 | 146.251 | 12 | Near Moe, Vic. |
| 1.7 | ML | 29/10/2012 | 04:31:35 | -38.41 | 145.984 | 0 | Near Korumburra, Vic. |
| 2.3 | ML | 30/10/2012 | 16:43:22 | -37.752 | 146.276 | 15 | Baw Baw, Vic. |
| 2.4 | ML | 31/10/2012 | 08:35:55 | -38.193 | 145.749 | 15 | SW of Drouin, Vic. |
| 2.6 | ML | 22/12/2012 | 07:41:20 | -38.462 | 145.824 | 10 | Near Korumburra, Vic. |

Figure 13.2 Isoseismal map of the June 19th ML 5.4 Moe earthquake (GA)

Figure 13.3 Isoseismal map of the July 20th ML 4.4 Moe earthquake (GA)

## Victoria Seismicity Report

Environmental Systems and Services

Compiled by: Elodie Borleis

## Seismicity

This report deals primarily with the area covered by the ES&S Victorian seismic network.Twenty-six events with magnitude greater or equal to ML 2.5 were located in Victoria by ES&S in 2012.

From 2009 to present there has been an exceptionally high occurrence of seismicity related to significant earthquakes recorded in Gippsland, Victoria.On June 19th 2012 a magnitude ML 5.4 earthquake was recorded, the largest recorded in Victoria since the magnitude 5.7 Mt Hotham earthquake in 1966.The fault ruptured along a region approximately 10 kilometres south of Moe, 125 kilometres southeast of Melbourne.This earthquake was felt right across Victoria, with reports of it being felt from as far away as Deniliquin in NSW - around 330km from the epicentre. Data from the intensity reports sent to ES&S for the earthquake were used to create an isoseismal map.There have been over 300 aftershocks recorded to date, the largest of which was a magnitude ML 4.3 recorded on July 20th.

### Network

The Victorian seismic monitoring network operated by the ES&S Seismology Research Centre now comprises 32 seismographs and accelerographs throughout Victoria.

Moondarra Reservoir MOON was retired in 2012.Glenmaggie GLMM and Willung South WILL were both ungraded and are now transmitting continuously back to the ES&S lab in Richmond, Victoria.

### Procedures

Standard procedures for earthquake location are:

Unusual, abnormal or unknown blasts are sometimes located and included in the catalogue (with a code to indicate the source type)

Overseas events are not located

Events outside the network operated by ES&S (in eastern Australia) are occasionally located.

Events that trigger the ES&S alarm service (operating in eastern Australia) are typically located within 30 minutes.

More detailed analyses of these events, as well as smaller events that did not trigger the alarm, are generally conducted within a week.

Data from GA sites and other independent stations are obtained whenever possible.

Locations are calculated using eqFocus software.

Figure 13.4 ES&S Victorian Seismic Network

Table 13.3 Victorian magnitude ML 2.5+ earthquakes recorded by Environmental Systems & Services

| Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Mag | Mmi | Acc | Location |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2012-01-02 | 1246 | -38.562 | 146.370 | 14 | 2.7 | 0 | C | Foster |
| 2012-02-25 | 0003 | -37.429 | 145.963 | 10 | 2.5 | 0 | C | Lake Mountain |
| 2012-03-24 | 0506 | -38.428 | 145.804 | 9 | 3.3 | 0 | C | Korumburra |
| 2012-04-23 | 0029 | -38.523 | 143.095 | 15 | 2.7 | 0 | C | Timboon |
| 2012-05-02 | 2155 | -38.406 | 145.851 | 7 | 2.6 | 0 | C | Korumburra |
| 2012-05-25 | 0345 | -38.406 | 145.851 | 8 | 2.5 | 0 | C | Korumburra |
| 2012-06-15 | 1737 | -38.065 | 144.326 | 7 | 2.5 | 3 | C | Geelong |
| 2012-06-19 | 1053 | -38.283 | 146.271 | 17 | 5.4 | 6 | B | Moe |
| 2012-06-19 | 1058 | -38.291 | 146.246 | 12 | 2.6 | 0 | C | Moe |
| 2012-06-19 | 1100 | -38.290 | 146.252 | 14 | 3.0 | 0 | B | Moe |
| 2012-06-19 | 1114 | -38.276 | 146.205 | 17 | 3.1 | 0 | D | Moe |
| 2012-06-19 | 1143 | -38.298 | 146.182 | 18 | 2.6 | 0 | D | Moe |
| 2012-06-19 | 1344 | -38.298 | 148.421 | 10 | 2.6 | 0 | E | Off Lakes Entrance |
| 2012-06-19 | 1437 | -38.285 | 146.462 | 11 | 3.3 | 0 | B | Churchill |
| 2012-06-30 | 0302 | -38.288 | 146.287 | 15 | 2.8 | 0 | C | Moe |
| 2012-07-08 | 1027 | -38.261 | 146.222 | 9 | 3.1 | 0 | B | Moe |
| 2012-07-18 | 0809 | -38.220 | 146.218 | 12 | 2.8 | 0 | B | Moe |
| 2012-07-20 | 0442 | -38.245 | 146.252 | 11 | 2.8 | 0 | B | Moe |

Figure 13.5 Map of 2012 Victorian Earthquakes detected by ES&S (circle size scales with magnitude).

Figure 13.6 ES&S Isoseismal map of the June 19th ML 5.4 Moe earthquake

# Western Australia

Figure 14.1 Map of 2012 Western Australian Earthquakes recorded by GA (circle size scales with magnitude).

Appendix A.4 includes a table detailing all 2012 Western Australian Earthquakes detected by Geoscience Australia.

Table 14.1 Western Australian Top 9 Earthquakes 2012

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4.0 | ML | 28/11/2012 | 22:35:22 | -27.757 | 125.943 | 0 | SW of Warburton, WA. |
| 3.7 | ML | 28/12/2012 | 12:04:44 | -24.344 | 112.396 | 3 | Offshore Western Australia. |
| 3.6 | ML | 08/11/2012 | 03:56:55 | -29.898 | 126.992 | 0 | Nullarbor Plain, WA. |
| 3.5 | ML | 18/07/2012 | 09:20:55 | -30.429 | 117.794 | 2 | Beacon, WA. |
| 3.5 | ML | 12/10/2012 | 19:27:14 | -22.881 | 127.681 | 10 | W of Lake Mackay, WA. |
| 3.4 | ML | 29/03/2012 | 05:05:21 | -29.031 | 114.224 | 3 | SW of Geraldton, WA. |
| 3.4 | ML | 09/03/2012 | 06:16:26 | -30.410 | 117.840 | 8 | Beacon, WA. |
| 3.4 | ML | 25/03/2012 | 16:57:41 | -30.405 | 117.843 | 10 | Beacon, WA. |
| 3.4 | ML | 01/12/2012 | 17:19:07 | -16.195 | 127.661 | 10 | SW of Kununurra, WA. |

Table 14.2 Western Australian Top 11 Earthquakes

| Magnitude | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- |
| 7.3 | 29/04/1941 | 1:35:39 | -26.920 | 115.800 | 0 | Meeberrie WA. |
| 6.9 | 14/10/1968 | 2:58:50 | -31.620 | 116.980 | 10 | Meckering WA. |
| 6.7 | 24/03/1970 | 10:35:17 | -22.050 | 126.610 | 8 | L Mckay WA. |
| 6.6 | 16/08/1929 | 21:28:23 | -16.990 | 120.660 | 33 | Broome WA. |
| 6.6 | 23/04/1979 | 5:45:10 | -16.660 | 120.270 | 26 | Broome WA. |
| 6.5 | 05/01/1885 | 14:25:00 | -29.000 | 114.000 | 0 | Near Geraldton, WA. |
| 6.3 | 10/08/1997 | 9:20:31 | -16.050 | 124.423 | 10 | Collier Bay, WA. |
| 6.2 | 8-Feb-1920 | 5:24:30 | -35.000 | 111.000 | 33 | Offshore WA. |
| 6.2 | 6/05/1978 | 19:52:19 | -19.550 | 126.560 | 17 | L Mckay WA. |
| 6.2 | 2/06/1979 | 9:47:59 | -30.827 | 117.179 | 3 | Cadoux WA. |

# Australian Region

Geoscience Australia detects and locates earthquakes of magnitude 5 and higher in the Australian region, which for earthquakes encompasses Latitude 10 degrees to -70 degrees and Longitude 85 degrees to -150 degrees. Appendix A.5 includes a table detailing all 2012 Magnitude 5+ Regional Earthquakes detected by Geoscience Australia.

Figure 15.1 Map of 2012 Magnitude 5+ Australian Region Earthquakes recorded by GA.

Table 15.1 Australian Region Top 10 Earthquakes 2012

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8.6 | Mwp | 11/04/2012 | 08:38:32 | 02.277 | 92.981 | 0 | Off W Coast of Northern Sumatra, Indonesia. |
| 8.2 | Mwp | 11/04/2012 | 10:43:11 | 00.801 | 92.411 | 30 | Off W Coast of Northern Sumatra, Indonesia. |
| 7.3 | Mwp | 02/02/2012 | 13:34:40 | -17.749 | 167.243 | 22 | Vanuatu Islands. |
| 7.2 | Mwp | 10/01/2012 | 18:37:00 | 02.508 | 93.187 | 20 | Off W Coast of Northern Sumatra. |
| 7.1 | Mwp | 10/12/2012 | 16:53:09 | -06.634 | 129.823 | 151 | Banda Sea. |
| 7.0 | Mwp | 06/02/2012 | 03:49:18 | 09.931 | 123.278 | 63 | Cebu, Philippines. |
| 6.8 | Mwp | 12/10/2012 | 00:31:27 | -04.910 | 134.060 | 9 | Irian Jaya Region, Indonesia. |
| 6.7 | Mwp | 03/03/2012 | 12:19:58 | -22.164 | 170.404 | 30 | SE of Loyalty Islands, New Caledonia. |
| 6.7 | Mwp | 21/03/2012 | 22:15:07 | -06.301 | 146.078 | 123 | Northwest of Lae, PNG. |

# World

Geoscience Australia detects and locates any earthquakes in the World of Magnitude 6 and higher. Appendix A.6 includes a table detailing all 2012 Magnitude 6+ Earthquakes detected by Geoscience Australia.

Figure 16.1 Map of 2012 Magnitude 6+ World Earthquakes recorded by GA.

Table 16.1 World Top 8 Earthquakes 2012

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8.6 | Mwp | 11/04/2012 | 08:38:32 | 2.277 | 92.981 | 0 | Off W Coast of Northern Sumatra, Indonesia. |
| 8.2 | Mwp | 11/04/2012 | 10:43:11 | 0.801 | 92.411 | 30 | Off W Coast of Northern Sumatra, Indonesia. |
| 7.7 | Mwp | 14/08/2012 | 02:59:42 | 49.784 | 145.126 | 626 | Sea of Okhotsk, Russia. |
| 7.7 | Mwp | 28/10/2012 | 03:04:11 | 52.769 | -131.927 | 18 | Queen Charlotte Islands Region. |
| 7.6 | Mwp | 31/08/2012 | 12:47:35 | 10.838 | 126.704 | 35 | Philippine Islands Region. |
| 7.6 | Mwp | 05/09/2012 | 14:42:08 | 9.996 | -85.318 | 40 | Off the Coast of Costa Rica. |
| 7.4 | Mwp | 07/11/2012 | 16:35:50 | 14.083 | -91.916 | 42 | Offshore Guatemala. |
| 7.4 | Mwp | 20/03/2012 | 18:02:49 | 16.662 | -98.188 | 20 | Oaxaca, Mexico. |

References

Burbidge, D.R. (ed.), 2012. The 2012 Australian Earthquake Hazard Map. Record 2012/71. Geoscience Australia: Canberra.

Clark, D., McPherson, A. and Collins, C. D. N. 2011. Australia’s seismogenic neotectonic record: a case for heterogeneous intraplate deformation. Geoscience Australia Record 2011/11, 95 pp.

Gaull, B.A. and Gregson, P.J., 1991. A new local magnitude scale for Western Australian earthquakes.Australian Journal of Earth Sciences. Vol. 38. pp. 251-260

Greenhalgh, S.A. and Singh, R., 1986. A revised Magnitude Scale for South Australian Earthquakes. Bulletin of the Seismological Society of America. Vol. 76, No. 3 pp. 757-769.

Leonard M. and Clark D. 2011. A record of stable continental region earthquakes from Western Australia spanning the late Pleistocene: Insights for contemporary seismicity. Earth and Planetary Science Letters DOI:10.1016/j.espl.2011.06.035.

Michael-Leiba, M. and Malafant, K., 1992. A new local magnitude scale for southeastern Australia. BMR Journal of Australian Geology & Geophysics. Vol. 13. pp. 201-205.

Rynn, J.M.W. and Weatherley, D., 2012 : Seismicity 1866 – 2009 . In “ Geology of Queensland “ (Edited by P.A. Jell), State Government of Queensland, Geological Survey of Queensland Publication, Chapter 11, pp 763 – 769.

Antelope® Software: http://www.brtt.com/software.html

Appendices

Australian Earthquakes

Table A.1 Australian Earthquakes 2012 (Recorded by Geoscience Australia)

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.1 | ML | 01/01/2012 | 16:37:04 | -31.652 | 117.118 | 10 | E of Meckering, WA. |
| 2.0 | ML | 01/01/2012 | 16:37:05 | -31.709 | 117.066 | 17 | SE of Meckering, WA. |
| 2.6 | ML | 02/01/2012 | 12:46:51 | -38.631 | 146.349 | 62 | N of Toora, VIC. |
| 1.7 | ML | 02/01/2012 | 19:18:28 | -32.517 | 138.363 | 10 | Hammond, SA. |
| 3.1 | ML | 03/01/2012 | 05:21:35 | -29.945 | 142.648 | 3 | SE of Tibooburra, NSW. |
| 2.0 | ML | 03/01/2012 | 10:53:38 | -33.315 | 136.978 | 0 | N of Cowell, SA. |
| 1.7 | ML | 03/01/2012 | 12:05:53 | -36.273 | 149.792 | 11 | NW of Cobargo, NSW. |
| 2.3 | ML | 04/01/2012 | 19:55:23 | -32.186 | 138.916 | 10 | NE of Carrieton, SA. |
| 1.8 | ML | 05/01/2012 | 11:46:56 | -34.291 | 148.935 | 0 | Near Frogmore, NSW. |
| 1.9 | ML | 06/01/2012 | 08:45:07 | -30.479 | 117.108 | 0 | NW of Burakin, WA. |
| 1.6 | ML | 06/01/2012 | 09:55:02 | -34.323 | 148.888 | 0 | Frogmore, NSW, |
| 1.7 | ML | 06/01/2012 | 11:58:51 | -34.284 | 148.936 | 0 | Frogmore, NSW. |
| 1.8 | ML | 07/01/2012 | 13:17:45 | -34.302 | 149.229 | 10 | NW of Crookwell, NSW. |
| 2.8 | ML | 09/01/2012 | 18:53:03 | -30.516 | 139.247 | 10 | SW of Arkaroola, SA. |
| 3.3 | ML | 10/01/2012 | 18:44:01 | -33.308 | 118.222 | 0 | NE of Nyabing, WA. |
| 2.5 | ML | 11/01/2012 | 03:37:04 | -14.809 | 132.037 | 12 | SW of Katherine, WA. |
| 3.3 | ML | 12/01/2012 | 01:22:13 | -38.566 | 140.929 | 19 | Offshore, S of Mt Gambier, SA. |
| 2.0 | ML | 12/01/2012 | 12:10:08 | -32.140 | 117.151 | 8 | E of Beverley, WA. |
| 2.7 | ML | 13/01/2012 | 19:02:13 | -30.681 | 121.554 | 4 | Kalgoorlie, WA. |
| 2.5 | ML | 13/01/2012 | 20:46:28 | -42.034 | 145.674 | 5 | Near Queenstown, TAS. |
| 2.8 | ML | 15/01/2012 | 21:33:45 | -31.502 | 144.599 | 11 | W of Cobar, NSW. |
| 2.2 | ML | 16/01/2012 | 06:41:22 | -30.620 | 117.542 | 0 | SE of Burakin, WA. |
| 2.1 | ML | 17/01/2012 | 17:21:03 | -30.828 | 118.764 | 10 | SE of Bonnie Rock, WA. |
| 1.7 | ML | 17/01/2012 | 18:54:32 | -31.729 | 117.028 | 7 | S of Meckering, WA. |
| 1.9 | ML | 18/01/2012 | 12:16:38 | -36.261 | 148.874 | 0 | Near Berridale, NSW. |
| 1.8 | ML | 18/01/2012 | 14:57:28 | -30.283 | 117.684 | 0 | NW of Beacon, WA. |
| 1.0 | ML | 18/01/2012 | 16:19:52 | -33.097 | 137.857 | 15 | SW of Port Germein, SA. |
| 2.3 | ML | 19/01/2012 | 01:26:07 | -35.264 | 135.829 | 10 | S of Port Lincoln, SA. |
| 2.0 | ML | 20/01/2012 | 12:39:45 | -30.318 | 117.665 | 14 | NW of Beacon, WA. |
| 2.0 | ML | 20/01/2012 | 20:49:33 | -31.664 | 117.058 | 19 | SE of Meckering, WA. |
| 2.1 | ML | 21/01/2012 | 17:51:49 | -33.777 | 148.714 | 16 | Cowra, NSW. |
| 2.0 | ML | 21/01/2012 | 21:19:06 | -35.029 | 149.024 | 3 | Murrumbateman, NSW. |
| 3.4 | ML | 22/01/2012 | 20:18:53 | -42.042 | 145.544 | 11 | Near Queenstown, TAS. |
| 2.0 | ML | 24/01/2012 | 00:20:16 | -31.145 | 116.683 | 16 | E of Calingiri, WA. |
| 3.0 | ML | 24/01/2012 | 11:42:30 | -30.449 | 116.935 | 11 | SW of Kalannie, WA. |
| 1.9 | ML | 24/01/2012 | 12:20:44 | -30.438 | 116.932 | 4 | SW of Kalannie, WA. |
| 2.8 | ML | 24/01/2012 | 18:34:17 | -42.035 | 145.530 | 10 | Near Queenstown, TAS. |
| 2.5 | ML | 27/01/2012 | 19:18:46 | -34.168 | 139.535 | 20 | SW of Morgan, SA. |
| 1.5 | ML | 28/01/2012 | 14:13:30 | -34.516 | 148.846 | 0 | Near Boorowa, NSW. |
| 2.9 | ML | 28/01/2012 | 19:52:56 | -19.784 | 133.926 | 16 | SW of Tennant Creek, NT. |
| 2.1 | ML | 28/01/2012 | 20:17:33 | -32.050 | 138.711 | 10 | SE of Hawker, SA. |
| 2.7 | ML | 29/01/2012 | 13:45:02 | -32.183 | 118.988 | 10 | NE of Hyden, WA. |
| 2.3 | ML | 01/02/2012 | 13:03:52 | -33.054 | 138.601 | 15 | Mannanarie, SA. |
| 2.3 | ML | 01/02/2012 | 19:46:19 | -33.126 | 138.616 | 5 | Mannanarie, SA. |
| 2.3 | ML | 01/02/2012 | 19:57:23 | -33.073 | 138.632 | 0 | Mannanarie, SA. |
| 1.9 | ML | 03/02/2012 | 13:49:24 | -33.063 | 138.627 | 0 | Mannanarie, SA. |
| 1.8 | ML | 03/02/2012 | 15:41:40 | -34.809 | 149.203 | 1 | Gunning, NSW. |
| 2.1 | ML | 03/02/2012 | 19:58:03 | -32.575 | 116.783 | 1 | W of Pingelly, WA. |
| 3.3 | ML | 04/02/2012 | 08:41:05 | -35.143 | 133.319 | 0 | W of Port Lincoln, SA. |
| 2.1 | ML | 04/02/2012 | 15:30:52 | -34.650 | 149.102 | 0 | Dalton, NSW. |
| 2.3 | ML | 05/02/2012 | 01:52:41 | -33.292 | 139.661 | 10 | NE of Burra, SA. |
| 2.6 | ML | 06/02/2012 | 10:57:41 | -32.941 | 151.468 | 6 | Brunkerville, NSW. |
| 2.2 | ML | 08/02/2012 | 19:53:06 | -30.683 | 118.442 | 9 | SE of Bonnie Rock, WA. |
| 2.4 | ML | 09/02/2012 | 22:48:50 | -33.178 | 113.896 | 30 | W of Bunbury, WA. |
| 1.9 | ML | 14/02/2012 | 21:44:34 | -32.090 | 138.307 | 14 | SW of Hawker, SA. |
| 1.6 | ML | 15/02/2012 | 01:31:40 | -36.175 | 148.787 | 11 | Lake Eucumbene, NSW. |
| 1.8 | ML | 15/02/2012 | 23:01:41 | -34.703 | 151.037 | 10 | E of Kiama, NSW. |
| 2.0 | ML | 16/02/2012 | 04:33:54 | -32.627 | 117.983 | 7 | SW of Condinin, WA. |
| 3.3 | ML | 16/02/2012 | 10:48:30 | -27.054 | 147.980 | 0 | SW of Roma, Queensland. |
| 2.0 | ML | 16/02/2012 | 12:04:28 | -36.488 | 147.635 | 10 | Near Lake Dartmouth, VIC. |
| 1.8 | ML | 16/02/2012 | 20:49:04 | -30.943 | 117.801 | 12 | SW of Bencubbin, WA. |
| 1.9 | ML | 16/02/2012 | 22:13:06 | -30.979 | 117.805 | 17 | SW of Bencubbin, WA. |
| 2.9 | ML | 18/02/2012 | 04:02:15 | -24.096 | 113.603 | 10 | Lake Macleod, WA. |
| 2.0 | ML | 18/02/2012 | 12:21:24 | -30.448 | 116.909 | 7 | NE of Ballidu, WA. |
| 1.9 | ML | 18/02/2012 | 16:42:40 | -33.313 | 118.582 | 1 | SE of Lake Grace, WA. |
| 1.3 | ML | 20/02/2012 | 07:48:30 | -34.066 | 139.311 | 0 | NE of Eudunda, SA. |
| 1.7 | ML | 20/02/2012 | 10:14:26 | -30.403 | 117.749 | 10 | NW of Beacon, WA. |
| 1.9 | ML | 20/02/2012 | 15:25:58 | -33.613 | 117.670 | 14 | NE of Katanning, WA. |
| 1.9 | ML | 20/02/2012 | 15:52:43 | -33.605 | 117.642 | 10 | NE of Katanning, WA. |
| 1.8 | ML | 21/02/2012 | 00:28:04 | -33.177 | 138.107 | 6 | Napperby, SA. |
| 1.9 | ML | 21/02/2012 | 01:50:42 | -32.052 | 138.537 | 10 | SE of Hawker, SA. |
| 1.9 | ML | 21/02/2012 | 09:21:03 | -34.654 | 150.794 | 0 | Jamberoo, NSW. |
| 2.3 | ML | 22/02/2012 | 03:54:22 | -29.719 | 115.866 | 13 | Yarra Yarra Lakes, WA. |
| 2.5 | ML | 23/02/2012 | 22:16:36 | -30.500 | 118.651 | 10 | NE of Bonnie Rock, WA. |
| 1.4 | ML | 24/02/2012 | 02:46:51 | -33.488 | 138.628 | 11 | Spalding, SA. |
| 2.4 | ML | 24/02/2012 | 17:51:19 | -27.276 | 119.636 | 0 | SE of Wilson, WA. |
| 2.8 | ML | 25/02/2012 | 01:16:46 | -40.408 | 149.228 | 18 | E of Flinders Island, TAS. |
| 1.8 | ML | 25/02/2012 | 06:46:45 | -33.963 | 148.423 | 4 | SW of Cowra, NSW. |
| 1.5 | ML | 25/02/2012 | 21:38:56 | -30.437 | 116.913 | 6 | NE of Ballidu. |
| 2.3 | ML | 26/02/2012 | 10:29:26 | -37.802 | 140.020 | 10 | S of Southend, SA. |
| 2.4 | ML | 26/02/2012 | 14:25:02 | -34.019 | 135.906 | 7 | Brooker, SA. |
| 2.3 | ML | 26/02/2012 | 16:51:02 | -32.935 | 151.469 | 5 | Brunkerville, NSW. |
| 1.9 | ML | 29/02/2012 | 12:44:36 | -34.295 | 149.075 | 0 | NE of Boorowa, NSW. |
| 1.3 | ML | 01/03/2012 | 16:45:33 | -33.598 | 138.373 | 0 | Koolunga, SA. |
| 2.1 | ML | 01/03/2012 | 16:53:30 | -33.323 | 118.011 | 5 | NW of Nyabing, WA. |
| 2.0 | ML | 01/03/2012 | 18:23:48 | -30.403 | 117.765 | 10 | NW of Beacon, WA. |
| 2.2 | ML | 01/03/2012 | 18:53:32 | -30.395 | 117.792 | 0 | NW of Beacon, WA. |
| 1.9 | ML | 02/03/2012 | 18:32:08 | -33.056 | 138.737 | 15 | NE of Jamestown, SA. |
| 2.3 | ML | 05/03/2012 | 15:19:07 | -36.622 | 147.898 | 10 | NE of Benambra, VIC. |
| 3.1 | ML | 05/03/2012 | 17:20:52 | -16.820 | 120.616 | 33 | NW of Broome, WA. |
| 1.7 | ML | 06/03/2012 | 04:15:27 | -35.032 | 149.031 | 0 | Murrumbateman area, NSW. |
| 2.3 | ML | 06/03/2012 | 18:28:43 | -31.068 | 138.560 | 17 | NW of Blinman, SA. |
| 2.8 | ML | 06/03/2012 | 22:34:20 | -37.594 | 139.639 | 10 | SW of Beachport, SA. |
| 2.4 | ML | 07/03/2012 | 08:54:47 | -31.190 | 118.349 | 10 | NE of Merredin, WA. |
| 2.0 | ML | 08/03/2012 | 06:45:17 | -30.870 | 121.100 | 0 | Near Kalgoorlie, WA. |
| 2.5 | ML | 09/03/2012 | 02:32:30 | -30.400 | 117.821 | 12 | Beacon, WA. |
| 3.4 | ML | 09/03/2012 | 06:16:26 | -30.410 | 117.840 | 8 | Beacon, WA. |
| 3.3 | ML | 09/03/2012 | 06:59:47 | -30.397 | 117.822 | 9 | Beacon, WA. |
| 2.3 | ML | 09/03/2012 | 08:27:14 | -30.407 | 117.716 | 0 | Beacon, WA. |
| 2.1 | ML | 09/03/2012 | 16:50:44 | -30.565 | 117.619 | 0 | SW of Beacon, WA. |
| 1.8 | ML | 09/03/2012 | 17:42:44 | -34.580 | 148.534 | 14 | Galong, NSW. |
| 2.9 | ML | 10/03/2012 | 05:30:04 | -34.125 | 117.005 | 10 | SW of Kojonup, WA. |
| 2.3 | ML | 11/03/2012 | 01:21:51 | -30.478 | 116.964 | 11 | NW of Burakin, WA. |
| 2.7 | ML | 11/03/2012 | 18:21:51 | -36.329 | 148.755 | 15 | NE of Jindabyne, NSW. |
| 2.1 | ML | 12/03/2012 | 05:25:12 | -30.397 | 117.803 | 5 | Beacon, WA. |
| 2.7 | ML | 12/03/2012 | 12:06:51 | -33.145 | 138.307 | 17 | Laura, SA. |
| 1.6 | ML | 12/03/2012 | 13:30:08 | -36.313 | 148.722 | 15 | NE of Jindabyne, NSW. |
| 2.7 | ML | 13/03/2012 | 18:48:14 | -28.795 | 116.347 | 0 | NE of Morawa, WA. |
| 3.0 | ML | 13/03/2012 | 20:26:01 | -34.134 | 117.186 | 15 | S of Kojonup, WA. |
| 1.8 | ML | 14/03/2012 | 16:21:48 | -36.326 | 148.628 | 12 | Jindabyne, NSW. |
| 2.1 | ML | 15/03/2012 | 03:54:14 | -33.489 | 150.156 | 7 | Lithgow, NSW. |
| 2.0 | ML | 15/03/2012 | 19:20:17 | -32.551 | 138.527 | 10 | S of Carrieton, SA. |
| 4.3 | ML | 16/03/2012 | 11:24:36 | -26.174 | 131.824 | 15 | Musgrave Ranges, SA. |
| 3.0 | ML | 16/03/2012 | 13:04:01 | -27.009 | 113.952 | 5 | N of Kalbarri, WA. |
| 2.1 | ML | 16/03/2012 | 16:19:15 | -36.302 | 148.858 | 0 | Barridale, NSW. |
| 1.3 | ML | 17/03/2012 | 18:38:17 | -33.496 | 138.894 | 15 | N of Burra, SA. |
| 2.3 | ML | 17/03/2012 | 22:42:50 | -34.132 | 117.225 | 0 | S of Kojonup, WA. |
| 2.1 | ML | 19/03/2012 | 21:02:52 | -31.951 | 117.237 | 10 | NW of Quairading, WA. |
| 3.8 | ML | 20/03/2012 | 03:25:23 | -26.144 | 132.287 | 10 | Musgrave Ranges, SA. |
| 2.6 | ML | 20/03/2012 | 15:28:07 | -33.872 | 151.782 | 0 | Tasman Sea, E of Sydney, NSW. |
| 3.9 | ML | 21/03/2012 | 17:07:09 | -28.231 | 136.001 | 0 | SE of Oodnadatta, SA. |
| 1.9 | ML | 22/03/2012 | 19:02:12 | -32.614 | 137.764 | 0 | S of Port Augusta, SA. |
| 5.7 | ML | 23/03/2012 | 09:25:19 | -26.124 | 132.124 | 0 | Near Ernabella, SA (MW 5.4). |
| 1.8 | ML | 23/03/2012 | 14:52:12 | -36.031 | 148.539 | 0 | Lake Eucumbene, NSW. |
| 2.3 | ML | 23/03/2012 | 16:24:42 | -33.337 | 138.760 | 9 | SE of Jamestown, SA. |
| 1.3 | ML | 23/03/2012 | 18:11:29 | -33.358 | 138.738 | 0 | SE of Jamestown, SA. |
| 3.1 | ML | 24/03/2012 | 05:06:02 | -38.443 | 145.849 | 8 | Korumburra, VIC. |
| 2.0 | ML | 24/03/2012 | 21:09:24 | -30.481 | 117.698 | 6 | Beacon, WA. |
| 2.7 | ML | 25/03/2012 | 14:52:08 | -30.406 | 117.817 | 12 | Beacon, WA. |
| 1.9 | ML | 25/03/2012 | 15:07:07 | -30.439 | 117.758 | 0 | Beacon, WA. |
| 2.3 | ML | 25/03/2012 | 15:40:50 | -30.375 | 117.827 | 10 | Beacon, WA. |
| 3.4 | ML | 25/03/2012 | 16:57:41 | -30.405 | 117.843 | 10 | Beacon, WA. |
| 3.9 | ML | 25/03/2012 | 20:53:20 | -30.324 | 136.821 | 0 | Near Andamooka, SA. |
| 1.7 | ML | 25/03/2012 | 22:36:05 | -33.443 | 138.700 | 0 | NE of Spalding, SA. |
| 2.2 | ML | 26/03/2012 | 03:08:45 | -30.422 | 117.802 | 10 | Beacon, WA. |
| 1.6 | ML | 26/03/2012 | 03:14:18 | -30.498 | 117.689 | 10 | Beacon, WA. |
| 2.3 | ML | 26/03/2012 | 17:53:18 | -28.615 | 115.962 | 1 | N of Morawa, WA. |
| 1.7 | ML | 26/03/2012 | 23:40:50 | -31.560 | 116.931 | 10 | Meckering, WA. |
| 2.2 | ML | 27/03/2012 | 03:41:44 | -32.146 | 118.845 | 10 | N of Hyden, WA. |
| 3.4 | ML | 29/03/2012 | 05:05:21 | -29.031 | 114.224 | 3 | SW of Geraldton, WA. |
| 1.6 | ML | 29/03/2012 | 17:31:08 | -34.561 | 148.669 | 0 | Near Boorowa, NSW. |
| 2.0 | ML | 29/03/2012 | 21:16:55 | -34.630 | 148.902 | 0 | NE of Bowning, NSW. |
| 3.6 | ML | 30/03/2012 | 06:27:09 | -26.209 | 132.186 | 10 | Near Ernabella, SA. |
| 1.6 | ML | 30/03/2012 | 08:54:50 | -32.659 | 138.872 | 20 | NE of Orroroo, SA. |
| 2.3 | ML | 31/03/2012 | 08:02:05 | -31.189 | 138.789 | 20 | SE of Blinman, SA. |
| 1.8 | ML | 31/03/2012 | 15:18:39 | -34.867 | 149.406 | 0 | SE of Gunning, NSW. |
| 2.4 | ML | 06/04/2012 | 18:18:53 | -29.464 | 118.485 | 10 | W of Lake Barlee, WA. |
| 1.8 | ML | 07/04/2012 | 09:42:43 | -33.359 | 118.173 | 3 | Near Merilup, WA. |
| 1.8 | ML | 07/04/2012 | 14:28:09 | -33.473 | 138.679 | 7 | Spalding, SA. |
| 2.1 | ML | 07/04/2012 | 21:30:54 | -30.373 | 117.806 | 5 | Beacon, WA. |
| 2.4 | ML | 08/04/2012 | 06:19:10 | -19.841 | 134.042 | 0 | Tennant Creek, NT. |
| 3.7 | ML | 08/04/2012 | 11:56:06 | -26.090 | 131.901 | 0 | Near Ernabella, SA. |
| 2.1 | ML | 08/04/2012 | 13:35:26 | -30.389 | 117.756 | 0 | Beacon, WA. |
| 1.9 | ML | 08/04/2012 | 23:00:41 | -30.407 | 117.768 | 4 | Beacon, WA. |
| 1.9 | ML | 09/04/2012 | 11:07:14 | -30.420 | 117.757 | 17 | Beacon, WA. |
| 2.3 | ML | 09/04/2012 | 14:27:57 | -34.347 | 150.953 | 12 | NE of Wollongong. NSW. |
| 1.7 | ML | 09/04/2012 | 21:36:51 | -34.824 | 149.210 | 9 | S of Dalton, NSW. |
| 2.0 | ML | 11/04/2012 | 06:53:10 | -31.227 | 118.217 | 13 | N of Merredin, WA. |
| 1.8 | ML | 11/04/2012 | 18:07:00 | -31.239 | 117.451 | 0 | Wyalkatchem, WA. |
| 3.6 | ML | 12/04/2012 | 06:50:58 | -25.937 | 132.917 | 10 | W of Kulgera, NT. |
| 2.3 | ML | 13/04/2012 | 22:55:13 | -33.276 | 118.963 | 0 | E of Lake Grace, WA. |
| 1.9 | ML | 18/04/2012 | 17:00:53 | -35.662 | 148.309 | 11 | S of Talbingo, NSW. |
| 3.7 | ML | 19/04/2012 | 19:09:16 | -35.231 | 148.603 | 4 | South of Wee Jasper, NSW. |
| 3.6 | ML | 19/04/2012 | 21:03:46 | -25.917 | 132.892 | 0 | Near Kulgera, NT. |
| 2.1 | ML | 20/04/2012 | 14:51:58 | -31.236 | 118.285 | 16 | N of Merredin, WA. |
| 2.1 | ML | 20/04/2012 | 16:47:08 | -31.238 | 118.180 | 11 | N of Merredin, WA. |
| 2.4 | ML | 20/04/2012 | 19:01:46 | -31.702 | 117.070 | 10 | SE of Meckering, WA. |
| 2.1 | ML | 20/04/2012 | 21:13:52 | -31.202 | 118.152 | 7 | N of Merredin, WA. |
| 2.1 | ML | 20/04/2012 | 21:14:50 | -31.218 | 118.127 | 8 | NW of Merredin, WA. |
| 2.1 | ML | 20/04/2012 | 21:19:50 | -31.224 | 118.143 | 8 | NW of Merredin, WA. |
| 2.1 | ML | 20/04/2012 | 21:21:34 | -31.191 | 118.134 | 5 | NW of Merredin, WA. |
| 3.0 | ML | 21/04/2012 | 01:33:15 | -31.222 | 118.203 | 11 | N of Merredin, WA. |
| 1.9 | ML | 21/04/2012 | 06:37:59 | -36.025 | 148.430 | 10 | W of Lake Eucumbene, NSW. |
| 2.1 | ML | 22/04/2012 | 18:50:04 | -33.599 | 117.759 | 10 | Nyabing, WA. |
| 2.0 | ML | 22/04/2012 | 19:39:33 | -31.244 | 118.171 | 11 | NW of Merredin, WA. |
| 2.6 | ML | 23/04/2012 | 00:29:05 | -38.590 | 143.051 | 10 | Port Campbell, VIC. |
| 2.3 | ML | 23/04/2012 | 02:13:36 | -31.184 | 118.344 | 7 | NE of Merredin, WA. |
| 1.9 | ML | 23/04/2012 | 11:59:39 | -31.209 | 118.162 | 8 | NW of Merredin, WA. |
| 1.6 | ML | 24/04/2012 | 18:01:39 | -32.477 | 117.268 | 5 | NE of Pingelly, WA. |
| 1.7 | ML | 24/04/2012 | 18:47:01 | -32.468 | 117.279 | 12 | NE of Pingelly, WA. |
| 1.7 | ML | 24/04/2012 | 19:33:13 | -32.531 | 117.297 | 10 | E of Pingelly, WA. |
| 1.7 | ML | 24/04/2012 | 19:34:43 | -32.485 | 117.275 | 7 | NE of Pingelly, WA. |
| 1.7 | ML | 24/04/2012 | 19:40:06 | -32.517 | 117.288 | 10 | E of Pingelly, WA. |
| 1.7 | ML | 24/04/2012 | 19:42:55 | -32.491 | 117.337 | 11 | NE of Pingelly, WA. |
| 1.7 | ML | 24/04/2012 | 19:48:55 | -32.519 | 117.285 | 2 | E of Pingelly, WA. |
| 1.8 | ML | 24/04/2012 | 23:33:22 | -32.515 | 117.292 | 6 | E of Pingelly, WA. |
| 1.7 | ML | 25/04/2012 | 00:26:34 | -32.490 | 117.260 | 9 | E of Pingelly, WA. |
| 1.8 | ML | 25/04/2012 | 05:21:31 | -30.525 | 117.324 | 7 | E of Burakin, WA. |
| 2.2 | ML | 26/04/2012 | 09:50:18 | -29.142 | 116.875 | 1 | Mongers Lake, WA. |
| 2.4 | ML | 26/04/2012 | 14:57:26 | -31.377 | 138.980 | 0 | SE of Blinman, SA. |
| 2.1 | ML | 27/04/2012 | 16:18:54 | -33.375 | 148.068 | 0 | Forbes, NSW. |
| 2.5 | ML | 29/04/2012 | 14:34:53 | -33.331 | 118.004 | 12 | E of Dumbleyung, WA. |
| 2.1 | ML | 30/04/2012 | 12:00:20 | -33.104 | 138.701 | 4 | NE of Jamestown, SA. |
| 1.7 | ML | 30/04/2012 | 12:22:25 | -33.119 | 138.695 | 0 | NE of Jamestown, SA. |
| 1.5 | ML | 30/04/2012 | 14:11:21 | -33.138 | 138.678 | 0 | NE of Jamestown, SA. |
| 1.8 | ML | 30/04/2012 | 14:28:21 | -33.115 | 138.713 | 2 | NE of Jamestown, SA. |
| 2.0 | ML | 01/05/2012 | 12:18:08 | -36.652 | 148.516 | 1 | SE of Thredbo, NSW. |
| 2.3 | ML | 01/05/2012 | 13:35:26 | -32.684 | 138.377 | 10 | NW of Orroroo, SA. |
| 1.8 | ML | 01/05/2012 | 19:24:51 | -31.782 | 117.380 | 7 | SE of Cunderdin, WA. |
| 2.4 | ML | 01/05/2012 | 21:57:24 | -33.572 | 138.549 | 12 | Spalding, SA. |
| 1.9 | ML | 02/05/2012 | 01:45:48 | -31.561 | 117.611 | 10 | NW of Kellerberrin, WA. |
| 2.6 | ML | 02/05/2012 | 21:55:02 | -38.431 | 145.850 | 0 | Korumburra, Vic. |
| 2.0 | ML | 03/05/2012 | 14:27:42 | -33.291 | 118.794 | 10 | E of Lake Grace, WA. |
| 1.2 | ML | 04/05/2012 | 20:48:33 | -33.565 | 138.414 | 0 | Near Spalding, SA. |
| 2.7 | ML | 05/05/2012 | 05:05:16 | -29.789 | 137.779 | 10 | SW of Marree, SA. |
| 2.1 | ML | 05/05/2012 | 11:56:52 | -33.214 | 138.677 | 11 | Near Jamestown, SA. |
| 2.4 | ML | 06/05/2012 | 03:37:34 | -38.647 | 147.073 | 0 | Off Woodside Beach, VIC. |
| 2.1 | ML | 06/05/2012 | 07:24:29 | -33.207 | 137.120 | 0 | 47km SW of Whyalla, SA. |
| 2.2 | ML | 06/05/2012 | 20:01:08 | -38.472 | 147.061 | 0 | Near Darriman, VIC. |
| 1.9 | ML | 07/05/2012 | 16:48:14 | -36.296 | 148.797 | 10 | Near Berridale, NSW. |
| 2.7 | ML | 08/05/2012 | 02:55:26 | -28.634 | 116.099 | 10 | N of Morawa. |
| 2.2 | ML | 08/05/2012 | 10:42:00 | -30.272 | 117.729 | 10 | NW of Beacon, WA. |
| 2.5 | ML | 08/05/2012 | 15:18:50 | -30.217 | 117.841 | 16 | N of Beacon, WA. |
| 2.8 | ML | 10/05/2012 | 22:28:49 | -23.903 | 115.850 | 10 | SW of Nanutarra, WA. |
| 1.8 | ML | 11/05/2012 | 20:35:18 | -30.261 | 117.717 | 5 | NW of Beacon, WA. |
| 1.8 | ML | 14/05/2012 | 08:47:46 | -33.070 | 138.708 | 0 | Yongala, SA. |
| 2.0 | ML | 16/05/2012 | 06:50:53 | -30.686 | 117.432 | 9 | N of Koorda, WA. |
| 1.7 | ML | 19/05/2012 | 16:30:10 | -33.358 | 116.166 | 0 | Collie, WA. |
| 2.2 | ML | 24/05/2012 | 02:01:37 | -30.590 | 117.678 | 6 | SW of Beacon, WA. |
| 3.1 | ML | 24/05/2012 | 19:06:31 | -31.297 | 141.732 | 12 | NE of Broken Hill, NSW. |
| 2.5 | ML | 25/05/2012 | 03:45:38 | -38.214 | 145.846 | 10 | Near Korumburra, VIC. |
| 1.9 | ML | 25/05/2012 | 14:12:37 | -30.602 | 117.614 | 12 | SW of Beacon, WA. |
| 2.0 | ML | 25/05/2012 | 14:54:19 | -30.609 | 117.613 | 13 | SW of Beacon, WA. |
| 2.0 | ML | 25/05/2012 | 17:12:03 | -30.477 | 117.694 | 17 | W of Beacon, WA. |
| 2.2 | ML | 26/05/2012 | 16:03:01 | -32.579 | 138.916 | 10 | Near Orroroo, SA. |
| 2.2 | ML | 27/05/2012 | 16:59:34 | -31.274 | 117.668 | 10 | Near Wyalkatchem, WA. |
| 1.8 | ML | 29/05/2012 | 20:17:26 | -33.260 | 118.793 | 0 | SW of Newdegate, WA. |
| 1.4 | ML | 01/06/2012 | 17:47:33 | -34.619 | 148.908 | 10 | Near Bowning, NSW. |
| 2.8 | ML | 03/06/2012 | 09:20:02 | -17.083 | 129.173 | 15 | SE of Kununurra, NT. |
| 2.4 | ML | 04/06/2012 | 04:09:24 | -33.381 | 138.376 | 10 | Georgetown, SA. |
| 3.2 | ML | 05/06/2012 | 09:12:10 | -34.337 | 148.698 | 12 | N of Boorowa, NSW. |
| 2.2 | ML | 05/06/2012 | 09:45:51 | -34.254 | 148.763 | 10 | N of Boorowa, NSW. |
| 2.3 | ML | 05/06/2012 | 17:42:03 | -34.349 | 148.746 | 0 | N of Boorowa, NSW. |
| 2.1 | ML | 06/06/2012 | 10:26:00 | -31.951 | 138.066 | 0 | Near Hawker, SA. |
| 2.0 | ML | 06/06/2012 | 22:15:30 | -34.288 | 148.955 | 1 | NE of Boorowa, NSW. |
| 2.0 | ML | 07/06/2012 | 17:10:05 | -36.324 | 148.804 | 7 | Berridale, NSW. |
| 4.2 | ML | 08/06/2012 | 11:31:00 | -30.759 | 150.413 | 0 | NW of Tamworth, NSW. |
| 4.1 | ML | 08/06/2012 | 11:32:39 | -30.748 | 150.400 | 0 | NW of Tamworth, NSW. |
| 1.8 | ML | 09/06/2012 | 17:12:32 | -31.242 | 117.429 | 0 | Wyalkatchem, WA. |
| 1.8 | ML | 09/06/2012 | 19:54:40 | -30.364 | 117.680 | 12 | NW of Beacon, WA. |
| 1.8 | ML | 09/06/2012 | 22:16:15 | -30.427 | 117.654 | 8 | W of Beacon, WA. |
| 2.1 | ML | 10/06/2012 | 16:28:38 | -31.615 | 116.994 | 0 | Meckering, WA. |
| 2.7 | ML | 11/06/2012 | 21:07:20 | -32.082 | 138.645 | 0 | SE of Hawker, SA. |
| 2.4 | ML | 12/06/2012 | 01:23:52 | -31.606 | 138.598 | 8 | NE of Hawker, SA. |
| 1.6 | ML | 12/06/2012 | 18:09:14 | -33.414 | 138.108 | 10 | Crystal Brook, SA. |
| 1.9 | ML | 13/06/2012 | 13:27:28 | -34.402 | 148.708 | 0 | Boorowa, NSW. |
| 2.3 | ML | 13/06/2012 | 14:31:08 | -34.372 | 148.628 | 20 | NW of Boorowa, NSW. |
| 2.2 | ML | 14/06/2012 | 23:06:17 | -30.798 | 117.761 | 0 | Bencubbin, WA. |
| 2.6 | ML | 15/06/2012 | 17:37:01 | -38.037 | 144.323 | 0 | NW of Geelong, Vic. |
| 2.7 | ML | 16/06/2012 | 19:46:02 | -35.074 | 149.084 | 10 | N of Canberra, NSW. |
| 2.2 | ML | 17/06/2012 | 17:01:20 | -31.573 | 117.033 | 10 | Meckering, WA. |
| 1.7 | ML | 17/06/2012 | 20:01:43 | -31.683 | 117.070 | 6 | Meckering, WA. |
| 1.6 | ML | 17/06/2012 | 20:20:24 | -31.723 | 117.083 | 1 | SE of Meckering, WA. |
| 2.0 | ML | 18/06/2012 | 01:52:35 | -31.211 | 116.570 | 3 | Bolgart, WA. |
| 5.4 | ML | 19/06/2012 | 10:53:29 | -38.304 | 146.200 | 10 | SW of Moe, Vic. |
| 3.1 | ML | 19/06/2012 | 11:14:47 | -38.346 | 146.198 | 4 | SW of Moe, Vic. |
| 3.1 | ML | 19/06/2012 | 14:37:34 | -38.293 | 146.180 | 10 | SW of Moe, Vic. |
| 2.4 | ML | 19/06/2012 | 15:18:23 | -38.299 | 146.196 | 6 | SW of Moe, Vic. |
| 2.7 | ML | 19/06/2012 | 16:55:44 | -38.312 | 146.237 | 14 | SW of Moe, Vic. |
| 2.4 | ML | 19/06/2012 | 18:00:41 | -38.212 | 146.324 | 10 | SE of Moe, Vic. |
| 2.7 | ML | 19/06/2012 | 19:17:17 | -38.282 | 146.223 | 10 | SW of Moe, Vic. |
| 2.4 | ML | 20/06/2012 | 13:40:28 | -33.611 | 142.726 | 2 | SE of Broken Hill, NSW. |
| 1.8 | ML | 20/06/2012 | 16:58:51 | -30.735 | 116.237 | 0 | Near Moora, WA. |
| 3.7 | ML | 22/06/2012 | 09:16:03 | -34.108 | 148.644 | 14 | N of Boorowa, NSW. |
| 2.2 | ML | 22/06/2012 | 14:21:16 | -33.060 | 138.781 | 6 | Yongala, SA. |
| 2.3 | ML | 22/06/2012 | 15:55:29 | -34.125 | 148.672 | 0 | N of Boorowa, NSW. |
| 2.4 | ML | 22/06/2012 | 17:56:34 | -38.474 | 145.849 | 0 | Korumburra, VIC. |
| 2.7 | ML | 22/06/2012 | 19:38:36 | -38.463 | 145.758 | 0 | Korumburra, VIC. |
| 2.8 | ML | 23/06/2012 | 15:40:54 | -30.672 | 121.557 | 10 | NE of Kalgoorlie, WA. |
| 2.8 | ML | 23/06/2012 | 15:55:25 | -30.649 | 121.554 | 1 | NE of Kalgoorlie, WA. |
| 2.0 | ML | 23/06/2012 | 17:17:48 | -34.212 | 148.659 | 11 | N of Boorowa, NSW. |
| 1.9 | ML | 23/06/2012 | 21:25:42 | -35.162 | 139.193 | 0 | Murray Bridge, SA. |
| 2.2 | ML | 23/06/2012 | 21:33:16 | -30.491 | 117.719 | 17 | Beacon, WA. |
| 2.5 | ML | 26/06/2012 | 13:40:36 | -31.651 | 138.529 | 15 | NE of Hawker, SA. |
| 2.2 | ML | 26/06/2012 | 20:07:02 | -33.331 | 136.980 | 15 | SW of Whyalla, SA. |
| 2.2 | ML | 27/06/2012 | 23:37:53 | -34.130 | 148.706 | 12 | N of Boorowa, NSW. |
| 2.3 | ML | 28/06/2012 | 04:58:59 | -31.686 | 117.067 | 5 | Meckering, WA. |
| 2.3 | ML | 28/06/2012 | 11:22:50 | -30.457 | 117.684 | 10 | W of Beacon, WA. |
| 3.2 | ML | 29/06/2012 | 12:28:56 | -23.931 | 136.882 | 10 | Simpson Desert, NT. |
| 2.0 | ML | 30/06/2012 | 01:08:43 | -34.169 | 148.726 | 0 | N of Boorowa, NSW. |
| 3.1 | ML | 30/06/2012 | 03:02:35 | -38.320 | 146.269 | 12 | S of Moe, Vic. |
| 2.0 | ML | 30/06/2012 | 08:32:49 | -34.125 | 148.714 | 10 | N of Boorowa, NSW. |
| 2.2 | ML | 30/06/2012 | 09:56:03 | -34.073 | 148.631 | 10 | N of Boorowa, NSW. |
| 2.6 | ML | 01/07/2012 | 16:17:51 | -31.575 | 138.812 | 10 | NE of Hawker, SA. |
| 2.6 | ML | 04/07/2012 | 09:15:04 | -34.158 | 150.352 | 12 | W of Picton, NSW. |
| 2.2 | ML | 05/07/2012 | 17:21:09 | -33.447 | 136.953 | 12 | Near Cowell, SA. |
| 2.0 | ML | 06/07/2012 | 07:58:07 | -35.634 | 138.565 | 17 | SW of Victor Harbor, SA. |
| 2.5 | ML | 08/07/2012 | 09:33:51 | -29.131 | 116.603 | 10 | E of Morawa, WA. |
| 2.9 | ML | 08/07/2012 | 10:27:33 | -38.339 | 146.176 | 14 | Near Moe, Vic. |
| 1.6 | ML | 09/07/2012 | 01:23:45 | -30.552 | 117.035 | 0 | W of Burakin, WA. |
| 1.9 | ML | 09/07/2012 | 14:34:05 | -32.161 | 139.461 | 12 | Yunta, SA. |
| 1.7 | ML | 14/07/2012 | 02:07:25 | -30.657 | 117.396 | 0 | N of Koorda, WA. |
| 2.0 | ML | 15/07/2012 | 12:27:44 | -34.220 | 138.553 | 8 | Owen, SA. |
| 2.5 | ML | 16/07/2012 | 17:11:45 | -35.735 | 137.873 | 15 | Kangaroo Island, SA. |
| 2.4 | ML | 16/07/2012 | 22:04:43 | -38.291 | 146.192 | 0 | SW of Moe, Vic. |
| 2.4 | ML | 17/07/2012 | 01:56:00 | -38.159 | 146.256 | 0 | Moe, Vic. |
| 2.4 | ML | 17/07/2012 | 21:22:10 | -30.621 | 117.429 | 0 | N of Koorda, WA. |
| 2.3 | ML | 17/07/2012 | 21:54:49 | -30.648 | 117.429 | 9 | N of Koorda, WA. |
| 2.3 | ML | 17/07/2012 | 22:13:34 | -30.630 | 117.437 | 1 | N of Koorda, WA. |
| 2.4 | ML | 17/07/2012 | 22:51:24 | -30.629 | 117.469 | 0 | N of Koorda, WA. |
| 2.0 | ML | 18/07/2012 | 02:44:15 | -30.575 | 117.487 | 0 | N of Koorda, WA. |
| 2.7 | ML | 18/07/2012 | 08:09:04 | -38.088 | 146.271 | 0 | N of Moe, Vic. |
| 3.5 | ML | 18/07/2012 | 09:20:55 | -30.429 | 117.794 | 2 | Beacon, WA. |
| 2.1 | ML | 18/07/2012 | 11:39:49 | -30.637 | 117.448 | 3 | N of Koorda, WA. |
| 2.2 | ML | 18/07/2012 | 12:37:48 | -30.426 | 117.759 | 1 | Beacon, WA. |
| 3.2 | ML | 18/07/2012 | 16:35:18 | -30.405 | 117.880 | 3 | Beacon, WA. |
| 1.9 | ML | 19/07/2012 | 12:05:32 | -30.630 | 117.450 | 11 | N of Koorda, WA. |
| 2.3 | ML | 19/07/2012 | 16:21:28 | -30.435 | 117.754 | 5 | Beacon, WA. |
| 2.9 | ML | 20/07/2012 | 04:42:01 | -38.335 | 146.191 | 13 | Near Moe, Vic. |
| 4.4 | ML | 20/07/2012 | 09:11:30 | -38.307 | 146.230 | 0 | Near Moe, Vic. |
| 2.2 | ML | 20/07/2012 | 09:25:08 | -38.230 | 146.239 | 1 | Near Moe, Vic. |
| 2.1 | ML | 20/07/2012 | 09:30:42 | -38.165 | 146.282 | 4 | Near Moe, Vic. |
| 2.8 | ML | 20/07/2012 | 10:00:16 | -38.302 | 146.205 | 0 | Near Moe, Vic. |
| 2.7 | ML | 20/07/2012 | 10:08:35 | -32.064 | 149.516 | 10 | Near Dunedoo, NSW. |
| 2.2 | ML | 20/07/2012 | 11:38:42 | -38.296 | 146.210 | 0 | Near Moe, Vic. |
| 2.9 | ML | 20/07/2012 | 17:02:28 | -27.303 | 113.708 | 8 | NW of Kalbarri, WA. |
| 2.1 | ML | 21/07/2012 | 05:25:58 | -30.406 | 117.629 | 7 | Beacon, WA. |
| 2.5 | ML | 21/07/2012 | 14:03:26 | -32.875 | 148.090 | 0 | NW of Parkes, NSW. |
| 2.4 | ML | 22/07/2012 | 01:53:09 | -37.082 | 147.141 | 10 | Dinner Plain, Vic. |
| 1.5 | ML | 23/07/2012 | 11:38:44 | -35.386 | 148.671 | 8 | S of Clive, NSW. |
| 2.2 | ML | 23/07/2012 | 12:20:59 | -35.985 | 136.282 | 10 | W of Kangaroo Island, SA. |
| 2.0 | ML | 24/07/2012 | 17:33:24 | -38.268 | 146.292 | 10 | S of Moe, Vic. |
| 2.2 | ML | 24/07/2012 | 20:47:48 | -35.805 | 137.999 | 12 | Kangaroo Island, SA. |
| 2.4 | ML | 25/07/2012 | 18:31:03 | -38.592 | 147.324 | 10 | SE of Seaspray, Vic. |
| 2.1 | ML | 25/07/2012 | 20:36:04 | -30.440 | 117.790 | 0 | Beacon, WA. |
| 1.7 | ML | 27/07/2012 | 18:56:41 | -34.368 | 148.610 | 2 | NW of Boorowa, NSW. |
| 2.2 | ML | 27/07/2012 | 21:00:09 | -35.556 | 137.380 | 2 | Kangaroo Island, SA. |
| 2.0 | ML | 28/07/2012 | 11:43:34 | -30.651 | 117.391 | 0 | NW of Koorda, WA. |
| 1.8 | ML | 29/07/2012 | 13:15:24 | -30.960 | 117.116 | 0 | S of Manmanning, WA. |
| 2.3 | ML | 29/07/2012 | 16:51:56 | -28.021 | 120.355 | 10 | SW of Leinster, WA. |
| 3.5 | ML | 30/07/2012 | 17:25:19 | -37.740 | 150.505 | 2 | SE of Mallacoota, Vic. |
| 3.2 | ML | 30/07/2012 | 17:28:26 | -37.811 | 150.583 | 17 | SE of Mallacoota, Vic. |
| 2.9 | ML | 30/07/2012 | 19:56:05 | -37.827 | 150.576 | 15 | SE of Mallacoota, Vic. |
| 2.9 | ML | 31/07/2012 | 00:08:04 | -38.627 | 146.636 | 12 | Near Yarram, Vic. |
| 3.1 | ML | 31/07/2012 | 01:06:58 | -37.843 | 150.428 | 0 | SE of Mallacoota, Vic. |
| 1.7 | ML | 31/07/2012 | 15:51:45 | -34.807 | 149.140 | 0 | Near Dalton, NSW. |
| 2.2 | ML | 04/08/2012 | 07:38:03 | -31.269 | 118.175 | 10 | N of Merredin, WA. |
| 2.3 | ML | 04/08/2012 | 08:05:25 | -30.589 | 117.487 | 0 | N of Koorda, WA. |
| 2.3 | ML | 04/08/2012 | 08:38:38 | -31.100 | 118.351 | 1 | SE of Mukinbudin, WA. |
| 2.3 | ML | 04/08/2012 | 13:22:56 | -33.379 | 117.761 | 3 | Dumbleyung, WA. |
| 1.9 | ML | 04/08/2012 | 20:34:57 | -34.171 | 148.689 | 10 | N of Boorowa, NSW. |
| 2.1 | ML | 05/08/2012 | 07:11:13 | -30.491 | 117.068 | 0 | NW of Burakin, WA. |
| 2.0 | ML | 06/08/2012 | 19:08:50 | -31.236 | 117.445 | 0 | Wyalkatchem, WA. |
| 2.1 | ML | 07/08/2012 | 17:42:29 | -35.424 | 137.518 | 10 | N of Kingscote, SA. |
| 1.8 | ML | 08/08/2012 | 13:48:15 | -31.507 | 116.811 | 0 | NE of Northam, WA. |
| 2.1 | ML | 08/08/2012 | 21:46:43 | -30.427 | 117.680 | 0 | Near Beacon, WA. |
| 2.1 | ML | 08/08/2012 | 22:23:05 | -30.470 | 117.716 | 0 | Near Beacon, WA. |
| 2.3 | ML | 09/08/2012 | 01:17:27 | -30.481 | 117.718 | 7 | Beacon, WA. |
| 2.3 | ML | 09/08/2012 | 01:36:56 | -30.450 | 117.731 | 7 | Near Beacon, WA. |
| 2.8 | ML | 09/08/2012 | 04:20:52 | -30.436 | 117.770 | 4 | Near Beacon, WA. |
| 2.8 | ML | 09/08/2012 | 04:33:58 | -30.411 | 117.836 | 6 | Near Beacon, WA. |
| 1.9 | ML | 09/08/2012 | 04:41:50 | -30.411 | 117.727 | 0 | Near Beacon, WA. |
| 2.6 | ML | 09/08/2012 | 07:38:29 | -30.447 | 117.786 | 2 | Near Beacon, WA. |
| 2.1 | ML | 09/08/2012 | 07:55:36 | -30.505 | 117.685 | 10 | Near Beacon, WA. |
| 2.3 | ML | 09/08/2012 | 08:44:48 | -30.460 | 117.792 | 11 | Beacon, WA. |
| 1.6 | ML | 09/08/2012 | 10:36:23 | -30.146 | 117.190 | 7 | Near Kalannie, WA. |
| 1.9 | ML | 09/08/2012 | 11:25:30 | -30.428 | 117.775 | 13 | Beacon, WA. |
| 1.3 | ML | 09/08/2012 | 13:28:19 | -35.619 | 149.174 | 12 | Near Michelago, NSW. |
| 2.3 | ML | 09/08/2012 | 20:30:24 | -30.484 | 117.747 | 9 | Beacon, WA. |
| 2.2 | ML | 09/08/2012 | 20:45:22 | -30.471 | 117.720 | 9 | Beacon, WA. |
| 2.1 | ML | 09/08/2012 | 20:47:16 | -30.480 | 117.715 | 4 | Beacon, WA. |
| 2.3 | ML | 09/08/2012 | 20:59:11 | -30.454 | 117.724 | 9 | Beacon, WA. |
| 2.2 | ML | 11/08/2012 | 17:21:08 | -30.436 | 117.742 | 2 | Beacon, WA. |
| 2.2 | ML | 11/08/2012 | 17:27:14 | -30.423 | 117.794 | 9 | Beacon, WA. |
| 2.0 | ML | 11/08/2012 | 17:30:54 | -30.448 | 117.740 | 10 | Beacon, WA. |
| 2.4 | ML | 11/08/2012 | 17:37:47 | -30.530 | 118.568 | 10 | E of Bonnie Rock, WA. |
| 2.7 | ML | 12/08/2012 | 14:01:08 | -32.998 | 148.745 | 8 | S of Cumnock, NSW. |
| 2.5 | ML | 13/08/2012 | 02:55:28 | -30.410 | 117.711 | 0 | Near Beacon, WA. |
| 2.3 | ML | 13/08/2012 | 05:36:12 | -30.671 | 117.424 | 10 | Near Beacon, WA. |
| 2.5 | ML | 13/08/2012 | 07:54:42 | -33.488 | 148.316 | 1 | Eugowra, NSW. |
| 2.4 | ML | 13/08/2012 | 09:17:02 | -30.470 | 117.782 | 10 | Beacon, WA. |
| 2.2 | ML | 13/08/2012 | 12:09:13 | -30.591 | 117.466 | 0 | N of Koorda, WA. |
| 2.8 | ML | 14/08/2012 | 00:51:56 | -30.452 | 117.789 | 10 | Beacon, WA. |
| 2.3 | ML | 14/08/2012 | 14:12:23 | -30.414 | 117.720 | 5 | Near Beacon, WA. |
| 2.4 | ML | 14/08/2012 | 16:16:04 | -30.489 | 118.530 | 10 | Near Bonnie Rock, WA. |
| 2.5 | ML | 14/08/2012 | 16:59:34 | -31.877 | 116.467 | 0 | W of York, WA. |
| 2.1 | ML | 14/08/2012 | 17:51:07 | -31.841 | 116.479 | 0 | W of York, WA. |
| 2.1 | ML | 15/08/2012 | 10:07:29 | -30.426 | 117.785 | 0 | Beacon, WA. |
| 1.9 | ML | 15/08/2012 | 19:19:15 | -30.385 | 117.727 | 0 | Near Beacon, WA. |
| 2.6 | ML | 17/08/2012 | 01:07:30 | -32.260 | 138.496 | 10 | S of Hawker, SA. |
| 2.1 | ML | 17/08/2012 | 04:22:14 | -30.458 | 117.756 | 3 | Beacon, WA. |
| 2.1 | ML | 17/08/2012 | 07:32:41 | -30.443 | 117.741 | 3 | Near Beacon, WA. |
| 2.3 | ML | 17/08/2012 | 09:20:42 | -30.463 | 117.770 | 5 | Beacon, WA. |
| 2.0 | ML | 17/08/2012 | 11:39:30 | -34.454 | 148.751 | 5 | Boorowa, NSW. |
| 2.0 | ML | 19/08/2012 | 04:29:42 | -30.457 | 117.693 | 15 | Near Beacon, WA. |
| 2.0 | ML | 19/08/2012 | 12:45:53 | -36.734 | 148.533 | 0 | S of Jindabyne, NSW. |
| 3.4 | ML | 19/08/2012 | 16:27:19 | -27.598 | 135.324 | 10 | Near Oodnadatta, SA. |
| 2.0 | ML | 21/08/2012 | 02:22:00 | -34.454 | 150.344 | 10 | Bowral, NSW. |
| 2.7 | ML | 21/08/2012 | 06:25:21 | -34.333 | 149.228 | 0 | Near Crookwell, NSW. |
| 1.9 | ML | 23/08/2012 | 13:50:49 | -34.340 | 148.905 | 0 | Near Boorowa, NSW. |
| 1.9 | ML | 24/08/2012 | 02:03:08 | -33.546 | 138.121 | 10 | Mundoora, SA. |
| 2.9 | ML | 25/08/2012 | 06:27:33 | -29.445 | 141.739 | 0 | Near Tibooburra, NSW. |
| 2.0 | ML | 26/08/2012 | 22:24:54 | -30.651 | 117.422 | 8 | N of Koorda, WA. |
| 2.0 | ML | 26/08/2012 | 22:26:47 | -30.673 | 117.410 | 5 | NW of Koorda, WA. |
| 2.0 | ML | 26/08/2012 | 23:15:16 | -30.647 | 117.423 | 7 | N of Koorda, WA. |
| 2.5 | ML | 27/08/2012 | 04:56:40 | -30.731 | 117.325 | 10 | NW of Koorda, WA. |
| 2.4 | ML | 27/08/2012 | 23:46:02 | -30.416 | 117.824 | 5 | Beacon, WA. |
| 1.9 | ML | 28/08/2012 | 01:18:48 | -30.677 | 117.412 | 0 | N of Koorda, WA. |
| 1.9 | ML | 28/08/2012 | 01:49:09 | -30.644 | 117.455 | 0 | N of Koorda, WA. |
| 2.7 | ML | 28/08/2012 | 04:32:50 | -30.427 | 117.736 | 4 | Near Beacon, WA. |
| 2.0 | ML | 28/08/2012 | 08:01:37 | -30.660 | 117.469 | 0 | Koorda, WA. |
| 2.8 | ML | 28/08/2012 | 19:42:42 | -30.438 | 117.773 | 5 | Beacon, WA. |
| 2.4 | ML | 28/08/2012 | 19:45:06 | -30.493 | 117.710 | 10 | Near Beacon, WA. |
| 2.4 | ML | 28/08/2012 | 19:48:57 | -30.499 | 117.613 | 0 | SW of Beacon, WA. |
| 2.4 | ML | 28/08/2012 | 21:59:16 | -30.414 | 117.762 | 3 | Beacon, WA. |
| 2.3 | ML | 31/08/2012 | 12:21:28 | -32.568 | 138.476 | 7 | Near Orroroo, SA. |
| 1.7 | ML | 01/09/2012 | 14:14:31 | -30.439 | 116.869 | 0 | Near Ballidu, WA. |
| 1.9 | ML | 01/09/2012 | 14:29:53 | -34.042 | 135.758 | 10 | Near Cummins, SA. |
| 2.0 | ML | 02/09/2012 | 08:06:15 | -31.963 | 117.236 | 6 | NW of Quairading, WA. |
| 2.1 | ML | 02/09/2012 | 14:44:35 | -31.947 | 117.232 | 0 | NW of Quairading, WA. |
| 2.8 | ML | 03/09/2012 | 10:06:03 | -28.654 | 115.990 | 9 | SE of Mullewa, WA. |
| 2.5 | ML | 03/09/2012 | 19:13:46 | -37.499 | 151.240 | 0 | Tasman sea. |
| 1.9 | ML | 03/09/2012 | 20:49:25 | -30.434 | 116.880 | 0 | NE of Ballidu, WA. |
| 2.5 | ML | 04/09/2012 | 09:21:58 | -30.495 | 117.705 | 10 | Near Beacon, WA. |
| 1.9 | ML | 05/09/2012 | 04:24:32 | -31.785 | 116.182 | 0 | Near Mundaring, WA. |
| 2.3 | ML | 05/09/2012 | 20:36:22 | -30.480 | 117.685 | 14 | Near Beacon, WA. |
| 2.6 | ML | 06/09/2012 | 12:56:07 | -33.320 | 118.561 | 0 | NE of Pingrup, WA. |
| 2.2 | ML | 07/09/2012 | 06:57:39 | -30.635 | 117.474 | 12 | N of Koorda, WA. |
| 1.8 | ML | 08/09/2012 | 04:53:41 | -31.662 | 117.041 | 5 | Meckering, WA. |
| 2.4 | ML | 08/09/2012 | 08:36:38 | -30.387 | 117.797 | 0 | Beacon, WA. |
| 1.8 | ML | 09/09/2012 | 03:12:40 | -34.363 | 148.604 | 12 | Boorowa, NSW. |
| 3.1 | ML | 09/09/2012 | 08:41:56 | -38.607 | 140.855 | 9 | S of Port Macdonnell, SA. |
| 3.0 | ML | 09/09/2012 | 09:23:52 | -38.639 | 140.805 | 10 | S of Port Macdonnell, SA. |
| 2.8 | ML | 10/09/2012 | 03:16:46 | -24.438 | 113.395 | 0 | Lake Macleod, WA. |
| 3.2 | ML | 10/09/2012 | 06:23:52 | -33.449 | 151.844 | 2 | Offshore E of Terrigal, NSW. |
| 1.9 | ML | 10/09/2012 | 08:32:16 | -30.695 | 117.310 | 5 | NW of Koorda, WA. |
| 3.0 | ML | 11/09/2012 | 07:13:29 | -26.114 | 132.019 | 0 | NW of Ernabella SA. |
| 1.4 | ML | 11/09/2012 | 08:04:30 | -30.738 | 117.352 | 0 | Near Koorda, WA. |
| 1.8 | ML | 12/09/2012 | 06:08:08 | -30.547 | 117.569 | 9 | N of Koorda, WA. |
| 1.8 | ML | 12/09/2012 | 17:14:57 | -30.594 | 117.509 | 8 | N of Koorda, WA. |
| 2.3 | ML | 12/09/2012 | 20:34:13 | -38.509 | 145.770 | 0 | Korumburra, Vic. |
| 2.1 | ML | 13/09/2012 | 21:12:51 | -34.366 | 148.858 | 0 | NE of Boorowa, NSW. |
| 2.3 | ML | 14/09/2012 | 13:23:05 | -30.446 | 116.998 | 10 | SW of Kalannie, WA. |
| 3.5 | ML | 14/09/2012 | 17:10:04 | -20.951 | 129.475 | 5 | NE of Lake Mackay, NT. |
| 1.6 | ML | 14/09/2012 | 18:38:20 | -31.574 | 117.804 | 5 | Near Kellerberrin, WA. |
| 2.3 | ML | 14/09/2012 | 21:13:10 | -25.949 | 116.055 | 0 | NW of Meekatharra, WA. |
| 2.2 | ML | 15/09/2012 | 13:05:37 | -30.649 | 117.407 | 0 | N of Koorda, WA. |
| 1.8 | ML | 15/09/2012 | 17:30:08 | -30.695 | 117.364 | 5 | NW of Koorda, WA. |
| 1.9 | ML | 15/09/2012 | 18:26:42 | -34.143 | 148.980 | 10 | NE of Boorowa, NSW. |
| 3.0 | ML | 16/09/2012 | 15:57:17 | -39.566 | 150.002 | 4 | E of Flinders Island, Australia. |
| 2.0 | ML | 16/09/2012 | 18:02:05 | -30.646 | 117.479 | 2 | N of Koorda, WA. |
| 2.1 | ML | 16/09/2012 | 20:47:16 | -30.720 | 117.541 | 6 | NE of Koorda, WA. |
| 1.9 | ML | 17/09/2012 | 06:23:47 | -35.188 | 150.983 | 8 | Off shore Jervis Bay, NSW. |
| 2.8 | ML | 17/09/2012 | 13:06:42 | -34.171 | 150.796 | 6 | Appin, NSW. |
| 2.1 | ML | 19/09/2012 | 08:56:42 | -30.636 | 117.382 | 0 | N of Koorda, WA. |
| 2.3 | ML | 20/09/2012 | 12:41:54 | -33.496 | 136.489 | 0 | North of Cleve, SA. |
| 1.9 | ML | 20/09/2012 | 17:39:28 | -31.277 | 117.612 | 7 | SE of Wyalkatchem, WA. |
| 1.8 | ML | 20/09/2012 | 20:41:47 | -34.475 | 148.594 | 0 | Near Boorowa, NSW. |
| 2.5 | ML | 21/09/2012 | 08:53:09 | -30.425 | 117.705 | 0 | Near Beacon, WA. |
| 2.1 | ML | 21/09/2012 | 19:27:20 | -42.035 | 145.896 | 10 | NE of Queenstown, Tas. |
| 1.9 | ML | 22/09/2012 | 17:50:39 | -35.076 | 150.434 | 13 | Wandandian, NSW. |
| 2.4 | ML | 22/09/2012 | 19:32:58 | -30.597 | 117.434 | 8 | N of Koorda, WA. |
| 2.3 | ML | 22/09/2012 | 21:21:23 | -30.632 | 117.467 | 12 | N of Koorda, WA. |
| 2.9 | ML | 23/09/2012 | 16:29:13 | -26.330 | 151.561 | 0 | Ballogie, Qld. |
| 2.2 | ML | 24/09/2012 | 03:04:35 | -35.702 | 138.004 | 15 | NE of Penneshaw, SA. |
| 2.5 | ML | 24/09/2012 | 15:37:14 | -30.812 | 138.826 | 10 | NE of Blinman, SA. |
| 2.2 | ML | 25/09/2012 | 21:32:30 | -31.447 | 117.741 | 2 | North Kellerberrin, WA. |
| 1.9 | ML | 26/09/2012 | 00:29:03 | -30.532 | 117.065 | 0 | Burakin, WA. |
| 3.5 | ML | 26/09/2012 | 05:22:35 | -26.054 | 132.163 | 13 | N of Ernabella, SA. |
| 2.3 | ML | 26/09/2012 | 14:03:31 | -31.439 | 117.765 | 2 | N of Kellerberrin, WA. |
| 3.0 | ML | 26/09/2012 | 17:37:29 | -30.996 | 121.072 | 10 | Bullabulling, WA. |
| 2.9 | ML | 27/09/2012 | 15:56:53 | -19.700 | 133.988 | 0 | Tennant Creek, NT. |
| 2.5 | ML | 01/10/2012 | 13:05:20 | -43.429 | 147.544 | 16 | Off Bruny Island, Tas. |
| 2.4 | ML | 01/10/2012 | 17:08:56 | -38.115 | 149.384 | 17 | Off Point Hicks, Vic. |
| 1.9 | ML | 01/10/2012 | 21:32:47 | -30.411 | 117.761 | 0 | Near Beacon, WA. |
| 1.9 | ML | 01/10/2012 | 22:07:17 | -30.409 | 117.762 | 0 | Near Beacon, WA. |
| 2.7 | ML | 02/10/2012 | 11:38:31 | -36.855 | 144.111 | 0 | SW of Bendigo, Vic. |
| 2.2 | ML | 05/10/2012 | 05:19:28 | -36.594 | 147.975 | 10 | Near Mt Murphy, Vic. |
| 2.5 | ML | 05/10/2012 | 14:46:33 | -30.734 | 139.223 | 10 | NE of Blinman, SA. |
| 3.2 | ML | 07/10/2012 | 03:20:28 | -27.768 | 120.630 | 0 | Near Leinster, WA. |
| 2.2 | ML | 07/10/2012 | 14:09:08 | -30.456 | 117.692 | 3 | W of Beacon, WA. |
| 1.8 | ML | 08/10/2012 | 04:32:29 | -32.577 | 138.561 | 10 | S of Carrieton, SA. |
| 1.7 | ML | 10/10/2012 | 14:15:21 | -33.824 | 139.130 | 12 | Near Burra SA. |
| 3.1 | ML | 11/10/2012 | 22:47:09 | -37.893 | 143.591 | 10 | Rokewood, Vic. |
| 3.5 | ML | 12/10/2012 | 19:27:14 | -22.881 | 127.681 | 10 | W of Lake Mackay, WA. |
| 2.4 | ML | 13/10/2012 | 13:57:41 | -19.769 | 133.897 | 0 | Tennant Creek, NT. |
| 1.5 | ML | 14/10/2012 | 07:53:12 | -36.208 | 148.901 | 0 | Wambrook, NSW. |
| 3.5 | ML | 17/10/2012 | 18:16:57 | -30.648 | 141.877 | 15 | N of Broken Hill, NSW. |
| 1.8 | ML | 17/10/2012 | 22:24:12 | -31.291 | 117.671 | 4 | SW of Trayning, WA. |
| 1.9 | ML | 18/10/2012 | 17:41:40 | -36.548 | 148.653 | 2 | S of Jindabyne, NSW. |
| 1.7 | ML | 26/10/2012 | 12:37:37 | -31.723 | 117.049 | 0 | Meckering, WA. |
| 3.0 | ML | 27/10/2012 | 04:43:14 | -38.310 | 146.251 | 12 | Near Moe, Vic. |
| 1.7 | ML | 29/10/2012 | 04:31:35 | -38.410 | 145.984 | 0 | Near Korumburra, Vic. |
| 2.3 | ML | 30/10/2012 | 16:43:22 | -37.752 | 146.276 | 15 | Baw Baw, Vic. |
| 2.1 | ML | 30/10/2012 | 20:46:23 | -30.657 | 117.383 | 1 | NW of Koorda, WA. |
| 2.1 | ML | 30/10/2012 | 21:47:38 | -30.592 | 117.454 | 2 | N of Koorda, WA. |
| 2.4 | ML | 31/10/2012 | 08:35:55 | -38.193 | 145.749 | 15 | SW of Drouin, Vic. |
| 2.7 | ML | 31/10/2012 | 21:54:00 | -33.362 | 120.807 | 0 | W of Cascade, WA. |
| 4.5 | MB | 05/11/2012 | 19:31:49 | -11.546 | 129.045 | 39 | Offshore Bathurst Island, NT. |
| 3.6 | ML | 08/11/2012 | 03:56:55 | -29.898 | 126.992 | 0 | Nullarbor Plain, WA. |
| 1.8 | ML | 08/11/2012 | 18:33:42 | -36.316 | 149.359 | 10 | SE of Cooma, NSW. |
| 2.2 | ML | 10/11/2012 | 17:11:12 | -30.367 | 117.721 | 8 | NW of Beacon, WA. |
| 1.7 | ML | 11/11/2012 | 16:11:46 | -34.269 | 149.099 | 0 | E of Frogmore, NSW. |
| 3.0 | ML | 13/11/2012 | 07:47:42 | -28.375 | 139.910 | 10 | SW of Innamincka, SA. |
| 1.5 | ML | 13/11/2012 | 09:09:19 | -33.270 | 138.267 | 13 | Near Laura, SA. |
| 2.8 | ML | 14/11/2012 | 20:23:03 | -30.322 | 117.802 | 2 | Near Beacon, WA. |
| 1.6 | ML | 15/11/2012 | 09:04:42 | -32.823 | 116.887 | 0 | NW of Narrogin, WA. |
| 2.2 | ML | 16/11/2012 | 03:16:18 | -33.391 | 138.455 | 7 | Near Georgetown, SA. |
| 2.1 | ML | 16/11/2012 | 10:38:03 | -30.433 | 117.744 | 0 | Beacon, WA. |
| 2.1 | ML | 16/11/2012 | 11:10:25 | -30.665 | 117.438 | 6 | N of Koorda, WA. |
| 1.8 | ML | 16/11/2012 | 11:48:45 | -30.652 | 117.401 | 0 | N of Koorda, WA. |
| 1.7 | ML | 16/11/2012 | 19:48:16 | -33.538 | 138.651 | 6 | Near Spalding, SA. |
| 2.1 | ML | 18/11/2012 | 02:09:17 | -30.686 | 117.418 | 5 | NW of Koorda, WA. |
| 2.0 | ML | 19/11/2012 | 23:46:34 | -33.631 | 136.738 | 15 | NW of Cowell, SA. |
| 3.0 | ML | 20/11/2012 | 19:12:51 | -27.817 | 140.661 | 18 | Near Innamincka, SA. |
| 3.3 | ML | 21/11/2012 | 06:21:45 | -22.974 | 128.603 | 10 | Near Lake Mackay, WA. |
| 2.8 | ML | 21/11/2012 | 12:17:20 | -27.763 | 140.848 | 10 | Innamincka, SA. |
| 2.9 | ML | 22/11/2012 | 02:48:38 | -27.782 | 140.924 | 10 | Near Innamincka, SA. |
| 2.7 | ML | 23/11/2012 | 17:03:58 | -28.599 | 137.802 | 10 | Lake Eyre, SA. |
| 2.7 | ML | 23/11/2012 | 17:11:24 | -27.813 | 140.791 | 10 | Near Innamincka, SA. |
| 2.3 | ML | 24/11/2012 | 22:54:40 | -40.458 | 145.197 | 13 | N of Smithton, Tas. |
| 2.6 | ML | 25/11/2012 | 06:53:24 | -27.787 | 140.721 | 5 | Near Innamincka, SA. |
| 2.7 | ML | 25/11/2012 | 15:06:56 | -27.969 | 140.822 | 10 | Near Innamincka, SA. |
| 2.6 | ML | 27/11/2012 | 04:01:26 | -27.587 | 140.482 | 0 | Near Innamincka, SA. |
| 2.5 | ML | 27/11/2012 | 04:26:09 | -27.820 | 140.619 | 0 | Near Innamincka, SA. |
| 3.6 | ML | 27/11/2012 | 07:13:07 | -27.793 | 140.751 | 0 | Near Innamincka, SA. |
| 3.2 | ML | 27/11/2012 | 08:02:59 | -27.837 | 140.711 | 5 | Near Innamincka, SA. |
| 2.9 | ML | 28/11/2012 | 16:37:17 | -27.895 | 140.460 | 10 | Near Innamincka, SA. |
| 2.8 | ML | 28/11/2012 | 16:46:38 | -27.867 | 140.663 | 10 | Near Innamincka, SA. |
| 2.8 | ML | 28/11/2012 | 17:09:29 | -27.830 | 140.704 | 10 | Near Innamincka, SA. |
| 4.0 | ML | 28/11/2012 | 22:35:22 | -27.757 | 125.943 | 0 | SW of Warburton, WA. |
| 2.7 | ML | 29/11/2012 | 03:57:10 | -27.830 | 140.614 | 0 | Near Innamincka, SA. |
| 2.8 | ML | 29/11/2012 | 07:32:34 | -27.933 | 140.671 | 0 | Near Innamincka, SA. |
| 2.9 | ML | 29/11/2012 | 08:55:37 | -27.885 | 140.738 | 10 | Near Innamincka, SA. |
| 3.3 | ML | 29/11/2012 | 14:53:40 | -27.832 | 140.758 | 14 | Near Innamincka, SA. |
| 2.9 | ML | 29/11/2012 | 15:21:07 | -27.804 | 140.789 | 10 | Near Innamincka, SA. |
| 2.8 | ML | 29/11/2012 | 17:03:01 | -27.871 | 140.741 | 10 | Near Innamincka, SA. |
| 3.0 | ML | 29/11/2012 | 17:48:46 | -27.838 | 140.764 | 10 | Near Innamincka, SA. |
| 3.2 | ML | 29/11/2012 | 18:19:24 | -27.818 | 140.676 | 12 | Near Innamincka, SA. |
| 3.9 | ML | 29/11/2012 | 19:21:18 | -27.756 | 140.659 | 5 | Near Innamincka, SA. |
| 3.2 | ML | 29/11/2012 | 19:29:25 | -27.740 | 140.650 | 10 | Near Innamincka, SA. |
| 3.2 | ML | 29/11/2012 | 19:45:35 | -27.757 | 140.687 | 10 | Near Innamincka, SA. |
| 3.0 | ML | 29/11/2012 | 20:05:08 | -27.730 | 140.674 | 10 | Near Innamincka, SA. |
| 3.5 | ML | 29/11/2012 | 20:31:30 | -27.811 | 140.587 | 8 | Near Innamincka, SA. |
| 3.3 | ML | 29/11/2012 | 22:08:19 | -27.785 | 140.625 | 12 | Near Innamincka, SA. |
| 3.3 | ML | 29/11/2012 | 22:26:52 | -27.763 | 140.592 | 10 | Near Innamincka, SA. |
| 3.1 | ML | 30/11/2012 | 07:52:42 | -27.704 | 140.531 | 0 | Near Innamincka, SA. |
| 3.1 | ML | 30/11/2012 | 16:24:45 | -32.374 | 122.560 | 14 | SE of Norseman, WA. |
| 1.6 | ML | 30/11/2012 | 17:43:00 | -32.807 | 138.265 | 10 | Near Melrose, SA. |
| 1.9 | ML | 30/11/2012 | 22:41:30 | -31.719 | 117.075 | 1 | Near Meckering, WA. |
| 3.4 | ML | 01/12/2012 | 17:19:07 | -16.195 | 127.661 | 10 | SW of Kununurra, WA. |
| 3.0 | ML | 01/12/2012 | 18:48:03 | -27.844 | 140.684 | 10 | Near Innamincka, SA. |
| 2.2 | ML | 02/12/2012 | 03:29:13 | -30.646 | 118.065 | 5 | SE of Beacon, WA. |
| 2.9 | ML | 04/12/2012 | 17:51:04 | -29.936 | 139.124 | 10 | NW of Yudnamutana, SA. |
| 3.5 | ML | 04/12/2012 | 20:17:43 | -25.785 | 151.473 | 10 | Near Gayndah, QLD. |
| 1.5 | ML | 05/12/2012 | 06:05:05 | -30.363 | 117.107 | 0 | Kalannie, WA. |
| 1.8 | ML | 06/12/2012 | 10:06:59 | -30.664 | 117.439 | 7 | N of Koorda, WA. |
| 1.4 | ML | 06/12/2012 | 15:58:29 | -30.505 | 117.687 | 0 | SW of Beacon, WA. |
| 1.5 | ML | 06/12/2012 | 22:00:06 | -30.661 | 117.420 | 8 | N of Koorda, WA. |
| 1.5 | ML | 07/12/2012 | 23:24:51 | -33.426 | 137.933 | 0 | W of Merriton, SA. |
| 2.2 | ML | 08/12/2012 | 09:33:14 | -33.188 | 148.724 | 0 | Manildra, NSW. |
| 2.6 | ML | 09/12/2012 | 07:41:04 | -31.232 | 117.608 | 0 | SE of Wyalkatchem, WA. |
| 2.1 | ML | 10/12/2012 | 16:17:01 | -30.394 | 117.699 | 0 | Near Beacon, WA. |
| 1.9 | ML | 10/12/2012 | 16:41:17 | -30.491 | 117.677 | 0 | Near Beacon, WA. |
| 1.9 | ML | 11/12/2012 | 10:57:06 | -32.144 | 117.168 | 0 | SW of Quairading, WA. |
| 2.1 | ML | 12/12/2012 | 01:12:32 | -30.479 | 117.671 | 5 | Near Beacon, WA. |
| 2.3 | ML | 12/12/2012 | 04:03:57 | -30.445 | 117.713 | 5 | Near Beacon, WA. |
| 2.6 | ML | 12/12/2012 | 08:30:48 | -32.192 | 119.177 | 0 | Hyden, WA. |
| 2.1 | ML | 12/12/2012 | 18:06:59 | -30.474 | 117.718 | 0 | Near Beacon, WA. |
| 2.5 | ML | 12/12/2012 | 20:48:35 | -32.261 | 118.824 | 10 | N of Hyden, WA. |
| 2.3 | ML | 12/12/2012 | 21:49:39 | -32.252 | 118.814 | 5 | N of Hyden, WA. |
| 2.4 | ML | 13/12/2012 | 06:35:56 | -34.276 | 147.314 | 0 | Near Temora, NSW. |
| 3.6 | ML | 14/12/2012 | 04:14:55 | -19.826 | 133.818 | 11 | SW of Tennant Creek, NT. |
| 2.7 | ML | 15/12/2012 | 17:11:53 | -37.525 | 151.245 | 0 | SE of Pambula, NSW. |
| 1.5 | ML | 15/12/2012 | 18:06:19 | -30.486 | 116.863 | 3 | NE of Ballidu, WA. |
| 2.2 | ML | 15/12/2012 | 22:40:08 | -30.636 | 117.423 | 2 | N of Koorda, WA. |
| 2.0 | ML | 17/12/2012 | 23:59:59 | -30.402 | 117.794 | 2 | Beacon, WA. |
| 1.9 | ML | 19/12/2012 | 15:16:12 | -34.427 | 150.541 | 10 | Near Mittagong, NSW. |
| 3.0 | ML | 19/12/2012 | 23:50:01 | -24.921 | 112.497 | 10 | W of Carnarvon, WA. |
| 1.6 | ML | 20/12/2012 | 14:27:39 | -31.660 | 117.039 | 6 | Meckering, WA. |
| 2.4 | ML | 20/12/2012 | 23:30:41 | -32.069 | 138.727 | 5 | SE of Hawker, SA. |
| 2.9 | ML | 21/12/2012 | 15:26:20 | -32.395 | 152.041 | 9 | Near Bulahdelah, NSW. |
| 2.4 | ML | 22/12/2012 | 06:08:49 | -30.417 | 117.752 | 0 | Near Beacon, WA. |
| 2.6 | ML | 22/12/2012 | 07:41:20 | -38.462 | 145.824 | 10 | Near Korumburra, Vic. |
| 2.8 | ML | 22/12/2012 | 09:29:16 | -32.529 | 122.512 | 10 | SE of Norseman, WA. |
| 2.2 | ML | 22/12/2012 | 15:36:32 | -30.455 | 117.686 | 8 | W of Beacon, WA. |
| 1.6 | ML | 23/12/2012 | 02:16:24 | -31.789 | 117.374 | 0 | SE of Cunderdin, WA. |
| 2.0 | ML | 25/12/2012 | 01:13:56 | -30.675 | 117.381 | 1 | NW of Koorda, WA. |
| 2.1 | ML | 25/12/2012 | 02:45:58 | -32.413 | 138.683 | 4 | Near Carrieton, SA. |
| 2.7 | ML | 26/12/2012 | 20:46:15 | -35.114 | 115.809 | 2 | Off coast of Windy Harbour, WA. |
| 3.7 | ML | 28/12/2012 | 12:04:44 | -24.344 | 112.396 | 3 | Offshore Western Australia. |
| 2.1 | ML | 28/12/2012 | 13:59:11 | -32.272 | 118.644 | 10 | Near Hyden, WA. |
| 1.9 | ML | 28/12/2012 | 17:52:44 | -19.842 | 133.982 | 0 | Tennant Creek, NT. |
| 2.0 | ML | 29/12/2012 | 08:26:46 | -19.846 | 134.014 | 0 | SW of Tennant Creek, NT. |
| 3.0 | ML | 29/12/2012 | 17:43:56 | -24.529 | 113.024 | 10 | NW of Carnarvon, WA. |

New South Wales Earthquakes

Table A.2 New South Wales Earthquakes 2012

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3.1 | ML | 03/01/2012 | 05:21:35 | -29.945 | 142.648 | 3 | SE of Tibooburra, NSW. |
| 1.7 | ML | 03/01/2012 | 12:05:53 | -36.273 | 149.792 | 11 | NW of Cobargo, NSW. |
| 1.8 | ML | 05/01/2012 | 11:46:56 | -34.291 | 148.935 | 0 | Near Frogmore, NSW. |
| 1.6 | ML | 06/01/2012 | 09:55:02 | -34.323 | 148.888 | 0 | Frogmore, NSW, |
| 1.7 | ML | 06/01/2012 | 11:58:51 | -34.284 | 148.936 | 0 | Frogmore, NSW. |
| 1.8 | ML | 07/01/2012 | 13:17:45 | -34.302 | 149.229 | 10 | NW of Crookwell, NSW. |
| 2.8 | ML | 15/01/2012 | 21:33:45 | -31.502 | 144.599 | 11 | W of Cobar, NSW. |
| 1.9 | ML | 18/01/2012 | 12:16:38 | -36.261 | 148.874 | 0 | Near Berridale, NSW. |
| 2.1 | ML | 21/01/2012 | 17:51:49 | -33.777 | 148.714 | 16 | Cowra, NSW. |
| 2.0 | ML | 21/01/2012 | 21:19:06 | -35.029 | 149.024 | 3 | Murrumbateman, NSW. |
| 1.5 | ML | 28/01/2012 | 14:13:30 | -34.516 | 148.846 | 0 | Near Boorowa, NSW. |
| 1.8 | ML | 03/02/2012 | 15:41:40 | -34.809 | 149.203 | 1 | Gunning, NSW. |
| 2.1 | ML | 04/02/2012 | 15:30:52 | -34.650 | 149.102 | 0 | Dalton, NSW. |
| 2.6 | ML | 06/02/2012 | 10:57:41 | -32.941 | 151.468 | 6 | Brunkerville, NSW. |
| 1.6 | ML | 15/02/2012 | 01:31:40 | -36.175 | 148.787 | 11 | Lake Eucumbene, NSW. |
| 1.8 | ML | 15/02/2012 | 23:01:41 | -34.703 | 151.037 | 10 | E of Kiama, NSW. |
| 1.9 | ML | 21/02/2012 | 09:21:03 | -34.654 | 150.794 | 0 | Jamberoo, NSW. |
| 1.8 | ML | 25/02/2012 | 06:46:45 | -33.963 | 148.423 | 4 | SW of Cowra, NSW. |
| 2.3 | ML | 26/02/2012 | 16:51:02 | -32.935 | 151.469 | 5 | Brunkerville, NSW. |
| 1.9 | ML | 29/02/2012 | 12:44:36 | -34.295 | 149.075 | 0 | NE of Boorowa, NSW. |
| 1.7 | ML | 06/03/2012 | 04:15:27 | -35.032 | 149.031 | 0 | Murrumbateman area, NSW. |
| 1.8 | ML | 09/03/2012 | 17:42:44 | -34.58 | 148.534 | 14 | Galong, NSW. |
| 2.7 | ML | 11/03/2012 | 18:21:51 | -36.329 | 148.755 | 15 | NE of Jindabyne, NSW. |
| 1.6 | ML | 12/03/2012 | 13:30:08 | -36.313 | 148.722 | 15 | NE of Jindabyne, NSW. |
| 1.8 | ML | 14/03/2012 | 16:21:48 | -36.326 | 148.628 | 12 | Jindabyne, NSW. |
| 2.1 | ML | 15/03/2012 | 03:54:14 | -33.489 | 150.156 | 7 | Lithgow, NSW. |
| 2.1 | ML | 16/03/2012 | 16:19:15 | -36.302 | 148.858 | 0 | Barridale, NSW. |
| 2.6 | ML | 20/03/2012 | 15:28:07 | -33.872 | 151.782 | 0 | Tasman Sea, E of Sydney, NSW. |
| 1.8 | ML | 23/03/2012 | 14:52:12 | -36.031 | 148.539 | 0 | Lake Eucumbene, NSW. |
| 1.6 | ML | 29/03/2012 | 17:31:08 | -34.561 | 148.669 | 0 | Near Boorowa, NSW. |
| 2.0 | ML | 29/03/2012 | 21:16:55 | -34.63 | 148.902 | 0 | NE of Bowning, NSW. |
| 1.8 | ML | 31/03/2012 | 15:18:39 | -34.867 | 149.406 | 0 | SE of Gunning, NSW. |
| 2.3 | ML | 09/04/2012 | 14:27:57 | -34.347 | 150.953 | 12 | NE of Wollongong. NSW. |
| 1.7 | ML | 09/04/2012 | 21:36:51 | -34.824 | 149.21 | 9 | S of Dalton, NSW. |
| 1.9 | ML | 18/04/2012 | 17:00:53 | -35.662 | 148.309 | 11 | S of Talbingo, NSW. |
| 3.7 | ML | 19/04/2012 | 19:09:16 | -35.231 | 148.603 | 4 | South of Wee Jasper, NSW. |
| 1.9 | ML | 21/04/2012 | 06:37:59 | -36.025 | 148.43 | 10 | W of Lake Eucumbene, NSW. |
| 2.1 | ML | 27/04/2012 | 16:18:54 | -33.375 | 148.068 | 0 | Forbes, NSW. |
| 2.0 | ML | 01/05/2012 | 12:18:08 | -36.652 | 148.516 | 1 | SE of Thredbo, NSW. |
| 1.9 | ML | 07/05/2012 | 16:48:14 | -36.296 | 148.797 | 10 | Near Berridale, NSW. |
| 3.1 | ML | 24/05/2012 | 19:06:31 | -31.297 | 141.732 | 12 | NE of Broken Hill, NSW. |
| 1.4 | ML | 01/06/2012 | 17:47:33 | -34.619 | 148.908 | 10 | Near Bowning, NSW. |
| 3.2 | ML | 05/06/2012 | 09:12:10 | -34.337 | 148.698 | 12 | N of Boorowa, NSW. |
| 2.2 | ML | 05/06/2012 | 09:45:51 | -34.254 | 148.763 | 10 | N of Boorowa, NSW. |
| 2.3 | ML | 05/06/2012 | 17:42:03 | -34.349 | 148.746 | 0 | N of Boorowa, NSW. |
| 2.0 | ML | 06/06/2012 | 22:15:30 | -34.288 | 148.955 | 1 | NE of Boorowa, NSW. |
| 2.0 | ML | 07/06/2012 | 17:10:05 | -36.324 | 148.804 | 7 | Berridale, NSW. |
| 4.2 | ML | 08/06/2012 | 11:31:00 | -30.759 | 150.413 | 0 | NW of Tamworth, NSW. |
| 4.1 | ML | 08/06/2012 | 11:32:39 | -30.748 | 150.4 | 0 | NW of Tamworth, NSW. |
| 1.9 | ML | 13/06/2012 | 13:27:28 | -34.402 | 148.708 | 0 | Boorowa, NSW. |
| 2.3 | ML | 13/06/2012 | 14:31:08 | -34.372 | 148.628 | 20 | NW of Boorowa, NSW. |
| 2.7 | ML | 16/06/2012 | 19:46:02 | -35.074 | 149.084 | 10 | N of Canberra, NSW. |
| 2.4 | ML | 20/06/2012 | 13:40:28 | -33.611 | 142.726 | 2 | SE of Broken Hill, NSW. |
| 3.7 | ML | 22/06/2012 | 09:16:03 | -34.108 | 148.644 | 14 | N of Boorowa, NSW. |
| 2.3 | ML | 22/06/2012 | 15:55:29 | -34.125 | 148.672 | 0 | N of Boorowa, NSW. |
| 2.0 | ML | 23/06/2012 | 17:17:48 | -34.212 | 148.659 | 11 | N of Boorowa, NSW. |
| 2.2 | ML | 27/06/2012 | 23:37:53 | -34.13 | 148.706 | 12 | N of Boorowa, NSW. |
| 2.0 | ML | 30/06/2012 | 01:08:43 | -34.169 | 148.726 | 0 | N of Boorowa, NSW. |
| 2.0 | ML | 30/06/2012 | 08:32:49 | -34.125 | 148.714 | 10 | N of Boorowa, NSW. |
| 2.2 | ML | 30/06/2012 | 09:56:03 | -34.073 | 148.631 | 10 | N of Boorowa, NSW. |
| 2.6 | ML | 04/07/2012 | 09:15:04 | -34.158 | 150.352 | 12 | W of Picton, NSW. |
| 2.7 | ML | 20/07/2012 | 10:08:35 | -32.064 | 149.516 | 10 | Near Dunedoo, NSW. |
| 2.5 | ML | 21/07/2012 | 14:03:26 | -32.875 | 148.09 | 0 | NW of Parkes, NSW. |
| 1.5 | ML | 23/07/2012 | 11:38:44 | -35.386 | 148.671 | 8 | S of Clive, NSW. |
| 1.7 | ML | 27/07/2012 | 18:56:41 | -34.368 | 148.61 | 2 | NW of Boorowa, NSW. |
| 1.7 | ML | 31/07/2012 | 15:51:45 | -34.807 | 149.14 | 0 | Near Dalton, NSW. |
| 1.9 | ML | 04/08/2012 | 20:34:57 | -34.171 | 148.689 | 10 | N of Boorowa, NSW. |
| 1.3 | ML | 09/08/2012 | 13:28:19 | -35.619 | 149.174 | 12 | Near Michelago, NSW. |
| 2.7 | ML | 12/08/2012 | 14:01:08 | -32.998 | 148.745 | 8 | S of Cumnock, NSW. |
| 2.5 | ML | 13/08/2012 | 07:54:42 | -33.488 | 148.316 | 1 | Eugowra, NSW. |
| 2.0 | ML | 17/08/2012 | 11:39:30 | -34.454 | 148.751 | 5 | Boorowa, NSW. |
| 2.0 | ML | 19/08/2012 | 12:45:53 | -36.734 | 148.533 | 0 | S of Jindabyne, NSW. |
| 2.0 | ML | 21/08/2012 | 02:22:00 | -34.454 | 150.344 | 10 | Bowral, NSW. |
| 2.7 | ML | 21/08/2012 | 06:25:21 | -34.333 | 149.228 | 0 | Near Crookwell, NSW. |
| 1.9 | ML | 23/08/2012 | 13:50:49 | -34.34 | 148.905 | 0 | Near Boorowa, NSW. |
| 2.9 | ML | 25/08/2012 | 06:27:33 | -29.445 | 141.739 | 0 | Near Tibooburra, NSW. |
| 1.8 | ML | 09/09/2012 | 03:12:40 | -34.363 | 148.604 | 12 | Boorowa, NSW. |
| 3.2 | ML | 10/09/2012 | 06:23:52 | -33.449 | 151.844 | 2 | Offshore E of Terrigal, NSW. |
| 2.1 | ML | 13/09/2012 | 21:12:51 | -34.366 | 148.858 | 0 | NE of Boorowa, NSW. |
| 1.9 | ML | 15/09/2012 | 18:26:42 | -34.143 | 148.98 | 10 | NE of Boorowa, NSW. |
| 1.9 | ML | 17/09/2012 | 06:23:47 | -35.188 | 150.983 | 8 | Off shore Jervis Bay, NSW. |
| 2.8 | ML | 17/09/2012 | 13:06:42 | -34.171 | 150.796 | 6 | Appin, NSW. |
| 1.8 | ML | 20/09/2012 | 20:41:47 | -34.475 | 148.594 | 0 | Near Boorowa, NSW. |
| 1.9 | ML | 22/09/2012 | 17:50:39 | -35.076 | 150.434 | 13 | Wandandian, NSW. |
| 1.5 | ML | 14/10/2012 | 07:53:12 | -36.208 | 148.901 | 0 | Wambrook, NSW. |
| 3.5 | ML | 17/10/2012 | 18:16:57 | -30.648 | 141.877 | 15 | N of Broken Hill, NSW. |
| 1.9 | ML | 18/10/2012 | 17:41:40 | -36.548 | 148.653 | 2 | S of Jindabyne, NSW. |
| 1.8 | ML | 08/11/2012 | 18:33:42 | -36.316 | 149.359 | 10 | SE of Cooma, NSW. |
| 1.7 | ML | 11/11/2012 | 16:11:46 | -34.269 | 149.099 | 0 | E of Frogmore, NSW. |
| 2.2 | ML | 08/12/2012 | 09:33:14 | -33.188 | 148.724 | 0 | Manildra, NSW. |
| 2.4 | ML | 13/12/2012 | 06:35:56 | -34.276 | 147.314 | 0 | Near Temora, NSW. |
| 2.7 | ML | 15/12/2012 | 17:11:53 | -37.525 | 151.245 | 0 | SE of Pambula, NSW. |
| 1.9 | ML | 19/12/2012 | 15:16:12 | -34.427 | 150.541 | 10 | Near Mittagong, NSW. |
| 2.9 | ML | 21/12/2012 | 15:26:20 | -32.395 | 152.041 | 9 | Near Bulahdelah, NSW. |

South Australian Earthquakes

Table A.3 South Australian Earthquakes 2012 Recorded by Geoscience Australia

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.7 | ML | 02/01/2012 | 19:18:28 | -32.517 | 138.363 | 10 | Hammond, SA. |
| 2.0 | ML | 03/01/2012 | 10:53:38 | -33.315 | 136.978 | 0 | N of Cowell, SA. |
| 2.3 | ML | 04/01/2012 | 19:55:23 | -32.186 | 138.916 | 10 | NE of Carrieton, SA. |
| 2.8 | ML | 09/01/2012 | 18:53:03 | -30.516 | 139.247 | 10 | SW of Arkaroola, SA. |
| 3.3 | ML | 12/01/2012 | 01:22:13 | -38.566 | 140.929 | 19 | Offshore, S of Mt Gambier, SA. |
| 1.0 | ML | 18/01/2012 | 16:19:52 | -33.097 | 137.857 | 15 | SW of Port Germein, SA. |
| 2.3 | ML | 19/01/2012 | 01:26:07 | -35.264 | 135.829 | 10 | S of Port Lincoln, SA. |
| 2.5 | ML | 27/01/2012 | 19:18:46 | -34.168 | 139.535 | 20 | SW of Morgan, SA. |
| 2.1 | ML | 28/01/2012 | 20:17:33 | -32.05 | 138.711 | 10 | SE of Hawker, SA. |
| 2.3 | ML | 01/02/2012 | 13:03:52 | -33.054 | 138.601 | 15 | Mannanarie, SA. |
| 2.3 | ML | 01/02/2012 | 19:46:19 | -33.126 | 138.616 | 5 | Mannanarie, SA. |
| 2.3 | ML | 01/02/2012 | 19:57:23 | -33.073 | 138.632 | 0 | Mannanarie, SA. |
| 1.9 | ML | 03/02/2012 | 13:49:24 | -33.063 | 138.627 | 0 | Mannanarie, SA. |
| 3.3 | ML | 04/02/2012 | 08:41:05 | -35.143 | 133.319 | 0 | W of Port Lincoln, SA. |
| 2.3 | ML | 05/02/2012 | 01:52:41 | -33.292 | 139.661 | 10 | NE of Burra, SA. |
| 1.9 | ML | 14/02/2012 | 21:44:34 | -32.09 | 138.307 | 14 | SW of Hawker, SA. |
| 1.3 | ML | 20/02/2012 | 07:48:30 | -34.066 | 139.311 | 0 | NE of Eudunda, SA. |
| 1.8 | ML | 21/02/2012 | 00:28:04 | -33.177 | 138.107 | 6 | Napperby, SA. |
| 1.9 | ML | 21/02/2012 | 01:50:42 | -32.052 | 138.537 | 10 | SE of Hawker, SA. |
| 1.4 | ML | 24/02/2012 | 02:46:51 | -33.488 | 138.628 | 11 | Spalding, SA. |
| 2.3 | ML | 26/02/2012 | 10:29:26 | -37.802 | 140.02 | 10 | S of Southend, SA. |
| 2.4 | ML | 26/02/2012 | 14:25:02 | -34.019 | 135.906 | 7 | Brooker, SA. |
| 1.3 | ML | 01/03/2012 | 16:45:33 | -33.598 | 138.373 | 0 | Koolunga, SA. |
| 1.9 | ML | 02/03/2012 | 18:32:08 | -33.056 | 138.737 | 15 | NE of Jamestown, SA. |
| 2.3 | ML | 06/03/2012 | 18:28:43 | -31.068 | 138.56 | 17 | NW of Blinman, SA. |
| 2.8 | ML | 06/03/2012 | 22:34:20 | -37.594 | 139.639 | 10 | SW of Beachport, SA. |
| 2.7 | ML | 12/03/2012 | 12:06:51 | -33.145 | 138.307 | 17 | Laura, SA. |
| 2.0 | ML | 15/03/2012 | 19:20:17 | -32.551 | 138.527 | 10 | S of Carrieton, SA. |
| 4.3 | ML | 16/03/2012 | 11:24:36 | -26.174 | 131.824 | 15 | Musgrave Ranges, SA. |
| 1.3 | ML | 17/03/2012 | 18:38:17 | -33.496 | 138.894 | 15 | N of Burra, SA. |
| 3.8 | ML | 20/03/2012 | 03:25:23 | -26.144 | 132.287 | 10 | Musgrave Ranges, SA. |
| 3.9 | ML | 21/03/2012 | 17:07:09 | -28.231 | 136.001 | 0 | SE of Oodnadatta, SA. |
| 1.9 | ML | 22/03/2012 | 19:02:12 | -32.614 | 137.764 | 0 | S of Port Augusta, SA. |
| 5.7 | ML | 23/03/2012 | 09:25:19 | -26.124 | 132.124 | 0 | Near Ernabella, SA (MW 5.4). |
| 2.3 | ML | 23/03/2012 | 16:24:42 | -33.337 | 138.76 | 9 | SE of Jamestown, SA. |
| 1.3 | ML | 23/03/2012 | 18:11:29 | -33.358 | 138.738 | 0 | SE of Jamestown, SA. |
| 3.9 | ML | 25/03/2012 | 20:53:20 | -30.324 | 136.821 | 0 | Near Andamooka, SA. |
| 1.7 | ML | 25/03/2012 | 22:36:05 | -33.443 | 138.7 | 0 | NE of Spalding, SA. |
| 3.6 | ML | 30/03/2012 | 06:27:09 | -26.209 | 132.186 | 10 | Near Ernabella, SA. |
| 1.6 | ML | 30/03/2012 | 08:54:50 | -32.659 | 138.872 | 20 | NE of Orroroo, SA. |
| 2.3 | ML | 31/03/2012 | 08:02:05 | -31.189 | 138.789 | 20 | SE of Blinman, SA. |
| 1.8 | ML | 07/04/2012 | 14:28:09 | -33.473 | 138.679 | 7 | Spalding, SA. |
| 3.7 | ML | 08/04/2012 | 11:56:06 | -26.09 | 131.901 | 0 | Near Ernabella, SA. |
| 2.4 | ML | 26/04/2012 | 14:57:26 | -31.377 | 138.98 | 0 | SE of Blinman, SA. |
| 2.1 | ML | 30/04/2012 | 12:00:20 | -33.104 | 138.701 | 4 | NE of Jamestown, SA. |
| 1.7 | ML | 30/04/2012 | 12:22:25 | -33.119 | 138.695 | 0 | NE of Jamestown, SA. |
| 1.5 | ML | 30/04/2012 | 14:11:21 | -33.138 | 138.678 | 0 | NE of Jamestown, SA. |
| 1.8 | ML | 30/04/2012 | 14:28:21 | -33.115 | 138.713 | 2 | NE of Jamestown, SA. |
| 2.3 | ML | 01/05/2012 | 13:35:26 | -32.684 | 138.377 | 10 | NW of Orroroo, SA. |
| 2.4 | ML | 01/05/2012 | 21:57:24 | -33.572 | 138.549 | 12 | Spalding, SA. |
| 1.2 | ML | 04/05/2012 | 20:48:33 | -33.565 | 138.414 | 0 | Near Spalding, SA. |
| 2.7 | ML | 05/05/2012 | 05:05:16 | -29.789 | 137.779 | 10 | SW of Marree, SA. |
| 2.1 | ML | 05/05/2012 | 11:56:52 | -33.214 | 138.677 | 11 | Near Jamestown, SA. |
| 2.1 | ML | 06/05/2012 | 07:24:29 | -33.207 | 137.12 | 0 | 47km SW of Whyalla, SA. |
| 1.8 | ML | 14/05/2012 | 08:47:46 | -33.07 | 138.708 | 0 | Yongala, SA. |
| 2.2 | ML | 26/05/2012 | 16:03:01 | -32.579 | 138.916 | 10 | Near Orroroo, SA. |
| 2.4 | ML | 04/06/2012 | 04:09:24 | -33.381 | 138.376 | 10 | Georgetown, SA. |
| 2.1 | ML | 06/06/2012 | 10:26:00 | -31.951 | 138.066 | 0 | Near Hawker, SA. |
| 2.7 | ML | 11/06/2012 | 21:07:20 | -32.082 | 138.645 | 0 | SE of Hawker, SA. |
| 2.4 | ML | 12/06/2012 | 01:23:52 | -31.606 | 138.598 | 8 | NE of Hawker, SA. |
| 1.6 | ML | 12/06/2012 | 18:09:14 | -33.414 | 138.108 | 10 | Crystal Brook, SA. |
| 2.2 | ML | 22/06/2012 | 14:21:16 | -33.06 | 138.781 | 6 | Yongala, SA. |
| 1.9 | ML | 23/06/2012 | 21:25:42 | -35.162 | 139.193 | 0 | Murray Bridge, SA. |
| 2.5 | ML | 26/06/2012 | 13:40:36 | -31.651 | 138.529 | 15 | NE of Hawker, SA. |
| 2.2 | ML | 26/06/2012 | 20:07:02 | -33.331 | 136.98 | 15 | SW of Whyalla, SA. |
| 2.6 | ML | 01/07/2012 | 16:17:51 | -31.575 | 138.812 | 10 | NE of Hawker, SA. |
| 2.2 | ML | 05/07/2012 | 17:21:09 | -33.447 | 136.953 | 12 | Near Cowell, SA. |
| 2.0 | ML | 06/07/2012 | 07:58:07 | -35.634 | 138.565 | 17 | SW of Victor Harbor, SA. |
| 1.9 | ML | 09/07/2012 | 14:34:05 | -32.161 | 139.461 | 12 | Yunta, SA. |
| 2.0 | ML | 15/07/2012 | 12:27:44 | -34.22 | 138.553 | 8 | Owen, SA. |
| 2.5 | ML | 16/07/2012 | 17:11:45 | -35.735 | 137.873 | 15 | Kangaroo Island, SA. |
| 2.2 | ML | 23/07/2012 | 12:20:59 | -35.985 | 136.282 | 10 | W of Kangaroo Island, SA. |
| 2.2 | ML | 24/07/2012 | 20:47:48 | -35.805 | 137.999 | 12 | Kangaroo Island, SA. |
| 2.2 | ML | 27/07/2012 | 21:00:09 | -35.556 | 137.38 | 2 | Kangaroo Island, SA. |
| 2.1 | ML | 07/08/2012 | 17:42:29 | -35.424 | 137.518 | 10 | N of Kingscote, SA. |
| 2.6 | ML | 17/08/2012 | 01:07:30 | -32.26 | 138.496 | 10 | S of Hawker, SA. |
| 3.4 | ML | 19/08/2012 | 16:27:19 | -27.598 | 135.324 | 10 | Near Oodnadatta, SA. |
| 1.9 | ML | 24/08/2012 | 02:03:08 | -33.546 | 138.121 | 10 | Mundoora, SA. |
| 2.3 | ML | 31/08/2012 | 12:21:28 | -32.568 | 138.476 | 7 | Near Orroroo, SA. |
| 1.9 | ML | 01/09/2012 | 14:29:53 | -34.042 | 135.758 | 10 | Near Cummins, SA. |
| 3.1 | ML | 09/09/2012 | 08:41:56 | -38.607 | 140.855 | 9 | S of Port Macdonnell, SA. |
| 3.0 | ML | 09/09/2012 | 09:23:52 | -38.639 | 140.805 | 10 | S of Port Macdonnell, SA. |
| 3.0 | ML | 11/09/2012 | 07:13:29 | -26.114 | 132.019 | 0 | NW of Ernabella SA. |
| 2.3 | ML | 20/09/2012 | 12:41:54 | -33.496 | 136.489 | 0 | North of Cleve, SA. |
| 2.2 | ML | 24/09/2012 | 03:04:35 | -35.702 | 138.004 | 15 | NE of Penneshaw, SA. |
| 2.5 | ML | 24/09/2012 | 15:37:14 | -30.812 | 138.826 | 10 | NE of Blinman, SA. |
| 3.5 | ML | 26/09/2012 | 05:22:35 | -26.054 | 132.163 | 13 | N of Ernabella, SA. |
| 2.5 | ML | 05/10/2012 | 14:46:33 | -30.734 | 139.223 | 10 | NE of Blinman, SA. |
| 1.8 | ML | 08/10/2012 | 04:32:29 | -32.577 | 138.561 | 10 | S of Carrieton, SA. |
| 1.7 | ML | 10/10/2012 | 14:15:21 | -33.824 | 139.13 | 12 | Near Burra SA. |
| 3.0 | ML | 13/11/2012 | 07:47:42 | -28.375 | 139.91 | 10 | SW of Innamincka, SA. |
| 1.5 | ML | 13/11/2012 | 09:09:19 | -33.27 | 138.267 | 13 | Near Laura, SA. |
| 2.2 | ML | 16/11/2012 | 03:16:18 | -33.391 | 138.455 | 7 | Near Georgetown, SA. |
| 1.7 | ML | 16/11/2012 | 19:48:16 | -33.538 | 138.651 | 6 | Near Spalding, SA. |
| 2.0 | ML | 19/11/2012 | 23:46:34 | -33.631 | 136.738 | 15 | NW of Cowell, SA. |
| 3.0 | ML | 20/11/2012 | 19:12:51 | -27.817 | 140.661 | 18 | Near Innamincka, SA. |
| 2.8 | ML | 21/11/2012 | 12:17:20 | -27.763 | 140.848 | 10 | Innamincka, SA. |
| 2.9 | ML | 22/11/2012 | 02:48:38 | -27.782 | 140.924 | 10 | Near Innamincka, SA. |
| 2.7 | ML | 23/11/2012 | 17:03:58 | -28.599 | 137.802 | 10 | Lake Eyre, SA. |
| 2.7 | ML | 23/11/2012 | 17:11:24 | -27.813 | 140.791 | 10 | Near Innamincka, SA. |
| 2.6 | ML | 25/11/2012 | 06:53:24 | -27.787 | 140.721 | 5 | Near Innamincka, SA. |
| 2.7 | ML | 25/11/2012 | 15:06:56 | -27.969 | 140.822 | 10 | Near Innamincka, SA. |
| 2.6 | ML | 27/11/2012 | 04:01:26 | -27.587 | 140.482 | 0 | Near Innamincka, SA. |
| 2.5 | ML | 27/11/2012 | 04:26:09 | -27.82 | 140.619 | 0 | Near Innamincka, SA. |
| 3.6 | ML | 27/11/2012 | 07:13:07 | -27.793 | 140.751 | 0 | Near Innamincka, SA. |
| 3.2 | ML | 27/11/2012 | 08:02:59 | -27.837 | 140.711 | 5 | Near Innamincka, SA. |
| 2.9 | ML | 28/11/2012 | 16:37:17 | -27.895 | 140.46 | 10 | Near Innamincka, SA. |
| 2.8 | ML | 28/11/2012 | 16:46:38 | -27.867 | 140.663 | 10 | Near Innamincka, SA. |
| 2.8 | ML | 28/11/2012 | 17:09:29 | -27.83 | 140.704 | 10 | Near Innamincka, SA. |
| 2.7 | ML | 29/11/2012 | 03:57:10 | -27.83 | 140.614 | 0 | Near Innamincka, SA. |
| 2.8 | ML | 29/11/2012 | 07:32:34 | -27.933 | 140.671 | 0 | Near Innamincka, SA. |
| 2.9 | ML | 29/11/2012 | 08:55:37 | -27.885 | 140.738 | 10 | Near Innamincka, SA. |
| 3.3 | ML | 29/11/2012 | 14:53:40 | -27.832 | 140.758 | 14 | Near Innamincka, SA. |
| 2.9 | ML | 29/11/2012 | 15:21:07 | -27.804 | 140.789 | 10 | Near Innamincka, SA. |
| 2.8 | ML | 29/11/2012 | 17:03:01 | -27.871 | 140.741 | 10 | Near Innamincka, SA. |
| 3.0 | ML | 29/11/2012 | 17:48:46 | -27.838 | 140.764 | 10 | Near Innamincka, SA. |
| 3.2 | ML | 29/11/2012 | 18:19:24 | -27.818 | 140.676 | 12 | Near Innamincka, SA. |
| 3.9 | ML | 29/11/2012 | 19:21:18 | -27.756 | 140.659 | 5 | Near Innamincka, SA. |
| 3.2 | ML | 29/11/2012 | 19:29:25 | -27.74 | 140.65 | 10 | Near Innamincka, SA. |
| 3.2 | ML | 29/11/2012 | 19:45:35 | -27.757 | 140.687 | 10 | Near Innamincka, SA. |
| 3.0 | ML | 29/11/2012 | 20:05:08 | -27.73 | 140.674 | 10 | Near Innamincka, SA. |
| 3.5 | ML | 29/11/2012 | 20:31:30 | -27.811 | 140.587 | 8 | Near Innamincka, SA. |
| 3.3 | ML | 29/11/2012 | 22:08:19 | -27.785 | 140.625 | 12 | Near Innamincka, SA. |
| 3.3 | ML | 29/11/2012 | 22:26:52 | -27.763 | 140.592 | 10 | Near Innamincka, SA. |
| 3.1 | ML | 30/11/2012 | 07:52:42 | -27.704 | 140.531 | 0 | Near Innamincka, SA. |
| 1.6 | ML | 30/11/2012 | 17:43:00 | -32.807 | 138.265 | 10 | Near Melrose, SA. |
| 3.0 | ML | 01/12/2012 | 18:48:03 | -27.844 | 140.684 | 10 | Near Innamincka, SA. |
| 2.9 | ML | 04/12/2012 | 17:51:04 | -29.936 | 139.124 | 10 | NW of Yudnamutana, SA. |
| 1.5 | ML | 07/12/2012 | 23:24:51 | -33.426 | 137.933 | 0 | W of Merriton, SA. |
| 2.4 | ML | 20/12/2012 | 23:30:41 | -32.069 | 138.727 | 5 | SE of Hawker, SA. |
| 2.1 | ML | 25/12/2012 | 02:45:58 | -32.413 | 138.683 | 4 | Near Carrieton, SA. |

Western Austral ian Earthquakes

Table A.4 Western Australian Earthquakes 2012

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.1 | ML | 01/01/2012 | 16:37:04 | -31.652 | 117.118 | 10 | E of Meckering, WA. |
| 2.0 | ML | 01/01/2012 | 16:37:05 | -31.709 | 117.066 | 17 | SE of Meckering, WA. |
| 1.9 | ML | 06/01/2012 | 08:45:07 | -30.479 | 117.108 | 0 | NW of Burakin, WA. |
| 3.3 | ML | 10/01/2012 | 18:44:01 | -33.308 | 118.222 | 0 | NE of Nyabing, WA. |
| 2.5 | ML | 11/01/2012 | 03:37:04 | -14.809 | 132.037 | 12 | SW of Katherine, WA. |
| 2.0 | ML | 12/01/2012 | 12:10:08 | -32.140 | 117.151 | 8 | E of Beverley, WA. |
| 2.7 | ML | 13/01/2012 | 19:02:13 | -30.681 | 121.554 | 4 | Kalgoorlie, WA. |
| 2.2 | ML | 16/01/2012 | 06:41:22 | -30.620 | 117.542 | 0 | SE of Burakin, WA. |
| 2.1 | ML | 17/01/2012 | 17:21:03 | -30.828 | 118.764 | 10 | SE of Bonnie Rock, WA. |
| 1.7 | ML | 17/01/2012 | 18:54:32 | -31.729 | 117.028 | 7 | S of Meckering, WA. |
| 1.8 | ML | 18/01/2012 | 14:57:28 | -30.283 | 117.684 | 0 | NW of Beacon, WA. |
| 2.0 | ML | 20/01/2012 | 12:39:45 | -30.318 | 117.665 | 14 | NW of Beacon, WA. |
| 2.0 | ML | 20/01/2012 | 20:49:33 | -31.664 | 117.058 | 19 | SE of Meckering, WA. |
| 2.0 | ML | 24/01/2012 | 00:20:16 | -31.145 | 116.683 | 16 | E of Calingiri, WA. |
| 3.0 | ML | 24/01/2012 | 11:42:30 | -30.449 | 116.935 | 11 | SW of Kalannie, WA. |
| 1.9 | ML | 24/01/2012 | 12:20:44 | -30.438 | 116.932 | 4 | SW of Kalannie, WA. |
| 2.7 | ML | 29/01/2012 | 13:45:02 | -32.183 | 118.988 | 10 | NE of Hyden, WA. |
| 2.1 | ML | 03/02/2012 | 19:58:03 | -32.575 | 116.783 | 1 | W of Pingelly, WA. |
| 2.2 | ML | 08/02/2012 | 19:53:06 | -30.683 | 118.442 | 9 | SE of Bonnie Rock, WA. |
| 2.4 | ML | 09/02/2012 | 22:48:50 | -33.178 | 113.896 | 30 | W of Bunbury, WA. |
| 2.0 | ML | 16/02/2012 | 04:33:54 | -32.627 | 117.983 | 7 | SW of Condinin, WA. |
| 1.8 | ML | 16/02/2012 | 20:49:04 | -30.943 | 117.801 | 12 | SW of Bencubbin, WA. |
| 1.9 | ML | 16/02/2012 | 22:13:06 | -30.979 | 117.805 | 17 | SW of Bencubbin, WA. |
| 2.9 | ML | 18/02/2012 | 04:02:15 | -24.096 | 113.603 | 10 | Lake Macleod, WA. |
| 2.0 | ML | 18/02/2012 | 12:21:24 | -30.448 | 116.909 | 7 | NE of Ballidu, WA. |
| 1.9 | ML | 18/02/2012 | 16:42:40 | -33.313 | 118.582 | 1 | SE of Lake Grace, WA. |
| 1.7 | ML | 20/02/2012 | 10:14:26 | -30.403 | 117.749 | 10 | NW of Beacon, WA. |
| 1.9 | ML | 20/02/2012 | 15:25:58 | -33.613 | 117.670 | 14 | NE of Katanning, WA. |
| 1.9 | ML | 20/02/2012 | 15:52:43 | -33.605 | 117.642 | 10 | NE of Katanning, WA. |
| 2.3 | ML | 22/02/2012 | 03:54:22 | -29.719 | 115.866 | 13 | Yarra Yarra Lakes, WA. |
| 2.5 | ML | 23/02/2012 | 22:16:36 | -30.500 | 118.651 | 10 | NE of Bonnie Rock, WA. |
| 2.4 | ML | 24/02/2012 | 17:51:19 | -27.276 | 119.636 | 0 | SE of Wilson, WA. |
| 1.5 | ML | 25/02/2012 | 21:38:56 | -30.437 | 116.913 | 6 | NE of Ballidu. |
| 2.1 | ML | 01/03/2012 | 16:53:30 | -33.323 | 118.011 | 5 | NW of Nyabing, WA. |
| 2.0 | ML | 01/03/2012 | 18:23:48 | -30.403 | 117.765 | 10 | NW of Beacon, WA. |
| 2.2 | ML | 01/03/2012 | 18:53:32 | -30.395 | 117.792 | 0 | NW of Beacon, WA. |
| 3.1 | ML | 05/03/2012 | 17:20:52 | -16.820 | 120.616 | 33 | NW of Broome, WA. |
| 2.4 | ML | 07/03/2012 | 08:54:47 | -31.190 | 118.349 | 10 | NE of Merredin, WA. |
| 2.0 | ML | 08/03/2012 | 06:45:17 | -30.870 | 121.100 | 0 | Near Kalgoorlie, WA. |
| 2.5 | ML | 09/03/2012 | 02:32:30 | -30.400 | 117.821 | 12 | Beacon, WA. |
| 3.4 | ML | 09/03/2012 | 06:16:26 | -30.410 | 117.840 | 8 | Beacon, WA. |
| 3.3 | ML | 09/03/2012 | 06:59:47 | -30.397 | 117.822 | 9 | Beacon, WA. |
| 2.3 | ML | 09/03/2012 | 08:27:14 | -30.407 | 117.716 | 0 | Beacon, WA. |
| 2.1 | ML | 09/03/2012 | 16:50:44 | -30.565 | 117.619 | 0 | SW of Beacon, WA. |
| 2.9 | ML | 10/03/2012 | 05:30:04 | -34.125 | 117.005 | 10 | SW of Kojonup, WA. |
| 2.3 | ML | 11/03/2012 | 01:21:51 | -30.478 | 116.964 | 11 | NW of Burakin, WA. |
| 2.1 | ML | 12/03/2012 | 05:25:12 | -30.397 | 117.803 | 5 | Beacon, WA. |
| 2.7 | ML | 13/03/2012 | 18:48:14 | -28.795 | 116.347 | 0 | NE of Morawa, WA. |
| 3.0 | ML | 13/03/2012 | 20:26:01 | -34.134 | 117.186 | 15 | S of Kojonup, WA. |
| 3.0 | ML | 16/03/2012 | 13:04:01 | -27.009 | 113.952 | 5 | N of Kalbarri, WA. |
| 2.3 | ML | 17/03/2012 | 22:42:50 | -34.132 | 117.225 | 0 | S of Kojonup, WA. |
| 2.1 | ML | 19/03/2012 | 21:02:52 | -31.951 | 117.237 | 10 | NW of Quairading, WA. |
| 2.0 | ML | 24/03/2012 | 21:09:24 | -30.481 | 117.698 | 6 | Beacon, WA. |
| 2.7 | ML | 25/03/2012 | 14:52:08 | -30.406 | 117.817 | 12 | Beacon, WA. |
| 1.9 | ML | 25/03/2012 | 15:07:07 | -30.439 | 117.758 | 0 | Beacon, WA. |
| 2.3 | ML | 25/03/2012 | 15:40:50 | -30.375 | 117.827 | 10 | Beacon, WA. |
| 3.4 | ML | 25/03/2012 | 16:57:41 | -30.405 | 117.843 | 10 | Beacon, WA. |
| 2.2 | ML | 26/03/2012 | 03:08:45 | -30.422 | 117.802 | 10 | Beacon, WA. |
| 1.6 | ML | 26/03/2012 | 03:14:18 | -30.498 | 117.689 | 10 | Beacon, WA. |
| 2.3 | ML | 26/03/2012 | 17:53:18 | -28.615 | 115.962 | 1 | N of Morawa, WA. |
| 1.7 | ML | 26/03/2012 | 23:40:50 | -31.560 | 116.931 | 10 | Meckering, WA. |
| 2.2 | ML | 27/03/2012 | 03:41:44 | -32.146 | 118.845 | 10 | N of Hyden, WA. |
| 3.4 | ML | 29/03/2012 | 05:05:21 | -29.031 | 114.224 | 3 | SW of Geraldton, WA. |
| 2.4 | ML | 06/04/2012 | 18:18:53 | -29.464 | 118.485 | 10 | W of Lake Barlee, WA. |
| 1.8 | ML | 07/04/2012 | 09:42:43 | -33.359 | 118.173 | 3 | Near Merilup, WA. |
| 2.1 | ML | 07/04/2012 | 21:30:54 | -30.373 | 117.806 | 5 | Beacon, WA. |
| 2.1 | ML | 08/04/2012 | 13:35:26 | -30.389 | 117.756 | 0 | Beacon, WA. |
| 1.9 | ML | 08/04/2012 | 23:00:41 | -30.407 | 117.768 | 4 | Beacon, WA. |
| 1.9 | ML | 09/04/2012 | 11:07:14 | -30.420 | 117.757 | 17 | Beacon, WA. |
| 2.0 | ML | 11/04/2012 | 06:53:10 | -31.227 | 118.217 | 13 | N of Merredin, WA. |
| 1.8 | ML | 11/04/2012 | 18:07:00 | -31.239 | 117.451 | 0 | Wyalkatchem, WA. |
| 2.3 | ML | 13/04/2012 | 22:55:13 | -33.276 | 118.963 | 0 | E of Lake Grace, WA. |
| 2.1 | ML | 20/04/2012 | 14:51:58 | -31.236 | 118.285 | 16 | N of Merredin, WA. |
| 2.1 | ML | 20/04/2012 | 16:47:08 | -31.238 | 118.180 | 11 | N of Merredin, WA. |
| 2.4 | ML | 20/04/2012 | 19:01:46 | -31.702 | 117.070 | 10 | SE of Meckering, WA. |
| 2.1 | ML | 20/04/2012 | 21:13:52 | -31.202 | 118.152 | 7 | N of Merredin, WA. |
| 2.1 | ML | 20/04/2012 | 21:14:50 | -31.218 | 118.127 | 8 | NW of Merredin, WA. |
| 2.1 | ML | 20/04/2012 | 21:19:50 | -31.224 | 118.143 | 8 | NW of Merredin, WA. |
| 2.1 | ML | 20/04/2012 | 21:21:34 | -31.191 | 118.134 | 5 | NW of Merredin, WA. |
| 3.0 | ML | 21/04/2012 | 01:33:15 | -31.222 | 118.203 | 11 | N of Merredin, WA. |
| 2.1 | ML | 22/04/2012 | 18:50:04 | -33.599 | 117.759 | 10 | Nyabing, WA. |
| 2.0 | ML | 22/04/2012 | 19:39:33 | -31.244 | 118.171 | 11 | NW of Merredin, WA. |
| 2.3 | ML | 23/04/2012 | 02:13:36 | -31.184 | 118.344 | 7 | NE of Merredin, WA. |
| 1.9 | ML | 23/04/2012 | 11:59:39 | -31.209 | 118.162 | 8 | NW of Merredin, WA. |
| 1.6 | ML | 24/04/2012 | 18:01:39 | -32.477 | 117.268 | 5 | NE of Pingelly, WA. |
| 1.7 | ML | 24/04/2012 | 18:47:01 | -32.468 | 117.279 | 12 | NE of Pingelly, WA. |
| 1.7 | ML | 24/04/2012 | 19:33:13 | -32.531 | 117.297 | 10 | E of Pingelly, WA. |
| 1.7 | ML | 24/04/2012 | 19:34:43 | -32.485 | 117.275 | 7 | NE of Pingelly, WA. |
| 1.7 | ML | 24/04/2012 | 19:40:06 | -32.517 | 117.288 | 10 | E of Pingelly, WA. |
| 1.7 | ML | 24/04/2012 | 19:42:55 | -32.491 | 117.337 | 11 | NE of Pingelly, WA. |
| 1.7 | ML | 24/04/2012 | 19:48:55 | -32.519 | 117.285 | 2 | E of Pingelly, WA. |
| 1.8 | ML | 24/04/2012 | 23:33:22 | -32.515 | 117.292 | 6 | E of Pingelly, WA. |
| 1.7 | ML | 25/04/2012 | 00:26:34 | -32.490 | 117.260 | 9 | E of Pingelly, WA. |
| 1.8 | ML | 25/04/2012 | 05:21:31 | -30.525 | 117.324 | 7 | E of Burakin, WA. |
| 2.2 | ML | 26/04/2012 | 09:50:18 | -29.142 | 116.875 | 1 | Mongers Lake, WA. |
| 2.5 | ML | 29/04/2012 | 14:34:53 | -33.331 | 118.004 | 12 | E of Dumbleyung, WA. |
| 1.8 | ML | 01/05/2012 | 19:24:51 | -31.782 | 117.380 | 7 | SE of Cunderdin, WA. |
| 1.9 | ML | 02/05/2012 | 01:45:48 | -31.561 | 117.611 | 10 | NW of Kellerberrin, WA. |
| 2.0 | ML | 03/05/2012 | 14:27:42 | -33.291 | 118.794 | 10 | E of Lake Grace, WA. |
| 2.7 | ML | 08/05/2012 | 02:55:26 | -28.634 | 116.099 | 10 | N of Morawa. |
| 2.2 | ML | 08/05/2012 | 10:42:00 | -30.272 | 117.729 | 10 | NW of Beacon, WA. |
| 2.5 | ML | 08/05/2012 | 15:18:50 | -30.217 | 117.841 | 16 | N of Beacon, WA. |
| 2.8 | ML | 10/05/2012 | 22:28:49 | -23.903 | 115.850 | 10 | SW of Nanutarra, WA. |
| 1.8 | ML | 11/05/2012 | 20:35:18 | -30.261 | 117.717 | 5 | NW of Beacon, WA. |
| 2.0 | ML | 16/05/2012 | 06:50:53 | -30.686 | 117.432 | 9 | N of Koorda, WA. |
| 1.7 | ML | 19/05/2012 | 16:30:10 | -33.358 | 116.166 | 0 | Collie, WA. |
| 2.2 | ML | 24/05/2012 | 02:01:37 | -30.590 | 117.678 | 6 | SW of Beacon, WA. |
| 1.9 | ML | 25/05/2012 | 14:12:37 | -30.602 | 117.614 | 12 | SW of Beacon, WA. |
| 2.0 | ML | 25/05/2012 | 14:54:19 | -30.609 | 117.613 | 13 | SW of Beacon, WA. |
| 2.0 | ML | 25/05/2012 | 17:12:03 | -30.477 | 117.694 | 17 | W of Beacon, WA. |
| 2.2 | ML | 27/05/2012 | 16:59:34 | -31.274 | 117.668 | 10 | Near Wyalkatchem, WA. |
| 1.8 | ML | 29/05/2012 | 20:17:26 | -33.260 | 118.793 | 0 | SW of Newdegate, WA. |
| 1.8 | ML | 09/06/2012 | 17:12:32 | -31.242 | 117.429 | 0 | Wyalkatchem, WA. |
| 1.8 | ML | 09/06/2012 | 19:54:40 | -30.364 | 117.680 | 12 | NW of Beacon, WA. |
| 1.8 | ML | 09/06/2012 | 22:16:15 | -30.427 | 117.654 | 8 | W of Beacon, WA. |
| 2.1 | ML | 10/06/2012 | 16:28:38 | -31.615 | 116.994 | 0 | Meckering, WA. |
| 2.2 | ML | 14/06/2012 | 23:06:17 | -30.798 | 117.761 | 0 | Bencubbin, WA. |
| 2.2 | ML | 17/06/2012 | 17:01:20 | -31.573 | 117.033 | 10 | Meckering, WA. |
| 1.7 | ML | 17/06/2012 | 20:01:43 | -31.683 | 117.07 | 6 | Meckering, WA. |
| 1.6 | ML | 17/06/2012 | 20:20:24 | -31.723 | 117.083 | 1 | SE of Meckering, WA. |
| 2.0 | ML | 18/06/2012 | 01:52:35 | -31.211 | 116.570 | 3 | Bolgart, WA. |
| 1.8 | ML | 20/06/2012 | 16:58:51 | -30.735 | 116.237 | 0 | Near Moora, WA. |
| 2.8 | ML | 23/06/2012 | 15:40:54 | -30.672 | 121.557 | 10 | NE of Kalgoorlie, WA. |
| 2.8 | ML | 23/06/2012 | 15:55:25 | -30.649 | 121.554 | 1 | NE of Kalgoorlie, WA. |
| 2.2 | ML | 23/06/2012 | 21:33:16 | -30.491 | 117.719 | 17 | Beacon, WA. |
| 2.3 | ML | 28/06/2012 | 04:58:59 | -31.686 | 117.067 | 5 | Meckering, WA. |
| 2.3 | ML | 28/06/2012 | 11:22:50 | -30.457 | 117.684 | 10 | W of Beacon, WA. |
| 2.5 | ML | 08/07/2012 | 09:33:51 | -29.131 | 116.603 | 10 | E of Morawa, WA. |
| 1.6 | ML | 09/07/2012 | 01:23:45 | -30.552 | 117.035 | 0 | W of Burakin, WA. |
| 1.7 | ML | 14/07/2012 | 02:07:25 | -30.657 | 117.396 | 0 | N of Koorda, WA. |
| 2.4 | ML | 17/07/2012 | 21:22:10 | -30.621 | 117.429 | 0 | N of Koorda, WA. |
| 2.3 | ML | 17/07/2012 | 21:54:49 | -30.648 | 117.429 | 9 | N of Koorda, WA. |
| 2.3 | ML | 17/07/2012 | 22:13:34 | -30.630 | 117.437 | 1 | N of Koorda, WA. |
| 2.4 | ML | 17/07/2012 | 22:51:24 | -30.629 | 117.469 | 0 | N of Koorda, WA. |
| 2.0 | ML | 18/07/2012 | 02:44:15 | -30.575 | 117.487 | 0 | N of Koorda, WA. |
| 3.5 | ML | 18/07/2012 | 09:20:55 | -30.429 | 117.794 | 2 | Beacon, WA. |
| 2.1 | ML | 18/07/2012 | 11:39:49 | -30.637 | 117.448 | 3 | N of Koorda, WA. |
| 2.2 | ML | 18/07/2012 | 12:37:48 | -30.426 | 117.759 | 1 | Beacon, WA. |
| 3.2 | ML | 18/07/2012 | 16:35:18 | -30.405 | 117.88 | 3 | Beacon, WA. |
| 1.9 | ML | 19/07/2012 | 12:05:32 | -30.630 | 117.450 | 11 | N of Koorda, WA. |
| 2.3 | ML | 19/07/2012 | 16:21:28 | -30.435 | 117.754 | 5 | Beacon, WA. |
| 2.9 | ML | 20/07/2012 | 17:02:28 | -27.303 | 113.708 | 8 | NW of Kalbarri, WA. |
| 2.1 | ML | 21/07/2012 | 05:25:58 | -30.406 | 117.629 | 7 | Beacon, WA. |
| 2.1 | ML | 25/07/2012 | 20:36:04 | -30.440 | 117.790 | 0 | Beacon, WA. |
| 2.0 | ML | 28/07/2012 | 11:43:34 | -30.651 | 117.391 | 0 | NW of Koorda, WA. |
| 1.8 | ML | 29/07/2012 | 13:15:24 | -30.960 | 117.116 | 0 | S of Manmanning, WA. |
| 2.3 | ML | 29/07/2012 | 16:51:56 | -28.021 | 120.355 | 10 | SW of Leinster, WA. |
| 2.2 | ML | 04/08/2012 | 07:38:03 | -31.269 | 118.175 | 10 | N of Merredin, WA. |
| 2.3 | ML | 04/08/2012 | 08:05:25 | -30.589 | 117.487 | 0 | N of Koorda, WA. |
| 2.3 | ML | 04/08/2012 | 08:38:38 | -31.100 | 118.351 | 1 | SE of Mukinbudin, WA. |
| 2.3 | ML | 04/08/2012 | 13:22:56 | -33.379 | 117.761 | 3 | Dumbleyung, WA. |
| 2.1 | ML | 05/08/2012 | 07:11:13 | -30.491 | 117.068 | 0 | NW of Burakin, WA. |
| 2.0 | ML | 06/08/2012 | 19:08:50 | -31.236 | 117.445 | 0 | Wyalkatchem, WA. |
| 1.8 | ML | 08/08/2012 | 13:48:15 | -31.507 | 116.811 | 0 | NE of Northam, WA. |
| 2.1 | ML | 08/08/2012 | 21:46:43 | -30.427 | 117.68 | 0 | Near Beacon, WA. |
| 2.1 | ML | 08/08/2012 | 22:23:05 | -30.470 | 117.716 | 0 | Near Beacon, WA. |
| 2.3 | ML | 09/08/2012 | 01:17:27 | -30.481 | 117.718 | 7 | Beacon, WA. |
| 2.3 | ML | 09/08/2012 | 01:36:56 | -30.450 | 117.731 | 7 | Near Beacon, WA. |
| 2.8 | ML | 09/08/2012 | 04:20:52 | -30.436 | 117.770 | 4 | Near Beacon, WA. |
| 2.8 | ML | 09/08/2012 | 04:33:58 | -30.411 | 117.836 | 6 | Near Beacon, WA. |
| 1.9 | ML | 09/08/2012 | 04:41:50 | -30.411 | 117.727 | 0 | Near Beacon, WA. |
| 2.6 | ML | 09/08/2012 | 07:38:29 | -30.447 | 117.786 | 2 | Near Beacon, WA. |
| 2.1 | ML | 09/08/2012 | 07:55:36 | -30.505 | 117.685 | 10 | Near Beacon, WA. |
| 2.3 | ML | 09/08/2012 | 08:44:48 | -30.460 | 117.792 | 11 | Beacon, WA. |
| 1.6 | ML | 09/08/2012 | 10:36:23 | -30.146 | 117.190 | 7 | Near Kalannie, WA. |
| 1.9 | ML | 09/08/2012 | 11:25:30 | -30.428 | 117.775 | 13 | Beacon, WA. |
| 2.3 | ML | 09/08/2012 | 20:30:24 | -30.484 | 117.747 | 9 | Beacon, WA. |
| 2.2 | ML | 09/08/2012 | 20:45:22 | -30.471 | 117.720 | 9 | Beacon, WA. |
| 2.1 | ML | 09/08/2012 | 20:47:16 | -30.480 | 117.715 | 4 | Beacon, WA. |
| 2.3 | ML | 09/08/2012 | 20:59:11 | -30.454 | 117.724 | 9 | Beacon, WA. |
| 2.2 | ML | 11/08/2012 | 17:21:08 | -30.436 | 117.742 | 2 | Beacon, WA. |
| 2.2 | ML | 11/08/2012 | 17:27:14 | -30.423 | 117.794 | 9 | Beacon, WA. |
| 2.0 | ML | 11/08/2012 | 17:30:54 | -30.448 | 117.740 | 10 | Beacon, WA. |
| 2.4 | ML | 11/08/2012 | 17:37:47 | -30.530 | 118.568 | 10 | E of Bonnie Rock, WA. |
| 2.5 | ML | 13/08/2012 | 02:55:28 | -30.410 | 117.711 | 0 | Near Beacon, WA. |
| 2.3 | ML | 13/08/2012 | 05:36:12 | -30.671 | 117.424 | 10 | Near Beacon, WA. |
| 2.4 | ML | 13/08/2012 | 09:17:02 | -30.470 | 117.782 | 10 | Beacon, WA. |
| 2.2 | ML | 13/08/2012 | 12:09:13 | -30.591 | 117.466 | 0 | N of Koorda, WA. |
| 2.8 | ML | 14/08/2012 | 00:51:56 | -30.452 | 117.789 | 10 | Beacon, WA. |
| 2.3 | ML | 14/08/2012 | 14:12:23 | -30.414 | 117.720 | 5 | Near Beacon, WA. |
| 2.4 | ML | 14/08/2012 | 16:16:04 | -30.489 | 118.530 | 10 | Near Bonnie Rock, WA. |
| 2.5 | ML | 14/08/2012 | 16:59:34 | -31.877 | 116.467 | 0 | W of York, WA. |
| 2.1 | ML | 14/08/2012 | 17:51:07 | -31.841 | 116.479 | 0 | W of York, WA. |
| 2.1 | ML | 15/08/2012 | 10:07:29 | -30.426 | 117.785 | 0 | Beacon, WA. |
| 1.9 | ML | 15/08/2012 | 19:19:15 | -30.385 | 117.727 | 0 | Near Beacon, WA. |
| 2.1 | ML | 17/08/2012 | 04:22:14 | -30.458 | 117.756 | 3 | Beacon, WA. |
| 2.1 | ML | 17/08/2012 | 07:32:41 | -30.443 | 117.741 | 3 | Near Beacon, WA. |
| 2.3 | ML | 17/08/2012 | 09:20:42 | -30.463 | 117.770 | 5 | Beacon, WA. |
| 2.0 | ML | 19/08/2012 | 04:29:42 | -30.457 | 117.693 | 15 | Near Beacon, WA. |
| 2.0 | ML | 26/08/2012 | 22:24:54 | -30.651 | 117.422 | 8 | N of Koorda, WA. |
| 2.0 | ML | 26/08/2012 | 22:26:47 | -30.673 | 117.410 | 5 | NW of Koorda, WA. |
| 2.0 | ML | 26/08/2012 | 23:15:16 | -30.647 | 117.423 | 7 | N of Koorda, WA. |
| 2.5 | ML | 27/08/2012 | 04:56:40 | -30.731 | 117.325 | 10 | NW of Koorda, WA. |
| 2.4 | ML | 27/08/2012 | 23:46:02 | -30.416 | 117.824 | 5 | Beacon, WA. |
| 1.9 | ML | 28/08/2012 | 01:18:48 | -30.677 | 117.412 | 0 | N of Koorda, WA. |
| 1.9 | ML | 28/08/2012 | 01:49:09 | -30.644 | 117.455 | 0 | N of Koorda, WA. |
| 2.7 | ML | 28/08/2012 | 04:32:50 | -30.427 | 117.736 | 4 | Near Beacon, WA. |
| 2.0 | ML | 28/08/2012 | 08:01:37 | -30.660 | 117.469 | 0 | Koorda, WA. |
| 2.8 | ML | 28/08/2012 | 19:42:42 | -30.438 | 117.773 | 5 | Beacon, WA. |
| 2.4 | ML | 28/08/2012 | 19:45:06 | -30.493 | 117.710 | 10 | Near Beacon, WA. |
| 2.4 | ML | 28/08/2012 | 19:48:57 | -30.499 | 117.613 | 0 | SW of Beacon, WA. |
| 2.4 | ML | 28/08/2012 | 21:59:16 | -30.414 | 117.762 | 3 | Beacon, WA. |
| 1.7 | ML | 01/09/2012 | 14:14:31 | -30.439 | 116.869 | 0 | Near Ballidu, WA. |
| 2.0 | ML | 02/09/2012 | 08:06:15 | -31.963 | 117.236 | 6 | NW of Quairading, WA. |
| 2.1 | ML | 02/09/2012 | 14:44:35 | -31.947 | 117.232 | 0 | NW of Quairading, WA. |
| 2.8 | ML | 03/09/2012 | 10:06:03 | -28.654 | 115.990 | 9 | SE of Mullewa, WA. |
| 1.9 | ML | 03/09/2012 | 20:49:25 | -30.434 | 116.880 | 0 | NE of Ballidu, WA. |
| 2.5 | ML | 04/09/2012 | 09:21:58 | -30.495 | 117.705 | 10 | Near Beacon, WA. |
| 1.9 | ML | 05/09/2012 | 04:24:32 | -31.785 | 116.182 | 0 | Near Mundaring, WA. |
| 2.3 | ML | 05/09/2012 | 20:36:22 | -30.480 | 117.685 | 14 | Near Beacon, WA. |
| 2.6 | ML | 06/09/2012 | 12:56:07 | -33.320 | 118.561 | 0 | NE of Pingrup, WA. |
| 2.2 | ML | 07/09/2012 | 06:57:39 | -30.635 | 117.474 | 12 | N of Koorda, WA. |
| 1.8 | ML | 08/09/2012 | 04:53:41 | -31.662 | 117.041 | 5 | Meckering, WA. |
| 2.4 | ML | 08/09/2012 | 08:36:38 | -30.387 | 117.797 | 0 | Beacon, WA. |
| 2.8 | ML | 10/09/2012 | 03:16:46 | -24.438 | 113.395 | 0 | Lake Macleod, WA. |
| 1.9 | ML | 10/09/2012 | 08:32:16 | -30.695 | 117.310 | 5 | NW of Koorda, WA. |
| 1.4 | ML | 11/09/2012 | 08:04:30 | -30.738 | 117.352 | 0 | Near Koorda, WA. |
| 1.8 | ML | 12/09/2012 | 06:08:08 | -30.547 | 117.569 | 9 | N of Koorda, WA. |
| 1.8 | ML | 12/09/2012 | 17:14:57 | -30.594 | 117.509 | 8 | N of Koorda, WA. |
| 2.3 | ML | 14/09/2012 | 13:23:05 | -30.446 | 116.998 | 10 | SW of Kalannie, WA. |
| 1.6 | ML | 14/09/2012 | 18:38:20 | -31.574 | 117.804 | 5 | Near Kellerberrin, WA. |
| 2.3 | ML | 14/09/2012 | 21:13:10 | -25.949 | 116.055 | 0 | NW of Meekatharra, WA. |
| 2.2 | ML | 15/09/2012 | 13:05:37 | -30.649 | 117.407 | 0 | N of Koorda, WA. |
| 1.8 | ML | 15/09/2012 | 17:30:08 | -30.695 | 117.364 | 5 | NW of Koorda, WA. |
| 2.0 | ML | 16/09/2012 | 18:02:05 | -30.646 | 117.479 | 2 | N of Koorda, WA. |
| 2.1 | ML | 16/09/2012 | 20:47:16 | -30.720 | 117.541 | 6 | NE of Koorda, WA. |
| 2.1 | ML | 19/09/2012 | 08:56:42 | -30.636 | 117.382 | 0 | N of Koorda, WA. |
| 1.9 | ML | 20/09/2012 | 17:39:28 | -31.277 | 117.612 | 7 | SE of Wyalkatchem, WA. |
| 2.5 | ML | 21/09/2012 | 08:53:09 | -30.425 | 117.705 | 0 | Near Beacon, WA. |
| 2.4 | ML | 22/09/2012 | 19:32:58 | -30.597 | 117.434 | 8 | N of Koorda, WA. |
| 2.3 | ML | 22/09/2012 | 21:21:23 | -30.632 | 117.467 | 12 | N of Koorda, WA. |
| 2.2 | ML | 25/09/2012 | 21:32:30 | -31.447 | 117.741 | 2 | North Kellerberrin, WA. |
| 1.9 | ML | 26/09/2012 | 00:29:03 | -30.532 | 117.065 | 0 | Burakin, WA. |
| 2.3 | ML | 26/09/2012 | 14:03:31 | -31.439 | 117.765 | 2 | N of Kellerberrin, WA. |
| 3.0 | ML | 26/09/2012 | 17:37:29 | -30.996 | 121.072 | 10 | Bullabulling, WA. |
| 1.9 | ML | 01/10/2012 | 21:32:47 | -30.411 | 117.761 | 0 | Near Beacon, WA. |
| 1.9 | ML | 01/10/2012 | 22:07:17 | -30.409 | 117.762 | 0 | Near Beacon, WA. |
| 3.2 | ML | 07/10/2012 | 03:20:28 | -27.768 | 120.63 | 0 | Near Leinster, WA. |
| 2.2 | ML | 07/10/2012 | 14:09:08 | -30.456 | 117.692 | 3 | W of Beacon, WA. |
| 3.5 | ML | 12/10/2012 | 19:27:14 | -22.881 | 127.681 | 10 | W of Lake Mackay, WA. |
| 1.8 | ML | 17/10/2012 | 22:24:12 | -31.291 | 117.671 | 4 | SW of Trayning, WA. |
| 1.7 | ML | 26/10/2012 | 12:37:37 | -31.723 | 117.049 | 0 | Meckering, WA. |
| 2.1 | ML | 30/10/2012 | 20:46:23 | -30.657 | 117.383 | 1 | NW of Koorda, WA. |
| 2.1 | ML | 30/10/2012 | 21:47:38 | -30.592 | 117.454 | 2 | N of Koorda, WA. |
| 2.7 | ML | 31/10/2012 | 21:54:00 | -33.362 | 120.807 | 0 | W of Cascade, WA. |
| 3.6 | ML | 08/11/2012 | 03:56:55 | -29.898 | 126.992 | 0 | Nullarbor Plain, WA. |
| 2.2 | ML | 10/11/2012 | 17:11:12 | -30.367 | 117.721 | 8 | NW of Beacon, WA. |
| 2.8 | ML | 14/11/2012 | 20:23:03 | -30.322 | 117.802 | 2 | Near Beacon, WA. |
| 1.6 | ML | 15/11/2012 | 09:04:42 | -32.823 | 116.887 | 0 | NW of Narrogin, WA. |
| 2.1 | ML | 16/11/2012 | 10:38:03 | -30.433 | 117.744 | 0 | Beacon, WA. |
| 2.1 | ML | 16/11/2012 | 11:10:25 | -30.665 | 117.438 | 6 | N of Koorda, WA. |
| 1.8 | ML | 16/11/2012 | 11:48:45 | -30.652 | 117.401 | 0 | N of Koorda, WA. |
| 2.1 | ML | 18/11/2012 | 02:09:17 | -30.686 | 117.418 | 5 | NW of Koorda, WA. |
| 3.3 | ML | 21/11/2012 | 06:21:45 | -22.974 | 128.603 | 10 | Near Lake Mackay, WA. |
| 4.0 | ML | 28/11/2012 | 22:35:22 | -27.757 | 125.943 | 0 | SW of Warburton, WA. |
| 3.1 | ML | 30/11/2012 | 16:24:45 | -32.374 | 122.560 | 14 | SE of Norseman, WA. |
| 1.9 | ML | 30/11/2012 | 22:41:30 | -31.719 | 117.075 | 1 | Near Meckering, WA. |
| 3.4 | ML | 01/12/2012 | 17:19:07 | -16.195 | 127.661 | 10 | SW of Kununurra, WA. |
| 2.2 | ML | 02/12/2012 | 03:29:13 | -30.646 | 118.065 | 5 | SE of Beacon, WA. |
| 1.5 | ML | 05/12/2012 | 06:05:05 | -30.363 | 117.107 | 0 | Kalannie, WA. |
| 1.8 | ML | 06/12/2012 | 10:06:59 | -30.664 | 117.439 | 7 | N of Koorda, WA. |
| 1.4 | ML | 06/12/2012 | 15:58:29 | -30.505 | 117.687 | 0 | SW of Beacon, WA. |
| 1.5 | ML | 06/12/2012 | 22:00:06 | -30.661 | 117.420 | 8 | N of Koorda, WA. |
| 2.6 | ML | 09/12/2012 | 07:41:04 | -31.232 | 117.608 | 0 | SE of Wyalkatchem, WA. |
| 2.1 | ML | 10/12/2012 | 16:17:01 | -30.394 | 117.699 | 0 | Near Beacon, WA. |
| 1.9 | ML | 10/12/2012 | 16:41:17 | -30.491 | 117.677 | 0 | Near Beacon, WA. |
| 1.9 | ML | 11/12/2012 | 10:57:06 | -32.144 | 117.168 | 0 | SW of Quairading, WA. |
| 2.1 | ML | 12/12/2012 | 01:12:32 | -30.479 | 117.671 | 5 | Near Beacon, WA. |
| 2.3 | ML | 12/12/2012 | 04:03:57 | -30.445 | 117.713 | 5 | Near Beacon, WA. |
| 2.6 | ML | 12/12/2012 | 08:30:48 | -32.192 | 119.177 | 0 | Hyden, WA. |
| 2.1 | ML | 12/12/2012 | 18:06:59 | -30.474 | 117.718 | 0 | Near Beacon, WA. |
| 2.5 | ML | 12/12/2012 | 20:48:35 | -32.261 | 118.824 | 10 | N of Hyden, WA. |
| 2.3 | ML | 12/12/2012 | 21:49:39 | -32.252 | 118.814 | 5 | N of Hyden, WA. |
| 1.5 | ML | 15/12/2012 | 18:06:19 | -30.486 | 116.863 | 3 | NE of Ballidu, WA. |
| 2.2 | ML | 15/12/2012 | 22:40:08 | -30.636 | 117.423 | 2 | N of Koorda, WA. |
| 2.0 | ML | 17/12/2012 | 23:59:59 | -30.402 | 117.794 | 2 | Beacon, WA. |
| 3.0 | ML | 19/12/2012 | 23:50:01 | -24.921 | 112.497 | 10 | W of Carnarvon, WA. |
| 1.6 | ML | 20/12/2012 | 14:27:39 | -31.660 | 117.039 | 6 | Meckering, WA. |
| 2.4 | ML | 22/12/2012 | 06:08:49 | -30.417 | 117.752 | 0 | Near Beacon, WA. |
| 2.8 | ML | 22/12/2012 | 09:29:16 | -32.529 | 122.512 | 10 | SE of Norseman, WA. |
| 2.2 | ML | 22/12/2012 | 15:36:32 | -30.455 | 117.686 | 8 | W of Beacon, WA. |
| 1.6 | ML | 23/12/2012 | 02:16:24 | -31.789 | 117.374 | 0 | SE of Cunderdin, WA. |
| 2.0 | ML | 25/12/2012 | 01:13:56 | -30.675 | 117.381 | 1 | NW of Koorda, WA. |
| 2.7 | ML | 26/12/2012 | 20:46:15 | -35.114 | 115.809 | 2 | Off coast of Windy Harbour, WA. |
| 3.7 | ML | 28/12/2012 | 12:04:44 | -24.344 | 112.396 | 3 | Offshore Western Australia. |
| 2.1 | ML | 28/12/2012 | 13:59:11 | -32.272 | 118.644 | 10 | Near Hyden, WA. |
| 3.0 | ML | 29/12/2012 | 17:43:56 | -24.529 | 113.024 | 10 | NW of Carnarvon, WA. |

Australian Region Earthquakes

Table A.5 Australian Region Earthquakes (Magnitude 5+) 2012

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5.1 | MB | 01/01/2012 | 00:50:05 | -11.303 | 166.451 | 57 | Santa Cruz Islands. |
| 5.0 | MB | 01/01/2012 | 12:27:50 | -43.927 | 173.881 | 0 | Off E. Coast of S. Island, New Zealand. |
| 5.3 | MB | 01/01/2012 | 16:45:02 | -43.551 | 173.138 | 0 | South Island, New Zealand. |
| 5.2 | MB | 01/01/2012 | 18:09:08 | 4.670 | 96.521 | 43 | Northern Sumatra, Indonesia. |
| 5.2 | MB | 01/01/2012 | 21:07:27 | 2.148 | 128.423 | 79 | Halmahera, Indonesia. |
| 5.3 | MB | 04/01/2012 | 04:47:36 | -14.705 | 167.400 | 241 | Vanuatu Islands. |
| 5.0 | MB | 04/01/2012 | 15:35:52 | -7.460 | 128.748 | 163 | Banda Sea. |
| 5.1 | MB | 05/01/2012 | 01:13:41 | -17.729 | -173.002 | 66 | Tonga Islands Region. |
| 5.3 | MB | 05/01/2012 | 23:14:49 | -0.833 | 99.060 | 38 | Mentawai Islands, Indonesia. |
| 5.2 | MB | 06/01/2012 | 04:10:52 | -37.755 | 179.916 | 0 | Off E Coast of N Island, NZ. |
| 5.1 | MB | 07/01/2012 | 03:21:25 | -3.773 | 138.074 | 90 | Irian Jaya, Indonesia. |
| 5.4 | MB | 07/01/2012 | 04:30:24 | -4.213 | 152.849 | 60 | New Britain Region, PNG. |
| 5.7 | MB | 07/01/2012 | 07:31:44 | 1.641 | 127.455 | 136 | Halmahera, Indonesia. |
| 5.1 | MB | 07/01/2012 | 10:15:44 | -18.338 | -174.521 | 77 | Tonga Islands. |
| 5.5 | MB | 07/01/2012 | 17:24:50 | -5.180 | 145.320 | 56 | Eastern New Guinea Reg, PNG. |
| 5.6 | MB | 07/01/2012 | 18:38:06 | -28.170 | -176.673 | 0 | Kermadec Islands Region. |
| 5.4 | MB | 08/01/2012 | 00:35:54 | -17.742 | -177.993 | 650 | Fiji Islands Region. |
| 5.1 | MB | 08/01/2012 | 21:51:50 | -17.217 | 168.156 | 69 | Vanuatu Islands. |
| 6.6 | Mwp | 09/01/2012 | 04:07:18 | -10.517 | 165.185 | 54 | Santa Cruz Islands. |
| 5.3 | MB | 09/01/2012 | 20:13:08 | -0.794 | 133.288 | 17 | Irian Jaya Region, Indonesia. |
| 5.3 | MB | 10/01/2012 | 01:45:41 | -16.936 | 171.747 | 39 | Vanuatu Islands Region. |
| 5.2 | MB | 10/01/2012 | 13:08:35 | -17.309 | -172.365 | 0 | Tonga Islands Region. |
| 7.2 | Mwp | 10/01/2012 | 18:37:00 | 2.508 | 93.187 | 20 | Off W Coast of Northern Sumatra. |
| 5.2 | MB | 10/01/2012 | 21:00:50 | 2.401 | 93.127 | 10 | Off W Coast of Northern Sumatra. |
| 5.0 | MB | 11/01/2012 | 00:20:20 | -8.025 | 125.661 | 31 | Timor Region. |
| 5.7 | MB | 11/01/2012 | 22:21:33 | -6.913 | 123.424 | 640 | Banda Sea, Indonesia. |
| 5.4 | MB | 12/01/2012 | 07:08:01 | -18.074 | -171.992 | 0 | Tonga Islands Region. |
| 5.0 | MB | 13/01/2012 | 04:12:43 | -6.605 | 150.552 | 40 | New Britain Region, PNG. |
| 5.2 | MB | 13/01/2012 | 17:49:33 | -33.326 | -179.556 | 163 | Kermadec Islands Region. |
| 5.0 | MB | 13/01/2012 | 20:03:47 | 2.488 | 96.246 | 48 | Northern Sumatra, Indonesia. |
| 5.3 | MB | 13/01/2012 | 20:25:27 | -33.193 | -177.641 | 68 | South of Kermadec Islands. |
| 5.1 | MB | 16/01/2012 | 08:26:17 | -0.005 | 123.169 | 178 | Sulawesi, Indonesia. |
| 5.3 | MB | 16/01/2012 | 10:26:46 | -0.205 | 125.523 | 62 | Southern Molucca Sea. |
| 5.2 | MB | 16/01/2012 | 18:39:46 | -19.013 | -172.537 | 0 | Tonga Islands Region. |
| 5.0 | MB | 17/01/2012 | 09:23:18 | -4.502 | 133.360 | 0 | Irian Jaya Region, Indonesia. |
| 5.3 | MB | 17/01/2012 | 10:00:21 | 6.510 | 126.065 | 96 | Mindanao, Philippines. |
| 5.4 | MB | 18/01/2012 | 01:19:36 | -1.010 | 126.920 | 42 | Southern Molucca Sea. |
| 5.9 | Mwp | 18/01/2012 | 12:50:21 | -0.936 | 126.766 | 28 | Southern Molucca Sea. |
| 5.3 | MB | 18/01/2012 | 13:44:23 | -1.073 | 126.738 | 45 | Southern Molucca Sea. |
| 5.2 | MB | 18/01/2012 | 14:18:05 | -1.127 | 126.714 | 40 | Southern Molucca Sea. |
| 5.3 | MB | 18/01/2012 | 17:01:23 | -11.034 | 165.595 | 22 | Santa Cruz Islands. |
| 5.4 | MB | 18/01/2012 | 17:02:53 | -11.085 | 165.478 | 0 | Santa Cruz Islands. |
| 5.0 | MB | 19/01/2012 | 04:06:34 | -10.804 | 165.694 | 0 | Santa Cruz Islands. |
| 6.1 | Mwp | 19/01/2012 | 05:58:42 | -11.040 | 165.625 | 38 | Santa Cruz Islands. |
| 6.2 | Mwp | 19/01/2012 | 06:48:45 | -46.780 | 165.761 | 0 | Off W Coast of S Island, New Zealand. |
| 5.4 | MB | 19/01/2012 | 09:01:54 | -46.775 | 165.634 | 0 | Off W. Coast of S. Island, NZ. |
| 5.0 | MB | 20/01/2012 | 07:31:38 | -23.259 | 179.170 | 566 | South of Fiji Islands. |
| 5.7 | MB | 20/01/2012 | 20:32:51 | -8.565 | 119.614 | 109 | Flores Region, Indonesia. |
| 5.3 | MB | 20/01/2012 | 22:50:53 | 4.747 | 125.332 | 58 | Talaud Islands, Indonesia. |
| 6.2 | Mwp | 24/01/2012 | 00:52:05 | -24.727 | 178.769 | 594 | South of Fiji Islands. |
| 5.5 | MB | 25/01/2012 | 06:02:30 | -46.818 | 165.722 | 0 | Off W Coast of S Island, NZ. |
| 5.3 | MB | 25/01/2012 | 07:17:02 | -5.186 | 152.271 | 64 | New Britain Region, PNG. |
| 5.0 | MB | 26/01/2012 | 23:04:15 | -3.469 | 126.922 | 30 | Seram, Indonesia. |
| 5.4 | MB | 27/01/2012 | 00:06:45 | -1.564 | 138.375 | 11 | Near N Coast of Irian Jaya, New Guinea. |
| 5.1 | MB | 27/01/2012 | 09:54:30 | 2.804 | 93.003 | 0 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.1 | MB | 27/01/2012 | 17:16:27 | -10.352 | 116.025 | 22 | South of Lombok, Indonesia. |
| 6.2 | Mwp | 28/01/2012 | 17:42:54 | -29.270 | -176.697 | 56 | Kermadec Islands, New Zealand. |
| 5.4 | MB | 28/01/2012 | 17:45:55 | -29.518 | -176.733 | 106 | Kermadec Islands. |
| 5.4 | MB | 28/01/2012 | 18:17:58 | -29.332 | -176.807 | 88 | Kermadec Islands. |
| 5.5 | MB | 28/01/2012 | 18:49:06 | -29.127 | -176.858 | 54 | Kermadec Islands. |
| 5.5 | MB | 28/01/2012 | 19:05:44 | -29.184 | -176.832 | 69 | Kermadec Islands, New Zealand. |
| 5.3 | MB | 28/01/2012 | 19:37:30 | -4.211 | 140.730 | 96 | Irian Jaya, Indonesia. |
| 5.0 | MB | 29/01/2012 | 00:06:53 | 5.882 | 124.835 | 418 | Mindanao, Philippines. |
| 5.6 | MB | 30/01/2012 | 13:20:36 | 2.127 | 96.635 | 47 | Simeulue, Indonesia. |
| 5.1 | MB | 30/01/2012 | 18:25:20 | -2.874 | 129.490 | 46 | Seram, Indonesia. |
| 5.5 | MB | 31/01/2012 | 07:26:10 | -21.830 | -179.366 | 601 | Fiji Islands Region. |
| 5.2 | MB | 31/01/2012 | 19:58:02 | -14.207 | 167.347 | 174 | Vanuatu Islands. |
| 5.8 | MB | 01/02/2012 | 07:14:28 | -3.619 | 102.259 | 71 | Southern Sumatra, Indonesia. |
| 5.6 | MB | 01/02/2012 | 18:27:12 | -6.428 | 130.135 | 164 | Banda Sea. |
| 5.5 | MB | 02/02/2012 | 06:46:29 | -6.657 | 149.892 | 39 | New Britain Region, PNG. |
| 5.5 | MB | 02/02/2012 | 09:32:15 | -6.657 | 149.888 | 38 | New Britain Region, PNG. |
| 7.3 | Mwp | 02/02/2012 | 13:34:40 | -17.749 | 167.243 | 22 | Vanuatu Islands. |
| 5.1 | MB | 02/02/2012 | 13:48:25 | -17.706 | 167.405 | 96 | Vanuatu Islands. |
| 5.2 | MB | 02/02/2012 | 13:57:10 | -17.399 | 167.409 | 47 | Vanuatu Islands. |
| 5.6 | MB | 02/02/2012 | 17:27:11 | -17.953 | 167.372 | 68 | Vanuatu Islands. |
| 5.1 | MB | 02/02/2012 | 17:28:49 | -17.563 | 167.305 | 30 | Vanuatu Islands. |
| 5.0 | MB | 02/02/2012 | 18:00:31 | -40.241 | 173.691 | 198 | Cook Strait, New Zealand. |
| 6.4 | Mwp | 03/02/2012 | 03:46:27 | -17.397 | 167.331 | 50 | Vanuatu Islands. |
| 5.5 | MB | 03/02/2012 | 04:33:12 | -17.483 | 167.397 | 66 | Vanuatu Islands. |
| 5.8 | MB | 03/02/2012 | 06:01:42 | -17.370 | 167.350 | 40 | Vanuatu Islands. |
| 5.0 | MB | 03/02/2012 | 10:58:41 | 1.701 | 127.801 | 200 | Halmahera, Indonesia. |
| 5.0 | MB | 03/02/2012 | 14:25:37 | -17.859 | 167.788 | 82 | Vanuatu Islands. |
| 5.1 | MB | 03/02/2012 | 17:40:04 | -0.331 | 99.137 | 79 | Southern Sumatra, Indonesia. |
| 5.8 | Mwp | 04/02/2012 | 07:40:10 | -20.456 | -173.949 | 0 | Tonga Islands. |
| 5.2 | MB | 04/02/2012 | 17:45:34 | -19.038 | 167.907 | 63 | Vanuatu Islands. |
| 5.2 | MB | 05/02/2012 | 00:05:54 | -5.450 | 134.017 | 12 | Aru Islands Region, Indonesia. |
| 6.0 | Mwp | 05/02/2012 | 00:15:40 | -18.959 | 169.002 | 160 | Vanuatu Islands. |
| 6.3 | Mwp | 05/02/2012 | 16:40:40 | -17.927 | 167.336 | 12 | Vanuatu Islands. |
| 5.2 | MB | 05/02/2012 | 16:46:44 | 5.771 | 123.957 | 507 | Mindanao, Philippines. |
| 7.0 | Mwp | 06/02/2012 | 03:49:18 | 9.931 | 123.278 | 63 | Cebu, Philippines. |
| 6.3 | Mwp | 06/02/2012 | 10:10:29 | 9.719 | 123.217 | 94 | Cebu, Philippines. |
| 6.1 | Mwp | 06/02/2012 | 11:33:44 | 9.581 | 123.069 | 69 | Cebu, Philippines. |
| 5.4 | MB | 06/02/2012 | 11:40:29 | 9.711 | 123.140 | 141 | Negros, Philippines. |
| 5.5 | MB | 08/02/2012 | 01:58:36 | -29.254 | -176.189 | 52 | Kermadec Islands Region, New Zealand. |
| 5.1 | MB | 09/02/2012 | 02:39:42 | -3.354 | 130.759 | 46 | Seram, Indonesia. |
| 5.1 | MB | 09/02/2012 | 05:09:12 | -3.422 | 130.811 | 52 | Seram, Indonesia. |
| 5.2 | MB | 09/02/2012 | 05:18:47 | -15.260 | 167.658 | 122 | Vanuatu Islands. |
| 5.5 | MB | 09/02/2012 | 05:59:03 | -7.840 | 155.979 | 26 | Solomon Islands. |
| 5.6 | MB | 09/02/2012 | 18:52:49 | -58.161 | 158.306 | 22 | Macquarie Island Region. |
| 5.2 | MB | 09/02/2012 | 19:20:49 | -59.717 | 159.833 | 12 | Macquarie Island Region. |
| 5.9 | Mwp | 10/02/2012 | 01:47:33 | -17.894 | -178.321 | 580 | Fiji Islands Region. |
| 5.6 | Mwp | 10/02/2012 | 05:34:02 | -0.368 | 132.805 | 3 | Irian Jaya Region, Indonesia |
| 5.2 | MB | 10/02/2012 | 15:49:48 | -29.679 | -176.933 | 59 | Kermadec Islands. |
| 5.4 | MB | 11/02/2012 | 05:10:44 | -20.854 | -176.172 | 96 | Tonga Islands. |
| 5.7 | Mwp | 11/02/2012 | 07:06:51 | -1.050 | 126.852 | 39 | Southern Molucca Sea. |
| 5.1 | MB | 11/02/2012 | 15:23:10 | -9.984 | 160.319 | 72 | Solomon Islands Region. |
| 5.1 | MB | 13/02/2012 | 09:09:06 | -18.054 | -172.046 | 0 | Tonga Islands Region. |
| 5.1 | MB | 13/02/2012 | 10:12:04 | -32.955 | -177.803 | 59 | Kermadec Islands Region, New Zealand. |
| 5.0 | MB | 13/02/2012 | 14:47:02 | -6.363 | 124.372 | 19 | Banda Sea, Sunda Arc. |
| 6.5 | Mwp | 14/02/2012 | 08:20:01 | -10.421 | 161.243 | 110 | Solomon Islands. |
| 5.3 | MB | 15/02/2012 | 01:49:37 | -19.246 | -177.080 | 337 | Fiji Islands Region. |
| 5.7 | MB | 15/02/2012 | 14:07:56 | -7.626 | 146.948 | 41 | Eastern New Guinea Reg, PNG. |
| 5.1 | MB | 15/02/2012 | 18:31:54 | 6.212 | 92.820 | 18 | Nicobar Islands, India Region. |
| 5.0 | MB | 16/02/2012 | 11:12:40 | -8.468 | 118.341 | 6 | Sumbawa Region, Indonesia. |
| 5.1 | MB | 17/02/2012 | 01:57:34 | -19.538 | 169.286 | 99 | Vanuatu Islands. |
| 5.1 | MB | 17/02/2012 | 11:26:59 | -14.933 | 166.816 | 34 | Vanuatu Islands. |
| 5.4 | MB | 18/02/2012 | 04:50:21 | -5.249 | 151.508 | 71 | New Britain Region, PNG. |
| 5.7 | MB | 18/02/2012 | 19:15:17 | -20.704 | -173.569 | 0 | Tonga Islands. |
| 5.2 | MB | 18/02/2012 | 22:58:49 | 2.465 | 126.963 | 67 | Northern Molucca Sea, Indonesia. |
| 5.3 | MB | 19/02/2012 | 23:33:08 | -17.727 | -178.448 | 575 | Fiji Islands Region. |
| 5.2 | MB | 20/02/2012 | 02:28:17 | 1.894 | 99.683 | 190 | Northern Sumatra, Indonesia. |
| 5.2 | MB | 20/02/2012 | 20:59:07 | -4.780 | 151.518 | 185 | New Britain Region, PNG. |
| 5.2 | MB | 21/02/2012 | 04:36:24 | -19.098 | -177.363 | 574 | Fiji Islands Region. |
| 5.3 | MB | 21/02/2012 | 09:10:34 | -16.148 | -173.874 | 95 | Tonga Islands. |
| 5.1 | MB | 22/02/2012 | 00:50:44 | -17.686 | 167.211 | 76 | Vanuatu Islands. |
| 5.8 | Mwp | 22/02/2012 | 10:45:23 | -5.253 | 133.588 | 19 | Aru Islands Region, Indonesia. |
| 5.3 | MB | 22/02/2012 | 12:02:45 | 4.937 | 94.320 | 39 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.0 | MB | 23/02/2012 | 12:01:15 | -17.833 | -178.530 | 577 | Fiji Islands Region. |
| 5.4 | MB | 23/02/2012 | 13:25:30 | -5.158 | 151.936 | 69 | New Britain Region, PNG. |
| 5.2 | MB | 23/02/2012 | 18:26:15 | -28.817 | -175.816 | 0 | Kermadec Islands Region. |
| 5.0 | MB | 24/02/2012 | 14:13:05 | -4.935 | 102.878 | 62 | Southern Sumatra, Indonesia. |
| 5.3 | MB | 25/02/2012 | 05:06:21 | -25.502 | 179.796 | 517 | South of Fiji Islands. |
| 5.6 | MB | 26/02/2012 | 05:21:23 | -24.649 | -176.857 | 8 | South of Fiji Islands. |
| 5.2 | MB | 26/02/2012 | 13:49:56 | -24.533 | -176.923 | 0 | South of Fiji Islands. |
| 5.2 | MB | 26/02/2012 | 16:27:14 | -17.990 | -178.411 | 593 | Fiji Islands Region. |
| 5.3 | MB | 27/02/2012 | 03:01:50 | -24.729 | -176.994 | 0 | South of Fiji Islands. |
| 5.3 | MB | 27/02/2012 | 14:54:46 | -24.479 | -176.869 | 27 | South of Fiji Islands. |
| 5.3 | MB | 28/02/2012 | 23:52:33 | -7.482 | 128.207 | 159 | Banda Sea. |
| 5.3 | MB | 29/02/2012 | 07:05:11 | -12.481 | 166.514 | 63 | Santa Cruz Islands. |
| 5.6 | MB | 29/02/2012 | 16:03:59 | -17.646 | 168.012 | 57 | Vanuatu Islands. |
| 5.2 | MB | 29/02/2012 | 21:59:31 | -17.852 | 168.237 | 0 | Vanuatu Islands Region. |
| 5.3 | MB | 01/03/2012 | 00:59:54 | -17.631 | 168.050 | 0 | Vanuatu Islands Region. |
| 5.0 | MB | 02/03/2012 | 04:28:31 | -7.992 | 146.528 | 11 | Eastern New Guinea Region. |
| 5.3 | MB | 02/03/2012 | 05:09:54 | -8.129 | 146.250 | 0 | Eastern New Guinea Region. |
| 5.1 | MB | 03/03/2012 | 04:48:39 | 9.488 | 126.618 | 74 | Mindanao, Philippines. |
| 6.7 | Mwp | 03/03/2012 | 12:19:58 | -22.164 | 170.404 | 30 | SE of Loyalty Islands, New Caledonia. |
| 5.7 | MB | 04/03/2012 | 00:51:38 | -17.720 | -178.511 | 556 | Fiji Islands Region. |
| 5.5 | MB | 04/03/2012 | 12:49:07 | -21.512 | 169.909 | 50 | SE of Loyalty Islands. |
| 5.1 | MB | 04/03/2012 | 16:05:10 | -22.208 | -174.273 | 0 | Tonga Islands Region. |
| 5.3 | MB | 04/03/2012 | 23:17:05 | -4.399 | 102.509 | 62 | Southern Sumatra, Indonesia. |
| 5.3 | MB | 05/03/2012 | 06:55:31 | 4.273 | 97.219 | 43 | Northern Sumatra, Indonesia. |
| 5.4 | MB | 05/03/2012 | 08:12:32 | 1.983 | 128.765 | 38 | Halmahera, Indonesia. |
| 5.1 | MB | 05/03/2012 | 12:45:44 | -7.115 | 129.303 | 169 | Banda Sea. |
| 5.6 | MB | 06/03/2012 | 02:52:44 | 8.447 | 93.798 | 38 | Nicobar Islands. |
| 5.0 | MB | 06/03/2012 | 11:36:23 | -0.793 | 100.114 | 105 | Southern Sumatra, Indonesia. |
| 5.4 | MB | 07/03/2012 | 07:19:43 | -19.078 | -175.768 | 234 | Tonga Islands. |
| 6.0 | Mwp | 07/03/2012 | 12:00:44 | -2.823 | 139.048 | 43 | Near North Coast of Irian Jaya, Indonesia. |
| 5.2 | MB | 08/03/2012 | 03:51:33 | -17.870 | 167.583 | 32 | Vanuatu Islands. |
| 6.3 | MB | 09/03/2012 | 07:09:55 | -19.054 | 169.739 | 45 | Vanuatu Islands. |
| 5.0 | MB | 10/03/2012 | 16:01:02 | -4.306 | 104.941 | 33 | Southern Sumatra, Indonesia. |
| 5.0 | MB | 10/03/2012 | 16:08:37 | -6.947 | 129.502 | 206 | Banda Sea. |
| 5.0 | MB | 11/03/2012 | 16:08:07 | -17.030 | 168.674 | 273 | Vanuatu Islands. |
| 5.1 | MB | 13/03/2012 | 18:23:43 | -23.656 | -179.823 | 550 | South of Fiji Islands. |
| 5.1 | MB | 13/03/2012 | 23:16:25 | -17.496 | 167.132 | 88 | Vanuatu Islands. |
| 5.1 | MB | 14/03/2012 | 19:07:44 | -45.197 | 166.654 | 9 | Off West Coast of South Island, NZ. |
| 6.0 | MB | 14/03/2012 | 21:13:13 | -5.767 | 151.109 | 67 | New Britain region, PNG. |
| 5.0 | MB | 15/03/2012 | 10:28:57 | -7.419 | 126.603 | 318 | Banda Sea. |
| 5.2 | MB | 15/03/2012 | 23:45:04 | -20.692 | -178.132 | 592 | Fiji Islands Region. |
| 5.2 | MB | 16/03/2012 | 17:18:34 | -22.468 | 171.325 | 87 | SE of Loyalty Islands. |
| 5.2 | MB | 17/03/2012 | 01:38:06 | -7.043 | 123.283 | 627 | Banda Sea. |
| 5.6 | MB | 17/03/2012 | 01:49:28 | -7.023 | 123.360 | 622 | Banda Sea. |
| 5.0 | MB | 19/03/2012 | 05:04:55 | -0.995 | 119.860 | 4 | Minahassa, Indonesia. |
| 5.9 | MB | 19/03/2012 | 13:42:35 | -4.410 | 143.870 | 109 | New Guinea, PNG. |
| 5.4 | MB | 19/03/2012 | 23:46:30 | -14.416 | -175.203 | 0 | Samoa Islands Region. |
| 5.2 | MB | 20/03/2012 | 15:05:58 | -19.644 | 169.438 | 92 | Vanuatu Islands. |
| 6.3 | Mwp | 20/03/2012 | 17:56:20 | -3.922 | 140.296 | 79 | Irian Jaya, Indonesia. |
| 5.3 | MB | 21/03/2012 | 18:54:48 | -7.745 | 127.182 | 146 | Banda Sea. |
| 6.7 | Mwp | 21/03/2012 | 22:15:07 | -6.301 | 146.078 | 123 | Northwest of Lae, PNG. |
| 5.2 | MB | 22/03/2012 | 00:21:35 | 3.474 | 126.096 | 104 | Talaud Islands, Indonesia. |
| 5.2 | MB | 22/03/2012 | 14:35:10 | -21.293 | -173.744 | 0 | Tonga Islands. |
| 5.7 | ML | 23/03/2012 | 09:25:19 | -26.124 | 132.124 | 0 | Near Ernabella, SA (MW 5.4). |
| 5.1 | MB | 23/03/2012 | 14:35:41 | -16.520 | -174.495 | 189 | Tonga Islands. |
| 5.1 | MB | 23/03/2012 | 15:04:06 | -17.510 | 167.745 | 66 | Vanuatu Islands. |
| 5.0 | MB | 25/03/2012 | 06:43:45 | -19.946 | -176.239 | 220 | Fiji Islands Region. |
| 5.4 | MB | 25/03/2012 | 13:59:21 | -18.451 | -178.666 | 496 | Fiji Islands Region. |
| 5.0 | MB | 25/03/2012 | 14:18:43 | -21.441 | -178.537 | 557 | Fiji Islands Region. |
| 5.2 | MB | 28/03/2012 | 08:54:12 | -4.455 | 101.069 | 0 | Southwest of Sumatra, Indonesia. |
| 5.1 | MB | 30/03/2012 | 08:55:58 | -14.843 | 167.499 | 200 | Vanuatu Islands. |
| 5.2 | MB | 30/03/2012 | 09:38:58 | -21.051 | -178.691 | 591 | Fiji Islands Region. |
| 5.1 | MB | 30/03/2012 | 19:51:08 | -6.683 | 154.600 | 73 | Offshore Solomon Islands. |
| 5.3 | MB | 30/03/2012 | 22:02:11 | 4.656 | 95.107 | 58 | Northern Sumatra, Indonesia. |
| 5.2 | MB | 30/03/2012 | 23:05:16 | -5.865 | 147.738 | 46 | Eastern New Guinea Region, PNG. |
| 5.2 | MB | 31/03/2012 | 09:04:40 | -23.441 | 179.500 | 552 | South of Fiji Islands. |
| 5.3 | MB | 31/03/2012 | 17:06:45 | -20.172 | -175.095 | 108 | Tonga Islands. |
| 5.7 | MB | 01/04/2012 | 21:44:01 | -4.497 | 153.637 | 102 | New Ireland Region, PNG. |
| 5.3 | MB | 02/04/2012 | 08:17:01 | -16.827 | -173.561 | 0 | Tonga Islands. |
| 5.2 | MB | 02/04/2012 | 19:29:51 | -17.647 | -178.611 | 558 | Fiji Islands Region. |
| 5.0 | MB | 02/04/2012 | 21:14:48 | -7.009 | 149.658 | 68 | New Britain Region, PNG. |
| 5.0 | MB | 03/04/2012 | 10:42:33 | -4.313 | 153.676 | 110 | New Ireland Region, PNG. |
| 5.1 | MB | 03/04/2012 | 11:27:00 | -7.069 | 125.324 | 490 | Banda Sea. |
| 5.8 | MB | 03/04/2012 | 23:42:16 | -5.667 | 133.986 | 0 | Aru Islands Region, Indonesia. |
| 5.2 | MB | 04/04/2012 | 20:27:27 | -3.892 | 152.282 | 16 | New Ireland Region, PNG. |
| 6.4 | Mwp | 06/04/2012 | 16:15:56 | -4.543 | 153.532 | 96 | New Ireland Region, PNG. |
| 5.0 | MB | 06/04/2012 | 17:49:36 | -6.512 | 131.120 | 50 | Tanimbar Islands Region, Indonesia. |
| 5.2 | MB | 06/04/2012 | 19:24:12 | -3.329 | 100.351 | 3 | Southern Sumatra, Indonesia. |
| 5.9 | MB | 07/04/2012 | 11:58:06 | -6.809 | 149.633 | 29 | New Britain Region, PNG. |
| 5.1 | MB | 07/04/2012 | 20:09:39 | -6.017 | 130.770 | 73 | Banda Sea. |
| 8.6 | Mwp | 11/04/2012 | 08:38:32 | 2.277 | 92.981 | 0 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.9 | MB | 11/04/2012 | 09:27:55 | 1.202 | 91.679 | 4 | North Indian Ocean. |
| 5.5 | MB | 11/04/2012 | 09:51:39 | 2.606 | 90.324 | 10 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.3 | MB | 11/04/2012 | 10:01:17 | 2.597 | 90.305 | 2 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.2 | MB | 11/04/2012 | 10:08:29 | 2.695 | 90.083 | 13 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.3 | MB | 11/04/2012 | 10:21:13 | 2.711 | 92.442 | 0 | Off W Coast of Northern Sumatra, Indonesia. |
| 8.2 | Mwp | 11/04/2012 | 10:43:11 | 0.801 | 92.411 | 30 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.4 | MB | 11/04/2012 | 11:34:02 | 1.386 | 92.929 | 0 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.2 | MB | 11/04/2012 | 12:10:53 | 1.438 | 92.632 | 15 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.1 | MB | 11/04/2012 | 13:12:03 | -18.356 | -175.281 | 232 | Tonga Islands. |
| 5.7 | MB | 11/04/2012 | 13:42:44 | 2.198 | 93.583 | 37 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.4 | MB | 11/04/2012 | 13:58:05 | 1.493 | 90.747 | 6 | North Indian Ocean. |
| 5.0 | MB | 11/04/2012 | 14:08:40 | 1.115 | 92.053 | 0 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.3 | MB | 11/04/2012 | 14:34:21 | 1.153 | 90.515 | 28 | North Indian Ocean. |
| 5.4 | MB | 11/04/2012 | 14:54:27 | 1.333 | 91.749 | 0 | North Indian Ocean. |
| 5.1 | MB | 11/04/2012 | 15:46:50 | 2.932 | 92.109 | 0 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.5 | MB | 11/04/2012 | 16:04:22 | 3.348 | 92.764 | 6 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.1 | MB | 11/04/2012 | 16:21:49 | -10.590 | 113.712 | 34 | S of Java, Indonesia. |
| 5.3 | MB | 11/04/2012 | 16:58:09 | 2.662 | 90.198 | 7 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.0 | MB | 11/04/2012 | 18:15:39 | 1.842 | 93.955 | 13 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.6 | MB | 11/04/2012 | 18:54:49 | 2.543 | 92.617 | 48 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.3 | MB | 11/04/2012 | 19:04:22 | 1.167 | 92.172 | 27 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.3 | MB | 11/04/2012 | 21:36:07 | 1.943 | 90.916 | 0 | North Indian Ocean. |
| 5.1 | MB | 11/04/2012 | 22:15:24 | 0.654 | 92.412 | 0 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.1 | MB | 12/04/2012 | 11:01:59 | -6.066 | 103.369 | 15 | Southwest of Sumatra, Indonesia. |
| 5.4 | MB | 12/04/2012 | 13:09:50 | 2.500 | 93.403 | 47 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.2 | MB | 12/04/2012 | 14:46:32 | 3.423 | 94.003 | 64 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.0 | MB | 12/04/2012 | 15:01:15 | -7.629 | 106.932 | 26 | Java, Indonesia. |
| 5.1 | MB | 12/04/2012 | 15:07:35 | 3.661 | 92.664 | 28 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.2 | MB | 12/04/2012 | 20:21:51 | 3.698 | 92.655 | 0 | Off W Coast of Northern Sumatra. |
| 5.1 | MB | 12/04/2012 | 21:42:19 | 1.118 | 92.803 | 0 | Off W Coast of Northern Sumatra. |
| 5.0 | MB | 13/04/2012 | 19:35:09 | -4.504 | 102.593 | 62 | Southern Sumatra, Indonesia. |
| 5.1 | MB | 14/04/2012 | 12:18:24 | 2.554 | 90.312 | 0 | Off W Coast of Northern Sumatra. |
| 5.4 | MB | 14/04/2012 | 15:21:53 | 0.290 | 92.106 | 0 | Off W Coast of Northern Sumatra. |
| 5.9 | MB | 14/04/2012 | 19:26:41 | -6.795 | 105.345 | 51 | Sunda Strait, Indonesia. |
| 5.4 | MB | 14/04/2012 | 19:31:47 | -6.975 | 105.064 | 46 | Sunda Strait, Indonesia. |
| 6.1 | MB | 14/04/2012 | 22:05:31 | -18.971 | 168.949 | 57 | Vanuatu Islands. |
| 6.2 | MB | 15/04/2012 | 05:57:37 | 2.537 | 90.192 | 0 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.8 | MB | 16/04/2012 | 02:17:52 | -2.690 | 121.912 | 26 | Sulawesi, Indonesia. |
| 5.2 | MB | 16/04/2012 | 09:46:26 | 0.841 | 92.338 | 12 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.1 | MB | 16/04/2012 | 16:05:52 | 0.257 | 92.203 | 0 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.5 | MB | 16/04/2012 | 18:01:16 | -2.612 | 121.920 | 21 | Sulawesi, Indonesia. |
| 5.1 | MB | 16/04/2012 | 20:16:38 | -19.012 | 168.910 | 49 | Vanuatu Islands Region. |
| 5.2 | MB | 17/04/2012 | 03:24:24 | 5.330 | 94.482 | 54 | Northern Sumatra, Indonesia. |
| 6.5 | MB | 17/04/2012 | 07:13:50 | -5.509 | 147.142 | 208 | Eastern New Guinea Region, PNG. |
| 5.6 | MB | 17/04/2012 | 08:51:28 | -31.495 | -176.954 | 27 | Kermadec Islands Region. |
| 5.4 | MB | 17/04/2012 | 19:45:41 | 2.832 | 92.565 | 38 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.4 | MB | 18/04/2012 | 01:18:36 | -13.265 | 167.170 | 194 | Vanuatu Islands. |
| 5.3 | MB | 18/04/2012 | 18:38:06 | -0.603 | 131.941 | 0 | Irian Jaya Region, Indonesia. |
| 5.6 | MB | 18/04/2012 | 19:52:35 | -0.547 | 132.005 | 10 | Irian Jaya Region, Indonesia. |
| 5.0 | MB | 19/04/2012 | 15:46:54 | -1.907 | 134.387 | 31 | Irian Jaya Region, Indonesia. |
| 5.3 | MB | 19/04/2012 | 17:23:41 | 1.149 | 126.579 | 59 | Northern Molucca Sea. |
| 5.1 | MB | 20/04/2012 | 18:02:01 | -58.179 | 159.708 | 0 | Macquarie Island Region. |
| 5.2 | MB | 20/04/2012 | 19:34:11 | 3.368 | 93.967 | 55 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.4 | MB | 20/04/2012 | 22:19:44 | 3.261 | 93.798 | 8 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.7 | MB | 20/04/2012 | 22:29:59 | 3.235 | 93.803 | 27 | Off W Coast of Northern Sumatra, Indonesia. |
| 6.3 | Mwp | 20/04/2012 | 23:14:33 | 2.189 | 93.365 | 44 | Off W Coast of Northern Sumatra, Indonesia. |
| 6.3 | MB | 21/04/2012 | 01:16:55 | -1.807 | 134.371 | 29 | Near North coast of Papua, Indonesia. |
| 5.7 | MB | 21/04/2012 | 01:25:15 | -1.741 | 134.217 | 28 | Near North coast of Papua, Indonesia. |
| 5.4 | MB | 21/04/2012 | 12:36:10 | -21.792 | 170.419 | 42 | Southeast of Loyalty Islands. |
| 5.1 | MB | 21/04/2012 | 14:11:22 | -19.016 | 168.945 | 43 | Vanuatu Islands. |
| 5.2 | MB | 21/04/2012 | 23:27:34 | -17.444 | -178.742 | 535 | Fiji Islands Region. |
| 5.2 | MB | 22/04/2012 | 03:17:28 | 1.516 | 91.638 | 0 | North Indian Ocean. |
| 5.0 | MB | 22/04/2012 | 09:17:03 | -32.666 | -178.790 | 198 | South of Kermadec Islands. |
| 5.2 | MB | 22/04/2012 | 11:44:39 | -9.406 | 158.466 | 53 | Solomon Islands. |
| 5.2 | MB | 22/04/2012 | 12:20:54 | -52.546 | 140.349 | 74 | West of Macquarie Island. |
| 5.2 | MB | 22/04/2012 | 13:11:25 | -52.918 | 140.113 | 64 | West of Macquarie Island. |
| 5.0 | MB | 22/04/2012 | 15:37:33 | -6.490 | 131.846 | 47 | Tanimbar Islands Region, Indonesia. |
| 5.1 | MB | 22/04/2012 | 23:05:12 | 7.798 | 94.116 | 64 | Nicobar Islands, India Region. |
| 5.1 | MB | 23/04/2012 | 02:14:14 | -23.730 | -175.491 | 0 | Tonga Islands Region. |
| 5.0 | MB | 23/04/2012 | 03:08:06 | -17.754 | -174.959 | 249 | Tonga Islands Region. |
| 5.0 | MB | 23/04/2012 | 07:21:49 | -23.935 | -175.413 | 3 | Tonga Islands Region. |
| 5.1 | MB | 23/04/2012 | 10:57:15 | -23.744 | -175.564 | 0 | Tonga Islands Region. |
| 5.3 | MB | 23/04/2012 | 14:54:54 | -7.365 | 129.215 | 117 | Banda Sea. |
| 5.5 | MB | 23/04/2012 | 17:36:17 | -28.517 | -176.960 | 92 | Kermadec Islands Region. |
| 5.7 | MB | 23/04/2012 | 21:21:47 | 0.307 | 125.468 | 71 | Northern Molucca Sea. |
| 5.4 | MB | 24/04/2012 | 14:57:17 | 9.146 | 93.739 | 53 | Nicobar Islands, India Region. |
| 5.5 | MB | 24/04/2012 | 15:15:39 | -23.976 | -175.478 | 31 | Tonga Islands Region. |
| 5.2 | MB | 24/04/2012 | 21:37:33 | -17.643 | -178.537 | 550 | Fiji Islands Region. |
| 5.4 | MB | 25/04/2012 | 03:23:18 | -19.844 | -175.322 | 125 | Tonga Islands. |
| 5.8 | MB | 25/04/2012 | 07:42:24 | 8.979 | 94.051 | 14 | Nicobar Islands, India Region. |
| 5.1 | MB | 25/04/2012 | 07:53:28 | 9.043 | 94.130 | 76 | Nicobar Islands, India Region. |
| 5.3 | MB | 25/04/2012 | 16:31:00 | 8.941 | 93.868 | 22 | Nicobar Islands, India Region. |
| 5.2 | MB | 25/04/2012 | 20:17:57 | -4.199 | 151.660 | 279 | New Britain Region, PNG. |
| 5.2 | MB | 25/04/2012 | 23:43:12 | -23.537 | -175.195 | 86 | Tonga Islands Region. |
| 5.0 | MB | 26/04/2012 | 06:38:30 | -18.562 | -177.446 | 610 | Fiji Islands Region. |
| 5.5 | MB | 26/04/2012 | 19:21:42 | 2.689 | 94.480 | 3 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.3 | MB | 27/04/2012 | 10:29:42 | -2.635 | 121.940 | 35 | Sulawesi, Indonesia. |
| 6.5 | MB | 28/04/2012 | 10:08:04 | -18.610 | -174.649 | 91 | Tonga Islands. |
| 5.4 | MB | 28/04/2012 | 18:00:37 | -3.294 | 100.484 | 30 | Southern Sumatra, Indonesia. |
| 5.3 | MB | 28/04/2012 | 19:21:04 | -5.357 | 152.270 | 52 | New Britain Region, PNG. |
| 5.4 | MB | 29/04/2012 | 03:00:47 | 1.987 | 93.718 | 35 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.4 | MB | 29/04/2012 | 03:52:43 | 1.702 | 93.654 | 82 | Off W Coast of Northern Sumatra, Indonesia. |
| 6.0 | Mwp | 29/04/2012 | 08:09:05 | 2.792 | 94.510 | 19 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.1 | MB | 29/04/2012 | 22:35:25 | -29.975 | -173.499 | 0 | Kermadec Islands Region. |
| 5.1 | MB | 30/04/2012 | 16:56:39 | -8.304 | 105.391 | 0 | South of Java, Indonesia. |
| 5.3 | MB | 30/04/2012 | 18:11:44 | -5.736 | 128.464 | 359 | Banda Sea. |
| 5.1 | MB | 30/04/2012 | 19:12:52 | 3.622 | 127.541 | 67 | Talaud Islands, Indonesia. |
| 5.4 | MB | 30/04/2012 | 19:15:35 | -20.767 | -175.376 | 121 | Tonga Islands. |
| 5.2 | MB | 30/04/2012 | 20:17:45 | -29.610 | -176.049 | 46 | Kermadec Islands Region. |
| 5.2 | MB | 30/04/2012 | 21:40:37 | 3.348 | 92.906 | 0 | Off West Coast of Northern Sumatra. |
| 5.3 | MB | 01/05/2012 | 13:59:43 | -2.607 | 122.035 | 28 | Sulawesi, Indonesia. |
| 5.0 | MB | 02/05/2012 | 09:22:29 | -5.499 | 152.378 | 53 | New Britain Region, PNG. |
| 5.9 | MB | 02/05/2012 | 12:17:53 | -54.989 | 144.180 | 0 | West of Macquarie Island. |
| 5.2 | MB | 03/05/2012 | 11:10:42 | -23.677 | 179.163 | 548 | South of Fiji Islands. |
| 5.2 | MB | 03/05/2012 | 16:14:19 | -5.498 | 152.174 | 55 | New Britain Region, PNG. |
| 5.0 | MB | 04/05/2012 | 02:31:33 | -5.119 | 151.648 | 70 | New Britain Region, PNG. |
| 5.4 | MB | 04/05/2012 | 10:40:55 | 9.037 | 126.972 | 82 | Mindanao, Philippines. |
| 5.3 | MB | 05/05/2012 | 18:43:19 | 5.705 | 126.559 | 99 | Mindanao, Philippines. |
| 5.5 | MB | 05/05/2012 | 20:23:28 | -21.252 | -173.846 | 0 | Tonga Islands. |
| 5.0 | MB | 07/05/2012 | 03:27:22 | -25.276 | 179.847 | 496 | South of Fiji Islands. |
| 5.0 | MB | 07/05/2012 | 17:49:38 | -16.962 | -177.118 | 397 | Fiji Islands Region. |
| 5.4 | MB | 08/05/2012 | 00:33:00 | -14.387 | 167.173 | 199 | Vanuatu Islands. |
| 5.5 | MB | 09/05/2012 | 04:35:02 | 6.281 | 127.634 | 66 | Philippine Islands Region. |
| 5.2 | MB | 10/05/2012 | 07:31:31 | 0.112 | 122.331 | 171 | Minahassa Peninsula, Sulawesi. |
| 5.2 | MB | 10/05/2012 | 19:11:28 | 1.383 | 126.434 | 62 | Northern Molucca Sea. |
| 5.1 | MB | 11/05/2012 | 01:31:58 | -6.025 | 155.105 | 203 | Solomon Islands. |
| 5.6 | MB | 12/05/2012 | 13:23:18 | 2.191 | 126.106 | 86 | Northern Molucca Sea. |
| 5.2 | MB | 12/05/2012 | 14:34:45 | -23.306 | -175.765 | 15 | Tonga Islands Region. |
| 5.1 | MB | 12/05/2012 | 20:39:31 | -3.388 | 136.017 | 17 | Irian Jaya, Indonesia. |
| 5.6 | MB | 13/05/2012 | 02:00:15 | 4.399 | 92.458 | 27 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.3 | MB | 13/05/2012 | 04:42:49 | -58.377 | 158.132 | 20 | Macquarie Island Region, New Zealand. |
| 5.2 | MB | 13/05/2012 | 04:46:05 | -0.173 | 123.007 | 95 | Sulawesi, Indonesia. |
| 5.2 | MB | 13/05/2012 | 18:24:49 | -3.816 | 128.722 | 123 | Seram, Indonesia. |
| 5.1 | MB | 14/05/2012 | 19:28:48 | 1.267 | 126.449 | 61 | Northern Molucca Sea. |
| 5.2 | MB | 15/05/2012 | 19:22:20 | -24.219 | -175.438 | 0 | South of Tonga Islands. |
| 5.8 | MB | 16/05/2012 | 00:59:34 | -5.571 | 149.729 | 158 | New Britain Region, PNG. |
| 5.2 | MB | 16/05/2012 | 01:28:20 | -24.113 | -175.612 | 49 | South of Tonga Islands. |
| 5.0 | MB | 17/05/2012 | 08:05:20 | -17.778 | -178.343 | 593 | Fiji Islands Region. |
| 5.2 | MB | 17/05/2012 | 21:58:59 | 3.432 | 92.872 | 30 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.0 | MB | 17/05/2012 | 23:50:09 | -2.635 | 122.006 | 46 | Sulawesi, Indonesia. |
| 5.1 | MB | 20/05/2012 | 05:29:48 | -6.528 | 148.897 | 55 | New Britain Region, PNG. |
| 5.0 | MB | 20/05/2012 | 21:15:22 | -18.878 | 169.581 | 88 | Vanuatu Islands. |
| 5.2 | MB | 20/05/2012 | 22:15:46 | -3.235 | 143.889 | 24 | Near N Coast of New Guinea, PNG. |
| 5.2 | MB | 22/05/2012 | 02:38:50 | 2.455 | 126.942 | 59 | Northern Molucca Sea. |
| 5.5 | MB | 23/05/2012 | 23:00:02 | -49.878 | 139.310 | 56 | Antarctic Ridge, Southern Ocean. |
| 5.8 | MB | 23/05/2012 | 23:19:17 | -50.293 | 139.580 | 0 | Antarctic Ridge, Southern Ocean. |
| 5.3 | MB | 25/05/2012 | 02:44:55 | -43.278 | 172.974 | 38 | Christchurch, New Zealand. |
| 5.0 | MB | 25/05/2012 | 12:10:28 | -30.221 | -177.533 | 40 | Kermadec Islands. |
| 5.3 | MB | 26/05/2012 | 00:51:31 | -20.816 | 168.740 | 57 | Loyalty Islands. |
| 5.2 | MB | 26/05/2012 | 17:18:00 | -4.785 | 102.941 | 48 | Southern Sumatra, Indonesia. |
| 5.2 | MB | 27/05/2012 | 03:50:20 | -2.840 | 99.974 | 31 | Southern Sumatra, Indonesia. |
| 5.1 | MB | 28/05/2012 | 13:15:06 | -0.039 | 123.515 | 178 | Minahassa Peninsula, Sulawesi. |
| 5.0 | MB | 28/05/2012 | 16:44:21 | 3.706 | 118.607 | 6 | Celebes Sea. |
| 5.8 | MB | 29/05/2012 | 03:33:23 | -22.007 | -179.311 | 582 | South of Fiji Islands. |
| 5.0 | MB | 30/05/2012 | 06:57:07 | -6.272 | 130.288 | 154 | Banda Sea. |
| 5.6 | MB | 31/05/2012 | 23:01:02 | -0.908 | 133.285 | 40 | Irian Jaya Region, Indonesia. |
| 5.0 | MB | 01/06/2012 | 06:18:52 | 2.575 | 126.211 | 104 | Northern Molucca Sea. |
| 5.7 | MB | 01/06/2012 | 06:56:23 | -0.882 | 133.299 | 49 | Irian Jaya Region, Indonesia. |
| 5.3 | MB | 01/06/2012 | 13:31:43 | -20.146 | -177.617 | 520 | Fiji Islands Region. |
| 5.1 | MB | 03/06/2012 | 14:51:27 | -21.855 | -179.207 | 589 | Fiji Islands Region. |
| 5.8 | MB | 04/06/2012 | 11:18:14 | -7.703 | 106.332 | 62 | Java, Indonesia. |
| 5.2 | MB | 04/06/2012 | 14:18:51 | -15.953 | -177.468 | 13 | Fiji Islands Region. |
| 5.1 | MB | 04/06/2012 | 16:39:58 | -0.663 | 127.857 | 0 | Halmahera, Indonesia. |
| 5.1 | MB | 06/06/2012 | 02:24:46 | -10.676 | 161.562 | 33 | Solomon Islands. |
| 5.2 | MB | 06/06/2012 | 04:17:30 | -0.943 | 126.719 | 40 | Southern Molucca Sea. |
| 5.0 | MB | 07/06/2012 | 00:02:24 | -22.715 | -174.354 | 0 | Tonga Islands Region. |
| 5.2 | MB | 07/06/2012 | 01:10:34 | 1.468 | 126.657 | 71 | Northern Molucca Sea. |
| 5.6 | MB | 07/06/2012 | 09:01:51 | -20.123 | -176.170 | 275 | Fiji Islands Region. |
| 5.1 | MB | 07/06/2012 | 21:10:09 | -30.075 | -177.206 | 28 | Kermadec Islands. |
| 5.2 | MB | 09/06/2012 | 08:49:32 | -3.014 | 127.787 | 80 | Ceram Sea. |
| 5.2 | MB | 11/06/2012 | 13:31:47 | 6.691 | 123.732 | 621 | Mindanao, Philippines. |
| 5.2 | MB | 11/06/2012 | 20:56:07 | -5.828 | 148.071 | 42 | New Britain Region, PNG. |
| 5.5 | MB | 12/06/2012 | 05:59:38 | -5.616 | 105.513 | 167 | Sunda Strait, Indonesia. |
| 5.6 | MB | 12/06/2012 | 13:48:36 | -31.408 | -178.122 | 61 | Kermadec Islands Region. |
| 5.2 | MB | 14/06/2012 | 11:04:04 | -22.123 | 179.898 | 580 | South of Fiji Islands. |
| 5.6 | MB | 14/06/2012 | 20:17:27 | 1.184 | 126.853 | 97 | Northern Molucca Sea. |
| 6.0 | MB | 15/06/2012 | 01:14:11 | 5.634 | 126.612 | 74 | Mindanao, Philippines. |
| 5.6 | MB | 15/06/2012 | 08:43:27 | -3.385 | 146.215 | 0 | Bismarck Sea, PNG. |
| 5.8 | MB | 17/06/2012 | 03:43:18 | -8.501 | 160.530 | 136 | Solomon Islands. |
| 5.6 | MB | 17/06/2012 | 17:26:23 | -20.493 | -173.925 | 0 | Tonga Islands. |
| 5.2 | MB | 18/06/2012 | 12:50:42 | 1.970 | 129.338 | 38 | Halmahera, Indonesia. |
| 5.0 | MB | 18/06/2012 | 18:13:54 | -0.042 | 123.421 | 185 | Sulawesi, Indonesia. |
| 5.2 | MB | 19/06/2012 | 04:27:56 | -2.927 | 100.856 | 53 | Southern Sumatra, Indonesia. |
| 5.2 | MB | 19/06/2012 | 04:37:33 | -20.448 | -178.460 | 597 | Fiji Islands Region. |
| 5.4 | ML | 19/06/2012 | 10:53:29 | -38.304 | 146.200 | 10 | SW of Moe, Vic. |
| 5.7 | MB | 19/06/2012 | 21:13:32 | -3.090 | 99.573 | 0 | Southwest of Sumatra, Indonesia. |
| 5.6 | MB | 20/06/2012 | 00:41:51 | -0.960 | 133.137 | 61 | Irian Jaya, Indonesia. |
| 5.6 | MB | 20/06/2012 | 09:59:04 | -4.959 | 103.084 | 75 | Southern Sumatra, Indonesia. |
| 5.5 | MB | 20/06/2012 | 21:55:53 | 0.074 | 123.535 | 143 | Minahassa Peninsula, Indonesia. |
| 5.1 | MB | 20/06/2012 | 23:20:56 | -8.659 | 157.269 | 15 | Solomon Islands. |
| 5.1 | MB | 21/06/2012 | 05:18:41 | -25.589 | 179.954 | 501 | South of Fiji Islands. |
| 5.5 | MB | 21/06/2012 | 22:34:34 | -18.098 | -178.140 | 482 | Fiji Islands Region. |
| 5.4 | MB | 21/06/2012 | 23:53:02 | -32.844 | -178.720 | 74 | South of Kermadec Islands, NZ. |
| 5.1 | MB | 22/06/2012 | 00:03:25 | -33.512 | 179.867 | 148 | South of Kermadec Islands, NZ. |
| 5.3 | MB | 22/06/2012 | 02:20:05 | -33.014 | -178.232 | 0 | South of Kermadec Islands, NZ. |
| 6.1 | Mwp | 22/06/2012 | 04:31:29 | -54.119 | 158.780 | 113 | Macquarie Island Region. |
| 6.3 | Mwp | 23/06/2012 | 04:34:54 | 3.014 | 97.971 | 107 | Northern Sumatra Indonesia. |
| 5.0 | MB | 23/06/2012 | 18:37:19 | -21.138 | -179.164 | 601 | Fiji Islands Region. |
| 5.1 | MB | 24/06/2012 | 01:20:46 | -7.197 | 129.167 | 144 | Banda Sea. |
| 5.4 | MB | 25/06/2012 | 03:09:06 | -27.375 | -177.465 | 192 | Kermadec Islands Region. |
| 5.1 | MB | 25/06/2012 | 08:24:29 | -6.160 | 147.002 | 97 | Eastern New Guinea Region, PNG. |
| 5.5 | MB | 26/06/2012 | 12:32:30 | -21.662 | -176.578 | 134 | Fiji Islands Region. |
| 5.0 | MB | 27/06/2012 | 06:56:45 | -15.649 | -173.611 | 44 | Tonga Islands. |
| 5.3 | MB | 27/06/2012 | 12:56:59 | -8.362 | 119.922 | 33 | Flores Region, Indonesia. |
| 5.0 | MB | 28/06/2012 | 12:23:14 | -17.411 | -178.744 | 532 | Fiji Islands Region. |
| 5.8 | MB | 28/06/2012 | 15:52:03 | -13.475 | 166.654 | 34 | Vanuatu Islands. |
| 5.1 | MB | 29/06/2012 | 09:20:52 | 1.205 | 97.243 | 42 | Northern Sumatra, Indonesia. |
| 5.1 | MB | 29/06/2012 | 10:01:58 | -25.638 | -177.723 | 303 | South of Fiji Islands. |
| 5.1 | MB | 29/06/2012 | 22:37:07 | -33.358 | -177.792 | 53 | South of Kermadec Islands. |
| 5.4 | MB | 30/06/2012 | 16:24:36 | -22.016 | 171.129 | 82 | Southeast of Loyalty Islands. |
| 5.3 | MB | 01/07/2012 | 04:23:26 | -7.875 | 128.604 | 4 | Banda Sea. |
| 5.0 | MB | 01/07/2012 | 07:49:53 | 5.506 | 125.406 | 141 | Mindanao, Philippines. |
| 5.4 | MB | 01/07/2012 | 23:02:30 | -17.722 | -178.173 | 573 | Fiji Islands Region. |
| 5.1 | MB | 03/07/2012 | 02:05:02 | -23.886 | -175.330 | 1 | Tonga Islands Region. |
| 5.1 | MB | 03/07/2012 | 03:49:36 | -10.770 | 163.448 | 124 | Solomon Islands. |
| 6.2 | Mwp | 03/07/2012 | 10:36:13 | -40.063 | 174.017 | 217 | South Taranaki Bight, New Zealand. |
| 5.1 | MB | 04/07/2012 | 10:26:25 | -5.747 | 147.287 | 99 | Eastern New Guinea Region, PNG. |
| 5.2 | MB | 04/07/2012 | 14:36:34 | -30.764 | -177.836 | 34 | Kermadec Islands, New Zealand. |
| 5.7 | MB | 05/07/2012 | 08:14:19 | 5.101 | 126.658 | 60 | Mindanao, Philippines. |
| 6.2 | Mwp | 06/07/2012 | 02:28:26 | -14.693 | 167.394 | 202 | Vanuatu Islands. |
| 5.4 | MB | 06/07/2012 | 02:52:23 | -17.649 | -178.392 | 572 | Fiji Islands Region. |
| 5.2 | ML | 07/07/2012 | 00:50:27 | -39.080 | 175.820 | 91 | North Island, NZ. |
| 5.6 | MB | 07/07/2012 | 03:35:33 | -4.736 | 153.237 | 79 | New Ireland Region, PNG. |
| 5.0 | MB | 07/07/2012 | 04:34:55 | -6.337 | 103.781 | 24 | Southwest of Sumatra, Indonesia. |
| 5.0 | MB | 07/07/2012 | 13:38:18 | 9.695 | 125.969 | 52 | Mindanao, Philippines. |
| 5.2 | MB | 09/07/2012 | 12:25:45 | -0.204 | 122.614 | 79 | Sulawesi, Indonesia. |
| 5.1 | MB | 10/07/2012 | 02:28:10 | -24.406 | -176.114 | 12 | South of Fiji Islands. |
| 5.0 | MB | 10/07/2012 | 09:05:17 | -21.640 | -178.949 | 570 | Fiji Islands Region. |
| 5.3 | MB | 10/07/2012 | 10:27:00 | -1.855 | 134.282 | 38 | Irian Jaya Region, Indonesia. |
| 5.1 | MB | 10/07/2012 | 18:09:44 | -23.204 | -179.744 | 554 | South of Fiji Islands. |
| 5.5 | MB | 11/07/2012 | 00:16:13 | -8.473 | 156.522 | 42 | Solomon Islands. |
| 5.6 | MB | 11/07/2012 | 11:12:42 | -25.855 | -177.117 | 115 | South of Fiji Islands. |
| 5.0 | MB | 12/07/2012 | 01:22:05 | -50.987 | 119.224 | 63 | Western Indian Antarctic Ridge. |
| 5.6 | MB | 12/07/2012 | 04:11:14 | -10.675 | 166.116 | 112 | Santa Cruz Islands. |
| 5.3 | MB | 13/07/2012 | 04:50:40 | -17.956 | -178.396 | 593 | Fiji Islands Region. |
| 5.0 | MB | 13/07/2012 | 05:18:03 | 0.391 | 123.813 | 204 | Minahassa Peninsula, Indonesia. |
| 5.2 | MB | 13/07/2012 | 14:01:22 | -38.751 | 176.537 | 48 | North Island, New Zealand. |
| 5.2 | MB | 13/07/2012 | 17:57:05 | -8.256 | 108.959 | 60 | Java, Indonesia. |
| 5.8 | MB | 16/07/2012 | 16:33:11 | -1.348 | 137.097 | 19 | Near North Coast of Irian Jaya, New Guinea. |
| 5.6 | Mwp | 16/07/2012 | 16:33:12 | -1.465 | 137.005 | 10 | Near North Coast of Irian Jaya, New Guinea. |
| 5.4 | MB | 17/07/2012 | 12:38:49 | -31.287 | -179.206 | 9 | Kermadec Islands Region, New Zealand. |
| 5.2 | MB | 17/07/2012 | 16:56:46 | -7.385 | 128.687 | 186 | Banda Sea. |
| 5.2 | MB | 18/07/2012 | 04:33:59 | -4.520 | 102.678 | 65 | Southern Sumatra, Indonesia. |
| 5.8 | MB | 18/07/2012 | 08:09:43 | -20.751 | -178.266 | 498 | Fiji Islands Region. |
| 5.8 | MB | 20/07/2012 | 14:03:13 | -21.077 | -179.123 | 625 | Fiji Islands Region. |
| 5.0 | MB | 20/07/2012 | 17:45:04 | -8.593 | 123.858 | 40 | Flores Region, Indonesia. |
| 5.4 | MB | 20/07/2012 | 18:14:50 | -8.288 | 123.695 | 4 | Flores Region, Indonesia. |
| 5.1 | MB | 20/07/2012 | 23:22:53 | -22.917 | -176.846 | 121 | South of Fiji Islands. |
| 5.7 | MB | 21/07/2012 | 04:54:04 | -38.035 | -179.861 | 0 | E of North Island, NZ. |
| 5.6 | MB | 21/07/2012 | 06:12:04 | -19.268 | 173.826 | 49 | Vanuatu Islands Region. |
| 5.2 | MB | 21/07/2012 | 12:46:30 | -5.420 | 129.254 | 239 | Banda Sea. |
| 5.1 | MB | 21/07/2012 | 20:47:43 | -15.039 | -173.023 | 0 | Tonga Islands. |
| 5.2 | MB | 22/07/2012 | 05:39:14 | -4.198 | 153.628 | 50 | New Ireland Region, PNG. |
| 5.3 | MB | 22/07/2012 | 07:02:18 | -5.109 | 149.463 | 49 | New Britain Region, PNG. |
| 5.2 | MB | 22/07/2012 | 13:36:59 | -14.923 | 167.398 | 129 | Vanuatu Islands. |
| 5.3 | MB | 22/07/2012 | 16:56:00 | -21.672 | -174.722 | 0 | Tonga Islands. |
| 5.1 | MB | 22/07/2012 | 17:05:39 | -21.725 | -174.710 | 0 | Tonga Islands. |
| 5.3 | MB | 23/07/2012 | 00:22:04 | -2.744 | 135.432 | 0 | Irian Jaya Region, Indonesia. |
| 5.2 | MB | 23/07/2012 | 19:46:59 | -30.262 | -175.799 | 0 | Kermadec Islands, NZ. |
| 5.0 | MB | 24/07/2012 | 03:02:57 | 7.537 | 126.339 | 57 | Mindanao, Philippines. |
| 6.0 | MB | 25/07/2012 | 00:27:47 | 2.643 | 96.074 | 38 | Northern Sumatra, Indonesia. |
| 6.5 | Mwp | 25/07/2012 | 11:20:30 | -9.749 | 159.998 | 53 | Solomon Islands. |
| 5.2 | MB | 25/07/2012 | 13:27:51 | 5.592 | 126.471 | 123 | Mindanao, Philippines. |
| 5.1 | MB | 25/07/2012 | 19:15:37 | -0.351 | 124.478 | 64 | Southern Molucca Sea. |
| 5.1 | MB | 25/07/2012 | 21:43:29 | -9.711 | 159.963 | 36 | Solomon Islands. |
| 5.2 | MB | 26/07/2012 | 06:12:13 | -26.133 | -178.491 | 351 | South of Fiji Islands. |
| 5.2 | MB | 26/07/2012 | 10:20:01 | -20.706 | -174.087 | 0 | Tonga Islands Region. |
| 5.6 | MB | 26/07/2012 | 10:48:17 | -21.060 | 169.762 | 0 | Southeast of Loyalty Islands. |
| 5.2 | MB | 26/07/2012 | 23:58:37 | -4.288 | 152.369 | 175 | New Britain Region, PNG. |
| 5.0 | MB | 27/07/2012 | 13:25:29 | 0.387 | 100.049 | 164 | Northern Sumatra, Indonesia. |
| 6.2 | MB | 28/07/2012 | 20:04:01 | -4.742 | 153.173 | 75 | New Ireland Region, PNG. |
| 5.1 | MB | 29/07/2012 | 03:04:39 | 7.898 | 126.749 | 65 | Mindanao, Philippines. |
| 5.3 | MB | 29/07/2012 | 14:21:18 | -6.074 | 104.272 | 70 | Sunda Strait, Indonesia. |
| 5.3 | MB | 31/07/2012 | 02:20:12 | -20.640 | -175.398 | 50 | Tonga Islands. |
| 5.2 | MB | 31/07/2012 | 06:50:14 | -4.795 | 102.957 | 59 | Southern Sumatra, Indonesia. |
| 5.2 | MB | 01/08/2012 | 00:20:41 | 1.354 | 124.913 | 88 | Minahassa Sulawesi, Indonesia. |
| 5.2 | MB | 01/08/2012 | 09:24:55 | -36.268 | 179.107 | 90 | East of North Island, New Zealand. |
| 5.7 | MB | 01/08/2012 | 20:43:44 | -4.430 | 144.616 | 77 | Eastern New Guinea Region, PNG. |
| 5.5 | MB | 02/08/2012 | 03:15:30 | -6.337 | 155.004 | 67 | Solomon Islands. |
| 5.2 | MB | 02/08/2012 | 05:17:12 | -37.497 | 179.790 | 12 | Off E Coast of North Island, NZ. |
| 5.5 | MB | 02/08/2012 | 07:37:44 | -6.981 | 131.215 | 59 | Tanimbar Islands Region, Indonesia. |
| 6.0 | Mwp | 02/08/2012 | 09:56:42 | -4.700 | 153.433 | 61 | New Ireland Region, PNG. |
| 5.3 | MB | 02/08/2012 | 10:26:10 | -35.591 | -179.668 | 0 | E of North Island, New Zealand. |
| 5.4 | MB | 03/08/2012 | 02:52:15 | -16.661 | -173.910 | 86 | Tonga Islands. |
| 5.5 | MB | 03/08/2012 | 04:09:05 | -7.081 | 155.717 | 93 | Solomon Islands. |
| 5.4 | MB | 03/08/2012 | 11:59:01 | -11.476 | 165.148 | 27 | Santa Cruz Islands. |
| 5.1 | MB | 03/08/2012 | 13:42:06 | -35.806 | -178.786 | 6 | East of North Island, NZ. |
| 5.1 | MB | 04/08/2012 | 09:56:22 | -35.678 | -179.599 | 25 | East of North Island, NZ. |
| 5.4 | MB | 04/08/2012 | 11:24:16 | 4.873 | 96.492 | 39 | Northern Sumatra, Indonesia. |
| 5.2 | MB | 04/08/2012 | 12:54:31 | -35.807 | -179.913 | 51 | East of North Island, NZ. |
| 5.4 | MB | 05/08/2012 | 13:55:10 | -20.958 | -178.248 | 472 | Fiji Islands Region. |
| 5.1 | MB | 05/08/2012 | 22:40:47 | -16.557 | -173.623 | 47 | Tonga Islands. |
| 5.4 | MB | 06/08/2012 | 18:55:13 | 2.673 | 128.468 | 209 | Halmahera, Indonesia. |
| 5.0 | MB | 07/08/2012 | 23:48:02 | -17.507 | -178.633 | 554 | Fiji Islands Region. |
| 5.3 | MB | 08/08/2012 | 10:58:07 | -16.028 | 177.895 | 219 | Fiji Islands. |
| 5.1 | MB | 08/08/2012 | 16:14:05 | -6.417 | 146.703 | 107 | Eastern New Guinea Region. |
| 5.2 | MB | 09/08/2012 | 11:04:25 | -8.809 | 116.386 | 110 | Lombok Region, Indonesia. |
| 5.0 | MB | 09/08/2012 | 17:24:03 | -18.103 | -178.703 | 558 | Fiji Islands Region. |
| 5.0 | MB | 10/08/2012 | 00:33:09 | -14.135 | 167.413 | 214 | Vanuatu Islands. |
| 5.2 | MB | 10/08/2012 | 02:19:22 | 1.388 | 127.032 | 83 | Halmahera, Indonesia. |
| 5.4 | MB | 10/08/2012 | 04:41:45 | 1.930 | 96.996 | 42 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.2 | MB | 10/08/2012 | 07:17:46 | 7.575 | 126.983 | 65 | Mindanao, Philippines. |
| 5.3 | MB | 15/08/2012 | 07:54:37 | -16.852 | -173.962 | 93 | Tonga Islands. |
| 5.2 | MB | 16/08/2012 | 06:35:40 | -18.339 | -177.781 | 561 | Fiji Islands Region. |
| 5.1 | MB | 17/08/2012 | 08:47:37 | -6.889 | 154.502 | 52 | Solomon Islands. |
| 5.0 | MB | 17/08/2012 | 16:09:34 | -6.999 | 154.326 | 41 | Solomon Islands. |
| 6.5 | Mwp | 18/08/2012 | 09:41:53 | -1.446 | 120.102 | 7 | Sulawesi, Indonesia. |
| 5.8 | MB | 18/08/2012 | 10:23:27 | -13.359 | 172.484 | 0 | Vanuatu Islands Region. |
| 6.4 | Mwp | 18/08/2012 | 15:31:45 | 2.540 | 128.778 | 47 | Halmahera, Indonesia. |
| 6.1 | Mwp | 18/08/2012 | 17:44:58 | -15.565 | -172.653 | 0 | Samoa Islands Region. |
| 5.5 | MB | 19/08/2012 | 01:57:05 | 4.977 | 94.706 | 56 | Off W Coast of Northern Sumatra, Indonesia. |
| 6.1 | MB | 19/08/2012 | 22:41:52 | -4.947 | 144.623 | 91 | Near N Coast of New Guinea, PNG. |
| 5.5 | MB | 21/08/2012 | 03:09:39 | -4.722 | 103.003 | 66 | Southern Sumatra, Indonesia. |
| 5.6 | MB | 21/08/2012 | 17:39:36 | 0.005 | 91.984 | 0 | Off W Coast of Sumatra, Indonesia. |
| 5.3 | MB | 21/08/2012 | 17:51:37 | 0.326 | 91.981 | 0 | Off W Coast of Sumatra, Indonesia. |
| 5.4 | MB | 22/08/2012 | 10:13:51 | -19.873 | -173.496 | 0 | Tonga Islands. |
| 5.9 | MB | 22/08/2012 | 11:38:20 | -6.174 | 147.262 | 83 | Eastern New Guinea Region, PNG. |
| 5.1 | MB | 23/08/2012 | 17:21:36 | -7.265 | 119.385 | 413 | Flores Sea, Indonesia. |
| 5.4 | MB | 24/08/2012 | 04:43:45 | -26.802 | -176.413 | 0 | S of Fiji Islands. |
| 5.8 | MB | 24/08/2012 | 10:01:56 | -33.241 | -179.196 | 58 | S of Kermadec Islands, NZ. |
| 5.1 | MB | 26/08/2012 | 06:37:04 | -19.652 | -174.136 | 58 | Tonga Islands. |
| 6.4 | MB | 26/08/2012 | 15:05:38 | 2.100 | 126.881 | 94 | Northern Molucca Sea. |
| 5.3 | MB | 27/08/2012 | 03:47:20 | 2.041 | 126.915 | 102 | Northern Molucca Sea. |
| 5.2 | MB | 27/08/2012 | 09:01:22 | 2.250 | 99.070 | 137 | Northern Sumatra, Indonesia. |
| 5.1 | MB | 27/08/2012 | 16:00:50 | -26.793 | -176.025 | 79 | Kermadec Islands Region. |
| 5.1 | MB | 27/08/2012 | 17:54:20 | 3.591 | 126.896 | 0 | Talaud Islands, Indonesia. |
| 5.6 | MB | 28/08/2012 | 15:51:05 | 4.452 | 92.959 | 6 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.4 | MB | 29/08/2012 | 09:49:13 | 3.396 | 122.723 | 552 | Celebes Sea. |
| 5.1 | MB | 29/08/2012 | 13:34:32 | -17.607 | 168.442 | 115 | Vanuatu Islands. |
| 5.3 | MB | 30/08/2012 | 06:56:39 | -3.557 | 130.928 | 0 | Seram, Indonesia. |
| 5.1 | MB | 30/08/2012 | 07:13:25 | -3.553 | 130.880 | 5 | Seram, Indonesia. |
| 5.1 | MB | 30/08/2012 | 10:53:59 | -50.204 | 114.190 | 9 | Western Indian-Antarctic Ridge. |
| 5.2 | MB | 30/08/2012 | 12:18:53 | -49.943 | 114.118 | 93 | Western Indian-Antarctic Ridge. |
| 5.1 | MB | 30/08/2012 | 19:00:08 | -8.086 | 128.657 | 40 | Timor Sea. |
| 5.2 | MB | 31/08/2012 | 18:21:36 | -4.968 | 151.919 | 102 | New Britain Region, PNG. |
| 5.3 | MB | 01/09/2012 | 19:46:02 | -21.460 | -173.910 | 11 | Tonga Islands. |
| 5.6 | MB | 03/09/2012 | 06:49:56 | 6.494 | 123.886 | 56 | Mindanao, Philippines. |
| 5.1 | MB | 03/09/2012 | 14:26:40 | -10.923 | 113.848 | 0 | South of Java, Indonesia. |
| 6.4 | MB | 03/09/2012 | 18:23:03 | -10.818 | 113.880 | 0 | South of Java, Indonesia. |
| 5.7 | MB | 03/09/2012 | 19:44:27 | 7.791 | 125.086 | 39 | Mindanao, Philippines. |
| 5.1 | MB | 04/09/2012 | 04:27:13 | -10.884 | 113.912 | 13 | S of Java, Indonesia. |
| 5.2 | MB | 04/09/2012 | 12:07:25 | -10.907 | 113.748 | 0 | South of Java, Indonesia. |
| 5.1 | MB | 04/09/2012 | 12:49:09 | -10.905 | 113.596 | 48 | South of Java, Indonesia. |
| 5.4 | MB | 04/09/2012 | 13:31:01 | -10.952 | 113.771 | 0 | South of Java, Indonesia. |
| 5.1 | MB | 05/09/2012 | 00:48:02 | -10.966 | 113.707 | 5 | S of Java, Indonesia. |
| 6.1 | MB | 05/09/2012 | 13:09:15 | -12.561 | 166.452 | 52 | Santa Cruz Islands. |
| 5.0 | MB | 05/09/2012 | 22:48:23 | -17.430 | -177.438 | 433 | Fiji Islands Region. |
| 5.4 | MB | 06/09/2012 | 02:22:18 | 4.395 | 126.883 | 60 | Talaud Islands, Indonesia. |
| 5.5 | MB | 06/09/2012 | 06:46:34 | -3.881 | 140.369 | 40 | Irian Jaya, Indonesia. |
| 5.7 | MB | 06/09/2012 | 14:16:16 | -21.876 | -176.007 | 98 | Tonga Islands Region. |
| 5.2 | MB | 07/09/2012 | 12:30:31 | 2.417 | 128.261 | 158 | Halmahera, Indonesia. |
| 5.2 | MB | 08/09/2012 | 04:11:46 | -9.759 | 160.621 | 40 | Solomon Islands. |
| 5.0 | MB | 08/09/2012 | 04:29:01 | -9.441 | 125.873 | 0 | Timor Region. |
| 6.2 | MB | 08/09/2012 | 10:51:45 | -3.261 | 135.154 | 27 | Irian Jaya Region, Indonesia. |
| 5.3 | MB | 08/09/2012 | 16:27:43 | 2.430 | 126.871 | 84 | Northern Molucca Sea. |
| 5.2 | MB | 09/09/2012 | 14:36:40 | -30.487 | -177.749 | 82 | Kermadec Islands, New Zealand. |
| 5.1 | MB | 09/09/2012 | 16:49:20 | -3.257 | 135.090 | 40 | Irian Jaya Region, Indonesia. |
| 5.1 | MB | 10/09/2012 | 07:51:12 | -6.885 | 129.609 | 155 | Banda Sea. |
| 5.1 | MB | 10/09/2012 | 09:46:24 | -15.390 | -171.552 | 0 | Samoa Islands Region. |
| 5.2 | MB | 10/09/2012 | 23:14:31 | 0.546 | 98.441 | 63 | Northern Sumatra, Indonesia. |
| 5.4 | MB | 11/09/2012 | 14:21:39 | -10.772 | 113.729 | 32 | South of Java, Indonesia. |
| 5.4 | MB | 12/09/2012 | 04:28:13 | -5.066 | 152.405 | 66 | New Britain Region, PNG. |
| 5.5 | MB | 12/09/2012 | 11:27:52 | -10.028 | 161.260 | 110 | Solomon Islands. |
| 5.4 | MB | 13/09/2012 | 01:54:59 | -18.663 | 175.837 | 55 | Fiji Islands Region. |
| 5.3 | MB | 13/09/2012 | 23:40:02 | -10.853 | 113.845 | 10 | South of Java, Indonesia. |
| 6.2 | MB | 14/09/2012 | 04:51:44 | -3.226 | 100.535 | 0 | Southern Sumatra, Indonesia. |
| 5.8 | MB | 15/09/2012 | 16:32:21 | -10.881 | 113.795 | 6 | South of Java, Indonesia. |
| 5.3 | MB | 15/09/2012 | 17:35:40 | -10.988 | 113.801 | 0 | South of Java, Indonesia. |
| 5.4 | MB | 16/09/2012 | 13:02:51 | -18.316 | -174.532 | 95 | Tonga Islands. |
| 5.5 | MB | 17/09/2012 | 20:07:56 | -5.744 | 150.166 | 102 | New Britain Region, PNG. |
| 5.2 | MB | 17/09/2012 | 21:15:45 | -14.850 | 167.575 | 154 | Vanuatu Islands. |
| 5.1 | MB | 18/09/2012 | 01:46:47 | 1.359 | 125.985 | 74 | Northern Molucca Sea, Indonesia. |
| 5.1 | MB | 18/09/2012 | 03:23:45 | -6.327 | 103.504 | 45 | Southwest of Sumatra, Indonesia. |
| 5.2 | MB | 18/09/2012 | 08:05:39 | 4.423 | 126.336 | 36 | Talaud Islands, Indonesia. |
| 5.2 | MB | 18/09/2012 | 15:59:38 | 3.730 | 92.632 | 12 | Off W Coast of Northern Sumatra. |
| 5.1 | MB | 19/09/2012 | 00:32:12 | 1.298 | 127.170 | 144 | Halmahera, Indonesia. |
| 5.5 | MB | 20/09/2012 | 05:40:13 | -20.428 | -175.988 | 158 | Tonga Islands. |
| 5.0 | MB | 20/09/2012 | 15:52:36 | 2.104 | 126.919 | 103 | Northern Molucca Sea. |
| 5.3 | MB | 20/09/2012 | 20:47:45 | 0.030 | 98.923 | 47 | Northern Sumatra, Indonesia. |
| 5.2 | MB | 20/09/2012 | 21:03:40 | -20.477 | -178.379 | 538 | Fiji Islands Region. |
| 5.7 | MB | 20/09/2012 | 21:21:05 | 5.959 | 126.461 | 107 | Mindanao, Philippines. |
| 5.3 | MB | 21/09/2012 | 03:25:48 | -25.601 | 178.711 | 546 | South of Fiji Islands. |
| 5.3 | MB | 24/09/2012 | 01:18:11 | -19.874 | -176.653 | 101 | Fiji Islands Region. |
| 5.6 | MB | 24/09/2012 | 10:31:20 | -21.001 | -174.007 | 0 | Tonga Islands. |
| 5.9 | MB | 25/09/2012 | 03:06:51 | -15.468 | -173.738 | 107 | Tonga Islands. |
| 5.1 | MB | 25/09/2012 | 05:37:58 | -25.484 | 178.471 | 612 | South of Fiji Islands. |
| 5.0 | MB | 25/09/2012 | 10:41:27 | 9.405 | 126.884 | 39 | Mindanao, Philippines. |
| 5.3 | MB | 26/09/2012 | 01:41:36 | -21.054 | -174.075 | 0 | Tonga Islands. |
| 5.0 | MB | 26/09/2012 | 05:24:55 | -31.904 | -177.874 | 143 | Kermadec Islands Region. |
| 5.5 | MB | 26/09/2012 | 06:45:13 | -18.759 | -175.409 | 112 | Tonga Islands. |
| 5.4 | MB | 26/09/2012 | 17:15:51 | -25.660 | -177.119 | 90 | South of Fiji Islands. |
| 6.2 | Mwp | 27/09/2012 | 23:54:00 | -8.969 | 157.044 | 60 | Solomon Islands. |
| 5.1 | MB | 28/09/2012 | 02:08:27 | -18.115 | -172.685 | 65 | Tonga Islands Region. |
| 5.2 | MB | 28/09/2012 | 23:22:32 | -8.878 | 157.649 | 22 | Solomon Islands. |
| 5.3 | MB | 29/09/2012 | 11:24:31 | 6.180 | 92.764 | 14 | Nicobar Islands, India Region. |
| 5.2 | MB | 29/09/2012 | 16:25:42 | -7.024 | 124.131 | 594 | Banda Sea. |
| 5.7 | MB | 02/10/2012 | 03:33:28 | -18.796 | -174.817 | 74 | Tonga Islands. |
| 5.4 | MB | 02/10/2012 | 06:25:34 | 8.179 | 123.245 | 63 | Mindanao, Philippines. |
| 5.3 | MB | 02/10/2012 | 11:35:45 | -20.873 | -174.166 | 0 | Tonga Islands. |
| 5.0 | MB | 02/10/2012 | 17:37:22 | -2.873 | 101.926 | 110 | Southern Sumatra, Indonesia. |
| 5.2 | MB | 03/10/2012 | 06:26:16 | -6.260 | 152.824 | 29 | New Britain Region, PNG. |
| 5.7 | MB | 03/10/2012 | 13:32:38 | -0.390 | 97.687 | 21 | Southwest of Sumatra, Indonesia. |
| 5.3 | MB | 03/10/2012 | 18:28:32 | 5.823 | 127.025 | 140 | Philippine Islands Region. |
| 5.1 | MB | 04/10/2012 | 06:38:45 | -8.356 | 123.540 | 0 | Flores Region, Indonesia. |
| 5.0 | MB | 05/10/2012 | 11:19:27 | -22.774 | -175.890 | 40 | Tonga Islands Region. |
| 5.1 | MB | 05/10/2012 | 15:44:06 | 0.035 | 123.683 | 107 | Minahassa Peninsula, Sulawesi. |
| 5.2 | MB | 06/10/2012 | 07:56:26 | -25.162 | -177.345 | 137 | South of Fiji Islands. |
| 5.2 | MB | 07/10/2012 | 08:36:35 | -5.664 | 151.920 | 71 | New Britain Region, PNG. |
| 6.3 | MB | 08/10/2012 | 11:43:30 | -4.575 | 129.168 | 0 | Banda Sea. |
| 5.2 | MB | 08/10/2012 | 12:20:51 | -4.529 | 129.214 | 19 | Banda Sea. |
| 5.1 | MB | 09/10/2012 | 07:14:55 | -2.665 | 139.296 | 13 | Near N Coast of Irian Jaya. |
| 5.6 | MB | 09/10/2012 | 07:49:49 | -2.743 | 139.206 | 0 | Near N Coast of Irian Jaya. |
| 6.6 | Mwp | 09/10/2012 | 12:32:05 | -61.041 | 154.372 | 20 | Balleny Islands Region. |
| 5.0 | MB | 11/10/2012 | 05:47:15 | 4.431 | 125.848 | 192 | Talaud Islands, Indonesia. |
| 5.3 | MB | 11/10/2012 | 17:07:49 | 1.471 | 92.531 | 0 | Off W Coast of Northern Sumatra. |
| 6.8 | Mwp | 12/10/2012 | 00:31:27 | -4.910 | 134.060 | 9 | Irian Jaya Region, Indonesia. |
| 5.3 | MB | 12/10/2012 | 01:34:47 | -4.855 | 133.911 | 0 | Irian Jaya Region Indonesia. |
| 5.3 | MB | 13/10/2012 | 10:03:57 | -11.057 | 163.156 | 43 | Solomon Islands Region. |
| 5.4 | MB | 13/10/2012 | 22:36:31 | -16.621 | -174.011 | 56 | Tonga Islands. |
| 5.3 | MB | 13/10/2012 | 23:07:55 | -2.548 | 142.300 | 19 | Near N Coast of New Guinea, PNG. |
| 5.1 | MB | 14/10/2012 | 03:42:24 | 4.436 | 125.134 | 36 | Talaud Islands, Indonesia. |
| 5.9 | MB | 14/10/2012 | 04:58:10 | -7.189 | 156.126 | 95 | Solomon Islands. |
| 5.1 | MB | 14/10/2012 | 06:16:50 | -2.590 | 142.067 | 36 | Near N Coast of New Guinea, PNG. |
| 5.0 | MB | 14/10/2012 | 13:18:55 | -10.196 | 161.625 | 90 | Solomon Islands. |
| 5.1 | MB | 15/10/2012 | 15:55:01 | 5.977 | 125.259 | 87 | Mindanao, Philippines. |
| 5.3 | MB | 15/10/2012 | 20:19:00 | -20.292 | -178.053 | 538 | Fiji Islands Region. |
| 5.4 | MB | 16/10/2012 | 23:43:05 | -38.475 | 176.052 | 193 | North Island, New Zealand. |
| 5.7 | MB | 17/10/2012 | 04:42:31 | 4.182 | 124.640 | 344 | Celebes Sea. |
| 5.0 | MB | 17/10/2012 | 05:49:26 | -4.979 | 151.679 | 181 | New Britain Region, PNG. |
| 5.2 | MB | 17/10/2012 | 15:58:56 | -1.252 | 126.809 | 37 | Southern Molucca Sea. |
| 5.6 | MB | 17/10/2012 | 17:23:36 | -18.893 | -173.973 | 0 | Tonga Islands. |
| 5.5 | MB | 18/10/2012 | 01:27:19 | -54.214 | 144.106 | 34 | West of Macquarie Island. |
| 5.3 | MB | 18/10/2012 | 04:14:32 | -8.073 | 123.722 | 35 | Flores Region, Indonesia. |
| 5.1 | MB | 18/10/2012 | 09:00:03 | -6.502 | 97.885 | 0 | SW of Sumatra, Indonesia. |
| 5.0 | MB | 19/10/2012 | 01:19:55 | -4.906 | 152.048 | 184 | New Britain, PNG. |
| 5.6 | Mwp | 19/10/2012 | 15:34:15 | -30.292 | -176.674 | 78 | Kermadec Islands Region. |
| 5.1 | MB | 19/10/2012 | 17:22:12 | -21.189 | 176.509 | 90 | South of Fiji Islands. |
| 5.1 | MB | 20/10/2012 | 16:29:47 | -3.155 | 100.958 | 0 | Southern Sumatra, Indonesia. |
| 6.6 | Mwp | 20/10/2012 | 23:00:34 | -13.543 | 166.695 | 55 | Vanuatu Islands. |
| 5.0 | MB | 21/10/2012 | 17:51:36 | -24.179 | -174.222 | 0 | South of Tonga Islands. |
| 5.0 | MB | 23/10/2012 | 04:36:09 | -24.681 | -179.958 | 522 | S of Fiji Islands. |
| 5.2 | MB | 23/10/2012 | 09:33:26 | -57.466 | 147.992 | 47 | Macquarie Island Region. |
| 6.0 | Mwp | 23/10/2012 | 09:39:27 | -22.159 | 171.914 | 96 | Southeast of Loyalty Islands. |
| 5.2 | MB | 23/10/2012 | 18:00:55 | -7.244 | 147.603 | 104 | Eastern New Guinea, PNG. |
| 5.1 | MB | 25/10/2012 | 09:26:00 | -17.990 | -178.211 | 634 | Fiji Islands Region. |
| 5.2 | MB | 25/10/2012 | 10:09:43 | 0.640 | 124.381 | 160 | Minahassa Peninsula, Sulawesi. |
| 5.6 | MB | 25/10/2012 | 18:40:38 | -19.741 | -177.989 | 596 | Fiji Islands Region. |
| 5.2 | MB | 25/10/2012 | 18:57:02 | -4.065 | 140.325 | 4 | Irian Jaya, Indonesia. |
| 5.0 | MB | 26/10/2012 | 04:24:30 | -17.976 | -178.295 | 610 | Fiji Islands Region. |
| 5.0 | MB | 26/10/2012 | 06:28:44 | -0.422 | 133.015 | 18 | Irian Jaya Region, Indonesia. |
| 5.3 | MB | 27/10/2012 | 20:13:02 | -6.179 | 151.660 | 31 | New Britain Region, PNG. |
| 5.4 | MB | 27/10/2012 | 20:15:03 | -21.334 | 169.471 | 33 | Vanuatu Islands. |
| 5.0 | MB | 28/10/2012 | 01:43:30 | 9.824 | 127.074 | 119 | Philippine Islands Region. |
| 5.4 | MB | 29/10/2012 | 02:22:47 | 1.105 | 98.619 | 78 | Northern Sumatra, Indonesia. |
| 5.4 | MB | 29/10/2012 | 06:28:33 | -8.284 | 123.503 | 8 | Flores Region, Indonesia. |
| 5.7 | MB | 29/10/2012 | 15:29:52 | -0.165 | 125.365 | 48 | Southern Molucca Sea. |
| 5.1 | MB | 29/10/2012 | 19:49:05 | -0.374 | 133.007 | 27 | Irian Jaya Region, Indonesia. |
| 5.1 | MB | 29/10/2012 | 21:20:32 | -4.793 | 153.266 | 55 | New Ireland Region, PNG. |
| 5.7 | MB | 29/10/2012 | 21:38:41 | -15.569 | -172.358 | 0 | Samoa Islands Region. |
| 5.2 | MB | 30/10/2012 | 07:12:29 | -4.619 | 152.017 | 161 | New Britain Region, PNG. |
| 5.5 | MB | 30/10/2012 | 07:12:36 | -17.870 | -178.421 | 572 | Fiji Islands Region. |
| 5.4 | MB | 30/10/2012 | 10:59:29 | -18.139 | -174.288 | 0 | Tonga Islands. |
| 5.2 | MB | 01/11/2012 | 03:31:25 | -5.644 | 129.107 | 251 | Banda Sea. |
| 5.2 | MB | 01/11/2012 | 09:48:02 | -20.149 | -177.540 | 475 | Fiji Islands Region. |
| 5.3 | MB | 01/11/2012 | 13:26:12 | -5.324 | 145.643 | 99 | Eastern New Guinea Region, PNG. |
| 5.7 | Mwp | 01/11/2012 | 14:12:00 | -6.773 | 107.476 | 127 | Java, Indonesia. |
| 5.3 | MB | 01/11/2012 | 23:37:20 | 1.107 | 122.022 | 56 | Minahassa Peninsula, Indonesia. |
| 5.6 | MB | 02/11/2012 | 01:22:14 | -7.808 | 121.624 | 281 | Flores Sea. |
| 5.0 | MB | 02/11/2012 | 14:02:49 | 4.561 | 125.686 | 190 | Talaud Islands, Indonesia. |
| 6.2 | Mwp | 02/11/2012 | 18:17:37 | 9.157 | 126.470 | 73 | Mindanao, Philippines. |
| 5.2 | MB | 03/11/2012 | 20:49:16 | -21.764 | -176.205 | 154 | Fiji Islands Region. |
| 5.3 | MB | 04/11/2012 | 00:16:33 | -14.782 | 167.460 | 147 | Vanuatu Islands. |
| 5.1 | MB | 04/11/2012 | 12:13:01 | -22.284 | 171.416 | 85 | Southeast of Loyalty Islands. |
| 5.2 | MB | 04/11/2012 | 23:46:40 | 4.576 | 125.064 | 58 | Talaud Islands, Indonesia. |
| 5.1 | MB | 05/11/2012 | 00:30:01 | -0.145 | 125.396 | 52 | Southern Molucca Sea. |
| 5.2 | MB | 05/11/2012 | 00:55:39 | 4.512 | 124.947 | 31 | Talaud Islands, Indonesia. |
| 5.0 | MB | 05/11/2012 | 11:59:29 | 1.431 | 128.210 | 93 | Halmahera, Indonesia. |
| 5.1 | MB | 05/11/2012 | 18:43:44 | 3.566 | 125.856 | 73 | Talaud Islands, Indonesia. |
| 5.9 | MB | 06/11/2012 | 01:36:25 | 1.326 | 122.272 | 60 | Minahassa, Sulawesi, Indonesia. |
| 5.4 | MB | 06/11/2012 | 01:42:29 | 1.224 | 122.094 | 63 | Minahassa, Sulawesi, Indonesia. |
| 5.3 | MB | 06/11/2012 | 03:36:21 | -21.429 | -178.636 | 499 | Fiji Islands Region. |
| 5.2 | MB | 06/11/2012 | 14:20:38 | -3.085 | 149.366 | 25 | Bismarck Sea, PNG. |
| 5.8 | MB | 07/11/2012 | 23:42:18 | -8.633 | 148.119 | 113 | Eastern New Guinea Region, PNG. |
| 5.2 | MB | 08/11/2012 | 07:39:42 | -14.580 | 167.445 | 114 | Vanuatu Islands. |
| 5.2 | MB | 08/11/2012 | 17:38:10 | -49.556 | 117.257 | 36 | Western Indian-Antarctic Ridge. |
| 5.1 | MB | 08/11/2012 | 17:56:33 | -18.485 | -172.776 | 0 | Tonga Islands Region. |
| 5.5 | MB | 09/11/2012 | 05:51:41 | -42.605 | 124.537 | 1 | South of Australia, Indian Ocean. |
| 5.3 | MB | 09/11/2012 | 15:07:46 | -7.938 | 117.627 | 0 | Bali Sea. |
| 5.4 | MB | 09/11/2012 | 19:59:44 | 1.424 | 97.924 | 0 | Northern Sumatra, Indonesia. |
| 5.1 | MB | 10/11/2012 | 06:05:19 | -7.473 | 106.417 | 2 | Java, Indonesia. |
| 5.2 | MB | 10/11/2012 | 09:06:52 | -9.207 | 120.596 | 0 | Sumba Region, Indonesia. |
| 5.2 | MB | 11/11/2012 | 02:55:43 | -11.558 | 166.339 | 87 | Santa Cruz Islands. |
| 5.0 | MB | 11/11/2012 | 07:52:35 | -5.851 | 130.151 | 0 | Banda Sea. |
| 5.5 | MB | 11/11/2012 | 14:56:14 | -10.559 | 164.774 | 65 | Santa Cruz Islands Region. |
| 5.3 | MB | 11/11/2012 | 21:33:07 | -5.562 | 151.858 | 50 | New Britain Region, PNG. |
| 5.5 | MB | 13/11/2012 | 23:23:50 | -21.131 | -175.723 | 37 | Tonga Islands. |
| 5.4 | MB | 14/11/2012 | 05:21:48 | 9.961 | 122.508 | 105 | Negros, Philippines. |
| 5.3 | MB | 14/11/2012 | 07:41:50 | -20.553 | -173.695 | 0 | Tonga Islands. |
| 5.5 | MB | 14/11/2012 | 18:22:31 | -14.398 | 173.904 | 610 | Fiji Islands Region. |
| 5.3 | MB | 14/11/2012 | 23:41:55 | -5.729 | 151.557 | 59 | New Britain Region, PNG. |
| 5.5 | MB | 15/11/2012 | 08:21:52 | -3.220 | 148.441 | 42 | Bismarck Sea, PNG. |
| 5.1 | MB | 15/11/2012 | 21:55:08 | -16.874 | -174.843 | 196 | Tonga Islands. |
| 5.0 | MB | 16/11/2012 | 12:48:21 | -5.694 | 151.596 | 125 | New Britain Region, PNG. |
| 5.6 | MB | 17/11/2012 | 02:51:28 | -15.017 | 167.358 | 160 | Vanuatu Islands. |
| 6.1 | Mwp | 17/11/2012 | 05:12:55 | -18.241 | -172.326 | 0 | Tonga Islands Region. |
| 5.3 | MB | 17/11/2012 | 09:06:22 | -22.089 | 171.999 | 105 | Southeast of Royalty Islands. |
| 5.1 | MB | 17/11/2012 | 21:02:55 | -5.726 | 151.760 | 56 | New Britain Region, PNG. |
| 5.5 | MB | 18/11/2012 | 20:01:26 | 5.947 | 125.943 | 125 | Mindanao, Philippines. |
| 6.0 | MB | 19/11/2012 | 09:44:38 | -5.822 | 151.700 | 42 | New Britain Region, PNG. |
| 5.1 | MB | 19/11/2012 | 10:16:02 | -6.049 | 151.645 | 68 | New Britain Region, PNG. |
| 5.2 | MB | 19/11/2012 | 10:55:13 | -5.831 | 151.708 | 45 | New Britain Region, PNG. |
| 5.0 | MB | 19/11/2012 | 11:51:24 | -5.646 | 151.698 | 51 | New Britain Region, PNG. |
| 5.0 | MB | 20/11/2012 | 15:28:05 | 0.760 | 126.197 | 62 | Northern Molucca Sea. |
| 5.2 | MB | 20/11/2012 | 16:10:41 | -5.924 | 151.815 | 29 | New Britain Region, PNG. |
| 5.1 | MB | 21/11/2012 | 01:18:44 | -5.791 | 151.727 | 44 | New Britain Region, PNG. |
| 5.8 | MB | 21/11/2012 | 16:46:37 | -11.467 | 117.896 | 9 | South of Sumbawa, Indonesia. |
| 5.0 | MB | 22/11/2012 | 06:32:25 | 4.884 | 126.717 | 109 | Talaud Islands, Indonesia. |
| 5.0 | MB | 22/11/2012 | 09:09:31 | -9.364 | 114.964 | 49 | South of Bali, Indonesia. |
| 5.2 | MB | 22/11/2012 | 14:16:42 | -17.839 | -178.407 | 594 | Fiji Islands Region. |
| 5.2 | MB | 22/11/2012 | 18:01:05 | -4.525 | 102.825 | 0 | Southern Sumatra, Indonesia. |
| 5.0 | MB | 22/11/2012 | 21:29:20 | -11.490 | 117.926 | 8 | South of Sumbawa, Indonesia. |
| 5.2 | MB | 23/11/2012 | 00:40:23 | 1.619 | 126.085 | 97 | Northern Molucca Sea. |
| 5.3 | MB | 24/11/2012 | 20:28:50 | 6.437 | 123.493 | 617 | Mindanao, Philippines. |
| 5.1 | MB | 25/11/2012 | 07:14:50 | 5.747 | 126.565 | 37 | Mindanao, Philippines. |
| 5.2 | MB | 26/11/2012 | 07:23:43 | 9.900 | 126.651 | 54 | Mindanao, Philippines. |
| 5.1 | MB | 26/11/2012 | 07:58:25 | -20.781 | -179.124 | 604 | Fiji Islands Region. |
| 5.0 | MB | 26/11/2012 | 08:51:39 | -19.160 | -170.941 | 0 | Tonga Islands Region. |
| 5.5 | MB | 27/11/2012 | 02:59:09 | -3.030 | 129.318 | 32 | Seram, Indonesia. |
| 5.1 | MB | 27/11/2012 | 11:58:19 | -2.684 | 138.744 | 51 | Irian Jaya, Indonesia. |
| 5.1 | MB | 28/11/2012 | 01:42:15 | -1.927 | 138.616 | 9 | Near N Coast of Irian Jaya, Indonesia. |
| 5.8 | MB | 28/11/2012 | 08:56:15 | -5.409 | 131.146 | 44 | Banda Sea. |
| 5.3 | MB | 29/11/2012 | 00:30:30 | -20.981 | -178.529 | 537 | Fiji Islands Region. |
| 5.0 | MB | 29/11/2012 | 08:11:11 | -27.970 | -178.149 | 265 | Kermadec Islands Region. |
| 5.6 | MB | 29/11/2012 | 11:10:27 | -3.710 | 145.401 | 27 | Near North Coast of PNG. |
| 5.5 | MB | 30/11/2012 | 11:20:42 | -18.823 | -175.442 | 239 | Tonga Islands. |
| 6.0 | MB | 02/12/2012 | 00:54:26 | -16.971 | 167.666 | 61 | Vanuatu Islands. |
| 5.2 | MB | 02/12/2012 | 13:32:42 | -20.555 | -174.000 | 0 | Tonga Islands Region. |
| 5.2 | MB | 02/12/2012 | 21:00:50 | -21.531 | -179.277 | 600 | Fiji Islands Region. |
| 5.1 | MB | 04/12/2012 | 03:54:49 | -19.702 | -177.564 | 563 | Fiji Islands Region. |
| 5.3 | MB | 05/12/2012 | 23:08:36 | -20.602 | -178.788 | 617 | Fiji Islands Region. |
| 5.0 | MB | 06/12/2012 | 02:24:24 | 2.559 | 90.432 | 0 | Off W Coast of Northern Sumatra, Indonesia. |
| 5.1 | MB | 07/12/2012 | 02:25:46 | -5.284 | 151.609 | 109 | New Britain Region, PNG. |
| 6.1 | Mwp | 07/12/2012 | 18:19:05 | -38.343 | 176.266 | 136 | North Island, New Zealand. |
| 5.7 | MB | 07/12/2012 | 19:50:22 | -7.593 | 147.041 | 131 | Eastern New Guinea Region, PNG. |
| 5.2 | MB | 08/12/2012 | 08:11:56 | 2.512 | 128.345 | 156 | Halmahera, Indonesia. |
| 5.0 | MB | 08/12/2012 | 12:26:44 | -31.646 | -178.478 | 51 | Kermadec Islands Region. |
| 5.8 | MB | 08/12/2012 | 16:35:18 | -7.265 | 143.908 | 18 | Near South Coast of New Guinea, PNG. |
| 5.1 | MB | 08/12/2012 | 19:00:13 | -7.382 | 143.754 | 41 | Near South Coast of New Guinea, PNG. |
| 5.1 | MB | 08/12/2012 | 21:46:08 | -14.774 | 167.307 | 119 | Vanuatu Islands Region. |
| 5.2 | MB | 09/12/2012 | 02:50:46 | -3.427 | 145.356 | 2 | Near N Coast of New Guinea, PNG. |
| 5.3 | MB | 09/12/2012 | 18:30:07 | -3.669 | 141.093 | 48 | New Guinea, PNG. |
| 5.7 | MB | 09/12/2012 | 20:52:40 | -16.213 | -174.159 | 139 | Tonga Islands. |
| 5.9 | MB | 09/12/2012 | 21:45:38 | 6.543 | 126.236 | 98 | Mindanao, Philippines. |
| 7.1 | Mwp | 10/12/2012 | 16:53:09 | -6.634 | 129.823 | 151 | Banda Sea. |
| 5.4 | MB | 10/12/2012 | 17:34:35 | 3.610 | 127.044 | 62 | Talaud Islands, Indonesia. |
| 5.3 | MB | 10/12/2012 | 20:47:51 | -6.516 | 129.945 | 168 | Banda Sea. |
| 6.0 | MB | 11/12/2012 | 06:18:31 | 0.449 | 126.283 | 61 | Northern Molucca Sea. |
| 5.3 | MB | 11/12/2012 | 12:18:18 | -18.798 | 167.828 | 13 | Vanuatu Islands Region. |
| 5.1 | MB | 11/12/2012 | 23:16:37 | -18.795 | 167.455 | 153 | Vanuatu Islands. |
| 5.4 | MB | 13/12/2012 | 00:53:28 | -30.666 | -177.837 | 50 | Kermadec Islands, NZ. |
| 5.0 | MB | 14/12/2012 | 02:34:52 | -5.077 | 153.110 | 48 | New Ireland Region, PNG. |
| 5.4 | MB | 14/12/2012 | 16:52:36 | -15.145 | -177.945 | 0 | Fiji Islands Region. |
| 5.2 | MB | 15/12/2012 | 07:33:12 | -4.749 | 153.519 | 53 | New Ireland Region, PNG. |
| 6.0 | Mwp | 15/12/2012 | 19:30:05 | -4.703 | 153.065 | 81 | New Ireland Region, PNG. |
| 6.3 | Mwp | 17/12/2012 | 09:16:31 | -0.718 | 123.901 | 47 | Minahassa Peninsula, Sulawesi. |
| 5.0 | MB | 18/12/2012 | 08:27:01 | -6.322 | 120.660 | 0 | Flores Sea. |
| 5.2 | MB | 19/12/2012 | 11:51:46 | -18.927 | -175.299 | 184 | Tonga Islands. |
| 5.0 | MB | 20/12/2012 | 02:11:19 | -7.445 | 128.574 | 151 | Banda Sea. |
| 5.4 | MB | 20/12/2012 | 13:38:04 | -7.132 | 129.302 | 146 | Banda Sea. |
| 5.7 | MB | 20/12/2012 | 21:47:31 | 0.504 | 126.313 | 43 | Northern Molucca Sea. |
| 6.6 | Mwp | 21/12/2012 | 22:28:09 | -14.366 | 167.365 | 208 | Vanuatu Islands. |
| 5.4 | MB | 22/12/2012 | 03:50:11 | 5.370 | 94.393 | 70 | Northern Sumatra, Indonesia. |
| 5.2 | MB | 23/12/2012 | 00:30:00 | -13.048 | 167.131 | 260 | Vanuatu Islands. |
| 5.1 | MB | 24/12/2012 | 12:58:22 | 4.139 | 126.535 | 40 | Talaud Islands, Indonesia. |
| 5.4 | MB | 24/12/2012 | 23:24:50 | -55.690 | 146.778 | 10 | West of Macquarie Island. |
| 5.3 | MB | 25/12/2012 | 07:34:45 | 0.985 | 125.585 | 89 | Northern Molucca Sea. |
| 5.2 | MB | 25/12/2012 | 08:46:29 | -7.913 | 101.251 | 0 | Southwest of Sumatra, Indonesia. |
| 5.4 | MB | 26/12/2012 | 02:24:31 | -5.928 | 147.292 | 77 | Eastern New Guinea Region, PNG. |
| 5.6 | MB | 26/12/2012 | 02:58:22 | -20.541 | -178.377 | 580 | Fiji Islands Region. |
| 5.2 | MB | 26/12/2012 | 05:17:34 | -24.460 | -179.635 | 513 | South of Fiji Islands. |
| 5.0 | MB | 26/12/2012 | 19:08:03 | 0.965 | 98.708 | 84 | Northern Sumatra, Indonesia. |
| 5.4 | MB | 27/12/2012 | 09:41:55 | -18.195 | -173.390 | 0 | Tonga Islands. |
| 5.3 | MB | 28/12/2012 | 10:35:44 | -28.917 | -176.063 | 0 | Kermadec Islands Region. |
| 5.6 | MB | 28/12/2012 | 17:32:17 | -0.161 | 122.919 | 104 | Minahassa Peninsula, Sulawesi. |
| 5.1 | MB | 29/12/2012 | 07:56:10 | -3.402 | 148.571 | 97 | Bismarck Sea, PNG. |
| 5.2 | MB | 29/12/2012 | 07:59:25 | -3.554 | 148.475 | 81 | Bismarck Sea, PNG. |
| 5.2 | MB | 30/12/2012 | 11:33:43 | 1.476 | 126.583 | 74 | Northern Molucca Sea. |
| 5.1 | MB | 30/12/2012 | 19:25:58 | -1.446 | 101.398 | 88 | Southern Sumatra, Indonesia. |
| 5.0 | MB | 31/12/2012 | 20:02:15 | -61.410 | 154.730 | 95 | Balleny Islands Region. |

World Earthquakes

Table A.6 World Earthquakes Magnitude 6+ 2012

| Magnitude | Magnitude Type | Date | Time (UTC) | Latitude | Longitude | Depth (Km) | Location |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6.8 | Mwp | 01/01/2012 | 05:27:55 | 31.416 | 138.155 | 349 | Izu Islands, Japan Region. |
| 6.6 | Mwp | 09/01/2012 | 04:07:18 | -10.517 | 165.185 | 54 | Santa Cruz Islands. |
| 7.2 | Mwp | 10/01/2012 | 18:37:00 | 2.508 | 93.187 | 20 | Off W Coast of Northern Sumatra. |
| 6.6 | MB | 15/01/2012 | 13:40:18 | -60.765 | -55.858 | 10 | South Shetland Islands. |
| 6.2 | MB | 15/01/2012 | 14:21:32 | -60.852 | -55.886 | 14 | South Shetland Islands. |
| 6.1 | Mwp | 19/01/2012 | 05:58:42 | -11.040 | 165.625 | 38 | Santa Cruz Islands. |
| 6.2 | Mwp | 19/01/2012 | 06:48:45 | -46.780 | 165.761 | 0 | Off W Coast of S Island, New Zealand. |
| 6.2 | Mwp | 21/01/2012 | 18:47:16 | 14.963 | -93.087 | 80 | Offshore Chiapas, Mexico. |
| 6.3 | Mwp | 22/01/2012 | 05:53:40 | -56.485 | -25.081 | 3 | South Sandwich Islands Region. |
| 6.2 | Mwp | 23/01/2012 | 16:04:54 | -36.415 | -73.015 | 30 | Offshore Bio-Bio, Chile. |
| 6.2 | Mwp | 24/01/2012 | 00:52:05 | -24.727 | 178.769 | 594 | South of Fiji Islands. |
| 6.2 | Mwp | 28/01/2012 | 17:42:54 | -29.270 | -176.697 | 56 | Kermadec Islands, New Zealand. |
| 6.3 | Mwp | 30/01/2012 | 05:11:01 | -14.179 | -75.644 | 39 | Near the Coast of Central Peru. |
| 7.3 | Mwp | 02/02/2012 | 13:34:40 | -17.749 | 167.243 | 22 | Vanuatu Islands. |
| 6.4 | Mwp | 03/02/2012 | 03:46:27 | -17.397 | 167.331 | 50 | Vanuatu Islands. |
| 6.0 | Mwp | 05/02/2012 | 00:15:40 | -18.959 | 169.002 | 160 | Vanuatu Islands. |
| 6.3 | Mwp | 05/02/2012 | 16:40:40 | -17.927 | 167.336 | 12 | Vanuatu Islands. |
| 7.0 | Mwp | 06/02/2012 | 03:49:18 | 9.931 | 123.278 | 63 | Cebu, Philippines. |
| 6.3 | Mwp | 06/02/2012 | 10:10:29 | 9.719 | 123.217 | 94 | Cebu, Philippines. |
| 6.1 | Mwp | 06/02/2012 | 11:33:44 | 9.581 | 123.069 | 69 | Cebu, Philippines. |
| 6.0 | Mwp | 14/02/2012 | 06:21:58 | 36.154 | 141.494 | 10 | Near the East Coast of Honshu, Japan. |
| 6.5 | Mwp | 14/02/2012 | 08:20:01 | -10.421 | 161.243 | 110 | Solomon Islands. |
| 6.7 | Mwp | 26/02/2012 | 06:17:20 | 51.731 | 95.921 | 12 | Southwestern Siberia, Russia. |
| 6.7 | Mwp | 03/03/2012 | 12:19:58 | -22.164 | 170.404 | 30 | SE of Loyalty Islands, New Caledonia. |
| 6.1 | MB | 05/03/2012 | 07:46:10 | -28.227 | -63.242 | 550 | Santiago Del Estero, Argentina. |
| 6.0 | Mwp | 07/03/2012 | 12:00:44 | -2.823 | 139.048 | 43 | Near North Coast of Irian Jaya, Indonesia. |
| 6.3 | MB | 09/03/2012 | 07:09:55 | -19.054 | 169.739 | 45 | Vanuatu Islands. |
| 6.9 | Mwp | 14/03/2012 | 09:08:38 | 40.899 | 144.923 | 27 | Off the E Coast of Honshu, Japan. |
| 6.1 | Mwp | 14/03/2012 | 10:49:25 | 40.799 | 144.770 | 10 | Off the E Coast of Honshu, Japan. |
| 6.0 | MB | 14/03/2012 | 21:13:13 | -5.767 | 151.109 | 67 | New Britain region, PNG. |
| 6.3 | Mwp | 20/03/2012 | 17:56:20 | -3.922 | 140.296 | 79 | Irian Jaya, Indonesia. |
| 7.4 | Mwp | 20/03/2012 | 18:02:49 | 16.662 | -98.188 | 20 | Oaxaca, Mexico. |
| 6.7 | Mwp | 21/03/2012 | 22:15:07 | -6.301 | 146.078 | 123 | Northwest of Lae, PNG. |
| 7.1 | Mwp | 25/03/2012 | 22:37:07 | -35.183 | -71.792 | 35 | Maule, Chile. |
| 6.0 | Mwp | 27/03/2012 | 11:00:44 | 39.866 | 142.116 | 10 | Near the East coast of Honshu, Japan. |
| 6.0 | Mwp | 02/04/2012 | 17:36:43 | 16.456 | -98.296 | 9 | Oaxaca, Mexico. |
| 6.4 | Mwp | 06/04/2012 | 16:15:56 | -4.543 | 153.532 | 96 | New Ireland Region, PNG. |
| 8.6 | Mwp | 11/04/2012 | 08:38:32 | 2.277 | 92.981 | 0 | Off W Coast of Northern Sumatra, Indonesia. |
| 8.2 | Mwp | 11/04/2012 | 10:43:11 | 0.801 | 92.411 | 30 | Off W Coast of Northern Sumatra, Indonesia. |
| 6.1 | MB | 11/04/2012 | 11:53:36 | 2.934 | 89.491 | 10 | North Indian Ocean. |
| 6.5 | Mwp | 11/04/2012 | 22:55:11 | 18.272 | -102.732 | 20 | Michoacan, Mexico. |
| 6.2 | Mwp | 12/04/2012 | 07:06:01 | 28.841 | -113.072 | 10 | Gulf of California. |
| 6.9 | Mwp | 12/04/2012 | 07:15:49 | 28.790 | -113.142 | 10 | Gulf of California. |
| 6.2 | Mwp | 14/04/2012 | 10:56:19 | -57.588 | -65.414 | 10 | Drake Passage. |
| 6.1 | MB | 14/04/2012 | 22:05:31 | -18.971 | 168.949 | 57 | Vanuatu Islands. |
| 6.2 | MB | 15/04/2012 | 05:57:37 | 2.537 | 90.192 | 0 | Off W Coast of Northern Sumatra, Indonesia. |
| 6.7 | Mwp | 17/04/2012 | 03:50:17 | -32.701 | -71.484 | 37 | Offshore Valparaiso, Chile. |
| 6.5 | MB | 17/04/2012 | 07:13:50 | -5.509 | 147.142 | 208 | Eastern New Guinea Region, PNG. |
| 6.2 | Mwp | 17/04/2012 | 19:03:55 | -59.099 | -16.693 | 1 | East of South Sandwich Islands. |
| 6.3 | Mwp | 20/04/2012 | 23:14:33 | 2.189 | 93.365 | 44 | Off W Coast of Northern Sumatra, Indonesia. |
| 6.3 | MB | 21/04/2012 | 01:16:55 | -1.807 | 134.371 | 29 | Near North coast of Papua, Indonesia. |
| 6.5 | MB | 28/04/2012 | 10:08:04 | -18.610 | -174.649 | 91 | Tonga Islands. |
| 6.0 | Mwp | 29/04/2012 | 08:09:05 | 2.792 | 94.510 | 19 | Off W Coast of Northern Sumatra, Indonesia. |
| 6.0 | Mwp | 01/05/2012 | 22:43:38 | 14.466 | -92.909 | 36 | Offshore Chiapas, Mexico. |
| 6.2 | Mwp | 14/05/2012 | 10:00:39 | -17.816 | -69.749 | 98 | Tarapaca, Chile. |
| 6.2 | Mwp | 18/05/2012 | 02:00:41 | -44.594 | -80.073 | 10 | Off the Coast of Aisen, Chile. |
| 6.0 | Mwp | 20/05/2012 | 02:03:52 | 44.800 | 11.192 | 5 | Northern Italy. |
| 6.4 | Mwp | 20/05/2012 | 07:20:38 | 39.727 | 143.037 | 16 | Off East Coast of Honshu, Japan. |
| 6.2 | Mwp | 24/05/2012 | 22:47:47 | 72.994 | 5.651 | 9 | Norwegian Sea. |
| 6.2 | Mwp | 04/06/2012 | 00:45:16 | 5.287 | -82.580 | 10 | South of Panama. |
| 6.4 | Mwp | 04/06/2012 | 03:15:25 | 5.507 | -82.457 | 6 | South of Panama. |
| 6.0 | Mwp | 07/06/2012 | 16:03:18 | -15.919 | -72.516 | 100 | Southern Peru. |
| 6.0 | Mwp | 09/06/2012 | 21:00:18 | 24.522 | 122.337 | 71 | Taiwan Region. |
| 6.0 | MB | 15/06/2012 | 01:14:11 | 5.634 | 126.612 | 74 | Mindanao, Philippines. |
| 6.1 | Mwp | 16/06/2012 | 22:18:48 | 15.574 | 119.609 | 35 | Luzon, Philippines. |
| 6.4 | Mwp | 17/06/2012 | 20:32:20 | 38.894 | 141.946 | 32 | Near East Coast of Honshu, Japan. |
| 6.0 | Mwp | 19/06/2012 | 15:56:33 | 53.364 | 171.593 | 14 | Near Islands, Aleutian Islands, Alaska. |
| 6.1 | Mwp | 22/06/2012 | 04:31:29 | -54.119 | 158.780 | 113 | Macquarie Island Region. |
| 6.3 | Mwp | 23/06/2012 | 04:34:54 | 3.014 | 97.971 | 107 | Northern Sumatra Indonesia. |
| 6.1 | Mwp | 24/06/2012 | 03:15:03 | 57.601 | 163.218 | 17 | Near East Coast of Kamchatka, Russia. |
| 6.3 | Mwp | 29/06/2012 | 21:07:33 | 43.444 | 84.725 | 18 | Northern Xinjiang, China. |
| 6.2 | Mwp | 03/07/2012 | 10:36:13 | -40.063 | 174.017 | 217 | South Taranaki Bight, New Zealand. |
| 6.2 | Mwp | 06/07/2012 | 02:28:26 | -14.693 | 167.394 | 202 | Vanuatu Islands. |
| 6.0 | Mwp | 20/07/2012 | 06:10:26 | 49.418 | 155.935 | 23 | Kuril Islands. |
| 6.0 | MB | 25/07/2012 | 00:27:47 | 2.643 | 96.074 | 38 | Northern Sumatra, Indonesia. |
| 6.5 | Mwp | 25/07/2012 | 11:20:30 | -9.749 | 159.998 | 53 | Solomon Islands. |
| 6.7 | Mwp | 26/07/2012 | 05:33:32 | -17.593 | 66.363 | 10 | Mauritius - Reunion Region. |
| 6.2 | MB | 28/07/2012 | 20:04:01 | -4.742 | 153.173 | 75 | New Ireland Region, PNG. |
| 6.0 | Mwp | 29/07/2012 | 12:22:15 | 14.339 | -92.364 | 36 | Offshore Chiapas, Mexico. |
| 6.1 | Mwp | 02/08/2012 | 09:38:31 | -8.379 | -74.245 | 143 | Central Peru, South America. |
| 6.0 | Mwp | 02/08/2012 | 09:56:42 | -4.700 | 153.433 | 61 | New Ireland Region, PNG. |
| 6.2 | Mwp | 10/08/2012 | 18:37:43 | 52.658 | -167.425 | 13 | Fox Islands, Aleutian Islands, Alaska. |
| 6.4 | Mwp | 11/08/2012 | 12:23:18 | 38.322 | 46.888 | 10 | Northwestern Iran. |
| 6.3 | Mwp | 11/08/2012 | 12:34:35 | 38.324 | 46.759 | 10 | Northwestern Iran. |
| 6.3 | Mwp | 12/08/2012 | 10:47:06 | 35.681 | 82.516 | 11 | Xinjiang-Xizang Border Region. |
| 7.7 | Mwp | 14/08/2012 | 02:59:42 | 49.784 | 145.126 | 626 | Sea of Okhotsk, Russia. |
| 6.5 | Mwp | 18/08/2012 | 09:41:53 | -1.446 | 120.102 | 7 | Sulawesi, Indonesia. |
| 6.4 | Mwp | 18/08/2012 | 15:31:45 | 2.540 | 128.778 | 47 | Halmahera, Indonesia. |
| 6.1 | Mwp | 18/08/2012 | 17:44:58 | -15.565 | -172.653 | 0 | Samoa Islands Region. |
| 6.1 | MB | 19/08/2012 | 22:41:52 | -4.947 | 144.623 | 91 | Near N Coast of New Guinea, PNG. |
| 6.4 | MB | 26/08/2012 | 15:05:38 | 2.100 | 126.881 | 94 | Northern Molucca Sea. |
| 7.3 | Mwp | 27/08/2012 | 04:37:20 | 12.278 | -88.528 | 20 | Offshore El Salvador. |
| 6.8 | Mwp | 30/08/2012 | 13:43:25 | 71.461 | -10.919 | 10 | Jan Mayen Island Region. |
| 7.6 | Mwp | 31/08/2012 | 12:47:35 | 10.838 | 126.704 | 35 | Philippine Islands Region. |
| 6.4 | MB | 03/09/2012 | 18:23:03 | -10.818 | 113.880 | 0 | South of Java, Indonesia. |
| 6.1 | MB | 05/09/2012 | 13:09:15 | -12.561 | 166.452 | 52 | Santa Cruz Islands. |
| 7.6 | Mwp | 05/09/2012 | 14:42:08 | 9.996 | -85.318 | 40 | Off the Coast of Costa Rica. |
| 6.2 | MB | 08/09/2012 | 10:51:45 | -3.261 | 135.154 | 27 | Irian Jaya Region, Indonesia. |
| 6.0 | Mwp | 09/09/2012 | 05:39:21 | 49.429 | 155.537 | 59 | Kuril Islands. |
| 6.2 | MB | 14/09/2012 | 04:51:44 | -3.226 | 100.535 | 0 | Southern Sumatra, Indonesia. |
| 6.2 | Mwp | 25/09/2012 | 23:45:26 | 24.835 | -110.152 | 10 | Gulf of California. |
| 6.7 | Mwp | 26/09/2012 | 23:39:59 | 51.583 | -178.200 | 41 | Andreanof Islands, Alaska. |
| 6.2 | Mwp | 27/09/2012 | 23:54:00 | -8.969 | 157.044 | 60 | Solomon Islands. |
| 7.3 | Mwp | 30/09/2012 | 16:31:36 | 1.916 | -76.355 | 168 | Colombia. |
| 6.2 | Mwp | 01/10/2012 | 22:21:45 | 39.853 | 143.047 | 10 | Off E Coast of Honshu, Japan. |
| 6.0 | MB | 08/10/2012 | 06:26:22 | 25.125 | -109.698 | 10 | Gulf of California. |
| 6.3 | MB | 08/10/2012 | 11:43:30 | -4.575 | 129.168 | 0 | Banda Sea. |
| 6.6 | Mwp | 09/10/2012 | 12:32:05 | -61.041 | 154.372 | 20 | Balleny Islands Region. |
| 6.8 | Mwp | 12/10/2012 | 00:31:27 | -4.910 | 134.060 | 9 | Irian Jaya Region, Indonesia. |
| 6.6 | Mwp | 20/10/2012 | 23:00:34 | -13.543 | 166.695 | 55 | Vanuatu Islands. |
| 6.0 | Mwp | 23/10/2012 | 09:39:27 | -22.159 | 171.914 | 96 | Southeast of Loyalty Islands. |
| 6.5 | Mwp | 24/10/2012 | 00:45:34 | 10.121 | -85.314 | 20 | Costa Rica. |
| 7.7 | Mwp | 28/10/2012 | 03:04:11 | 52.769 | -131.927 | 18 | Queen Charlotte Islands Region. |
| 6.3 | Mwp | 28/10/2012 | 18:54:22 | 52.633 | -132.701 | 8 | Queen Charlotte Islands Region. |
| 6.2 | Mwp | 30/10/2012 | 02:49:03 | 52.324 | -131.908 | 10 | Queen Charlotte Islands Region. |
| 6.2 | Mwp | 02/11/2012 | 18:17:37 | 9.157 | 126.470 | 73 | Mindanao, Philippines. |
| 7.4 | Mwp | 07/11/2012 | 16:35:50 | 14.083 | -91.916 | 42 | Offshore Guatemala. |
| 6.3 | Mwp | 08/11/2012 | 02:01:51 | 49.185 | -128.528 | 17 | Vancouver Island, Canada Region. |
| 6.8 | Mwp | 11/11/2012 | 01:12:38 | 23.014 | 95.942 | 10 | Myanmar. |
| 6.5 | Mwp | 11/11/2012 | 22:15:00 | 14.164 | -92.167 | 27 | Offshore Guatemala. |
| 6.4 | Mwp | 12/11/2012 | 20:42:15 | 57.544 | -142.889 | 55 | Gulf of Alaska. |
| 6.1 | Mwp | 14/11/2012 | 19:02:06 | -29.157 | -71.272 | 62 | Coquimbo, Chile. |
| 6.4 | Mwp | 16/11/2012 | 18:12:40 | 49.269 | 155.472 | 29 | Kuril Islands. |
| 6.1 | Mwp | 17/11/2012 | 05:12:55 | -18.241 | -172.326 | 0 | Tonga Islands Region. |
| 6.0 | MB | 19/11/2012 | 09:44:38 | -5.822 | 151.700 | 42 | New Britain Region, PNG. |
| 6.0 | MB | 02/12/2012 | 00:54:26 | -16.971 | 167.666 | 61 | Vanuatu Islands. |
| 7.3 | Mwp | 07/12/2012 | 08:18:24 | 37.889 | 144.090 | 36 | Off The East Coast of Honshu, Japan. |
| 6.2 | MB | 07/12/2012 | 08:31:15 | 37.939 | 143.763 | 29 | Off The East Coast of Honshu, Japan. |
| 6.1 | Mwp | 07/12/2012 | 18:19:05 | -38.343 | 176.266 | 136 | North Island, New Zealand. |
| 7.1 | Mwp | 10/12/2012 | 16:53:09 | -6.634 | 129.823 | 151 | Banda Sea. |
| 6.0 | MB | 11/12/2012 | 06:18:31 | 0.449 | 126.283 | 61 | Northern Molucca Sea. |
| 6.3 | Mwp | 14/12/2012 | 10:36:02 | 31.213 | -119.560 | 11 | Off The West Coast of Baja California. |
| 6.0 | Mwp | 15/12/2012 | 19:30:05 | -4.703 | 153.065 | 81 | New Ireland Region, PNG. |
| 6.3 | Mwp | 17/12/2012 | 09:16:31 | -0.718 | 123.901 | 47 | Minahassa Peninsula, Sulawesi. |
| 6.6 | Mwp | 21/12/2012 | 22:28:09 | -14.366 | 167.365 | 208 | Vanuatu Islands. |

Geoscience Australia Seismic Data Downloading Procedure using SEEDLINK

Purpose

This guideline describes the process of downloading Geoscience Australia (GA) Australian National Seismograph Network (ANSN) seismic data with freeware software using the SEEDLINK protocol.At present, GA is looking to streamline its approach for preparing seismic data for use by external clients that does not have the overheads associated with the autoDRM client currently used.International users of GA seismic data with Antelope licenses retrieve GA data using orb2orb processes using SEEDLINK protocol.

GA has developed the process described in this document, using freely downloadable software, to obtain real-time data feeds, locally store, manage and analyse data acquired by the ANSN.

Definitions

GA			Geoscience Australia

SEEDLINK		A robust data transmission intended for use on the internet or private circuits that support TCP/IP

Real-time		Continuous data with latency typically less than one minute

Antelope		Seismic program produced by Boulder Real Time Technologies (BRTT) which is an integrated collection of programs for real-time data collection and seismic analysis

autoDRM		Automatic Data Request Manager is a software package that allows email to retrieve seismic datapackets from a collection agency

Watcom		Compiler

Slinktool		Command line SEEDLINK client

SeisGram2K	Java based interactive visualization and analysis of earthquake 	seismograms

Procedure

Installation

Go to “My Documents” on a windows based pc and make a new folder called “GA Data” (or whatever name you want to call it).Double click on this directory and make another file called “archive”.

If using on a windows based pc, connect to the internet and visit http://www.openwatcom.org/index.php/Download .Scroll down and click on “ftp” from the openwatcom.org mirror site for the installer’s version.

Download open-watcom-c-win32-1.8.exe and save in the GA Data directory in My Documents.This may take some time as the file is 77Mb.

Go back to the GA Data directory and click on the executable (*.exe file).Click I Agree and allow the program to save to the C drive.Allow a selective install; click next, next and next.Allow to modify local machine environment variables and allow to modify associations.Click OK when finished and reboot your pc.

Once rebooted, go back to the internet and download http://www.iris.edu/pub/programs/SeedLink/slinktool-4.1b.tar.gz and save in the GA Data directory.

Go to this file in the GA Data directory and right click.If you have an extraction program such as 7-zip or WinZip, extract here should unzip and untar the file and make a slinktool folder.The file slinktool-4.1b.zip can also be downloaded and unzipped if you come across problems.These same commands can be done in a Unix/Linux environment using gunzip and tar –xvf on the file.

Open up a command prompt window (Programs – Accessories – Command Prompt) in windows, type in cd c:WATCOM to change into the compiler directory.Type dir to do a listing of what is available in this directory.Type owsetenv.bat to start the watcom environment.

Type cd .. to get out of WATCOM and then cd Documents and Settings\local admin\My Documents\GA Data\slinktool .The tab key will write the rest of each line (i.e. Docu(Tab) will write out Documents and Settings).

Type wmake –f Makefile.wat to compile the slinktool program.After about one minute, an executable file will be created.In LINUX, a “make gcc” or in UNIX, a “make” will also compile the program.If using a solaris UNIX system, read the README file to locate a line which must be unhashed in the src/Makefile file.

To view the waveforms that you have downloaded, go to http://alomax.free.fr/seisgram/ver53/SeisGram2K_install.html and click on SeisGram2K53.jar and save in GA Data.This file will be an executable java file and can be run just by double clicking.If it doesn’t, update your java version.

All of the compiling has taken place and only will need to be completed if reinstalling – not starting a new download screen

Configuration

In a command prompt window, type c:Documents and Settings\local admin\My Documents\GA Data\slinktool .Type in slinktool –h to access a help file with all of the possible options.

A simple line for downloading direct from the GA Seismic SEEDLINK server is slinktool [options] [host][:][port]

For our data, an example is

slinktool –S “AU_PTPS”,”AU_BBOO”,”AU_WHYH” –SDS \archive 192.104.43.82:18000

slinktool –S “AU_BBOO” –BUD \archive 192.104.43.82:18000

The program will start storing data into the archive directory after about 20 seconds.The line will appear as though nothing is happening – this is what you want.Error messages will come up if a particular station is down or the command line is typed incorrectly.

On average, each channel will download between 3 and 6 Megabytes per day or for 50 stations over an entire year, approximately 230 Gigabytes.If you have download usage allowances, please be aware.If you have download space restrictions, burning to a DVD once per week will keep your archive size small.

Using the SDS file structure, data can be viewed in newer versions of EQSuite by ES&S.More testing of other seismogram viewers will be carried out later.

Other options in slinktool can also be played around with that could improve its capabilities.The network timeout could be very useful for flaky or dial-up connections.

To stop the acquisition of data, type Control+C.

Stations Exported through SEEDLINK

- The easiest way to see what stations are coming through via SEEDLINK protocol is to visit http://www.iris.edu/mda/AU .

Updates to the station list (and associated metadata) are always occurring so regularly check this list.

Stations recently upgraded will be offline for several weeks before all QA/QC tests have been completed and station has been implemented into the production environment for dissemination through SEEDLINK.

Web accessible station status, descriptions and locations are currently not available in the public domain for JUMP stations, other station information is available through IRIS links at http://www.iris.washington.edu/bud_stuff/dmc/ .

Limitations

As this software is free, there is no guarantee on the programs involved being problem free.

Where the SEEDLINK server has crashed, data will not backfill using this process (only real time).For research purposes, backup data can be requested from IRIS.GA is currently looking into the viability of being able to perform this function in the distant future.

IRIS Seismic Data Downloading Procedure using SEEDLINK

Purpose

This guideline describes the process of downloading Incorporated Research Institutions for Seismology (IRIS) Seismic data using freeware software using the SEEDLINK protocol.At present, GA is looking to aid the collection and analysis of seismic data for its external clients to use within their own warning and research centres.International users of GA seismic data with antelope licenses have been able to pull data using orb2orb processes using SEEDLINK protocol.GA has now come up with a process, using freely downloadable software, to grab a feed of the real-time data, store, manage and analyses the data currently being produced.

Definitions

GA			Geoscience Australia

SEEDLINK		A robust data transmission intended for use on the internet or private circuits that support TCP/IP

Real-time		Continuous data with less than one minute latency

Antelope		Seismic program which is an integrated collection of programs for data collection and seismic analysis

autoDRM		Automatic Data Request Manager is a software package that allows email to retrieve seismic datapackets from a collection agency

Watcom		Compiler

Slinktool		Command line SEEDLINK client

SeisGram2K		Java based interactive visualization and analysis of earthquake seismograms

Procedure

Installation

Go to “My Documents” on a windows based pc and make a new folder called “IRIS Data” (or whatever name you want to call it).Double click on this directory and make another file called “archive”.

If using on a windows based pc, connect to the internet and visit http://www.openwatcom.org/index.php/Download .Scroll down and click on “ftp” from the openwatcom.org mirror site for the installer’s version.

Download open-watcom-c-win32-1.8.exe and save in the IRIS Data directory in My Documents.This may take some time as the file is 77Mb.

Go back to the IRIS Data directory and click on the executable (*.exe file).Click I Agree and allow the program to save to the C drive.Allow a selective install; click next, next and next.Allow to modify local machine environment variables and allow to modify associations.Click OK when finished and reboot your pc.

Once rebooted, go back to the internet and download http://www.iris.edu/pub/programs/SeedLink/slinktool-4.1b.tar.gz and save in the IRIS Data directory.

Go to this file in the IRIS Data directory and right click.If you have an extraction program such as 7-zip or WinZip, extract here should unzip and untar the file and make a slinktool folder.The file slinktool-4.1b.zip can also be downloaded and unzipped if you come across problems.These same commands can be done in a Unix/Linux environment using gunzip and tar –xvf on the file.

Open up a command prompt window (Programs – Accessories – Command Prompt) in windows, type in cd c:WATCOM to change into the compiler directory.Type dir to do a listing of what is available in this directory.Type owsetenv.bat to start the watcom environment.

Type cd .. to get out of WATCOM and then cd Documents and Settings\local admin\My Documents\IRIS Data\slinktool .The tab key will write the rest of each line (i.e. Docu(Tab) will write out Documents and Settings).

Type wmake –f Makefile.wat to compile the slinktool program.After about one minute, an executable file will be created.In LINUX, a “make gcc” or in UNIX, a “make” will also compile the program.If using a solaris UNIX system, read the README file to locate a line which must be unhashed in the src/Makefile file.

To view the waveforms that you have downloaded, go to http://alomax.free.fr/seisgram/ver53/SeisGram2K_install.html and click on SeisGram2K53.jar and save in IRIS Data.This file will be an executable java file and can be run just by double clicking.If it doesn’t, update your java version.

All of the compiling has taken place and only will need to be completed if reinstalling – not starting a new download screen

Configuration

In a command prompt window, type c:Documents and Settings\local admin\My Documents\IRIS Data\slinktool .Type in slinktool –h to access a help file with all of the possible options.

A simple line for downloading direct from the IRIS Seismic SEEDLINK server is slinktool [options] [host][:][port]

For IRIS data, an example is

slinktool –S “IU_KNTN”,”AU_BBOO”,”NZ_WPVZ” –SDS \archive rtserve.iris.washington.edu:18000

slinktool –S “AU_BBOO” –BUD \archive rtserve.iris.washington.edu:18000

The program will start storing data into the archive directory after about 20 seconds.The line will appear as though nothing is happening – this is what you want.Error messages will come up if a particular station is down or the command line is typed incorrectly.

On average, each channel will download between 3 and 6 Megabytes per day or for 50 stations over an entire year, approximately 230 Gigabytes.If you have download usage allowances, please be aware.If you have download space restrictions, burning to a DVD once per week will keep your archive size small.

Using the SDS file structure, data can be viewed in newer versions of EQSuite by ES&S.More testing of other seismogram viewers will be carried out later.

Other options in slinktool can also be played around with that could improve its capabilities.The network timeout could be very useful for flaky or dial-up connections.

To stop the acquisition of data, type Control+C.

Stations Exported through SEEDLINK

The easiest way to see what stations are coming through via SEEDLINK protocol is to visit http://www.iris.edu/mda/ .

Updates to the station list (and associated metadata) are always occurring so regularly check this list.

Stations recently upgraded will be offline for several weeks before all QA/QC tests have been completed and station has been implemented into the production environment for dissemination through SEEDLINK.

Limitations

As this software is free, there is no guarantee on the programs involved being problem free.

Where the SEEDLINK server has crashed, data will not backfill using this process (only real time).For research purposes, backup data can be requested from IRIS.