Oceanic Shoals Commonwealth Marine Reserve (Timor Sea) Biodiversity Survey

GA0339/SOL5650 - Post-Survey Report

Geoscience Australia
RECORD 2013/38

S.L. Nichol1, F.J.F. Howard1, J. Kool1, M. Stowar2, P. Bouchet3, L. Radke1, J. Siwabessy1, R. Przeslawski1, K. Picard1 , B. Alvarez de Glasby4 , J. Colquhoun2, T. Letessier,3 A. Heyward2

- Geoscience Australia

- Australian Institute of Marine Science

- Oceans Institute, University of Western Australia

- Museum and Art Gallery of the Northern Territory

Department of Industry

Minister for Industry: The Hon Ian Macfarlane MP
Parliamentary Secretary: The Hon Bob Baldwin MP
Secretary: Ms Glenys Beauchamp PSM

Geoscience Australia

Chief Executive Officer: Dr Chris Pigram
This paper is published with the permission of the CEO, Geoscience Australia

© Commonwealth of Australia (Geoscience Australia) 2013

With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence. (http://www.creativecommons.org/licenses/by/3.0/au/deed.en)

Geoscience Australia has tried to make the information in this product as accurate as possible. However, it does not guarantee that the information is totally accurate or complete. Therefore, you should not solely rely on this information when making a commercial decision.

Geoscience Australia is committed to providing web accessible content wherever possible. If you are having difficulties with accessing this document please contact clientservices@ga.gov.au.

ISSN 2201-702X (PDF)

ISBN 978-1-922201-74-4 (PDF)

GeoCat 76658

Bibliographic reference: Nichol, S.L., Howard, F.J.F., Kool, J., Stowar, M., Bouchet, P., Radke, L., Siwabessy, J., Przeslawski, R., Picard, K., Alvarez de Glasby, B., Colquhoun, J., Letessier, T. & Heyward, A. 2013. Oceanic Shoals Commonwealth Marine Reserve (Timor Sea) Biodiversity Survey: GA0339/SOL5650 – Post Survey Report. Record 2013/38. Geoscience Australia: Canberra.

Version: 1307

Contents

List of Figures	v

List of Tables	viii

Executive Summary	1

1 Introduction	3

1.1 Background and Survey Aims	3

1.2 Study Area	4

2 Methods	7

2.1 Survey Overview	7

2.2 Sampling Overview	7

2.3 Geophysical Data Acquisition and Processing	10

2.3.1 Bathymetric Mapping	10

2.3.2 Backscatter Acquisition and Processing	11

2.3.3 Sub-Bottom Profiler Acquisition and Processing	11

2.4 Oceanographic Data Acquisition	11

2.4.1 Water Column Measurements and Sampling	11

2.4.2 Ocean Current Drifters	12

2.5 Physical Sampling	13

2.5.1 Sedimentology	13

2.5.2 Geochemistry	14

2.5.3 Biology	15

2.5.3.1 Towed-video	16

2.5.3.2 Infaunal Sampling	16

2.5.3.3 Epifaunal Sampling	17

2.5.3.4 Biological Material for Biodiscovery	17

2.5.3.5 Sampling of demersal fish diversity and abundance	18

2.5.3.6 Video sampling (SISSTAs) and visual surveys of marine predators	19

3 Preliminary Results & Interpretations	21

3.1 Seabed Characteristics Area 1	22

3.2 Seabed Characteristics Area 2	29

3.3 Seabed Characteristics Area 3	33

3.4 Seabed Characteristics Area 4	39

3.5 All Areas Summary and Comparison	44

3.6 Sub-Bottom Profile Results	47

3.7 Initial Ocean Drifter Results	49

4 Conclusions and Future Work	51

Acknowledgements	52

References	53

Appendix A. Scientific and Technical Crew	55

Appendix B. Survey Details for Sub-bottom Lines	56

Appendix C. Survey Details for CTD Deployments	57

Appendix D. Survey details for Ocean Current Drifter Deployments	59

Appendix E. Survey Details for Surface Sediment Grab (GR) and Box Core (BC) Samples	60

Appendix F. Survey Details for Surface Water Samples	63

Appendix G. Survey Details for Geochemical Samples	64

Appendix H. Geochemical Techniques	67

Appendix I. Survey Details for Towed-Video Transects	69

Appendix J. Survey Details for Benthic Sled Transects	76

Appendix K. Biodiscovery Samples	78

Appendix L. Survey Details for Baited Remote Underwater Video Stations (BRUVS)	84

Appendix M. Survey Details for Baited Underwater Water Column Video (SISSTAs)	86

Appendix N. Geomorphic Mapping Method	88

Appendix O. Survey Leader Daily Log	90

List of Figures

Figure 1.1 Location map of areas surveyed during the Oceanic Shoals Commonwealth Marine Reserve Biodiversity Survey.	4

Figure 1.2 Map showing geomorphic features within the western part of the Oceanic Shoals Commonwealth Marine Reserve, with survey areas indicated. Geomorphic features are based on the Australian 250 m bathymetry grid (published October 2006), as described by Heap and Harris (2008).	5

Figure 2.1 Kongsberg EM3002D multibeam sonar system installed on RV Solander, showing: (a) POS MV motion reference unit, data acquisition and processing units, and; (b) single head mounting deployed in the moon-pool.	10

Figure 2.2 Applied Acoustics sub-bottom “Sparker” system, showing: (a) CSP-D generator unit and; (b) Squid-2000 sound source.	11

Figure 2.3 Seabird Electronics SBE-911plus Livewire CTD prior to deployment.	12

Figure 2.4 Ocean drifter prior to deployment.	13

Figure 2.5 Images of the sediment sampling equipment employed on this survey. (a) A Smith McIntyre grab full of sediment, sitting in its frame. (b) A returned box corer, full of sediment.	14

Figure 2.6 (a) Extraction of sediment cores from the Smith McIntyre grab; (b) Incubation of cores under different light treatments; (c) Example sediment core complete with stirrer, and; (d) Example of a core, post-incubation, showing sulphide accumulation around a worm burrow. The sulphide build-up is due to the occurrence of reactive carbon in the degrading worm.	15

Figure 2.7 Images of the AIMS towed-video system, showing: (a) Deployment of AIMS towed-video system of the stern of the RV Solander; (b) Monitor output from the USBL acoustic tracking system, and; (c) Monitor showing real-time video footage as seen by the towed-video system.	16

Figure 2.8 Images of the elutriating process used for infaunal sampling, showing: (a) Sediment collected from a grab sample in Nally Bin prior to elutriating; (b) Scientist elutriating grab sample with a 500 μm sieve to collect suspended organisms, and; (c) Photograph of organisms collected by sieve after 5 minutes of elutriation.	17

Figure 2.9 Examples of the epibenthic sampling process using the AIMS sled, showing: (a) Deployment of the epibenthic sled from the stern of the RV Solander; (b) A full sled net being released onto the sorting table, and; (c) Biological samples from a sled being sorted into broad taxonomic groups.	17

Figure 2.10 Examples of biodiscovery operations, including: (a) Biologists and crew of RV Solander sorting through a epibenthic sled sample for materials that have biodiscovery potential; (b) Sponges (centre) and octocorals (in green bucket) selected for further analysis, and; (c) Senior biologist identifying, cataloguing and processing samples with high biodiscovery potential.	18

Figure 2.11 Imagery obtained from BRUVS deployed during the Oceanic Shoals survey: (a) Video frame grab of a giant Queensland groper (Epinephelus lanceolatus); (b) Still image taken by a camera mounted to a BRUVS frame showing a spangled emperor (Lethrinus nebulosus) and big-eye trevally (Caranx sexfasciatus).	19

Figure 2.12 Examples of high-order marine species observed within the Oceanic Shoals CMR, including: (a) Bottlenose dolphin (Tursiops truncatus); (b) Killer whales (Orcinus orca); (c) Mid-water video observations of Australian blacktip shark (Carcharhinus tilstoni) and; (d) Manta ray (Manta birostris).	20

Figure 3.1 False colour bathymetry image of Area 1 with sub-bottom profile (sparker) lines and sampling stations indicated.	24

Figure 3.2 False colour backscatter image of Area 1 with towed video transects, benthic sled, baited remote underwater video (BRUVS) and baited mid-water column video (SISSTAs) stations indicated.	25

Figure 3.3 Local-scale geomorphic features mapped in Area 1 interpreted from bathymetry. Images are 30% transparent polygons overlying hill-shaded bathymetric images. Boxes indicate the location of key geomorphic features shown in Figure 3.4.	26

Figure 3.4 Hill shaded bathymetry (a-d) and backscatter (e-h) images of representative geomorphic features in Area 1, including: bank (a and e); sparse pockmark fields (b and f) with and without ‘tails’; dense pockmark fields (c and g), and; terraced and notched morphology of the sides of banks (d and h). Although these specific examples are in Area 1 they are representative of the other areas surveyed.	27

Figure 3.5 Representative still images from the northernmost bank in Area 1, showing: (a) live mushroom corals; (b) coral rubble; (c) moderate to dense epifaunal coverage (cnidarians, sponges); (d) the flank of the bank supporting scattered epifauna (sponges, Mopsella gorgonian), and; (e) high levels of bioturbation on the lower flank.	28

Figure 3.6 False colour bathymetry image of Area 2 with sub-bottom profile (sparker) lines and sampling stations indicated.	30

Figure 3.7 False colour backscatter image of Area 2 with towed video transects, benthic sled, baited remote underwater video (BRUVS) and baited mid-water column video (SISSTAs) stations indicated.	30

Figure 3.8 Local-scale geomorphic features mapped in Area 2 interpreted from bathymetry. Images are 30% transparent polygons overlying hill-shaded bathymetric images.	31

Figure 3.9 Representative still images from the main bank in Area 2, showing: (a) sparse epifauna on bioturbated sandy substrate; (b) a gorgonian growing on sand; (c) larger habitat-forming epifauna (Ianthella sponges, Mopsella gorgonian; (d) moderate to dense coverage of epifauna on the lower terrace of the bank, and; (e) moderate concentration of bioturbation on the lower flank.	32

Figure 3.10 False colour bathymetry image of Area 3 with sampling stations indicated.	34

Figure 3.11 False colour backscatter image of Area 3 with towed video transects and benthic sled, baited remote underwater video (BRUVS) and baited mid-water column video (SISSTAs) stations indicated.	35

Figure 3.12 Local-scale geomorphic features mapped in Area 3 interpreted from bathymetry. Images are 30% transparent polygons overlying hill-shaded bathymetric images.	36

Figure 3.13 Representative still images from the main bank in Area 3, showing: (a) high density of burrows in silty sediment; (b) benthic algae; (c) scattered epifauna (sponges, gorgonians); (d) very sparse epifauna; (e) and (f) high levels of bioturbation on the lower terraces.	38

Figure 3.14 False colour bathymetry image of Area 4 with sampling stations indicated.	40

Figure 3.15 False colour backscatter image of Area 4 with towed video transects indicated (Benthic sled, BRUVS and SISSTAs were not deployed in this survey area).	41

Figure 3.16 Local-scale geomorphic features mapped in Area 4 interpreted from bathymetry. Images are 30% transparent polygons overlying hill-shaded bathymetric images.	42

Figure 3.17 Representative still images from the main bank in Area 4, showing: (a) the top of the bank with conspicuous sea urchins; (b) mushroom corals; (c) a barrel sponge Xestospongia; (d) the flank of the bank supporting encrusting and low-relief epifauna, and; (e) the margin of the bank showing scattered epifauna and low levels of levels of bioturbation.	43

Figure 3.18 Representative still images from geomorphic features in the study area: (a) Moderate octocoral coverage at station 16CAM12 (~70 m depth); (b) Limited epifauna and bioturbation at station 32CAM22 (~105 m depth); (c) Burrows at station 57CAM42 (~110 m depth); (d) Scleractinian coral at station 16CAM13 (~35 m depth); (e) Habitat-forming gorgonian (Mopsella spp.) at station 67CAM48 (~48 m depth), and; (f) Absence of epifauna at station 47CAM33 (~68 m depth).	45

Figure 3.19 Mean species richness and biomass as estimated on-board from sled collections, based on: (a) survey area, and; (b) banks that were sampled. Numbers in parentheses indicate the number of sleds deployed, and error bars are standard error mean.	46

Figure 3.20 Section of sub-bottom profile from line 01 in Area 1, showing signal penetration to >100 m below a soft sediment plain through a sedimentary succession characterised by repeated channel cut-and-fill and vertical accretion depths based on sound velocity of 1500 m/s.	47

Figure 3.21 Section of sub-bottom profile from line 01 in Area 2, showing limited signal penetration into a carbonate bank and terrace and penetration to >100 m below the adjacent plain, with the latter characterised by channel cut-and-fill and vertical accretion. Depths based on sound velocity of 1500 m/s.	48

Figure 3.22 Section of sub-bottom profile from line 01 in Area 3, showing signal penetration to >100 m below the seabed through a sedimentary succession characterised by vertical accretion and draping across the edge of a bank. Depths based on sound velocity of 1500 m/s.	49

Figure 3.23 Trajectory plots for drifters released during the Oceanic Shoals survey as of 5 June 2013. Different colours represent different drifters. Tracks that end with a solid circle represent drifters that have terminated transmission and arrowheads represent ongoing transmission. Drifter 53522 (red track-line) was deployed on the return transit to Broome.	50

List of Tables

Table 2.1 List of sampling and tow video operations by station. Deployment details for BRUVS, SISSTAs and drifters are listed separately in the Appendices. An asterisk denotes an unsuccessful deployment. Detailed descriptions of sampling are included in the voyage leader log (Appendix O)	8

Table 3.1 Areas mapped by acoustic multibeam and sub-bottom profile activities in the Oceanic Shoals CMR Survey (SOL5650), excluding transit lines.	21

Table 3.2 Summary of the geomorphic features mapped with definitions and datasets used to produce them. Feature definitions were based on those used by Heap and Harris (2008).	21

Table 3.3 Summary statistics for areas surveyed and their geomorphic diversity.	22

Table 3.4 Summary of physical variables for geomorphic features mapped in Area 1.	23

Table 3.5 Summary of physical variables for geomorphic features mapped in Area 2.	31

Table 3.6 Summary of physical variables for geomorphic features mapped in Area 3.	37

Table 3.7 Summary of physical variables for geomorphic features mapped in Area 4.	43

Table A.1 Role and affiliation of personnel on survey GA0339/SOL5650.	55

Table B.1 Survey details for sub-bottom profile lines.	56

Table C.1 Survey details for CTD deployments.	57

Table D.1 Survey details for ocean current drifter deployments.	59

Table E.1 Surface sediment grab (GR) and box core (BC) sample locations, water depths and field descriptions (including Munsell colour code, where noted).	60

Table F.1 Details of underway water samples collected during transits.	63

Table G.1 Details of the geochemical analyses undertaken on sediment samples. Grab numbers from Smith-McIntyre grabs (GRXXX) and box cores (BCXXX) are shown in the columns. The designation B_XX corresponds to the extensions that are applied to the sample numbers in the corporate database, in which B designates a geochemistry sample and XX designates a sub-sample type. For example, SOL5650/060GR045B_B1 refers to a geochemistry sample from station 060 and grab 45 designated for chlorin, porosity and mineralogy analysis. SOD denotes sediment oxygen demand.	64

Table H.1 Details of shipboard laboratory processing and analytical techniques used to prepare and analyse the geochemistry sub-samples. Sub-sample codes (B_B1 etc) correspond to the file extension assigned to the sub-sample types in the shipboard database.	67

Table I.1 Towed-video transect details.	69

Table J.1 Benthic sled transect details. All transects were deployed over banks. Biota weight includes the total wet weight of all live samples collected.	76

Table K.1 Biological specimens processed for biodiscovery purposes.	78

Table L.1 Baited remote underwater videos (BRUV) station details.	84

Table M.1 Baited mid water column video (SISSTAs) station details. (* denotes camera not recovered)	86

Table N.1 Summary of the geomorphic features mapped with definitions and datasets used to produce them. Feature definitions were based on those used by Heap & Harris (2008).	88

Table O.1 Sampling operations completed on Thursday, 13 September 2012.	91

Table O.2 Sampling operations completed on Friday, 14 September 2012.	91

Table O.3 Sampling operations completed on Saturday, 15 September 2012.	92

Table O.4 Sampling operations completed on Sunday, 16 September 2012.	93

Table O.5 Sampling operations completed on Monday, 17 September 2012.	94

Table O.6 Sampling operations completed on Tuesday, 18 September 2012.	95

Table O.7 Sub-bottom profile lines completed on Wednesday, 19 September 2012.	96

Table O.8 Sampling operations completed on Wednesday, 19 September 2012.	96

Table O.9 Sampling operations completed on Thursday, 20 September 2012.	97

Table O.10 Sampling operations completed on Friday, 21 September 2012.	99

Table O.11 Sampling operations completed on Saturday, 22 September 2012.	100

Table O.12 Sampling operations completed on Sunday, 23 September 2012.	101

Table O.13 Sub-bottom profile lines completed on Monday, 24 September 2012.	102

Table O.14 Sampling operations completed on Monday, 24 September 2012.	102

Table O.15 Sampling operations completed on Wednesday, 26 September 2012.	103

Table O.16 Sampling operations completed on Thursday, 27 September 2012.	105

Table O.17 Sampling operations completed on Friday, 28 September 2012.	106

Table O.18 Sampling operations completed on Saturday, 29 September 2012.	106

Table O.19 Sampling operations completed on Sunday, 30 September 2012.	107

Table O.20 Sub-bottom profile lines completed on Monday, 1 October 2012.	108

Table O.21 Sampling operations completed on Monday, 1 October 2012.	109

Table O.22 Sampling operations completed on Tuesday, 2 October 2012.	109

Table O.23 Sampling operations completed on Monday, 1 October 2012.	110

Table O.24 Sampling operations completed on Wednesday, 3 October 2012.	111

Table O.25 Drifters deployed during the survey	111

Executive Summary

This report provides details of activities undertaken by the Australian Institute of Marine Science (AIMS), Geoscience Australia (GA), the University of Western Australia and the Museum and Art Gallery of the Northern Territory during a marine biodiversity survey to the Oceanic Shoals Commonwealth Marine Reserve (Timor Sea) in 2012. The survey was an activity within the Australian Government’s National Environmental Research Program Marine Biodiversity Hub and the key component of Theme 4 – Regional Biodiversity Discovery to Support Marine Bioregional Plans. Data collected during the survey will be used to support research being undertaken in other Themes of the Marine Biodiversity Hub, including the modelling of ecosystem processes for the northern region, and to support the work programs of the Department of the Environment (previously Department of Sustainability, Environment, Water, Population and Communities). These data will be made publicly available, via the Marine Biodiversity Hub website and the Australian Ocean Data Network Portal, adding to the knowledge base of Australian tropical shelf habitats and contributing to the long term management of these poorly understood areas.

The survey was undertaken between 12 September and 6 October 2012 on the AIMS Research Vessel, RV Solander, and acquired data from four areas in the western sector of the Oceanic Shoals Commonwealth Marine Reserve. Covering 507 km2, these areas incorporate a variety of seabed geomorphic features across water depths that range from 30 to 180 m, including carbonate banks, terraces and pinnacles, as well as soft sediment plains and valleys. The carbonate banks and terraces in particular are recognised in the North and North-West Marine Region Plans as a Key Ecological Feature with potential as a biodiversity hotspot. Data acquired from each survey area included: continuous high-resolution multibeam sonar bathymetry and acoustic backscatter; sub-bottom acoustic profiles along representative transects; physical samples of seabed sediments and infauna (hard- and soft-bodied organisms); physical samples of epibenthic biota (sponges & corals); video and still camera observations of seabed habitats and associated biological communities, including demersal and pelagic fish; observations of high-order pelagic predators (whales, dolphins), and; oceanographic measurements of the water column from CTD (conductivity, temperature, depth) casts and from deployment of sea surface drifters. In total, samples and/or video observations were collected at 70 stations.

Key observations from the survey are as follows:

The geomorphic diversity of the Oceanic Shoals Commonwealth Marine Reserve is well represented in the western part of the reserve, with numerous banks and terraces providing hard substrate for benthic communities;

The epibenthic biodiversity on banks appears to vary as a function of water depth and related light and turbidity conditions, with shallower banks (<45 m) supporting more biodiversity than deeper banks, including hard corals;

Species richness and endemism of sponges in the western sector of the Oceanic Shoals CMR may not be as high as those in the eastern sector, with sponges from the west comparatively dominated by species that are common across northern Australia (to be confirmed by taxonomic analysis);

Spatial gradients in epibenthic biodiversity exist as a possible function of marked changes in substrate, light and turbidity levels along the depth transition from bank to terrace to plain;

Tidal currents play an important role in regulating levels of suspended sediment (turbidity) and in redistributing sediment across the plains and around banks and terraces, with some smaller banks partly buried by sediment;

Demersal fish communities respond to spatial patterns in benthic biodiversity, occurring in larger and more diverse populations on the shallower, less turbid banks, and;

A wide variety of high-order pelagic fish species occur in these waters.

The data and information collected on this survey will be used with pre-existing data collected from the eastern part of the Oceanic Shoals CMR to develop regional scale maps and models of benthic habitats, ecosystem processes and associated biodiversity. This analysis is part of ongoing research within the Marine Biodiversity Hub and will enable an assessment of the uniqueness of habitats and biological communities observed in this survey. It will also allow the Oceanic Shoals CMR to be placed into the broader regional context of tropical northern Australia. Importantly, all available materials and data have been subject to consistent sampling and processing protocols, ensuring a uniform data standard for analysis and interpretation. Combined with the sampling design used in this survey, these consistent data standards allow the information to be potentially used as a baseline for long-term monitoring of the CMR.

# Introduction

This report provides details of activities jointly undertaken by the Australian Institute of Marine Science (AIMS), Geoscience Australia (GA), the University of Western Australia and the Museum and Art Gallery of the Northern Territory during a marine biodiversity survey to the Oceanic Shoals Commonwealth Marine Reserve in the Timor Sea in September-October 2012 (Figure 1.1). The survey was an activity within the Australian Government’s National Environmental Research Program Marine Biodiversity Hub and is the key component of Theme 4 – Regional Biodiversity Discovery to Support Marine Bioregional Plans.

Data collected during the survey will be used to support research being undertaken in other Themes of the Marine Biodiversity Hub, in particular the development of regional scale models of ecosystem processes and resultant biodiversity. This and related products will also support the work programs of the Department of the Environment (previously Department of Sustainability, Environment, Water, Population and Communities), including informing the management of the North and North-West Marine Regions. Overall, these new data will also add to the knowledge base of Australian tropical shelf habitats and contribute to the long term management of these poorly understood areas.

## Background and Survey Aims

Marine habitats in Northern Australia host globally significant levels of biodiversity but remain poorly known (Richardson et al., 2009; Tittensor et al., 2010; Fisher et al., 2011). This biodiversity faces rapidly increasing pressures from human activities (i.e. offshore energy industry and fishing), while extensive regional-scale knowledge gaps threaten to compromise efforts to conserve and manage it (Commonwealth of Australia, 2008). This survey was designed to begin to fill these knowledge gaps in one of the most poorly known regions of Northern Australia, the Oceanic Shoals Commonwealth Marine Reserve (CMR), through representative sampling of both the physical environment and biological communities.

The purpose of the survey was to collect geophysical and biological data on the shallow seabed environments for targeted areas within the Oceanic Shoals CMR (Figure 1.1). Proclaimed as a Commonwealth Marine Reserve in November 2012, the Oceanic Shoals CMR is part of the National Representative System of Marine Protected Areas. The Reserve incorporates extensive areas of carbonate banks and terraces that are recognised in the North and North-West Marine Region Plans as a Key Ecological Feature (Commonwealth of Australia, 2012a, b). Previous seabed surveys to the Timor Sea undertaken by AIMS and Geoscience Australia in 2009 and 2010 provided information on habitats and shelf processes for the carbonate banks in the eastern part of the Oceanic Shoals CMR (Heap et al., 2010; Anderson et al., 2011), and these were summarised in a regional overview of seabed habitats and geo-hazards by Przeslawski et al. (2011). This survey extended this work by focusing on shelf habitats (< 200 m water depth) of the western part of the Oceanic Shoals CMR and included potential biodiversity hotspots such as pinnacles and banks. The survey also adds to previous research of open ocean shoals and reefs of the Timor Sea undertaken by AIMS (Heyward et al., 1997).

## Study Area

The Oceanic Shoals CMR covers 71,740 km2 and incorporates a range of large-scale geomorphic features on the continental shelf, including parts of the Van Diemen Rise, Malita Shelf Valley and the Londonderry Rise (Figure 1.1). Both the Van Diemen Rise and Londonderry Rise are characterised by a complex morphology of shallow flat-topped carbonate banks and terraces, separated by valleys and plains (Van Andel and Veevers, 1967; Heap and Harris, 2008). Water depths range from 10 m on the shallowest banks to over 200 m in valleys. In contrast, the Malita Shelf Valley is a broad basin that separates the two rises with water depths of 100 to 150 m. Overall, this seabed morphology is interpreted as a drowned landscape of coastal lowlands, estuarine valleys and nearshore environments that formed during the Late Quaternary Period and was inundated during the post-glacial rise in sea level of the Late Pleistocene to Holocene Epochs (West et al., 1992).

Figure 1.1 Location map of areas surveyed during the Oceanic Shoals Commonwealth Marine Reserve Biodiversity Survey.

Today, these tropical shelf environments are influenced by a range of oceanographic conditions. The Indonesian Through-Flow delivers relatively warm (~26 to 31°C) low salinity water from the north, whereas strong tidal flows, wind driven waves and seasonal cyclones influence sediment transport particularly across the banks. Coupled with sediment discharged from coastal river systems, these re-suspended seabed sediments contribute to a turbid nearshore and shelf environment that is likely to suppress photosynthetic activity at the seabed, and therefore may be an important determinant of the composition of benthic assemblages across the region.

The survey acquired data across four areas located along the transition between the Malita Shelf Valley and Londonderry Rise and spanning a cross-shelf transect of approximately 110 km
(Figure 1-2). Each area was positioned to encompass a variety of seabed geomorphic features and water depths, as identified in the national geomorphic features database (Heap and Harris, 2008), as follows:

Area 1 is the southernmost survey area covering 18 km x 4.2 km along a north-south orientation and incorporates a bank and terrace feature at the southern end, several pinnacles that rise to about 40 m water depth and plains in up to 150 m water depth.

Figure 1.2 Map showing geomorphic features within the western part of the Oceanic Shoals Commonwealth Marine Reserve, with survey areas indicated. Geomorphic features are based on the Australian 250 m bathymetry grid (published October 2006), as described by Heap and Harris (2008).

Area 2 is located 30 km to the northwest of Area 1, covers 18.6 km x 7.6 km on a northwest to southeast alignment and includes a bank and terrace at the northern end, pinnacles, valley and plains. Water depths range from 40-135 m.

Area 3 is located 34 km directly to the north of Area 2, covers 22 km x 7 km along an east-west orientation and is characterised by a broad bank and terrace feature in the western half, pinnacles and surrounding plains in the eastern half. Water depths range from 55-110 m.

Area 4 is located 5 km to the north of Area 3, covers 7.2 km x 3.6 km on an east-west orientation and includes two banks that rise to 40 m water depth surrounded by plains in up to 115 m water depth.

# Methods

## Survey Overview

Survey SOL5650/GA0339 was undertaken between 12 September and 6 October 2012, using the AIMS 35 m research vessel Solander with a science and technical support crew from AIMS, Geoscience Australia, University of Western Australia and the Museum and Art Gallery of the Northern Territory (Appendix A). The survey comprised a series of geophysical mapping and biophysical sampling activities in four areas within the Oceanic Shoals CMR. In each survey area, multibeam sonar data (bathymetry and backscatter) was collected to provide 100% spatial coverage, with sampling stations selected before the survey. Site selection used a spatially-balanced random stratified method (termed a Generalised Random-Tessellation Stratified [GRTS] design; Stevens and Olsen, 2004), that was weighted to shallow areas (<50 m water depth). Sampling included non-destructive techniques (towed underwater video, stills photography, baited underwater video) to record benthic and pelagic biota, plus physical sampling of sediments and epibenthic biota (grabs, benthic sled tows). Water column measurements (conductivity, temperature, turbidity) and sea surface samples were also taken at representative stations and during transits. To broadly characterise the sub-surface geology, shallow (<70 m) sub-surface data was collected across three of the survey areas (Areas 1, 2 and 3) with a sub-bottom profiler.

## Sampling Overview

Sampling was undertaken at 70 stations across the four survey areas to characterise the geophysical and biological properties of the seabed and water column within each area; with sea surface samples collected at a further nine stations during transits (Table 2.1). At each station, the type of sampling undertaken and the gear used was determined by the seabed composition, water depth and visibility. In particular, deployment of baited remote underwater video stations (BRUVS) was limited to water depths less than ~60 m due to high turbidity and low light conditions.

Sampling activities within the four survey areas included:

Towed underwater video transects at 52 stations to characterise seabed types and benthic communities;

Collection of replicate sediment samples at 61 stations for sedimentology, geochemistry and infauna analyses using either a Smith-McIntyre grab (29 stations) or a box corer (32 stations);

Benthic sled tows at 22 stations to sample epibenthos;

Deployment of BRUVS at 56 stations to characterise demersal fish communities;

Deployment of Stereo Imagery System for Shark and Tuna Assessments (SISSTAs) units at 120 stations to characterise pelagic fish communities;

Deployment of a Conductivity Temperature Depth (CTD) instrument at 63 stations to characterise water column properties, and;

Deployment of 10 ocean drifters to map surface ocean currents in the region.

Table 2.1 List of sampling and tow video operations by station. Deployment details for BRUVS, SISSTAs and drifters are listed separately in the Appendices. An asterisk denotes an unsuccessful deployment. Detailed descriptions of sampling are included in the voyage leader log (Appendix O)

| Survey Area | Station | Tow Video | Sediment Grab | Box Core | Benthic Sled | CTD | Water Sample |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transit | 01 | - | - | - | - | - | 01 |
| Transit | 02 | - | - | - | - | - | 02 |
| Transit | 03 | - | - | - | - | - | 03 |
| Transit | 04 | - | - | - | - | - | 04 |
| 1 | 05 | 01 | 01 | - | - | 01 | - |
| 1 | 06 | 02 | 02, 03 | - | 01 | 02 | - |
| 1 | 07 | 03 | 04, 05 | - | - | 03 | - |
| 1 | 08 | 04 | 06, 07 | - | - | 04 | - |
| 1 | 09 | 05 | 08, 09 | - | - | - | - |
| 1 | 10 | 06 | 010, 11, 12 | - | - | 05 | - |
| 1 | 11 | 07 | 13, 14 | - | - | 06 | - |
| 1 | 12 | 08 | - | - | - | 07 | - |
| 1 | 13 | 09 | - | - | - | - | - |
| 1 | 14 | 10 | 15, 16, 17, 18 | - | 02 | 08 | - |
| 1 | 15 | 11 | 19, 20 | - | 03 | 09 | - |
| 1 | 16 | 12, 13 | 21, 22 | - | 04 | 10 | - |
| 1 | 17 | 14 | - | 01, 02 | - | 11 | - |
| 1 | 18 | - | 23, 24 | - | - | 12 | - |
| 1 | 19 | - | 25, 26 | - | - | 13 | 05 |
| 1 | 20 | - | - | 03, 04 | - | 14 | - |
| 1 | 21 | - | - | 05, 06 | - | 15 | - |
| 1 | 22 | - | - | 07, 08 | - | 16 | - |
| 1 | 23 | - | - | - | - | 17 | 06 |
| 2 | 24 | 15 | - | 09, 10, 11 | 05 | 18 | - |
| 2 | 25 | 16 | 27 | 12 | 06 | 19 | - |
| 2 | 26 | 17 | 28, 29 | - | 07 | 20 | 07 |
| 2 | 27 | 18 | - | 14, 15 | - | 21 | - |
| 2 | 28 | 19 | - | 16, 17 | - | 22 | - |
| 2 | 29 | 20 | 30, 31 | - | 08 | 23 | - |
| 2 | 30 | - | - | - | - | 24 | 08 |
| 2 | 31 | 21 | - | 18, 19 | 09 | 25 | - |
| 2 | 32 | 22 | - | 20, 21 | - | 26 | - |
| 2 | 33 | 23 | - | 22, 23 | - | 27 | - |
| 2 | 34 | 24 | - | 24, 25, 26 | - | 28 | - |
| 2 | 35 | - | - | - | - | 29 | - |
| 2 | 36 | 25 | - | 27, 28 | 10 | 30 | - |
| 2 | 37 | 26 | 32 | 29 | 11 | 31 | - |
| 2 | 38 | 27 | - | 30, 31 | 12 | 32 | - |
| 2 | 39 | 28 | - | 32, 33 | - | 33 | - |
| 2 | 40 | 29 | - | 34, 35 | - | 34 | - |
| 2 | 41 | - | 33, 34 | - | - | 35 | - |
| 2 | 42 | - | 35, 36 | - | - | 36 | - |
| Transit | 43 | - | - | - | - | - | 09 |
| 3 | 44 | 30 | 37 | 36 | 13* | 37 | - |
| 3 | 45 | 31 | - | 37, 38 | - | 38 | - |
| 3 | 46 | 32 | - | 39, 40 | 14 | 39 | - |
| 3 | 47 | 33 | 38, 39 | - | 15 | 40 | - |
| 3 | 48 | - | - | - | - | 41 | 10 |
| 3 | 49 | 34 | 40, 41 | - | - | 42 | - |
| 3 | 50 | 35 | - | 41, 42 | - | 43 | - |
| 3 | 51 | 36 | - | 43, 44 | - | 44 | - |
| 3 | 52 | 37 | - | 45, 46 | - | 45 | - |
| 3 | 53 | 38 | - | 47, 48 | 16 | 46 | - |
| 3 | 54 | 39 | - | 49, 50 | - | 47 | - |
| 3 | 55 | 40 | 42, 43 | - | 17 | 48 | - |
| 3 | 56 | 41 | - | 51, 52 | 18 | 49 | - |
| 3 | 57 | 42 | - | 53, 54, 55 | - | 50 | - |
| 3 | 58 | 43 | - | 56, 57 | - | 51 | - |
| 3 | 59 | - | - | - | - | 52 | - |
| 3 | 60 | - | 44, 45 | - | - | - | - |
| 3 | 61 | 44 | - | 58, 59 | 19 | 53 | - |
| 3 | 62 | 45 | - | 60, 61 | - | 54 | - |
| 3 | 63 | 46 | - | 62, 63 | 20 | 55 | - |
| 3 | 64 | - | - | - | - | - | 11 |
| 3 | 65 | - | 46, 47 | - | - | 56 | - |
| 3 | 66 | - | 48, 49 | - | - | 57 | - |
| 4 | 67 | 47 | 50 | - | - | 58 | - |
| 4 | 68 | - | 51, 52 | - | - | 59 | - |
| 4 | 69 | 48 | - | - | - | - | - |
| 4 | 70 | 49 | - | - | - | - | - |
| Transit | 71 | - | - | - | - | - | 12 |
| 2 | 72 | - | - | - | - | 60 | - |
| 1 | 73 | 50 | 53, 54 | - | 21 | 61 | - |
| 1 | 74 | 51 | 55 | 64 | 22 | 62 | - |
| 1 | 75 | 52 | - | 65, 66 | - | 63 | - |
| Transit | 76 | - | - | - | - | - | 13 |
| Transit | 77 | - | - | - | - | - | 14 |
| Transit | 78 | - | - | - | - | - | 15 |
| Transit | 79 | - | - | - | - | - | 16 |

## Geophysical Data Acquisition and Processing

### Bathymetric Mapping

High-resolution bathymetric data were collected from each survey area using a Kongsberg EM3002D (300 kHz) multibeam sonar system mounted in single head configuration (Figure 2.1). The multibeam sonar data were processed during the survey using Caris HIPS & SIPS v7.1 SP2 software and included: i) applying algorithms that compensated for tide and vessel pitch, roll and heave (e.g. elevation errors caused by dynamic draft of the vessel); ii) using sound velocity profiles (collected while underway and at several stations in each survey area) to correct for variations in the speed of sound through the water column, and; iii) software filters and visual inspection of each swath line to remove any remaining artefacts and noisy data (e.g. nadir noise and data outliers).

Figure 2.1 Kongsberg EM3002D multibeam sonar system installed on RV Solander, showing: (a) POS MV motion reference unit, data acquisition and processing units, and; (b) single head mounting deployed in the moon-pool.

Tidal corrections were also applied to bathymetric data, based on a co-tidal solution. This solution is derived from the average tidal amplitude at multiple tide stations weighted inversely to the distance between the tide station and the point of interest. For each survey area, a tidal zone was defined by a closed polygon and included a primary tide station of known location and two or more secondary tide stations. Predicted tides were then generated using tidal constituents requested from the National Tidal Centre within the Bureau of Meteorology for these predefined tide stations. A Zone Definition File (.zdf) for defining the zone, the primary and secondary tide stations, and the associated predicted tides was created and used in Caris HIPS & SIPS V7.1 SP2. The final surface grid (bathymetric map) was then produced to ≤10 cm vertical resolution and 2 m horizontal resolution (cell size of the surface grid).

### Backscatter Acquisition and Processing

Seabed reflectance (backscatter), which is a surrogate for substrate hardness, was processed using the multibeam backscatter CMST-GA MB Process v10.10.17.0 toolbox software co-developed by the Centre for Marine Science and Technology (CMST) at Curtin University of Technology and Geoscience Australia (described in Gavrilov et al., 2005). Processing steps included correction for transmission loss and ensonification area, removal of the system model, calculation of the incidence angle and removal of angular dependence. The angularly equalised backscatter strengths were normalised to the backscatter strength at an angle of 25°. The final processed backscatter data were then gridded to 2 m horizontal resolution.

### Sub-Bottom Profiler Acquisition and Processing

Shallow (<100 metres below sea floor) sub-surface data were collected along representative transects in Areas 1, 2, and 3 using an Applied Acoustics CSP-D “Sparker” sub-bottom system (Figure 2.2; Appendix B). The system comprised a Squid-2000 sound source (2000 J) towed 18 m astern of the vessel at a depth of ~30 cm and a single channel hydrophone for data collection that was deployed 33 m astern approximately 1 m below the surface. Vessel speeds during acquisition ranged from 3.5 to 4 knots (6.5 – 7.5 km h-1) in Beaufort Sea States of 0 to 2. Data was processed using a C-View (Applied Acoustics Engineering) processing unit firing at a rate of 1 second with a record length of 500 milliseconds. The data were processed by applying a band-pass filter (60-600 Hz) and converted to SEGY for further processing. Depths were converted from two-way travel time (TWT) using 1,500 m s-1 as the speed of sound in the water column and shallow sub-surface sediments.

Figure 2.2 Applied Acoustics sub-bottom “Sparker” system, showing: (a) CSP-D generator unit and; (b) Squid-2000 sound source.

## Oceanographic Data Acquisition

### Water Column Measurements and Sampling

To characterise the vertical structure of the water column across the banks, terraces and valleys of each survey area, measurements were collected by CTD casts at 63 stations (Table 2.1). At each CTD station, a Seabird Electronics SBE-911plus Livewire CTD was deployed using the hydrographic wire (Figure 2.3), and lowered through the water column to within 1–2 m of the seabed. A conducting wire (Livewire) between the ship and the CTD allowed serial information to be transmitted down to the CTD and real-time data to be transmitted back. During each deployment, the CTD continually measured conductivity (mS/cm), irradiance (%), depth (m), fluorescence (mg/m3), nitrogen saturation (mg/l), oxygen saturation (mg/l), salinity (PSU) and temperature (degrees C). In addition to CTD casts, a hull-mounted Seabird Electronics SBE-21 thermosalinograph measured temperature and conductivity (≈salinity) of the surface water (<5 m below the sea surface) every 10 seconds for the duration of the voyage.

Samples of surface waters were also collected at 16 stations during transits (9 samples) and from within the survey areas (7 samples), using the on-board seawater circulation unit. At each station, approximately 10 litres of seawater was collected in buckets and filtered to retain suspended solids and chlorophyll-a, respectively. The timing of sample collection was scheduled to coincide with the overpass of the MODIS satellite, which remotely senses sea surface colour as a proxy for suspended solids and chlorophyll content in ocean waters.

Figure 2.3 Seabird Electronics SBE-911plus Livewire CTD prior to deployment.

### Ocean Current Drifters

To provide insight into the circulation pattern of sea surface waters across the survey areas, ten ocean drifters were deployed at strategic locations during the survey. The drifters were sourced from the Global Ocean Drifter Program (GDP) run by the National Oceanic & Atmospheric Administration (NOAA) in the United States (www.aoml.noaa.gov/phod). The drifters are expendable instruments weighing 20 kg and comprise a surface buoy and subsurface drogue (sea anchor) designed to drift in response to near-surface currents (Figure 2.4). The particular drifter model deployed collects sea surface temperature and transmits this data to the Argos satellite array, with the location of a drifter calculated from the Doppler shift of its transmission signal as seen by satellites (with 16-20 fixes taken per day). Interpolated positions are periodically loaded onto the GDP website for free download. Details of the drifters deployed during the survey are listed in Appendix D.

Figure 2.4 Ocean drifter prior to deployment.

## Physical Sampling

### Sedimentology

The principal aim of the sedimentology component of the survey was to determine the texture and composition of the surface sediments for seabed characterisation and to provide samples for analysis of infauna. For physical sedimentological analysis, unconsolidated sediments were collected at 62 stations using a Smith-McIntyre grab or a box corer (Table 2.1, Figure 2.5; Appendix E).

Up to 100 g of bulk sediment was sub-sampled from the top 2 cm of grabs and box cores for texture and compositional analysis. Samples were submitted to the GA sedimentology laboratory for measurement of gravel, sand and mud content by sieve separation and of particle size distributions (including mean, median, standard deviation, skewness and kurtosis indices) by laser diffraction using a Malvern Mastersizer 2000 particle size analyser. Separate sample splits were submitted for measurement of carbonate content using the carbonate digestion method (Müller and Gastner, 1971).

Figure 2.5 Images of the sediment sampling equipment employed on this survey. (a) A Smith McIntyre grab full of sediment, sitting in its frame. (b) A returned box corer, full of sediment.

### Geochemistry

The principal aims of the geochemistry component of the survey were to: (i) determine the geochemical composition of surface sediment for seabed characterisation; and (ii) quantify the levels of reactive bulk organic matter and chlorin pigments in sediments that support biodiversity in the region. Unconsolidated surface sediments designated for geochemical analysis were collected at 57 stations using either a Smith-McIntyre grab (10L, 0.1 m2 opening) or a box core (4.5L, 0.023 m2 opening). The grabs were sub-sampled into seven or eight separate containers for the parameters listed in Appendix G, Table G.1. An account of the shipboard processing, laboratory pre-processing and analytic techniques to be undertaken on these samples is provided in Appendix H, Table H.1 and Figure 2.6. The chlorin, sediment oxygen demand and bottle/core incubation data will be used to estimate levels of reactive organic matter that support biodiversity in the region. The elemental data, including Total Organic Carbon (TOC), Total Nitrogen (TN) and their isotopic ratios, will be used in conjunction with the organic matter abundance/reactivity indicators in multivariate analyses to deduce the major geochemical gradients in the region.

Figure 2.6 (a) Extraction of sediment cores from the Smith McIntyre grab; (b) Incubation of cores under different light treatments; (c) Example sediment core complete with stirrer, and; (d) Example of a core, post-incubation, showing sulphide accumulation around a worm burrow. The sulphide build-up is due to the occurrence of reactive carbon in the degrading worm.

### Biology

The aim of the biological component of this survey was to characterise biological diversity and assemblages on prominent geomorphic features using several sampling methods. To quantify the bio-physical structure of these seabed environments, 57 stations (37 from banks, 13 plain, 6 terraces, and 1 valley) were surveyed and/or sampled for biology. At each of these stations, a towed-video transect was undertaken to characterise the physical structure of the seabed by substratum type, geomorphology, and topographic relief; while the biological assemblage was quantified as the presence of macro-organisms and the percentage cover of key taxa, such as sponges and octocorals. Infaunal animals (living beneath the seafloor) were sampled from 57 stations and 111 sediment samples, and epibenthic organisms (living on the seabed) were successfully sampled from 21 stations over banks.

#### Towed-video

Seabed habitats and benthic macro-organisms were surveyed by video and stills camera along 52 transects (17 in Area 1, 15 in Area 2, 17 in Area 3, and 3 in Area 4: Table 2.1; Appendix I) using the AIMS towed-camera system (Figure 2.7). This system was fitted with a single forward-facing video camera (Watec colour D250 model, 4 mm lens) and two high-resolution still cameras (Sea&Sea DX2G 12 mega pixel: one forward-facing and one downward-facing) and their associated lights (Figure 2.7a). At each station, an approximately 1500-m long video transect (approximately 50 minute duration) was completed, along with high-resolution still photographs captured at least every 5 seconds. Most transects ran over multiple geomorphic features and depths in order to capture habitat transitions and gradients. The camera system was deployed from the stern of the RV Solander, and towed at 0.5 to 1.5 knots at an altitude of 0.5 - 2 m above the seabed. To accurately correlate the position of seabed video and images with physical features in multibeam maps, the position of the towed-camera system was tracked using a USBL (Ultra-short Baseline) acoustic tracking system (Figure 2.7b). Video footage was transmitted in real-time to the surface via coaxial cable to enable observers to characterise the seabed environment and allow the winch operator to regulate the altitude of the towed-camera system (Figure 2.7c). Video was characterised in real-time on the vessel using the AIMS’ TOWVID software and classification scheme.

Figure 2.7 Images of the AIMS towed-video system, showing: (a) Deployment of AIMS towed-video system of the stern of the RV Solander; (b) Monitor output from the USBL acoustic tracking system, and; (c) Monitor showing real-time video footage as seen by the towed-video system.

#### Infaunal Sampling

Seabed infauna was sampled from 57 stations using either a Smith-McIntyre grab or box corer (Table 2.1, Appendix E). For each sample collected, a small amount of sediment (<100 g wet weight) was removed for sedimentological analysis (Section 2.5.1). For some samples, geochemical sub-samples were also removed. The remaining sediment was then released into a 52 litre Nally Bin (Figure 2.8a), excess water was decanted through a 500 μm sieve, and the sample including the bin was weighed. Sediments were then separated from infauna by elutriating in water for 5 minutes (Figure 2.8b), with infauna collected on a 500 μm sieve at the mouth of a release pipe (Figure 2.8c). Excess water was then drained from the Nally bin, and the remaining sediments (heavy fraction) searched by hand for heavier organisms (e.g. gastropods, ophiuroids, holothurians, heart-urchins and hermit crabs). These larger organisms and the elutriated material retained on the sieve were then photographed. Larger organisms were preserved/fixed in ethanol (most taxa) or formalin (polychaete worms) to be sent to the Museum & Art Gallery of the Northern Territory (MAGNT) for cataloguing and taxonomic identifications. The elutriate material was removed and preserved in ethanol to be sent to Geoscience Australia for sorting and identification to operational taxonomic unit (OTU).

Figure 2.8 Images of the elutriating process used for infaunal sampling, showing: (a) Sediment collected from a grab sample in Nally Bin prior to elutriating; (b) Scientist elutriating grab sample with a 500 μm sieve to collect suspended organisms, and; (c) Photograph of organisms collected by sieve after 5 minutes of elutriation.

#### Epifaunal Sampling

Seabed epibenthos (epifauna and epiflora) was successfully sampled from 21 stations (all banks) using AIMS’s epibenthic sled (Table 2.1; Figure 2.9; Appendix J). The sled is 1.5 m wide x 1 m high and fitted with a 6-m long 45 mm stretch diamond-mesh net. At each station, the sled was towed along the seabed at 1.5-2 knots for approximately 50 m. Upon retrieval, the net was emptied onto a large sorting table where researchers separated biological specimens into broad taxonomic groups (e.g. sponges, gorgonians, crinoids) (Figure 2.9b-c). The total number of animals (abundance), species (species richness) and wet weight (±10 g) for each taxonomic group was then recorded, and representatives of each taxa were photographed. Sample weights <100 g were not specified. Taxonomic voucher specimens were then preserved in ethanol (most taxa) or formalin (worms, fishes and ascidians) for post-survey identification and cataloguing by Museum Victoria (Echinoderms) and the MAGNT (all other taxa). Biological collections of infauna and epibenthos will be identified to species-level or lowest operational taxonomic units (OTU) and, in conjunction with infaunal identifications, will contribute to subsequent publications on the regional-scale biodiversity of the Timor Sea.

Figure 2.9 Examples of the epibenthic sampling process using the AIMS sled, showing: (a) Deployment of the epibenthic sled from the stern of the RV Solander; (b) A full sled net being released onto the sorting table, and; (c) Biological samples from a sled being sorted into broad taxonomic groups.

#### Biological Material for Biodiscovery

Since 2002, the Museum and Art Gallery of the Territory (MAGNT) has been engaged in a cooperative biodiscovery program with the US-based Coral Reef Research Foundation and the National Cancer Institute (NCI). With the collaboration of the Australian Bioresources Library (ABL) at AIMS, samples collected from Commonwealth waters have been included in this collection program. These samples are used to test and synthesise potentially useful compounds for pharmaceutical use. Biodiscovery samples collected in the current survey will be exported to the NCI under existing agreements and permits through the ABL, which will allow biodiscovery research and commercialisation while protecting the resource provider interests in the collections. These will add to over 1000 samples which have already been sent to NCI for screening.

From biota collected from the benthic sled, selected samples (generally with weights greater than 50 g) were prepared for biodiscovery research and stored in plastic bags at -20o C (Figure 2.10a-c). A total of 139 biodiscovery samples were collected on this survey, including primarily sponges as well as bryozoa, octocorals, echinoderms, and ascidians (Appendix K).

Figure 2.10 Examples of biodiscovery operations, including: (a) Biologists and crew of RV Solander sorting through a epibenthic sled sample for materials that have biodiscovery potential; (b) Sponges (centre) and octocorals (in green bucket) selected for further analysis, and; (c) Senior biologist identifying, cataloguing and processing samples with high biodiscovery potential.

#### Sampling of demersal fish diversity and abundance

Baited Remote Underwater Video Stations (BRUVS) have been widely used in tropical Australian waters (and elsewhere) to sample demersal fish, shark, ray and sea snake communities (e.g. Cappo et al., 2004; 2007; Schultz et al., 2012). BRUVS are generally considered the best non-destructive sampling method for fishes of all sizes and from all functional groups in non-diveable water depths. As a visual technique, the use of BRUVS is reliant on adequate underwater visibility in order to effectively identify and quantify fish visitation to the video stations.

The ability to use BRUVS to sample demersal fishes in the Oceanic Shoals survey area was limited by very poor visibility (generally <5 m) at the seabed due to tidally mobile soft sediments and a consequent lack of ambient light in waters deeper than 60 m. As a result, initial plans to deploy BRUVS throughout each survey area according to a GRTS sampling design were abandoned in favour of limited sampling only in shoal areas of ~60 m or less water depth.

During the survey, 56 stereo BRUVS were deployed for one hour according to a regular random sampling design, with minimum spacing of 400 m to ensure independence among samples (Appendix L). This included 16 BRUVS on single banks of <60 m water depth in each of Areas 1 and 2, and 24 BRUVS on each of two banks of <60 m water depth in Area 3. This yielded 56 hours of video footage which was subsequently analysed and all species and their relative abundances recorded.

Detailed analysis of the fish community data is presently ongoing and will be reported elsewhere. Preliminary analysis reveals the presence of a diversity of commercially important (e.g. deep water snappers such as Pristipomoides sp.) and non-commercially exploited species of fishes including sharks (Figure 2.11). Some sea snakes were also recorded. The extremely poor visibility was a limitation of the use of BRUVS in this study area and the consequent limited field of view of these samples, as well as the inability to reliably quantify benthic habitat in the field of view, is likely to affect the direct comparability of the fish community recorded with other surveys in the broader region where visibility is typically better. Nevertheless, the data obtained will provide some insight into the fish communities of the shallower waters of the Oceanic Shoals region and their associations with other shoal areas that have previously been surveyed in northern Australia.

Figure 2.11 Imagery obtained from BRUVS deployed during the Oceanic Shoals survey: (a) Video frame grab of a giant Queensland groper (Epinephelus lanceolatus); (b) Still image taken by a camera mounted to a BRUVS frame showing a spangled emperor (Lethrinus nebulosus) and big-eye trevally (Caranx sexfasciatus).

#### Video sampling (SISSTAs) and visual surveys of marine predators

High-order mobile predators such as seabirds, marine mammals, large pelagic fish (e.g. billfish, tuna, or marlin) and sharks play a key role in maintaining biodiversity and are widely recognised as sentinels of ocean health. As such, they are often regarded as valuable ecosystem indicators and used to inform conservation planning and wildlife management initiatives. Prior to this survey, available data on the distribution and abundance patterns of these species in the Oceanic Shoals region were very limited.

To fill in these knowledge gaps, the following methods were conducted:

Visual surveys of cetaceans and seabirds from the RV Solander’s top deck, both within and in transit between survey areas. Angles and distances to sightings were measured using reticle binoculars and additional data (group size, group composition, behaviour etc.) were recorded and logged into a purpose-built database.

Mid-water video monitoring using a new baited stereo imagery system (named SISSTAs – Stereo Imagery System for Shark and Tuna Assessments). These mid-water systems are fitted with a bait chamber to attract wildlife into the field of view of two high-resolution GoPro Hero2 cameras mounted on a central frame. The cameras are set up in stereo to allow for the calculation of individual animal lengths, and units are anchored to the seabed and tethered to a line of surface buoys to enable sampling at a variety of pre-determined depths. Units were deployed manually in sets of 10 and soaked for a minimum of two hours. Video data were uploaded and backed up immediately following unit retrieval.

During 18 hours of observations, 102 sightings of more than 300 seabirds, fish, turtles, and cetaceans were made. Mid-water systems were deployed at 117 stations at an average rate of 10 deployments per day, generating in excess of 700 hours of underwater video footage (Appendix M). Preliminary review of footage indicates that a wide range of pelagic species were observed by the systems (Figure 2.12).

Figure 2.12 Examples of high-order marine species observed within the Oceanic Shoals CMR, including: (a) Bottlenose dolphin (Tursiops truncatus); (b) Killer whales (Orcinus orca); (c) Mid-water video observations of Australian blacktip shark (Carcharhinus tilstoni) and; (d) Manta ray (Manta birostris).

# Preliminary Results & Interpretations

Multibeam sonar (including bathymetry, backscatter and water column) data were collected from a total area of 507 km2, divided between the four survey areas as follows: 165 km2 in Area 1; 155 km2 in Area 2; 158 km2 in Area 3, and; 29 km2 in Area 4 (Table 3.1). In tandem with the bathymetric mapping 141.6 line-km of sub-bottom profiles were collected over three of the survey areas, as follows: 63.6 km in Area 1; 63 km in Area 2 and 15 km in Area 3 (Table 3.1; Appendix B).

Table 3.1 Areas mapped by acoustic multibeam and sub-bottom profile activities in the Oceanic Shoals CMR Survey (SOL5650), excluding transit lines.

| Survey Area | Acoustic Multibeam (km2) | Sub-bottom Profile (km) |
| --- | --- | --- |
| 1 | 165 | 63.6 |
| 2 | 155 | 63.0 |
| 3 | 158 | 15 |
| 4 | 29 | 0 |
| Total | 507 | 141.6 |

Within these four areas six geomorphic features were identified and mapped: bank; depression; mound; plain; scarp, and; terrace (Table 3.2). No single area contained all features, with Areas 1 and 2 being more geomorphically diverse than Areas 3 and 4, with five and four features respectively (Table 3.3). The method used to construct these interpreted geomorphic maps is described in 0.

Table 3.2 Summary of the geomorphic features mapped with definitions and datasets used to produce them. Feature definitions were based on those used by Heap and Harris (2008).

| Feature | Definition | Datasets Used |
| --- | --- | --- |
| Bank | Local or regional areas of elevated seafloor with one or more steep sides | 2 m x 2 m Bathymetric Grids; Slope; 1 m Contour |
| Depression | Generally enclosed bathymetric lows on the seafloor that would act as a sediment trap. | Slope; 1 m Contour |
| Mound | Local areas of elevated seafloor with gently sloping rounded sides. | Slope; 1 m Contour |
| Plain | Extensive, flat or gently sloping areas | 2 m x 2 m Bathymetric Grid; Slope; 1 m Contour |
| Scarp | Elongated and comparatively steep (≥10o) slope separating more gently sloping areas | Slope |
| Terrace | Relatively flat or gently sloping seafloor with a moderately sloping to steep rise on one side and a moderately sloping to steep drop on the other side. | 2 m x 2 m Bathymetric Grids; Slope; 1 m Contour |

Table 3.3 Summary statistics for areas surveyed and their geomorphic diversity.

|  | Min Depth (m) | Max Depth (m) | Modal Depth (m) | Number of Geomorphic Feature Types |
| --- | --- | --- | --- | --- |
| Area 1 | 35 | 180 | 109 | 5 |
| Area 2 | 50 | 146 | 111 | 5 |
| Area 3 | 57 | 169 | 106 | 4 |
| Area 4 | 42 | 125 | 112 | 4 |
| All Areas | 35 | 180 | 109 | 6 |

## Seabed Characteristics Area 1

Area 1 is the southernmost area and has the greatest range of water depths, varying between 35 and 180 m (Table 3.4; Figure 3.1). Covering 165 km2, Area 1 incorporates five of the six main geomorphic features mapped, including 81 km2 of plains, 69 km2 of banks, 12 km2 of depressions, 5 km2 of terraces and <1 km2 of scarps (Table 3.4).

Eight banks were identified and mapped in Area 1 (Figure 3.3). They have a modal depth of 74 m but exist in water depths ranging between 45 and 118 m. Banks have flat tops and relatively steep sides, ranging between 2 and 35o (Figure 3.4 a and e; 0), although the northern and western margins of banks generally have lower gradients than the southern and eastern margins. Banks and the scarps that define the edges of some banks, had the highest average backscatter values of Area 1, at -35 ± 3 dB (Table 3.4; Figure 3.2), indicating that they are composed of harder substrate than other features. This was confirmed by grab and box corer samples taken on the banks which generally consisted of gravelly coarse sand, gravelly mud and sandy gravels (Appendix E).

The tops of banks in Area 1 have a bimodal depth distribution occurring at either 75 m (n = 7), or 35 m (n = 1). The deeper banks (at 75 m) could represent drowned coral reefs that formed during lower sea level but today occur within the lower limits of the photic zone. The shallow bank in Area 1 also contains a terrace that corresponds to the 75 m depth of the other banks, indicating that this surface represents a second stage of development, built atop of the earlier stage. Banks are also characterised by several smaller scale terraces notched into their flanks (Figure 3.4d and h). This notched morphology was probably formed at sea level and therefore may contain information on previous sea-level conditions during the Late Quaternary period.

Pockmarks are a seabed feature also commonly associated with the banks in Area 1, where they have formed fields of dense overlapping clusters around the base of banks and in lower density on the tops of banks (Figure 3.4 c and g). Individual pockmarks have a diameter around 13-22 m and a depth of approximately 0.6-1.4 m. Such pockmark clustering around banks has previously been shown to indicate the presence of past or current fluid or gas seepage both within this study region (George and Cauquil, 2010) and more generally (Hovland, 1990; O’Brien et al., 2002). They therefore could indicate a similar correlation between seeps and bank locations in the survey area.

Based on observations from still images (Figure 3.5), the top of the northernmost bank is dominated by sparse to moderate coverage of sessile invertebrates interspersed with expanses of coral rubble and live mushroom corals. The terrace has similar epifaunal communities with the exception of the absence of rubble and mushroom corals. The bank margin supports sparser epifaunal communities likely due to increased dominance of unconsolidated sediments; however, burrows and other signs of bioturbation suggest an abundant infaunal community.

Table 3.4 Summary of physical variables for geomorphic features mapped in Area 1.

|  | Area (km2) | Percentage Total (%) | Min Depth (m) | Max Depth (m) | Modal Depth (m) | Average Slope (o) | Average Backscatter (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bank | 68.71 | 41 | 35 | 118 | 74 | 2.6 ± 2.1 | -35 ± 3 |
| Depression | 12.24 | 7 | 112 | 180 | 124 | 3.3 ± 2.9 | -38 ± 3 |
| Plain | 80.73 | 48 | 94 | 125 | 109 | 1.8 ± 1.8 | -40 ± 4 |
| Scarp | 0.56 | <1 | 45 | 159 | 66 | 16.6 ± 9.3 | -35 ± 3 |
| Terrace | 5.01 | 3 | 82 | 97 | 86 | 5.1 ± 3.8 | -43 ± 3 |
| Total | 167.26 | 100 | 35 | 180 | 109 | 2.4 ± 2.4 | -38 ± 4 |

Depressions occur between bank features in Area 1, with two large and deep depressions (~30 m and ~60 m deep) in the northeast part of the area (Figure 3.3). These deep depressions are unique to Area 1 and are characterised by steep sides along some of their margins. The other depression features identified in Area 1 were generally much shallower (6-10 m). Depressions had a moderate average backscatter value of -38 ± 3 dB (Table 3.4) indicating that they could be of mixed composition, with the steeper flanks generally returning higher backscatter values than the depression floors (Figure 3.2). No grab or box corer samples were taken within these features to confirm this. The location of these features between banks could indicate that they are formed by tidal current scour that is focused by the banks.

Plains are the most widespread geomorphic feature in Area 1 with a modal depth of 109 m (Table 3.4) and an increase in water depth towards the north (gradient ~0.07o). Grabs and box cores taken from plains in Area 1 generally returned sandy silts (Appendix E) which is reflected in the plains low average backscatter value of -40 ± 4 dB (Table 3.4). Individual, non-clustering, pockmarks (with depths between 0.5-2.1 m and diameters between 16-32 m) are also ubiquitous across the soft sediment plains, with a representative example illustrated in Figure 3.4b and f. Pockmarks are seen to have a higher backscatter signal than the surrounding plain (Figure 3.4f) indicating they are composed of a harder substrate and/or larger grainsize. Furthermore, it can be seen that the closer these individual pockmark features are to banks the more frequently they tend to have an asymmetric plan shape, perhaps indicating stronger tidal current action with proximity to banks. If this is correct the direction of the asymmetry could be used to reconstruct local current direction around banks.

Finally, a large terrace feature is mapped in the southwest corner of Area 1 (Figure 3.3) occurring at ~86 m. This terrace contains dense overlapping clusters of pockmarks (Figure 3.4c and g) and returned the lowest average backscatter values -43 ± 3 dB indicating that it is composed of soft sediment with small grain size. This was confirmed by a grab sample which contained poorly-sorted, slightly-sandy silt (Appendix E).

Figure 3.1 False colour bathymetry image of Area 1 with sub-bottom profile (sparker) lines and sampling stations indicated.

Figure 3.2 False colour backscatter image of Area 1 with towed video transects, benthic sled, baited remote underwater video (BRUVS) and baited mid-water column video (SISSTAs) stations indicated.

Figure 3.3 Local-scale geomorphic features mapped in Area 1 interpreted from bathymetry. Images are 30% transparent polygons overlying hill-shaded bathymetric images. Boxes indicate the location of key geomorphic features shown in Figure 3.4.

Figure 3.4 Hill shaded bathymetry (a-d) and backscatter (e-h) images of representative geomorphic features in Area 1, including: bank (a and e); sparse pockmark fields (b and f) with and without ‘tails’; dense pockmark fields (c and g), and; terraced and notched morphology of the sides of banks (d and h). Although these specific examples are in Area 1 they are representative of the other areas surveyed.

Figure 3.5 Representative still images from the northernmost bank in Area 1, showing: (a) live mushroom corals; (b) coral rubble; (c) moderate to dense epifaunal coverage (cnidarians, sponges); (d) the flank of the bank supporting scattered epifauna (sponges, Mopsella gorgonian), and; (e) high levels of bioturbation on the lower flank.

## Seabed Characteristics Area 2

Area 2 is located 32 km northwest of Area 1 and sits in water depths ranging between 50 and 146 m (Table 3.5; Figure 3.6). Covering 159 km2, Area 2 incorporates five of the six main geomorphic features mapped, including 84 km2 of plains, 41 km2 of terraces, 31 km2 of banks, 2 km2 of depressions and <1 km2 of scarps (Table 3.5). The seafloor characteristics and environmental setting of Area 2 are very similar to those found in Area 1.

Nine banks were mapped in Area 2, ranging in water depth at their base from 50 to 123 m (Figure 3.8). They have generally flat tops with six of the banks rising to 78 m water depth. The other three bank tops in Area 2 occur at 90 m, 85 m and 60 m water depth. As in Area 1, the flanks of the banks are notched by small terraces and there is a tendency for the northern margins to have shallower slopes. This relationship breaks down, however, where banks are clustered close together, as occurs in the centre of Area 2 (Figure 3.8). Banks and their scarps had the highest average backscatter values in Area 2, at -36 ± 2 dB (Figure 3.7; Table 3.5) indicating that they are composed of a harder substrate than the other features. This was confirmed by grab and box corer samples on bank features which generally consist of poorly sorted muddy coarse sand (Appendix E).

Dense clusters of overlapping pockmarks (Figure 3.4 c and g) are associated with most bank features in Area 2, generally occurring on the flanks and adjacent to the bases of banks and to a lesser degree their tops. This is not always the case. For example, the most northerly bank in Area 2 contains almost 100% coverage of dense pockmark clusters on its flat top, and relatively smooth and pockmark free flanks.

Based on observations from still images (Figure 3.9), the top of the most southern bank is dominated by unconsolidated sediment with little evidence of bioturbation and only scattered epifauna (small sponges, gorgonians, and ascidians). The seafloor of the terrace includes a much higher proportion of harder substrate as indicated by the moderate coverage of large sponges (Iantella) and gorgonians (Mopsella). The bank margin is also dominated by unconsolidated sediment but with increased signs of bioturbation.

Plains are the most widespread geomorphic feature in Area 2, and have a modal water depth of 112 m (Figure 3.6), dipping slightly in a north-westerly direction (~0.15°). Grabs and box cores taken from plains in Area 2 generally returned silt or slightly sand silt, with the exception of grabs taken near banks (e.g. Station 34) which contained greater amounts of coarse sand (Appendix E). The soft substrate composition of plains was also reflected in their average backscatter value of -44 ± 3 dB, which was the lowest of all features in Area 2. Pockmarks are ubiquitous across the soft sediment plains in Area 2. Unlike Area 1, both dense clusters of pockmarks along with individual larger pockmarks are common on plains. The fields of dense clusters tended to occur in the deeper section of the plain, located in the centre of Area 2, or adjacent to bank features (Figure 3.6). Similar to Area 1, many individual pockmarks tended to have an asymmetric plan-shape, particularly where they occur close to banks.

Figure 3.6 False colour bathymetry image of Area 2 with sub-bottom profile (sparker) lines and sampling stations indicated.

Figure 3.7 False colour backscatter image of Area 2 with towed video transects, benthic sled, baited remote underwater video (BRUVS) and baited mid-water column video (SISSTAs) stations indicated.

Figure 3.8 Local-scale geomorphic features mapped in Area 2 interpreted from bathymetry. Images are 30% transparent polygons overlying hill-shaded bathymetric images.

Table 3.5 Summary of physical variables for geomorphic features mapped in Area 2.

|  | Area (km2) | Percentage Total (%) | Min Depth (m) | Max Depth (m) | Modal Depth (m) | Average Slope (o) | Average Backscatter (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bank | 30.73 | 19 | 57 | 123 | 78 | 3.0 ± 2.8 | -36 ± 2 |
| Depression | 2.28 | 1 | 97 | 138 | 115 | 3.7 ± 3.1 | -43 ± 5 |
| Plain | 84.49 | 53 | 89 | 146 | 112 | 2.7 ± 2.7 | -44 ± 3 |
| Scarp | 0.51 | <1 | 62 | 111 | 96 | 16.5 ± 10.4 | -36 ± 2 |
| Terrace | 40.53 | 26 | 89 | 137 | 111 | 2.7 ± 2.5 | -43 ± 4 |
| Total | 158.54 | 100 | 57 | 146 | 111 | 2.8 ± 2.5 | -42 ± 5 |

Terraces cover 41 km2, making them the second most extensive feature in Area 2. They have a modal depth of 111 m (Table 3.5), sloping slightly in a south-easterly direction (0.15-0.35o). The eastern boundary between the plain and terrace is sharp with a steep slope (2-6°) between the two features. The western boundary is more gradual and less distinct. Grab and box core samples from the terrace returned slightly sandy silt which is reflected in their low average backscatter values of -43 ± 3 dB (Table 3.5). Similar to plains in Area 2, both fields of dense clusters of pockmarks and fields of individual, non-clustering pockmark features overprint terrace features.

Depressions cover 2 km2 in Area 2 with a modal depth of 115 m. They are shallow (3-10 m deep), elongate features that occur between banks, perhaps indicating that they are formed by tidal current scour that is focused by these features. Depressions have a low average backscatter value of -43 ± 5 dB (Table 3.5) indicating that they are composed of relatively soft sediment. The one box core sample from depressions in the northern part of Area 2 (Station 32), however, returned muddy, very coarse sand (Appendix E). This relatively hard substrate, when compared to the silt of the plains and terraces, is probably a consequence of the proximity of this depression to the northernmost bank, with coarse sediment being sourced from this prominent feature.

Figure 3.9 Representative still images from the main bank in Area 2, showing: (a) sparse epifauna on bioturbated sandy substrate; (b) a gorgonian growing on sand; (c) larger habitat-forming epifauna (Ianthella sponges, Mopsella gorgonian; (d) moderate to dense coverage of epifauna on the lower terrace of the bank, and; (e) moderate concentration of bioturbation on the lower flank.

## Seabed Characteristics Area 3

Area 3 is located 34 km north of Area 2 and in water depths ranging between 51 and 169 m (Table 3.6; Figure 3.10). Covering 159 km2, Area 3 incorporates four of the six main geomorphic features mapped, including: 113 km2 of plains; 45 km2 of banks; and <1 km2 of mounds and scarps (Table 3.6). The seafloor characteristics and environmental setting is very similar to those observed in all other areas surveyed.

Twenty-two banks were identified and mapped in Area 3, which was the greatest amount in any area surveyed (Figure 3.12). They have a modal depth of 88 m but exist in water depths ranging between 51 and 113 m. Most banks have an elliptical footprint with long axes oriented in a southeast to northwest direction. Banks and their scarps have the highest average backscatter values in Area 3, at -38 ± 4 and -35 ± 3 dB (Table 3.6), indicating that they are composed of a harder substrate than other geomorphic features. This was confirmed by grab and box corer samples on bank features which generally consisted of poorly sorted, muddy coarse sand (Appendix E).

The banks in this area can be clustered by the depth of their tops. The most frequent bank top occurs at 84-85 m (n = 10), followed by 80-92 m (n = 5), 90-91 m (n = 4), 62 m (n = 1), 52 m (n = 1), and 101 m (n = 1). The bank tops with the greater depth ranges have rounder tops, in contrast to the generally flat tops of banks with a narrow range or single value. The two shallowest banks are also the largest in Area 3 and both have lower terraces at 84-85 m depth, representing former sea-levels (Figure 3.12). The top of the largest bank in Area 3 has a slight dip towards the northwest (gradient ~0.14o) from 58 m to 65 m, but this is the only bank top to display this form.

Dense clusters of overlapping pockmarks are commonly associated with most bank features in Area 3 (Figure 3.4c and g). They generally occur on the flanks and adjacent to the bases of banks. More rounded banks also frequently had pockmark clusters on their tops while tops that were relatively flat were free from pockmark clusters.

Based on observations from still images, the top of the largest bank supports very few epifauna, with a seafloor comprised of unconsolidated sediment and limited bioturbation (Figure 3.13). The southern bank top shows more signs of bioturbation, as well as benthic algae, possibly due to comparatively lower benthic disturbance on the south of the bank. The flanks of the bank support differing biological assemblages, with the northern flanks showing only very sparse epifauna and bioturbation, and the southern flank showing more epifauna, benthic algae, and bioturbation. The margins of the bank show the most bioturbation signs, with few epifauna and no observed benthic algae.

Figure 3.10 False colour bathymetry image of Area 3 with sampling stations indicated.

Figure 3.11 False colour backscatter image of Area 3 with towed video transects and benthic sled, baited remote underwater video (BRUVS) and baited mid-water column video (SISSTAs) stations indicated.

Figure 3.12 Local-scale geomorphic features mapped in Area 3 interpreted from bathymetry. Images are 30% transparent polygons overlying hill-shaded bathymetric images.

Table 3.6 Summary of physical variables for geomorphic features mapped in Area 3.

|  | Area (km2) | Percentage Total (%) | Min Depth (m) | Max Depth (m) | Modal Depth (m) | Average Slope (o) | Average Backscatter (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bank | 44.95 | 28 | 50 | 113 | 88 | 3.3 ± 2.8 | -38 ± 4 |
| Mound | 0.49 | 0.5 | 85 | 112 | 107 | 5.1 ± 4.4 | -43 ± 4 |
| Plain | 112.57 | 71 | 90 | 146 | 106 | 2.5 ± 3.0 | -47 ± 2 |
| Scarp | 0.78 | 0.5 | 52 | 109 | 94 | 19.2 ± 11.9 | -35 ± 3 |
| Total | 158.78 | 100 | 50 | 146 | 106 | 2.9 ± 3.3 | -44 ± 5 |

Plains are the most widespread geomorphic feature in Area 3 with a modal depth of 106 m (Table 3.6), sloping slightly in a north-easterly direction (~0.02°). Grabs and box cores from plains in Area 3 returned well-sorted silt (Appendix E). This composition is reinforced by the low average backscatter value of -47 ± 2 dB for plains which is not only the lowest of all features in Area 3 but the lowest out of all plains in all areas. Pockmarks are ubiquitous across the soft sediment plains in Area 3, with depths of 0.5 to 2.1 m and diameters ranging from 16 to 32 m. Most pockmarks are circular, but in some cases have an asymmetric plan-shape. Fields of dense clusters of overlapping pockmarks are also common on plains, but only where they are adjacent to banks and mounds. These fields of dense clusters coincide with a higher backscatter return, particularly noticeable in the east of Area 3 (Figure 3.11).

Finally, mounds are a feature unique to Area 3, where three were mapped (Figure 3.12). They have a low, rounded profile, ranging between 3-10 m in height and although characterised by a low average backscatter of -43 ± 4 dB, they correspond to areas of distinctly higher backscatter than the adjacent plain (Figure 3.11). Like banks they are covered on their flanks by dense clusters of overlapping pockmarks.

Figure 3.13 Representative still images from the main bank in Area 3, showing: (a) high density of burrows in silty sediment; (b) benthic algae; (c) scattered epifauna (sponges, gorgonians); (d) very sparse epifauna; (e) and (f) high levels of bioturbation on the lower terraces.

## Seabed Characteristics Area 4

Area 4 is the northern-most survey area and sits in water depths ranging between 42 and 124 m (Table 3.7; Figure 3.14). Covering 29 km2, Area 4 incorporates four of the six main geomorphic features mapped, including: 20 km2 of plains, 7l m2 of banks and <1km2 of depressions and scarps. The seabed characteristics and environmental setting of Area 4 was similar to all other areas surveyed.

Two prominent and large banks were mapped in Area 4 (Figure 3.16). They have a modal depth of 90 m but exist in water depths ranging between 42 and 111 m. The flanks and scarps of banks in Area 4 tend to be very steep (ranging between 2 and 70°) with the northern and western margins less steep than the southern and eastern margins. Two grabs taken from banks in Area 4 returned muddy, medium to coarse carbonate sand (Appendix E). This is consistent with the high average backscatter values for banks and scarps in Area 4 of -36 ± 3 dB and -35 ± 3, respectively (Figure 3.15; Table 3.7).

The flat tops of both banks occur at different depths with the larger, western bank having a shallower and flatter top, at 44 m, than the eastern bank, which has a crestal depth of 80-90 m. The western bank, also has a large terrace at 80-90 m. Fungoid (disc) coral was found in the grab sample at Station 68 (Appendix E) on the top of the western, shallower bank, indicating that the top of this bank is a habitat for coral. Similar to the banks in the other survey areas, banks in Area 4 were notched with smaller-scale terraces.

Unlike banks in the other survey areas, the tops and flanks of banks in Area 4 were generally free of dense clusters of pockmarks. However, the areas of plain adjacent to banks, along with the less steep margins of banks, did contain dense clusters of pockmarks.

Based on observations from still images (Figure 3.17), the top of the western bank has moderate coverage of epifauna, including habitat-forming barrel sponges (Xestospongia), fields of mushroom corals and conspicuous fauna including species not observed in abundance at other banks (e.g. long spiny sea urchin and nubbed anemones). The flank has sparser epifauna, while the bank margin is dominated by unconsolidated sediment with few signs of bioturbation.

Plains are the most widespread geomorphic feature in Area 4, and have a modal depth of 112 m (Table 3.7). They return a low average backscatter value of -43 ± 4 dB indicating that they are likely to be composed of soft sediments. While no sediment samples were taken from plain features in Area 4, grab and box core samples from plains in the other areas indicate that it is highly likely that the plains are composed of silt or slightly sandy silt. Pockmarks are also ubiquitous across the soft sediment plains in Area 4. Unlike the other areas surveyed, dense clusters of overlapping pockmarks are by far the most common type in plains in Area 4. Fields of individual, larger pockmarks did occur but these often are coincident with the fields of larger pockmarks.

A single depression occurs in Area 4 between the two large banks (Figure 3.16) that it was likely formed by tidal current scour focused by these large banks. It is characterised by a modal depth of 114 m and is up to 6 m deep (Table 3.7). Although no physical samples were taken the depression has a low average backscatter of -43±45 dB indicating that it is formed in soft sediment. The entire depression is overprinted by dense clusters of overlapping pockmarks.

Figure 3.14 False colour bathymetry image of Area 4 with sampling stations indicated.

Figure 3.15 False colour backscatter image of Area 4 with towed video transects indicated (Benthic sled, BRUVS and SISSTAs were not deployed in this survey area).

Figure 3.16 Local-scale geomorphic features mapped in Area 4 interpreted from bathymetry. Images are 30% transparent polygons overlying hill-shaded bathymetric images.

Table 3.7 Summary of physical variables for geomorphic features mapped in Area 4.

|  | Area (km2) | Percentage Total (%) | Min Depth (m) | Max Depth (m) | Modal Depth (m) | Average Slope (o) | Average Backscatter (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bank | 7.39 | 26 | 42 | 111 | 90 | 3.2 ± 2.4 | -36 ± 3 |
| Depression | 0.60 | 2 | 113 | 119 | 114 | 3.3 ± 2.8 | -43 ± 3 |
| Plain | 20.40 | 71 | 101 | 124 | 112 | 3.1 ± 3.0 | -43 ± 4 |
| Scarp | 0.32 | 1 | 45 | 108 | 98 | 21.0 ± 12.6 | -35 ± 3 |
| Total | 28.71 | 100 | 42 | 124 | 112 | 3.3 ± 3.4 | -41 ± 5 |

Figure 3.17 Representative still images from the main bank in Area 4, showing: (a) the top of the bank with conspicuous sea urchins; (b) mushroom corals; (c) a barrel sponge Xestospongia; (d) the flank of the bank supporting encrusting and low-relief epifauna, and; (e) the margin of the bank showing scattered epifauna and low levels of levels of bioturbation.

## All Areas Summary and Comparison

Geomorphic features across all survey areas shared some common biological and habitat characteristics. Across all areas, banks and scarps were composed of the coarsest and hardest substrate as shown by grab samples returning poorly sorted, coarse muddy sand sometimes with gravel inclusions. This was reinforced by high average backscatter values for these features, relative to depressions, mounds, plains and terraces (Table 3.4; Table 3.5; Table 3.6; Table 3.7). Plains, terraces and mounds were composed of soft substrate with low average backscatter values. Grabs and box cores from these features contained predominately well sorted silt or sandy silt. Depressions were divided into two different types: deep, steep-sided features which had high backscatter values; and shallower features that had much lower backscatter values. Only one deep depression was sampled for sedimentology over all survey areas (Station 16, Area 1). It returned poorly sorted muddy, silty gravel. None of the shallow depressions were sampled for sedimentology but their average backscatter values were very similar to plains and it is inferred that their substrate is similar to these features (i.e. sandy silt to silt).

The distribution of epibenthic and infaunal communities across the survey areas shows clear associations with geomorphology and substrate types. Terraces and plains were distinguished by an almost complete lack of noticeable epifauna in the underwater video and still images (Figure 3.18b). However, burrows and mounds were very common at some stations, particularly in Area 3, indicating unconsolidated sediment with abundant or rich infaunal communities over some plains and terraces (Figure 3.18c). The single station in the deep depression showed patches of moderate to dense sponge and octocoral gardens (Figure 3.18a), indicating a hard substrate and higher epifaunal biodiversity compared to the surrounding plains. Banks were more likely than plains or terraces to have moderate to dense biological coverage and the only geomorphic feature to support reef-forming corals (Figure 3.18d). Based on observations from the video and still images, banks had no other consistent biological characteristics, with some banks supporting sponge and octocoral gardens (e.g. northern bank in Area 1) (Figure 3.18e) and others barren of epifauna (e.g. westernmost bank in Area 2) (Figure 3.18f).

Preliminary on-board estimates from sled collections showed differences in species richness and biomass between the three main survey areas, with Area 1 having the highest number of species and catch and Area 2 having the lowest number of species and catch (Figure 3.19a). These patterns may not be inherent to the survey areas, and instead may reflect the disproportionate numbers of sleds deployed on three banks, one in each area (Figure 3.2; Figure 3.7; Figure 3.11; Figure 3.15). Indeed, patterns of estimated species richness and biomass on only these three banks showed the same patterns as the survey areas, with the main bank in Area 1 having the highest number of species and catch and the main bank in Area 2 having the lowest (Figure 3.19b). Nonetheless, species richness and biomass varied considerably within each bank, as shown by the large error bars in Figure 3.19. For example, within the main bank in Area 1, Station 14 returned 3.9 kg representing approximately 35 species while Station 16 on the same bank returned over 64 kg representing approximately 89 species.

While taxonomic identifications remain pending, on board observations indicate that species richness and endemism of sponges in the western sector of the Oceanic Shoals CMR are not as high as those in the eastern sector (as sampled in 2009 and 2010; Przeslawski et al., 2011), with sponges from the west comparatively dominated by common species from northern Australia. These results are preliminary and will be verified after species-level identifications are complete.

Figure 3.18 Representative still images from geomorphic features in the study area: (a) Moderate octocoral coverage at station 16CAM12 (~70 m depth); (b) Limited epifauna and bioturbation at station 32CAM22 (~105 m depth); (c) Burrows at station 57CAM42 (~110 m depth); (d) Scleractinian coral at station 16CAM13 (~35 m depth); (e) Habitat-forming gorgonian (Mopsella spp.) at station 67CAM48 (~48 m depth), and; (f) Absence of epifauna at station 47CAM33 (~68 m depth).

Figure 3.19 Mean species richness and biomass as estimated on-board from sled collections, based on: (a) survey area, and; (b) banks that were sampled. Numbers in parentheses indicate the number of sleds deployed, and error bars are standard error mean.

## Sub-Bottom Profile Results

Sub-bottom profiles were acquired along five lines in Area 1, traversing bank, terrace and plain features (Figure 3.1; Appendix B). Signal penetration was greatest through the soft sediment plains, reaching depths of >100 m below the seabed and resolving the internal stratigraphy to metre scale. In contrast, penetration across banks and terraces was poor due to their acoustically hard carbonate surfaces, with sub-bottom profiles imaging only the seabed and a multiple thereof (Figure 3.20).

For the plains, the sub-bottom profile records a succession of acoustic facies characterised by moderate- to high-amplitude continuous reflectors. The geometry of bounding reflectors ranges from horizontal to wavy and locally irregular, with acoustic facies between the bounding surfaces characterised by conformable low-amplitude reflectors, local clinoform sets and intervals of massive structure. Facies thickness ranges from 10-30 m. This succession is interpreted as a record of horizontal and lateral accretion of the seabed, with the latter having occurred in channels as part of a repeated cut-and-fill process.

Figure 3.20 Section of sub-bottom profile from line 01 in Area 1, showing signal penetration to >100 m below a soft sediment plain through a sedimentary succession characterised by repeated channel cut-and-fill and vertical accretion depths based on sound velocity of 1500 m/s.

In Area 2, sub-bottom profiles were acquired along four lines, providing cross-sections over several banks and terraces, and adjacent plains (Figure 3.6, Appendix B). Signal penetration across the banks and terraces was limited to the upper few metres of sediment cover, below which the record is reflector-free (Figure 3.21). Penetration into the plains extended to >100 m below the seabed, with the record characterised by horizontal to wavy, continuous high-amplitude reflectors defining bounding surfaces. Seismic facies between bounding surfaces are 5-30 m thick and are characterised by low amplitude horizontal reflectors, local clinoform sets and reflector free intervals. Again, the record below the plains is interpreted as a sedimentary succession of vertical accretion with localised channel cut-and-fill.

Figure 3.21 Section of sub-bottom profile from line 01 in Area 2, showing limited signal penetration into a carbonate bank and terrace and penetration to >100 m below the adjacent plain, with the latter characterised by channel cut-and-fill and vertical accretion. Depths based on sound velocity of 1500 m/s.

In Area 3, sub-bottom profiles were acquired along one 15-km line that crossed two banks and a plain feature in the north-west sector of the area (Figure 3.10; Appendix B). Signal penetration was again greatest through the sedimentary succession beneath the plains, reaching to 100 m below the seabed and characterised by continuous, horizontal to wavy high-amplitude reflectors (Figure 3.22). Penetration into the carbonate banks was limited to the upper few metres of sediment cover, below which the acoustic record is reflector-free. This sub-bottom line also revealed the partial burial of the bank feature by sediments, as evidenced by the onlap stratigraphic relationship between the convex wedge of sediment at the seabed and the base of the bank. This wedge is 10-15 m thick and is interpreted as the product of accretion of fine-grained sediment during the Holocene, potentially incorporating deposits laid down during the post-glacial transgression and sea-level highstand.

Figure 3.22 Section of sub-bottom profile from line 01 in Area 3, showing signal penetration to >100 m below the seabed through a sedimentary succession characterised by vertical accretion and draping across the edge of a bank. Depths based on sound velocity of 1500 m/s.

## Initial Ocean Drifter Results

Trajectory plots for the 10 ocean drifters deployed during the survey display complex and varied patterns for the period to 5 June 2013 (Figure 3.23). Of the nine drifters deployed within the Oceanic Shoals CMR, three tracked south into Joseph Bonaparte Gulf, three washed ashore in Bonaparte Gulf and are no longer transmitting, one drifted north-northeast then turned west-southwest, one was recovered by another survey vessel, and one drifted north and west toward East Timor and has stopped transmitting. The single drifter deployed during the return transit to Broome has followed an anti-clockwise circuit north from the Kimberley region then turned south along the edge of the continental shelf. These early results suggest that current patterns in the Timor Sea are complicated, and likely a result of strong tidal influences. As of 30 June 2013, five of the ten drifters remained in active transmission.

Figure 3.23 Trajectory plots for drifters released during the Oceanic Shoals survey as of 5 June 2013. Different colours represent different drifters. Tracks that end with a solid circle represent drifters that have terminated transmission and arrowheads represent ongoing transmission. Drifter 53522 (red track-line) was deployed on the return transit to Broome.

# Conclusions and Future Work

The Oceanic Shoals Commonwealth Marine Reserve Biodiversity Survey has provided new insights into the benthic and pelagic communities and habitats of a Key Ecological Feature (the carbonate banks and terraces) within the North Marine Region. The survey acquired data from a combined area of 507 km2 which equates to less than one per cent of the ~72,000 km2 covered by the Oceanic Shoals CMR. The areas mapped nonetheless represent the range of seabed environments known to exist on the banks and terraces of the CMR.

Key observations from the survey are as follows:

The geomorphic diversity of the Oceanic Shoals Commonwealth Marine Reserve is well represented in the western part of the reserve, with numerous banks and terraces providing hard substrate for benthic communities;

The epibenthic biodiversity on banks appears to vary as a function of water depth and related light and turbidity conditions, with shallower banks (<45 m) supporting more biodiversity than deeper banks, including hard corals;

Species richness and endemism of sponges in the western sector of the Oceanic Shoals CMR may not be as high as those in the eastern sector, with sponges from the west comparatively dominated by species that are common across northern Australia (to be confirmed by taxonomic analysis);

Spatial gradients in epibenthic biodiversity exist as a possible function of marked changes in substrate, light and turbidity levels along the depth transition from bank to terrace to plain;

Tidal currents play an important role in regulating levels of suspended sediment (turbidity) and in redistributing sediment across the plains and around banks and terraces, with some smaller banks partly buried by sediment;

Demersal fish communities respond to spatial patterns in benthic biodiversity, occurring in larger and more diverse populations on the shallower, less turbid banks, and;

A wide variety of high-order pelagic fish species occur in these waters.

The data and information collected on this survey will be used with pre-existing data collected from the eastern part of the Oceanic Shoals CMR to develop regional scale maps and models of benthic habitats, ecosystem processes and associated biodiversity. This analysis is part of ongoing research within the Marine Biodiversity Hub and will enable an assessment of the uniqueness of habitats and biological communities observed in this survey. It will also allow the Oceanic Shoals CMR to be placed into the broader regional context of tropical northern Australia. Importantly, all available materials and data have been subject to consistent sampling and processing protocols, ensuring a uniform data standard for analysis and interpretation. Combined with the sampling design used in this survey, these consistent data standards allow the information to be potentially used as a baseline for long-term monitoring of the CMR.

Datasets, imagery and related research products from this survey will be made available to all stakeholders via the Marine Biodiversity Hub website and the Australian Ocean Data Network Portal.

Acknowledgements

The Oceanic Shoals Commonwealth Marine Reserve Biodiversity Survey was undertaken by the Marine Biodiversity Hub, a collaborative partnership supported through funding from the Australian Government’s National Environmental Research Program (NERP). NERP Marine Biodiversity Hub partners include the Institute for Marine and Antarctic Studies at the University of Tasmania, CSIRO, Geoscience Australia, Australian Institute of Marine Science, Museum Victoria, Charles Darwin University and the University of Western Australia. Participants of the survey acknowledge and thank the master and crew of the RV Solander for their expertise and support in conducting a highly successful survey. We also thank the Field and Engineering Support staff at Geoscience Australia (GA), in particular Ian Atkinson for his support with installation and operation of multibeam sonar and sub-bottom profiler systems, and with laboratory work during the survey. The Museum and Art Gallery of the Northern Territory (MAGNT) and Museum Victoria are acknowledged for archiving biological specimens. The draft report benefited from reviews by Jodie Smith (GA), Chris Carson (GA), Julian Caley (AIMS) and Nic Bax (CSIRO). This record is published with permission of the Chief Executive Officers of Geoscience Australia and the Australian Institute of Marine Science.

References

Anderson, T.J., Nichol, S.L., Radke, L., Heap, A.D., Battershill, C., Hughes, M., Siwabessy, P.J., Barrie, V., Glasby, B., Tran, M. and Daniell, J. 2011. Seabed environments of the eastern Joseph Bonaparte Gulf, Northern Australia, GA0325-SOL5117 Post-Survey Report. Geoscience Australia Record 2010/08, Canberra, 61 pp.

Cappo, M., Speare, P. and De’ath, G. 2004. Comparison of baited remote underwater video stations (BRUVS) and prawn (shrimp) trawls for assessments of fish biodiversity in inter-reefal areas of the Great Barrier Reef Marine Park. Journal of Experimental Marine Biology and Ecology 302, 123-152.

Cappo, M., Speare, P. and De’ath, G. 2007. Inter-reef vertebrate communities of the Great Barrier Reef Marine Park determined by baited remote underwater video stations. Marine Ecology Progress Series 350, 209-221.

Commonwealth of Australia. 2008. The North Marine Region Bioregional Plan: Bioregional Profile. Department of the Environment, Water, Heritage and the Arts, Canberra, 241 pp.

Commonwealth of Australia. 2012a. Marine Bioregional Plan for the North Marine Region. Department of Sustainability, Environment, Water, Population and Communities, Canberra, 191 pp.

Commonwealth of Australia. 2012b. Marine Bioregional Plan for the North-west Marine Region. Department of Sustainability, Environment, Water, Population and Communities, Canberra, 260 pp.

Fisher, R., Radford, B.T., Knowlton, N., Brainard, R.E., Michaelis, F.B. and Caley, M.J. 2011. Global mismatch between research effort and conservation needs of tropical coral reefs. Conservation Letters 4, 64-72.

Gavrilov, A.N., Duncan, A.J., McCauley, R.D., Parnum, I.M., Penrose, J.D., Siwabessy, P.J.W., Woods, A.J. and Tseng, Y-T. 2005. Characterisation of the seafloor in Australia's coastal zone using acoustic techniques. Proceedings of First International Conference on Underwater Acoustic Measurements: Technologies and Results. Heraklion, Crete, Greece, June 2005. Vol. 1, 1075-1080.

George T. and Cauquil, E. 2010 A multi-disciplinary site investigation for the assessment of drilling geohazards and environmental impact within the northern Bonaparte Basin. Preview 2010 (148), 41–44.

Heap, A.D., Przeslawski, R., Radke, L.C., Trafford, J. and Battershill, C. 2010. Seabed Environments of the Eastern Joseph Bonaparte Gulf, Northern Australia: SOL4934 - Post Survey Report. Geoscience Australia Record 2010/09, Canberra, 81 pp.

Heap, A.D. and Harris, P.T. 2008. Geomorphology of the Australian margin and adjacent seafloor. Australian Journal of Earth Sciences 55, 555-585.

Heyward, A., Pinceratto, E. and Smith, L. 1997. Big Bank Shoals of the Timor Sea: An Environmental Resource Atlas. BHP Petroleum, Melbourne.

Hovland, M. 1990. Do carbonate reefs form due to fluid seepage? Terra Nova, 2(1), 8-18.

Müller, G. and Gastner, M. 1971. The ‘‘Karbonat-Bombe’’, a simple device for the determination of the carbonate content in sediments, soils, and other materials. Neues Jahrbuch für Mineralogie Monatshefte 10, 446–469.

O’Brien, G. W., Glenn, K., Lawrence, G., Williams, A. K., Webster, M., Burns, S. and Cowley, R. 2002. Influence of hydrocarbon migration and seepage on benthic communities in the Timor Sea, Australia, APPEA Journal 42(1), 225–239.

Przeslawski, R., Daniell, J., Anderson, T., Barrie, V., Heap, A., Hughes, M., Li, J., Potter, A., Radke, L., Siwabessy, J., Tran, M., Whiteway, T., Nichol, S. 2011. Seabed habitats and hazards of the Joseph Bonaparte Gulf and Timor Sea, northern Australia. Geoscience Australia Record 2011/40, Canberra, 156 pp.

Richardson, A.J., Hobday, A.J., Poloczanska, E.S. 2009. Australia’s Marine Life. In, Report Card of Marine Climate Change for Australia 2009. Eds. E.S. Poloczanska, A.J. Hobday and A.J. Richardson, NCCARF Publication 05/09.

Schultz, A.L., Malcolm, H.A., Bucher, D.J., Smith, S.D.A. 2012. Effects of reef proximity on the structure of fish assemblages of unconsolidated substrata. PLoS One 7(11). Doi:10.1371/journal.pone.0049437

Stevens, D.L. and Olsen, A.R. 2004. Spatially balanced sampling of natural resources. Journal of the American Statistical Association, 99 (No. 465), 262-278.

Tittensor, D.P., Mora, C., Jetz, W., Lotze, H.K., Ricard, D., Vanden Berghe, E., Worm, B. 2010. Global patterns and predictors of marine biodiversity across taxa. Nature 466, 1098-1103.

Van Andel, T.H., and Veevers, J.J. 1967. Morphology and sediments of the Timor Sea. Bureau of Mineral Resources, Geology and Geophysics Bulletin 83, Canberra, 172 pp.

West, B.G., Conolly, J.R., Blevin, J.E., Miyazaki, S. and Vuckovic, V. 1992. Petroleum Prospectivity of the East Malita Graben area, Bonaparte Basin. Bureau of Mineral Resources, Geology and Geophysics Record 1992/24.

Scientific and Technical Crew

Table A.1 Role and affiliation of personnel on survey GA0339/SOL5650.

| Main Role | Crew Member |
| --- | --- |
| AIMS Voyage Leader | Marcus Stowar (AIMS)1 |
| GA Chief Scientist (Sedimentology) | Scott Nichol (GA)2 |
| Scientist (Geochemistry) | Lynda Radke (GA)2 |
| Scientist (Marine ecology) | Johnathan Kool (GA)2 |
| Scientist (Marine ecology/Biodiscovery) | Belinda Alvarez de Glasby (MAGNT)3 |
| Scientist (Marine ecology) | Tom Letessier (UWA)4 |
| Scientist (Marine mammals) | Phil Bouchet (UWA)4 |
| Multibeam acquisition & processing | Justy Siwabessy (GA)2 |
| Multibeam acquisition & processing | Kim Picard (GA)2 |
| Multibeam & sub-bottom profiler acquisition | Ian Atkinson (GA)2 |
| Towed Video Technician | Jamie Colquhoun (AIMS)1 |

1. AIMS – Australian Institute of Marine Science, 2. GA – Geoscience Australia; 3. MAGNT – Museum and Art Gallery of the Northern Territory; 4. UWA - University of Western Australia

Survey Details for Sub-bottom Lines

Table B.1 Survey details for sub-bottom profile lines.

| Area | Line No. | Length (km) | Start Latitude | Start Longitude | End Latitude | End Longitude |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.1 | 18.5 | -12.0573 | 127.4319 | -12.2242 | 127.4317 |
| 1 | 1.2 | 18.0 | -12.0672 | 127.4036 | -12.2278 | 127.4039 |
| 1 | 1.3 | 10.3 | -12.2214 | 127.4075 | -12.1522 | 127.4614 |
| 1 | 1.4 | 8.5 | -12.1522 | 127.4556 | -12.1522 | 127.3833 |
| 1 | Test_3 | 8.3 | -12.1506 | 127.3858 | -12.0753 | 127.3858 |
| 2 | 2.1 | 19.8 | -11.7803 | 126.9694 | -11.9155 | 127.1139 |
| 2 | 2.2 | 23.0 | -11.9108 | 127.0853 | -11.8386 | 127.0853 |
| 2 | 2.3 | 12.5 | -11.8050 | 127.0505 | -11.8483 | 127.0001 |
| 2 | 2.4 | 7.7 | -11.8197 | 126.9894 | -11.8003 | 127.0433 |
| 3 | 3.1 | 15 | -11.3919 | 127.0419 | -11.4208 | 126.9192 |

Survey Details for CTD Deployments

Table C.1 Survey details for CTD deployments.

| Survey Area | Station | Sample ID | Latitude | Longitude | Water depth (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 05 | SOL5650/005CTD001 | -12.21690 | 127.39397 | 84 |
| 1 | 06 | SOL5650/006CTD002 | -12.20900 | 127.42360 | 74 |
| 1 | 07 | SOL5650/007CTD003 | -12.18973 | 127.40072 | 76 |
| 1 | 08 | SOL5650/008CTD004 | -12.19272 | 127.40487 | 72 |
| 1 | 10 | SOL5650/010CTD005 | -12.19663 | 127.45323 | 89 |
| 1 | 11 | SOL5650/011CTD006 | -12.19898 | 127.44568 | 99 |
| 1 | 12 | SOL5650/012CTD007 | -12.19718 | 127.44092 | 98 |
| 1 | 14 | SOL5650/014CTD008 | -12.07332 | 127.43090 | 29 |
| 1 | 15 | SOL5650/015CTD009 | -12.07157 | 127.42400 | 56 |
| 1 | 16 | SOL5650/016CTD010 | -12.09007 | 127.42557 | 126 |
| 1 | 17 | SOL5650/017CTD011 | -12.10477 | 127.39930 | 110 |
| 1 | 18 | SOL5650/018CTD012 | -12.07640 | 127.42877 | 31 |
| 1 | 19 | SOL5650/019CTD013 | -12.08097 | 127.43043 | 49 |
| 1 | 20 | SOL5650/020CTD014 | -12.19747 | 127.39318 | 72 |
| 1 | 21 | SOL5650/021CTD015 | -12.20537 | 127.43607 | 77 |
| 1 | 22 | SOL5650/022CTD016 | -12.20823 | 127.44542 | 84 |
| 1 | 23 | SOL5650/023CTD017 | -12.21700 | 127.39702 | 86 |
| 2 | 24 | SOL5650/024CTD018 | -11.88682 | 127.09420 | 78 |
| 2 | 25 | SOL5650/025CTD019 | -11.89568 | 127.06962 | 73 |
| 2 | 26 | SOL5650/026CTD020 | -11.89193 | 127.09473 | 62 |
| 2 | 27 | SOL5650/027CTD021 | -11.87830 | 127.11590 | 116 |
| 2 | 28 | SOL5650/028CTD022 | -11.85733 | 127.03692 | 127 |
| 2 | 29 | SOL5650/029CTD023 | -11.89327 | 127.09652 | 60 |
| 2 | 30 | SOL5650/030CTD024 | -11.92632 | 127.07750 | 107 |
| 2 | 31 | SOL5650/031CTD025 | -11.79200 | 126.97793 | 78 |
| 2 | 32 | SOL5650/032CTD026 | -11.79317 | 126.99492 | 97 |
| 2 | 33 | SOL5650/033CTD027 | -11.84542 | 127.05523 | 127 |
| 2 | 34 | SOL5650/034CTD028 | -11.88003 | 127.09270 | 108 |
| 2 | 35 | SOL5650/035CTD029 | -11.90318 | 127.11667 | 106 |
| 2 | 36 | SOL5650/036CTD030 | -11.81700 | 127.04547 | 107 |
| 2 | 37 | SOL5650/037CTD031 | -11.81778 | 127.02990 | 110 |
| 2 | 38 | SOL5650/038CTD032 | -11.80713 | 126.96720 | 77 |
| 2 | 39 | SOL5650/039CTD033 | -11.78008 | 127.00950 | 90 |
| 2 | 40 | SOL5650/040CTD034 | -11.80800 | 126.99058 | 99 |
| 2 | 41 | SOL5650/041CTD035 | -11.88967 | 127.06682 | 62 |
| 2 | 42 | SOL5650/042CTD036 | -11.88838 | 127.06950 | 74 |
| 3 | 44 | SOL5650/044CTD037 | -11.41017 | 126.92277 | 88 |
| 3 | 45 | SOL5650/045CTD038 | -11.45030 | 126.92107 | 106 |
| 3 | 46 | SOL5650/046CTD039 | -11.43483 | 126.93770 | 75 |
| 3 | 47 | SOL5650/047CTD040 | -11.42067 | 126.94477 | 64 |
| 3 | 48 | SOL5650/048CTD041 | -11.43192 | 126.94270 | 61 |
| 3 | 49 | SOL5650/049CTD042 | -11.40693 | 126.92902 | 81 |
| 3 | 50 | SOL5650/050CTD043 | -11.40532 | 126.96090 | 99 |
| 3 | 51 | SOL5650/051CTD044 | -11.43203 | 126.98772 | 92 |
| 3 | 52 | SOL5650/052CTD045 | -11.43740 | 126.99717 | 95 |
| 3 | 53 | SOL5650/053CTD046 | -11.44230 | 126.96760 | 78 |
| 3 | 54 | SOL5650/054CTD047 | -11.42373 | 126.96570 | 86 |
| 3 | 55 | SOL5650/055CTD048 | -11.43397 | 126.95358 | 59 |
| 3 | 56 | SOL5650/056CTD049 | -11.39937 | 127.02052 | 75 |
| 3 | 57 | SOL5650/057CTD050 | -11.44318 | 127.09513 | 110 |
| 3 | 58 | SOL5650/058CTD051 | -11.44780 | 127.03050 | 107 |
| 3 | 59 | SOL5650/059CTD52 | -11.4030 | 127.0278 | 52 |
| 3 | 61 | SOL5650/061CTD053 | -11.44123 | 126.97107 | 82 |
| 3 | 62 | SOL5650/062CTD054 | -11.41310 | 126.94082 | 64 |
| 3 | 63 | SOL5650/063CTD055 | -11.39983 | 126.91263 | 99 |
| 3 | 65 | SOL5650/065CTD056 | -11.42470 | 126.94357 | 62 |
| 3 | 66 | SOL5650/066CTD057 | -11.41280 | 126.93593 | 65 |
| 4 | 67 | SOL5650/067CTD058 | -11.33650 | 126.96023 | 73 |
| 4 | 68 | SOL5650/068CTD059 | -11.32833 | 126.95320 | 43 |
| 4 | 72 | SOL5650/072CTD060 | -11.88920 | 127.07497 | 75 |
| 1 | 73 | SOL5650/073CTD061 | -12.06463 | 127.42538 | 73 |
| 1 | 74 | SOL5650/074CTD062 | -12.07720 | 127.44508 | 73 |
| 1 | 75 | SOL5650/075CTD063 | -12.14323 | 127.44580 | 110 |

Survey details for Ocean Current Drifter Deployments

Table D.1 Survey details for ocean current drifter deployments.

| Drifter ID | WMO Code1 | Release Date | Julian Day | Latitude | Longitude |
| --- | --- | --- | --- | --- | --- |
| 101918 | 53576 | 16/09/2012 | 260 | -12° 10.8806’ | 127° 26.8244’ |
| 101884 | 53575 | 17/09/2012 | 261 | -12° 08.0388’ | 127° 26.0007’ |
| 101765 | 53601 | 18/09/2012 | 262 | -12° 05.3788’ | 127° 23.6460’ |
| 101914 | 53520 | 20/09/2012 | 264 | -11° 52.3790’ | 127° 07.7039’ |
| 1019172 | 53521 | 21/09/2012 | 265 | -11° 56.1269’ | 127° 05.1850’ |
| 101753 | 53577 | 22/09/2012 | 266 | -11° 50.2650’ | 127° 05.2010’ |
| 101754 | 53578 | 24/09/2012 | 268 | -11° 53.9040’ | 127° 06.3839’ |
| 101756 | 53582 | 26/09/2012 | 270 | -11° 26.7590’ | 127° 54.2120’ |
| 101755 | 53523 | 3/10/2012 | 277 | -12° 08.4460’ | 127° 27.3470 |
| 101752 | 53522 | 4/10/2012 | 278 | -14° 16.7408’ | 124° 50.9317’ |

WMO – World Meteorological Organisation code. Used as drifter identifier on Global Drifter Program website.

Drifter 101917 was recovered by another survey vessel operating in the area on 27 September 2012. The drifter was later returned to Pearl Marine Engineering in Darwin then redeployed in the Arafura Sea on a Southern Surveyor transit survey (SS2012t07) on 18 October 2012; location approximately 20 nM north of the Wessel Islands.

Survey Details for Surface Sediment Grab (GR) and Box Core (BC) Samples

Table E.1 Surface sediment grab (GR) and box core (BC) sample locations, water depths and field descriptions (including Munsell colour code, where noted).

| Area | Station | Sample ID | Latitude | Longitude | Water Depth (m) | Sample Description |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 05 | GR001 | -12.2178 | 127.3947 | 84 | Slightly sandy silt; fragments of fine shell; poorly sorted |
| 1 | 06 | GR002 | -12.2094 | 127.4224 | 70 | Slightly gravelly silt with shell fragments; poorly sorted; 5Y 5/2 olive grey |
| 1 | 07 | GR004 | -12.1840 | 127.4006 | 76 | Slightly granular silt with shell fragments; poorly sorted; bivalves; 5Y 5/2 olive grey |
| 1 | 08 | GR006 | -12.1924 | 127.4043 | 73 | Gravelly silt and shell fragments; very poorly sorted; 5Y 5/2 olive grey |
| 1 | 09 | GR008 | -12.2004 | 127.4308 | 73 | Gravelly sandy silt; very poorly sorted; olive grey; 5Y 5/2 olive grey |
| 1 | 10 | GR010 | -12.1956 | 127.4521 | 106 | Gravelly mud; poorly sorted; 5Y 5/2 olive grey |
| 1 | 10 | GR011 | -12.1988 | 127.4522 | 104 | Gravelly mud |
| 1 | 11 | GR013 | -12.1987 | 127.4471 | 100 | Mud & gravel; poorly sorted; shell fragments; 5Y 4/2 olive grey |
| 1 | 11 | GR014 | -12.1985 | 127.4469 | 100 | Gravelly mud; poorly sorted; 5Y 4/2 olive grey |
| 1 | 14 | GR015 | -12.0742 | 127.4346 | 30 | Very coarse sandy gravel; mud trace; carbonate; very poorly sorted; 2.5Y 6/3 light yellowish brown |
| 1 | 14 | GR017 | -12.0677 | 127.4361 | 51 | Gravelly coarse sand; trace mud; poorly sorted; carbonate; |
| 1 | 15 | GR019 | -12.0713 | 127.4239 | 59 | Very poorly sorted muddy, sand & gravel; cemented clasts; carbonate; 5Y 4/2 olive grey |
| 1 | 16 | GR021 | -12.0895 | 127.4259 | 125 | Very poorly sorted muddy dandy gravel; carbonate clasts cemented; 5Y 4/2 olive grey |
| 1 | 17 | BC001 | -12.1048 | 127.3996 | 110 | Sandy silt; poorly sorted; 5Y 4/2 olive grey |
| 1 | 18 | GR023 | -12.0749 | 127.4265 | 32 | Very poorly sorted sandy gravel; carbonate; mud trace; 2.5Y 5/3 light olive brown |
| 1 | 19 | GR025 | -12.0806 | 127.4297 | 49 | Poorly sorted gravelly coarse sand; 2.5Y 5/2 greyish brown |
| 1 | 20 | BC003 | -12.1977 | 127.3940 | 73 | Sandy silt; shell fragments in sand fraction; 5Y 4/3 olive |
| 1 | 21 | BC005 | -12.2058 | 127.4364 | 77 | Muddy coarse sand and gravel; poorly sorted; 5Y 4/3 olive |
| 1 | 22 | BC007 | -12.2089 | 127.4457 | 81 | Muddy sandy gravel; very poorly sorted; 5Y 4/3 olive |
| 2 | 24 | BC009 | -11.8868 | 127.0935 | 78 | Muddy coarse sand; 5Y 4/3 olive |
| 2 | 25 | BC012 | -11.8969 | 127.0690 | 71 | No field description |
| 2 | 26 | GR028 | -11.8900 | 127.0950 | 73 | Muddy coarse sand; poorly sorted; 5Y 4/2 olive grey |
| 2 | 27 | BC014 | -11.8784 | 127.1166 | 116 | Silt; 5Y 4/3 olive |
| 2 | 28 | BC016 | -11.8581 | 127.0373 | 127 | Slightly sandy silt; 5Y 4/3 olive |
| 2 | 29 | GR030 | -11.8932 | 127.0951 | 58 | Muddy coarse to medium sand; 5Y 5/3 olive grey |
| 2 | 31 | BC018 | -11.7910 | 126.9772 | 76 | Muddy coarse sand |
| 2 | 32 | BC020 | -11.7944 | 126.9931 | 95 | Muddy coarse to very coarse sand; poorly sorted; 5Y 4/3 olive |
| 2 | 33 | BC022 | -11.8459 | 127.0559 | 127 | Silt; 5Y 4/3 olive |
| 2 | 34 | BC024 | -11.8799 | 127.0920 | 108 | Sandy mud; sand coarse to very coarse; poorly sorted; 5Y 4/3 olive |
| 2 | 36 | BC027 | -11.8178 | 127.0468 | 110 | Muddy very coarse sand; very poorly sorted; 5Y 4/3 olive |
| 2 | 37 | BC029 | -11.8177 | 127.0296 | 110 | Muddy coarse to very coarse sand; very poorly sorted; 5Y 2/3 olive |
| 2 | 38 | BC030 | -11.8075 | 126.9673 | 77 | Muddy coarse to very coarse sand; very poorly sorted; 5Y 4/2 olive grey |
| 2 | 39 | BC032 | -11.7801 | 127.0092 | 90 | Slightly sandy silt; moderately well sorted |
| 2 | 40 | BC034 | -11.8083 | 126.9905 | 99 | Slightly sandy silt; well sorted; 5Y 4/2 olive grey |
| 2 | 41 | GR033 | -11.8896 | 127.0670 | 64 | Slightly muddy coarse to very coarse sand & gravel; very poorly sorted; 5Y 4/3 olive |
| 2 | 42 | GR035 | -11.8887 | 127.0745 | 72 | Muddy coarse sand; poorly sorted; 5Y 2/3 olive |
| 3 | 44 | GR037 | -11.4102 | 126.9227 | 88 | Slightly sandy silt; well sorted; 5Y 5/3 olive grey |
| 3 | 45 | BC037 | -11.4504 | 126.9212 | 106 | Silt; well sorted; 5Y 5/2 grey |
| 3 | 46 | BC039 | -11.4338 | 126.9383 | 60 | Slightly sandy silt; moderate sorting; 5Y 5/3 olive |
| 3 | 47 | GR038 | -11.4204 | 126.9446 | 64 | Sandy silt; poorly sorted; sand fraction medium to coarse, angular; 5Y 5/3 olive |
| 3 | 49 | GR040 | -11.4069 | 126.9291 | 81 | Muddy coarse sand; poorly sorted; 5Y 2/3 olive |
| 3 | 50 | BC041 | -11.4051 | 126.9616 | 99 | Silt; well sorted; 5Y 5/2 olive grey |
| 3 | 51 | BC043 | -11.4310 | 126.9888 | 95 | Silt; well sorted; 5Y 5/3 olive |
| 3 | 52 | BC045 | -11.4360 | 126.9971 | 96 | Silt; well sorted; 5Y 5/3 olive |
| 3 | 53 | BC047 | -11.4426 | 126.9670 | 79 | Muddy coarse sand; poorly sorted; 5Y 4/3 olive |
| 3 | 54 | BC049 | -11.4244 | 126.9660 | 85 | Muddy medium to coarse sand; poorly sorted; 5Y 2/3 olive grey |
| 3 | 55 | GR042 | -11.4342 | 126.9542 | 57 | Muddy medium to coarse sand; poorly sorted; carbonates; 5Y 5/3 olive |
| 3 | 56 | BC051 | -11.3993 | 127.0205 | 76 | Muddy coarse sand; poorly sorted; 5Y 5/3 olive |
| 3 | 57 | BC053 | -11.4433 | 127.0951 | 110 | Silt; well sorted; 5Y 5/3 olive |
| 3 | 58 | BC056 | -11.4483 | 127.0306 | 108 | Silt; well sorted; 5Y 5/3 olive |
| 3 | 60 | GR044 | -11.4032 | 127.0245 | 51 | Slightly muddy medium to coarse sand; moderately well sorted; carbonates; 5Y 5/3 olive |
| 3 | 60 | GR045 | -11.4033 | 127.0249 | 52 | Muddy medium to coarse sand; 5Y 5/3 olive |
| 3 | 61 | BC058 | -11.4412 | 126.9713 | 84 | Muddy medium to coarse sand; poorly sorted; 5Y 5/3 olive |
| 3 | 62 | BC060 | -11.4091 | 126.9396 | 77 | Muddy coarse sand; poorly sorted; 5Y 5/3 olive |
| 3 | 63 | BC062 | -11.3994 | 126.9121 | 98 | Silt; well sorted; 5Y 5/3 olive |
| 3 | 65 | GR046 | -11.4257 | 126.9445 | 62 | Sandy mud; sand fraction coarse to very coarse carbonate; poorly sorted; 5Y 5/3 olive |
| 3 | 66 | GR048 | -11.4143 | 126.9355 | 65 | Muddy coarse sand; poorly sorted; 5Y 5/3 olive |
| 4 | 67 | GR050 | -11.3368 | 126.9622 | 71 | Muddy medium to coarse sand; poorly sorted; 5Y 5/3 olive |
| 4 | 68 | GR051 | -11.3284 | 126.9543 | 42 | Muddy coarse to very coarse sand; poorly sorted; fungoid corals (discs); 5Y 5/3 olive |
| 1 | 73 | GR053 | -12.0647 | 127.4257 | 72 | Slightly muddy medium to coarse sand; poorly sorted; carbonates; 5Y 5/3 olive |
| 1 | 74 | GR055 | -12.0775 | 127.4448 | 73 | Muddy medium to coarse sand; carbonate; poorly sorted; 5Y 5/3 olive |
| 1 | 75 | BC065 | -12.1440 | 127.4458 | 110 | Sandy silt; poorly sorted; sand fraction fine to coarse; carbonates; 5Y 5/3 olive |

Survey Details for Surface Water Samples

Table F.1 Details of underway water samples collected during transits.

| Station | Sample ID | Latitude | Longitude | UTC Date | UTC Time |
| --- | --- | --- | --- | --- | --- |
| 01 | SOL5650/001WS001 | -15.84167 | 122.71850 | 13/9/2012 | 02:20 |
| 02 | SOL5650/002WS002 | -15.40833 | 123.01950 | 13/9/2012 | 05:15 |
| 03 | SOL5650/003WS003 | -13.18350 | 126.00383 | 14/9/2012 | 02:05 |
| 04 | SOL5650/004WS004 | -12.95717 | 126.29200 | 14/9/2012 | 05:05 |
| 19 | SOL5650/019WS005 | -12.08088 | 127.43057 | 18/9/2012 | 04:45 |
| 23 | SOL5650/023WS006 | -12.21718 | 127.39683 | 19/9/2012 | 04:58 |
| 26 | SOL5650/026WS007 | -11.89480 | 127.09093 | 20/9/2012 | 05:40 |
| 30 | SOL5650/030WS008 | -11.92660 | 127.07742 | 21/9/2012 | 05:01 |
| 43 | SOL5650/043WS009 | -11.53238 | 127.44635 | 26/9/2012 | 01:55 |
| 48 | SOL5650/048WS010 | -11.43192 | 126.94270 | 27/9/2012 | 04:53 |
| 64 | SOL5650/064WS011 | -11.43635 | 126.96218 | 1/10/2012 | 01:48 |
| 71 | SOL5650/071WS012 | -11.64008 | 127.00917 | 2/10/2012 | 04:49 |
| 76 | SOL5650/076WS013 | -14.22344 | 124.94283 | 4/10/2012 | 01:50 |
| 77 | SOL5650/077WS014 | -14.33861 | 124.75250 | 5/10/2012 | 04:50 |
| 78 | SOL5650/078WS015 | -16.96980 | 122.23950 | 5/10/2012 | 01:50 |
| 79 | SOL5650/079WS016 | -17.34467 | 122.00225 | 5/10/2012 | 04:56 |

Survey Details for Geochemical Samples

Table G.1 Details of the geochemical analyses undertaken on sediment samples. Grab numbers from Smith-McIntyre grabs (GRXXX) and box cores (BCXXX) are shown in the columns. The designation B_XX corresponds to the extensions that are applied to the sample numbers in the corporate database, in which B designates a geochemistry sample and XX designates a sub-sample type. For example, SOL5650/060GR045B_B1 refers to a geochemistry sample from station 060 and grab 45 designated for chlorin, porosity and mineralogy analysis. SOD denotes sediment oxygen demand.

| Station | Sub-sample type |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Chlorins,Porosity, Mineralogy (0-2.0 cm) (B_B1) | Chlorophyll abc (0-0.5 cm) (B_B2) | Porosity (0-0.5 cm) (B_B3) | SOD (0-2.0 cm) (B_C1) | Bottle incubation (0-2.0 cm) (B_C2) | Core incubation (B_C3) | Elements, Carbonate, Specific Surface Area (0-2.0 cm) (B_D1) | Spare (~0-2.0 cm) (B_E1) | Specimens (A_S1) |
| 005 | & | & | & | & | & |  | GR001 | & |  |
| 006 | GR003 | GR003 | GR003 | GR003 | GR003 |  | GR003 | GR003 |  |
| 007 | GR005 | GR005 | GR005 | GR005 | GR005 |  | GR005 | GR005 |  |
| 008 | GR007 | GR007 | GR007 | GR007 | GR007 |  | GR007 | GR007 |  |
| 009 | GR009 | GR009 | GR009 | GR009 | GR009 |  | GR009 | # |  |
| 010 | GR012 | GR012 | GR012 | GR012 | GR012 |  | GR012 | GR012 |  |
| 011 | GR014 | GR014 | GR014 | GR014 | GR014 |  | GR014 | GR014 |  |
| 014 | GR016 | GR016 | GR016 | GR016 | GR016 |  | GR016 | GR016 |  |
| 014 | GR018 | GR018 | GR018 | GR018 | GR018 | GR018 | GR018 | # |  |
| 015 | GR020 | GR020 | GR020 | GR020 | GR020 |  | GR020 | GR020 |  |
| 016 | GR022 | GR022 | GR022 | GR022 | GR022 |  | GR022 | GR022 |  |
| 017 | BC002 | BC002 | BC002 | @ | BC002 |  | BC002 | # |  |
| 018 | GR024 | GR024 | GR024 | GR024 | GR024 | GR024 | GR024 | GR024 |  |
| 019 | GR026 | x | x | GR026 | GR026 | GR026 | GR026 | GR026 |  |
| 020 | BC004 | BC004 | BC004 | BC004 | BC004$ |  | BC004 | # |  |
| 021 | BC006 | BC006 | BC006 | BC006 | BC006 |  | BC006 | # |  |
| 022 | BC008 | BC008 | BC008 | BC008 | BC008 |  | BC008 | # |  |
| 024 | BC011 | BC011 | BC011 | BC011 | BC011 |  | BC011 | # |  |
| 025 | GR027 | GR027 | GR027 | GR027 | GR027 | GR027% | GR027 | # |  |
| 026 | GR029 | GR029 | GR029 | GR029 | GR029 | GR029% | GR029 | GR029 |  |
| 027 | BC015 | BC015 | BC015 | BC015 | BC015 |  | BC015 | BC015 |  |
| 028 | BC017 | BC017 | BC017 | BC017 | BC017 |  | BC017 | BC017 |  |
| 029 | BC031 | BC031 | BC031 | BC031 | BC031 | BC031 | BC031 | BC031 |  |
| 031 | BC019 | BC019 | BC019 | BC019 | BC019 |  | BC019 | BC019 |  |
| 032 | BC021 | BC021 | BC021 | BC021 | BC021 |  | BC021 | BC021 |  |
| 033 | BC023 | BC023 | BC023 | BC023 | BC023 | BC023% | BC023 | BC023+ |  |
| 034 | BC026 | BC026 | BC026 | BC026 | BC026 | BC026% | BC026 | BC026+ |  |
| 036 | BC028 | x | x | BC028 | BC028 | BC028% | BC028 | BC028+ |  |
| 037 | GR032 | GR032 | GR032 | GR032 | GR032 |  | GR032 | GR032 |  |
| 038 | BC031 | BC031 | BC031 | BC031 | BC031 | BC031% | BC031 | BC031+ |  |
| 039 | BC033 | x | x | BC033 | BC033 |  | BC033 | # |  |
| 040 | BC035 | BC035 | BC035 | BC035 | BC035 | BC035% | BC035 | BC035+ |  |
| 041 | GR034 | GR034 | GR034 | GR034 | GR034 | GR034% | GR034 | GR034 |  |
| 042 | GR036 | GR036 | GR036 | GR036 | GR036 | GR036% | GR036 | GR036 |  |
| 044 | BC036 | BC036 | BC036 | BC036 | BC036 | BC036% | BC036 | BC036+ |  |
| 045 | BC038 | BC038 | BC038 | BC038 | BC038 |  | BC038 | BC038 |  |
| 046 | BC040 | BC040 | BC040 | BC040 | BC040 |  | BC040 | BC040 |  |
| 047 | GR039 | GR039 | GR039 | GR039 | GR039 | GR039% | GR039 | GR039 |  |
| 049 | GR041 | GR041 | GR041 | GR041 | GR041 | GR041% | GR041 | GR041 |  |
| 050 | GR042 | GR042 | GR042 | GR042 | GR042 |  | GR042 | # |  |
| 051 | BC044 | BC044 | BC044 | BC044 | BC044 | BC044% | BC044 | BC044 |  |
| 052 | BC046 | BC046 | BC046 | BC046 | BC046 | BC046% | BC046 | BC046+ |  |
| 053 | BC048 | BC048 | BC048 | BC048 | BC048 |  | BC048 | # |  |
| 054 | BC050 | BC050 | BC050 | BC050 | BC050 |  | BC050 | BC050 |  |
| 055 | GR043 | GR043 | GR043 | GR043 | GR043 | GR043% | GR043 | GR043 |  |
| 056 | BC052 | BC052 | BC052 | BC052 | BC052 |  | BC052 | # |  |
| 057 | BC054 | BC054 | BC054 | BC054 | BC054 | BC054% | BC054 | # |  |
| 058 | BC057 | BC057 | BC057 | BC057 | BC057 |  | BC057 | BC057 |  |
| 060 | GR048 | GR048 | GR048 | GR048 | GR048 | GR048 | GR048 | GR048 | GR048* |
| 061 | BC059 | BC059 | BC059 | BC059 | BC059 | BC059% | BC059 | # |  |
| 062 | BC061 | BC061 | BC061 | BC061 | BC061 |  | BC061 | # |  |
| 063 | BC063 | BC063 | BC063 | BC063 | BC063 |  | BC063 | BC063 |  |
| 065 | GR047 | GR047 | GR047 | GR047 | GR047 | GR047% | GR047 | GR047 |  |
| 066 | GR049 | GR049 | GR049 | GR049 | GR049 | GR049% | GR049 | GR049 |  |
| 068 | GR052 | x | x | GR052 | GR052 |  | GR052 | # |  |
| 073 | GR054 | GR054 | GR054 | GR054 | GR054 | GR054% | GR054 | GR054 |  |
| 074 | BC064 | BC064 | BC064 | BC064 | BC064 | BC064% | BC064 | # |  |
| 075 | GR066 | GR066 | GR066 | GR066 | GR066 |  | GR066 | # |  |

(&) failed grab; (#) insufficient material; (x) inadequate surface; (@) lost sample; (%); dark incubation only; ($) Initial sample (T=0) only; (+) used sediment from incubated cores; (x) slumped surface on grab/core; (*) apparent carbonate cement

Geochemical Techniques

Table H.1 Details of shipboard laboratory processing and analytical techniques used to prepare and analyse the geochemistry sub-samples. Sub-sample codes (B_B1 etc) correspond to the file extension assigned to the sub-sample types in the shipboard database.

| Sub-sample | Shipboard processing | Parameters measured | Laboratory pre-processing | Analytic procedures and sample post processing |
| --- | --- | --- | --- | --- |
| B_B1 | 7.5 ml samples of surface sediment (0-2 cm) were syringed into plastic container wrapped in Al foil. The samples were frozen. | Porosity and wet/dry bulk densities (0.0-2.0cm) | Freeze-dry | Weight difference after drying and after correction for seawater salts (porosity) and normalisation to wet/dry volumes (bulk density). |
|  |  | Total chlorins and chlorin indices | Triple extraction in 100% acetone after freeze-drying and grinding (in dark). | Fluorometry |
|  |  | Mineralogy | 10% Zinc oxide added to dried samples | X-Ray diffraction |
| B_B2 | 4 ml samples of surface sediment (0-0.5 cm) were syringed into plastic bags. The samples were wrapped in Al foil and frozen. | Chlorophyll a,b,c and phaeophytin | Thaw in refrigerator and then extracted in 90% acetone | Extracts analysed by spectrophotometry (630, 647, 664 and 750 nm). Individual pigments quantified by trichometric equations and expressed on a per g dry wt basis utilising data from B_B3. |
| B_B3 | 4 ml samples of surface sediment (0-0.5 cm) were syringed into plastic bags. Samples were frozen. | Porosity and wet/dry bulk densities (0-0.5cm) | Freeze-dry | Weight difference after drying and after correction for seawater salts (porosity) and normalisation to wet/dry volumes (bulk density). |
| B_C1 | Bulk sub-sample (6.5 ml) of surface sediment (0-2 cm) incubated in BOD bottles for ~24 hrs in the dark at SST. Dissolved oxygen concentrations (and saturation values) were measured at the start and finish of each incubation. | Sediment oxygen demand | N/A | Results expressed on a per g dwt basis utilising B_B1 results. |
| B_C2 | Salinity, temperature and pH were measured on pore waters extracted from sub-samples C_D1. These pore waters were then filtered (0.45 µm) into 3 ml gas-tight vials (pre-charged with 0.025 HgCl2) within 1 hr of collection (T=0). The procedure was repeated on pore waters from an additional bulk sample collected as per C_D1 and incubated for ~24 hrs at SST (T=1). All samples were refrigerated prior to laboratory analysis. | CO2 production rates | Samples brought to room temperature in dark. | 1. Dissolved inorganic carbon (DIC) determined using a DIC analyser and infrared-based CO2 detector. (Geoscience Australia) 2. CO2 production rates calculated by concentration differences (T=1 – T=0) over the incubation period, after correction for CaCO3 fluxes. Results expressed on a per g dwt basis utilising B_B1 data. |
| B_C3 | A 4.5cm diameter core of surface sediment overlain by 0.118L of local seawater was incubated in the dark for ~4 hours at ~26oC after a ~3 hour pre-incubation period. Dissolved oxygen levels were measured at ~30 minute intervals. Samples for DIC analysis were taken at the start and finish of the incubation. At a few of the shallower stations cores were incubated at the following light levels: 0, ~5, ~25 and ~50 PAR. Blank cores (local seawater only) accompanied all incubations. | CO2 production/consumption rates/ total oxygen consumption/production ratesrates | DIC samples brought to room temperature in dark. | 1. Dissolved inorganic carbon (DIC) determined using a DIC analyser and infrared-based CO2 detector. (Geoscience Australia) 2. CO2/O2 production/consumption rates were calculated by concentration differences (T=1 – T=0) over the incubation period. Results expressed on a daily per m2 basis. |
| B_D1 | Surface sediment (0-2 cm) was syringed into acid-washed falcon vials. Pore waters were removed within 20 minutes of collection. Residual sediment was frozen for transport to the laboratory. | Major, minor, trace and rare earth elements | 1. Freeze-dry 2. Grind in tungsten carbide mill. | X-Ray Fluorescence and ICP AES (Geoscience Australia) |
|  |  | Bulk carbonate | 1. Freeze-dry 2. Grind | Carbonate Bomb (Geoscience Australia) |
|  |  | Particle surface area | 1. Freeze dry. 2. Slow heating to 350oC (12 hours). | 5-point BET (Geoscience Australia) |
|  |  | TOC, TN and C & N isotopes | 1. Freeze-dry 2. Grind 3. Acid treatment | Mass spectrometry |
| B_E1 | Spare sample. Surface sediment (~0-2 cm) was scooped into plastic bag. Samples were immediately frozen. | TBD | TBD | TBD |
| B_S1 | Carbonate specimens. Bagged and refrigerated. | TBD | TBD | TBD |

Survey Details for Towed-Video Transects

Table I.1 Towed-video transect details.

| Area | Station | Start Latitude | Start Longitude | Start Depth (m) | End Latitude | End Longitude | End Depth (m) | Duration (min) | No. Stills | Observations & Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 005CAM001 | -12.21815 | 127.39682 | 86 | -12.21800 | 127.39784 | 72 | 12 | 0 | USBL not logging; intermittent video signal; ended tow - plan to rerun with new cable. |
| 1 | 005CAM001.1 | -12.21590 | 127.39422 | 85 | -12.20538 | 127.40145 | 72 | 44 | 264 | Pockmarks, muddy; very sparse epibenthos; occasional urchin and crinoid, soft coral, small fish; High turbidity; small rocky outcrops; 800m slight increase in abundance upslope after 800 - 900m (< 75 m water depth). |
| 1 | 006CAM002 | -12.20878 | 127.42330 | 71 | -12.20257 | 127.43480 | 70 | 40 | 240 | Soft muddy; very sparse epibenthos; occasional pockmarks; very occasional crinoids, gorgonians and crinoids at 1400m. |
| 1 | 007CAM003 | -12.18530 | 127.39628 | 73 | -12.18540 | 127.40280 | 95 | 34 | 204 | Soft muddy substrate; very sparse barren epibenthos; pockmarks; low mounds; bioturbated; current forcing camera onto its side; very occasional crinoids; soft corals. |
| 1 | 008CAM004 | -12.19615 | 127.40213 | 73 | -12.18562 | 127.40960 | 96 | 39 | 234 | Soft muddy substrate; barren; pockmarks; burrows; very occasional crinoids; soft coral - on small rocky outcrops- very localised. |
| 1 | 009CAM005 | -12.20160 | 127.43143 | 74 | -12.19075 | 127.43476 | 103 | 47 | 282 | Muddy substrate; bioturbated; occasional gorgonian; sea whip; sponge - as small patches; fish; strong current. |
| 1 | 010CAM006 | -12.20843 | 127.44685 | 88 | -12.19770 | 127.45330 | 99 | 38 | 228 | Muddy substrate; soft; shell grit on surface; barren; occasional crinoid; burrows; fish; brittle stars; prawn; ray; soft coral; gorgonian; very sparse; more biota on northern edge of channel. |
| 1 | 011CAM007 | -12.20550 | 127.43892 | 91 | -12.19617 | 127.44883 | 89 | 42 | 252 | Strong tides; bioturbated; soft muddy substrate very sparse to barren epibenthos; very occasional crinoid; brittle stars; slightly more abundant at end of the tow across terrace on north side of channel; octocorals; crinoids; sea pens. |
| 1 | 012CAM008 | -12.19533 | 127.44455 | 88 | -12.19798 | 127.43270 | 100 | 46 | 276 | Soft muddy substrate; ray; sparse epibenthos; occasional crinoids, gorgonians; bioturbated, pockmarks, octocorals; prawns. USBL dropped out 08:21 - resumed 08:28. Most of channel floor barren mud with numerous mounds and pockmarks; numerous brittle stars. |
| 1 | 013CAM009 | -12.20660 | 127.43560 | 75 | -12.20683 | 127.43527 | 75 | 2 | 0 | No visibility; high suspended sediment load on bottom; tow terminated early |
| 1 | 014CAM010 | -12.07450 | 127.43548 | 30 | -12.06152 | 127.43670 | 89 | 44 | 264 | Strong tidal current, 2kn +; Hard substrate on edge of shoal; hard corals; lots of fish; irregular with mounds; gravel patches; red algae; patchy cover and epibenthos; sponges in patches; mushroom corals, very high cover in extensive continuous fields; abundance reduces below 50m W.D.; visibility OK; Large bommie at 70m covered in sponge and fish; gorgonian community below 72m. |
| 1 | 015CAM011 | -12.07073 | 127.42447 | 61 | -12.06173 | 127.43388 | 89 | 43 | 258 | Sandy gravelly substrate; sparse to patchy epibenthos; gorgonians, sponges, (relatively flat to undulating); crinoids; ~70 m = ledges with cover of sponges, gorgonians; Tide steering tow to NE; becomes sparse to barren below 70 m; with occasional bommie outcrop in sponges. |
| 1 | 016CAM012 | -12.08947 | 127.42677 | 118 | -12.08777 | 127.42735 | 110 | 5 | 30 | Muddy gravel; sparse to patchy epibenthos cover; sponges; gorgonians; sea whips; crinoids; rocky outcrops; terminated after 5 minutes due to steep rise from 100m to 30m over 100m distance. |
| 1 | 016CAM013 | -12.07805 | 127.43100 | 35 | -12.08707 | 127.42258 | 107 | 47 | 282 | Start sandy gravel, mounds; sponges, hard corals, red algae, crinoids, fish, continuous cover; steep drop offs to 70m; cover becomes sparse, occasional patches of gorgonians, sponges to 90m. |
| 1 | 017CAM014 | -12.10277 | 127.39908 | 110 | -12.09122 | 127.39505 | 111 | 55 | 330 | Soft muddy substrate; barren; sparse pockmarks; very sparse soft corals and fish. |
| 2 | 024CAM015 | -11.88680 | 127.09523 | 78 | -11.88903 | 127.10368 | 87 | 70 | 420 | Muddy soft substrate, very turbid, barren, sparse patches of hardground with sponges, gorgonians, bommies ~800m. |
| 2 | 025CAM016 | -11.89710 | 127.07040 | 71 | -11.89710 | 127.08037 | 71 | 70 | 420 | Soft muddy sand substrate; very sparse epifauna; 02:53_ stuck on bommie - 5min to release; very patchy epibenthos growing on isolated rocky outcrops - latter 500m of tow. |
| 2 | 026CAM017 | -11.89287 | 127.09500 | 61 | -11.90452 | 127.10122 | 84 | 54 | 324 | Soft sandy substrate; sparse sponges; gorgonians in localised patches; low visibility; fish; bommies; semi-continuous sponge gardens across crest of shoal; excellent tow transit across edge of platform on crest of shoal and onto lower shoal terrace, where biota are sparse. |
| 2 | 027CAM018 | -11.87965 | 127.11588 | 116 | -11.88327 | 127.11740 | 115 | 31 | 186 | Soft muddy substrate; barren; very poor visibility; terminated tow at 500m due to poor visibility and lack of epibenthic community. |
| 2 | 028CAM019 | -11.85868 | 127.03623 | 126 | -11.86310 | 127.03540 | 123 | 20 | 120 | Soft muddy substrate; barren; small fish; 1 or 2 crinoids; pockmarks/burrows; very turbid; low visibility; tow terminated at 500 m due to absence of epibenthos. |
| 2 | 029CAM020 | -11.89360 | 127.09842 | 63 | -11.89360 | 127.09842 | 63 | 4 | 24 | No USBL signal at start - tow terminated after 5 min to fix USBL. |
| 2 | 029CAM020_1 | -11.89428 | 127.09825 | 60 | -11.90388 | 127.10677 | 88 | 64 | 384 | muddy sand substrate; gorgonian; sponges in small scattered patches, 10s metres apart; rocky mounds after 01:40 UTC with sponges, crinoids, gorgonians, density of epibenthos increases; trevally school; hooked up under rock overhang 02:10 - had to turn boat to free up towvid; raised camera into water column while repositioning 02:14; resume 02:15; rich in gorgonian at start of lower terrace. |
| 2 | 031CAM021 | -11.79337 | 126.97838 | 77 | -11.80403 | 126.98572 | 90 | 64 | 384 | Soft muddy substrate; barren; very occasional soft coral; gorgonians; soft corals; increased abundance across step/ledge to 85m at 07:32; barren below 85-90m. |
| 2 | 032CAM022 | -11.79552 | 126.99378 | 93 | -11.80728 | 126.99842 | 93 | 42 | 252 | Soft muddy substrate; pockmarks; fish; very irregular bed across pockmarks; barren; best video and pockmarks thus far. |
| 2 | 033CAM023 | -11.84728 | 127.05692 | 127 | -11.85025 | 127.05590 | 127 | 17 | 102 | Soft muddy substrate; barren; terminated at 500m due to lack of bathy change at this station - substrate uniformly barren. |
| 2 | 034CAM024 | -11.88135 | 127.09223 | 104 | -11.87757 | 127.09328 | 111 | 18 | 108 | Soft muddy coarse sand substrate; low visibility; turbid; barren; pockmarks after 02:40 min. |
| 2 | 036CAM025 | -11.81475 | 127.03858 | 82 | -11.82093 | 127.05007 | 111 | 52 | 312 | No USBL at start to 06:49; mixed soft and hard substrate with patches of sponges, gorgonians, soft corals on outcrop including bommies; good visibility; semi-continuous communities of gorgonians, sponges, fish; steep drop off onto sparse muddy sands; becomes barren with distance from shoal; pockmarks. |
| 2 | 037CAM026 | -11.81868 | 127.03333 | 76 | -11.81217 | 127.02643 | 108 | 53 | 318 | Mixed substrate; muddy sand and rocky outcrop with sponge; gorgonians - relatively sparse; good visibility; rocky outcrops concentrated on edge of shoal- well colonised by biology. Excellent tow video. Improved visibility probably associated with neap tides (weaker current). Note that this tow is on the lee side with respect to dominant tides - sheltered aspect - substrate becomes barren after 09:15 UTC - Mounds at 09:21UTC. |
| 2 | 038CAM027 | -11.80768 | 126.96805 | 77 | -11.81757 | 126.97727 | 104 | 59 | 354 | Muddy sand and gravel substrate; pockmarks; mostly barren; improved visibility - likely due to weaker tidal currents on neaps; good video of pockmark holes and mounds; sparse patches of sponges on irregular substrate (rock?) across small ledges (80-90 m W.D.); barren and muddier below 90 m; pockmarks. |
| 2 | 039CAM028 | -11.78048 | 127.01048 | 90 | -11.77778 | 127.01365 | 98 | 16 | 96 | Soft muddy substrate; mounds, pockmarks, barren; terminated at 500 m due to the lack of variation across uniform bathymetry. |
| 2 | 040CAM029 | -11.80948 | 126.99110 | 100 | -11.82332 | 126.98855 | 110 | 53 | 318 | Soft muddy substrate; barren; moderate visibility; tow line crosses small knoll; sparse epibenthic community of sponges, sea whips, soft corals, gorgonians on hardground in 100-105 m W.D. |
| 3 | 044CAM030 | -11.41050 | 126.92420 | 88 | -11.41832 | 126.93482 | 63 | 43 | 258 | Soft muddy substrates; barren; pockmarks and small soft mounds; bioturbated; sparse patch of octocorals, gorgonians, sponges at 06:40 on mounds to 06:45 up to 65m - bommie at 06:47; reef-like. |
| 3 | 045CAM031 | -11.45150 | 126.92027 | 95 | -11.43893 | 126.92602 | 104 | 59 | 354 | Soft muddy substrate; mostly barren; isolated octocoral, gorgonian on small mounds; pockmarks, bioturbated. |
| 3 | 046CAM032 | -11.43402 | 126.93842 | 62 | -11.44693 | 126.93498 | 108 | 44 | 264 | Muddy sandy substrate; sparse to patchy epibenthos, sponges, gorgonians; good light and visibility; becomes very sparse after drop off into 80 m +; barren and pockmarked; soft mud. |
| 3 | 047CAM033 | -11.41918 | 126.94457 | 64 | -11.41195 | 126.94910 | 881 | 49 | 294 | Sandy muddy substrate; barren at start - irregular seabed, mounds; good light and visibility; occasional octocoral, small sponge, fish, bommies with 100% cover of biology; bommies spaced 10's metres; concentration of biology on small ledge drop off to 75 m. |
| 3 | 049CAM034 | -11.40628 | 126.92817 | 84 | -11.39768 | 126.91885 | 91 | 49 | 294 | Muddy sandy substrate with irregular mounds, patches of biology on mounds; gorgonians, sponges, sea whips, soft corals; becomes barren below 85 m; numerous pockmarks and mounds; very sparse small patches of biology. |
| 3 | 050CAM035 | -11.40462 | 126.96122 | 99 | -11.39188 | 126.96232 | 81 | 48 | 288 | Soft muddy substrate; bioturbated; barren; pockmarks; rocky outcrops at 10:12 near end of tow (82m); patch of hydroids, crinoids. |
| 3 | 051CAM036 | -11.43092 | 126.98882 | 95 | -11.41962 | 126.99582 | 104 | 51 | 306 | Soft muddy substrate; pockmarks; irregular mounds; barren; bioturbated. |
| 3 | 052CAM037 | -11.43623 | 126.99827 | 96 | -11.43603 | 127.01115 | 105 | 44 | 264 | Soft muddy substrate; barren; pockmarks and burrows. |
| 3 | 053CAM038 | -11.43918 | 126.96487 | 56 | -11.45032 | 126.97190 | 91 | 44 | 264 | Muddy sand and gravel substrate on bank platform; irregular surface; patchy to thin epibenthos-small sponges, gorgonians, sea whips; good light and visibility; bommies and reef at 06:38 - benthos increases in abundance; large barrel sponges; abrupt transition over drop off to 78 m with muddier substrate; still patchy gorgonians and sponges - very irregular bed with rocky outcrop; grades to barren mud by 06:45; strongly pockmarked. |
| 3 | 054CAM039 | -11.42683 | 126.96030 | 62 | -11.41752 | 126.96945 | 95 | 38 | 228 | Soft muddy substrate; barren, occasional bommie with cover of gorgonians, sponges, octocorals, fish; steep drop off at 09:35 to 82 m; rocky at base with gorgonians, sponges assemblages in patches becomes sparse with distance from shoal - pockmarked muds. |
| 3 | 055CAM040 | -11.43477 | 126.95460 | 59 | -11.44968 | 126.95403 | 103 | 52 | 6 | Muddy sand and gravel substrate; sparse to patchy epibenthos; bioturbated; generally flat and barren across top of shoal platform; good light and visibility; bed roughness high and benthos cover toward edge and drop off; becomes muddy and sparse on flats below shoal; pockmarks; couple of patches of sponges across lower platform edge (95 m); stills camera failed. |
| 3 | 055CAM040_1 | -11.43675 | 126.95483 | 59 | -11.44345 | 126.95493 | 85 | 24 | 144 | As above – partial re-run of transect across |
| 3 | 056CAM041 | -11.40055 | 127.02057 | 72 | -11.40080 | 127.02065 | 74 | 21 | 126 | Muddy sand substrate; barren at start then rises abruptly 20 m to top of shoal; hooked up on ledge; pull out and redeploy down-slope. |
| 3 | 056CAM041_1 | -11.40182 | 127.02313 | 50 | -11.39285 | 127.03227 | 108 | 40 | 240 | Sandy gravel (?) with semi-continuous sponge garden; lots of fish; good light and visibility; steep drop off to 77 m; muddy sand with gorgonians, sponges; becomes barren with distance from shoal and pockmarks. |
| 3 | 057CAM042 | -11.44437 | 127.09490 | 110 | -11.45208 | 127.08058 | 90 | 66 | 396 | Soft muddy substrate; barren; pockmarks; burrows; 10 m ledge at end of tow with gorgonians, crinoids, sponge community. |
| 3 | 058CAM043 | -11.45052 | 127.03443 | 81 | -11.43987 | 127.02733 | 106 | 38 | 228 | Transect starts on shoal platform perimeter; rocky outcrops (bommies) with gorgonians, sponge cover; soft muddy substrate in between with sparse cover. |
| 3 | 061CAM044 | -11.43950 | 126.98153 | 56 | -11.44523 | 126.98068 | 93 | 41 | 246 | Start of tow on shoal edge; rich assemblage of gorgonians, sponges on rocky substrate; becomes sparse to barren on flats below shoal; soft muddy substrate. |
| 3 | 062CAM045 | -11.41237 | 126.94013 | 64 | -11.39950 | 126.93783 | 88 | 50 | 300 | Tow start at edge of shoal platform; rocky with patches of sponge and gorgonians growth; bommies also with good cover of sponges; becomes barren muddy substrate (82 m W.D.); pockmarks; poor visibility; USBL cut out near end. |
| 3 | 063CAM046 | -11.39855 | 126.91085 | 97 | -11.39555 | 126.90540 | 100 | 17 | 102 | Night time tow; soft muddy substrate; barren tow crosses low rise/ knoll with patchy sponges and gorgonians, octocorals communities on muddy sand (?) with rocky (?) outcrops (85 m); semi-continuous cover across knoll; terminated tow at 600 m at western edge of survey area. |
| 4 | 067CAM047 | -11.32767 | 126.95342 | 44 | -11.33712 | 126.96223 | 73 | 60 | 360 | Hard pavement substrate; semi-continuous cover of soft and hard corals, gorgonians, sponges (including acropora); good light and visibility; sand in patches; bommies toward edge of platform; becomes sparse to barren below 65 m after drop off. |
| 4 | 069CAM048 | -11.32802 | 126.95400 | 43 | -11.32540 | 126.95038 | 75 | 13 | 78 | Hardground with semi-continuous cover of sponges, corals (hard and soft), gorgonians, reef fish; becomes sparse below 65 m. |
| 4 | 070CAM049 | -11.32968 | 126.95798 | 77 | -11.32650 | 126.95798 | 77 | 15 | 90 | Hardground with semi-continuous cover of sponges, corals (hard and soft), gorgonians, crinoids; becomes sparse to barren below 60 m after drop off. |
| 1 | 073CAM050 | -12.07165 | 127.42402 | 31 | -12.05950 | 127.42402 | 99 | 43 | 258 | Tow start on crest of shoal platform; with sand/gravel patches; patches of sponges; hard corals; gorgonians; good light and visibility; becomes sparse to patchy off platform (W.D > 55m; substrate flat - no pockmarks on terrace (sandy); bommies on low ridges with gorgonians and sponges. |
| 1 | 074CAM051 | -12.07118 | 127.43513 | 31 | -12.07697 | 127.44807 | 89 | 40 | 240 | and sandy gravel substrate; large 2D bedforms (31m) of gravel (~0.5m high); gravel in troughs; very sparse epibenthos; bedform field extends 10's m; transition to rocky substrate, irregular with sponges, corals, fish; bommies; 2nd bedform field after outcrops still in 31 m W.D.; 2nd area of irregular rocky outcrop, rich in hard of soft corals, fish. Transition off shoal to muddy substrate with occasional bommie. |
| 1 | 075CAM052 | -12.12818 | 127.44778 | 110 | -12.13933 | 127.45395 | 107 | 49 | 294 | Start on shoal platform, running south; mixed soft and hard substrate with coral patches of gorgonians, sponges on outcrops; poor visibility; edge of shoal characterised by small outcrops with gorgonians and sponge growth, fish; becomes barren; muddy substrate off the shoal. |

Survey Details for Benthic Sled Transects

Table J.1 Benthic sled transect details. All transects were deployed over banks. Biota weight includes the total wet weight of all live samples collected.

| Area | Station | Sample | Start Latitude | Start Longitude | Start Depth (m) | End Latitude | End Longitude | End Depth (m) | Biota wgt (kg) | Sample Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 6 | SOL5650/006BS001 | -12.2039 | 127.4323 | 72.4 | -12.2037 | 127.4327 | 74.3 | 0.348 |  |
| 1 | 14 | SOL5650/014BS002 | -12.0692 | 127.4361 | 36.5 | -12.0696 | 127.4361 | 35 | 3.96 | Carbonate gravel with sponges |
| 1 | 15 | SOL5650/015BS003 | -12.0688 | 127.4275 | 61.2 | -12.0684 | 127.4279 | 64.9 | 1.15 | Carbonate gravel with sponges |
| 1 | 16 | SOL5650/016BS004 | -12.0801 | 127.4290 | 44.9 | -12.0803 | 127.4296 | 46.7 | 64.57 | Sponges |
| 2 | 24 | SOL5650/024BS005 | -11.8864 | 127.0971 | 83.7 | -11.8862 | 127.0964 | 82.9 | 0.14 | Mostly mud; limited epibenthos |
| 2 | 25 | SOL5650/025BS006 | -11.8920 | 127.0753 | 74.1 | -11.8920 | 127.0750 | 74.2 | 0.32 | Mostly mud; limited epibenthos |
| 2 | 26 | SOL5650/026BS007 | -11.8993 | 127.0981 | 58 | -11.8989 | 127.0979 | 58 | 0.76 | Sponges, corals, cemented carbonates |
| 2 | 29 | SOL5650/029BS008 | -11.8990 | 127.1024 | 57.9 | -11.8989 | 127.1027 | 58.4 | 18.02 | Sponges, gorgonians |
| 2 | 31 | SOL5650/031BS009 | -11.7945 | 126.9860 | 82.7 | -11.7949 | 126.9856 | 82.3 | 2.12 | Large cobble sized block of coral - sampled in pieces for analysis (C-14) |
| 2 | 36 | SOL5650/036BS010 | -11.8168 | 127.0438 | 80.8 | -11.8165 | 127.0441 | 81.3 | 2.6 | Biota in muddy sand |
| 2 | 37 | SOL5650/037BS011 | -11.8188 | 127.0310 | 91.7 | -11.8182 | 127.0314 | 91.6 | 40.53 |  |
| 2 | 38 | SOL5650/038BS012 | -11.8097 | 126.9699 | 82.5 | -11.8096 | 126.9706 | 82.7 | 0.16 | Mostly sandy mud; shell fragments; cobble sized fragment of calcareous cement retained |
| 3 | 44 | SOL5650/044BS013 | -11.4181 | 126.9328 | 70.7 | -11.4177 | 126.9327 | 72.6 | 0 | Net broke, no sample (likely due to large amount of mud) |
| 3 | 46 | SOL5650/046BS014 | -11.4335 | 126.9375 | 63.7 | -11.4330 | 126.9371 | 63.7 | 8.92 | Muddy sand, carbonate rock sample retained |
| 3 | 47 | SOL5650/047BS015 | -11.4119 | 126.9491 | 81 | -11.4116 | 126.9493 | 81.8 | 0.36 | Mostly sandy mud, some biota |
| 3 | 53 | SOL5650/053BS016 | -11.4405 | 126.9657 | 56.6 | -11.4399 | 126.9653 | 57 | 4.76 | Small amount of biota in muddy sand, carbonate rock retained |
| 3 | 55 | SOL5650/055BS017 | -11.4395 | 126.9513 | 58.1 | -11.4400 | 126.9519 | 59.8 | 19.23 | Muddy sand and biology; sponge included; carbonate rock retained |
| 3 | 56 | SOL5650/056BS018 | -11.4015 | 127.0263 | 52.5 | -11.4015 | 127.0257 | 52.3 | 17.77 | All biota, no mud |
| 3 | 61 | SOL5650/061BS019 | -11.4385 | 126.9684 | 57.3 | -11.4393 | 126.9682 | 58.1 | 5.44 | Biota and carbonate rocks |
| 3 | 63 | SOL5650/063BS020 | -11.3972 | 126.9083 | 86.4 | -11.3969 | 126.9078 | 87.2 | 6.34 | Several large slabs of cemented carbonates with coral base - retained |
| 1 | 73 | SOL5650/073BS021 | -12.0683 | 127.4276 | 67.9 | -12.0688 | 127.4269 | 66.1 | 4.18 |  |
| 1 | 74 | SOL5650/074BS022 | -12.0748 | 127.4435 | 55.9 | -12.0754 | 127.4437 | 65.8 | 5.82 | Rubble, shells and biology; geological samples of corals retained |

Biodiscovery Samples

Table K.1 Biological specimens processed for biodiscovery purposes.

| Area | Station Number | Sample Number | ID Number | Phylum | Family | Genus | Species | Common Name | Weight (g) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 16 | 16BS04 | 10382 | Porifera |  |  |  | Sponge | 50 |
| 3 | 53 | 53BS16 | 10219 | Porifera | Irciniidae | Ircinia | Finger Balls | Sponge | 110 |
| 3 | 53 | 53BS16 | 10220 | Porifera | Irciniidae | Ircinia | Old Man Fingers | Sponge | <100 |
| 1 | 14 | 14BS02 | 10264 | Porifera |  |  |  | Sponge | 100 |
| 1 | 15 | 15BS03 | 10290 | Porifera |  | Ianthella |  | Sponge | 100 |
| 1 | 16 | 16BS04 | 10365 | Porifera |  |  |  | Sponge | 100 |
| 1 | 16 | 16BS04 | 10369 | Bryozoa |  |  |  | Bryozoan | 100 |
| 1 | 16 | 16BS04 | 10377 | Porifera |  | Reniochalina | Sponge |  | 100 |
| 1 | 16 | 16BS04 | 10380 | Porifera | Dictyonellidae | Stylissa | carteri | Sponge | 100 |
| 1 | 16 | 16BS04 | 10388 | Porifera |  |  |  | Sponge | 100 |
| 2 | 29 | 29BS08 | 10490 | Porifera |  |  |  | Sponge | 100 |
| 2 | 31 | 31BS09 | MAGNT0024 | Chordata |  |  |  | Ascidian | <100 |
| 2 | 36 | 36BS10 | MAGNT0056 | Porifera |  | Oceanapia | Sponge |  | 100 |
| 2 | 36 | 36BS10 | MAGNT0057 | Porifera |  | Microscleroderma | Sponge |  | 100 |
| 2 | 36 | 36BS10 | MAGNT0058 | Porifera |  | Aka? |  | Sponge | 100 |
| 2 | 36 | 36BS10 | MAGNT0059 | Porifera |  | Spongia |  | Sponge | 100 |
| 3 | 46 | 46BS14 | 10177 | Porifera |  | Suberea? |  | Sponge | 200 |
| 3 | 46 | 46BS14 | 10180 |  |  | Geodia? |  |  |  |
| 3 | 46 | 46BS14 | 10181 | Porifera |  | Cymbastella | Sponge |  | 100 |
| 3 | 46 | 46BS14 | 10184 | Porifera |  | Thorecta |  | Sponge | 100 |
| 3 | 55 | 55BS17 | 10233 | Porifera |  | Ircinia |  | Sponge | 1600 |
| 3 | 55 | 55BS17 | 10235 | Porifera |  | Clathria |  | Sponge | 150 |
| 3 | 56 | 56BS18 | 20776 | Porifera |  |  |  | Sponge | 100 |
| 3 | 56 | 56BS18 | 20818 | Cnidaria |  |  |  | soft coral | 110 |
| 3 | 61 | 61BS19 | 20836 | Porifera |  | Cymbastella | Sponge |  | 120 |
| 3 | 73 | 73BS21 | 20942 |  |  |  |  | Gorgonian | 100 |
| 3 | 46 | 46BS14 | 10186 | Porifera | Thorectidae |  |  | Sponge | 110 |
| 2 | 29 | 29BS08 | 10485 | Porifera |  | Coelocarteria | Sponge |  | 120 |
| 3 | 46 | 46BS14 | 10189 | Cnidaria |  |  |  | Gorgonian | 120 |
| 3 | 46 | 46BS14 | 10187 | Porifera |  | Reniochalina | Sponge |  | 150 |
| 3 | 46 | 46BS14 | 10188 | Porifera |  | Axechina |  | Sponge | 150 |
| 3 | 55 | 55BS17 | 20743 | Porifera |  | Echinodyctium | Sponge |  | 200 |
| 3 | 56 | 56BS18 | 10249 | Porifera | Thorectidae |  | Yellow Mucus | Sponge | 200 |
| 3 | 53 | 53BS16 | 10221 | Cnidaria |  |  |  | Gorgonian | <100 |
| 3 | 55 | 55BS17 | 20746 | Porifera |  |  |  | Sponge | 180 |
| 1 | 74 | 74BS22 | 20991 |  |  |  |  | Sponge | 200 |
| 1 | 16 | 16BS04 | 10375 | Cnidaria |  |  |  | Gorgonian | 190 |
| 1 | 14 | 14BS02 | 10265 | Porifera |  |  |  | Sponge | 200 |
| 1 | 14 | 14BS02 | 10267 | Porifera |  |  |  | Sponge | 200 |
| 1 | 15 | 15BS03 | 10310 | Cnidaria |  |  |  | Soft coral | 200 |
| 1 | 16 | 16BS04 | 10366 | Porifera |  |  |  | Sponge | 200 |
| 1 | 16 | 16BS04 | 10367 | Porifera |  |  |  | Sponge | 200 |
| 1 | 16 | 16BS04 | 10379 | Porifera |  | Cymbastella | Sponge |  | 200 |
| 2 | 29 | 29BS08 | 10488 | Porifera |  | Ircinia |  | Sponge | 200 |
| 3 | 46 | 46BS14 | 10176 | Porifera |  | Xestospongia | Sponge |  | 2000 |
| 3 | 46 | 46BS14 | 10179 | Porifera |  |  |  | Sponge | 200 |
| 3 | 46 | 46BS14 | 10182 | Porifera | Thorectidae |  |  | Sponge | 200 |
| 3 | 46 | 46BS14 | 10185 | Porifera |  | Oceanapia | Sponge |  | 200 |
| 3 | 46 | 46BS14 | 10191 | Cnidaria |  |  |  | Gorgonian | 200 |
| 3 | 53 | 53BS16 | 10216 | Porifera | Irciniidae | Ircinia |  | Sponge | 300 |
| 3 | 55 | 55BS17 | 10234 | Porifera |  | Thorecta |  | Sponge | 300 |
| 3 | 55 | 55BS17 | 10239 | Porifera |  | Spongia |  | Sponge | 400 |
| 3 | 56 | 56BS18 | 20786 | Porifera |  | Ircinia | Reptant Fingers | Sponge | 230 |
| 1 | 74 | 74BS22 | 20981 |  |  |  |  | Sponge | <100 |
| 1 | 16 | 16BS04 | 10374 | Cnidaria |  |  |  | Gorgonian | 210 |
| 1 | 16 | 16BS04 | 10376 | Echinodermata |  |  | Crinoid |  | 210 |
| 3 | 55 | 55BS17 | 20739 | Porifera | Thorectidae |  | Yellow Mucus | Sponge | 210 |
| 2 | 36 | 36BS10 | MAGNT0055 | Porifera |  | Theonella | Sponge |  | 240 |
| 3 | 46 | 46BS14 | 10183 | Porifera | Thorectidae |  |  | Sponge | 250 |
| 3 | 53 | 53BS16 | 10215 | Porifera |  | Microscleroderma | Sponge |  | 250 |
| 3 | 55 | 55BS17 | 10232 | Porifera | Thorectidae |  | Corneto | Sponge | 250 |
| 1 | 16 | 16BS04 | 10391 | Porifera |  | Ectyoplasia | Sponge |  | 270 |
| 3 | 61 | 61BS19 | 20837 | Porifera |  | Cymbastella | Salad Bowl | Sponge | 300 |
| 1 | 14 | 14BS02 | 10263 | Porifera |  |  |  | Sponge | 300 |
| 1 | 16 | 16BS04 | 10384 | Porifera |  | Ianthella |  | Sponge | 300 |
| 1 | 16 | 16BS04 | 10386 | Porifera |  |  |  | Sponge | 300 |
| 2 | 37 | 37BS11 | MAGNT0079 | Porifera |  | Oceanapia | Sponge |  | 300 |
| 3 | 46 | 46BS14 | 10173 | Cnidaria |  |  |  | Gorgonian | 1000 |
| 3 | 53 | 53BS16 | 10212 | Porifera |  | Oceanapia | Sponge |  | 480 |
| 3 | 56 | 56BS18 | 10242 | Porifera |  | Ianthella | basta | Sponge | 300 |
| 3 | 56 | 56BS18 | 10247 | Porifera |  | Coelocarteria | Orange | Sponge | 400 |
| 1 | 73 | 73BS21 | 20925 | Porifera |  |  |  | Sponge | 410 |
| 1 | 15 | 15BS03 | 10304 | Cnidaria |  | Tubipora |  | Tubipora | 350 |
| 3 | 56 | 56BS18 | 10243 | Porifera |  | Ianthella | flabelliformis | Sponge | 350 |
| 3 | 61 | 61BS19 | 20835 | Porifera |  | Thorecta | Corneto | Sponge | 380 |
| 1 | 16 | 16BS04 | 10383 | Porifera |  | Theonella | Sponge |  | 370 |
| 1 | 16 | 16BS04 | 10381 | Echinodermata |  |  | Crinoid |  | 400 |
| 1 | 16 | 16BS04 | 10385 | Cnidaria |  | Nephtia Cf. | Soft coral |  | 400 |
| 1 | 16 | 16BS04 | 10390 | Porifera |  | Thorecta |  | Sponge | 400 |
| 2 | 29 | 29BS08 | 10486 | Porifera |  | Spheciospongia | Sponge |  | 400 |
| 3 | 63 | 63BS20 | 20879 | Porifera |  | Xestospongia | Sponge |  | 400 |
| 3 | 63 | 63BS20 | 20881 | Echinodermata |  |  | Holothurian |  | 400 |
| 1 | 74 | 74BS22 | 20984 |  |  |  |  | Sponge | 400 |
| 3 | 56 | 56BS18 | 10248 | Porifera |  | Spheciospongia | purpurea? | Sponge | 600 |
| 2 | 29 | 29BS08 | 10482 | Porifera |  | Xestospongia | Sponge |  | 420 |
| 3 | 53 | 53BS16 | 10214 | Porifera | Thorectidae |  | Mucus | Sponge | 420 |
| 3 | 53 | 53BS16 | 10222 | Cnidaria |  |  |  | Gorgonian | 470 |
| 3 | 56 | 56BS18 | 10240 | Porifera |  | Pseudoceratina | Sponge |  | 600 |
| 3 | 46 | 46BS14 | 10170 | Cnidaria |  |  |  | Gorgonian | 700 |
| 3 | 55 | 55BS17 | 10223 | Porifera |  | Ianthella |  | Sponge | 600 |
| 3 | 63 | 63BS20 | 20880 | Chordata |  |  |  | Ascidian | 500 |
| 1 | 74 | 74BS22 | 20990 |  |  |  |  | Sponge | 500 |
| 1 | 16 | 16BS04 | 10387 | Porifera |  | Oceanapia | Sponge |  | 600 |
| 2 | 29 | 29BS08 | 10483 | Porifera |  | Oceanapia | Sponge |  | 600 |
| 2 | 29 | 29BS08 | 10484 | Porifera |  | Oceanapia | Sponge |  | 600 |
| 2 | 29 | 29BS08 | 10487 | Porifera |  | Theonella | Sponge |  | 600 |
| 2 | 37 | 37BS11 | MAGNT0080 | Porifera |  |  |  | Sponge | 600 |
| 1 | 73 | 73BS21 | 20943 |  |  |  |  | Soft Coral | 700 |
| 1 | 74 | 74BS22 | 20989 |  |  |  |  | Sponge | 700 |
| 1 | 16 | 16BS04 | 10371 | Porifera |  |  |  | Sponge | 650 |
| 1 | 74 | 74BS22 | 20974 |  |  |  |  | Sponge | 800 |
| 3 | 46 | 46BS14 | 10172 | Cnidaria |  |  |  | Gorgonian | 300 |
| 3 | 53 | 53BS16 | 10211 | Porifera |  | Oceanapia | Sponge |  | 800 |
| 3 | 55 | 55BS17 | 20751 | Cnidaria |  |  |  | Gorgonian | 700 |
| 1 | 74 | 74BS22 | 20999 |  |  |  |  | Sponge | 700 |
| 1 | 16 | 16BS04 | 10393 | Porifera |  | Ianthella |  | Sponge | 800 |
| 2 | 29 | 29BS08 | 10489 | Porifera |  | Cinachyrella | Sponge |  | 800 |
| 3 | 55 | 55BS17 | 10230 | Porifera |  | Oceanapia | Sponge |  | 800 |
| 3 | 46 | 46BS14 | 10171 | Cnidaria |  |  |  | Gorgonian | 500 |
| 3 | 55 | 55BS17 | 10224 | Porifera |  | Ianthella |  | Sponge | 900 |
| 2 | 37 | 37BS11 | MAGNT0076 | Porifera |  | Xestospongia | Sponge |  | 17500 |
| 3 | 46 | 46BS14 | 10175 | Porifera |  | Oceanapia | Sponge |  | 1600 |
| 3 | 55 | 55BS17 | 10228 | Porifera |  | Thorecta |  | Sponge | 1399 |
| 3 | 55 | 55BS17 | 10229 | Porifera |  | Oceanapia | Sponge |  | 1300 |
| 3 | 55 | 55BS17 | 10231 | Porifera |  | Xestospongia | Sponge |  | 2000 |
| 1 | 16 | 16BS04 | 10395 | Porifera |  | Echinodyctium | Sponge |  | 1050 |
| 2 | 36 | 36BS10 | MAGNT0053 | Porifera |  | Xestospongia | Sponge |  | 5000 |
| 2 | 37 | 37BS11 | MAGNT0077 | Porifera |  | Theonella | Sponge |  | 16700 |
| 3 | 46 | 46BS14 | 10174 | Cnidaria |  |  |  | Gorgonian | 300 |
| 3 | 53 | 53BS16 | 10213 | Porifera |  | Xestospongia | testudinaria | Sponge | 1500 |
| 3 | 55 | 55BS17 | 10227 | Porifera |  |  |  | Sponge | 2000 |
| 3 | 56 | 56BS18 | 10245 | Porifera |  | Xestospongia | testudinaria | Sponge | 11000 |
| 3 | 63 | 63BS20 | 20878 | Porifera |  | Rhabdastrella | Sponge |  | 1300 |
| 1 | 73 | 73BS21 | 20923 | Porifera |  |  |  | Sponge | 1600 |
| 1 | 73 | 73BS21 | 20924 | Porifera |  |  |  | Sponge | 300 |
| 1 | 14 | 14BS02 | 10262 | Porifera |  |  |  | Sponge | 1600 |
| 1 | 16 | 16BS04 | 10389 | Porifera |  | Scleritoderma | Sponge |  | 1200 |
| 1 | 16 | 16BS04 | 10392 | Porifera |  | Coelocarteria | Sponge |  | 1200 |
| 1 | 16 | 16BS04 | 10394 | Porifera |  | Petrosia |  | Sponge | 1200 |
| 1 | 16 | 16BS04 | 10396 | Porifera |  | Xetospongia | testudinaria | Sponge | 30000 |
| 2 | 29 | 29BS08 | 10479 | Cnidaria |  |  |  | Gorgonian | 3920 |
| 2 | 29 | 29BS08 | 10480 | Porifera |  | Xestospongia | Sponge |  | 5000 |
| 2 | 29 | 29BS08 | 10481 | Porifera |  | Petrosia |  | Sponge | 1200 |
| 2 | 31 | 31BS09 | MAGNT0025 | Porifera |  | Xestospongia | Sponge |  | 1200 |
| 2 | 36 | 36BS10 | MAGNT0054 | Porifera |  | Xestospongia | Sponge |  | 5600 |
| 3 | 55 | 55BS17 | 10226 | Porifera |  | Suberea |  | Sponge | 5000 |
| 3 | 61 | 61BS19 | 20834 | Porifera |  | Thorecta |  | Sponge | 2500 |
| 3 | 56 | 56BS18 | 10246 | Porifera |  | Ircinia |  | Sponge | 1600 |
| 2 | 37 | 37BS11 | MAGNT0078 | Porifera |  | Ircinia |  | Sponge | 5000 |

Survey Details for Baited Remote Underwater Video Stations (BRUVS)

Table L.1 Baited remote underwater videos (BRUV) station details.

| Area | Rig | Camera | Latitude | Longitude | Depth (m) |
| --- | --- | --- | --- | --- | --- |
| 1 | 5 | 8 | -12.0777 | 127.432 | 33 |
| 1 | 5 | 9 | -12.0777 | 127.4247 | 43 |
| 1 | 6 | 10 | -12.0762 | 127.4274 | 34 |
| 1 | 4 | 11 | -12.0747 | 127.4316 | 31 |
| 1 | 3 | 12 | -12.0712 | 127.4341 | 33 |
| 1 | 7 | 13 | -12.074 | 127.4432 | 60 |
| 1 | 8 | 14 | -12.0777 | 127.4424 | 54 |
| 1 | 8 | 1 | -12.0736 | 127.4248 | 38 |
| 1 | 7 | 2 | -12.0701 | 127.4276 | 46 |
| 1 | 2 | 3 | -12.068 | 127.4301 | 58 |
| 1 | 1 | 4 | -12.0683 | 127.4376 | 47 |
| 1 | 3 | 5 | -12.07 | 127.4403 | 45 |
| 1 | 4 | 6 | -12.0727 | 127.4381 | 32 |
| 1 | 6 | 7 | -12.0764 | 127.4355 | 31 |
| 1 | 1 | 15 | -12.0796 | 127.4373 | 48 |
| 1 | 2 | 16 | -12.0806 | 127.432 | 49 |
| 2 | 3 | 24 | -11.8883 | 127.0781 | 77 |
| 2 | 3 | 25 | -11.8914 | 127.0739 | 74 |
| 2 | 7 | 26 | -11.8914 | 127.0841 | 67 |
| 2 | 6 | 27 | -11.8903 | 127.0959 | 77 |
| 2 | 8 | 28 | -11.8979 | 127.0938 | 59 |
| 2 | 4 | 29 | -11.9003 | 127.0921 | 67 |
| 2 | 5 | 30 | -11.8992 | 127.1017 | 58 |
| 2 | 2 | 17 | -11.8913 | 127.0635 | 61 |
| 2 | 1 | 18 | -11.8941 | 127.0668 | 64 |
| 2 | 5 | 19 | -11.8924 | 127.0727 | 74 |
| 2 | 4 | 20 | -11.8972 | 127.0784 | 73 |
| 2 | 8 | 21 | -11.8975 | 127.0868 | 63 |
| 2 | 6 | 22 | -11.8917 | 127.0886 | 61 |
| 2 | 7 | 23 | -11.8874 | 127.0839 | 77 |
| 2 | 1 | 31 | -11.8949 | 127.1006 | 61 |
| 2 | 2 | 32 | -11.8982 | 127.1064 | 62 |
| 3 | 5 | 33 | -11.4014 | 127.0209 | 55 |
| 3 | 4 | 34 | -11.4004 | 127.022 | 57 |
| 3 | 1 | 35 | -11.4029 | 127.0221 | 55 |
| 3 | 2 | 36 | -11.4009 | 127.0237 | 52 |
| 3 | 6 | 37 | -11.4022 | 127.0243 | 51 |
| 3 | 7 | 38 | -11.4014 | 127.0262 | 52 |
| 3 | 3 | 39 | -11.4036 | 127.0264 | 54 |
| 3 | 8 | 40 | -11.403 | 127.029 | 56 |
| 3 | 8 | 41 | -11.411 | 126.9341 | 68 |
| 3 | 7 | 42 | -11.414 | 126.9424 | 66 |
| 3 | 3 | 43 | -11.4143 | 126.9501 | 67 |
| 3 | 5 | 44 | -11.4176 | 126.9551 | 68 |
| 3 | 6 | 45 | -11.4266 | 126.9594 | 63 |
| 3 | 1 | 46 | -11.4299 | 126.9656 | 62 |
| 3 | 4 | 47 | -11.4364 | 126.9622 | 58 |
| 3 | 2 | 48 | -11.4421 | 126.9654 | 58 |
| 3 | 2 | 49 | -11.438 | 126.956 | 60 |
| 3 | 4 | 50 | -11.4311 | 126.9529 | 61 |
| 3 | 1 | 51 | -11.4306 | 126.9464 | 62 |
| 3 | 6 | 52 | -11.4326 | 126.9382 | 62 |
| 3 | 5 | 53 | -11.4258 | 126.9377 | 62 |
| 3 | 3 | 54 | -11.423 | 126.9438 | 65 |
| 3 | 7 | 55 | -11.4197 | 126.9454 | 65 |
| 3 | 8 | 56 | -11.4172 | 126.9359 | 65 |

Survey Details for Baited Underwater Water Column Video (SISSTAs)

Table M.1 Baited mid water column video (SISSTAs) station details. (* denotes camera not recovered)

| Station | Area | Latitude | Longitude | Depth |
| --- | --- | --- | --- | --- |
| 1 | 1 | -12.1498 | 127.4533 | 105 |
| 2 | 1 | -12.142 | 127.4302 | 110 |
| 3 | 1 | -12.1552 | 127.4156 | 109 |
| 4 | 1 | -12.1611 | 127.4016 | 107 |
| 5 | 1 | -12.1625 | 127.3888 | 104 |
| 6 | 1 | -12.1802 | 127.3957 | 102 |
| 7 | 1 | -12.1749 | 127.4041 | 99 |
| 8 | 1 | -12.1858 | 127.4298 | 106 |
| 9 | 1 | -12.1845 | 127.4499 | 74 |
| 10 | 1 | -12.1668 | 127.4547 | 114 |
| 11 | 1 | -12.1415 | 127.395 | 108 |
| 12 | 1 | -12.1448 | 127.4034 | 109 |
| 13 | 1 | -12.1354 | 127.4144 | 77 |
| 14 | 1 | -12.1438 | 127.4153 | 109 |
| 15 | 1 | -12.1348 | 127.4323 | 93 |
| 16 | 1 | -12.1343 | 127.4412 | 90 |
| 17 | 1 | -12.1304 | 127.4478 | 79 |
| 18 | 1 | -12.1191 | 127.4353 | 72 |
| 19* | 1 | -12.1086 | 127.4379 | 91 |
| 20 | 1 | -12.1017 | 127.4359 | 125 |
| 21 | 1 | -12.0647 | 127.3989 | 106 |
| 22 | 1 | -12.0734 | 127.4039 | 106 |
| 23 | 1 | -12.0669 | 127.4159 | 76 |
| 24 | 1 | -12.0722 | 127.4315 | 31 |
| 25 | 1 | -12.0655 | 127.4529 | 166 |
| 26 | 1 | -12.079 | 127.4452 | 75 |
| 27 | 1 | -12.0866 | 127.4375 | 93 |
| 28 | 1 | -12.0862 | 127.4167 | 103 |
| 29 | 1 | -12.0944 | 127.4048 | 112 |
| 30 | 1 | -12.0955 | 127.3872 | 111 |
| 31 | 1 | -12.2197 | 127.445 | 74 |
| 32 | 1 | -12.201 | 127.4529 | 118 |
| 33 | 1 | -12.1935 | 127.4422 | 95 |
| 34 | 1 | -12.2002 | 127.4202 | 71 |
| 35 | 1 | -12.185 | 127.4143 | 77 |
| 36 | 1 | -12.1888 | 127.4058 | 85 |
| 37 | 1 | -12.202 | 127.4097 | 71 |
| 38 | 1 | -12.2167 | 127.4075 | 90 |
| 39 | 1 | -12.2182 | 127.396 | 87 |
| 40 | 1 | -12.194 | 127.3879 | 75 |
| 41 | 2 | -11.8765 | 127.1159 | 117 |
| 42 | 2 | -11.8895 | 127.1141 | 113 |
| 43 | 2 | -11.9032 | 127.1229 | 103 |
| 44 | 2 | -11.9004 | 127.1118 | 102 |
| 45 | 2 | -11.896 | 127.1092 | 81 |
| 46 | 2 | -11.9001 | 127.1064 | 75 |
| 47 | 2 | -11.9073 | 127.1043 | 109 |
| 48 | 2 | -11.9116 | 127.1011 | 111 |
| 49 | 2 | -11.9225 | 127.0812 | 109 |
| 50 | 2 | -11.9284 | 127.0775 | 107 |
| 51 | 2 | -11.8946 | 127.0914 | 90 |
| 52 | 2 | -11.8975 | 127.0767 | 74 |
| 53 | 2 | -11.8998 | 127.0828 | 75 |
| 54 | 2 | -11.8945 | 127.0834 | 68 |
| 55 | 2 | -11.8957 | 127.0942 | 60 |
| 56 | 2 | -11.897 | 127.1011 | 60 |
| 57 | 2 | -11.8916 | 127.0993 | 78 |
| 58 | 2 | -11.8831 | 127.0999 | 112 |
| 59 | 2 | -11.8812 | 127.1067 | 116 |
| 60 | 2 | -11.8762 | 127.1045 | 116 |
| 61 | 2 | -11.8461 | 127.0823 | 127 |
| 62 | 2 | -11.8651 | 127.0785 | 116 |
| 63 | 2 | -11.8585 | 127.073 | 120 |
| 64 | 2 | -11.8524 | 127.0637 | 126 |
| 65 | 2 | -11.8684 | 127.0676 | 120 |
| 66 | 2 | -11.8651 | 127.0573 | 121 |
| 67 | 2 | -11.8618 | 127.0458 | 123 |
| 68 | 2 | -11.8737 | 127.0464 | 120 |
| 69 | 2 | -11.8704 | 127.0376 | 122 |
| 70 | 2 | -11.8676 | 127.0242 | 126 |
| 71 | 3 | -11.4463 | 126.9109 | 103 |
| 72 | 3 | -11.448 | 126.9204 | 108 |
| 73 | 3 | -11.4467 | 126.9281 | 108 |
| 74 | 3 | -11.4873 | 126.9411 | 109 |
| 75 | 3 | -11.4391 | 126.9446 | 79 |
| 76 | 3 | -11.4315 | 126.9423 | 63 |
| 77 | 3 | -11.4195 | 126.938 | 65 |
| 78 | 3 | -11.4172 | 126.9305 | 85 |
| 79 | 3 | -11.4235 | 126.927 | 90 |
| 80 | 3 | -11.4344 | 126.9268 | 96 |
| 81 | 3 | -11.4493 | 126.9663 | 94 |
| 82 | 3 | -11.4454 | 126.9501 | 92 |
| 83 | 3 | -11.4373 | 126.9505 | 62 |
| 84 | 3 | -11.4318 | 126.9521 | 61 |
| 85 | 3 | -11.4259 | 126.959 | 63 |
| 86 | 3 | -11.4329 | 126.9605 | 59 |
| 87 | 3 | -11.4369 | 126.9707 | 84 |
| 88 | 3 | -11.4383 | 126.9878 | 90 |
| 89 | 3 | -11.4296 | 126.9952 | 100 |
| 90 | 3 | -11.4236 | 126.9857 | 99 |
| 91 | 3 | -11.4138 | 126.9964 | 106 |
| 92 | 3 | -11.4118 | 126.9862 | 105 |
| 93 | 3 | -11.4177 | 126.9801 | 100 |
| 94 | 3 | -11.4259 | 126.9719 | 91 |
| 95 | 3 | -11.4164 | 126.9627 | 90 |
| 96 | 3 | -11.4094 | 126.9653 | 100 |
| 97 | 3 | -11.402 | 126.9716 | 104 |
| 98 | 3 | -11.3959 | 126.9738 | 106 |
| 99 | 3 | -11.3972 | 216.9892 | 105 |
| 100 | 3 | -11.4024 | 126.9958 | 108 |
| 101 | 3 | -11.4168 | 126.9081 | 96 |
| 102 | 3 | -11.4035 | 126.9079 | 91 |
| 103 | 3 | -11.3935 | 126.9087 | 99 |
| 104 | 3 | -11.3929 | 126.9189 | 99 |
| 105 | 3 | -11.3941 | 126.9322 | 89 |
| 106 | 3 | -11.4022 | 126.9481 | 99 |
| 107 | 3 | -11.4127 | 126.9526 | 85 |
| 108 | 3 | -11.4248 | 126.9344 | 60 |
| 109 | 3 | -11.4121 | 126.9235 | 86 |
| 110 | 3 | -11.408 | 126.9259 | 91 |
| 111 | 2 | -11.8656 | 127.1088 | 120 |
| 112 | 2 | -11.8653 | 127.095 | 119 |
| 113 | 2 | -11.8757 | 127.09 | 111 |
| 114 | 2 | -11.8745 | 127.0836 | 114 |
| 115 | 2 | -11.8815 | 127.0832 | 104 |
| 116 | 2 | -1.2E+07 | 127.0789 | 79 |
| 117 | 2 | -11.8871 | 127.0686 | 78 |
| 118 | 2 | -11.8821 | 127.0638 | 100 |
| 119 | 2 | -11.8847 | 127.0573 | 111 |
| 120 | 2 | -11.8853 | 127.0501 | 114 |

Geomorphic Mapping Method

Interpreted local-scale geomorphic maps were produced for each survey area using multibeam bathymetry grids at 2 m resolution and bathymetric derivatives (e.g. slope; curvature; 1-m contours). Maps were produced in ArcGIS at a scale of 1:1 000 using a combination of both automatic extraction and hand-digitisation of polygons. Six geomorphic features, summarised in Table N.1, were identified and mapped following Heap & Harris (2008).

Table N.1 Summary of the geomorphic features mapped with definitions and datasets used to produce them. Feature definitions were based on those used by Heap & Harris (2008).

| Feature | Definition | Datasets Used |
| --- | --- | --- |
| Bank | Local or regional areas of elevated seafloor with one or more steep sides | 2 m x 2 m Bathymetric Grids; Slope; 1 m Contour |
| Depression | Generally enclosed bathymetric lows on the seafloor that would act as a sediment trap. | Slope; 1 m Contour |
| Mound | Local areas of elevated seafloor with gently sloping rounded sides. | Slope; 1 m Contour |
| Plain | Extensive, flat or gently sloping areas | 2 m x 2 m Bathymetric Grid; Slope; 1 m Contour |
| Scarp | Elongated and comparatively steep (≥10o) slope separating more gently sloping areas | Slope |
| Terrace | Relatively flat or gently sloping seafloor with a moderately sloping to steep rise on one side and a moderately sloping to steep drop on the other side. | 2 m x 2 m Bathymetric Grids; Slope; 1 m Contour |

Banks were delimited using slope and curvature rasters overlain by 1-m contours, with edges defined as the point where their steep flanks met a relatively flat plain or terrace, as observed from changes in contour spacing. Since many bank features had multiple terraces the deepest break in slope was selected as the outer boundary that encompassed the entire bank. Some banks do not have distinct transitions or sharp changes in slope and in these cases the boundaries were determined by tracing along a contour that best and most smoothly connected two areas where there was a distinct transition in slope, while avoiding large and abrupt changes in elevation.

Depressions were generally defined from closed sets of contours that represented at least 2 m bathymetric change at a 1:8 000 scale. Some depressions, such as those in Area 1, were very large (10s of meters deep) and extended out of the surveyed area. These larger, deeper depression boundaries were mapped along the top of the greatest change in slope, in a similar but inverse approach to the banks.

Mounds were mapped in a similar way to banks but were distinguished from banks on the basis of having a more rounded form and a smaller change in elevation, generally 2-5 m.

Plains were defined as the generally flat regions between all other features, with relatively large spacing between 1-m contours.

Scarps were extracted by an automated process employing the ArcGIS spatial analyst and conversion toolboxes, and slope raster for each area. Firstly, raster calculator was used to select all areas with a slope ≥10o. These areas were then converted to polygons using the raster to polygon tool. Polygons were subsequently filtered, retaining only those that were ≥10 000 m2, using the select tool. The holes inside scarp polygons were removed by creating a feature dataset typology enforcing a must not have gaps condition. Finally, each remaining scarp was selected and used as a mask for the multibeam bathymetry using the extract by mask tool to check that the change in depth over each polygon is ≥ 10 ± 1 m. A polygon was only labelled as a scarp if all these criteria were met.

Terraces were defined as flat regions, in much the same way as plains. However, they are also characterised by a clear break in slope along the edge. Where plains and terraces were adjacent to one another the boundary of the terrace was defined to include the flank between the plain and terrace, determined as the point where slope changed from relatively steep to relatively flat, using 1 m contour spacing.

Polygons representing geomorphic features were then imported into Fledermaus 3D-visualisation software to check for consistency before being cross-checked by another experienced observer for quality control.

Survey Leader Daily Log

Oceanic Shoals Marine Biodiversity Survey SOL5650

Note: all times are in Western Australian Time

Wednesday, 12 September 2012

In Port - Broome (Roebuck Bay)

GA science crew arrived at wharf at 13:30 to assist with loading and setup of labs and gear stowage.

SN met with AIMS Cruise Leader (Marcus Stowar) and Skipper (Chris Davis) to discuss voyage plan and the plan for the following 48 hrs. Aim for next 48 hours is to transit to Area 1 (southern-most grid)

Departed Broome at 19:00 hrs.

Welcome on board meeting with First Mate (Wayne) followed by safety briefing and tour of vessel for all science staff and support crew.

Completed test of multibeam sonar motion reference unit outside Roebuck Bay (20:30-22:00). Sonar logging commenced 23:30.

Calibration patch test of multibeam sonar heads not completed due to shallow water on transit; aim to complete patch test later on in the transit in deeper water.

Smooth seas for start of transit.

Thursday, 13 September 2012

Transit continued overnight in smooth seas.

Deployed continuous plankton recorder (CPR) at 08:00 (-16o13’03.1, 122o 30’50.6)

Emergency drill to muster station held 08:30; all went smoothly & to plan.

Science crew spent the morning sorting & testing sampling gear.

Collected surface water samples at two stations to coincide with overpass of MODIS satellite (01WS01 at 10:13; 02WS02 at 13:15). Samples filtered for lab analysis of Chlorophyll-a and suspended solids (TSM).

Reviewed operation of CTD and underway thermosalinograph (TSG) with Marcus. CTD log includes depth, temperature, salinity, fluorescence & transmissivity (all recorded as HEX files that require conversion to TXT). The underway TSG data includes temperature (C), salinity (PSU), fluorescence (mg/m^3) & turbidity (NTU). Data is logged automatically to AIMS Townsville where it is QC’d and loaded to the AIMS website for public download.

Satellite link for internet lost during the early evening. Marcus working with AIMS IT support in Townsville to rectify.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:00 to review and plan for the following day. No major issues.

Weather fine and warm. Sea conditions excellent with swell <0.5m and light south-easterly breeze.

Table O.1 Sampling operations completed on Thursday, 13 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 001 | -15.8416 | 122.7185 | 0 | Underway water sample (WS001) |
| 002 | -15.4083 | 123.0195 | 0 | Underway water sample (WS002) |

Friday, 14 September 2012

Transit continued overnight in smooth seas.

Satellite internet link still down. AIMS IT still working on the problem.

Retrieved CPR at 09:00 to allow for calibration patch test of the multibeam system. Upon recovery we found the CPR deployment had failed due to jamming of the silk roll on the cassette. Johnathan Kool worked to successfully remove the damaged cassette unit. We plan to redeploy using a second cassette on the return transit.

Sound velocity profile (SVP) at 09:00 (13o16’20.5, 125o 55’49.6)

Patch test completed between 11:15 and 11:45. No issues.

Collected surface water samples at two stations to coincide with overpass of MODIS satellite (03WS03 at 10:05; 04WS04 at 13:05). Samples filtered for lab analysis of Chlorophyll-a and suspended solids (TSM).

Arrived Area 1 at 20:15. Deployed tide gauge adjacent to the southwest corner of Area 1 in 80 m water depth (-12o 13.825, 127o 22.01).

Sound velocity profile at 20:40 (-12o 13.825, 127o 22.01). SVP only logged to 45m (battery problem). Will fix tomorrow and re-do the SVP.

Final check on tow video setup ahead of sampling tomorrow; all ok including a functioning and accurate USBL positioning system (AIMS USBL).

Commenced swath mapping Area 1 at 21:00; running east-west lines from the southern edge.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:30 to review the activities from today and plan for the following day. No major issues.

Weather fine and warm. Sea conditions excellent with swell <0.5m and light easterly breeze.

Table O.2 Sampling operations completed on Friday, 14 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 003 | -13.1835 | 126.0038 | 0 | Underway water sample (WS003) |
| 004 | -12.9572 | 126.2920 | 0 | Underway water sample (WS004) |

Saturday, 15 September 2012

Continued swath mapping Area 1 overnight, running east-west lines and continued uninterrupted until 07:00 Saturday. Data quality is excellent with minimal noise to remove during on-board processing. Based on the current acquisition rate we expect to complete Area 1 mapping over four nights, as originally planned. The processed bathymetry images reveal the banks in the southern part of Area 1 to be mantled in soft sediment and characterised by extensive fields of pockmarks with hardground areas very limited in extent.

Sampling commenced at 07:30 Saturday, with five stations completed by 18:00. Sampling activities included tow-video transects (1.5 km length), sediment grabs and CTD profiles at each station, with one benthic sled tow. Tow videos across the banks in 70-80 m water depth were characterised by very sparse epibenthos on soft muddy substrate. Visibility is very limited due to high suspended sediment content in strong tidal currents. The single sled tow yielded a small amount of biology mixed with mud. A second attempted sled tow failed with the loss of the tow net during recovery. We have a spare to continue with. All other sampling gear is functioning well, including the USBL on the tow video system. Minor issue with the Smith-Mac grab sampler not triggering on sites with very soft mud.

The satellite internet connection was re-established today.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:15 to review the activities from today and plan for the following day. No major issues.

Weather fine and warm. Sea conditions excellent with swell <0.5m and light easterly breeze.

Table O.3 Sampling operations completed on Saturday, 15 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 005 | -12.2178 | 127.3947 | 85 | Camera tow (CAM001) Grab – Smith McIntyre (GR001) CTD (CTD001) |
| 006 | -12.2093 | 127.4224 | 71 | Camera tow (CAM002) Grab – Smith McIntyre (GR002) Grab – Smith McIntyre (GR003) Benthic sled (BS001) CTD (CTD002) |
| 007 | -12.1902 | 127.4005 | 76 | Camera tow (CAM003) Grab – Smith McIntyre (GR004) Grab – Smith McIntyre (GR005) CTD (CTD003) |
| 008 | -12.1927 | 127.4046 | 73 | Camera tow (CAM004) Grab – Smith McIntyre (GR006) Grab – Smith McIntyre (GR007) CTD (CTD004) |
| 009 | -12.2004 | 127.4305 | 74 | Camera tow (CAM005) Grab – Smith McIntyre (GR008) Grab – Smith McIntyre (GR009) |

Sunday, 16 September 2012

Multibeam swath mapping in Area 1 continued overnight uninterrupted until 07:30 Sunday. Data quality remains very good with a slight increase in noise across the deeper and muddier areas. We remain on track for completing the mapping of Area 1 on Tuesday night. Processed bathymetry continues to show extensive areas of soft sediment with numerous pockmarks across banks, plains and channels.

Sampling commenced from 07:30 with the deployment of underwater video units (SISSTAS) at 10 stations for the observation of pelagic fish communities. Deployment and retrieval of the video units took 4 hrs, with the remaining time used for sampling at four stations. We also deployed a test BRUV in the middle of the day to check for light quality at 70 m – result not promising as the video was too dark for fish observations.

We may need to consider revising the number of SISSTAS stations if other operations are at risk of not being completed. However, at this stage we are confident of achieving 40 SISSTAS in total as the number of BRUV stations will be reduced due to the low visibility at the bed.

Tow video (1.5 km length), grabs (including one triplicate sample for analysis of fine scale infaunal variation) and CTD were completed at each station. Tow videos incorporated bank and channel environments in 70-100 m water depth with all areas characterised by very sparse epibenthos on soft muddy substrate. Visibility for video remains very limited due to high suspended sediment content in strong tidal currents; one tow video was aborted due to zero visibility. The benthic sled was not deployed today due to very low abundance of epibenthic biological material. Repeat issue with the Smith-Mac grab not triggering in deeper sites where mud is very soft (soupy). Station 12 failed to yield a grab due to this problem.

A note on locations of tow video lines, grabs, CTD and sleds. For all stations we have taken the planned waypoint as the start point for the tow video with the direction of the tow determined by tidal currents. Grabs have been taken at the waypoint in some cases, and in other cases at points along the towvid line that represent key depositional environments (e.g. across a bathymetric gradient or mid channel for CTD). So in some cases we don’t have a grab or CTD at the exact waypoint. Sleds are located according to presence of abundant biota (e.g. along narrow ledges at the margins of banks; so again, not on the waypoint. Key point here is that our swath mapping and tow-vid observations provide us with valuable information for more informed sampling; so taking the GRTS waypoint as a start point for observations rather than the focal point for all observations.

The first ocean drifter (ID 101918) was deployed at 17:50 within the southern sector of Area 1.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:15 to review the activities from today and plan for the following day. No major issues.

Weather fine and warm. Sea conditions excellent with negligible swell & no wind.

Table O.4 Sampling operations completed on Sunday, 16 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 010 | -12.2084 | 127.4468 | 88 | Camera tow (CAM006) Grab – Smith McIntyre (GR010) Grab – Smith McIntyre (GR011) Grab – Smith McIntyre (GR012) CTD (CTD005) |
| 011 | -12.2053 | 127.4389 | 91 | Camera tow (CAM007) Grab – Smith McIntyre (GR013) Grab – Smith McIntyre (GR014) CTD (CTD006) |
| 012 | -12.1953 | 127.4445 | 88 | Camera tow (CAM008) CTD (CTD007) |
| 013 | -12.2066 | 127.4356 | 75 | Camera tow (CAM009) |

Monday, 17 September 2012

Multibeam swath mapping in Area 1 continued overnight until 07:30 Monday. Mapping of Area 1 is now more than 75% complete, with the processed bathymetry showing areas of banks not evident on the chart. Data quality remains very good and we are still on track for completing the mapping of Area 1 on Tuesday night. This will allow time for deployment of the sub-bottom profiler prior to completing our work in this Area on Wednesday.

Sampling resumed from 07:30 with the deployment of underwater video units (SISSTAS) at a further 10 stations across the central sector of the grid. This was followed by sampling at four stations in the northern sector. Three stations were located on a bank that rises to 30 m, with tow video (1.5 km length), grabs and CTD completed at each station. Tow videos on the bank revealed highly diverse epibenthic communities of hard corals and sponge gardens, with grabs recovering very coarse carbonate gravel and sand. Two epibenthic sleds on the bank yielded excellent biological samples for biodiscovery research. This bank feature will be the focus of the baited underwater video (BRUV) deployment planned for tomorrow. A total of 16 BRUV stations are planned.

The box corer was successfully deployed at Station 17 in 110 m water depth. Initial grab sample attempts at this site failed due to soft muds preventing the Smith-Mac from triggering.

A second ocean drifter (ID 101884) was deployed at 17:45 within the central sector of Area 1.

Recovery of SISSTAS units went smoothly, but with the loss of one camera unit that broke off its mooring tether. UWA have spares.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:15 to review the activities from today and plan for the following day. Some concern with the amount of time required to deploy and recover the SISSTAS units; due mostly to transit distances between sites. Key point being that it consumes valuable daylight hours that could be used for other sampling activities.

Weather fine and warm. Sea conditions excellent with swell <0.5 m and no wind.

Table O.5 Sampling operations completed on Monday, 17 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 014 | -12.0742 | 127.4345 | 30 | Camera tow (CAM010) Grab – Smith McIntyre (GR015) Grab – Smith McIntyre (GR016) Grab – Smith McIntyre (GR017) Grab – Smith McIntyre (GR018) CTD (CTD008) Benthic sled (BS002) |
| 015 | -12.0713 | 127.4239 | 59 | Camera tow (CAM011) Grab – Smith McIntyre (GR019) Grab – Smith McIntyre (GR020) CTD (CTD009) Benthic sled (BS003) |
| 016 | -12.0894 | 127.4258 | 125 | Camera tow (CAM012) Camera tow (CAM013) Grab – Smith McIntyre (GR021) Grab – Smith McIntyre (GR022) CTD (CTD010) Benthic sled (BS004) |
| 017 | -12.1048 | 127.3996 | 110 | CTD (CTD011) Box core (BC001) Box core (BC002) |

Tuesday, 19 September 2012

Multibeam swath mapping in Area 1 continued overnight until 07:30 Tuesday. Mapping of Area 1 is now more than 95% complete and will be completed tonight. Deployment of the sub-bottom profiler is scheduled to follow the completion of the swath mapping, with a single north-south line planned to intersect a series of three banks and channels.

Sampling resumed from 07:30 with the deployment of underwater video units (SISSTAS) at a further 10 stations across the northern sector of the grid. This was followed by deployment of BRUV video units at 16 stations located across the northern shoal of Area 1. Water depths of BRUV deployment ranged from 30 to 60 m with visibility satisfactory for the identification of pelagic fish communities. Initial checks on BRUV videos showed some fish present on the shoal, with fewer fish observed at the demersal video (SISSTAS) stations.

The deployment and recovery of the SISSTAS and BRUV units occupied most of the day, with opportunistic sediment grabs and CTD casts collected between video deployments. The grabs were taken from the shoal in water depths less than 60 m to provide sediment for the benthic photosynthesis work being led by Lynda. Initial results from on-board processing of these samples are encouraging.

Completed a camera tow at station 17 (left over from yesterday). This tow in 110 m water depth was characterised by soft muddy substrate with very sparse epibenthic communities with only an occasional soft coral and small sponge noted.

We also collected a surface water sample on the shoal to coincide with the MODIS satellite overpass.

A third ocean drifter (ID 101765) was deployed at 18:00 within the northern sector of Area 1.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:15 to review the activities from today and plan for the following day. Continued concern with the amount of time required to deploy and recover the SISSTAS units. Discussed with Tom Letessier the potential for modifying the sampling design for SISSTAS for Area 2.

All on board are well (despite a few minor colds) and are in fine spirits.

Weather fine and warm. Clear skies. Swell <0.5 m. Light easterly breeze.

Table O.6 Sampling operations completed on Tuesday, 18 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 017 | -12.1048 | 127.3996 | 110 | Camera tow (CAM014) |
| 018 | -12.0749 | 127.4265 | 32 | Grab – Smith McIntyre (GR023) Grab – Smith McIntyre (GR024) CTD (CTD012) |
| 019 | -12.0806 | 127.4296 | 49 | Grab – Smith McIntyre (GR025) Grab – Smith McIntyre (GR026) CTD (CTD013) |

Wednesday, 19 September 2012

Multibeam swath mapping in Area 1 continued until 03:30 at which time all lines were completed. Minor infilling to complete across the top of the northern shoal.

Sub-bottom profiler deployed at 04:00 in very calm seas (sea state 0). A single north to south profile 18.5 km along the length of Area 1 was completed by 07:15. Data very good quality with minimal noise. Excellent imaging to >100 m depth across the channels, showing multiple seismic facies including clinoforms and truncations. In contrast, the banks are characterised by a structureless (massive) seismic facies.

Table O.7 Sub-bottom profile lines completed on Wednesday, 19 September 2012.

| Line | Start of Line Latitude | Start of Line Longitude | End of Line Latitude | End of Line Longitude | Distance (km) |
| --- | --- | --- | --- | --- | --- |
| 1 | -12.0573 | 127.4319 | -12.2242 | 127.4317 | 18.5 |

Sampling resumed at 08:00 with the deployment of SISSTAS video units at 10 stations across the southern sector of Area 1. Deployment completed by 09:00.

Sighting of school of dolphins and a sun fish following SISSTAS deployment. Details captured by Phil Bouchet (UWA).

Sampling at three stations on the southern shoal of Area 1 commenced 09:30 and completed by 11:20. Operations included CTD and box cores (for sedimentology, infauna and geochemistry) at three Priority 1 waypoints that were not sampled directly on 15/9/12. These now provide a full complement of samples at Priority 1 waypoints.

Recovered SISSTAS between 11:30 and 13:10.

Collected underway water sample (WS006) at 12:58 to coincide with MODIS satellite overpass. Also competed a CTD cast (STN023CTD017) at the same location.

Transited to southwest of Area 1 to collect tide gauge at 13:50.

Transited to northern end of Area 1 to fill gaps in multibeam coverage across the shallow part of the northern shoal. Completed by 15:50.

Commenced transit to Area 2 at 16:00. Deployed tide gauge at 17:40 (-11.9239, 127.1441; 110 m water depth).

Arrived Area 2 at 18:00 and commenced swath mapping, running northwest-southeast lines.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:00 to review the activities from today and plan for the following day. No issues. Marcus in email contact with AIMS and UWA regarding SISSTAS design.

All on board are well (despite a few minor colds) and are in good spirits.

Weather fine and warm. Clear skies. No swell No wind.

Table O.8 Sampling operations completed on Wednesday, 19 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 020 | -12.1977 | 127.3940 | 73 | CTD (CTD014) Box core (BC003) Box core (BC004) |
| 021 | -12.2058 | 127.4364 | 77 | CTD (CTD015) Box core (BC005) Box core (BC006) |
| 022 | -12.2089 | 127.4457 | 81 | CTD (CTD016) Box core (BC007) Box core (BC008) |
| 023 | -12.2172 | 127.3968 | 86 | CTD (CTD017) WS (WS006) |

Thursday, 20 September 2012

Multibeam swath mapping of Area 2 continued overnight without interruption. Data quality remains excellent, with coverage to this point including a large shoal that rises to 50 m in the southern sector of the grid and adjacent channels up to 130 m deep. Processed backscatter data is also of high quality, allowing us to identify the broad sediment types on the banks and channels and select sampling techniques accordingly.

Sampling activities occupied the full day with the completion of five stations on and adjacent to the main shoal. Sampling operations included CTD casts, grabs (at shallow sandy sites), box cores (at deeper muddy sites) and tow video at all sites, plus benthic sled tows at two sites where epibenthic communities were observed in videos. Benthic communities range from sponge gardens across the shoal to barren mud substrate in the channels, with clear community transitions related to depth and exposure. The first sled haul collected >500 kg of mud and shell, requiring us to tow the net in the water for 45 min prior to bringing on board. The second sled across the top of the shoal recovered an excellent sample of sponges & gorgonians.

Tow video quality remains limited due to high turbidity; however the quality of still photos is generally good. Turbidity will also limit the deployment of BRUV video units to the shallowest parts of the banks.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:00 to review the activities from today and plan for the following day. No issues. Marcus received revised sampling design for SISSTAS; now to be concentrated into a smaller part of the Grid to reduce transit times during deployment/recovery.

All on board are well (those with colds are on the mend) and are in good spirits.

Weather fine and warm. Clear skies. Swell <0.5m. No wind.

Table O.9 Sampling operations completed on Thursday, 20 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 024 | -11.8857 | 127.0932 | 79 | Camera tow (CAM015) Benthic sled (BS05) CTD (CTD018) Box core (BC009) Box core (BC010) Box core (BC011) |
| 025 | -11.8960 | 127.0699 | 73 | Camera tow (CAM016) Benthic sled (BS06) Grab– Smith McIntyre (GR027) CTD (CTD019) Box core (BC012) |
| 026 | -11.8920 | 127.0945 | 62 | Camera tow (CAM017) Benthic sled (BS07) Grab– Smith McIntyre (GR028) Grab– Smith McIntyre (GR029) CTD (CTD020) Box core (BC013) Water sample (WS007) |
| 027 | -11.8785 | 127.1154 | 116 | Camera tow (CAM018) CTD (CTD021) Box core (BC014) Box core (BC015) |
| 028 | -11.8585 | 127.0361 | 126 | Camera tow (CAM019) CTD (CTD022) Box core (BC016) Box core (BC017) |

Friday, 21 September 2012

Multibeam swath mapping of Area 2 continued overnight without interruption. Data quality remains excellent, with coverage now showing a series of small banks not marked on the charts. Our current thinking is that these features, which rise to 75 m water depth, may be drowned coral patch reefs similar to those mapped in the Gulf of Carpentaria. There is also an indication in the multibeam data that there may be buried reefs in the deeper channels; these will be targeted on our sub-bottom profile lines planned for Sunday night. Pockmarks are again a pervasive feature across the muddy channels. Tow video across the pockmark fields shows a highly irregular seabed with holes very well defined and up to 1 metre deep.

Sampling activities today included deployment and recovery of SISSTAS video units at 10 stations in the southern sector of Area 2, interspersed by sampling at three stations. Operations included CTD casts, grabs (at shallow sandy sites), box cores (at deeper muddy sites) and tow video. Two benthic sled tows were also completed, both retrieving good quantities of biological materials (sponges, gorgonians, soft corals etc). Initial processing of these materials points to very high biodiversity, with one sled yielding 20 species of gorgonians alone. One sled also collected a cobble-sized piece of fossil coral that likely forms the for epibenthic communities observed in tow videos and also in multibeam as small bommies. Our objective is to continue to target these sites for further sampling.

Deployed ocean drifter number four (ID 101914) at 07:30.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:15 to review the activities from today and plan for the following day. No issues. SISSTAS deployment noted as taking less time today, so all ok to proceed with 40 stations planned for each grid. Presence of seismic survey vessel MV Geo Atlantic to the northwest noted, with skipper to contact that vessel tomorrow to discuss intentions in the coming days.

All on board are well (colds continue to subside) and in good spirits.

Weather fine and warm. Clear skies. Swell <0.5m. No wind.

Table O.10 Sampling operations completed on Friday, 21 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 029 | -11.8932 | 127.0951 | 58 | Camera tow (CAM020, 20_1) Benthic sled (BS08) CTD (CTD023) Grab - Smith McIntyre (GR030) Grab - Smith McIntyre (GR031) |
| 030 | -11.9266 | 127.0774 | 107 | CTD (CTD024) Water Sample (WS008) |
| 031 | -11.7910 | 126.9772 | 76 | Camera tow (CAM021) Benthic sled (BS09) CTD (CTD025) Box core (BC018) Box core (BC019) |
| 032 | -11.7940 | 126.9938 | 95 | Camera tow (CAM022) CTD (CTD026) Box core (BC020) Box core (BC021) |

Saturday, 22 September 2012

Multibeam swath mapping of Area 2 continued overnight without interruption. Mapping over the last 24 hrs occurred along the western side of the grid where the seabed is mostly >110 m deep and generally flat and pockmarked. We are well on track to complete Area 2 during Sunday night, allowing 8-10 hrs for sub-bottom profiles.

Sampling activities today included deployment and recovery of SISSTAS video units at a further 10 stations in the southern sector of Area 2, with one unit not recovered (failed clip attachment). A pod of false killer whales was observed following the ship during the retrieval of the SISSTAS units. Many photos taken.

Sampling was completed at four stations today. Operations included CTD casts, grabs (at shallow sandy sites), box cores (at deeper muddy sites) and tow video. A CTD cast was also completed at 12:55 to coincide with the MODIS satellite overpass.

Deployed ocean drifter number five (ID 101917) at 07:30.

A set of triplicate box cores was collected for analysis of fine scale patterns of infaunal biodiversity (four sites with triplicate samples now completed).

Two benthic sled tows were also completed, both retrieving good quantities of diverse biological materials (sponges, gorgonians, soft corals) along with block of fossil coral and carbonate conglomerate. Our observations from tow video suggest that these communities occur in greatest abundance on the edges of banks on small mounds (bommies). There also appears to be an aspect effect on epibenthic communities, with higher concentrations of sponge and gorgonian communities observed on the north to northwest side of banks. This is the lee side with respect to net tidal flow, as suggested by sediment drifts around the banks. More work needed to substantiate this hypothesis however.

Our port call in Darwin for refuelling has been confirmed for early next week. Return transit time to Darwin, including refuelling, is estimated at 45 hrs. We are scheduled to depart Area 2 early afternoon on Monday (24/9) and return to start work in Area 3 by Wednesday (26/9) afternoon. Based on our current work rates, this will leave us sufficient time to complete Area 3 and possibly leave time for additional sampling in key areas of Area 1.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:30 to review the activities from today and plan for the following day. No issues. No further information on the seismic vessel MV Geo Atlantic.

All on board are well (colds continue to abate) and in good spirits.

Weather fine and warm. Scattered light cloud. Swell 0.5 m. Wind 5-10 knots from the southwest. Forecast for 10-15 knots maybe.

Table O.11 Sampling operations completed on Saturday, 22 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 033 | -11.8458 | 127.0559 | 127 | Camera tow (CAM023) CTD (CTD027) Box core (BC022) Box core (BC023) |
| 034 | -11.8798 | 127.0920 | 108 | Camera tow (CAM024) CTD (CTD028) Box core (BC024) Box core (BC025) Box core (BC026) |
| 035 | -11.9033 | 127.1292 | 106 | CTD (CTD029) |
| 036 | -11.8178 | 127.0468 | 110 | Camera tow (CAM025) CTD (CTD030) Box core (BC027) Box core (BC028) Benthic sled (BS010) |
| 037 | -11.8183 | 127.0294 | 110 | Camera tow (CAM026) CTD (CTD031) Box core (BC029) Grab – Smith McIntyre (GR032) Benthic sled (BS011) |

Sunday, 23 Sunday 2012

Multibeam swath mapping of Area 2 continued overnight without interruption. Mapping over the last 24 hrs occurred along the eastern side of the grid where the seabed is mostly >110 m deep and generally flat and pockmarked. We have one line remaining to complete tonight plus some hole-filling and additional mapping around the edge of the main southern shoal to improve resolution of the bathymetry grid on the steep sides of the shoal.

Processing of acoustic backscatter is complete for Area 1 and Area 2. Gridded to a horizontal resolution of 2 m both grids are excellent products for showing subtle textural variations across the seabed. Key patterns include a strong backscatter contrast between the sandy crests of banks (high backscatter intensity) and surrounding muddy channels and plains (low backscatter intensity). Pockmarks are also clearly differentiated by backscatter, with the floor of the pockmark returning a higher backscatter signal than the surrounding seabed. This is consistent with video observations which show sandier sediment within pockmarks; possibly a product of erosion/winnowing of fines associated with pockmark development.

Sampling activities today included deployment and recovery of SISSTAS video units at a further 10 stations in the central sector of Area 2 in the deeper (off-feature) part of the grid.

Deployed ocean drifter number six (ID 101753) at 07:25.

Sampling was completed at three stations today. Operations included CTD casts, box cores, tow video and a benthic sled at one station. The sled tow intersected a narrow ridge (hardground) recovered mostly sandy mud with only a small number of epibenthic organisms. The sled also brought up cobble-sized clasts of cemented carbonate sediment that is weakly bedded (stratified) – representative sample retained.

The camera tow at STN039 was terminated at 500 m due to lack of epibenthic organisms across a uniformly flat muddy seabed.

One of the CTD station casts (STN039) was taken 10 min before the MODIS satellite overpass.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:30 to review the activities from today and plan for the following day. No issues. No further information on the seismic vessel MV Geo Atlantic.

All on board are well and in good spirits.

Weather fine and warm. Scattered light cloud. Swell 0.5-1.0 m. Wind 5-10 knots from the southwest.

Table O.12 Sampling operations completed on Sunday, 23 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 038 | -11.8075 | 126.9673 | 77 | Camera tow (CAM027) CTD (CTD032) Box core (BC030) Box core (BC031) Benthic sled (BS012) |
| 039 | -11.7801 | 127.0092 | 90 | Camera tow (CAM028) CTD (CTD033) Box core (BC032) Box core (BC033) |
| 040 | -11.8083 | 126.9905 | 99 | Camera tow (CAM029) CTD (CTD034) Box core (BC034) Box core (BC035) |

Monday, 24 September 2012

Multibeam swath mapping of Area 2 was completed by 22:00 hours Sunday night. The sub-bottom profiler was deployed between 22:30 Sunday and 07:30 Monday. Four lines were completed, covering 65 km. Survey conditions very good, with wind 5-10 knots and swell less than 0.5 m. Data quality generally very good to excellent, with penetration to >150 m across the soft sediment areas between banks. No penetration across the carbonate banks, however.

Table O.13 Sub-bottom profile lines completed on Monday, 24 September 2012.

| Line | Start of Line Latitude | Start of Line Longitude | End of Line Latitude | End of Line Longitude | Distance (km) |
| --- | --- | --- | --- | --- | --- |
| 1 | -11.7803 | 126.9694 | -11.9155 | 127.1139 | 19.8 |
| 2 | -11.9108 | 127.0853 | -11.8386 | 127.0853 | 23.0 |
| 3 | -11.8050 | 127.0505 | -11.8483 | 127.0001 | 12.5 |
| 4 | -11.8197 | 126.9894 | -11.8003 | 127.0433 | 7.7 |

Sampling activities today included the successful deployment and recovery of BRUV underwater video units at 16 stations across the southern shoal of Area 2. Initial checks on videos show presence of a range of demersal fish in relative abundance.

Sampling at two stations also completed, with grabs and CTD casts. Sampling completed by 13:00 hrs.

Collected tide gauge from southwest corner of Area 2 at 13:30 and deployed drifter number seven (ID 101754) at the same location at 13:05.

Commenced transit to Darwin at 13:45.

Deployed continuous plankton recorder at 14:15 (-11o 56.4548”, 127o 17.1817)

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:30 to review the activities from today and plan for the port call in Darwin. No issues.

Weather fine and warm. Scattered light cloud. Swell 0.5-1.0 m. Wind 5-10 knots from the southwest.

Table O.14 Sampling operations completed on Monday, 24 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 041 | -11.8896 | 127.0669 | 64 | CTD (CTD035) Grab – Smith McIntyre (GR033) Grab – Smith McIntyre (GR034) |
| 042 | -11.8886 | 127.0745 | 72 | CTD (CTD036) Grab – Smith McIntyre (GR035) Grab – Smith McIntyre (GR036) |

Tuesday, 25 September 2012

Transit continued overnight in calm seas.

CPR retrieved 06:45 as water depths becoming too shallow for the tow line. Upon recovery we found the CPR deployment had failed due to jamming of the silk roll on the cassette. This is the second cassette that has failed. Decided not to redeploy and run the risk of damaging the gearing in main housing unit, and wasting time with deployment and recovery. JK contacted CSIRO to advise.

Arrived Darwin (Fishermans Wharf) at 10:20 (WA time). Commenced bunkering and re-provisioning. Ship crew and most of the science crew went ashore to run errands and purchase miscellaneous lab and office gear for the rest of the survey.

Departed Darwin at 14:00 (WA time) and set course for a 20 hr transit to Area 3.

Over the past 12 hours we have been in communication with the seismic survey vessel (MV Geo Atlantic, operated by Fugro) working in a large area of the Timor Sea that includes part of Area 3. The client representative from Fugro has advised that the MV Geo Atlantic should be crossing through our survey area in about 8 days. This will be after the scheduled completion of our work in Area 3. Following consultation with Andrew Heyward the decision has been made to proceed as per the voyage plan but remain in contact with Fugro.

All on board are well and looking forward to getting to work in the final survey grid.

Weather fine and warm. Scattered light cloud. Swell 0.5-1.0 m. Wind 10-15 knots (inshore) from the southwest.

Wednesday, 26 September 2012

Transit to Area 3 continued overnight on calm seas.

Further communication with the MV Geo Atlantic overnight has revised and clarified the impact of the seismic survey on our work in Area 3. The Geo Atlantic is operating a seismic array that is 1 km wide and 6000 m long, with exclusion distances of 2 Nm ahead and to each side, and 5 Nm astern. The survey pattern is following a ‘race-track’ path that crosses directly through the centre of Area 3 approximately every 52 hrs. The first crossing through Area 3 will be at approximately 02:00 Thursday during multibeam operations. We have advised Fugro that we will maintain a position outside the exclusion zone as the Geo Atlantic passes through Area 3; a period of 2-3 hrs. We expect to be able to continue mapping and have a line plan in place to accommodate the exclusion zone. At this stage we do not expect that the movements of the Geo Atlantic will affect our daytime sampling activities.

Collected surface water sample during transit to coincide with overpass of MODIS satellite (043WS009 at 09:53). Samples filtered for lab analysis of Chlorophyll-a and suspended solids (TSM).

Arrived into Area 3 at 13:10 and deployed a tide gauge outside the northwest corner of the Grid.

Sampling completed at three stations (priority 1 waypoints), with tow video at two stations and grabs/box cores and CTD casts at all three stations. Attempted a benthic sled at station 13 but was unsuccessful with the sled net almost lost in mud.

Commenced multibeam mapping at 18:30 following a patch test. Running east-west lines initially, starting from the southern edge of the Grid.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:15 to review the activities from today and plan for the arrival of the Geo Atlantic into Area 3 overnight.

Weather fine and warm. Scattered light cloud. Swell 0.5-1.0 m. Wind 5-10 knots from the southwest.

Table O.15 Sampling operations completed on Wednesday, 26 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 043 | -11.5324 | 127.4463 | 127 | Underway water sample (WS009) |
| 044 | -11.4101 | 126.9227 | 88 | Camera tow (CAM030) CTD (CTD037) Grab - Smith McIntyre (GR037) Box core (BC036) Benthic Sled (BS013) |
| 045 | -11.4504 | 126.9211 | 106 | Camera tow (CAM031) CTD (CTD038) Box core (BC037) Box core (BC038) |
| 046 | -11.4342 | 126.9379 | 61 | CTD (CTD039) Box core (BC039) Box core (BC040) Benthic Sled (BS014) – deployed on following day 27/9 |

Thursday, 27 September 2012

Multibeam swath mapping of Area 3 continued overnight without interruption. The Geo Atlantic crossed through the Grid between 00:00 and 03:00, during which time we mapped shorter east-west lines in the western sector of the Grid. No loss of mapping time.

Deployed drifter number eight (ID 101756) at 07:42 in the southwest part of Area 3.

Sampling activities today included deployment and recovery of SISSTAS video units at 10 stations in the western sector of Area 3 in an area that includes a small shoal feature and surrounding terrace. Deployment and recovery times for the SISSTAS have improved with the new set of clips being used on the moorings.

Sampling was completed at three stations today. Operations included CTD casts, box cores or grabs, tow video at all stations with the benthic sled deployed at two stations (STN46 & 47). The sled tows both recovered small amounts of biological material from sites with thin sediment cover and patchy outcrop of rock. Sled BS014 also brought up pebble-sized clasts of cemented carbonate sediment – representative samples retained.

The CTD cast at STN48 and underway water sample were both collected to coincide with the MODIS satellite overpass at 12:52.

Marcus Stowar received an email from the Geo Atlantic today advising us that they had recovered a drifter float (ID 101917 deployed on 21/9 in Area 2). We replied acknowledging it as ours and asking whether redeployment was possible. Awaiting reply.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:30 to review the activities from today and plan for tomorrow. No issues.

Weather fine and warm. Scattered cloud. Swell 0.5-1.0 m. Wind <5 knots from the southwest.

Table O.16 Sampling operations completed on Thursday, 27 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 047 | -11.4204 | 126.9446 | 64 | Camera tow (CAM033) CTD (CTD040) Grab - Smith McIntyre (GR038) Grab - Smith McIntyre (GR039) Benthic Sled (BS015) |
| 048 | -11.4319 | 126.9427 | 61 | CTD (CTD041) Underway water sample (WS010) |
| 049 | -11.4069 | 126.9291 | 81 | Camera tow (CAM034) CTD (CTD042) Grab - Smith McIntyre (GR040) Grab - Smith McIntyre (GR041) |
| 050 | -11.4051 | 126.9616 | 99 | Camera tow (CAM035) CTD (CTD043) Box core (BC041) Box core (BC042) |

Friday 28 September 2012

Multibeam swath mapping of Area 3 continued overnight without interruption. We are on track for completing the mapping of this Grid on Sunday night, allowing adequate time for sub-bottom profiling. Multibeam data quality continues to be excellent, allowing Justy and Kim to generate bathymetry and backscatter grids to 1 m resolution. At this resolution we are able to identify fine scale features such as bommies on the tops of banks and pockmarks across the deeper muddier areas. As in Areas 1 and 2, pockmarks are pervasive in this area and clearly evident on tow video as well defined holes (often providing shelter for demersal fish).

Sampling activities today included deployment and recovery of SISSTAS video units at 10 stations in the central sector of Area 3, followed by physical sampling at four stations. To date, the SISSTAS video deployments have seen very few pelagic fish in this Grid. Operations at sampling stations included CTD casts, box cores and tow video at all stations with the benthic sled deployed at one station at the edge of the main shoal. The sled tow recovered a large (>500 kg) haul of muddy sediment with only a small amount of biological material. Sled sampling continues to be a challenge in this area due to the patchy distribution of epibenthos (as seen in tow video). Nonetheless, we are recovering adequate amounts to characterize the key groups in these shoal communities (sponges, soft corals, gorgonians). We plan for more sled attempts tomorrow.

The Geo Atlantic has advised of an 8 hour delay in their 52 hour mapping cycle. This means that they will next pass through Area 3 about 2 pm Saturday. We have a sampling plan in place that can accommodate this interruption. The Geo Atlantic also advised that the drifter buoy they recovered was damaged and that they will attempt to return it to us in the next few days (via chaser vessel).

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:20 to review the activities from today and plan for tomorrow. No issues.

Weather fine and warm. Clear skies. Swell <0.5 m. No wind. Sea state 0 for most of the day.

Table O.17 Sampling operations completed on Friday, 28 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 051 | -11.4310 | 126.9888 | 95 | Camera tow (CAM036) CTD (CTD044) Box core (BC043) Box core (BC044) |
| 052 | -11.4360 | 126.9971 | 96 | Camera tow (CAM037) CTD (CTD045) Box core (BC045) Box core (BC046) |
| 053 | -11.4424 | 126.9671 | 78 | Camera tow (CAM038) CTD (CTD046) Box core (BC047) Box core (BC048) |
| 054 | -11.4243 | 126.9659 | 85 | Camera tow (CAM039) CTD (CTD047) Box core (BC049) Box core (BC050) |

Saturday, 29 September 2012

Multibeam swath mapping of Area 3 continued overnight without interruption. We are still on track for completing the mapping of this Grid on Sunday night, to be followed by sub-bottom profiling. Multibeam data quality continues to be excellent.

Sampling was completed at four stations today in water depths ranging from 57 m on the main shoal to 110 m in a channel pockmark field. Operations including CTD casts, box cores and tow video at all stations with the benthic sled deployed at two stations. Both sleds recovered approximately 20 kg of biological material from the edges of carbonate banks. Sampling also included a triplicate box core set of samples for fine-scale infaunal analysis at 110 m water depth.

The Geo Atlantic has advised that it is further delayed in its 52 hour mapping cycle. This means that they will next pass through Area 3 late Saturday night. We do not expect this to interrupt our swath mapping activities.

Weather fine and warm. Clear skies. Swell <0.5 m and easing. No wind. Sea state 0 for most of the day.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:20 to review the activities from today and plan for tomorrow. No issues.

Table O.18 Sampling operations completed on Saturday, 29 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 055 | -11.4341 | 126.9542 | 57 | Camera tow (CAM040 and 40_1) CTD (CTD048) Grab - Smith McIntyre (GR042) Grab - Smith McIntyre (GR043) Benthic Sled (BS017) |
| 056 | -11.3993 | 127.0205 | 76 | Camera tow (CAM041 and 41_1) CTD (CTD049) Box core (BC051) Box core (BC052) Benthic Sled (BS018) |
| 057 | -11.4432 | 126.0951 | 110 | Camera tow (CAM042) CTD (CTD050) Box core (BC053) Box core (BC054) Box core (BC055) |
| 058 | -11.4483 | 126.0306 | 108 | Camera tow (CAM043) CTD (CTD051) Box core (BC056) Box core (BC057) |

Sunday, 30 September 2012

Multibeam swath mapping of Area 3 continued overnight without interruption. Data quality continues to be excellent. We have approximately 10 hours mapping left to complete the Grid tonight, after which we will run a single representative sparker line across the Area.

Sampling activities today included successful deployment and recovery of SISSTAS water column video units at 10 stations, followed by BRUV seabed video units at 8 stations. The BRUV deployment focused on a small uncharted shoal in the northeast sector of the Grid that rises to 50 m water depth and has a rich cover of sponge and soft coral gardens on patchy sandy sediment. We attempted sediment grabs on this shoal, but with limited recovery due to the largely nature of the shoal.

Physical sampling was also completed today at four stations to complete the priority 1 sites and all but one of the priority two sites in Area 3. We now have good sampling coverage across all environments in this grid (banks, bank margins, channels and pockmark fields). Station operations included CTD casts, box cores (or grabs) and tow video at all stations with the benthic sled deployed at one station. The sled recovered a diverse collection of sponges and soft corals, plus further samples of cemented carbonates. We now have a range of materials (fossil carbonates, hard corals) for potential geochemical analysis and dating.

The Geo Atlantic is currently to the north of our survey area and will have no further impact on our activities. We will have departed Area 3 before it returns.

Evening briefing between Chris Davis, Marcus Stowar and Scott Nichol held at 19:40 to review the activities from today and plan for tomorrow. No issues.

Weather fine and warm. Clear skies. Swell <0.5 m. Wind 5 knots from the southwest.

Table O.19 Sampling operations completed on Sunday, 30 September 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 059 | -11.4030 | 127.0278 | 52 | CTD (CTD052) |
| 060 | -11.4032 | 127.0245 | 51 | Grab - Smith McIntyre (GR044) Grab - Smith McIntyre (GR045) |
| 061 | -11.4375 | 126.9683 | 56 | Camera tow (CAM044) CTD (CTD053) Box core (BC058) Box core (BC059) Benthic Sled (BS019) |
| 062 | -11.4091 | 126.9395 | 77 | Camera tow (CAM045) CTD (CTD054) Box core (BC060) Box core (BC061) |
| 063 | -11.3994 | 126.9121 | 99 | Camera tow (CAM046) CTD (CTD055) Box core (BC062) Box core (BC063) |

Monday, 1 October 2012

Multibeam swath mapping of Area 3 was completed overnight, ending at 04:30. We then deployed the sub-bottom profiler for three hours to collect a single sparker line that extended 15 km diagonally across the Grid from the northeast to southwest, crossing two carbonate banks and two channels. Sea conditions were favourable (sea state 1 – 2) and data quality is excellent with good resolution and penetration to >150 m.

Table O.20 Sub-bottom profile lines completed on Monday, 1 October 2012.

| Line | Start of Line Latitude | Start of Line Longitude | End of Line Latitude | End of Line Longitude | Distance (km) |
| --- | --- | --- | --- | --- | --- |
| 1 | -11.3919 | 127.0419 | -11.4208 | 126.9192 | 15 km |

Sampling activities today included successful deployment and recovery of SISSTAS water column video units at 10 stations, followed by BRUV seabed video units at 16 stations. These activities were interspersed by grab sampling and CTD at two stations on the main shoal to add to our coverage on this feature. We also collected an underway water sample at 09:50 to coincide with the MODIS satellite overpass.

Sampling also included a benthic sled at STN063. This station was sampled yesterday (30/9) with rich sponge and gorgonian gardens observed on tow video. The sled was successful, recovering approximately 200 kg of carbonate rock, sponges, crinoids, holothurians and other assorted biology. The carbonate rocks included a 50 kg slab of cemented calcareous materials (worm tubes, bivalves, gastropods, corals – photos taken) and an 8 kg boulder of coral with cemented carbonates above. This sample was retained for analysis.

Sampling ended at 17:30. Resumed swath mapping to fill in some holes in Area 3 then transited north to the “Bonus Shoal”. This shoal is located 3.5 Nm north of Area 3 and is marked on the chart as rising to 44 m. We have decided to take the opportunity to swath map this feature and collect tow video across the shoal platform.

Weather overcast and warm. Rain squall in the afternoon. Swell 0.5-1 m. Wind 10-15 knots from the northeast. Choppy sea.

Table O.21 Sampling operations completed on Monday, 1 October 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 063 | -11.3972 | 126.9082 | 88 | Benthic Sled (BS020) |
| 064 | -11.4363 | 126.9622 | 58 | Underway water sample (WS011) |
| 065 | -11.4256 | 126.9445 | 62 | CTD (CTD056) Grab - Smith McIntyre (GR046) Grab - Smith McIntyre (GR047) |
| 066 | -11.4143 | 126.9355 | 65 | CTD (CTD057) Grab - Smith McIntyre (GR048) Grab - Smith McIntyre (GR049) |

Tuesday, 2 October 2012

Multibeam mapping of “Bonus Shoal” was completed overnight, covering approximately 30 km2. The shoal is correctly charted, rising to 44 m with a lower platform at 76 m and fringing sediment lobes at 85 m. A second smaller shoal 2.5 km to the east is also correctly charted and rises to 77 m. Both banks are surrounded by soft sediment areas that are pockmarked.

Sampling activities on “Bonus Shoal” included three short tow video transects and CTD casts and grabs at two stations (note that only one grab collected at STN67 – second grab failed on irregular ground). Tow video showed the shoal platform is characterised by semi-continuous (cemented carbonates?) with occasional sand patches. Epibenthic cover is also semi-continuous and includes soft and hard corals (acropora noted), gorgonians, sponges (inc. large barrel sponges), crinoids and assorted biology. Large schools of reef fish also noted on video. Decision made not to deploy the benthic sled on this pristine ecosystem.

Sampling ended at 10:30. Transited to collect tide gauge at northwest corner of Area 3.

Transit to Area 2 commenced 11:15. Collected an underway water sample at 12:50 to coincide with the MODIS overpass (WS012).

Table O.22 Sampling operations completed on Tuesday, 2 October 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 067 | -11.3368 | 126.9622 | 77 | CTD (CTD058) Camera tow (CAM047) Grab - Smith McIntyre (GR050) |
| 068 | -11.3283 | 126.9543 | 42 | CTD (CTD059) Grab - Smith McIntyre (GR051) Grab - Smith McIntyre (GR052) |
| 069 | -11.3280 | 126.9540 | 43 | Camera tow (CAM048) |
| 070 | -11.3297 | 126.9545 | 42 | Camera tow (CAM049) |
| 071 | -11.6401 | 127.0092 | 100 | Underway water sample (WS012) |
| 072 | -11.8892 | 127.0750 | 75 | CTD (CTD060) |

Arrived in Area 2 at 14:15 to deploy SISSTAS units at 10 stations. This brings the total number of SISSTAS deployments in Area 2 to 40, as originally planned. Soak times were reduced to 2 hrs to allow recovery before dark.

Completed a CTD cast (STN072) while SISSTAS were soaking; also attempted a Smith-Mac grab at the same station but unsuccessful with the grab being triggered during the descent in choppy seas.

Transited from Area 2 to Area 1 at 17:30 to commence sparker lines and swath mapping along the western edge of the Grid.

Weather clear skies and warm. Swell 1-1.5 m. Wind 10-15 knots from the northeast. Choppy sea.

Wednesday, 3 October 2012

Sub-bottom profiling within Area 1 was completed overnight in good sea conditions (sea state 2). Three lines were run, including: line 2 - a north-south line across the wide channels in the western half of the grid; line 3 - a diagonal line across the banks and narrow channel in the southeast sector, and: line 4 - an east-west tie line across the centre of the grid. A test line was also logged (Test 3), running north along the western edge of Area 1. Data quality is excellent, with good resolution and penetration >150 m showing multiple episodes of (fluvial?) channel cut and fill that can be tied to sea-level cycles.

Table O.23 Sampling operations completed on Monday, 1 October 2012.

| Line | Start of Line Latitude | Start of Line Longitude | End of Line Latitude | End of Line Longitude | Distance (km) |
| --- | --- | --- | --- | --- | --- |
| 2 | -12.0672 | 127.4036 | -12.2278 | 127.4039 | 18.0 |
| 3 | -12.2214 | 127.4075 | -12.1522 | 127.4614 | 10.3 |
| 4 | -12.1522 | 127.4556 | -12.1522 | 127.3833 | 8.5 |
| Test 3 | -12.1506 | 127.3858 | -12.0753 | 127.3858 | 8.3 |

Sampling activities in Area 1 were finalised at three stations. Two were located on the northern shoal which rises to 31 m and the third in a channel in the centre of the grid. Station operations included CTD casts, camera tows, grabs/box cores, and benthic sleds at two stations. The camera tow at station 74 was a highlight, incorporating the bathymetric transition from the shoal platform at 31 m to the fringing muddy terrace at 89 m. The substrate ranged from sand and gravel on the platform with extensive (100s m2) 2D bedforms, to rocky outcrop on the platform edge and muddy sands on the terrace. Epibenthos varied from barren on the bedforms (these are presumably active during cyclones), to dense cover of large sponges and corals (including hard corals) on rocky substrates and patchy to sparse cover of smaller sponges and corals on the terrace. Visibility along the tow was also very good.

The two benthic sleds recovered good amounts of biological material, with the sled at station 74 also recovering a large (200 kg) haul of carbonate rubble. Several samples of fossil coral were retained for potential radiocarbon dating.

Table O.24 Sampling operations completed on Wednesday, 3 October 2012.

| Station | Latitude | Longitude | Depth (m) | Operations |
| --- | --- | --- | --- | --- |
| 073 | -12.0647 | 127.4257 | 72 | CTD (CTD061) Camera tow (CAM050) Grab - Smith McIntyre (GR053) Grab - Smith McIntyre (GR054) Benthic Sled (BS021) |
| 074 | -12.0774 | 127.4448 | 73 | CTD (CTD062) Camera tow (CAM051) Grab - Smith McIntyre (GR055) Box Core (BC064) Benthic Sled (BS022) |
| 075 | -12.1439 | 127.4458 | 110 | CTD (CTD063) Camera tow (CAM052) Box Core (BC065) Box Core (BC066) |

Following the completion of station 75 we deployed the ninth ocean drifter (ID 101755) and commenced the 50 hour return transit to Broome at 15:00.

Communications with the MV Geo Atlantic support vessel during the day to make alternate arrangements for the return of the damaged drifter (ID 101917). Agreed that Fugro would deliver the drifter to Pearl Marine Engineering on its next port call in Darwin. Fugro to advise Marcus Stowar when this occurs.

Weather clear skies and warm. Swell <0.5 m. Wind < 5 knots. Glassy seas in the afternoon.

Thursday, 4 October 2012

Transit continued overnight and throughout today in smooth seas.

Science crew used the day to pack samples, begin backups of digital data and continue some lab work.

Collected surface water samples at two stations to coincide with overpass of MODIS satellite (076WS013 at 09:50; 077WS014 at 12:50). Samples filtered for lab analysis of Chlorophyll-a and suspended solids (TSM).

Final drifter (ID 101752) deployed during transit at 12:15. Details of all drifter deployments are listed below.

Weather clear skies and warm. Swell <0.5 m. Wind < 5 knots.

Table O.25 Drifters deployed during the survey

| ID | UTC Date | JD | UTC Time | Latitude | Longitude | Depth (m) | Area |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 101918 | 16/9/12 | 260 | 09:48 | -12.1813 | 127.4471 | 74 | 1 |
| 101884 | 17/9/12 | 261 | 09:16 | -12.1334 | 127.4333 | 110 | 1 |
| 101765 | 18/9/12 | 262 | 10:09 | -12.0896 | 127.3941 | 110 | 1 |
| 101914 | 20/9/12 | 264 | 23:28 | -11.8729 | 127.1284 | 116 | 2 |
| 101917 | 21/9/12 | 265 | 23:32 | -11.9354 | 127.0864 | 101 | 2 |
| 101753 | 22/9/12 | 266 | 23:25 | -11.8377 | 127.0867 | 127 | 2 |
| 101754 | 24/9/12 | 268 | 05:06 | -11.8984 | 127.1064 | 62 | 2 |
| 101756 | 26/9/12 | 270 | 23:42 | -11.4460 | 126.9035 | 102 | 3 |
| 101755 | 3/10/12 | 277 | 06:38 | -12.1408 | 127.4558 | 108 | 1 |
| 101752 | 4/10/12 | 278 | 04:15 | -14.2790 | 124.8489 | 63 | Transit |

Friday, 5 October 2012

Transit continued overnight and throughout today in smooth to slightly choppy seas.

Packing and data backups completed.

Datasets held on GA portable hard drives in triplicate include:

Multibeam bathymetry – raw, processed and grids

Multibeam backscatter – raw, processed and grids

Port to Port navigation files (daily)

Sub-bottom profile data – raw and SEGY files plus screen shots (TIFFs)

Shipboard samples database

Station waypoint file with all sampling operations (from AIMS)

Tow video - USBL navigation files and CSV characterisation files (from AIMS)

ArcGIS files – shapefiles with all station operations

Geochemical sample database

Benthic ecology sample database

Benthic ecology specimen photos

CTD data files raw HEX files and converted CNV (text) files

Survey log and daily reports

Sediment samples bucket list

Deck photos (from GA, AIMS, UWA)

Discussed analysis of tow video & photos with Marcus Stowar and Jamie Colquhoun. AIMS will process the still photos including geo-referencing of individual photos and incorporating into the new AIMS database. No analysis of video tapes is intended, as these are generally low quality due to turbidity and in many cases the tow camera was flying too high above the bed to provide continuous video transects. This was intentional to provide the best quality downward facing still photos.

Confirmed with Marcus that data from the underway Thermosalinograph (TSG) will be available after the survey from the AIMS website, following QC by the data manager (Marcus to also send TSG data via email).

Arrived Broome at 16:30; at anchor overnight in Roebuck Bay.

Saturday, 6 October 2012

Solander tied up at Broome wharf at 07:00 and unloading of gear and samples followed, with all gear of-loaded by mid-afternoon.

All science crew disembarked late morning and checked into accommodation prior to flying out to Canberra on Sunday.