EDM Height Traversing Levelling Survey Report

Rarotonga, Cook Islands, December 2012

Geoscience Australia
Record 2014/15

S. J. K. Yates1, A. Lal2

National Geospatial Reference Systems, Minerals and Natural Hazards Division, Geoscience Australia GPO Box 378 Canberra ACT 2601

Secretariat of the Pacific Community (SPC).

Department of Industry

Minister for Industry: The Hon Ian Macfarlane MP
Parliamentary Secretary: The Hon Bob Baldwin MP
Secretary: Ms Glenys Beauchamp PSM

Geoscience Australia

Chief Executive Officer: Dr Chris Pigram
This paper is published with the permission of the CEO, Geoscience Australia

© Commonwealth of Australia (Geoscience Australia) 2014

With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence. (http://www.creativecommons.org/licenses/by/3.0/au/deed.en)

Geoscience Australia has tried to make the information in this product as accurate as possible. However, it does not guarantee that the information is totally accurate or complete. Therefore, you should not solely rely on this information when making a commercial decision.

Geoscience Australia is committed to providing web accessible content wherever possible. If you are having difficulties with accessing this document please email clientservices@ga.gov.au.

ISSN 2201-702X (PDF)

ISBN 978-1-925124-04-0 (PDF)

GeoCat 76781

Bibliographic reference: Yates, S. J. K. & Lal, A., 2014, EDM Height Traversing Levelling Survey Report: Rarotonga, Cook Islands, December 2012. Record 2014/15. Geoscience Australia, Canberra. http://dx.doi.org/10.11636/Record.2014.015

Contents

1 Introduction	1

2 The Survey	2

2.1 Bench Mark Locality Diagram	3

2.2 The Cook Islands Datum	3

2.3 Equipment	3

2.4 Method	4

2.5 Cook Islands 2012 Reduced Levels	4

2.6 Survey Support	5

2.7 Issues	5

3 Comparisons	6

3.1 Comparisons between 2012 and 2011 EDM Surveys	6

3.2 Combined Comparisons 1993 to 2012	7

3.3 Time Series of Bench Mark Movement relative to Fixed Deep Bench Mark BM27	8

4 Deep Driven Bench Mark Locality Diagrams	13

5 Temporary Holding Mark Locality Diagrams	19

6 CKISBM and CKIS Reference Mark Locality Diagram	23

7 References	24

# Introduction

This report outlines the high precision level survey completed between the Sea Level Fine Resolution Acoustic Measuring Equipment (SEAFRAME) tide gauge and continuous Global Navigation Satellite System (GNSS) station in Rarotonga, Cook Islands from 5th to 9th December 2012.

Personnel involved in the survey were Steve Yates, Project Officer, Geoscience Australia and Andrick Lal, Surveyor, Secretariat of the Pacific Community (SPC).

The Electronic Distance Measurement (EDM) height traversing levelling technique was employed to observe differences in height between the deep bench mark arrays in Rarotonga, which runs approximately 2.2 km from the Tide Gauge Sensor to the continuous GNSS (CGNSS) antenna. Previous levelling surveys have been conducted along the route using this technique in 2004, 2007, 2008, 2009 and 2011.

In addition, precise differential levelling surveys were performed along the deep bench mark array from 1993 to 2004 by the National Tidal Centre Australia (NTCA) with the survey in 2004 including a comparison between the precise levelling and EDM height traversing technique. This report contains a comparison between the 2012 and 2011 EDM height traversing results as well as a combined comparison since the first levelling survey.

# The Survey

The EDM height traversing levelling survey was carried out between the SEAFRAME tide gauge sensor benchmark, CGNSS station and the deep driven bench mark array:

BM26 	– deep driven benchmark

COO56 	– SEAFRAME sensor benchmark

COO10 	– Project plaque point

BM18 	– local survey department benchmark

BM33 	– deep driven bench mark

BM27 	– reference deep driven bench mark for the survey

BM34 	– deep driven bench mark

BM28 	– deep driven bench mark

BM35 	– deep driven bench mark

CKISBM 	– reference bench mark for the CGNSS pillar

Also included in the survey were:

Temporary holding marks – COO77, COO102, COO84, COO85, COO103, COO100, COO101 and COO103 which are permanent stainless steel bolt holding marks, drilled in concrete and glued in place.

CGNSS reference point (CKISBM) and the three CGNSS Reference Marks - RM1, RM2 and RM3.

COO103 was a new holding mark established during the 2012 visit replacing COO92 that was found destroyed.

All the deep bench marks were located and found in good order and undisturbed.

The EDM height traversing levelling technique was performed to the Class L2A specifications (ICSM, Standards and Practices for Control Surveys). After reduction an internal precision of 1mm√K or better was achieved, where K is distance in kilometres. This is well within the project specification of 2mm√K. A table of results and comparisons and the 2012 reduced levels are detailed later in this report.

## Bench Mark Locality Diagram

## The Cook Islands Datum

The Datum for the survey is Mean Sea Level (MSL).

All reduced levels for the Rarotonga levelling survey are relative to BM27 fixed at a RL of 4.7407 m. This value was determined by the NTCA in 1998.

## Equipment

Leica total station model TM30 (Serial No: 361441)

Leica precision prisms GPH1P (2).

Leica rigid tripod.

Stainless steel target poles supported by Leica telescopic bi-poles (2).

Shortened stainless steel target pole for the SEAFRAME sensor BM connection.

Leica cast iron change plates (2).

Kestral 4000 pocket weather tracker

## Method

The “Leap-Frog” EDM height traversing technique was employed for the Rarotonga tide gauge levelling survey. This technique involves setting up a total station (TCA1800L) midway between two target/reflectors (on reflector rods with struts). The targets remain at a particular change point for the back-sight and fore-sight observations. The instrument measures slope distances (±1mm) and vertical angle (1”) to derive height differences (between the instrument’s trunnion axis and the reflectors). In support of the slope distance observations, the ambient temperature, pressure and humidity are recorded (Kestral 4000 pocket weather tracker) and input into the instrument to apply the first velocity correction to the observed distances (Rüeger & Brunner, 1982). Four rounds of observations are taken to the back-sight and fore-sight targets from each instrument setup. All levelling runs started and finished with the same reflector and reflector rod, i.e. an even number of setups when the two reflector rod configuration was used. This eliminates any reflector rod zero error. This technique can also be performed using a single set-up / single rod configuration which is particularly useful when levelling between bench marks which are close together e.g. between the CGNSS RMs.

Reduction of the digital data was computed by the Geoscience Australia levelling program “leveling1.exe”. This program computes the height difference between the two reflectors.

## Cook Islands 2012 Reduced Levels

Table 2.1 Rarotonga, Cook Islands– Reduced levels relative to BM27 from 2012 survey
Date:	5 – 9 December 2012
Datum:	Mean Sea Level

| Point ID | Reduced Level 2012 | Type |
| --- | --- | --- |
| BM27 | 4.74074 | Stainless Steel Rod in Ground |
| COO77 | 4.87097 | Stainless Steel Pin in Concrete |
| COO103 | 5.19673 | Stainless Steel Pin in Concrete |
| COO100 | 4.99580 | Stainless Steel Pin in Concrete |
| BM34 | 4.65339 | Stainless Steel Rod in Ground |
| COO84 | 3.48512 | Stainless Steel Pin in Concrete |
| BM28 | 3.19548 | Stainless Steel Rod in Ground |
| BM35 | 3.36607 | Stainless Steel Rod in Ground |
| COO85 | 3.75602 | Stainless Steel Pin in Concrete |
| COO101 | 3.11670 | Stainless Steel Pin in Concrete |
| CKISBM | 3.25347 | Stainless Steel Pillar Pin |
| *CKIS | 4.5153 | Pillar Plate (ARP) |
| BM33 | 3.46006 | Stainless Steel Rod in Ground |
| BM18 | 3.59371 | Brass Plaque in Concrete |
| COO10 | 1.54535 | Project Plaque with Stainless Steel Pin in Concrete |
| COO56 | 2.67877 | Tide Gauge Stainless Steel Pin |
| CKISRM1 | 2.63417 | Stainless Steel Rod in Ground |
| CKISRM2 | 2.44432 | Stainless Steel Rod in Ground |
| CKISRM3 | 2.80718 | Stainless Steel Rod in Ground |

*The RL of the Reference Point CKIS (ARP) is derived from adding the static height difference of 1.2618 m (CKISBM to CKIS) to the levelled RL of CKISBM.

## Survey Support

The survey team very much appreciated the assistance from Lands and Survey Department (Ministry of Works) and the Meteorological Department.

Personnel consulted during the visit were:

Mr Arona Ngari		Director of the Cook Islands Meteorological Office.

Mr. Keu Mataora		Director, Ministry of Works.

Mr. Timoti Tangiruaine 	Project Officer, Ministry of Works

Mr. Vaipo Matoara		GIS, Ministry of Works.

Both departments have a keen interest in the PSLMP (Pacific Sea Level Monitoring Project) and are always supportive with any associated project matters.

## Issues

Customs clearance of the survey equipment should be arranged prior to the visit to Cook Islands.

It is also essential to arrange access and security passes to the airport area prior to arrival in Cook Islands. This can be done through personnel from Ministry of Works and Meteorological Office.

Unfortunately we were not able to access deep benchmark, BM26, as it is inside the closed compound and the gates were locked.

# Comparisons

## Comparisons between 2012 and 2011 EDM Surveys

Table 3 Rarotonga, Cook Islands EDM Height Traversing Levelling Comparison 2012 - 2011. BM27 - adopted fixed height of 4.74074m

| From | To | Levelled Ht. Diff. | RL 2012 | Misclose (mm) | Dist. (km) | 1mmk | RL 2011 | Difference (mm) 2012 - 2011 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BM27 |  |  | 4.74074 |  |  |  |  |  |
| COO77 | COO77 | 0.13023 | 4.87097 | 0.100 | 0.1855 | 0.431 | 4.87071 | 0.26 |
| COO103 | COO103 | 0.32577 | 5.19673 | 0.002 | 0.1246 | 0.353 | NEW MARK | NEW MARK |
| COO100 | COO100 | -0.20094 | 4.99580 | 0.100 | 0.2089 | 0.457 | 4.99638 | -0.58 |
| BM34 | BM34 | -0.34241 | 4.65339 | 0.050 | 0.0464 | 0.215 | 4.65346 | -0.07 |
| COO84 | COO84 | -1.16826 | 3.48512 | -0.375 | 0.1911 | 0.437 | 3.48561 | -0.49 |
| BM28 | BM28 | -0.28965 | 3.19548 | -0.190 | 0.1234 | 0.351 | 3.19608 | -0.60 |
| BM35 | BM35 | 0.17059 | 3.36607 | -0.063 | 0.1409 | 0.375 | 3.3664 | -0.33 |
| COO85 | COO85 | 0.38994 | 3.75602 | 0.238 | 0.1709 | 0.413 | 3.75672 | -0.70 |
| COO101 | COO101 | -0.63932 | 3.11670 | -0.188 | 0.1537 | 0.392 | 3.11715 | -0.45 |
|  | CKISBM | 0.13678 | 3.25347 | -0.250 | 0.0761 | 0.276 | 3.25284 | 0.63 |
| BM27 |  |  | 4.74074 |  |  |  |  |  |
| BM33 | BM33 | -1.28068 | 3.46006 | -0.458 | 0.2109 | 0.459 | 3.46016 | -0.10 |
| BM18 | BM18 | 0.13365 | 3.59371 | -0.350 | 0.2219 | 0.471 | 3.59431 | -0.60 |
| COO10 | COO10 | -2.04836 | 1.54535 | 0.050 | 0.0552 | 0.235 | 1.54635 | -1.00 |
|  | COO56 | 1.13342 | 2.67877 | -0.040 | 0.0226 | 0.150 | 2.68169 | -2.92 |
|  |  |  | Misclose for all bays levelled = | -1.374 | 1.9319 | 1.38993 |  |  |
| CKISBM |  |  | 3.25347 |  |  |  |  |  |
|  | CKISRM1 | -0.61931 | 2.63417 | 0.010 | 0.0324 | 0.180 | 2.63319 | 0.98 |
| CKISBM |  |  | 3.25347 |  |  |  |  |  |
|  | CKISRM2 | -0.80915 | 2.44432 | 0.002 | 0.0253 | 0.159 | 2.44348 | 0.84 |
| CKISBM |  |  | 3.25347 |  |  |  |  |  |
|  | CKISRM3 | -0.44629 | 2.80718 | 0.000 | 0.0305 | 0.175 | 2.80616 | 1.02 |

*The RL of the Reference Point CKIS (ARP) is derived from adding the static height difference of 1.2618 m (CKISBM to CKIS) to the levelled RL of CKISBM.

All levelling was performed within the project specifications of 2k

## Combined Comparisons 1993 to 2012

Examination of the level survey results (holding BM27 as the fixed reference bench mark) indicate no significant displacement of any of the deep driven bench marks and the CGNSS benchmark, but movement was detected with COO56, the SEAFRAME Sensor Bench Mark.

Table 2 Rarotonga, Cook Islands – bench mark Comparison of the RL’s for Precise Differential Levelling (1993 – 2004) and EDM Height Traversing (2004 – 2012).

| Year | BM 18 | BM 26 | BM 28 | BM 29 | BM 33 | BM 34 | BM 35 | COO 10 | COO 56 | CKISBM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1993.1 | 3.6180 | 2.1587 | 3.1986 | 2.6141 |  |  |  | 1.5487 |  |  |
| 1994.9 | 3.6170 | 2.1564 | 3.1973 | 2.6121 |  |  |  | 1.5484 |  |  |
| 1996.5 | 3.6156 | 2.1549 | 3.1980 | 2.6132 |  |  |  | 1.5481 |  |  |
| 1998.5 | 3.6155 | 2.1539 | 3.1978 | 2.6133 |  |  |  | 1.5489 | 2.6843 |  |
| 1999.9 | 3.6147 | 2.1523 | 3.1974 | 2.6121 |  |  |  | 1.5484 | 2.6844 |  |
| 2001.5 | 3.6181 | 2.1545 | 3.1976 | 2.6134 | 3.4606 | 4.6542 | 3.3664 | 1.5518 | 2.6904 |  |
| 2002.9 | 3.6147 | 2.1499 | 3.1984 | 2.6131 | 3.4615 | 4.6555 | 3.3680 | 1.5485 | 2.6867 | 3.2560 |
| 2004.6 | 3.6131 | 2.1474 | 3.1971 |  | 3.4615 | 4.6555 | 3.3680 | 1.5472 | 2.6855 | 3.2547 |

| Year | BM 18 | BM 26 | BM 28 | BM 29 | BM 33 | BM 34 | BM 35 | COO 10 | COO 56 | CKISBM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2004.6 |  | 2.1487 | 3.1972 |  | 3.4609 | 4.6546 | 3.3671 | 1.5481 | 2.6864 | 3.2562 |
| 2007.1 | 3.6101 | 2.1461 | 3.1976 |  | 3.4603 | 4.6543 | 3.3677 | 1.5473 | 2.6831 | 3.2561 |
| 2008.5 | 3.5965 | 2.1455 | 3.1958 |  | 3.4606 | 4.6525 | 3.3656 | 1.5474 | 2.6820 | 3.2535 |
| 2009.9 | 3.5965 | 2.1455 | 3.1957 |  | 3.4603 | 4.6534 | 3.3657 | 1.5472 | 2.6819 | 3.2521 |
| 2011.4 | 3.5943 | 2.1431 | 3.1961 |  | 3.4602 | 4.6535 | 3.3664 | 1.5464 | 2.6817 | 3.2528 |
| 2012.9 | 3.5937 |  | 3.1955 |  | 3.4601 | 4.6534 | 3.3661 | 1.5454 | 2.6788 | 3.2535 |

## Time Series of Bench Mark Movement relative to Fixed Deep Bench Mark BM27

Precise Differential Levelling: 1993 - 2004	
EDM Height Traversing: 2004 onwards

According to the 2012 survey, the SEAFRAME Sensor Benchmark, COO56, moved by -2.92mm (dropped); whereas all the other benchmarks showing insignificant movement. This indicates that COO56 at the tide gauge may have been disturbed.

# Deep Driven Bench Mark Locality Diagrams

# Temporary Holding Mark Locality Diagrams

# CKISBM and CKIS Reference Mark Locality Diagram

# References

Rüeger, J.M. & Brunner, F.K. 1982, ‘EDM Height Traversing versus Geodetic Levelling’, The Canadian Surveyor, vol. 36, no. 1, pp. 69-87.

Inter-Government Committee on Surveying and Mapping (ICSM) 2002, Standards and Practices for Control Surveys, SP1, Ver. 1.5, May 2002.