EDM Height Traversing Levelling Survey Report

Lautoka, Fiji Islands, February 2013

Geoscience Australia
Record 2014/16

S. J. K. Yates1, A. Lal2

National Geospatial Reference Systems, Minerals and Natural Hazards Division, Geoscience Australia GPO Box 378 Canberra ACT 2601

Secretariat of the Pacific Community (SPC)

Department of Industry

Minister for Industry: The Hon Ian Macfarlane MP
Parliamentary Secretary: The Hon Bob Baldwin MP
Secretary: Ms Glenys Beauchamp PSM

Geoscience Australia

Chief Executive Officer: Dr Chris Pigram
This paper is published with the permission of the CEO, Geoscience Australia

© Commonwealth of Australia (Geoscience Australia) 2014

With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence. (http://www.creativecommons.org/licenses/by/3.0/au/deed.en)

Geoscience Australia has tried to make the information in this product as accurate as possible. However, it does not guarantee that the information is totally accurate or complete. Therefore, you should not solely rely on this information when making a commercial decision.

Geoscience Australia is committed to providing web accessible content wherever possible. If you are having difficulties with accessing this document please email clientservices@ga.gov.au.

ISSN 2201-702X (PDF)

ISBN 978-1-925124-05-7 (PDF)

GeoCat 76782

Bibliographic reference: Yates, S. J. K. & Lal, A., 2014, EDM Height Traversing Levelling Survey Report: Lautoka, Fiji Islands, February 2013. Record 2014/16. Geoscience Australia, Canberra. http://dx.doi.org/10.11636/Record.2014.016

Contents

1 Introduction	1

2 The Survey	2

2.1 Bench Mark Locality Diagram	3

2.2 The Fiji Islands Datum	3

2.3 Equipment	3

2.4 Method	4

2.5 Lautoka 2013 Reduced Levels	4

2.6 Survey Support	5

2.7 Issues	5

3 Comparisons	6

3.1 Comparisons between 2013 and 2011 EDM Surveys	6

3.2 Combined Comparisons 1992 to 2013	7

3.3 Time Series of Bench Mark Movement relative to Fixed Deep Bench Mark LAUTBM	8

4 Deep Bench Mark Locality Diagrams	12

5 LAUTBM and LAUT Reference Mark Locality Diagrams	18

6 Temporary Holding Mark Locality Diagrams	22

7 References	30

# Introduction

This report outlines the levelling survey completed during the visit to Lautoka, Fiji from 17th to 24th February 2013.

Personnel involved in the survey were Steve Yates, Project Officer, Geoscience Australia and Andrick Lal, Surveyor, Secretariat of the Pacific Community (SPC).

This is the sixth Electronic Distance Measurement (EDM) height traversing levelling survey of the deep bench mark array in Lautoka, Fiji, which runs approximately 3 km from the tide gauge sensor benchmark (SEAFRAME – Sea Level Fine Resolution Acoustic Measuring Equipment) at the wharf to the continuous GNSS (Global Navigation Satellite System) Station located in the Fiji Sugar Corporation (FSC) Compound. The previous levelling surveys using this technique were performed in 2005, 2007, 2008, 2010 and 2011.

Precise Differential Levelling surveys were performed on eight occasions between 1992 and 2003 by the National Tidal Centre Australia (NTCA) and the survey in 2005 included a comparison between the precise levelling and EDM height traversing technique. This report contains a comparison between the 2011 and 2013 EDM height traversing results as well as a combined comparison since the first levelling survey.

# The Survey

The EDM Height Traversing levelling survey was carried out between the five deep driven bench marks and from the CGNSS Station to the SEAFRAME Station:

LAUTBM	– CGNSS Benchmark

FIJ13 		– SEAFRAME sensor benchmark

BM3244 	– Deep Driven Benchmark

BM3245 	– Deep Driven Benchmark

BM3246 	– Deep Driven Benchmark

BM3247 	– Deep Driven Benchmark

BM3248 	– Deep Driven Benchmark

All the deep bench marks were located and found in good order and undisturbed except the primary benchmark BM3243 which, unfortunately is now covered with soil and a statute, making it impossible to recover and rendering it useless. Therefore, the LAUTBM was used as the primary, and fixed, benchmark and will be used as such for future surveys.

Included in the survey was the SEAFRAME tide gauge sensor bench mark, FIJ13 and the survey mark (project plaque) located adjacent to the tide gauge hut, FIJ12.

Also included in the survey were temporary holding marks – FIJ212, FIJ81, FIJ207, FIJ205, FIJ84, FIJ85, FIJ206, FIJ87, FIJ92, FIJ208, FIJ209, FIJ210, FIJ211 and FIJ214 (new mark) as well as the CGNSS bench mark – LAUTBM and the three CGNSS reference marks - RM1, RM2 and RM4 .

The EDM height traversing levelling technique was performed to the Class L2A specifications (ICSM, Standards and Practices for Control Surveys). After reduction an internal precision of 1mm√K or better was achieved, where K is distance in kilometres. This is well within the project specification of 2mm√K. A table of results and comparisons and the 2012 reduced levels are detailed later in this report.

## Bench Mark Locality Diagram

## The Fiji Islands Datum

The Datum for the survey is Tide Staff Zero (TSZ). Reduction of the data has been calculated holding LAUTBM (the Reference Bench Mark for the CGNSS Pillar) fixed with a Reduced Level of 31.3403m, this value was determined by the SPSLCMP survey team in July 2011, computed from the previously held fixed bench mark BM3243.

## Equipment

Leica total station model TM30 (Serial No: 361441)

Leica precision prisms GPH1P (2).

Leica rigid tripod.

Stainless steel target poles supported by Leica telescopic bi-poles (2).

Shortened stainless steel target pole for the SEAFRAME sensor BM connection.

Leica cast iron change plates (2).

Kestral 4000 pocket weather tracker

## Method

The “Leap-Frog” EDM height traversing technique was employed for the Lautoka tide gauge levelling survey. This technique involves setting up a total station (TCA1800L) midway between two target/reflectors (on reflector rods with struts). The targets remain at a particular change point for the back-sight and fore-sight observations. The instrument measures slope distances (±1mm) and vertical angle (1”) to derive height differences (between the instrument’s trunnion axis and the reflectors). In support of the slope distance observations, the ambient temperature, pressure and humidity are recorded (Kestral 4000 pocket weather tracker) and input into the instrument to apply the first velocity correction to the observed distances (Rüeger & Brunner, 1982). Four rounds of observations are taken to the back-sight and fore-sight targets from each instrument setup. All levelling runs started and finished with the same reflector and reflector rod, i.e. an even number of setups when the two reflector rod configuration was used. This eliminates any reflector rod zero error. This technique can also be performed using a single set-up / single rod configuration which is particularly useful when levelling between bench marks which are close together e.g. between the CGNSS RMs.

Reduction of the digital data was computed by the Geoscience Australia levelling program “leveling1.exe”. This program computes the height difference between the two reflectors.

## Lautoka 2013 Reduced Levels

Table 2.1 Lautoka, Fiji – Reduced levels from the 2013 survey
Date:	19th to 24th February 2013
Datum:	Tide Staff Zero

| Point ID | Reduced Level 2013 | Type |
| --- | --- | --- |
| LAUTBM | 31.3403 | Stainless Steel Pillar Pin |
| *LAUT | 32.2955 | Pillar Plate (ARP) |
| FIJ92 | 26.2538 | Stainless Steel Pin in Concrete |
| BM3247 | 20.9601 | Stainless Steel Rod in Ground |
| BM3246 | 7.8256 | Stainless Steel Rod in Ground |
| FIJ87 | 7.4018 | Stainless Steel Pin in Concrete |
| FIJ206 | 4.6409 | Stainless Steel Pin in Concrete |
| BM3244 | 4.1729 | Stainless Steel Rod in Ground |
| FIJ211 | 4.2342 | Stainless Steel Pin in Concrete |
| FIJ85 | 3.1427 | Stainless Steel Pin in Concrete |
| FIJ84 | 3.0463 | Stainless Steel Pin in Concrete |
| FJ210 | 3.2207 | Stainless Steel Pin in Concrete |
| FJ205 | 5.3255 | Stainless Steel Pin in Concrete |
| FJ212 | 3.1492 | Stainless Steel Pin in Concrete |
| FJ81 | 2.6684 | Stainless Steel Pin in Concrete |
| FJ207 | 4.1709 | Stainless Steel Pin in Concrete |
| FJ12 | 4.0713 | Project Plaque Stainless Steel Pin |
| FJ13 | 4.4458 | Stainless Steel Sensor Pin |
| BM3248 | 35.2356 | Stainless Steel Rod in Ground |
| FJ208 | 30.9335 | Stainless Steel Pin in Concrete |
| FJ209 | 25.0123 | Stainless Steel Pin in Concrete |
| FJ214 | 16.8872 | Masonry Nail in Concrete |
| BM3245 | 16.6377 | Stainless Steel Rod in Ground |

*The RL of the Reference Point LAUT (ARP) is derived from adding the static height difference of 0.9552 m (LAUTBM to LAUT) to the levelled RL of LAUTBM.

## Survey Support

The survey team very much appreciated the assistance from staff of the Lands Department Lautoka Office, the Lautoka Ports and the Fiji Sugar Corporation.

Personnel consulted during the visit were:

Mr. Litia Navusolo, Divisional Surveyor, Lands Department

Mr. Jekope Tubukalou, Technical Officer – Surveys, Lands Department

Email: jekope.tubikalou@govnet.gov.fj; Phone (679) 9525570/ 6661800

Mr. Atunaisa Ralulu, Technical Officer – Surveys, Lands Department

Email: arobanakadavu@govnet.gov.fj; Phone (679) 9214167/ 6661800

Mr. Emosi Navusolo, Supervisor Administration – Lautoka Ports

Email: emosi@fijiports.com.fj; Phone (679) 9907125/ 6662160

Mr. Viliame Savou, Production Manager – Fiji Sugar Corporation, Lautoka.

Phone (679) 9922006

The Lautoka Lands and Survey Department have always had a keen interest in the SPSLCMP and are always supportive with any associated project matters.

## Issues

A security pass is required in order to access the Lautoka Ports area, where the tide gauge is located. This must be arranged prior to the survey through the Lands Department or SOPAC (SPC) staff. This is also a hard-hat zone. A letter to the Ports Authority for accessibility is necessary prior to the visit.

# Comparisons

## Comparisons between 2013 and 2011 EDM Surveys

Table 3.1 Lautoka, Fiji EDM Height Traversing Levelling Comparison 2013 – 2011. LAUTBM - adopted fixed height of 31.34031m

| From | To | Levelled Ht. Diff. | RL 2013 | Misclose (mm) | Dist. (km) | 1mmk | RL 2011 | Difference (mm) 2013 - 2011 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LAUTBM |  |  | 31.34031 |  |  |  |  |  |
| FIJ92 | FIJ92 | -5.086550 | 26.25376 | -0.250 | 0.0697 | 0.264 | 26.25424 | -0.48 |
| BM3247 | BM3247 | -5.293656 | 20.96010 | 0.063 | 0.0932 | 0.305 | 20.95991 | 0.19 |
| BM3246 | BM3246 | -13.134526 | 7.82558 | -0.325 | 0.2184 | 0.467 | 7.82428 | 1.30 |
| FIJ87 | FIJ87 | -0.423735 | 7.40184 | -0.010 | 0.0809 | 0.284 | 7.39929 | 2.55 |
| FIJ206 | FIJ206 | -2.760930 | 4.64091 | 0.265 | 0.2251 | 0.474 | 4.63804 | 2.87 |
|  | BM3244 | -0.468025 | 4.17289 | 0.075 | 0.0362 | 0.190 | 4.17025 | 2.64 |
| FIJ206 |  |  | 4.64091 |  |  |  |  |  |
| FIJ211 | FIJ211 | -0.406728 | 4.23419 | -0.370 | 0.0985 | 0.314 | 4.23130 | 2.89 |
| FIJ85 | FIJ85 | -1.091521 | 3.14266 | -0.278 | 0.1784 | 0.422 | 3.14041 | 2.25 |
| FIJ84 | FIJ84 | -0.096325 | 3.04634 | 0.230 | 0.0803 | 0.283 | 3.04341 | 2.93 |
| FIJ210 | FIJ210 | 0.174340 | 3.22068 | 0.100 | 0.0769 | 0.277 | 3.21697 | 3.71 |
| FIJ205 | FIJ205 | 2.104775 | 5.32545 | -0.200 | 0.1287 | 0.359 | 5.32272 | 2.73 |
| FIJ212 | FIJ212 | -2.176263 | 3.14919 | 0.275 | 0.1570 | 0.396 | 3.14549 | 3.70 |
| FIJ81 | FIJ81 | -0.480819 | 2.66837 | -0.363 | 0.1810 | 0.425 | 2.66568 | 2.69 |
| FIJ207 | FIJ207 | 1.502563 | 4.17094 | -0.325 | 0.1724 | 0.415 | 4.16675 | 4.19 |
| FIJ12 | FIJ12 | -0.099686 | 4.07125 | 0.238 | 0.0467 | 0.216 | 4.06708 | 4.17 |
|  | FIJ13 | 0.374515 | 4.44577 | 0.010 | 0.0130 | 0.114 | 4.44091 | 4.86 |
| LAUTBM |  |  | 31.34031 |  |  |  |  |  |
| BM3248 | BM3248 | 3.895332 | 35.23564 | 0.388 | 0.1670 | 0.409 | 35.23661 | -0.97 |
| FIJ208 | FIJ208 | -4.302126 | 30.93352 | -0.175 | 0.1972 | 0.444 | 30.93406 | -0.54 |
| FIJ209 | FIJ209 | -5.921169 | 25.01235 | 0.288 | 0.1909 | 0.437 | 25.01221 | 0.14 |
| FIJ214 | FIJ214 | -8.125194 | 16.88715 | 0.188 | 0.1204 | 0.347 | New Mark | New Mark |
|  | BM3245 | -0.249476 | 16.63768 | 0.000 | 0.2164 | 0.465 | 16.63951 | -1.83 |
|  |  |  | Misclose for all bays levelled = | -0.177 | 2.748 | 1.658 |  |  |
| LAUTBM |  |  | 31.34031 |  |  |  |  |  |
|  | RM1 | -2.329155 | 29.01116 | -0.070 | 0.0443 | 0.210 | 29.01115 | 0.00 |
| LAUTBM |  |  | 31.34031 |  |  |  |  |  |
|  | RM2 | -1.00255 | 30.33776 | -0.040 | 0.0322 | 0.179 | 30.33782 | -0.06 |
| LAUTBM |  |  | 31.34031 |  |  |  |  |  |
|  | RM4 | -0.70364 | 30.63667 | 0.000 | 0.0348 | 0.186 | 30.63639 | 0.28 |

All levelling was performed within the project specifications of 2k

## Combined Comparisons 1992 to 2013

Table.3.2 Lautoka, Fiji - Comparison of the RL's for Precise Differential Levelling (1992 – 2005) and EDM Height Traversing (2005 - 2013).

| Year | BM 3244 | BM 3245 | BM 3246 | BM 3247 | BM 3248 | LAUTBM | FIJ 13 | FIJ 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1992.8 | 4.1601 | 16.6280 |  |  |  |  | 4.4326 | 4.0591 |
| 1994.6 | 4.1597 | 16.6270 |  |  |  |  | 4.4336 | 4.0597 |
| 1995.9 | 4.1606 | 16.6293 |  |  |  |  | 4.4345 | 4.0606 |
| 1997.5 | 4.1607 | 16.6293 |  |  |  |  | 4.4341 | 4.0608 |
| 1998.9 | 4.1612 | 16.6243 |  |  |  |  | 4.4342 | 4.0606 |
| 2001.0 | 4.1630 | 16.6314 |  |  |  |  | 4.4352 | 4.0611 |
| 2002.2 | 4.1632 | 16.6313 | 7.8170 | 20.9526 | 35.2283 | 31.3337 | 4.4348 | 4.0603 |
| 2003.4 | 4.1635 | 16.6327 | 7.8179 | 20.9533 | 35.2294 | 31.3338 | 4.4368 | 4.0624 |
| 2005.4 | 4.1679 | 16.6362 | 7.8219 | 20.9571 | 35.2332 | 31.3375 | 4.4396 | 4.0657 |

| Year | BM 3244 | BM 3245 | BM 3246 | BM 3247 | BM 3248 | LAUTBM | FIJ 13 | FIJ 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2005.4 | 4.1674 | 16.6366 | 7.8217 | 20.9569 | 35.2334 | 31.3379 | 4.4395 | 4.0655 |
| 2007.1 | 4.1679 | 16.6365 | 7.8220 | 20.9572 | 35.2333 | 31.3384 | 4.4413 | 4.0664 |
| 2008.8 | 4.1690 | 16.6379 | 7.8234 | 20.9586 | 35.2360 | 31.3399 | 4.4416 | 4.0672 |
| 2010.2 | 4.1700 | 16.6371 | 7.8244 | 20.9586 | 35.2347 | 31.3390 | 4.4426 | 4.0678 |
| 2011.6 | 4.1703 | 16.6395 | 7.8243 | 20.9599 | 35.2366 | 31.3403 | 4.4409 | 4.0671 |
| 2013.1 | 4.1729 | 16.6377 | 7.8256 | 20.9601 | 35.2356 | 31.3403 | 4.4458 | 4.0713 |

## Time Series of Bench Mark Movement relative to Fixed Deep Bench Mark LAUTBM

Precise Differential Levelling: 1992 - 2005	
EDM Height Traversing: 2005 onwards

The survey measured a significant movement on the Tide Gauge in Lautoka (+4.86mm) which indicates instability at the wharf. The deep driven benchmarks have also shown an unexplained movement between +1.3mm to +4.17mm range.

# Deep Bench Mark Locality Diagrams

# LAUTBM and LAUT Reference Mark Locality Diagrams

# Temporary Holding Mark Locality Diagrams

# References

Rüeger, J.M. & Brunner, F.K. 1982, ‘EDM Height Traversing versus Geodetic Levelling’, The Canadian Surveyor, vol. 36, no. 1, pp. 69-87.

Inter-Government Committee on Surveying and Mapping (ICSM) 2002, Standards and Practices for Control Surveys, SP1, Ver. 1.5, May 2002.