EDM Height Traversing Levelling Survey Report

Nauru, February 2012

Geoscience Australia
Record 2014/22

S. J. K. Yates1, A. Lal2

National Geospatial Reference Systems, Geospatial and Earth Monitoring Division, Geoscience Australia GPO Box 378 Canberra ACT 2601

Secretariat of the Pacific  Community (SPC)

Department of Industry

Minister for Industry: The Hon Ian Macfarlane MP
Parliamentary Secretary: The Hon Bob Baldwin MP
Secretary: Ms Glenys Beauchamp PSM

Geoscience Australia

Chief Executive Officer: Dr Chris Pigram
This paper is published with the permission of the CEO, Geoscience Australia

© Commonwealth of Australia (Geoscience Australia) 2014

With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence. (http://www.creativecommons.org/licenses/by/3.0/au/deed.en)

Geoscience Australia has tried to make the information in this product as accurate as possible. However, it does not guarantee that the information is totally accurate or complete. Therefore, you should not solely rely on this information when making a commercial decision.

Geoscience Australia is committed to providing web accessible content wherever possible. If you are having difficulties with accessing this document please email clientservices@ga.gov.au.

ISSN 2201-702X (PDF)

ISBN 978-1-925124-11-8 (PDF)

GeoCat 76785

Bibliographic reference: Yates, S. J. K. & Lal, A., 2014, EDM Height Traversing Levelling Survey Report: Nauru, February 2012. Record 2014/22. Geoscience Australia, Canberra. http://dx.doi.org/10.11636/Record.2014.022

Contents

1 Introduction	1

2 The Survey	2

2.1 Bench Mark Locality Diagram	3

2.2 The Nauru Datum	4

2.3 Equipment	4

2.4 Method	4

2.5 Nauru 2012 Reduced Levels	5

2.6 Survey Support	6

2.7 Issues	6

2.8 Description of Marks – Nauru	6

3 Comparisons	7

3.1 Comparisons between 2010 and 2012 Surveys	7

3.2 Combined Comparisons 1993 to 2012	8

3.3 Time Series of Bench Mark Movement relative to Fixed Deep Bench Mark NAU1	9

4 Deep driven Bench Mark Locality Diagrams	12

5 NAURBM and NAUR Reference Mark Locality Diagrams	27

6 References	31

# Introduction

This report outlines the levelling survey completed during the visit to Nauru from 6 to 15 February 2012.

Personnel involved in the survey were Steve Yates, Project Officer, Geoscience Australia and Andrick Lal, Surveyor, Secretariat of the Pacific Community (SPC).

This is the fifth Electronic Distance Measurement (EDM) height traversing levelling survey of the deep bench mark array in Nauru. The previous levelling surveys using this technique were performed in 2005, 2007, 2009 and 2010.

Precise Differential Levelling surveys were performed on nine occasions between 1993 and 2003 by the National Tidal Centre Australia (NTCA) and the survey in 2005 included a comparison between the precise levelling and EDM height traversing techniques. This report contains a comparison between the 2010 and 2012 EDM height traversing results as well as a combined comparison since the first levelling survey.

# The Survey

The EDM Height Traversing levelling survey was carried out between the 6 deep driven bench marks:

NAU1

NAU2

NAU36

NAU37

NAU38

NAU16

All the deep bench marks were located and found in good order and undisturbed. Included in the survey was the Sea Level Fine Resolution Acoustic Measuring Equipment (SEAFRAME) tide gauge sensor bench mark, NAU15 and the tide gauge benchmark located adjacent to the tide gauge hut, NAU14.

Also included in the survey were temporary holding marks – NAU28, NAU100-109, NAU33, NAU43, NAU44, NAU47, NAU53 as well as the Continuous Global Navigation Satellite System (CGNSS) bench mark NAURBM, the CGNSS reference point NAUR, and the three CGNSS reference marks, RM1, RM2 and RM3.

The EDM height traversing levelling technique was performed to the Class L2A specifications (ICSM, 2002). After reduction an internal precision of 1mm √K or better was achieved, where K is distance in kilometres. This is well within the project specification of 2mm √K. A table of results and comparisons and the 2012 reduced levels are detailed later in this report.

All the deep bench marks were located and found in good order and undisturbed.

The EDM height traversing levelling technique was performed to the Class L2A specifications (ICSM, Standards and Practices for Control Surveys). After reduction an internal precision of 1mm√K or better was achieved, where K is distance in kilometres. This is well within the project specification of 2mm√K. A table of results and comparisons and the 2012 reduced levels are detailed later in this report.

## Bench Mark Locality Diagram

## The Nauru Datum

The Datum for the survey is the Nauru Island Datum (NID). Reduction of the data has been calculated holding NAU1 fixed at 7.2930 metres, this value was determined by the NTC in 1993 by:

1993	Adopting the height of UH 1 as 4.130m NID

1994	Adopting the 1993 height of U2B as 3.8894m NID

## Equipment

Leica total station model TM30 (Serial No: 361441)

Leica precision prisms GPH1P (2).

Leica rigid tripod.

Stainless steel target poles supported by Leica telescopic bi-poles (2).

Shortened stainless steel target pole for the SEAFRAME sensor BM connection.

Leica cast iron change plates (2).

Kestral 4000 pocket weather tracker

## Method

The “Leap-Frog” EDM height traversing technique was employed for the Nauru tide gauge levelling survey. This technique involves setting up a total station (TCA1800L) midway between two target/reflectors (on reflector rods with struts). The targets remain at a particular change point for the back-sight and fore-sight observations. The instrument measures slope distances (±1mm) and vertical angle (1”) to derive height differences (between the instrument’s trunnion axis and the reflectors). In support of the slope distance observations, the ambient temperature, pressure and humidity are recorded (Kestral 4000 pocket weather tracker) and input into the instrument to apply the first velocity correction to the observed distances (Rüeger & Brunner, 1982). Four rounds of observations are taken to the back-sight and fore-sight targets from each instrument setup. All levelling runs started and finished with the same reflector and reflector rod, i.e. an even number of setups when the two reflector rod configuration was used. This eliminates any reflector rod zero error. This technique can also be performed using a single set-up / single rod configuration which is particularly useful when levelling between bench marks which are close together e.g. between the CGNSS RMs.

Reduction of the digital data was computed by the Geoscience Australia levelling program “leveling1.exe”. This program computes the height difference between the two reflectors.

## Nauru 2012 Reduced Levels

Table 2.1 Nauru – Reduced levels and heights difference relative to NAU1 from the 2012 survey
Date:	6 – 15 February 2012
Datum:	Nauru Island Datum

| POINT # | 2010 levelled diff. ht. | 2010 RL |
| --- | --- | --- |
| NAU14 | 3.4458 | 3.8481 |
| NAU15 | 1.2869 | 6.0052 |
| NAU36 | 0.0735 | 7.2190 |
| NAU28 | -0.5070 | 7.7987 |
| NAU2 | -0.4930 | 7.7846 |
| NAU105 | -0.4907 | 7.7824 |
| NAU100 | -0.9192 | 8.2100 |
| NAU106 | -0.7550 | 8.0458 |
| NAU37 | -0.4083 | 7.6988 |
| NAU38 | -0.2814 | 7.5722 |
| NAU107 | -0.4891 | 7.7797 |
| NAU33 | -0.1189 | 7.4094 |
| NAU16 | 0.5969 | 6.6935 |
| NAU108 | 0.4267 | 6.8638 |
| NAU43 | 1.3698 | 5.9197 |
| NAU44 | 0.7994 | 6.4896 |
| NAU109 | 1.5771 | 5.7107 |
| NAU47 | 1.4456 | 5.8421 |
| NAU101 | 0.9741 | 6.3134 |
| NAU102 | 0.9847 | 6.3031 |
| NAU103 | 1.6645 | 5.6225 |
| NAU104 | 2.0930 | 5.1937 |
| NAU53 | 1.8035 | 5.4825 |
| NAURBM | 2.0283 | 5.2580 |
| *NAUR |  | 6.2063 |
| NAUR | 1.0797 | 6.2133 |
| RM1 | 2.7406 | 4.5456 |
| RM2 | 2.9032 | 4.3831 |
| RM3 | 2.8052 | 4.4810 |

*The RL of the Reference Point NAUR (ARP) is derived from adding the static height difference of 0.9483 m (NAURBM to NAUR) to the levelled RL of NAURBM.

## Survey Support

The survey team very much appreciated the assistance from staff of the Lands Department Nauru Office.

Personnel consulted during the visit were:

Mr. Porthos Bop –Chief Surveyor, Lands Department

Mr.Wess Tsitsi – Senior Surveyor, Lands Department

The Nauru Lands and Survey Department provided a chainman to assist with the field survey. Both Porthos and Wess have always taken a keen interest in the SPSLCMP and are always supportive with any associated project matters.

## Issues

It is recommended to establish an additional deep driven bench mark approximately midway along the airport road between NAU16 and the CGPS bench mark NAURBM.

Also, the valve boxes for the CGPS reference marks, RM2 and RM3 require replacement.

## Description of Marks – Nauru

NAU1 is the bench mark held fixed with an RL = 7.2930 metres. The height of NAU1 was derived by NTC as described earlier.

Deep driven bench marks:

NAU1, NAU2, NAU36, NAU37, NAU38 and NAU16 are deep driven bench marks.

Other marks:

NAU15 is the SEAFRAME Sensor Bench Mark.

NAU 14 is the Project Plaque point, located adjacent to the tide gauge sensor.

NAURBM is the Reference Bench Mark for the CGPS Pillar.

NAUR is the CGPS mark.

All other points used with the prefix of NAU are temporary holding marks consisting of stainless steel bolts drilled in concrete and glued in situ with epoxy resin.

# Comparisons

## Comparisons between 2010 and 2012 Surveys

Table 3.1 Nauru EDM Height Traversing Levelling Comparison 2010 - 2012 and Table of Results. NAU 1 - adopted fixed height (NID) 7.29300 m

| From | To | Levelled Ht. Diff. | RL 2012 | Misclose (mm) | Dist. (km) | 1mmk | RL 2010 | Difference (mm) 2012 - 2010 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAU1 |  |  | 7.2930 |  |  |  |  |  |
| NAU14 | NAU14 | -3.4449 | 3.8481 | 0.090 | 0.109 | 0.330 | 3.8472 | 0.0009 |
|  | NAU15 | 2.1571 | 6.0052 | 0.050 | 0.007 | 0.084 | 6.0061 | -0.0009 |
| NAU1 |  |  | 7.2930 |  |  |  |  |  |
| NAU36 | NAU36 | -0.0741 | 7.2190 | 0.400 | 0.186 | 0.431 | 7.2195 | -0.0005 |
| NAU28 | NAU28 | 0.5798 | 7.7987 | 0.400 | 0.196 | 0.443 | 7.8000 | -0.0013 |
| NAU2 | NAU2 | -0.0141 | 7.7846 | 0.300 | 0.167 | 0.409 | 7.7860 | -0.0014 |
| NAU105 | NAU105 | -0.0022 | 7.7824 | 0.050 | 0.072 | 0.268 | 7.7837 | -0.0013 |
| NAU100 | NAU100 | 0.4276 | 8.2100 | -0.200 | 0.097 | 0.311 | 8.2122 | -0.0022 |
| NAU106 | NAU106 | -0.1642 | 8.0458 | 0.280 | 0.200 | 0.447 | 8.0480 | -0.0022 |
| NAU37 | NAU37 | -0.3470 | 7.6988 | -0.020 | 0.118 | 0.344 | 7.7013 | -0.0025 |
| NAU38 | NAU38 | -0.1267 | 7.5722 | 0.080 | 0.054 | 0.232 | 7.5744 | -0.0022 |
| NAU107 | NAU107 | 0.2075 | 7.7797 | 0.330 | 0.200 | 0.447 | 7.7821 | -0.0024 |
| NAU33 | NAU33 | -0.3703 | 7.4094 | 0.080 | 0.131 | 0.361 | 7.4119 | -0.0025 |
|  | NAU16 | -0.7158 | 6.6935 | -0.110 | 0.123 | 0.351 | 6.6961 | -0.0026 |
| NAU33 |  |  |  |  |  |  |  |  |
| NAU108 | NAU108 | -0.5455 | 6.8638 | 0.090 | 0.178 | 0.421 | 6.8663 | -0.0025 |
| NAU43 | NAU43 | -0.9441 | 5.9197 | -0.040 | 0.177 | 0.420 | 5.9232 | -0.0035 |
| NAU44 | NAU44 | 0.5699 | 6.4896 | 0.110 | 0.204 | 0.451 | 6.4936 | -0.0040 |
| NAU109 | NAU109 | -0.7789 | 5.7107 | -0.060 | 0.200 | 0.447 | 5.7159 | -0.0052 |
| NAU47 | NAU47 | 0.1315 | 5.8421 | -0.450 | 0.199 | 0.447 | 5.8474 | -0.0053 |
| NAU101 | NAU101 | 0.4713 | 6.3134 | 0.090 | 0.200 | 0.447 | 6.3189 | -0.0055 |
| NAU102 | NAU102 | -0.0104 | 6.3031 | 0.280 | 0.199 | 0.446 | 6.3083 | -0.0052 |
| NAU103 | NAU103 | -0.6806 | 5.6225 | 0.300 | 0.199 | 0.447 | 5.6285 | -0.0060 |
| NAU104 | NAU104 | -0.4288 | 5.1937 | 0.010 | 0.199 | 0.446 | 5.2000 | -0.0063 |
| NAU53 | NAU53 | 0.2888 | 5.4825 | 0.240 | 0.193 | 0.439 | 5.4895 | -0.0070 |
| NAURBM | NAURBM | -0.2245 | 5.2580 | 0.040 | 0.074 | 0.272 | 5.2647 | -0.0067 |
| NAURBM |  |  |  |  |  |  |  |  |
|  | RM1 | -0.7124 | 4.5456 |  | 0.020 | 0.142 | 4.5524 | -0.0068 |
| NAURBM |  |  |  |  |  |  |  |  |
|  | RM2 | -0.8749 | 4.3831 |  | 0.022 | 0.148 | 4.3898 | -0.0067 |
| NAURBM |  |  |  |  |  |  |  |  |
|  | RM3 | -0.7769 | 4.4810 |  | 0.024 | 0.155 | 4.4878 | -0.0068 |

An internal precision of 1mm √K was achieved for all level bays, the Project Specification is 2mm √K

## Combined Comparisons 1993 to 2012

Table 3.2 Nauru - Comparison of the RL's for Precise Differential Levelling (1993 – 2005) and EDM Height Traversing (2005 - 2012).

| Year | NAU1 | NAU2 | NAU36 | NAU37 | NAU38 | NAU16 | NAURBM | NAU14 | NAU15 | *NAUR | RM1 | RM2 | RM3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1993.5 | 7.2930 | 7.7837 |  |  |  |  |  | 3.8459 | 6.0044 |  |  |  |  |
| 1994.2 | 7.2930 | 7.7849 |  |  |  | 6.6915 |  | 3.8462 | 6.0041 |  |  |  |  |
| 1995.2 | 7.2930 | 7.7849 |  |  |  | 6.6923 |  | 3.8459 | 6.0038 |  |  |  |  |
| 1996.7 | 7.2930 | 7.7836 |  |  |  | 6.6923 |  | 3.8469 | 6.0048 |  |  |  |  |
| 1997.9 | 7.2930 | 7.7849 |  |  |  | 6.6920 |  | 3.8462 | 6.0027 |  |  |  |  |
| 1999.5 | 7.2930 | 7.7835 |  |  |  | 6.6916 |  | 3.8462 | 6.0032 |  |  |  |  |
| 2000.6 | 7.2930 | 7.7841 |  |  |  | 6.6920 |  | 3.8465 | 6.0050 |  |  |  |  |
| 2002.4 | 7.2930 | 7.7838 | 7.2191 | 7.6977 | 7.5711 | 6.6920 |  | 3.8464 | 6.0046 |  |  |  |  |
| 2003.9 | 7.2930 | 7.7842 | 7.2191 | 7.6979 | 7.5714 | 6.6915 | 5.2570 | 3.8472 | 6.0046 |  | 4.5446 | 4.382 | 4.4801 |
| 2005.8 | 7.2930 | 7.7846 | 7.2193 | 7.6987 | 7.5722 | 6.6934 |  | 3.8478 | 6.0064 |  |  |  |  |
| 2005.8 | 7.2930 | 7.7851 | 7.2191 | 7.6994 | 7.5729 | 6.6939 | 5.2613 | 3.8476 | 6.0070 | 6.2096 | 4.5489 | 4.3863 | 4.4843 |
| 2007.5 | 7.2930 | 7.7857 | 7.2192 | 7.6998 | 7.5731 | 6.6944 | 5.2625 | 3.8479 | 6.0070 | 6.2107 | 4.5499 | 4.3874 | 4.4854 |
| 2009.1 | 7.2930 | 7.7847 | 7.2186 | 7.6992 | 7.5725 | 6.6935 | 5.2604 | 3.8479 | 6.0073 | 6.2089 | 4.5481 | 4.3854 | 4.4834 |
| 2010.5 | 7.2930 | 7.7860 | 7.2195 | 7.7013 | 7.5744 | 6.6961 | 5.2647 | 3.8472 | 6.0061 | 6.2133 | 4.5524 | 4.3898 | 4.4878 |
| 2012.1 | 7.2930 | 7.7846 | 7.2190 | 7.6988 | 7.5722 | 6.6935 | 5.2580 | 3.8481 | 6.0052 | 6.2063 | 4.5456 | 4.3831 | 4.4810 |

*The RL of the Reference Point NAUR (ARP) is derived from adding the static height difference of 0.9483 m (NAURBM to NAUR) to the levelled RL of NAURBM.

The 2012 RL of NAUR is 6.2063 m

## Time Series of Bench Mark Movement relative to Fixed Deep Bench Mark NAU1

Precise Differential Levelling: 1993 - 2005	
EDM Height Traversing: 2005 onwards

# Deep driven Bench Mark Locality Diagrams

# NAURBM and NAUR Reference Mark Locality Diagrams

# References

Rüeger, J.M. & Brunner, F.K. 1982, ‘EDM Height Traversing versus Geodetic Levelling’, The Canadian Surveyor, vol. 36, no. 1, pp. 69-87.

Inter-Government Committee on Surveying and Mapping (ICSM) 2002, Standards and Practices for Control Surveys, SP1, Ver. 1.5, May 2002.