EDM Height Traversing Levelling Survey Report

Manus Island, Papua New Guinea, September 2012

Geoscience Australia
Record 2014/19

S. J. K. Yates1, A. Lal2

National Geospatial Reference Systems, Geospatial and Earth Monitoring Division, Geoscience Australia GPO Box 378 Canberra ACT 2601

Secretariat of the Pacific Community (SPC)

Department of Industry

Minister for Industry: The Hon Ian Macfarlane MP
Parliamentary Secretary: The Hon Bob Baldwin MP
Secretary: Ms Glenys Beauchamp PSM

Geoscience Australia

Chief Executive Officer: Dr Chris Pigram
This paper is published with the permission of the CEO, Geoscience Australia

© Commonwealth of Australia (Geoscience Australia) 2014

With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence. (http://www.creativecommons.org/licenses/by/3.0/au/deed.en)

Geoscience Australia has tried to make the information in this product as accurate as possible. However, it does not guarantee that the information is totally accurate or complete. Therefore, you should not solely rely on this information when making a commercial decision.

Geoscience Australia is committed to providing web accessible content wherever possible. If you are having difficulties with accessing this document please email clientservices@ga.gov.au.

ISSN 2201-702X (PDF)

ISBN 978-1-925124-08-8 (PDF)

GeoCat 76786

Bibliographic reference: Yates, S. J. K. & Lal, A., 2014, EDM Height Traversing Levelling Survey Report: Manus Island, PNG, September 2012. Record 2014/19. Geoscience Australia, Canberra. http://dx.doi.org/10.11636/Record.2014.019

Contents

1 Introduction	1

2 The Survey	2

2.1 Bench Mark Locality Diagram	3

2.2 The Manus Island Datum	3

2.3 Equipment	3

2.4 Method	3

2.5 Manus Island 2012 Reduced Levels	4

2.6 Survey Support	5

2.7 Issues and Correspondence	5

3 Comparisons	6

3.1 Comparisons between 2012 and 2010 EDM Surveys	6

3.2 Combined Comparisons 1994 to 2012	7

3.3 Time Series of Bench Mark Movement relative to Fixed Deep Bench Mark PNG1	8

4 Deep Driven Bench Mark Locality Diagrams	12

5 PNGMBM and PNGM Reference Mark Locality Diagrams	18

6 Temporary Holding Mark Locality Diagrams	22

7 References	25

# Introduction

This report outlines the high precision levelling survey completed between the Sea Level Fine Resolution Acoustic Measuring Equipment (SEAFRAME) tide gauge and Continuous Global Navigation Satellite System (CGNSS) station in Manus Island, Papua New Guinea from 9th to 12th September 2012.

Personnel involved in the survey were Stephen Yates, Project Officer, Geoscience Australia and Andrick Lal, Surveyor, Secretariat of the Pacific Community (SPC).

The Electronic Distance Measuring (EDM) height traversing levelling technique was employed to observe differences in height between the deep benchmark arrays in Manus Island, which runs approximately 1.5 km from the SEAFRAME tide gauge sensor to the continuous operating reference station CGNSS antenna. Previous levelling surveys using this technique have been conducted along the route in 2006, 2007, 2009 and 2010.

In addition, precise differential levelling surveys were performed along the deep benchmark (BM) array from 1994 to 2006 by the National Tidal Centre Australia (NTCA). This report contains a comparison between the 2010 and 2009 EDM height traversing results as well as a combined comparison since the first differential levelling survey in 1994.

# The Survey

The EDM height traversing levelling survey was carried out between the SEAFRAME tide gauge sensor, continuous CGNSS station and the deep driven bench marks:

PNGMBM	– GNSS Benchmark

PNG14 	– SEAFRAME sensor benchmark

PNG1 	– Deep Driven Benchmark

PNG2 	– Deep Driven Benchmark

PNG29 	– Deep Driven Benchmark

PNG30	– Deep Driven Benchmark

PNG3 	– Deep Driven Benchmark

PNG31 	– Deep Driven Benchmark

All the deep bench marks were located and found in good order and undisturbed. Also included in the survey were temporary holding marks PNG26, PNG28, PNG34 and PNG100. The benchmark, PNG20 (Iron bolt adjacent to the Tide Gauge Hut) was found disturbed therefore it was replaced with PNG101 (stainless steel bolt in concrete – radar sensor bolt)

The EDM height traversing levelling technique was performed to the Class L2A specifications (ICSM, Standards and Practices for Control Surveys). After reduction an internal precision of 1mm√K or better was achieved (zero order); well within the specifications of the project which is 2√K where K is the distance in kilometres. A table of results and comparisons and the 2012 reduced levels are detailed later in this report.

## Bench Mark Locality Diagram

## The Manus Island Datum

The Datum for the survey is Tide Staff Zero (TSZ). Reduction of the data has been computed holding PNG1 fixed at 2.2987675 metres. This value was determined by the NTCA by adopting the Tide Staff Zero (TGZ) from the 1994 survey.

## Equipment

Leica total station model TM30 (Serial No: 361441)

Leica precision prisms GPH1P (2).

Leica rigid tripod.

Stainless steel target poles supported by Leica telescopic bi-poles (2).

Shortened stainless steel target pole for the SEAFRAME sensor BM connection.

Leica cast iron change plates (2).

Kestral 4000 pocket weather tracker

## Method

The “Leap-Frog” EDM height traversing technique was employed for the Manus Island tide gauge levelling survey. This technique involves setting up a total station (TCA1800L) midway between two target/reflectors (on reflector rods with struts). The targets remain at a particular change point for the back-sight and fore-sight observations. The instrument measures slope distances (±1mm) and vertical angle (1”) to derive height differences (between the instrument’s trunnion axis and the reflectors). In support of the slope distance observations, the ambient temperature, pressure and humidity are recorded (Kestral 4000 pocket weather tracker) and input into the instrument to apply the first velocity correction to the observed distances (Rüeger & Brunner, 1982). Four rounds of observations are taken to the back-sight and fore-sight targets from each instrument setup. All levelling runs started and finished with the same reflector and reflector rod, i.e. an even number of setups when the two reflector rod configuration was used. This eliminates any reflector rod zero error. This technique can also be performed using a single set-up / single rod configuration which is particularly useful when levelling between bench marks which are close together e.g. between the CGNSS RMs.

Reduction of the digital data was computed by the Geoscience Australia levelling program “leveling1.exe”. This program computes the height difference between the two reflectors.

## Manus Island 2012 Reduced Levels

Table 2.1 Manus Island, Papua New Guinea – Reduced levels 2012 survey
Date:	9th – 12th September 2012
Datum:	Tide Staff Zero (TSZ) (1994)

| Point ID | Reduced Level | Type |
| --- | --- | --- |
| PNG1 | 2.2988 | Stainless Steel Rod in Ground |
| PNG29 | 4.4804 | Stainless Steel Rod in Ground |
| PNG34 | 8.5051 | Stainless Steel Pin in Concrete |
| PNG26 | 26.2898 | Stainless Steel Pin in Concrete |
| PNG3 | 34.1474 | Stainless Steel Rod in Ground |
| PNG28 | 31.1332 | Stainless Steel Pin in Concrete |
| PNG31 | 33.1101 | Stainless Steel Rod in Ground |
| PNGMBM | 37.6893 | Stainless Steel Pillar Pin |
| *PNGM | 38.6505 | Pillar Plate (ARP) |
| PNG100 | 3.3066 | Stainless Steel Pin in Concrete |
| PNG2 | 2.4957 | Stainless Steel Rod in Ground |
| PNG101 | 3.2889 | Threaded Stainless Steel Bolt |
| PNG14 | 4.5790 | Stainless Steel Pin |
| PNG30 | 3.2879 | Stainless Steel Rod in Ground |
| RM1 | 35.8871 | Stainless Steel Rod in Ground |
| RM2 | 37.3137 | Stainless Steel Rod in Ground |
| RM3 | 37.6004 | Stainless Steel Rod in Ground |
| PNGM_MET | 37.1943 | Top of Met Station |

*The RL of the Reference Point PNGM (ARP) is derived from adding the static height difference of 0.9612 m (PNGMBM to PNGM) to the levelled RL of PNGMBM.

## Survey Support

The survey team very much appreciated the assistance from staff of Lombrum Naval base, Manus Provincial Administration and the Manus Island Weather Service Office.

Personnel consulted during the visit were:-

Mr. Kanawi Sindol –Assistant Administrator, Manus Provincial Administration

Mr. Piten Nama –Manus Provincial Surveyor

Mr. Hymson Waffi – Officer In Charge, Manus Island Weather Service, Momote Aiprort

Mr Alois Tinglei – Survey Assistant, Manus Provincial Survey Office

Mr Mark Mohe – Electronics Technician, Papua New Guinea Defence Force, Lombrum, Manus Island. Phone (675) 71958058 Fax (675) 9701002

All personnel consulted had a keen interest in the project and were always supportive with any project associated matters.

## Issues and Correspondence

All project personnel must ensure that prior approval is required for entry into Lombrum Naval Base (PNG Defence Force) from the Commander; the local focal point have to be notified upon every visit to arrange for clearance into the Naval Base.

All correspondence and clearance to the Site was arranged via Mr. Kanawi Sindol.

The survey equipment was freighted via DHL to Mr Hymson Waffi of Manus Island Weather Service Office.

The vehicle used in the survey for commuting from Lorengau to Lombrum was hired from Travel Car Ltd.

# Comparisons

## Comparisons between 2012 and 2010 EDM Surveys

Table 3.1 Manus Island, PNG 2012 EDM Height Traversing Levelling & Comparison 2012 - 2010. PNG1 - adopted fixed height OF 2.29877m

| From | To | Levelled Ht. Diff. | RL 2012 | Misclose (mm) | Dist. (km) | 1mmk | RL 2010 | Difference (mm) 2012 - 2010 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PNG1 |  |  | 2.29877 |  |  |  | 2.2988 |  |
| PNG29 | PNG29 | 2.18160 | 4.48037 | 0.338 | 0.178 | 0.422 | 4.4799 | 0.47 |
| PNG34 | PNG34 | 4.02472 | 8.50509 | 0.324 | 0.115 | 0.340 | 8.5011 | 3.99 |
| PNG26 | PNG26 | 17.78475 | 26.28984 | 0.050 | 0.205 | 0.453 | 26.2869 | 2.94 |
| PNG3 | PNG3 | 7.85759 | 34.14743 | 0.425 | 0.183 | 0.427 | 34.1459 | 1.53 |
| PNG28 | PNG28 | -3.01424 | 31.13318 | -0.063 | 0.105 | 0.324 | 31.128 | 5.18 |
| PNG31 | PNG31 | 1.97687 | 33.11005 | -0.038 | 0.073 | 0.271 | 33.1032 | 6.85 |
|  | PNGMBM | 4.57929 | 37.68934 | 0.150 | 0.070 | 0.265 | 37.6851 | 4.24 |
| PNG1 |  |  | 2.29877 |  |  |  | 2.2988 |  |
| PNG100 | PNG100 | 1.00784 | 3.30661 | -0.093 | 0.103 | 0.321 | 3.3073 | -0.69 |
|  | PNG2 | -0.81095 | 2.49566 | 0.325 | 0.162 | 0.402 | 2.4963 | -0.64 |
| PNG1 |  |  | 2.29877 |  |  |  | 2.2988 |  |
| PNG101 | PNG101 | 0.99009 | 3.28886 | -0.138 | 0.147 | 0.383 | NEW MARK | NEW MARK |
|  | PNG14 | 1.29015 | 4.57901 | -0.150 | 0.016 | 0.128 | 4.5801 | -1.09 |
| PNG34 |  |  | 8.50509 |  |  |  | 8.5011 |  |
|  | PNG30 | -5.21715 | 3.28794 | -0.190 | 0.050 | 0.223 | 3.2866 | 1.34 |
|  |  |  | Misclosure for all bays levelled = | 0.942 | 1.407 | 1.186 |  |  |
| PNGMBM |  |  | 37.68934 |  |  |  | 37.6851 | 4.21 |
|  | RM1 | -1.80224 | 35.88710 | 0.060 | 0.025 | 0.158 | 35.8825 | 4.63 |
| PNGMBM |  |  | 37.68934 |  |  |  | 37.6851 | 4.21 |
|  | RM2 | -0.37561 | 37.31373 | -0.018 | 0.023 | 0.153 | 37.3096 | 4.10 |
| PNGMBM |  |  | 37.68934 |  |  |  | 37.6851 | 4.21 |
|  | RM3 | -0.08892 | 37.60043 | -0.170 | 0.024 | 0.155 | 37.5958 | 4.64 |
| PNGMBM |  |  | 37.68934 |  |  |  |  |  |
|  | PNGM_MET | -0.4950 | 37.19429 | -0.050 | 0.064 | 0.253 | NEW MARK | NEW MARK |

All levelling was performed within the project specifications of 2k

## Combined Comparisons 1994 to 2012

Table 3.2: Manus Island, PNG - Comparison of the RL's for Precise Differential Levelling (1994 - 2006) and EDM Height Traversing (2006 - 2012). Units are in millimetres (mm)

| Year | PNG1 | PNG2 | PNG3 | PNG14 | PNG29 | PNG30 | PNG31 | PNGMBM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1994 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |  |  |  |  |
| 1995 | 0.0000 | -0.0300 | -1.0300 | -0.8800 |  |  |  |  |
| 1997 | 0.0000 | -0.4800 | -1.7500 | -2.6800 |  |  |  |  |
| 1998 | 0.0000 | 0.1900 | -0.9900 | -1.4100 |  |  |  |  |
| 2000 | 0.0000 | -0.0200 | -0.5300 | -1.9000 |  |  |  |  |
| 2002.4 | 0.0000 | -0.1900 | -0.5200 | -1.8600 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| 2003.7 | 0.0000 | -4.9500 | -0.3900 | -2.7900 | 0.0800 | 0.5200 | 3.3500 | 2.5400 |
| 2006.1 | 0.0000 |  | 0.5100 | -0.9400 | 0.5900 |  | 1.8300 | 1.3900 |

| Year | PNG1 | PNG2 | PNG3 | PNG14 | PNG29 | PNG30 | PNG31 | PNGMBM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2006.1 | 0.0000 |  | 1.7900 | -1.6300 | 0.8800 |  | 3.3800 | 3.3000 |
| 2007.7 | 0.0000 | -5.0500 | 1.4200 | -2.0600 | 0.3100 | 0.3800 | 3.0100 | 2.2300 |
| 2009.5 | 0.0000 | -5.0500 | 0.3200 | -1.2000 | 0.5100 | 0.4800 | 1.2100 | 0.6300 |
| 2010.9 | 0.0000 | -4.7040 | -0.1820 | -1.1605 | 0.6800 | 0.5250 | -1.6985 | -1.0415 |
| 2012.6 | 0.0000 | -5.3885 | 1.3480 | -2.2915 | 1.1815 | 1.8180 | 5.1625 | 3.1705 |

Note 1 - PNG2 was not levelled in 2006

Note 2 - Unable to locate PNG30 in 2006

*The RL of the Reference Point PNGM (ARP) is derived from adding the static height difference of 0.9612 m (PNGMBM to PNGM) to the levelled RL of PNGMBM

## Time Series of Bench Mark Movement relative to Fixed Deep Bench Mark PNG1

The purpose of this survey is twofold: firstly, to provide accurate changes in land height to be used in computations of absolute sea level rise and secondly to provide accurate assessments of relative sea level changes due to localised deformation.

Note: PNG31, PNGMBM, RM1, RM2 and RM3 – All these sites are within close proximity of each other near the CGNSS Site. Results showed a ~5mm drop since 2006, but 2012 survey have showed that these marks have rebounded to ~ 4.5mm uplift in the time series of PNG2, PNG3, PNG14, PNG29 and PNG30.

Precise Differential Levelling: 1993 - 2004	
EDM Height Traversing: 2004 onwards

# Deep Driven Bench Mark Locality Diagrams

# PNGMBM and PNGM Reference Mark Locality Diagrams

# Temporary Holding Mark Locality Diagrams

# References

Rüeger, J.M. & Brunner, F.K. 1982, ‘EDM Height Traversing versus Geodetic Levelling’, The Canadian Surveyor, vol. 36, no. 1, pp. 69-87.

Inter-Government Committee on Surveying and Mapping (ICSM) 2002, Standards and Practices for Control Surveys, SP1, Ver. 1.5, May 2002.