EDM Height Traversing Levelling Survey Report

Pohnpei, Federated States of Micronesia, August 2012

Geoscience Australia
Record 2014/21

S. J. K. Yates1, A. Lal2

National Geospatial Reference Systems, Minerals and Natural Hazards Division, Geoscience Australia GPO Box 378 Canberra ACT 2601

Secretariat of the Pacific Community (SPC)

Department of Industry

Minister for Industry: The Hon Ian Macfarlane MP
Parliamentary Secretary: The Hon Bob Baldwin MP
Secretary: Ms Glenys Beauchamp PSM

Geoscience Australia

Chief Executive Officer: Dr Chris Pigram
This paper is published with the permission of the CEO, Geoscience Australia

© Commonwealth of Australia (Geoscience Australia) 2014

With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence. (http://www.creativecommons.org/licenses/by/3.0/au/deed.en)

Geoscience Australia has tried to make the information in this product as accurate as possible. However, it does not guarantee that the information is totally accurate or complete. Therefore, you should not solely rely on this information when making a commercial decision.

Geoscience Australia is committed to providing web accessible content wherever possible. If you are having difficulties with accessing this document please email clientservices@ga.gov.au.

ISSN 2201-702X (PDF)

ISBN 978-1-925124-10-1 (PDF)

GeoCat 76787

Bibliographic reference: Yates, S. J. K. & Lal, A., 2014, EDM Height Traversing Levelling Survey Report: Pohnpei, Federated States of Micronesia, August 2012. Record 2014/21. Geoscience Australia, Canberra. http://dx.doi.org/10.11636/Record.2014.021

Contents

1 Introduction	1

2 The Survey	2

2.1 Bench Mark Locality Diagram	3

2.2 The FSM Datum	3

2.3 Equipment	3

2.4 Method	4

2.5 Pohnpei 2012 Reduced Levels	4

2.6 Survey Support	5

2.7 Issues	5

2.8 Description of Marks – Pohnpei, FSM	6

3 Comparisons	7

3.1 Comparisons between 2112 and 2011 EDM Surveys	7

3.2 Combined Comparisons 2001 to 2012	8

3.3 Time Series of Bench Mark Movement relative to Fixed Deep Bench Mark FSM1	9

4 Deep Bench Mark Locality Diagrams	12

5 Temporary Bench Mark Locality Diagrams	17

6 POHNBM and POHN Reference Mark Locality Diagrams	30

7 References	34

# Introduction

This report outlines the levelling survey completed during the visit to Pohnpei, Federated States of Micronesia (FSM) from 2nd to 9th August 2012.

Personnel involved in the survey were Steve Yates, Project Officer, Geoscience Australia and Andrick Lal, Surveyor, Secretariat of the Pacific Community (SPC).

This is the fifth Electronic Distance Measuring (EDM) height traversing levelling survey of the deep bench mark array in Pohnpei, FSM which runs approximately 3.5 km from the tide gauge sensor bench mark  Sea Level Fine Resolution Acoustic Measuring Equipment (SEAFRAME) at the wharf to the continuous GNSS (Global Navigation Satellite System) Station located in the Pohnpei Weather Office Compound. The previous levelling surveys using this technique were performed in 2006, 2008, 2009 and 2011.

Precise Differential Levelling surveys were performed in 2001 and 2003 by the National Tidal Centre Australia (NTCA) and the survey in May – June 2006 included a comparison between the precise differential levelling and EDM height traversing technique. This report contains a comparison between the 2011 and 2012 EDM height traversing results as well as a combined comparison since the first levelling survey.

# The Survey

The EDM Height Traversing levelling survey was carried out between the five deep driven bench marks; from the CGNSS Station to the SEAFRAME Station.

POHNBM	– CGNSS Benchmark

FSM55 	– SEAFRAME sensor benchmark

FSM1 	– Deep Driven Benchmark

FSM2 	– Deep Driven Benchmark

FSM3 	– Deep Driven Benchmark

FSM4 	– Deep Driven Benchmark

FSM5 	– Deep Driven Benchmark

Included in the survey was FSM17, a stainless steel bolt located adjacent to the tide gauge hut.

Also included in the survey were temporary holding marks – FSM8, FSM10, FSM11, FSM12, FSM14, FSM16, FSM25, FSM27, FSM29, FSM31, FSM34, FSM100, FSM44, FSM47, FSM51 and the three CGNSS reference marks - RM1, RM2 and RM3. Temporary holding marks, FSM101, FSM102, FSM103 and FSM104 were placed during this survey, consisting of a stainless steel bolt drilled in concrete and then glued in place with quality epoxy resin. The following temporary holding marks, FSM36, FSM38, FSM43 and FSM49 were destroyed and not found.

All the deep bench marks were located and found in good order.

The EDM height traversing levelling technique was performed to the Class L2A specifications (ICSM, 2002). After reduction an internal precision of 1mm √K or better was achieved, where K is distance in kilometres. This is well within the project specification of 2mm √K. A table of results and comparisons and the 2012 reduced levels are detailed later in this report.

## Bench Mark Locality Diagram

## The FSM Datum

Reduction of the data has been calculated holding FSM1 fixed at 2.4382 metres.

This value was determined by the NTCA by:

2001 Adopting the height of PNIA; RL = 2.00000 metres (Arbitrary Datum)

2003 Adopting the height of FSM1 as derived in the 2001 survey

## Equipment

Leica total station model TM30 (Serial No: 361441)

Leica precision prisms GPH1P (2).

Leica rigid tripod.

Stainless steel target poles supported by Leica telescopic bi-poles (2).

Shortened stainless steel target pole for the SEAFRAME sensor BM connection.

Leica cast iron change plates (2).

Kestral 4000 pocket weather tracker

## Method

The “Leap-Frog” EDM height traversing technique was employed for the Pohnpei tide gauge levelling survey. This technique involves setting up a total station (TCA1800L) midway between two target/reflectors (on reflector rods with struts). The targets remain at a particular change point for the back-sight and fore-sight observations. The instrument measures slope distances (±1mm) and vertical angle (1”) to derive height differences (between the instrument’s trunnion axis and the reflectors). In support of the slope distance observations, the ambient temperature, pressure and humidity are recorded (Kestral 4000 pocket weather tracker) and input into the instrument to apply the first velocity correction to the observed distances (Rüeger & Brunner, 1982). Four rounds of observations are taken to the back-sight and fore-sight targets from each instrument setup. All levelling runs started and finished with the same reflector and reflector rod, i.e. an even number of setups when the two reflector rod configuration was used. This eliminates any reflector rod zero error. This technique can also be performed using a single set-up / single rod configuration which is particularly useful when levelling between bench marks which are close together e.g. between the CGNSS RMs.

Reduction of the digital data was computed by the Geoscience Australia levelling program “leveling1.exe”. This program computes the height difference between the two reflectors.

## Pohnpei 2012 Reduced Levels

Table 2.1 Pohnpei, FSM – Reduced levels from the 2013 survey
Date:	4th to 8th August 2012
Datum:	Arbitrary

| Point ID | Reduced Level 2012 | Type |
| --- | --- | --- |
| FSM1 | 2.4382 | Stainless Steel Rod in Ground |
| FSM17 | 3.0719 | Stainless Steel Pin in Concrete |
| FSM55 | 4.0319 | Stainless Steel Sensor Pin |
| FSM2 | 2.3421 | Stainless Steel Rod in Ground |
| FSM8 | 2.3679 | Stainless Steel Pin in Concrete |
| FSM10 | 2.0059 | Stainless Steel Pin in Concrete |
| FSM3 | 2.0677 | Stainless Steel Rod in Ground |
| FSM11 | 2.8590 | Stainless Steel Pin in Concrete |
| FSM12 | 2.7860 | Stainless Steel Pin in Concrete |
| FSM14 | 2.6041 | Stainless Steel Pin in Concrete |
| FSM16 | 2.4184 | Stainless Steel Pin in Concrete |
| FSM4 | 1.7543 | Stainless Steel Rod in Ground |
| FSM25 | 2.3944 | Stainless Steel Pin in Concrete |
| FSM27 | 2.5435 | Stainless Steel Pin in Concrete |
| FSM29 | 1.9428 | Metal Pin in ground |
| FSM31 | 3.0779 | Metal Pin in ground |
| FSM34 | 6.4778 | Stainless Steel Pin in Concrete |
| FSM101 | 14.7279 | Stainless Steel Pin in Concrete |
| FSM102 | 20.4777 | Stainless Steel Pin in Concrete |
| FSM5 | 20.5927 | Stainless Steel Rod in Ground |
| FSM100 | 21.8480 | Stainless Steel Pin in Concrete |
| FSM103 | 21.6609 | Stainless Steel Pin in Concrete |
| FSM44 | 22.6846 | Stainless Steel Pin in Concrete |
| FSM47 | 25.6969 | Masonry Nail in Concrete |
| FSM104 | 29.1914 | Stainless Steel Pin in Concrete |
| FSM51 | 37.5510 | Masonry Nail in Concrete |
| POHNBM | 38.0080 | Stainless Steel Pillar Pin |
| RM1 | 37.4375 | Stainless Steel Rod in Ground |
| RM2 | 36.7847 | Stainless Steel Rod in Ground |
| RM | 37.1586 | Stainless Steel Rod in Ground |
| NWSBM | 37.5037 | Top of Weather Sensor |

*The RL of the Reference Point POHN (ARP) is derived from adding the static height difference of 0.9520 m (POHNBM to POHN) to the levelled RL of POHNBM.

## Survey Support

The survey team very much appreciated the assistance from staff of the Pohnpei State Weather Office and SPC North Regional Office namely:

Personnel consulted during the visit were:

Mr. Eden Skilling – OIC, Pohnpei State Weather Office

Mr. Toshio Akinaga – Weather Officer, Pohnpei State Weather Office

Mr. Amena Yauvoli – SPC North Regional Office, Pohnpei

Ms Miyuki Norman – SPC North Regional Office, Pohnpei

All personnel consulted had a keen interest in the project and were always supportive with any project associated matters.

## Issues

Due to a recent change in field procedures, a labourer, Nixon David of Nett Village was hired to carry the Leica Total Station during the survey. This was arranged through the kind assistance of Jun Tamps, General Manager, Pacific Foods and Services Ltd – Phone (691) 320 8895.

To have access to the Pohnpei tide gauge, prior arrangement should be made with Fisheries.

## Description of Marks – Pohnpei, FSM

FSM1		- Deep Driven bench mark (BM) located adjacent to the Caroline Fisheries Bldg

FSM17	- Stainless Steel bolt in concrete below the SEAFRAME sensor bench mark

FSM55	- SEAFRAME Sensor Bench Mark

FSM2		- Deep Driven BM located next to the Continental Air Cargo depot.

FSM8		- Stainless Steel Pin near the Pohnpei airport entrance gate.

FSM10	- Stainless Steel Pin at the Fisheries Corporation compound entrance.

FSM3		- Deep driven BM located within the Fisheries Corporation compound.

FSM11	- Stainless Steel Pin located along the Deketik Causeway Road

FSM12	- Stainless Steel Pin located along the Deketik Causeway Road.

FSM14	- Stainless Steel Pin located along the Deketik Causeway Road.

FSM16	- Stainless Steel Pin located along the Deketik Causeway Road.

FSM4		- Deep Driven BM located along the Deketik Causeway Road.

FSM25	- Stainless Steel Pin located along the Deketik Causeway Road.

FSM27	- Stainless Steel Pin located along the Deketik Causeway Road.

FSM29	- Metal Pin located along the Deketik Causeway Road.

FSM31	- Metal Pin located along the Deketik Causeway Road

FSM34	- Stainless Steel Pin located corner Deketik Causeway Road and Kaselehlie Street.

FSM101	- Stainless Steel Pin located on a sewer manhole along Kaselehlie Street.

FSM102	- Stainless Steel Pin located on top of a concrete culvert along Kaselehlie Street.

FSM5		- Deep Driven BM located in the grassed area next to the Spanish Wall.

FSM100	- New stainless steel bolt located along Kaselehlie Street.

FSM103	- Stainless Steel Pin located on top of a concrete culvert along Kaselehlie Street.

FSM44	- Stainless Steel Pin located in front of A-one Fashion shop along Kaselehlie Street.

FSM47	- Masonry nail located at the end of guard rail along Kaselehlie Street.

FSM104	- Stainless Steel Pin near the Namiki Restaurant along Kaselehlie Street.

FSM51	- Pin near the north east corner to the weather office along Kaselehlie Street.

POHNBM 	- CGNSS Benchmark

POHN 	- CGNSS Reference Point

RM1, 2 & 4	- CGNSS Reference Mark

# Comparisons

## Comparisons between 2112 and 2011 EDM Surveys

Table 3.1 Pohnpei, FSM EDM Height Traversing Levelling Comparison 2012 - 2011. FSM1 - adopted fixed height of 2.43822m.

| From | To | Levelled Ht. Diff. | RL 2012 | Misclose (mm) | Dist. (km) | 1mmk | RL 2011 | Difference (mm) 2011 - 2012 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FSM1 |  |  | 2.4382 |  |  |  |  |  |
| FSM17 | FSM17 | 0.63366 | 3.0719 | 0.3833 | 0.1096 | 0.331 | 3.0713 | 0.58 |
|  | FSM55 | 0.95997 | 4.0319 | -0.0233 | 0.0213 | 0.146 | 4.0313 | 0.55 |
| FSM1 |  |  | 2.4382 |  |  |  |  |  |
| FSM2 | FSM2 | -0.09614 | 2.3421 | 0.0875 | 0.2501 | 0.500 | 2.3420 | 0.08 |
| FSM8 | FSM8 | 0.02584 | 2.3679 | -0.3125 | 0.1206 | 0.347 | 2.3682 | -0.28 |
| FSM10 | FSM10 | -0.36198 | 2.0059 | 0.3125 | 0.2324 | 0.482 | 2.0092 | -3.26 |
|  | FSM3 | 0.06175 | 2.0677 | 0.2250 | 0.0809 | 0.285 | 2.0683 | -0.62 |
| FSM10 |  |  | 2.0059 |  |  |  |  |  |
| FSM11 | FSM11 | 0.85303 | 2.8590 | 0.2875 | 0.1744 | 0.418 | 2.8590 | -0.04 |
| FSM12 | FSM12 | -0.07297 | 2.7860 | -0.0542 | 0.0929 | 0.305 | 2.7869 | -0.90 |
| FSM14 | FSM14 | -0.18191 | 2.6041 | 0.2625 | 0.1813 | 0.426 | 2.6079 | -3.81 |
| FSM16 | FSM16 | -0.18569 | 2.4184 | -0.1625 | 0.1839 | 0.429 | 2.4245 | -6.10 |
| FSM4 | FSM4 | -0.66415 | 1.7543 | -0.1050 | 0.0993 | 0.315 | 1.7546 | -0.35 |
| FSM25 | FSM25 | 0.64010 | 2.3944 | 0.1075 | 0.0942 | 0.307 | 2.3963 | -1.95 |
| FSM27 | FSM27 | 0.14914 | 2.5435 | 0.3792 | 0.1818 | 0.426 | 2.5442 | -0.71 |
| FSM29 | FSM29 | -0.60068 | 1.9428 | 0.3375 | 0.1838 | 0.429 | 1.9445 | -1.69 |
| FSM31 | FSM31 | 1.13508 | 3.0779 | -0.1875 | 0.1827 | 0.427 | 3.0784 | -0.51 |
| FSM34 | FSM34 | 3.39989 | 6.4778 | 0.3875 | 0.1558 | 0.395 | 6.4756 | 2.19 |
| FSM101 | FSM101 | 8.25008 | 14.7279 | 0.1625 | 0.0847 | 0.291 | New Mark | New Mark |
| FSM102 | FSM102 | 5.74987 | 20.4777 | -0.0125 | 0.0656 | 0.256 | New Mark | New Mark |
| FSM5 | FSM5 | 0.11496 | 20.5927 | 0.3025 | 0.0982 | 0.313 | 20.5923 | 0.40 |
| FSM100 | FSM100 | 1.25532 | 21.8480 | -0.3875 | 0.1791 | 0.423 | 21.8471 | 0.92 |
| FSM103 | FSM103 | -0.18717 | 21.6609 | -0.0125 | 0.0651 | 0.255 | New Mark | New Mark |
| FSM44 | FSM44 | 1.02377 | 22.6846 | 0.2075 | 0.1011 | 0.318 | 22.6839 | 0.72 |
| FSM47 | FSM47 | 3.01233 | 25.6969 | -0.0458 | 0.1968 | 0.444 | 25.6974 | -0.45 |
| FSM104 | FSM104 | 3.49444 | 29.1914 | -0.1000 | 0.0827 | 0.288 | New Mark | New Mark |
| FSM51 | FSM51 | 8.35962 | 37.5510 | 0.0625 | 0.1075 | 0.328 | 37.5502 | 0.80 |
| POHNBM | POHNBM | 0.45702 | 38.0080 | -0.3250 | 0.1415 | 0.376 | 38.0076 | 0.43 |
|  | NWSBM | -0.50432 | 37.5037 | 0.0125 | 0.0207 | 0.144 | New Mark | New Mark |
|  |  |  | Misclose for all bays levelled = | 1.789 | 3.488 | 1.868 |  |  |
| POHNBM |  |  | 38.0080 |  |  |  |  |  |
|  | RM1 | -0.57050 | 37.4375 | 0.1083 | 0.0303 | 0.174 | 37.4377 | -0.14 |
| POHNBM |  |  |  |  |  |  |  |  |
|  | RM2 | -1.22334 | 36.7847 | -0.0200 | 0.0214 | 0.146 | 36.7843 | 0.35 |
| POHNBM |  |  |  |  |  |  |  |  |
|  | RM4 | -0.84939 | 37.1586 | 0.0125 | 0.0216 | 0.147 | New Mark | New Mark |

All levelling was performed within the project specifications of 2k

## Combined Comparisons 2001 to 2012

Examination of the level survey results indicate that deep driven bench mark FSM3 is sinking at a rate of 2.3mm per year, relative to FSM1.

Table 3.2 Pohnpei, FSM - Comparison of the RL's for Precise Differential Levelling (2001 - 2006) and EDM Height Traversing (2006 - 2012).

| Year | FSM1 | FSM2 | FSM3 | FSM4 | FSM5 | FSM55 | FSM17 | POHNBM | *POHN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2001.95 | 2.4382 | 2.3417 | 2.0875 | 1.7553 | 20.5905 |  | 3.0757 |  |  |
| 2003.16 | 2.4382 | 2.3415 | 2.0816 | 1.7538 | 20.5881 | 4.0326 | 3.0750 | 38.0022 |  |
| 2006.41 | 2.4382 | 2.3419 | 2.0746 | 1.7554 | 20.5935 | 4.0330 | 3.0740 | 38.0100 |  |

| Year | FSM1 | FSM2 | FSM3 | FSM4 | FSM5 | FSM55 | FSM17 | POHNBM | *POHN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2006.42 | 2.4382 | 2.3426 | 2.0760 | 1.7557 | 20.5946 | 4.0314 | 3.0732 | 38.0091 | 38.9609 |
| 2008.17 | 2.4382 | 2.3415 | 2.0720 | 1.7540 | 20.5909 | 4.0305 | 3.0723 | 38.0060 | 38.9580 |
| 2009.41 | 2.4382 | 2.3416 | 2.0695 | 1.7538 | 20.5924 | 4.0316 | 3.0720 | 38.0091 | 38.9612 |
| 2011.14 | 2.4382 | 2.3420 | 2.0683 | 1.7546 | 20.5923 | 4.0313 | 3.0713 | 38.0076 | 38.9597 |
| 2012.59 | 2.4382 | 2.3421 | 2.0677 | 1.7543 | 20.5927 | 4.0319 | 3.0719 | 38.0080 | 38.9600 |

## Time Series of Bench Mark Movement relative to Fixed Deep Bench Mark FSM1

Precise Differential Levelling: 2001 - 2006	
EDM Height Traversing: 2006 onwards

# Deep Bench Mark Locality Diagrams

# Temporary Bench Mark Locality Diagrams

# POHNBM and POHN Reference Mark Locality Diagrams

# References

Rüeger, J.M. & Brunner, F.K. 1982, ‘EDM Height Traversing versus Geodetic Levelling’, The Canadian Surveyor, vol. 36, no. 1, pp. 69-87.

Inter-Government Committee on Surveying and Mapping (ICSM) 2002, Standards and Practices for Control Surveys, SP1, Ver. 1.5, May 2002.