EDM Height Traversing Levelling Survey Report

Apia, Western Samoa, November 2011

Geoscience Australia
Record 2014/24

S. J. K. Yates1, A. Lal2

National Geospatial Reference Systems, Geospatial and Earth Monitoring Division, Geoscience Australia GPO Box 378 Canberra ACT 2601

Secretariat of the Pacific Community (SPC)

Department of Industry

Minister for Industry: The Hon Ian Macfarlane MP
Parliamentary Secretary: The Hon Bob Baldwin MP
Secretary: Ms Glenys Beauchamp PSM

Geoscience Australia

Chief Executive Officer: Dr Chris Pigram
This paper is published with the permission of the CEO, Geoscience Australia

© Commonwealth of Australia (Geoscience Australia) 2014

With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence. (http://www.creativecommons.org/licenses/by/3.0/au/deed.en)

Geoscience Australia has tried to make the information in this product as accurate as possible. However, it does not guarantee that the information is totally accurate or complete. Therefore, you should not solely rely on this information when making a commercial decision.

Geoscience Australia is committed to providing web accessible content wherever possible. If you are having difficulties with accessing this document please email clientservices@ga.gov.au.

ISSN 2201-702X (PDF)

ISBN 978-1-925124-13-2 (PDF)

GeoCat 76788

Bibliographic reference: Yates, S. J. K. & Lal, A., 2014, EDM Height Traversing Levelling Survey Report: Apia, Western Samoa, November 2011. Record 2014/24. Geoscience Australia, Canberra. http://dx.doi.org/10.11636/Record.2014.024

Contents

1 Introduction	1

2 The Survey	2

2.1 Bench Mark Locality Diagram	3

2.2 The Western Samoa Datum	4

2.3 Equipment	4

2.4 Method	4

2.5 Survey Support	4

2.6 Issues	5

3 Comparisons	6

3.1 Comparisons between 2011 and 2010 Surveys	6

3.2 Combined Comparisons 1993 to 2011	7

3.3 Time Series of Bench Mark movement relative to Fixed Deep Bench Mark BM201	8

4 Deep Bench Mark Locality Diagrams	14

5 Western Samoa Reference Mark Locality Diagram	21

6 Temporary Holding Mark Locality Diagrams	22

7 References	32

# Introduction

This report outlines the high precision level survey completed between the Sea Level Fine Resolution Acoustic Measuring Equipment (SEAFRAME) tide gauge Bench Mark and the Continuous Global Navigation Satellite System (CGNSS) station in Apia, Western Samoa from 1 – 13 November 2011.

Personnel involved in the survey were Steve Yates, Project Officer, Geoscience Australia and Andrick Lal, Surveyor, Secretariat of the Pacific Community (SPC).

The Electronic Distance Measuring (EDM) height traversing levelling technique was employed to observe differences in height between the deep bench mark array in Apia, which runs approximately 5.5 km from the SEAFRAME tide gauge sensor to the CGNSS antenna. Previous levelling surveys have been conducted along the route using this technique in 2006, 2008 and 2010.

In addition, precise differential levelling surveys were performed along the deep bench mark array from 1992 to 2004 by the National Tidal Centre Australia (NTCA). This report contains a comparison between the 2011 and 2010 EDM height traversing results as well as a combined comparison since the first levelling survey.

# The Survey

The EDM height traversing levelling survey was carried out between the SEAFRAME tide gauge sensor, CGNSS station and the deep driven bench mark array:

SAM16	 - Project plaque at tide gauge

SAM17	 - SEAFRAME sensor benchmark

BM201	 - Deep driven BM, held fixed and located at the rear of the old Ministry of Resources, Energy and Environment Building along the Apia Port Road.

BM210	 - Deep driven BM along Matafagatele Street inside Church grounds.

BM212	 - Deep driven BM located at the end of Matafagatele Street and the start of the Main East Coast Road.

BM213	 - Deep driven BM outside a shop at the corner of the Main East Coast Road and Vaivase Road.

BM214	 - Deep Driven BM located corner of Vaivase and Fuaiupolu Roads.

BM215	 - Deep Driven BM located inside church grounds, Plantation Road.

BM220	 - Deep Driven BM located in front of Apia Park entrance gate

All the deep bench marks were located and found in good order and undisturbed. Also included in the survey were the permanent holding marks SAM 104, SAM 139, SAM 166, SAM 168, SAM 170, SAM 180, SAM 186, SAM 187, SAM 300, SAM301, SAM 303, SAM 304, SAM 305, SAM 307, SAM 308, SAM 309, SAM 310, SAM 311, SAM 312, SAM 313, SAM 314, SAM 315, SAM 316. SAM 310 – 316 are new holding marks established in 2011 and consist of domed stainless steel bolts drilled in concrete and glued in place with quality epoxy resin.

The EDM height traversing levelling technique was performed to the Class L2A specifications (ICSM, Standards and Practices for Control Surveys). After reduction an internal precision of 1mm√K or better was achieved; well within the specifications of the project which is 2√K where K is the distance in kilometres. A table of results and comparisons and the 2011 reduced levels are detailed later in this report.

## Bench Mark Locality Diagram

## The Western Samoa Datum

The Datum for the survey is Mean Sea Level (1993) as established by the National Tidal Centre Australia. Reduction of the data has been calculated holding BM201 fixed at 1.3292 m.

## Equipment

Leica total station model TCA1800

Leica precision prisms GPH1P (2).

Leica rigid tripod.

Stainless steel target poles supported by Leica telescopic bi-poles (2).

Shortened stainless steel target pole for the SEAFRAME sensor BM connection.

Leica cast iron change plates (2).

Kestral 4000 pocket weather tracker

## Method

The “Leap-Frog” EDM height traversing technique was employed for the Apia tide gauge levelling survey. This technique involves setting up a total station (TCA1800L) midway between two target/reflectors (on reflector rods with struts). The targets remain at a particular change point for the back-sight and fore-sight observations. The instrument measures slope distances (±1mm) and vertical angle (1”) to derive height differences (between the instrument’s trunnion axis and the reflectors). In support of the slope distance observations, the ambient temperature, pressure and humidity are recorded (Kestral 4000 pocket weather tracker) and input into the instrument to apply the first velocity correction to the observed distances (Rüeger & Brunner, 1982). Four rounds of observations are taken to the back-sight and fore-sight targets from each instrument setup. All levelling runs started and finished with the same reflector and reflector rod, i.e. an even number of setups when the two reflector rod configuration was used. This eliminates any reflector rod zero error. This technique can also be performed using a single set-up / single rod configuration which is particularly useful when levelling between bench marks which are close together e.g. between the CGNSS RMs.

Reduction of the digital data was computed by the Geoscience Australia levelling program “leveling1.exe”. This program computes the height difference between the two reflectors.

## Survey Support

Ms. Siosina Lui from the Ministry of Natural Resources & Environment provided valuable assistance in obtaining customs clearances for the surveying equipment. This is a lengthy process and seems to becoming more time consuming throughout most countries in the Pacific. Her assistance is greatly appreciated.

Senior Surveyor, Mr. Ueligitone Seiuli gave us valuable assistance and organised for a survey assistant to assist us for the levelling.

Other personnel consulted during the visit were Mr. Austelaia Titimaea (Assistant CEO) and Me Sale from the Meteorology Division of the Ministry of Natural Resources & Environment.

## Issues

Avoid levelling between SAM166 and the tide gauge bench mark during offloading of ships or increased port activity. Access inside the Ports Authority area is strictly prohibited and arrange security pass through the Weather Office or the Lands and Surveys staff well in advance. The Ports Authority area is closed on Saturdays, however, security is still on the gate. This is a good day to arrange to perform the survey.

Levelling along the reclaimed area around the wharf and along Matafagatele Street, especially around Apia Park Stadium should be carried out during quiet traffic periods. It is recommended to avoid areas with drainage systems underneath the footpath in that area as the heavy trucks which drive past cause significant ground shake.

# Comparisons

## Comparisons between 2011 and 2010 Surveys

Table 3.1 Apia, Western Samoa 2010 EDM Height Traversing Levelling Comparison 2011 - 2010. BM201 - adopted fixed height of 1.3292 m

| From | To | Levelled Ht. Diff. | RL 2012 | Misclose (mm) | Dist. (km) | 1mmk | RL 2011 | Difference (mm) 2012 - 2011 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BM201 |  |  | 1.3292 |  |  |  | 1.3292 | 0.0000 |
| SAM186 | SAM186 | 0.99194 | 2.3211 | 0.10 | 0.094 | 0.307 | 2.3215 | -0.0004 |
| SAM166 | SAM166 | -0.10283 | 2.2183 | 0.38 | 0.190 | 0.436 | 2.2189 | -0.0006 |
| SAM309 | SAM309 | -0.27260 | 1.9457 | 0.10 | 0.186 | 0.431 |  |  |
| SAM16 | SAM16 | 0.42758 | 2.3733 | 0.12 | 0.052 | 0.228 | 2.3742 | -0.0009 |
|  | SAM17 | 1.78467 | 4.1580 | 0.03 | 0.011 | 0.105 | 4.1610 | -0.0030 |
| SAM186 |  |  | 2.3211 |  |  |  | 2.3215 | -0.0004 |
| SAM180 | SAM180 | -0.12721 | 2.1939 | 0.23 | 0.103 | 0.321 |  |  |
| SAM104 | SAM104 | -0.05781 | 2.1361 | 0.23 | 0.103 | 0.321 |  |  |
| SAM168 | SAM168 | -0.35291 | 1.7832 | 0.35 | 0.141 | 0.375 | 1.7854 | -0.0022 |
| SAM307 | SAM307 | -0.05641 | 1.7268 | 0.41 | 0.213 | 0.462 |  |  |
|  | BM210 | -0.47484 | 1.2519 | 0.00 | 0.017 | 0.130 | 1.2537 | -0.0018 |
| SAM307 |  |  | 1.7268 |  |  |  |  |  |
| SAM311 | SAM311 | 0.36212 | 2.0889 | 0.41 | 0.203 | 0.451 | New Mark | New Mark |
| SAM170 | SAM170 | -0.14476 | 1.9442 | -0.04 | 0.154 | 0.392 | 1.9481 | -0.0039 |
| SAM300 | SAM300 | -0.48723 | 1.4569 | 0.43 | 0.199 | 0.446 | 1.4661 | -0.0092 |
| BM220 | BM220 | 0.06113 | 1.5181 | 0.19 | 0.184 | 0.429 | 1.5221 | -0.0040 |
| SAM301 | SAM301 | 0.09700 | 1.6151 | 0.03 | 0.206 | 0.454 | 1.6198 | -0.0047 |
| SAM187 | SAM187 | -0.23940 | 1.3757 | -0.28 | 0.192 | 0.438 | 1.3816 | -0.0059 |
| SAM312 | SAM312 | 0.03410 | 1.4098 | 0.39 | 0.201 | 0.448 | New Mark | New Mark |
| SAM308 | SAM308 | 0.08494 | 1.4947 | 0.30 | 0.101 | 0.318 |  |  |
| BM212 | BM212 | -0.11359 | 1.3811 | -0.01 | 0.145 | 0.381 | 1.3884 | -0.0073 |
| SAM303 | SAM303 | 0.70545 | 2.0866 | -0.22 | 0.212 | 0.460 | 2.0938 | -0.0072 |
| SAM313 | SAM313 | -0.32558 | 1.7610 | 0.43 | 0.205 | 0.453 | New Mark | New Mark |
| BM213 | BM213 | -0.58191 | 1.1791 | 0.28 | 0.204 | 0.452 | 1.1865 | -0.0074 |
| SAM304 | SAM304 | 0.51478 | 1.6938 | 0.07 | 0.076 | 0.276 | 1.7018 | -0.0080 |
| SAM314 | SAM314 | 1.95802 | 3.6519 | 0.11 | 0.200 | 0.447 | New Mark | New Mark |
| BM214 | BM214 | 4.46246 | 8.1143 | 0.25 | 0.172 | 0.415 | 8.1217 | -0.0074 |
| SAM305 | SAM305 | 1.88784 | 10.0022 | -0.04 | 0.145 | 0.381 | 10.0099 | -0.0077 |
| SAM315 | SAM315 | -0.13704 | 9.8651 | 0.25 | 0.184 | 0.429 | New Mark | New Mark |
| SAM139 | SAM139 | 7.79541 | 17.6605 | 0.30 | 0.156 | 0.395 | 17.6682 | -0.0077 |
| BM215 | BM215 | 3.94376 | 21.6043 | -0.28 | 0.092 | 0.303 | 21.6122 | -0.0079 |
| SAM310 | SAM310 | 8.15688 | 29.7612 | 0.14 | 0.166 | 0.407 | 29.7693 | -0.0081 |
| SAM316 | SAM316 | 0.53941 | 30.3006 | -0.35 | 0.224 | 0.473 | New Mark | New Mark |
|  | SAMOBM | 7.88299 | 38.1836 | 0.05 | 0.213 | 0.462 | 38.1914 | -0.0078 |
|  |  |  | Misclose for all bays levelled = | 3.63 | 4.411 | 4.190 |  |  |
| SAMOBM |  |  | 38.1836 |  |  |  | 38.1914 |  |
|  | RM1 | -1.7312 | 36.4524 | 0.06 | 0.017 | 0.130 | 36.4602 | -0.0078 |
| SAMOBM |  |  |  |  |  |  |  |  |
|  | RM4 | -0.7821 | 37.4015 | 0.02 | 0.025 | 0.158 | 37.4090 | -0.0075 |
| SAMOBM |  |  |  |  |  |  |  |  |
|  | RM3 | -1.2222 | 36.9614 | -0.02 | 0.028 | 0.167 | 36.9690 | -0.0076 |

## Combined Comparisons 1993 to 2011

Table 3.2 Apia, Western Samoa - Comparison of the RL's for Precise Differential Levelling (1993 - 2006) and EDM Height Traversing (2006 - 2011). Units are in metres.

| Year | BM201 | SAM16 | SAM17 | BM210 | BM220 | BM211 | BM212 | BM213 | BM214 | BM215 | SAMOBM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1993.1 | 1.3292 | 2.3843 | 4.1769 |  |  |  |  |  |  |  |  |
| 1994.8 | 1.3292 | 2.3812 | 4.1719 |  |  |  |  |  |  |  |  |
| 1996.4 | 1.3292 | 2.3796 | 4.1696 |  |  |  |  |  |  |  |  |
| 1998.4 | 1.3292 | 2.3783 | 4.1685 |  |  |  |  |  |  |  |  |
| 1999.8 | 1.3292 | 2.3777 | 4.1675 |  |  |  |  |  |  |  |  |
| 2001.4 | 1.3292 | 2.3768 | 4.1665 |  |  |  |  |  |  |  |  |
| 2002.9 | 1.3292 | 2.3751 | 4.1634 | 1.2535 |  | 1.4287 | 1.3846 | 1.1826 | 8.1172 | 21.6076 | 38.1869 |
| 2004.7 | 1.3292 | 2.3757 | 4.1627 | 1.2538 |  | 1.4297 | 1.3864 | 1.1842 | 8.1189 | 21.6103 | 38.1889 |
| 2006.8 | 1.3292 | 2.3739 | 4.162 | 1.2546 |  | 1.4310 | 1.3884 | 1.1874 | 8.1223 | 21.6130 | 38.1926 |
| 2006.8 | 1.3292 | 2.3751 | 4.1629 | 1.2557 |  | 1.4321 | 1.3898 | 1.1876 | 8.1218 | 21.6128 | 38.1921 |
| 2008.3 | 1.3292 | 2.3744 | 4.1613 | 1.2542 | 1.5237 | destroyed | 1.3880 | 1.1861 | 8.1215 | 21.6131 | 38.1919 |
| 2010.5 | 1.3292 | 2.3742 | 4.1610 | 1.2537 | 1.5221 | destroyed | 1.3884 | 1.1865 | 8.1217 | 21.6122 | 38.1914 |
| 2011.9 | 1.3292 | 2.3733 | 4.1580 | 1.2519 | 1.5181 | destroyed | 1.3811 | 1.1791 | 8.1143 | 21.6043 | 38.1836 |

*The RL of the Reference Point SAMO (ARP) is derived from adding the static height difference of 0.7282 m (SAMOBM to SAMO) to the levelled RL of SAMOBM.

**The 2011 RL of SAMO is 38.9118 m

## Time Series of Bench Mark movement relative to Fixed Deep Bench Mark BM201

There is a 3mm drop in SAM17 and if you note the graph below there has been a gradual drop in SAM17 since 1995. This could be caused by the ports area being reclaimed land; but this theory has yet to be confirmed. The holding mark SAM300 has dropped a further 9mm since the 2010 survey and indicates that it is an unstable mark and should be used as a temporary mark only.

**Although the internal accuracy of the 2011 levelling survey was well within the project specifications it is NOT advised to use the 2011 RL’s form holding mark SAM312 to the SAMOBM. From SAM312 an error has been detected in the survey of 7 to 8mm, and indicates an unexplained movement of the newly established mark SAM312 after closure of the levelling bay SAM187 to SAM312, this has resulted in the error being carried through to the subsequent levelling bays.

The purpose of these surveys is twofold: firstly, to provide accurate changes in land height to be used in computations of absolute sea level rise and secondly to provide accurate assessments of relative sea level rise. This is an excellent example to highlight the importance of measuring land height variations and the impact they can have on absolute sea level rise measurements. Furthermore, it provides useful information regarding localised deformation and the potential relative sea level height impacts.

Precise Differential Levelling: 1995 - 2006	
EDM Height Traversing: 2006 onwards

# Deep Bench Mark Locality Diagrams

# Western Samoa Reference Mark Locality Diagram

# Temporary Holding Mark Locality Diagrams

# References

Rüeger, J.M. & Brunner, F.K. 1982, ‘EDM Height Traversing versus Geodetic Levelling’, The Canadian Surveyor, vol. 36, no. 1, pp. 69-87.

Inter-Government Committee on Surveying and Mapping (ICSM) 2002, Standards and Practices for Control Surveys, SP1, Ver. 1.5, May 2002.