EDM Height Traversing Levelling Survey Report

Apia, Western Samoa, May 2013

Geoscience Australia
Record 2014/25

S. J. K. Yates1, A. Lal2

National Geospatial Reference Systems, Geospatial and Earth Monitoring Division, Geoscience Australia GPO Box 378 Canberra ACT 2601

Secretariat of the Pacific Community (SPC)

Department of Industry

Minister for Industry: The Hon Ian Macfarlane MP
Parliamentary Secretary: The Hon Bob Baldwin MP
Secretary: Ms Glenys Beauchamp PSM

Geoscience Australia

Chief Executive Officer: Dr Chris Pigram
This paper is published with the permission of the CEO, Geoscience Australia

© Commonwealth of Australia (Geoscience Australia) 2014

With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence. (http://www.creativecommons.org/licenses/by/3.0/au/deed.en)

Geoscience Australia has tried to make the information in this product as accurate as possible. However, it does not guarantee that the information is totally accurate or complete. Therefore, you should not solely rely on this information when making a commercial decision.

Geoscience Australia is committed to providing web accessible content wherever possible. If you are having difficulties with accessing this document please email clientservices@ga.gov.au.

ISSN 2201-702X (PDF)

ISBN 978-1-925124-14-9 (PDF)

GeoCat 76789

Bibliographic reference: Yates, S. J. K. & Lal, A., 2014, EDM Height Traversing Levelling Survey Report: Apia, Western Samoa, May 2013. Record 2014/25. Geoscience Australia, Canberra. http://dx.doi.org/10.11636/Record.2014.025

Contents

1 Introduction	1

2 The Survey	2

2.1 Bench Mark Locality Diagram	3

2.2 The Western Samoa Datum	3

2.3 Equipment	3

2.4 Method	4

2.5 Samoa 2013 Reduced Levels	4

2.6 Survey Support	6

2.7 Issues	6

3 Comparisons	7

3.1 Comparisons between 2013 and 2011 EDM Surveys	7

3.2 Combined Comparisons 1993 to 2013	9

3.3 Time Series of Bench Mark Movement relative to Fixed Deep Bench Mark BM201	10

4 Deep Bench Mark Locality Diagrams	15

5 Western Samoa Reference Mark Locality Diagram	22

6 Permanent Holding Mark Locality Diagrams	23

7 References	36

# Introduction

This report outlines the high precision level survey completed between the Sea Level Fine Resolution Acoustic Measuring Equipment (SEAFRAME) tide gauge and the Continuous Global Navigational Satellite System (CGNSS) station in Apia, Western Samoa from 21st May to 2nd June 2013.

Personnel involved in the survey were Steve Yates, Project Officer, Geoscience Australia and Andrick Lal, Surveyor, Secretariat of the Pacific Community (SPC).

The Electronic Distance Measurement (EDM) height traversing levelling technique was employed to observe differences in height between the deep bench mark arrays in Apia, which runs approximately 5.5 km from the tide gauge sensor to the CGNSS antenna. Previous levelling surveys have been conducted along the route using this technique in 2006, 2008, 2010, 2011 and 2013.

In addition, precise differential levelling surveys were performed along the deep bench mark array from 1992 to 2004 by the National Tidal Centre Australia (NTCA). This report contains a comparison between the 2013 and 2011 EDM height traversing results as well as a combined comparison since the first levelling survey.

# The Survey

The EDM height traversing levelling survey was carried out between the SEAFRAME tide gauge sensor, continuous GPS station and the deep driven bench mark array:

SAM16 	– SEAFRAME Project Plaque Bench mark

SAM17	– SEAFRAME Sensor Bench mark

BM201 	– Deep driven BM, held fixed, located at the rear of the X Bar along the Apia Port Road.

BM210	– Deep Driven Bench mark along Matafagatele Street inside LDS Church grounds.

BM212	– Deep Driven Bench mark located at the end of Matafagatele Street and the start of the Main East Coast Road inside the Church compounds.

BM213	– Deep Driven Bench mark outside a shop at the corner of the Main East Coast Road and Vaivase Road.

BM214	– Deep Driven Bench mark located corner of Vaivase and Fuaiupolu Roads.

BM215	– Deep Driven Bench mark located inside church grounds, Plantation Road.

BM220	– Deep Driven Bench mark located in front of Apia Park entrance gate

SAMO	– CGNSS reference point

SAMO RM1	– CGNSS reference mark

SAMO RM4	– CGNSS reference mark

SAMO RM3	– CGNSS reference mark

SAMOBM	– Level reference bench mark for the GNSS pillar

All the deep bench marks were located and found in good order and undisturbed. Also included in the survey were the permanent holding marks; SAM317, SAM166, SAM309, SAM180, SAM104, SAM105, SAM168, SAM307, SAM311, SAM170, SAM300, SAM301, SAM 187, SAM318, SAM308, SAM318, SAM303, SAM313, SAM304, SAM314, SAM305, SAM315, SAM139, SAM310 and SAM316.

Marks SAM317 and SAM318 are new holding marks established in 2013 and consist of cup shaped stainless steel bolts drilled in concrete and glued in place with quality epoxy resin.

The EDM height traversing levelling technique was performed to the Class L2A specifications (ICSM, Standards and Practices for Control Surveys). After reduction an internal precision of 1mm√K or better was achieved; well within the specifications of the project which is 2√K where K is the distance in kilometres. A table of results and comparisons and the 2013 reduced levels are detailed later in this report.

## Bench Mark Locality Diagram

## The Western Samoa Datum

The Datum for the survey is Mean Sea Level (1993) as established by the National Tidal Centre Australia.

The reference point for this survey is BM201 derived in 1993.

Reduction of the data has been calculated holding BM201 fixed at 1.3292 m.

## Equipment

Leica total station model TM30 (Serial No: 361441)

Leica precision prisms GPH1P (2).

Leica rigid tripod.

Stainless steel target poles supported by Leica telescopic bi-poles (2).

Shortened stainless steel target pole for the SEAFRAME sensor BM connection.

Leica cast iron change plates (2).

Kestral 4000 pocket weather tracker

## Method

The “Leap-Frog” EDM height traversing technique was employed for the Apia tide gauge levelling survey. This technique involves setting up a total station (TCA1800L) midway between two target/reflectors (on reflector rods with struts). The targets remain at a particular change point for the back-sight and fore-sight observations. The instrument measures slope distances (±1mm) and vertical angle (1”) to derive height differences (between the instrument’s trunnion axis and the reflectors). In support of the slope distance observations, the ambient temperature, pressure and humidity are recorded (Kestral 4000 pocket weather tracker) and input into the instrument to apply the first velocity correction to the observed distances (Rüeger & Brunner, 1982). Four rounds of observations are taken to the back-sight and fore-sight targets from each instrument setup. All levelling runs started and finished with the same reflector and reflector rod, i.e. an even number of setups when the two reflector rod configuration was used. This eliminates any reflector rod zero error. This technique can also be performed using a single set-up / single rod configuration which is particularly useful when levelling between bench marks which are close together e.g. between the CGNSS RMs.

Reduction of the digital data was computed by the Geoscience Australia levelling program “leveling1.exe”. This program computes the height difference between the two reflectors.

## Samoa 2013 Reduced Levels

Table 2.1 Apia, Samoa – Reduced levels and heights difference relative to BM201
Date:	22nd May to 1st June 2013
Datum:	Mean Sea Level

| Point ID | Reduced Level 2013 | Type |
| --- | --- | --- |
| BM201 | 1.32920 | Stainless Steel Rod in Ground |
| SAM317 | 2.35798 | Stainless Steel Pin in Concrete |
| SAM166 | 2.21730 | Stainless Steel Pin in Concrete |
| SAM309 | 1.94263 | Spike in Bitumen |
| SAM16 | 2.37280 | Stainless Steel Pin in Concrete |
| SAM17 | 4.15700 | Stainless Steel Pin |
| SAM180 | 2.19378 | Stainless Steel Pin in Concrete |
| SAM104 | 2.13621 | Stainless Steel Pin in Concrete |
| SAM105 | 2.10837 | Stainless Steel Pin in Concrete |
| SAM168 | 1.78381 | Stainless Steel Pin in Concrete |
| SAM307 | 1.72728 | Stainless Steel Pin in Concrete |
| BM210 | 1.25385 | Stainless Steel Rod in Ground |
| SAM311 | 2.09004 | Stainless Steel Pin in Concrete |
| SAM170 | 1.94638 | Stainless Steel Pin in Concrete |
| SAM300 | 1.45625 | Stainless Steel Pin in Concrete |
| BM220 | 1.52153 | Stainless Steel Rod in Ground |
| SAM301 | 1.61895 | Stainless Steel Pin in Concrete |
| SAM187 | 1.37890 | Masonry Nail in Concrete |
| SAM318 | 1.32165 | Stainless Steel Pin in Concrete |
| SAM319 | 1.48170 | Steel Pin in Concrete |
| BM212 | 1.38665 | Stainless Steel Rod in Ground |
| SAM303 | 2.09112 | Stainless Steel Pin in Concrete |
| SAM313 | 1.76564 | Stainless Steel Pin in Concrete |
| BM213 | 1.18411 | Stainless Steel Rod in Ground |
| SAM304 | 1.69869 | Stainless Steel Pin in Concrete |
| SAM314 | 3.65678 | Masonry Nail in Concrete |
| BM214 | 8.11896 | Stainless Steel Rod in Ground |
| SAM305 | 10.00738 | Stainless Steel Pin in Concrete |
| SAM315 | 9.87025 | Stainless Steel Pin in Concrete |
| SAM139 | 17.66525 | Stainless Steel Rod in Ground |
| BM215 | 21.61044 | Stainless Steel Rod in Ground |
| SAM310 | 29.76698 | Stainless Steel Pin in Concrete |
| SAM316 | 30.30625 | Stainless Steel Pin in Concrete |
| SAMOBM | 38.19008 | Stainless Steel Pillar Pin |
| *SAMO | 38.91828 | Pillar Plate (ARP) |

*The RL of the Reference Point SAMO (ARP) is derived from adding the static height difference of 0.7282 m (SAMOBM to SAMO) to the levelled RL of SAMOBM.

## Survey Support

Ms. Siosina Lui from the Ministry of Natural Resources & Environment provided valuable assistance in obtaining customs clearances for the surveying equipment. This is a lengthy process and seems to becoming more time consuming throughout most countries in the Pacific. The project is using Calcy Pacific Agencies as the local customs agent broker to make necessary arraangemnts for customs clearance.

Senior Surveyor Mr. Petanaia Taulu and Assistant Surveyor, Mr. Phillip Laulu from the Survey Department, provided us with valuable field assistance and support during the field survey.

Other personnel consulted during the visit were Mr. Austelaia Titimaea (CEO) from the Meteorology Division of the Ministry of Natural Resources & Environment.

## Issues

Avoid levelling between SAM317 and the tide gauge bench mark during offloading of ships or increased port activity. Access inside the Ports Authority area is strictly prohibited and it is necessary to arrange security passes through the Weather Office or the Lands and Surveys staff well in advance. The Ports Authority area is closed on Saturdays, however, security is still on the gate. This is the best day to perform the survey.

Levelling along the reclaimed area around the wharf and along Matafagatele Street, especially around Apia Park Stadium should be carried out during quiet traffic periods. It is recommended to avoid areas with drainage systems underneath the footpath in that area as the heavy trucks which drive past cause significant ground shake.

# Comparisons

## Comparisons between 2013 and 2011 EDM Surveys

Table 3.1 Apia, Western Samoa 2013 EDM Height Traversing Levelling Comparison 2013- 2011. BM201 - adopted fixed height OF 1.3292 m

| From | To | Levelled Ht. Diff. | RL 2013 | Misclose (mm) | Dist. (km) | 1mm√k | RL 2011 | Difference (mm) 2013 - 2011 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BM201 |  |  | 1.3292 |  |  |  |  |  |
| SAM317 | SAM317 | 1.0288 | 2.3580 | 0.080 | 0.096 | 0.310 |  | New Mark |
| SAM166 | SAM166 | -0.1407 | 2.2173 | -0.095 | 0.183 | 0.428 | 2.2183 | -1.00 |
| SAM309 | SAM309 | -0.2747 | 1.9426 | -0.163 | 0.186 | 0.431 | 1.9457 | -3.07 |
| SAM16 | SAM16 | 0.4302 | 2.3728 | -0.068 | 0.051 | 0.226 | 2.3733 | -0.50 |
|  | SAM17 | 1.7842 | 4.1570 | 0.000 | 0.020 | 0.142 | 4.158 | -1.00 |
| SAM317 |  |  | 2.3580 |  |  |  |  |  |
| SAM180 | SAM180 | -0.1642 | 2.1938 | 0.013 | 0.111 | 0.334 | 2.1939 | -0.12 |
| SAM104 | SAM104 | -0.0576 | 2.1362 | 0.013 | 0.094 | 0.307 | 2.1361 | 0.11 |
| SAM105 | SAM105 | -0.0278 | 2.1084 | 0.200 | 0.069 | 0.262 |  | New Mark |
| SAM168 | SAM168 | -0.3246 | 1.7838 | 0.171 | 0.071 | 0.266 | 1.7832 | 0.61 |
| SAM307 | SAM307 | -0.0565 | 1.7273 | -0.435 | 0.214 | 0.462 | 1.7268 | 0.48 |
| BM210 | BM210 | -0.4734 | 1.2539 | -0.021 | 0.029 | 0.169 | 1.2519 | 1.95 |
| SAM311 | SAM311 | 0.8362 | 2.0900 | 0.063 | 0.203 | 0.450 | 2.0889 | 1.14 |
| SAM170 | SAM170 | -0.1437 | 1.9464 | 0.363 | 0.154 | 0.392 | 1.9442 | 2.18 |
| SAM300 | SAM300 | -0.4901 | 1.4562 | -0.400 | 0.199 | 0.446 | 1.4569 | -0.65 |
| BM220 | BM220 | 0.0653 | 1.5215 | -0.150 | 0.185 | 0.431 | 1.5181 | 3.43 |
| SAM301 | SAM301 | 0.0974 | 1.6190 | 0.440 | 0.206 | 0.454 | 1.6151 | 3.85 |
| SAM187 | SAM187 | -0.2401 | 1.3789 | -0.040 | 0.192 | 0.439 | 1.3757 | 3.20 |
| SAM318 | SAM318 | -0.0573 | 1.3216 | -0.150 | 0.182 | 0.426 |  | New Mark |
| SAM319 | SAM308 | 0.1601 | 1.4817 | 0.333 | 0.098 | 0.314 |  | New Mark |
| BM212 | BM212 | -0.0950 | 1.3867 | -0.090 | 0.168 | 0.410 | 1.3811 | 5.55 |
| SAM303 | SAM303 | 0.7045 | 2.0911 | -0.210 | 0.212 | 0.461 | 2.0866 | 4.52 |
| SAM313 | SAM313 | -0.3255 | 1.7656 | 0.403 | 0.206 | 0.454 |  | New Mark |
| BM213 | BM213 | -0.5815 | 1.1841 | -0.290 | 0.202 | 0.450 | 1.1791 | 5.01 |
| SAM304 | SAM304 | 0.5146 | 1.6987 | -0.067 | 0.075 | 0.275 | 1.6938 | 4.89 |
| SAM314 | SAM314 | 1.9581 | 3.6568 | 0.440 | 0.200 | 0.447 | 3.6519 | 4.88 |
| BM214 | BM214 | 4.4622 | 8.1190 | -0.220 | 0.172 | 0.414 | 8.1143 | 4.66 |
| SAM305 | SAM305 | 1.8884 | 10.0074 | -0.300 | 0.145 | 0.381 | 10.0022 | 5.18 |
| SAM315 | SAM315 | -0.1371 | 9.8702 | -0.050 | 0.184 | 0.429 | 9.8651 | 5.15 |
| SAM139 | SAM139 | 7.7950 | 17.6653 | -0.190 | 0.157 | 0.396 | 17.6605 | 4.75 |
| BM215 | BM215 | 3.9452 | 21.6104 | -0.217 | 0.091 | 0.302 | 21.6043 | 6.14 |
| SAM310 | SAM310 | 8.1565 | 29.7670 | -0.388 | 0.167 | 0.408 | 29.7612 | 5.78 |
| SAM316 | SAM316 | 0.5393 | 30.3063 | -0.483 | 0.224 | 0.474 | 30.3006 | 5.65 |
|  | SAMOBM | 7.8838 | 38.1901 | 0.100 | 0.214 | 0.462 | 38.1836 | 6.48 |
|  |  |  | Misclose for all bays levelled = | -1.409 | 4.962 | 2.228 |  |  |
| SAMOBM |  |  | 38.1901 |  |  |  |  |  |
|  | RM1 | -1.73073 | 36.4594 | -0.038 | 0.036 | 0.189 | 36.4524 | 6.96 |
| SAMOBM |  |  | 38.1901 |  |  |  |  |  |
|  | RM4 | -0.7821 | 37.4080 | 0.019 | 0.027 | 0.164 | 37.4015 | 6.49 |
| SAMOBM |  |  | 38.1901 |  |  |  |  |  |
|  | RM3 | -1.22224 | 36.9678 | 0.013 | 0.034 | 0.185 | 36.9614 | 6.45 |

## Combined Comparisons 1993 to 2013

Table 3.2 Apia, Western Samoa - Comparison of the RL's for Precise Differential Levelling (1993 - 2006) and EDM Height Traversing (2006 – 2013)

| Year | SAM16 | SAM17 | BM210 | BM220 | BM212 | BM213 | BM214 | BM215 | SAMOBM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1993.1 | 2.3843 | 4.1769 |  |  |  |  |  |  |  |
| 1994.8 | 2.3812 | 4.1719 |  |  |  |  |  |  |  |
| 1996.4 | 2.3796 | 4.1696 |  |  |  |  |  |  |  |
| 1998.4 | 2.3783 | 4.1685 |  |  |  |  |  |  |  |
| 1999.8 | 2.3777 | 4.1675 |  |  |  |  |  |  |  |
| 2001.4 | 2.3768 | 4.1665 |  |  |  |  |  |  |  |
| 2002.9 | 2.3751 | 4.1634 | 1.2535 |  | 1.3846 | 1.1826 | 8.1172 | 21.6076 | 38.1869 |
| 2004.7 | 2.3757 | 4.1627 | 1.2538 |  | 1.3864 | 1.1842 | 8.1189 | 21.6103 | 38.1889 |
| 2006.8 | 2.3739 | 4.162 | 1.2546 |  | 1.3884 | 1.1874 | 8.1223 | 21.6130 | 38.1926 |
| 2006.8 | 2.3751 | 4.1629 | 1.2557 |  | 1.3898 | 1.1876 | 8.1218 | 21.6128 | 38.1921 |
| 2008.3 | 2.3744 | 4.1613 | 1.2542 | 1.5237 | 1.3880 | 1.1861 | 8.1215 | 21.6131 | 38.1919 |
| 2010.5 | 2.3742 | 4.1610 | 1.2537 | 1.5221 | 1.3884 | 1.1865 | 8.1217 | 21.6122 | 38.1914 |
| 2011.9 | 2.3733 | 4.1580 | 1.2519 | 1.5181 | 1.3811 | 1.1791 | 8.1143 | 21.6043 | 38.1836 |
| 2013.4 | 2.3728 | 4.1570 | 1.2539 | 1.5215 | 1.3867 | 1.1841 | 8.1190 | 21.6104 | 38.1901 |

## Time Series of Bench Mark Movement relative to Fixed Deep Bench Mark BM201

Please note the graph below, there has been a gradual drop in SAM17 and SAM16 (Tide gauge Station) since 1995. This could be caused by the ports area being reclaimed land; but this theory has yet to be confirmed.

The purpose of these surveys is twofold: firstly, to provide accurate changes in land height to be used in computations of absolute sea level rise and secondly to provide accurate assessments of relative sea level rise. This is an excellent example to highlight the importance of measuring land height variations and the impact they can have on absolute sea level rise measurements. Furthermore, it provides useful information regarding localised deformation and the potential relative sea level height impacts.

Precise Differential Levelling: 1995 - 2006	
EDM Height Traversing: 2005 onwards

# Deep Bench Mark Locality Diagrams

# Western Samoa Reference Mark Locality Diagram

# Permanent Holding Mark Locality Diagrams

# References

Rüeger, J.M. & Brunner, F.K. 1982, ‘EDM Height Traversing versus Geodetic Levelling’, The Canadian Surveyor, vol. 36, no. 1, pp. 69-87.

Inter-Government Committee on Surveying and Mapping (ICSM) 2002, Standards and Practices for Control Surveys, SP1, Ver. 1.5, May 2002.