EDM Height Traversing Levelling Survey Report

Nuku’alofa, Tonga, April 2013

Geoscience Australia
Record 2014/28

S. J. K. Yates1, A. Lal2

National Geospatial Reference Systems, Geospatial and Earth Monitoring Division, Geoscience Australia GPO Box 378 Canberra ACT 2601

Secretariat of the Pacific Community (SPC)

Department of Industry

Minister for Industry: The Hon Ian Macfarlane MP
Parliamentary Secretary: The Hon Bob Baldwin MP
Secretary: Ms Glenys Beauchamp PSM

Geoscience Australia

Chief Executive Officer: Dr Chris Pigram
This paper is published with the permission of the CEO, Geoscience Australia

© Commonwealth of Australia (Geoscience Australia) 2014

With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence. (http://www.creativecommons.org/licenses/by/3.0/au/deed.en)

Geoscience Australia has tried to make the information in this product as accurate as possible. However, it does not guarantee that the information is totally accurate or complete. Therefore, you should not solely rely on this information when making a commercial decision.

Geoscience Australia is committed to providing web accessible content wherever possible. If you are having difficulties with accessing this document please email clientservices@ga.gov.au.

ISSN 2201-702X (PDF)

ISBN 978-1-925124-17-0 (PDF)

GeoCat 76792

Bibliographic reference: Yates, S. J. K. & Lal, A., 2014, EDM Height Traversing Levelling Survey Report: Nuku’alofa, Tonga, April 2013. Record 2014/28. Geoscience Australia, Canberra. http://dx.doi.org/10.11636/Record.2014.028

Contents

1 Introduction	1

2 The Survey	2

2.1 Bench Mark Locality Diagram	3

2.2 The Tonga Datum	3

2.3 Equipment	3

2.4 Method	4

2.5 Tonga 2013 Reduced Levels	4

2.6 Survey Support	6

2.7 Issues	6

3 Comparisons	7

3.1 Comparisons between 2013 and 2011 EDM Survey	7

3.2 Combined Comparisons 1993 to 2013	8

3.3 Time Series of Bench Mark Movement relative to Fixed Deep Bench Mark TON1	9

4 Deep Bench Mark Locality Diagrams	14

5 Temporary Holding Mark Locality Diagrams	20

6 Reference Mark Locality Diagram	25

7 References	29

# Introduction

This report outlines the high precision level survey completed between the Sea Level Fine Resolution Acoustic Measuring Equipment (SEAFRAME) tide gauge and the Continuous Global Navigation Satellite System (CGNSS) station in Nuku’alofa, Tonga from 9th to 16th April 2013.

Personnel involved in the survey were Steve Yates, Project Officer, Geoscience Australia and Andrick Lal, Surveyor, Secretariat of Pacific Community (SPC).

The Electronic Distance Measurement (EDM) height traversing levelling technique was employed to observe differences in height between the deep bench mark arrays in Nuku’alofa, which runs approximately 2 km from the tide gauge sensor to the CGNSS antenna. Previous levelling surveys have been conducted along the route using this technique in 2005, 2007, 2008, 2010 and 2011.

In addition, precise differential levelling surveys were performed along the deep bench mark array from 1993 to 2005 by the National Tidal Centre Australia (NTCA) with the survey in 2005 including a comparison between the precise levelling and EDM height traversing technique. This report contains a comparison between the 2013 and 2011 EDM height traversing results as well as a combined comparison since the first levelling survey.

# The Survey

The EDM height traversing levelling survey was carried out between the SEAFRAME tide gauge sensor, the CGNSS station and the deep driven bench mark array:

TON15 	– project plaque point

TON16 	– SEAFRAME sensor benchmark

TON1 	– deep driven bench mark

TON60 	– deep driven bench mark

TON61 	– deep driven bench mark

TON62 	– deep driven bench mark

TON2 	– deep driven bench mark

TON3 	– deep driven bench mark

TONGBM 	– Levelling reference bench mark for the CGNSS pillar

All the deep bench marks were located and found in good order and undisturbed.

The temporary holding marks TON67 and TON101 was found to be destroyed and was replaced with TON109 and TON108 respectively. Also included in the survey were the holding marks TON100 and TON102 – 107.

The EDM height traversing levelling technique was performed to the Class L2A specifications (ICSM, Standards and Practices for Control Surveys). After reduction an internal precision of 1mm√K or better was achieved, where K is distance in kilometres. This is well within the project specification of 2mm√K. A table of results and comparisons and the 2011 reduced levels are detailed later in this report.

## Bench Mark Locality Diagram

## The Tonga Datum

The Datum for the survey is Mean Sea Level (1990).

Reduction of the data has been calculated holding TON1 fixed at 1.118552 metres, this value was determined by the National Tidal Centre Australia in October 1993 by:

Adopting the height of BM41

RL = 1.619 m MSL

Adopting the height of TON1 as derived from the 1993 survey

RL = 1.118552 metres.

## Equipment

Leica total station model TM30 (Serial No: 361441)

Leica precision prisms GPH1P (2).

Leica rigid tripod.

Stainless steel target poles supported by Leica telescopic bi-poles (2).

Shortened stainless steel target pole for the SEAFRAME sensor BM connection.

Leica cast iron change plates (2).

Kestral 4000 pocket weather tracker

## Method

The “Leap-Frog” EDM height traversing technique was employed for the Nuku’alofa tide gauge levelling survey. This technique involves setting up a total station (TCA1800L) midway between two target/reflectors (on reflector rods with struts). The targets remain at a particular change point for the back-sight and fore-sight observations. The instrument measures slope distances (±1mm) and vertical angle (1”) to derive height differences (between the instrument’s trunnion axis and the reflectors). In support of the slope distance observations, the ambient temperature, pressure and humidity are recorded (Kestral 4000 pocket weather tracker) and input into the instrument to apply the first velocity correction to the observed distances (Rüeger & Brunner, 1982). Four rounds of observations are taken to the back-sight and fore-sight targets from each instrument setup. All levelling runs started and finished with the same reflector and reflector rod, i.e. an even number of setups when the two reflector rod configuration was used. This eliminates any reflector rod zero error. This technique can also be performed using a single set-up / single rod configuration which is particularly useful when levelling between bench marks which are close together e.g. between the CGNSS RMs.

Reduction of the digital data was computed by the Geoscience Australia levelling program “leveling1.exe”. This program computes the height difference between the two reflectors.

## Tonga 2013 Reduced Levels

Table 2.1 Nuku’alofa, Tonga – Reduced levels and heights difference relative to TON1
Date:	9 to 16 April 2013
Datum:	Mean Sea Level (1990)

| Point ID | 2013 Reduced Level | Type |
| --- | --- | --- |
| TON1 | 1.11855 | Stainless Steel Rod in Ground |
| TON103 | 1.16440 | Stainless Steel Pin in Concrete |
| TON102 | 1.95456 | Stainless Steel Pin in Concrete |
| TON108 | 1.57005 | Stainless Steel Pin in Concrete |
| TON100 | 1.45682 | Stainless Steel Pin in Concrete |
| TON15 | 2.27119 | Stainless Steel Pin in Concrete |
| TON16 | 3.88896 | Stainless Steel Pin |
| TON60 | 1.44120 | Stainless Steel Rod in Ground |
| TON104 | 1.66906 | Stainless Steel Pin in Concrete |
| TON61 | 1.09140 | Stainless Steel Rod in Ground |
| TON109 | 1.37715 | Stainless Steel Pin in Concrete |
| TON62 | 1.11994 | Stainless Steel Rod in Ground |
| TON105 | 1.37567 | Stainless Steel Pin in Concrete |
| TON2 | 0.79853 | Stainless Steel Rod in Ground |
| TON107 | 1.67415 | Stainless Steel Pin in Bitumen |
| TON106 | 5.75891 | Stainless Steel Pin in Concrete |
| TON3 | 7.62395 | Stainless Steel Rod in Ground |
| TONGBM | 1.98194 | Stainless Steel Pillar Pin |
| *TONG | 2.94814 | Pillar Plate (ARP) |
| TONG RM1 | 0.74281 | Stainless Steel Rod in Ground |
| TONG RM2 | 0.80713 | Stainless Steel Rod in Ground |
| TONG RM3 | 0.89941 | Stainless Steel Rod in Ground |

*The RL of the Reference Point TONG (ARP) is derived from adding the static height difference of 0.9662 m (TONGBM to TONG) to the levelled RL of TONGBM.

## Survey Support

The survey team very much appreciated the assistance from Lands and Survey Department.

In particular Mr Aleki Pongi, to make clearance arrangements of the survey equipment from the Customs Authority; Mr Viliami Pahulu and Mr Tahi Kato, from the Geodetic Section.

We also contacted the primary contact of the project Mr Richard Kautoke of the GIS section.

## Issues

Whilst setting up or using the deep driven benchmark, TON60, which is out the front of the ANZ building, one must be aware that it is impossible to setup a telescopic tripod over the mark only the bipod can be levelled over the mark; the fence wall is very close to the benchmark.

Prior arrangements with the local counterpart should be made for the clearance of the survey equipment from the Customs Authority when it is shipped across.

# Comparisons

## Comparisons between 2013 and 2011 EDM Survey

Table 3.1 Results of Nuku’alofa, Tonga 2013 EDM Height Traversing Comparison 2013 – 2011. TON1 - adopted fixed height of 1.11855 m

| From | To | Levelled Ht. Diff. | RL 2013 | Misclose (mm) | Dist. (km) | 1mmk | RL 2011 | Difference (mm) 2013 - 2011 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TON1 |  |  | 1.11855 |  |  |  |  |  |
| TON103 | TON103 | 0.04585 | 1.16440 | -0.050 | 0.138 | 0.372 | 1.16473 | -0.33 |
| TON102 | TON102 | 0.79016 | 1.95456 | 0.079 | 0.101 | 0.318 | 1.95498 | -0.42 |
| TON108 | TON108 | -0.38450 | 1.57005 | 0.150 | 0.056 | 0.237 | 1.56984 | 0.21 |
| TON100 | TON100 | -0.11323 | 1.45682 | -0.004 | 0.085 | 0.292 | 1.45712 | -0.30 |
| TON15 | TON15 | 0.81437 | 2.27119 | 0.050 | 0.063 | 0.250 | 2.27063 | 0.56 |
|  | TON16 | 1.61778 | 3.88896 | -0.110 | 0.045 | 0.213 | 3.88840 | 0.56 |
| TON1 |  |  | 1.11855 |  |  |  |  |  |
| TON60 | TON60 | 0.32265 | 1.44120 | -0.052 | 0.031 | 0.176 | 1.44156 | -0.356 |
| TON104 | TON104 | 0.22786 | 1.66906 | -0.263 | 0.154 | 0.393 | 1.66993 | -0.870 |
| TON61 | TON61 | -0.57766 | 1.09140 | -0.173 | 0.176 | 0.420 | 1.09278 | -1.383 |
| TON109 | TON109 | 0.28576 | 1.37715 | 0.063 | 0.129 | 0.358 | New Mark | New Mark |
| TON62 | TON62 | -0.25722 | 1.11994 | -0.350 | 0.155 | 0.393 | 1.12155 | -1.614 |
|  | TONGBM | 0.86200 | 1.98194 | 0.013 | 0.070 | 0.264 | 1.98371 | -1.772 |
| TON109 |  |  | 1.37715 |  |  |  | New Mark | New Mark |
| TON105 | TON105 | -0.00148 | 1.37567 | -0.388 | 0.204 | 0.451 | 1.37897 | -3.301 |
| TON2 | TON2 | -0.57715 | 0.79853 | 0.108 | 0.032 | 0.178 | 0.80068 | -2.154 |
| TON107 | TON107 | 0.87562 | 1.67415 | 0.050 | 0.203 | 0.451 | 1.69119 | -17.041 |
| TON106 | TON106 | 4.08476 | 5.75891 | 0.438 | 0.189 | 0.434 | 5.76213 | -3.222 |
|  | TON3 | 1.86504 | 7.62395 | -0.354 | 0.166 | 0.407 | 7.62718 | -3.230 |
|  |  |  | Misclose for all bays levelled = | -0.793 | 1.995 | 5.607 |  |  |
| TONGBM |  |  | 1.98194 |  |  |  |  |  |
|  | RM1 | -1.23913 | 0.74281 | -0.033 | 0.039 | 0.198 | 0.74461 | -1.797 |
| TONGBM |  |  | 1.98194 |  |  |  |  |  |
|  | RM2 | -1.17481 | 0.80713 | -0.042 | 0.029 | 0.170 | 0.80888 | -1.754 |
| TONGBM |  |  | 1.98194 |  |  |  |  |  |
|  | RM3 | -1.08253 | 0.89941 | -0.017 | 0.052 | 0.228 | 0.90139 | -1.977 |

All levelling was performed within the project specifications of 2√k

## Combined Comparisons 1993 to 2013

Table 3.2 Nuku’alofa, Tonga - Comparison of the RL's for Precise Differential Levelling (1993 - 2005) and EDM Height Traversing (2005 - 2013).

| Year | TON1 | TON2 | TON3 | TON60 | TON61 | TON62 | TONGBM | TON15 | TON16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1993.79 | 1.1186 | 0.7978 | 7.6217 |  |  |  |  | 2.2710 | 3.8964 |
| 1994.71 | 1.1186 | 0.7981 | 7.6224 |  |  |  |  | 2.2707 | 3.8951 |
| 1995.88 | 1.1186 | 0.7973 | 7.6212 |  |  |  |  | 2.2716 | 3.8989 |
| 1997.46 | 1.1186 | 0.7974 | 7.6217 |  |  |  |  | 2.2701 | 3.8896 |
| 1998.88 | 1.1186 | 0.7979 | 7.6223 |  |  |  |  | 2.2698 | 3.8901 |
| 2000.38 | 1.1186 | 0.7968 | 7.6211 |  |  |  |  | 2.2695 | 3.8900 |
| 2002.13 | 1.1186 | 0.7981 | 7.6227 | 1.4504 | 1.0914 | 1.1198 | 1.9826 | 2.2703 | 3.8896 |
| 2003.46 | 1.1186 | 0.7978 | 7.6229 | 1.4500 | 1.0913 | 1.1194 | 1.9818 | 2.2700 | 3.8887 |
| 2005.46 | 1.1186 | 0.7981 | 7.6228 | 1.4497 | 1.0913 | 1.1196 | 1.9821 | 2.2699 | 3.8891 |

| Year | TON1 | TON2 | TON3 | TON60 | TON61 | TON62 | TONGBM | TON15 | TON16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2005.46 | 1.1186 | 0.7984 | 7.6232 | 1.4497 | 1.0921 | 1.1203 | 1.9828 | 2.2692 | 3.8882 |
| 2007.21 | 1.1186 | 0.7985 | 7.6239 | 1.4427 | 1.0920 | 1.1208 | 1.9832 | 2.2723 | 3.8960 |
| 2008.83 | 1.1186 | 0.7985 | 7.6237 | 1.4421 | 1.0918 | 1.1207 | 1.983 | 2.2714 | 3.8565 |
| 2010.30 | 1.1186 | 0.7994 | 7.6251 | 1.4418 | 1.0921 | 1.1208 | 1.9830 | 2.2718 | 3.8910 |
| 2011.40 | 1.1186 | 0.8009 | 7.6274 | 1.4418 | 1.0930 | 1.1218 | 1.9839 | 2.2706 | 3.8933 |
| 2013.27 | 1.1186 | 0.7985 | 7.6240 | 1.4412 | 1.0914 | 1.1199 | 1.9819 | 2.2712 | 3.8890 |

## Time Series of Bench Mark Movement relative to Fixed Deep Bench Mark TON1

The majority of the deep driven benchmarks only show minor variability when compared with previous surveys; however, there are a couple which require further discussion and investigation.

Firstly, the time series of TON16 (SEAFRAME sensor benchmark) shows the knock it received between early 2007 and late 2008 causing a ~40 mm drop in the height of the tide gauge and the results of the most recent survey show the height increase by ~35 mm after the tide gauge was straightened following the knock.

The other point which has undergone significant motion is TON60 which dropped by ~8 mm between mid 2005 and early 2007. Since that time two surveys have shown little motion at this site, however, there is an overall downward trend.

During this survey, temporary benchmark TON107 showed a significant drop by ~17 mm, this is due to its nature of establishment; it is a stainless pin in bitumen and have a tendency to move due to its location.

Precise Differential Levelling: 1993 - 2005	
EDM Height Traversing: 2005 onwards

# Deep Bench Mark Locality Diagrams

# Temporary Holding Mark Locality Diagrams

# Reference Mark Locality Diagram

# References

Rüeger, J.M. & Brunner, F.K. 1982, ‘EDM Height Traversing versus Geodetic Levelling’, The Canadian Surveyor, vol. 36, no. 1, pp. 69-87.

Inter-Government Committee on Surveying and Mapping (ICSM) 2002, Standards and Practices for Control Surveys, SP1, Ver. 1.5, May 2002.