EDM Height Traversing Levelling Survey Report

Port Vila, Vanuatu, October 2012

Geoscience Australia
RECORD 2014/31

S. J. K. Yates1, A. Lal2

National Geospatial Reference Systems, Minerals and Natural Hazards Division, Geoscience Australia GPO Box 378 Canberra ACT 2601

Ocean & Islands Programme, Applied Geoscience and Technology Division (SOPAC), Secretariat of the Pacific Community, Suva, Fiji Islands

Department of Industry

Minister for Industry: The Hon Ian Macfarlane MP
Parliamentary Secretary: The Hon Bob Baldwin MP
Secretary: Ms Glenys Beauchamp PSM

Geoscience Australia

Chief Executive Officer: Dr Chris Pigram
This paper is published with the permission of the CEO, Geoscience Australia

© Commonwealth of Australia (Geoscience Australia) 2014

With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence. (http://www.creativecommons.org/licenses/by/3.0/au/deed.en)

Geoscience Australia has tried to make the information in this product as accurate as possible. However, it does not guarantee that the information is totally accurate or complete. Therefore, you should not solely rely on this information when making a commercial decision.

Geoscience Australia is committed to providing web accessible content wherever possible. If you are having difficulties with accessing this document please email clientservices@ga.gov.au.

ISSN 2201-702X (PDF)

ISBN 978-1-925124-20-0 (PDF)

GeoCat 76794

Bibliographic reference: Yates S. J. K, Lal A., 2014. EDM Height Traversing Levelling Survey Report, Port Vila, Vanuatu, October 2012. Record 2014/31. Geoscience Australia, Canberra. http://dx.doi.org/10.11636/Record.2014.031

Contents

1 Introduction	1

1.1 The Survey	1

1.2 Bench Mark Locations – Port Vila	2

1.3 The Vanuatu Datum	2

1.4 Equipment	3

1.5 Method	3

1.6 Survey Support	3

1.7 Issues	3

1.8 Description of Marks – Port Vila, Vanuatu	4

1.9 Comparisons between 2012 and 2011 EDM Surveys	5

1.10 Combined Comparisons	7

1.11 Vanuatu 2012 Reduced Levels	8

1.12 Time Series of Bench Mark movement relative to Fixed Deep Bench Mark VAN3	9

2 Deep Bench Mark Locality Diagrams	13

3 Vanuatu Reference Mark Locality Diagrams	17

4 Temporary Holding Mark Locality Diagrams	21

5 References	32

# Introduction

This report outlines the high precision level survey completed between the SEAFRAME (Sea Level Fine Resolution Acoustic Measuring Equipment) tide gauge and continuous GPS (Global Positioning System) station and the newly established Global Navigation Satellite System (GNSS) Station in Port Vila, Vanuatu from 16th – 21st October 2012.

Personnel involved in the survey were Steve Yates, Project Officer, Geoscience Australia and Andrick Lal, Surveyor, Secretariat of the Pacific Islands Community (SPC).

The Electronic Distance Measurement (EDM) Height Traversing levelling technique was employed to observe differences in height between the deep bench mark arrays in Port Vila, which runs approximately 4km from the tide gauge sensor to the continuous GPS Station. Previous levelling surveys have been conducted along the route using this technique in 2006, 2008, 2009 and 2011.

In addition, precise differential levelling surveys were performed along the deep bench mark array from 1993 to 2004 by the National Tidal Centre (NTC) and the survey in 2006 included a comparison between the precise differential levelling and EDM height traversing technique. This report contains a comparison between the 2012 and 2011 EDM height traversing results as well as a combined comparison since the first levelling survey.

## The Survey

The EDM height traversing levelling survey was carried out between the SEAFRAME tide gauge sensor, continuous GNSS station and the deep driven bench mark array:

VAN16	– SEAFRAME sensor bench mark

VAN14	– wall mounted survey bench mark adjacent to VAN16

VAN2		– deep driven bench mark

VAN3		– reference deep driven bench mark

VAN100	– deep driven bench mark (within meteorology compound)

VAN101	– deep driven bench mark (within parliament compound)

VANUBM	– reference bench mark for the CGPS pillar (within parliament compound)

PTVLBM	– reference bench mark for the GNSS pillar (within meteorology compound)

PTVL		– GNSS Pillar bench mark (within meteorology compound)

Also included in the survey were temporary holding marks - VAN160, VAN143, VAN200, VAN207, VAN208, VAN204, VAN132, VAN201, VAN154, VAN209, VAN202, VAN205, VAN164, VAN206, VAN138, and VAN166

The Continuous GPS Reference Point - VANU and the three CGPS Reference Marks - RM1, RM2 and RM3 were also levelled. Two additional temporary holding marks were installed to support the levelling survey network (VAN210 and VAN211). These new marks are stainless steel bolts drilled in concrete and then glued in place with quality epoxy resin.

A new Continuous GNSS Station (PTVL) was established in the Meteorological Office compound to replace the existing VANU CGPS Station located at the parliament complex which will be decommissioned in the near future due to development and building of the cultural centre within the complex. The new reference benchmark, PTVLBM, for the PTVL GNSS Station and the three reference marks around the Pillar were levelled in the 2012 survey.

All the deep bench marks were located and found in good order and undisturbed.

The EDM Height Traversing levelling technique was performed to the Class L2A specifications. After reduction an internal precision of 1mm √K or better was achieved, where K is distance in kilometres. This is well within the project specification of 2mm √K. A table of results and comparisons and the 2012 reduced levels are detailed later in this report.

## Bench Mark Locations – Port Vila

## The Vanuatu Datum

All reduced levels for the Port Vila levelling survey are relative to VAN3 fixed at 23.54630 m (TGZ-ORSTOM). This value was determined by the NTC in 1993 by adopting the height of BC1 (VAN14) with RL of 2.819m MSL.

## Equipment

LEICA total station model TCA2003 and TM30 (Serial No: 361441)

LEICA precision prisms GPH1P (2)

LEICA rigid tripod

Stainless steel target poles supported by LEICA telescopic bi-poles (2)

Shortened stainless steel target pole for the SEAFRAME sensor BM connection

LEICA cast iron change plates (2)

KESTRAL 4000 pocket weather tracker

## Method

The “Leap-Frog” EDM-height-traversing technique was employed for the Port Vila tide gauge levelling survey. The "Leap-Frog" EDM-height-traversing technique involves setting up a total station (TCA2003 or TM30) midway between two target/reflectors (on reflector rods with struts). The targets remain at a particular change point for the back-sight and fore-sight observations. The instrument measures slope distances (1mm) and vertical angle to derive height differences (between the instrument’s trunnion axis and the reflectors). In support of the slope distance observations, the ambient temperature, pressure and humidity are recorded (KESTRAL 4000 pocket weather tracker) and input into the instrument to apply the first velocity correction to the observed distances (Rüeger & Brunner 1982). Four rounds of observations are taken to the back-sight and fore-sight targets from each instrument setup. All levelling bays started and finished with the same reflector and reflector rod, i.e. an even number of setups when the two reflector rod configuration was used – this eliminates any reflector rod zero error.

Reduction of the digital data was computed by the Geoscience Australia levelling program “leveling1.exe”. This program computes the height difference between the two reflectors at any one set-up. Results can also be gained with the EDM Height Traversing method by using a single set-up / single rod configuration. This “single set-up / single rod” configuration is particularly useful when levelling between bench marks which are close together e.g. between the CGNSS Bench Mark and the Reference Marks.

## Survey Support

Due to a recent change in field procedures, two labourers, Ben Nawi and Jayson Sokomanu were hired to carry the Leica total station in the carry case during the survey. This was arranged through the kind assistance of Mr Martin Sokomanu.

## Issues

Bench mark VAN101 is located within the Parliament House grounds and may be difficult to access on weekends.

Always ensure when the survey equipment is freighted to Port Vila, prior arrangements with Meteorological Office or Survey Office should be made for the clearance from Customs Authority for concession on tariffs.

## Description of Marks – Port Vila, Vanuatu

VAN 3 	– deep driven bench mark

VAN 2 	– deep driven bench mark next to bus shelter along wharf road

VAN 100 	– deep driven bench mark within meteorology office compound

VAN 101 	– deep driven bench mark within parliament compound

VAN 16 	– SEAFRAME sensor bench mark

VAN14 (BC 1) 	– wall mounted bench mark within the main wharf area

VANUBM 	– Reference bench mark for the CGPS pillar

VANU 	– CGPS reference point

VANU RM1 	– CGPS survey reference marks

VANU RM2	– CGPS survey reference marks

VANU RM3 	– CGPS survey reference marks.

PTVLBM 	– Reference bench mark for the GNSS pillar

PTVL 	– GNSS reference point

PTVL RM1	– GNSS survey reference mark

PTVL RM2	– GNSS survey reference mark

PTVL RM3 	– GNSS survey reference mark

Holding pins along survey route from tide gauge to CGPS (as listed previously) are all stainless steel bolts, drilled in concrete and glued in place with the exception of VAN154, a metal spike and VAN166, a bolt located at the base of a light pole along the entrance road to the VANU CGPS Pillar.

## Comparisons between 2012 and 2011 EDM Surveys

Table 1.1 Results of Port Vila, Vanuatu 2012 EDM Height Traversing Comparison 2012 – 2011. VAN3 - adopted fixed height of 23.5463m

| From | To | Levelled Ht. Diff. | RL 2012 | Misclose (mm) | Dist. (km) | 1mmk | RL 2011 | Difference (mm) 2012 - 2011 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VAN3 |  |  | 23.54630 |  |  |  |  |  |
| VAN160 | VAN160 | -4.55954 | 18.98676 | -0.013 | 0.180 | 0.424 | 18.9873 | -0.51 |
| VAN210 | VAN210 | -7.44061 | 11.54615 | 0.055 | 0.206 | 0.454 | New Mark | New Mark |
| VAN143 | VAN143 | 1.50430 | 13.05045 | 0.402 | 0.160 | 0.400 | 13.0512 | -0.71 |
| VAN2 | VAN2 | -3.04172 | 10.00873 | 0.070 | 0.246 | 0.496 | 10.0085 | 0.25 |
| VAN211 | VAN211 | -2.00914 | 7.99960 | 0.230 | 0.107 | 0.327 | New Mark | New Mark |
| VAN200 | VAN200 | -4.32661 | 3.67298 | 0.096 | 0.230 | 0.480 | 3.6729 | 0.07 |
| VAN207 | VAN207 | 0.00689 | 3.67988 | 0.213 | 0.167 | 0.408 | 3.6797 | 0.19 |
| VAN208 | VAN208 | -0.69202 | 2.98785 | -0.118 | 0.209 | 0.457 | 2.9860 | 1.88 |
| VAN16 | VAN16 | 1.98718 | 4.97503 | -0.150 | 0.086 | 0.293 | 4.9769 | -1.83 |
|  | VAN14 | -1.40846 | 3.56658 | 0.100 | 0.030 | 0.174 | 3.5649 | 1.69 |
| VAN3 |  |  | 23.54630 |  |  |  |  |  |
| VAN204 | VAN204 | -1.02470 | 22.52160 | 0.246 | 0.102 | 0.319 | 22.5215 | 0.14 |
| VAN132 | VAN132 | -5.29730 | 17.22430 | 0.344 | 0.157 | 0.397 | 17.2243 | 0.02 |
| VAN201 | VAN201 | -0.97169 | 16.25261 | -0.188 | 0.152 | 0.389 | 16.2527 | -0.11 |
| VAN154 | VAN154 | 3.84186 | 20.09447 | 0.050 | 0.166 | 0.407 | 20.0946 | -0.14 |
| VAN209 | VAN209 | 2.12873 | 22.22320 | -0.037 | 0.117 | 0.342 | 22.2230 | 0.22 |
| VAN202 | VAN202 | 1.93281 | 24.15601 | 0.183 | 0.079 | 0.281 | 24.1558 | 0.26 |
| VAN205 | VAN205 | 6.91594 | 31.07195 | 0.138 | 0.192 | 0.439 | 31.0717 | 0.24 |
| VAN164 | VAN164 | 2.68749 | 33.75945 | -0.213 | 0.170 | 0.412 | 33.7580 | 1.43 |
| VAN206 | VAN206 | 1.99019 | 35.74964 | 0.013 | 0.882 | 0.939 | 35.7495 | 0.14 |
| VAN138 | VAN138 | 0.04188 | 35.79152 | 0.025 | 0.127 | 0.356 | 35.7913 | 0.19 |
| VAN166 | VAN166 | -1.41426 | 34.37726 | 0.027 | 0.077 | 0.277 | New Mark | New Mark |
|  | VANUBM | -1.46627 | 32.91098 | 0.236 | 0.100 | 0.317 | 32.9114 | -0.45 |
| VAN202 |  |  | 24.15601 |  |  |  |  |  |
| VAN100 | VAN100 | -2.94399 | 21.21202 | -0.175 | 0.121 | 0.348 | 21.2120 | 0.01 |
| PTVLBM | PTVLBM | -0.56201 | 20.65002 | 0.040 | 0.019 | 0.139 | New Mark | New Mark |
|  | PTVL | 0.96809 | 21.61811 | -0.038 | 0.019 | 0.139 | New Mark | New Mark |
| VAN206 |  |  | 35.74964 |  |  |  |  |  |
|  | VAN101 | 0.36956 | 36.11920 | -0.084 | 0.020 | 0.140 | 36.1192 | 0.01 |
|  |  |  | Misclose for all bays levelled = | 1.452 | 4.119 | 2.030 |  |  |

All levelling was performed within the project specifications of 2k

| From | To | Levelled Ht. Diff. | RL 2012 | Misclose (mm) | Dist. (km) | 1mmk | RL 2011 | Difference (mm) 2012 - 2011 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VANUBM |  |  | 32.91098 |  |  |  |  |  |
|  | VANURM1 | -1.0599 | 31.85112 | -0.010 | 0.025 | 0.157 | 31.85349 | -2.37 |
| VANUBM |  |  | 32.91098 |  |  |  |  |  |
|  | VANURM2 | -0.6862 | 32.22482 | -0.130 | 0.033 | 0.181 | 32.22511 | -0.29 |
| VANUBM |  |  | 32.91098 |  |  |  |  |  |
|  | VANURM3 | -0.8043 | 32.10667 | -0.121 | 0.037 | 0.192 | 32.10710 | -0.43 |
| VAN100 |  |  | 21.21202 |  |  |  |  |  |
|  | PTVLRM1 | 0.4434 | 21.65546 | 0.000 | 0.029 | 0.170 | New Mark |  |
| VAN100 |  |  | 21.21202 |  |  |  |  |  |
|  | PTVLRM2 | -0.3544 | 20.85761 | 0.053 | 0.040 | 0.200 | New Mark |  |
| VAN100 |  |  | 21.21202 |  |  |  |  |  |
|  | PTVLRM3 | -1.0793 | 20.13270 | -0.010 | 0.039 | 0.197 | New Mark |  |
| PTVL |  |  | 20.65002 |  |  |  |  |  |
|  | PTVL_MET | -1.4393 | 19.21071 | 0.010 | 0.055 | 0.234 | New Mark |  |

## Combined Comparisons

Examination of the level survey results do not show any significant changes in reduced levels from the 2011 survey. All indications are that no immediate or significant displacement of any of the deep driven bench marks has occurred.

The reduced levels derived after the 2002 earthquake, which particularly impacted the Port Vila wharf area, agree to approximately 1mm.

Table 1.2 Port Vila, Vanuatu  – Deep Bench Mark final RL's for Precise Differential Levelling (1993 - 2006) and EDM Height Traversing (2006 - 2012).

| Year | VAN1 | VAN2 | VAN14 | VAN16 | VAN100 | VAN101 | VANUBM | PTVLBM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1993.0 | 2.8537 | 10.0886 | 3.5690 |  |  |  |  |  |
| 1994.0 | 2.8533 | 10.0894 | 3.5686 |  |  |  |  |  |
| 1995.2 | 2.8542 | 10.0883 | 3.5693 | 5.0085 |  |  |  |  |
| 1997.1 | 2.8554 | 10.0885 | 3.5702 | 5.0100 |  |  |  |  |
| 1998.2 | 2.8558 | 10.0892 | 3.5708 | 5.0087 |  |  |  |  |
| 1999.6 | 2.8545 | 10.0890 | 3.5703 | 5.0091 | 21.2136 | 36.1163 |  |  |
| 2001.2 | 2.8560 | 10.0894 | 3.5716 | 5.0112 | 21.2130 | 36.1159 |  |  |
| 2002.7 | 2.8489 | 10.0083 | 3.5637 | 4.9747 | 21.2133 | 36.1185 | 32.9096 |  |
| 2004.4 | 2.8498 | 10.0087 | 3.5642 | 4.9760 | 21.2127 | 36.1182 | 32.9092 |  |
| 2006.7 | Destroyed | 10.0083 | 3.5636 | 4.9729 | 21.2144 | 36.1213 | 32.9124 |  |

| Year | VAN1 | VAN2 | VAN14 | VAN16 | VAN100 | VAN101 | VANUBM | PTVLBM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2006.7 |  | 10.0088 | 3.5628 | 4.9713 | 21.2130 | 36.1198 | 32.9111 |  |
| 2008.3 |  | 10.0082 | 3.5622 | 4.9744 | 21.2129 | 36.1194 | 32.9108 |  |
| 2009.9 |  | 10.0086 | 3.5636 | 4.9742 | 21.2132 | 36.1207 | 32.9115 |  |
| 2011.2 |  | 10.0085 | 3.5649 | 4.9769 | 21.2120 | 36.1192 | 32.9114 |  |
| 2012.8 |  | 10.0087 | 3.5666 | 4.9750 | 21.2120 | 36.1192 | 32.9110 | 20.6500 |

The 2012 RL of VANU is 33.8644

*The RL of the Reference Point VANU (ARP) is derived from adding the static height difference of 0.9534 (VANUBM to VANU) to the 2012 levelled RL of VANUBM

## Vanuatu 2012 Reduced Levels

Table 1.3 Port Vila, Vanuatu – Reduced levels 2012 survey
Date:	16 – 21 October 2012
Datum:	TGZ-ORSTOM (1993)

| Point ID | 2012 Reduced Level | Type |
| --- | --- | --- |
| VAN3 | 23.5463 | Stainless Steel Rod in Ground |
| VAN160 | 18.9868 | Galvanised Bolt in Concrete |
| VAN210 | 11.5461 | Stainless Steel Pin in Concrete |
| VAN143 | 13.0504 | Stainless Steel Pin in Concrete |
| VAN2 | 10.0087 | Stainless Steel Rod in Ground |
| VAN211 | 7.9996 | Stainless Steel Pin in Concrete |
| VAN200 | 3.6730 | Stainless Steel Pin in Concrete |
| VAN207 | 3.6799 | Stainless Steel Pin in Concrete |
| VAN208 | 2.9879 | Stainless Steel Pin in Concrete |
| VAN16 | 4.9750 | Stainless Steel Pin |
| VAN14 | 3.5666 | Galvanised Bolt on Concrete Wall |
| VAN204 | 22.5216 | Stainless Steel Pin in Concrete |
| VAN132 | 17.2243 | Stainless Steel Pin in Concrete |
| VAN201 | 16.2526 | Stainless Steel Pin in Concrete |
| VAN154 | 20.0945 | Metal Spike in Concrete |
| VAN209 | 22.2232 | Stainless Steel Pin in Concrete |
| VAN202 | 24.1560 | Stainless Steel Pin in Concrete |
| VAN205 | 31.0720 | Stainless Steel Pin in Concrete |
| VAN164 | 33.7594 | Stainless Steel Pin in Concrete |
| VAN206 | 35.7496 | Stainless Steel Pin in Concrete |
| VAN138 | 35.7915 | Stainless Steel Pin in Concrete |
| VAN166 | 34.3773 | Galvanised Bolt in Concrete |
| VANUBM | 32.9110 | Stainless Steel Pillar Pin |
| *VANU | 33.8644 | Pillar Plate (ARP) |
| VAN100 | 21.2120 | Stainless Steel Rod in Ground |
| PTVLBM | 20.6500 | Stainless Steel Pillar Pin |
| PTVL | 21.6181 | Stainless Steel Plate |
| VAN101 | 36.1192 | Stainless Steel Rod in Ground |
| VANURM1 | 31.8511 | Stainless Steel Rod in Ground |
| VANURM2 | 32.2248 | Stainless Steel Rod in Ground |
| VANURM3 | 32.1067 | Stainless Steel Rod in Ground |
| PTVLRM1 | 21.6555 | Stainless Steel Rod in Ground |
| PTVLRM2 | 20.8576 | Stainless Steel Rod in Ground |
| PTVLRM3 | 20.1327 | Stainless Steel Rod in Ground |
| PTVL_MET | 19.2107 | Top of Met Station |

*The RL of the Reference Point VANU (ARP) is derived from adding the static height difference of 0.9534 (VANUBM to VANU) to the 2012 levelled RL of VANUBM.

## Time Series of Bench Mark movement relative to Fixed Deep Bench Mark VAN3

Precise Differential Levelling: 1993 - 2006	
EDM Height Traversing: 2006 onwards

# Deep Bench Mark Locality Diagrams

# Vanuatu Reference Mark Locality Diagrams

# Temporary Holding Mark Locality Diagrams

# References

Rüeger, J.M. & Brunner, F.K. 1982, ‘EDM Height Traversing versus Geodetic Levelling’, The Canadian Surveyor, vol. 36, no. 1, pp. 69-87.

Inter-Government Committee on Surveying and Mapping (ICSM) 2002, Standards and Practices for Control Surveys, SP1, Ver. 1.5, May 2002.