EDM Height Traversing Levelling Survey Report

Nauru, September 2013

Geoscience Australia
RECORD 2014/23

S. J. K. Yates1, A. Lal2

National Geospatial Reference Systems, Minerals and Natural Hazards Division, Geoscience Australia GPO Box 378 Canberra ACT 2601

Secretariat of the Pacific Community (SPC)

Department of Industry

Minister for Industry: The Hon Ian Macfarlane MP
Parliamentary Secretary: The Hon Bob Baldwin MP
Secretary: Ms Glenys Beauchamp PSM

Geoscience Australia

Chief Executive Officer: Dr Chris Pigram
This paper is published with the permission of the CEO, Geoscience Australia

© Commonwealth of Australia (Geoscience Australia) 2014

With the exception of the Commonwealth Coat of Arms and where otherwise noted, all material in this publication is provided under a Creative Commons Attribution 3.0 Australia Licence. (http://www.creativecommons.org/licenses/by/3.0/au/deed.en)

Geoscience Australia has tried to make the information in this product as accurate as possible. However, it does not guarantee that the information is totally accurate or complete. Therefore, you should not solely rely on this information when making a commercial decision.

Geoscience Australia is committed to providing web accessible content wherever possible. If you are having difficulties with accessing this document please email clientservices@ga.gov.au.

ISSN 2201-702X (PDF)

ISBN 978-1-925124-12-5 (PDF)

GeoCat 78480

Bibliographic reference: Yates S. J. K, Lal A., 2014. EDM Height Traversing Levelling Survey Report, Nauru, September 2013. Record 2014/23. Geoscience Australia, Canberra. http://dx.doi.org/10.11636/Record.2014.023

Contents

1 Introduction	1

2 The Survey	2

2.1 Bench Mark Locations – Nauru	3

2.2 The Nauru Datum	3

2.3 Equipment	3

2.4 Method	4

2.5 Nauru 2013 Reduced Levels	4

2.6 Survey Support	6

2.7 Issues	6

2.8 Description of Marks – Nauru	6

3 Comparisons	7

3.1 Comparisons between 2013 and 2012 EDM Surveys	7

3.2 Combined Comparisons	8

3.3 Time Series of Bench Mark movement relative to Fixed Deep Bench Mark NAU1	10

4 Deep Driven Bench Mark Locality Diagrams	14

5 Temporary Holding Mark Location Diagrams	20

6 NAURBM and NAUR Reference Mark Locality Diagrams	29

7 References	33

# Introduction

This report outlines the high precision level survey completed between the Sea Level Fine Resolution Acoustic Measuring Equipment (SEAFRAME) tide gauge and the Continuous Global Navigation Satellite System (CGNSS) Station in Nauru from 5th – 12th September 2013.

Personnel involved in the survey were Steve Yates, Project Officer, Geoscience Australia and Andrick Lal, Surveyor, Secretariat of the Pacific Community (SPC).

The Electronic Distance Measurement (EDM) Height Traversing levelling technique was employed to observe differences in height between the deep bench mark arrays in Nauru, which runs approximately 3.7km from the tide gauge sensor to the CGNSS Station. Previous levelling surveys have been conducted along the route using this technique in 2005, 2007, 2009, 2010 and 2012.

In addition, precise differential levelling surveys were performed along the deep bench mark array from 1993 to 2006 by the National Tidal Centre Australia (NTCA) and the survey in 2005 included a comparison between the precise differential levelling and EDM height traversing technique. This report contains a comparison between the 2013 and 2012 EDM height traversing results as well as a combined comparison since the first levelling survey in 1993.

# The Survey

The EDM height traversing levelling survey was carried out between the SEAFRAME tide gauge sensor, continuous GNSS station and the deep driven bench mark array:

NAU1	– reference deep driven bench mark

NAU2	– deep driven bench mark

NAU36	– deep driven bench mark

NAU37	– deep driven bench mark

NAU38	– deep driven bench mark

NAU16	– deep driven bench mark

NAU14	– SEAFRAME sensor bench mark

NAU15	– SEAFRAME Project plaque bench mark

NAURBM	– reference bench mark for the GNSS pillar

Also included in the survey were temporary holding marks – NAU28, NAU100, NAU101, NAU102, NAU103, NAU104, NAU105, NAU106, NAU107, NAU108 NAU109, NAU33, NAU43, NAU44, NAU47, NAU53

The Continuous GNSS (CGNSS) Reference Point - NAUR and the three CGNSS Reference Marks – RM1, RM2 and RM3 were also levelled. All the deep bench marks were located and found in good order and undisturbed.

The EDM Height Traversing levelling technique was performed to the Class L2A specifications. After reduction an internal precision of 1mm √K or better was achieved, where K is distance in kilometres. This is well within the project specification of 2mm √K. A table of results and comparisons and the 2013 reduced levels are detailed later in this report.

## Bench Mark Locations – Nauru

## The Nauru Datum

The Datum for the survey is the Nauru Island Datum (NID). Reduction of the data has been calculated holding NAU1 fixed at 7.2930 metres, this value was determined by the NTC in 1993 by:

1993	Adopting the height of UH 1 as 4.130m NID

1994	Adopting the 1993 height of U2B as 3.8894m NID

## Equipment

LEICA total station model TCA2003 (Serial No:  440883)

LEICA precision prisms GPH1P (2)

LEICA rigid tripod

Stainless steel target poles supported by LEICA telescopic bi-poles (2)

Shortened stainless steel target pole for the SEAFRAME sensor BM connection

LEICA cast iron change plates (2)

KESTRAL 4000 pocket weather tracker

## Method

The “Leap-Frog” EDM height traversing technique was employed for the Nauru tide gauge levelling survey. This technique involves setting up a total station (TCA1800L) midway between two target/reflectors (on reflector rods with struts). The targets remain at a particular change point for the back-sight and fore-sight observations. The instrument measures slope distances (±1mm) and vertical angle (1”) to derive height differences (between the instrument’s trunnion axis and the reflectors). In support of the slope distance observations, the ambient temperature, pressure and humidity are recorded (Kestral 4000 pocket weather tracker) and input into the instrument to apply the first velocity correction to the observed distances (Rüeger & Brunner, 1982). Four rounds of observations are taken to the back-sight and fore-sight targets from each instrument setup. All levelling runs started and finished with the same reflector and reflector rod, i.e. an even number of setups when the two reflector rod configuration was used. This eliminates any reflector rod zero error. This technique can also be performed using a single set-up / single rod configuration which is particularly useful when levelling between bench marks which are close together e.g. between the CGNSS RMs.

Reduction of the digital data was computed by the Geoscience Australia levelling program “leveling1.exe”. This program computes the height difference between the two reflectors.

## Nauru 2013 Reduced Levels

Table 2.1 Nauru – Reduced levels 2013 survey
Date:	5 – 12 September 2013
Datum:	Nauru Island Datum

| Point ID | Reduced Level 2013 | Type |
| --- | --- | --- |
| NAU1 | 7.2930 | Stainless Steel Rod in Ground |
| NAU14 | 3.8477 | Stainless Steel Pin in Concrete |
| NAU15 | 6.0033 | Stainless Steel Pin |
| NAU36 | 7.2189 | Stainless Steel Rod in Ground |
| NAU28 | 7.7998 | Stainless Steel Pin in Concrete |
| NAU2 | 7.7851 | Stainless Steel Rod in Ground |
| NAU105 | 7.7828 | Stainless Steel Pin in Concrete |
| NAU100 | 8.2108 | Stainless Steel Pin in Concrete |
| NAU106 | 8.0465 | Stainless Steel Pin in Concrete |
| NAU37 | 7.6995 | Stainless Steel Rod in Ground |
| NAU38 | 7.5726 | Stainless Steel Rod in Ground |
| NAU107 | 7.7801 | Stainless Steel Pin in Concrete |
| NAU33 | 7.4100 | Stainless Steel Pin in Concrete |
| NAU16 | 6.6939 | Stainless Steel Rod in Ground |
| NAU108 | 6.8652 | Stainless Steel Pin in Concrete |
| NAU43 | 5.9204 | Stainless Steel Pin in Concrete |
| NAU44 | 6.4905 | Stainless Steel Pin in Concrete |
| NAU109 | 5.7124 | Stainless Steel Pin in Concrete |
| NAU47 | 5.8440 | Stainless Steel Pin in Concrete |
| NAU101 | 6.3153 | Stainless Steel Pin in Concrete |
| NAU102 | 6.3042 | Stainless Steel Pin in Concrete |
| NAU103 | 5.6239 | Stainless Steel Pin in Concrete |
| NAU104 | 5.1953 | Stainless Steel Pin in Concrete |
| NAU53 | 5.4837 | Stainless Steel Pin in Concrete |
| *NAUR | 6.2065 | Pillar Plate (ARP) |
| NAURBM | 5.2593 | Stainless Steel Pillar Pin |
| RM1 | 4.5476 | Stainless Steel Rod in Ground |
| RM2 | 4.3842 | Stainless Steel Rod in Ground |
| RM3 | 4.4824 | Stainless Steel Rod in Ground |

*The RL of the Reference Point NAUR (ARP) is derived from adding the static height difference of 0.9472m (NAURBM to NAUR) to the 2013 levelled RL of NAURBM.

## Survey Support

The survey team very much appreciated the assistance from staff of the Lands Department Nauru Office.

Personnel consulted during the visit were:

Mr. Peniasi Nakautoga (peniasi.nakautoga@gmail.com) – Chief Surveyor, Lands Department

Mr.Wess Tsitsi – Senior Surveyor, Lands Department

The Nauru Lands and Survey Department provided a trainee surveyor, Sebastian Gobune and chainman, Martin Aukbar to assist with the field survey. Wess Tsitsi has always taken a keen interest in the PSLMP and is always supportive with any associated project matters.

## Issues

It is recommended to establish an additional deep driven bench mark approximately midway along the airport road between NAU16 and the CGNSS bench mark NAURBM.

Also, the valve boxes for the CGNSS reference marks, RM2 and RM3 require replacement.

## Description of Marks – Nauru

NAU1	– reference deep driven bench mark (within wharf compound)

NAU2	– deep driven bench mark (within private residential compound)

NAU36	– deep driven bench mark (within Nauru Phosphate Head Quarters compound)

NAU37	– deep driven bench mark (on the corner of Catholic Church Hall)

NAU38	– deep driven bench mark (on the corner of Catholic Church)

NAU16	– deep driven bench mark (within airport runaway compound)

NAU14	– SEAFRAME sensor bench mark (within the Boat Harbour)

NAU15	– SEAFRAME Project plaque bench mark (within the Boat Harbour)

NAURBM	– reference bench mark for the GNSS pillar

Holding pins along survey route from tide gauge to CGNSS (as listed previously) are all stainless steel bolts, drilled in concrete and glued in place.

# Comparisons

## Comparisons between 2013 and 2012 EDM Surveys

Table 3.1 Results of Nauru 2013 EDM Height Traversing Comparison 2013 – 2012. NAU1 - adopted fixed height of 7.2930m

| From | To | Levelled Diff. Ht. | RL 2013 | Misclose (mm) | Dist. (km) | 1mm√k | RL 2012 | Difference (mm) 2013 - 2012 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAU1 |  |  | 7.2930 |  |  |  | 7.2930 |  |
| NAU14 | NAU14 | -3.4453 | 3.8477 | -0.160 | 0.104 | 0.322 | 3.8481 | -0.36 |
|  | NAU15 | 2.1555 | 6.0033 | 0.000 | 0.009 | 0.095 | 6.0052 | -1.95 |
| NAU1 |  |  | 7.2930 |  |  |  | 7.2930 |  |
| NAU36 | NAU36 | -0.0741 | 7.2189 | 0.404 | 0.186 | 0.431 | 7.2190 | -0.09 |
| NAU28 | NAU28 | 0.5809 | 7.7998 | -0.340 | 0.197 | 0.444 | 7.7987 | 1.10 |
| NAU2 | NAU2 | -0.0147 | 7.7851 | -0.210 | 0.167 | 0.409 | 7.7846 | 0.51 |
| NAU105 | NAU105 | -0.0023 | 7.7828 | -0.067 | 0.072 | 0.268 | 7.7824 | 0.39 |
| NAU100 | NAU100 | 0.4280 | 8.2108 | 0.000 | 0.097 | 0.311 | 8.2100 | 0.80 |
| NAU106 | NAU106 | -0.1643 | 8.0465 | 0.210 | 0.200 | 0.447 | 8.0458 | 0.69 |
| NAU37 | NAU37 | -0.3470 | 7.6995 | 0.113 | 0.118 | 0.344 | 7.6988 | 0.67 |
| NAU38 | NAU38 | -0.1269 | 7.5726 | -0.133 | 0.054 | 0.232 | 7.5722 | 0.37 |
| NAU107 | NAU107 | 0.2075 | 7.7801 | 0.100 | 0.202 | 0.449 | 7.7797 | 0.38 |
| NAU33 | NAU33 | -0.3700 | 7.4100 | -0.040 | 0.130 | 0.361 | 7.4094 | 0.64 |
|  | NAU16 | -0.7161 | 6.6939 | 0.319 | 0.123 | 0.350 | 6.6935 | 0.39 |
| NAU33 |  |  | 7.4100 |  |  |  |  |  |
| NAU108 | NAU108 | -0.5448 | 6.8652 | 0.340 | 0.178 | 0.422 | 6.8638 | 1.43 |
| NAU43 | NAU43 | -0.9448 | 5.9204 | 0.040 | 0.177 | 0.420 | 5.9197 | 0.70 |
| NAU44 | NAU44 | 0.5701 | 6.4905 | -0.310 | 0.204 | 0.451 | 6.4896 | 0.90 |
| NAU109 | NAU109 | -0.7781 | 5.7124 | 0.030 | 0.198 | 0.445 | 5.7107 | 1.74 |
| NAU47 | NAU47 | 0.1316 | 5.8440 | 0.375 | 0.199 | 0.446 | 5.8421 | 1.89 |
| NAU101 | NAU101 | 0.4713 | 6.3153 | 0.010 | 0.201 | 0.448 | 6.3134 | 1.87 |
| NAU102 | NAU102 | -0.0111 | 6.3042 | 0.230 | 0.199 | 0.446 | 6.3031 | 1.07 |
| NAU103 | NAU103 | -0.6802 | 5.6239 | -0.320 | 0.200 | 0.447 | 5.6225 | 1.44 |
| NAU104 | NAU104 | -0.4287 | 5.1953 | 0.160 | 0.199 | 0.446 | 5.1937 | 1.58 |
| NAU53 | NAU53 | 0.2884 | 5.4837 | -0.050 | 0.193 | 0.439 | 5.4825 | 1.18 |
|  | NAURBM | -0.2244 | 5.2593 | 0.063 | 0.074 | 0.272 | 5.2580 | 1.31 |
|  |  |  | Misclose for all bays levelled = | 0.763 | 3.678 | 1.918 |  |  |
| NAURBM |  |  | 5.2593 |  |  |  |  |  |
|  | RM1 | -0.7117 | 4.5476 | 0.031 | 0.035 | 0.186 | 4.5456 | 2.01 |
| NAURBM |  |  | 5.2593 |  |  |  |  |  |
|  | RM2 | -0.8751 | 4.3842 | 0.01875 | 0.023 | 0.151 | 4.3831 | 1.14 |
| NAURBM |  |  | 5.2593 |  |  |  |  |  |
|  | RM3 | -0.7769 | 4.4824 | -0.0125 | 0.026 | 0.161 | 4.481 | 1.38 |

All levelling was performed within the project specifications of 2k

## Combined Comparisons

Examination of the level survey results do not show any significant changes in reduced levels from the 2013 survey. All indications are that no immediate or significant displacement of any of the deep driven bench marks has occurred.

Table 3.2 Nauru – Deep Bench Mark final RL's for Precise Differential Levelling (1993 - 2005) and EDM Height Traversing (2005 - 2013).

| Year | NAU1 | NAU2 | NAU36 | NAU37 | NAU38 | NAU16 | NAURBM | NAU14 | NAU15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1993.52 | 7.2930 | 7.7837 |  |  |  |  |  | 3.8459 | 6.0044 |
| 1994.17 | 7.2930 | 7.7849 |  |  |  | 6.6915 |  | 3.8462 | 6.0041 |
| 1995.23 | 7.2930 | 7.7849 |  |  |  | 6.6923 |  | 3.8459 | 6.0038 |
| 1996.72 | 7.2930 | 7.7836 |  |  |  | 6.6923 |  | 3.8469 | 6.0048 |
| 1997.92 | 7.2930 | 7.7849 |  |  |  | 6.6920 |  | 3.8462 | 6.0027 |
| 1999.45 | 7.2930 | 7.7835 |  |  |  | 6.6916 |  | 3.8462 | 6.0032 |
| 2000.64 | 7.2930 | 7.7841 |  |  |  | 6.6920 |  | 3.8465 | 6.0050 |
| 2002.42 | 7.2930 | 7.7838 | 7.2191 | 7.6977 | 7.5711 | 6.6920 |  | 3.8464 | 6.0046 |
| 2003.86 | 7.2930 | 7.7842 | 7.2191 | 7.6979 | 7.5714 | 6.6915 | 5.2570 | 3.8472 | 6.0046 |
| 2005.79 | 7.2930 | 7.7846 | 7.2193 | 7.6987 | 7.5722 | 6.6934 |  | 3.8478 | 6.0064 |

| Year | NAU1 | NAU2 | NAU36 | NAU37 | NAU38 | NAU16 | NAURBM | NAU14 | NAU15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2005.79 | 7.2930 | 7.7851 | 7.2191 | 7.6994 | 7.5729 | 6.6939 | 5.2613 | 3.8476 | 6.0070 |
| 2007.45 | 7.2930 | 7.7857 | 7.2192 | 7.6998 | 7.5731 | 6.6944 | 5.2625 | 3.8479 | 6.0070 |
| 2009.10 | 7.2930 | 7.7847 | 7.2186 | 7.6992 | 7.5725 | 6.6935 | 5.2604 | 3.8479 | 6.0073 |
| 2010.54 | 7.2930 | 7.7860 | 7.2195 | 7.7013 | 7.5744 | 6.6961 | 5.2647 | 3.8472 | 6.0061 |
| 2012.17 | 7.2930 | 7.7846 | 7.2190 | 7.6988 | 7.5722 | 6.6935 | 5.2580 | 3.8481 | 6.0052 |
| 2013.68 | 7.2930 | 7.7851 | 7.2189 | 7.6995 | 7.5726 | 6.6939 | 5.2593 | 3.8477 | 6.0033 |

The 2013 RL of NAUR is 6.2065m

*The RL of the Reference Point NAUR (ARP) is derived from adding the static height difference of 0.9472m (NAURBM to NAURI) to the 2013 levelled RL of NAURBM.

## Time Series of Bench Mark movement relative to Fixed Deep Bench Mark NAU1

Precise Differential Levelling: 1993 - 2005	
EDM Height Traversing: 2005 onwards

# Deep Driven Bench Mark Locality Diagrams

# Temporary Holding Mark Location Diagrams

# NAURBM and NAUR Reference Mark Locality Diagrams

# References

Rüeger, J.M. & Brunner, F.K. 1982, ‘EDM Height Traversing versus Geodetic Levelling’, The Canadian Surveyor, vol. 36, no. 1, pp. 69-87.

Inter-Government Committee on Surveying and Mapping (ICSM) 2002, Standards and Practices for Control Surveys, SP1, Ver. 1.5, May 2002.